3GPP TSG RAN Meeting #109												RP-252137

Beijing, China, September 15-18, 2025

Agenda Item:	8.3

Source:	Huawei, HiSilicon

Title:	Consideration on cell management and multi-carrier operation

Document for:	Discussion and decision

Introduction

In the RAN#108 meeting, a New SID: Study on 6G Radio was approved [1]. Its objectives include the following:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

In this contribution, we analyzed the current status of spectrum allocation and gave considerations for cell management and multi-carrier in 6GR, including efficient utilization of non-contiguous spectrum and flexible spectrum deployment.

Spectrum status

By observing the available 1.4~2.6 GHz spectrum of 60+ operators over the world, Figure 1 shows the distribution of FDD carrier bandwidth as well as the number of FDD carriers owned by each operator. It can be observed that the bandwidth no larger than 30MHz occupies 95% of contiguous 1.4~2.6 GHz FDD spectrum, and 90% operators own spectrum in more than one band within frequency range 1.4~2.6 GHz.

Figure 1. Non-contiguous FDD spectrum over 1.4~2.6GHz spectrum of 60+ operators

Similarly, Sub1-GHz (700/800/900 MHz) spectrum also faces the same situation. Figure 2 shows the status of non-contiguous FDD spectrum over sub1-GHz spectrum of 60+ operators. It can be observed that the bandwidth no larger than 15 MHz occupies 93% of contiguous sub1-GHz FDD spectrum, and 71% operators own spectrum in more than one band within sub1GHz frequency range.

Figure 2. Non-contiguous FDD spectrum over Sub1-GHz (700/800/900MHz) spectrum of 60+ operators

In addition to the aforementioned sub-3GHz bands, operators may have spectrum resources in around 3 GHz to 6 GHz range (such as 3.5 GHz, 4.9 GHz, etc.). Furthermore, candidate new spectrum resources have been discussed, ranging from around 7 to 24 GHz, such as 6.425 – 7.125 GHz (identified in WRC-23 meeting [2]), 7.125 – 8.400 GHz, and 14.800 – 15.350 GHz (recommended for WRC-27).

Figure 3. Candidate spectrum resources for operators

In this case, it is typical that one operator has multiple diverse spectrum resources as shown in Figure 3, and how to efficiently and flexibly utilize those spectrum resources will become a challenge faced by mobile operators.

Consideration on Multi-spectrum and Cell Management

Non-contiguous spectrum utilization efficiency

In NR, CA is widely used by operators to aggregate spectrum resources. The existing CA mechanism treats each carrier as an independent serving cell and assumes each carrier is deployed independently. This may introduce unnecessary overhead and latency, which are discussed in details below.

The CA procedures include (de)configuration and (de)activation of SCells, and such procedures will be triggered due to user mobility, different traffic type and network load control. If the changes happen frequently, e.g. the UE moves back and forth among several cells, these procedures will have to repeat multiple times and bring non-negligible signaling overhead, which leads to inefficient spectrum utilization. For example, the configuration overhead for a single SCell at low frequencies is approximately 500 bytes, while at high frequencies, it is about 1.5k bytes. Each SCell (de-)configuration and (de-)activation procedure also leads to a certain delay. For SCell (de-)configuration, considering RRC signaling transmission delay and UE processing delay (as shown in Table 1), the latency is around 20ms. For SCell activation, although MAC CE itself only contributes ~5ms latency, the whole procedure will also include HARQ transmission, A-TRS transmission, CSI measurement and valid CSI reporting as shown in Figure 4, with an overall delay of approximately 20~25 ms as defined in 38.133. Such a latency will reduce user perceived data rate and may not serve applications well such as immersive communication, mobile AI applications.

Table 1: UE performance requirements for RRC procedures for UEs [3]

| Procedure title: | Network -> UE | UE -> Network | Value [ms] | Notes |
| --- | --- | --- | --- | --- |
| RRC Connection Control Procedures |  |  |  |  |
| RRC reconfiguration | RRCReconfiguration | RRCReconfigurationComplete | 10 |  |
| RRC reconfiguration (scell addition/release) | RRCReconfiguration | RRCReconfigurationComplete | 16 |  |

Figure 4. SCell activation delay

Observation 1: The existing SCell (de)configuration and (de)activation still brings signaling overhead and latency to 40~45ms for the whole procedure, which is not efficient for spectrum utilization and not sufficient for user experience.

In addition, under the CA mechanism, cell management is performed on a per-carrier basis for each frequency band. In this case, every carrier needs to configure common control signaling, such as SSB, system information etc. which will cause ~10% overhead assuming the initial access bandwidth is 20MHz with 30kHz SCS, and lead to a decrease in spectral efficiency.

In NR, features like SSB-less have been introduced in Rel-15 for intra-band case, and Rel-18 for inter-band case. Rel-18 inter-band case can solve such overhead issue, but as an add-on feature, it may not be easy for wide commercialization due to accommodation of legacy UEs. In addition, SSB-less features only apply to CA framework, which means they can only be used when UE enters into RRC_CONNECTED mode. In other words, the PCell has to be fixed on a particular band to save overhead from other bands, and all UEs have to initiate random access to the PCell on the same band. This brings limitation for efficient spectrum utilization during initial access, and should be resolved in 6G Day One.

Observation 2: The existing CA framework brings overhead on common control signal up to 10% for each carrier, which is not efficient for spectrum utilization for both initial access and data transmission.

According to the above analysis, it is worthwhile to consider a new framework for multi-carrier operation and cell management for more efficient spectrum utilization, to further reduce the latency and the system overhead. Such a framework needs to be addressed in 6G Day One as this will impact fundamental design for initial access mechanism and cell modeling.

Proposal 1: A new framework for multi-carrier operation and cell management to enable efficient multi-spectrum utilization shall be supported in 6G Day One, to further reduce latency and system overhead compared with existing CA framework.

Flexible Spectrum Deployment

In NR, each cell has to have one FDD band or one TDD band, whose downlink and uplink are associated in a fixed manner. SUL can be configured as an additional uplink for a cell. This brings limitation that the uplink configuration has to be tightly coupled with downlink configuration, which may not be flexible enough to address various operators practical deployment. Figure 5 shows two examples on the advantage of flexible uplink and downlink paring.

- Example a: achieve higher DL throughput and better UL coverage.

In the example (a) in Figure 5, for a cell edge UE with a high DL throughput requirement, operating both UL and DL in high frequency bands would limit the UL coverage, while operating both in low frequency bands would not meet the high throughput needs. Therefore, it is considered that the DL could operate in the mid-to-high frequency bands (e.g. 4.9 GHz) to achieve high throughput efficiency, while the UL could operate in the low frequency bands (e.g. 1.8 GHz) to achieve better UL coverage.

- Example b: enable flexible scheduling and load balancing.

In the example (b) in Figure 5, an operator has two frequency bands, one is with high UL load (e.g. 3.5G Hz) and another is with low UL load (e.g. 2.1 GHz). Since different users may have varying UL communication requirements, if the UL and DL pairing is fixed, a UE operates in 3.5GHz DL, has to also operate in 3.5GHz UL, which  may lead to 3.5GHz UL congestion. To solve this issue, it is considered that BS can provide flexibility to schedule the UL in 3.5GHz and/or 2.1GHz bands. For a UE with heavy UL traffic (e.g. UE1), BS schedules both UL bands to improve UL experience, whereas a UE with light UL traffic (e.g. UE2) will be offloaded to the UL in 2.1GHz band only. This approach facilitates flexible scheduling and load balancing.

Figure 5. Two examples of flexible DL/UL pairing

The above examples show requirements on flexible uplink and downlink paring, which is also seen beneficial from UE side. In NR, UL performance improvement is achieved at the cost of always binding to DL CC capabilities, and by flexible DL/UL pairing, more UL carriers could be utilized based on traffic demands without mandating equivalent DL CC capabilities.

To support this, we need to reconsider how to model a cell. In existing cell framework, downlink is mandatory to be configured and uplink configuration shall be associated with the downlink, and the cell is always identified by the downlink. If to support such a flexible uplink and downlink paring, there is a need to decouple the configuration from the two, and in this case whether a cell with uplink only should also be regarded as a cell needs to be discussed. This will impact the cell modeling and definition. Correspondingly, the flexibility of DL and UL configuration for TDD bands shall also be studied together for such a flexible pairing, to timely adapt to UE’s UL traffic demands and low-latency feedback requirements.

Proposal 2: Flexible uplink and downlink paring and corresponding cell framework on modeling and identifying a cell shall be supported.

Conclusion

According to the above discussion, we have provided observations on existing CA framework as below:

Observation 1: The existing SCell (de)configuration and (de)activation still brings signaling overhead and latency to 40~45ms for the whole procedure, which is not efficient for spectrum utilization and not sufficient for user experience.

Observation 2: The existing CA framework brings overhead on common control signal up to 10% for each carrier, which is not efficient for spectrum utilization for both initial access and data transmission.

It is therefore proposed to adopt the below proposals into TR 38.914 operational requirements:

Proposal 1: A new framework for multi-carrier operation and cell management to enable efficient spectrum utilization shall be supported in 6G Day One, to further reduce latency and system overhead compared with existing CA framework.

Proposal 2: Flexible uplink and downlink paring and corresponding cell framework on modeling and identifying a cell shall be supported.

References

- RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO, Prague, Czech Republic, June 9-13, 2025

- https://www.gsma.com/connectivity-for-good/spectrum/wrc-series/

- 3GPP TS 38.331, NR Radio Resource Control (RRC) Protocol Specification