3GPP TSG-RAN Meeting #109			RP-252480

Beijing, China, September 15-18, 2025

Agenda item:	9.6.3.3

Source:	Rapporteur (NTT DOCOMO, INC., AT&T)

Title:	Summary for WI additional topological enhancements for NR

WI code(s):	NR_WAB_5GFemto

Leading WG:	TSG RAN WG3

Release:	Rel-19

# 1	Introduction

This WI comprises of Wireless Access Backhaul (WAB) and 5G Femto.

For WAB, the following main functionalities have been achieved in this WI:

-	Support of a WAB-node including a WAB-gNB and a WAB-MT.

-	Support of backhauling of the WAB-gNB’s NG, Xn and OAM traffic over the WAB-MT’s PDU session(s).

-	Support of Xn interface(s) by the WAB-gNB with the WAB-MTs serving BH RAN node and with other surrounding gNBs, including how to avoid setting up Xn between WAB-gNBs.

-	Defining the behaviour of WAB-node in case the authorization status of WAB-MT and/or WAB-gNB changes.

-	Network integration procedures for WAB nodes.

-	Handling of WAB-gNB’s traffic (including Xn, NG and OAM traffic) during WAB-node mobility, including the case where the WAB-MT’s BH PDU session changes.

-	Support the UE’s AMF change for UEs connected to, or camped on, a WAB-gNB.

-	UE’s ULI that reflect the WAB node’s location.

For 5G Femto, the following main functionalities have been achieved in this WI:

-	support NR Femto architecture with optional NR Femto GW for NG interface

-	support access control for NR Femtos reusing existing CAG functionality

For WAB, corresponding stage-2 descriptions are specified in TS 38.300, TS 38.401. NG-AP related functionalities are specified in TS38.413. Xn-AP related functionalities are specified in TS38.423. Location services related functionalities are specified in TS 38.305 and TS 38.455.

For 5G Femto, corresponding stage-2 descriptions are specified in TS 38.300. NG related functionality is specified in TS 38.410. NG-AP related functionalities are specified in TS38.413.

# 2	Description

WAB

- A WAB-node consists of a WAB-gNB and a WAB-MT.

- WAB node integration:

  - WAB node integration procedure consists of WAB-MT setup, WAB-gNB setup, WAB-gNB initialization, NG connection setup and Xn connection setup.

- Configuration of WAB node:

  - When WAB node moves, the parameters that enable the WAB-gNB to select and connect to the AMF(s) that serve the UE(s), the parameters that enable the WAB-gNB to connect to, and communicate with, the OAM system, and the parameters that the WAB-gNB should broadcast, e.g., the TAC(s), the cell ID(s), the RANAC(s) may need to be updated.

  - A WAB-MT obtains IP address(es) for the PDU sessions in the same manner as a legacy UE.

  - The TAC/RANAC of WAB-gNB’s cell is configured by the OAM. During the mobility of WAB-node, it can be reconfigured by the OAM, or it can remain unchanged in a certain geographical area.

- NG connection management:

  - Based on the OAM configuration, the WAB-gNB can set up NG interface with an AMF.

- WAB node authorization:

  - WAB-node authorization includes authorization of the WAB-MT’s backhaul support, and service authorization of the WAB-gNB, as defined in TS 23.501.

- User Location information for UEs served by a WAB-gNB:

  - For UEs served by a WAB-gNB, in addition to the User Location Information (ULI), the WAB-gNB also provides the core network with Additional ULI, which includes a TAI and a NR CGI pertinent to the WAB-gNB’s broadcasted PLMN/SNPN.

- PCI collision avoidance:

  - In WAB-node deployments, the legacy mechanism for PCI collision avoidance can be reused. PCI space can be partitioned by allocating a range of PCIs to WAB-gNB cells.

- WAB-MT mobility:

  - The WAB-MT reuses legacy mobility procedures defined for the UE.

- WAB-gNB mobility:

  - Due to the WAB-gNB’s mobility, the UEs moving together with the WAB-gNB may have to change AMF. For the AMF change, a new logical WAB-gNB is instantiated, which establishes NG connection(s) towards one or more new AMF(s). The new logical WAB-gNB may obtain from the OAM the configuration parameters needed to establish the connection(s) to the UE’s new AMF(s), based on, e.g., WAB-node’s location. The new logical WAB-gNB may shall activate one or more cell(s) with new TAC, cell ID, and PCI, which depend on the WAB-node’s current location.

- Xn management:

  - A WAB-gNB can establish an Xn connection with the BH-gNB serving the co-located WAB-MT, and with the neighbouring gNBs. During the setup or update of its Xn connections, the WAB-gNB can include an ID of the WAB-MT, to indicate that it is a WAB-gNB. In case the peer gNB is the WAB-MT’s BH-gNB, the WAB-MT ID makes the BH-gNB aware of the co-location of the WAB-MT and the WAB-gNB. The WAB-MT ID consists of the C-RNTI assigned to the WAB-MT by the BH-gNB, and the cell ID of BH-gNB´s cell serving the WAB-MT.

  - Establishment of Xn connections between two WAB-gNBs can be avoided. To achieve this, the WAB-gNB may reject the Xn setup initiated by another WAB-gNB, e.g., based on the presence of the WAB-MT ID received in the XN SETUP REQUEST message.

5G Femto

- Overall Architecture and Functional split:

  - The NR Femto node may directly connect to the 5GC. The NG-RAN architecture may also deploy an NR Femto Gateway (NR Femto GW) to allow the concentration of the NG-C interface between the NR Femto node and the 5GC.

  - An NR Femto node hosts the same functions as a gNB, with the some additional specifics in case of connection to the NR Femto GW as specified in TS 38.300.

  - AMF hosts routing of handover messages and DOWNLINK RAN CONFIGURATION TRANSFER message towards NR Femto GWs based on the Selected TAI contained in these messages.

  - The NR Femto GW hosts relaying UE-associated NGAP messages between the AMF and the NR Femto node serving the UE and terminating non-UE associated NGAP procedures towards the NR Femto node and towards the AMF. The NR Femto GW supports TAC and PLMN ID used by the NR Femto node. The Femto GW Relays the PATH SWITCH REQUEST message towards the AMF indicated by the GUAMI of the source AMF received from the NR Femto node.

- Access control:

  - Cells served by an NR Femto node may be deployed as part of a PNI-NPN in order to restrict access to UEs according to the respective subscription.

# 3	References

[1] RP-251395, “Revised SID: Study on 6G Scenarios and requirements” CMCC, Verizon, NTT DOCOMO, Deutsche Telekom

[2] R3-256013, (BL CR to 38.300 for Femto) Introduction of NR Femto Architecture and Protocol Aspects

[3] R3-256014, (BL CR to 38.401 for WAB) Support for Wireless Access Backhaul

[4] R3-256015, (BL CR to TS 38.413 for Femto) Support of NR Femto architecture with NR Femto Gateway

[5] R3-256016, (BL CR to 38.423 for WAB) Support for Wireless Access Backhaul

[6] R3-256044, (BL CR to 38.305 for WAB) Support of Location Service Involving WAB-Nodes

[7] R3-256045, (BL CR to 38.410 for Femto) Introduction of NR Femto in NGAP list of functions

[8] R3-256046, (BL CR to 38.413 for WAB) Support for Wireless Access Backhaul

[9] R3-256047, (BL CR to 38.455 for WAB) Support of Location Service Involving WAB-Nodes