3GPP TSG RAN Meeting #109								RP-252823

Beijing, China, September 15-18, 2025

Agenda Item:		9.6.1.1

Source:			Samsung

Title:				WI summary for WI Core part: NR MIMO Phase 5

WI code(s):			NR_MIMO_Ph5

leading WG:		RAN1

Release:			Rel-19

### 1	Introduction

This work item (WI) introduced enhanced specification support for several key aspects on multi-input multi-output (MIMO) operation where Rel-15, 16, 17, and 18 NR were found deficient in terms of signalling latency, overhead, as well as spectral efficiency and coverage.

First, in the legacy specification, the network (NW) may configure/activate frequent periodic or semi-persistent beam reporting or trigger frequent aperiodic beam reporting to acquire the best/preferred beam for data/control transmissions. However, this results in large UL reporting and DL control signaling overhead/latency. Since UEs have more accurate knowledge of beam quality variation, the UE-initiated beam reporting procedure is introduced to provide more up-to-date and lower-overhead/latency event-driven beam reporting.

Second, in the legacy specification, PMI-based CSI reporting is limited to 32 CSI-RS ports, which limited its usage for upper FR1 bands where up to 128 CSI-RS ports can be utilized by the NW at a gNB. For this purpose, the existing PMI-based CSI reporting needs to be extended to support up to 128 CSI-RS ports – for both fully digital and hybrid MIMO architectures. Additionally, to facilitate coherent joint transmission (CJT) deployment with non-ideal backhaul where the transmit-receive-points (TRPs) are not calibrated in time/frequency/phase, CJT calibration report is introduced.

Third, in the legacy specification, NR supports 1-port, 2-port, 4-port and 8-port PUSCH transmission. To accommodate additional UE antenna architectures, enhancements on operating 3 transmit antennas for non-coherent UL transmission are introduced, comprising the required RS enhancement to facilitate 3-port PUSCH transmission (SRS, PTRS, and DMRS) as well as 3-port codebook and non-codebook-based UL transmissions.

Fourth, while the legacy specification supports various multi-TRP schemes, they target symmetric scenarios where all TRPs support both DL transmission and UL reception. To accommodate asymmetric DL sTRP/UL mTRP scenario which occurs in heterogeneous networks, Rel-19 introduces three features: 1) per-TRP pathloss offset configuration, 2) two closed loop power control (CLPC) adjustment states for SRS separate from PUSCH, 3) two timing advance (TA) values without restriction on multi-DCI-based multi-TRP.

### 2	Description

2.1 	Support for UE-initiated/event-driven beam management

The NW-configured/triggered periodic, semi-persistent and aperiodic beam reporting (e.g., N best beams and corresponding L1-RSRPs) have been specified since 5G-NR Rel-15. However, due to the lack of timely knowledge of beam quality changes in the NW side, large UL reporting and DL control signaling overhead/latency are incurred. In Rel-19, the UE-initiated/event-driven beam management/report procedure is introduced to provide more up-to-date beam reporting, yet with reduced reporting overhead and latency.

The Rel-19 UE-initiated/event-driven beam management supports the following two modes of UE-initiated beam reporting.

- In Mode A, upon detection of an event, the UE transmits a beam report on a PUSCH resource indicated by a DCI in response to a one-bit UE-initiated report indication (UEIRI).

  - Step 1: UE transmits the one-bit UEIRI on a pre-configured PUCCH resource to request a resource on PUSCH to later transmit a beam report.

  - Step 2: UE detects the DCI format to indicate the PUSCH resource to transmit the beam report.

  - Step 3: The beam report is transmitted on the PUSCH resource indicated by the DCI.

- In Mode B, upon detection of an event, the UE transmits the beam report on a type-1 configured-grant (CG) PUSCH associated to a pre-configured PUCCH resource carrying a one-bit UEIRI.

  - Step 1: UE transmits the one-bit UEIRI on the PUCCH resource, notifying the NW that a beam report will be transmitted on the CG-PUSCH.

  - Step 2: UE transmits the beam report in the CG-PUSCH.

  - Note: The one-bit UEIRI in Step 1 is in a separate reporting instance from the beam report in Step 2.

In order to accommodate different usages (e.g., link failure detection, timely using the best transmission beam, or timely update for activated TCI state(s) to accelerate the subsequent indication), the following three events are specified:

- Event-1: Quality of the current beam, in terms of L1-RSRP, is worse than a configured threshold.

- Event-2: Quality of at least one new beam, in terms of L1-RSRP, becomes a configured threshold value better than the current beam.

- Event-7: Quality of at least one new beam, in terms of L1-RSRP, becomes a configured threshold value better than that of the RS derived from the activated TCI state with the Q-th best quality, where the value of Q is RRC-configured.

Finally, to better facilitate commercial cross-carrier deployment, Rel-19 also supports cross-CC UE-initiated/event-driven beam reporting via enabling the cross-CC new beam RS indication and cross-CC current beam RS indication (i.e., via carrier and CCofIndicatedTCI-r19 in RRC CSI-report configuration, respectively).

2.2 	CSI support for up to 128 ports and CJT calibration

First, to extend the legacy PMI-based CSI reporting to accommodate up to 128 CSI-RS ports, the following enhancements are introduced in Rel-19:

- For 48, 64, and 128 CSI-RS ports, NZP CSI-RS resource aggregation is introduced to construct a channel measurement resource (CMR) from multiple legacy NZP CSI-RS resources (each with ≤ 32 ports).

- An enhanced Type-I codebook is introduced for 48, 64, and 128 CSI-RS ports where only wideband spatial-domain (SD) basis selection is used. The enhanced Type-I codebook comprises two modes of SD basis selection: one resembling the Rel-15 Type-I (joint selection across layers), the other using free SD basis selection across layers. Furthermore, the legacy Rel-16 eType-II and Rel-17 FeType-II port selection codebooks are extended by simply adding new number-of-port parameter (i.e. N1 and N2) values for 48, 64, and 128 CSI-RS ports.

- To support hybrid architecture, the legacy CRI-based reporting – where the best (CRI, RI, PMI, CQI) value is reported – is extended to facilitate reporting multiple pairs of (CRI, RI, PMI, CQI) values. In addition, the Rel-16 eType-II codebook can also be used for a limited configuration.

- To support low-complexity UE implementation for >4 layers in TDD scenarios, SRS port grouping is introduced to facilitate correspondence (needed for reciprocity-based DL CSI acquisition) between SRS ports and the two codewords.

Second, to ensure that CJT can be efficiently deployed with non-ideal backhaul (e.g. inter-site CJT, distributed MIMO), the following aperiodic stand-alone CSI reports on PUSCH are introduced in Rel-19 to assist the NW in calibrating N>1 TRPs utilized for CJT transmission:

- Inter-TRP delay offset reporting, which provides the relative delay offsets between (N-1) TRPs and a reference TRP. The (N-1) relative delay offsets are accompanied with (M-1) one-bit indicators informing the NW whether the resulting delay offset-plus-spread exceeds the cyclic prefix (CP) length or not. Periodic CSI-RS for tracking (TRS) is used to measure the quantities.

- The inter-TRP delay offset reporting can also be linked with an aperiodic CSI report configured with the Rel-18 Type-II CJT codebook to facilitate delay compensation for PDSCH transmission.

- Inter-TRP frequency offset reporting, which provides the relative frequency offsets between (N-1) TRPs and a reference TRP. Periodic CSI-RS for tracking (TRS) is used to measure the quantities.

- Inter-TRP DL/UL phase offset reporting (intended for TDD scenarios), which provides the relative DL/UL phase offsets between (N-1) TRPs and a reference TRP. One-port CSI-RS for CSI, along with an associated SRS resource, is used to measure the quantities.

2.3 	Support for 3TX uplink transmission

For 3TX codebook-based PUSCH transmission, only non-coherent TPMI and full-power mode 0 transmission are introduced. Also, up to 2 SRS resources can be included in an SRS resource set for codebook-based transmission where each of SRS resources comprises 4 ports with the last port muted. For 3TX non-codebook-based PUSCH transmission, up to 3 SRS resources can be included in an SRS resource set for non-codebook-based transmission, with each SRS resource comprising 1 port.

For both 3TX codebook and non-codebook-based PUSCH transmission, a UE capability is introduced to support the maximum PUSCH rank up to 3. Additionally, the bit-width of PTRS-DMRS association field in DCI format 0_1 and 0_2 is reduced to 1 when two PTRS ports are configured.

For 3TX SRS antenna switching, 3T3R and 3T6R are introduced. The 3-port SRS transmission is achieved by reusing a 4-port SRS resource with the last port muted. The possible number of SRS resources/SRS resource sets for 3T3R and 3T6R is same as the legacy xTxR (x=1,2,4,8) and xTyR (y=2x, x=1,2,4), respectively.

2.4 	Support for asymmetric DL sTRP/UL mTRP

To facilitate asymmetric multi-TRP scenario, the following three features are introduced.

- Pathloss offset configuration for PUCCH, PUSCH, SRS, and PRACH

- When a certain TRP (termed an UL TRP) is configured not to perform DL transmission, if a UE is configured to transmit UL channel/signal to the UL TRP, the UE doesn’t know the pathloss value from the UL TRP. To remedy this issue, the NW can indicate/configure pathloss offset (difference in the pathloss between the UL TRP and the UE, from the pathloss between the main TRP and the UE, where the main TRP can perform both DL transmission (therefore transmitting pathloss RS) and UL reception.

- For PUCCH, PUSCH, and SRS, the pathloss offset is configured only for the TCI state(s) corresponding to the UL TRP. For PDCCH-order-triggered PRACH, whether/how the pathloss offset is applied is indicated by a 1-bit new field in the triggering DCI.

- Two CLPC adjustment states for SRS separate from PUSCH

- In the legacy specification, only up to one CLPC adjustment state for SRS separate from PUSCH can be configured per serving cell. To facilitate asymmetric multi-TRP scenarios, up to two CLPC adjustment states for SRS separate from PUSCH can now be configured per serving cell.

- To indicate one of two CLPC adjustment states for SRS transmit power control (TPC) command separate from PUSCH, closed loop indicator field is introduced in DCI format 2_3. In addition, both SRS TPC command field and closed loop indicator field are introduced in DCI format 1_1.

- Two TAs (2TA) without restriction on multi-DCI-based multi-TRP

- 2TA was introduced in Rel-18 only for multi-DCI-based multi-TRP. To facilitate asymmetric multi-TRP scenarios, any of the TCI states can now be associated with any one of the two TA groups (TAGs) without such restriction.

### 3	References

[1]	RP-242394, Samsung (Moderator), WID revision: NR MIMO Phase 5