3GPP TR 23.732 V0.3.0 (2018-10)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Study on User data interworking, Coexistence and Migration

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

3GPP, 5G, Architecture, Coexistence, Interworking, Migration, Study

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2018, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	6

2	References	6

3	Definitions, symbols and abbreviations	7

3.1	Definitions	7

3.2	Abbreviations	7

4	Architectural Assumptions and Principles	7

4.1	Architectural Assumptions	7

4.2	Architectural Principles	8

5	Key Issues	8

5.1	Key Issue 1: Interaction between UDM and HSS with separate repositories	8

5.1.1	Description	8

5.2	Key Issue 2: Supporting a common repository for both HSS and UDM	9

5.2.1	Description	9

6	Solutions	9

6.1	Solution #1: Retrieving subscription data from separate UDR repositories for EPS and 5GS subscription data using a UDR Translation Function (UTF)	9

6.1.1	Introduction	9

6.1.2	High-level Description	9

6.1.3	Services and Illustrated Procedures	10

6.1.3.1	General	10

6.1.3.2	5GS to EPS Interworking when N26 is not supported	12

6.1.3.2.1	5GS to EPS handover	12

6.1.3.2.2	EPS to 5GS handover	13

6.1.3.3	Terminating Access Domain Selection (T-ADS)	15

6.1.3.3.1	Alternative 1	15

6.1.3.3.2	Alternative 2	15

6.1.3.4	P-CSCF restoration	16

6.1.4	Impacts on existing services and interfaces	16

6.1.5	Evaluation	16

6.2	Solution #2: Direct SBA UDM-HSS Interworking	16

6.2.1	Introduction	16

6.2.2	High-level Description	16

6.2.3	Services and Illustrated Procedures	17

6.2.3.1	General	18

6.2.3.2	Subscriber Profile Indications	19

6.2.3.3	Authentication	19

6.2.3.4	Mobility Scenarios	20

6.2.3.4.1	Mobility from 5GC to EPC, single registration, with N26 Interface	20

6.2.3.4.2	Mobility from EPC to 5GC, single registration, with N26 Interface	21

6.2.3.4.3	Interworking without N26 Interface	22

6.2.3.5	T-ADS	23

6.2.3.6	P-CSCF Restoration	23

6.2.4	Impacts on existing services and interfaces	24

6.2.5	Evaluation	25

6.3	Solution #3: UDM/HSS FE interaction reusing legacy protocol	25

6.3.1	Introduction	25

6.3.2	Solution on Authentication Procedure	25

6.3.2.1	Architecture	25

6.3.2.2	Illustrated procedure	26

6.3.2.3	Impacts on existing services and interfaces	29

6.3.2.4	Evaluation	29

6.3.3	Solution on IP address continuity of interworking without N26 from 5GS to EPS	29

6.3.3.1	Architecture	29

6.3.3.2	Illustrated procedure	30

6.3.3.3	Impacts on existing services and interfaces	31

6.3.3.4	Evaluation	31

6.3.4	Solution on supporting T-ADS for IMS voice	31

6.3.4.1	Architecture	31

6.3.4.2	Illustrated procedure	32

6.3.4.3	Impacts on existing services and interfaces	33

6.3.4.4	Evaluation	33

6.4	Solution #4: Use of a Data Access Layer for interaction between HSS FE and UDM with common repository	33

6.4.1	Introduction	33

6.4.2	High-level Description	33

6.4.2.1	Assumptions	33

6.4.2.2	Architectural proposal	34

6.4.3	Services and Illustrated Procedures	35

6.4.3.1	General description	35

6.4.3.2	5GS procedures	35

6.4.3.3	EPS procedures	36

6.4.3.4	Handover procedures	36

6.4.3.4.1	5GS to EPS	36

6.4.3.4.2	EPS to 5GS	37

6.4.3.5	IMS interaction	37

6.4.3.5.1	Access to IMS data	37

6.4.3.5.2	P-CSCF Restoration procedures	38

6.4.3.5.3	Terminating domain selection information for IMS voice	39

6.4.4	Impacts on existing services and interfaces	40

6.4.4.1	General	40

6.4.4.2	Summary of impacts on UDM	41

6.4.4.3	Summary of impacts on HSS FE	41

6.4.4.4	Summary of impacts on EPS UDR	41

6.4.5	Evaluation	41

6.5	Solution #5: Use of a Data Access Layer for interaction between HSS FE and UDM with separate repositories	41

6.5.1	Introduction	41

6.5.2	High-level Description	41

6.5.2.1	Assumptions	41

6.5.2.2	Architectural proposal	42

6.5.3	Services and Illustrated Procedures	43

6.5.3.1	General description	43

6.5.3.2	5GS procedures	43

6.5.3.3	EPS procedures	44

6.5.3.4	Handover procedures	44

6.5.3.4.1	5GS to EPS	44

6.5.3.4.2	EPS to 5GS	45

6.5.3.5	IMS interaction	46

6.5.3.5.1	Access to IMS data	46

6.5.3.5.2	P-CSCF Restoration procedures	46

6.5.3.5.3	Terminating domain selection information for IMS voice	47

6.5.4	Impacts on existing services and interfaces	48

6.5.4.1	General	48

6.5.4.2	Summary of impacts on UDM	49

6.5.4.3	Summary of impacts on HSS FE	49

6.5.5	Evaluation	49

7	Evaluation	49

8	Conclusions	49

Annex A:	Change history	50

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The objective of this Technical Report is to investigate scenarios for co-existence of subscription data pertaining to legacy system (EPS) with data pertaining to the 5G system, when the HSS+UDM defined in Rel-15 is separated and both UDM and HSS are independent, and will propose solutions to support this deployment where necessary.

NOTE 1:	The subscription data referred to by this Technical Report comprises both static as well as dynamic data, including IMS, LCS, SMS data.

-	study diverse scenarios related to the storage of subscription data including, but not limited to, those of common repository for subscription data from EPS and 5GS, as well as separate repositories for EPS and 5GS subscription data,

-	investigate the relevant procedures in TSs 23.502 (for e.g. authentication, mobility, handover and procedures for IMS and SMS handling in interworking between 5G system and EPS),

-	determine if there is a need for interaction between the consumers of EPS and 5GS data, in the different interworking cases and deployment scenarios.

Existing procedures in 23.502 shall not be impacted.

Provisioning aspects are outside this study.

Direct interactions between the EPS UDR and the 5GS UDR are out of the scope of this TR (i.e. it is assumed that there is no direct interaction between these two repositories).

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.502: "Procedures for the 5G System".

[3]	3GPP TS 23.335: "User Data Convergence (UDC); Technical realization and information flows; Stage 2".

[4]	3GPP TS 29.335: "User Data Convergence (UDC); User data repository access protocol over the Ud interface; Stage 3".

[5]	3GPP TS 23.002: "Network architecture".

[6]	3GPP TS 23.501: "System Architecture for the 5G System".

[7]	3GPP TS 23.228: "IP Multimedia Subsystem".

[8]	3GPP TS 23.380: "IMS Restoration Procedures".

[9]	3GPP TS 29.505: "5G System; Usage of the Unified Data Repository services for Subscription Data".

[10]	3GPP TS 29.228: "IP Multimedia (IM) Subsystem Cx and Dx Interfaces; Signalling flows and message contents".

[11]	3GPP TS 29.328: "IP Multimedia (IM) Subsystem Sh interface; Signalling flows and message contents".

[12]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[13]	3GPP TS 33.401: "System Architecture Evolution (SAE) – Security architecture".

[14]	3GPP TS 23.794: "Study on Enhanced IMS to 5GC Integration".

[15]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[16]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5G Repository: Repository hosting subscription data for 5G UEs.

5GS UDR: term used to identify the Unified Data Repository defined in TS 23.501 [6] (clause 4.2.5).

EPS UDR: term used to identify the User Data Repository defined in TS 23.335 [3] (clause 4.2.3).

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core

5GS	5G System

5GS UDR	5G Unified Data Repository

HSS FE	Home Subscriber Server Front End

EPS	Enhanced Packet core System

EPS UDR	EPS User Data Repository

E-UTRAN	Evolved Universal mobile telecommunications system Terrestrial Radio Access Network

UDM	Unified Data Management

# 4	Architectural Assumptions and Principles

Editor's note:	This clause will list general architectural assumptions and principles for this study.

## 4.1	Architectural Assumptions

-	The 5GS data storage architecture defined in TS 23.501 [6], for structured data, is used as the baseline network architecture.

-	The User Data Convergence concept specified in TS 23.335 [3], is used for reference to EPS/legacy system architecture for data storage, when layered architecture is deployed (i.e. application front-end decoupled from data repository using the standard interface Ud defined in TS 29.335 [4]).

-	The Home Subscriber Server (HSS) architecture specified in clause 4.1.1 of TS 23.002 [5], is used for reference to EPS/legacy system architecture for data storage, when non-layered HSS architecture is deployed (i.e. monolithic deployment where user data are stored within the entity itself (or in an external entity via non standard interfaces)).

## 4.2	Architectural Principles

In the UDC concept used for reference, HSS FE and EPS UDR defined in TS 23.335 [3] are assumed to interact for any data required by the study: no standardization will be considered for the interface between HSS FE and EPS UDR.

# 5	Key Issues

## 5.1	Key Issue 1: Interaction between UDM and HSS with separate repositories

### 5.1.1	Description

This key issue, which assumes the HSS is deployed using the UDC architecture in TS 23.335 [3], is to study how the UDM and HSS with separate repositories interact in the scenario that the HSS FE and UDM are separately deployed and the user data of 5G and 4G/3G/2G are also stored separately in different repositories (i.e. EPS UDR storing user data for 4G/3G/2G and IMS domains and 5GS UDR storing user data for 5G domain).

The interaction between UDM and HSS FE with separate repositories should be based on specific procedures that require data from one to the other.

The solutions to this key issue need to:

-	Clarify what user data is involved for the interaction between UDM and HSS FE with separate repositories;

-	Study the scenarios of interaction between UDM and HSS FE with separate repositories, specifically:

-	IMS and 5GS interworking required for the following cases:

-	Terminating Access Domain Selection (T-ADS).

When requested by IMS, the HSS+UDM shall be able to query the serving AMF for T-ADS related information (see clause 5.16.3.6 in TS 23.501 [6]).

-	P-CSCF Restoration

As defined in clause 5.8.4 in TS 23.380 [8], upon request by IMS when P-CSCF failure is detected, the HSS+UDM shall send a P-CSCF restoration indication to either the SMF serving the IMS PDU session for the UE or to the AMF serving the UE.

-	Network Provided Location Information (NPLI).

As defined in clause Y.6 in TS 23.228 [7], an IMS AS can trigger the retrieval of the user location and/or UE Time Zone information from the AMF via the HSS+UDM.

-	EPS and 5GS interworking required for mobility cases from EPS to 5GS and from 5GS to EPS.

-	Avoid the impact on existing procedures between HSS/UDM and the rest of the network.

## 5.2	Key Issue 2: Supporting a common repository for both HSS and UDM

### 5.2.1	Description

This key issue studies network deployments where a common repository (acting as EPS UDR and 5GS UDR) is shared between HSS and UDM, analyses the interactions between the HSS, UDM, & common repository, and examines the potential of shared data fields in the repository.

The solutions to this key issue need to:

-	Study the scenarios of interaction between UDM and HSS FE with common repository, specifically:

-	IMS and 5GS interworking required for the following cases:

-	Terminating Access Domain Selection (T-ADS).

When requested by IMS, the HSS+UDM shall be able to query the serving AMF for T-ADS related information (see clause 5.16.3.6 in TS 23.501 [6]).

-	P-CSCF Restoration

As defined in clause 5.8.4 in TS 23.380 [8], upon request by IMS when P-CSCF failure is detected, the HSS+UDM shall send a P-CSCF restoration indication to either the SMF serving the IMS PDU session for the UE or to the AMF serving the UE.

-	Network Provided Location Information (NPLI).

As defined in clause Y.6 in TS 23.228 [7], an IMS AS can trigger the retrieval of the user location and/or UE Time Zone information from the AMF via the HSS+UDM.

-	EPS and 5GS interworking required for mobility cases from EPS to 5GS and from 5GS to EPS.

# 6	Solutions

## 6.1	Solution #1: Retrieving subscription data from separate UDR repositories for EPS and 5GS subscription data using a UDR Translation Function (UTF)

### 6.1.1	Introduction

Solution #1 addresses Key Issue 1.

### 6.1.2	High-level Description

It is proposed to introduce a UTF (UDR Translation Function) to allow subscription data exchange between the HSS FE and the UDM when each function has a separate UDR repository. The UTF is a logical function that can be standalone or collocated with the UDM or HSS FE. The assumption with the proposed architecture is that the UDR database of the UDM contains the related 5G subscription data, whereas the UDR of the HSS FE contains the related EPS and IMS subscription data.

The UTF function contains a translation table that allows conversion of an Nudr to a Ud request and vice versa. If the UDM requires information from the EPSUDR , the UTF allows facilitating translation of an Nudr request to Ud request. If the HSS FE requires information from the EPS UDR the UTF is responsible to convert the Ud request to an Nudr request targeting specific subscription data based on the semantics defined in TS 29.505 [9].

NOTE:	The UTF would require to implement a translation table supporting the HSS FE vendor specific Ud implementation.

The architecture is illustrated below:

Figure 6.1.2-1: UDM/HSS FE interworking architecture

### 6.1.3	Services and Illustrated Procedures

#### 6.1.3.1	General

When the HSS FE or UDM determine that subscription data needs to be fetched from a different UDR an Ud or Nudr message requesting subscription details is sent to UTF. The UTF translates the message into the corresponding message towards the HSS FE (Ud message) or UDM (Nudr message) that is proxied towards the UDR (Option 1). The UTF may also provide the translated message directly to the target UDR repository bypassing the target HSS FE or UDM (Option 2).

An example procedure is illustrated below:

Figure 6.1.3.1-1: Procedure to retrieve subscription data from separate UDR repository

The following cases have been identified where subscription data are retrieved from different UDR:

-	5GS to EPS interworking when no N26 is supported: In 5GS to EPS handover, the HSS FE retrieves from the 5GS UDR the PGW-C+SMF ID in order to establish a UE requested PDN connection. In EPS to 5GS handover the UDM retrieves from the EPS UDR the PGW-C+SMF ID in order to establish a PDU session.

-	Support of T-ADS: When IMS core interfaces with the HSS FE and the UE is camped on a 5G network the HSS FE retrieves from the 5GS UDR the IMS over PS support and current RAT type.

#### 6.1.3.2	5GS to EPS Interworking when N26 is not supported

##### 6.1.3.2.1	5GS to EPS handover

When the HSS FE requires the PGW-C+SMF ID, the UTF must translate the vendor specific Ud request from the HSS FE to an Nudr_DM_Query service request including a Data Subset "UE Context in SMF data" towards the 5GS UDR (the Nudr request may be proxied via the UDM to the 5GS UDR).

Figure 6.1.3.2.1-1: HSS FE retrieves PGW-C + SMF ID from 5G UDR

If during 5GS to EPS interworking the UE indicates that dual registration is not supported the HSS needs to notify the UDM to deregister the UE from the 5G network. Such notification can be supported indirectly via the UDR. The steps required are:

-	The HSS FE creates a vendor specific Ud request to delete the 5G subscription data of the UE from the 5G UDR

-	The UTF translates the request into an Nudr_DM_Update message including information to remove all related 5G subscription data for the UE from the 5G UDR

-	The 5G UDR deletes the subscription data and notify the UDM of the result using an Nudr_DM_Notify service operation

-	The UDM notifies the AMF of the deregistration by invoking the Nudm_UECM_DeregistrationNotify service operation

The steps are shown in the Figure below:

Figure 6.1.3.2.1-1: Indirect procedure for the HSS to notify the UDM to deregister the UE from the 5G network

##### 6.1.3.2.2	EPS to 5GS handover

When the UDM requires the PGW-C+SMF ID the UDM sends an Nudr_DM_Query service request including a Data Subset "UE Context in SMF data" towards the UTF to retrieve the address. The UTF translates the Nudr request into a vendor specific Ud request in order to retrieve the same information from the EPS UDR.

Figure 6.1.3.2.2-1: UDM retrieves PGW-C + SMF ID from EPS UDR

If during EPS to 5G interworking the UE indicates that dual registration is not supported the UDM needs to notify the HSS to deregister the UE from the EPS network. The steps required are:

-	The UDM creates an Nudr_DM_Update request to delete the EPS subscription data

-	The UTF translates the request into a vendor specific Ud request

-	The EPS UDR deletes the subscription data and notify the HSS of the result

-	The HSS notifies the MME of the deregistration by invoking a Cancel Location request.

The steps are shown in the Figure below:

Figure 6.1.3.2.2-2: Indirect procedure for the UDM to notify the HSS to deregister the UE from the EPS network

#### 6.1.3.3	Terminating Access Domain Selection (T-ADS)

##### 6.1.3.3.1	Alternative 1

This alternative assumes the IMS network interfaces only with the HSS FE over Sh interface.

When the HSS FE requires the homogeneous IMS voice over PS supporting indication and the UE is camped on 5G network, the HSS FE constructs a Ud request to retrieve the subscription information from the 5G UDR

NOTE 1:	It is assumed that the HSS FE first sends a request to retrieve the related information from the 4G UDR. When the 4G UDR responds that there is no 4G related subscription content the HSS FE determines that the user is camped on 5G access and re-sends the Ud message towards the UTF.

The UTF must translate the vendor specific Ud request from the HSS FE to an Nudr_DR_Get service request including a Data Subset "Amf3GPPAccessRegistration" towards the 5GS UDR (the Nudr request may be proxied via the UDM to the UDR).

NOTE 2:	For T-ADS it is not possible to obtain the most recent access network capabilities for domain selection. The T-ADS makes the decision based on whether IMS over PS session is supported homogeneously over all TAs served by the AMF.

Editor's note:	Details of this alternative when there is dual registration is FFS.

##### 6.1.3.3.2	Alternative 2

This alternative assumes that the IMS AS can interface to both HSS and UDM over Sh interface. In such a case the IMS AS can obtain the most recent access network capabilities for domain selection by interrogating the HSS and UDM.

Editor's note:	Details of this alternative are FFS.

#### 6.1.3.4	P-CSCF restoration

The P-CSCF restoration is an optional feature allowing the IMS network to notify the HSS or UDM to re-establish the IMS related PDU session in case of P-CSCF failure.

One approach to support this feature is the S-CSCF to support a Cx interface to both HSS and UDM. In case of P-CSCF failure, the S-CSCF reports failure to the UDM or HSS.

Editor's note:	It is FFS how the S-CSCF determines to send the P-CSCF failure indication to the UDM or HSS in case the UE is operating in dual registration mode.

The UDM would support limited Cx functionality to be notified by the S-CSCF of a P-CSCF failure. Once notified the UDF invokes the procedures described in clause 5.8 of TS 23.380 [8].

NOTE:	The IMS network interfaces with the HSS (i.e. via Cx/Sh interfaces) for any IMS related subscription information.

Editor's note:	Additional use cases are FFS.

### 6.1.4	Impacts on existing services and interfaces

The impacts are as follows:

-	HSS FE

-	Identifies that subscription information is stored on a UDR repository containing 5GS subscription data

-	For option 1: proxies a Ud request/answer from the UTF/UDR respectively

-	UDM

-	Identifies that subscription information is stored on a UDR repository containing EPS subscription data

-	For Option 1: proxies an Nudr request/answer from the UTF/UDR respectively.

-	S-CSCF

-	When the network uses the P-CSCF restoration procedure via the AMF, the S-CSCF interrogates the HSS FE and the UDM in order to identify the serving network and report the P-CSCF failure

### 6.1.5	Evaluation

Editor's note:	This clause provides an evaluation of the solution.

## 6.2	Solution #2: Direct SBA UDM-HSS Interworking

### 6.2.1	Introduction

This solution is applicable to key issues 1 and 2.

This solution proposes a new Service Based Interface between UDM and HSS.

### 6.2.2	High-level Description

This contribution proposes a service based interface between HSS and UDM. The architecture for this solution is outlined in the following figure:

Figure 6.2.2-1: Architecture for Direct UDM-HSS Interworking with independent UDRs

Figure 6.2.2-2: Architecture for Direct UDM-HSS Interworking with a common repository

A new reference point between UDM and HSS within the combined HSS/UDM (i.e. Nxx as depicted in Figures 6.2.2-1 and 6.2.2-2) is proposed. The new interface for interworking between HSS and UDM is proposed to be based on SBI principles.

### 6.2.3	Services and Illustrated Procedures

Figure 6.2.3-1 shows a high-level illustration of the EPS/IMS and 5GS interworking procedures that requires interworking via HSS/UDM for procedures triggered from the EPS/IMS.

#### 6.2.3.1	General

Figure 6.2.3-1: High Level Illustration of HSS-UDM Interworking

1.	IMS/EPC interacts with HSS over existing interfaces for the purpose of e.g. Updating Location of an MME in HSS, and/or requesting HSS to execute some actions in the access network (e.g. P-CSCF restoration) or retrieve some information from the access network (e.g. TADS or NPLI information).

2.	Depending on the type of request, the HSS may be able to serve the request and reply to the request with no interaction with UDM (e.g. for ULR from MME or P-CSCF restoration request from S-CSCF). In this case, the HSS replies the request as in step 6.

Otherwise, the HSS interacts with UDM over the new Nxx reference point proposed in this solution using new service operations under the Nudm service set. The definition of these new service operations is out of the scope of the description of this solution.

In either case, in a layered architecture, the HSS interacts with the 4G/IMS UDR (using Ud). The HSS does not need to retrieve any subscription data belonging to the 5G subscription profile.

3.	UDM receives the request from HSS over the new Nxx reference point and interacts with the 5GC as needed using existing defined procedures. UDM interacts with the 5G UDR (using Nudr). The UDM does not need to retrieve any subscription data belonging to the 4G/IMS subscription profile.

Editor's note:	Analysis on the existing defined procedures in UDM that are used is FFS.

Depending on the request, UDM may omit the request to the 5GC and reply to HSS as in step 5 based on information stored under the 5GC subscription profile (e.g. in case of a TADS requests for which the AMF has already informed UDM that "Homogeneous support for Voice over IP" is supported).

In case of P-CSCF restoration or AMF deregistration, the UDM does not wait for the acknowledgement from 5GC, it sends the response back like in step 5.

4.	5GC replies the request from UDM including the requested information (e.g. TADS, NPLI) or confirming the execution of the requested action (e.g. P-CSCF restoration, deregistration of the UE from the AMF).

5.	UDM responds to HSS including the requested information (e.g. TADS, NPLI) or confirming the execution of the requested action (e.g. for the P-CSCF restoration request, it confirms the deregistration of the UE from the AMF).

6.	HSS responds to EPC/IMS. Depending on the request, the response from HSS will include information received from 5GC via UDM (e.g. TADS, NPLI).

Figure 6.2.3-2 shows a high-level illustration of the 5GS and EPS/IMS interworking procedures that requires interworking via HSS/UDM for procedures triggered from the 5GC.

Figure 6.2.3-2: High Level Illustration of UDM-HSS Interworking

In this case, an interaction between the 5GC and the UDM (e.g. an AMF registration in UDM in the context of a mobility scenario from EPC to 5GC using interworking with N26 procedures) triggers a request from UDM to HSS over the new Nxx reference point proposed in this solution using new service operations under a new service set offered by the HSS, e.g. Nhss service set.

Similar interactions as described for the HSS-UDM interworking for procedures triggered from the EPC/IMS apply here too. Depending on the procedure, the interactions between the UDM and the 5GC, the UDM and HSS and HSS and the EPS, may be executed asynchronously (e.g. the UDM may not wait to the response from HSS, not even to trigger the request to HSS to send a response to the 5GC).

#### 6.2.3.2	Subscriber Profile Indications

Different subscriber types may exist in a network with Nxx interface. For example, some subscribers will use the full network, i.e. 5GC, the EPC and the IMS, while others might use 5G and IMS but no EPC, and legacy subscribers might just remain limited to EPC and IMS services without subscription to the 5GC. The different subscription types determine, whether the use of the Nxx interface is required for a subscriber. Thus, a "5GC subscription indication" is part of the HSS subscription data as well as a "SMS via 5G NAS support" indication. Similarly, an "EPC subscription indication" and an "IMS subscription" indication are stored in the UDM subscription data.

Editor's note:	Requirement for "IMS subscription indication" to be confirmed.

#### 6.2.3.3	Authentication

For users with credentials stored in the HSS(AuC), (i.e. users with also a 2/3/4G subscription) the UDM needs to retrieve the authentication vectors from the HSS. The collocation of the 5G and 4G/3G/2G credentials is required to avoid distributed vectors generation lacking coordinated Sequence numbers.

Editor's note:	This procedure may not be the only option pending SA WG3 LS response.

Figure 6.2.3.3-1 shows the interaction in an interworking scenario with N26 interface, when the UE attaches to the 5GC, but authentication data are stored in the EPC-UDR.

Figure 6.2.3.3-1: Authentication in 5GC with credentials in HSS

1.	The UDM receives a Nudm_UEAuthentication_Get Request, containing the SUPI of the user.

2.	The UDM queries the UDR for the user profile belonging to the SUPI.

3.	The UDM receives the user profile, which includes the IMSI, the authentication method, an EPC subscription indication, but no authentication data.

4.	The UDM sends a Nhss_UEAuthentication_Get request to the HSS, containing the IMSI, Serving network information, and the Authentication Method (which specifies whether the 5G AKA or the EAP-AKA authentication methods is used)

5.	If the EPC UDR is used, the HSS reads the IMSI, SQN and other data from the EPC-UDR.

6.	The AuC as part of the HSS generates the 5G AKA or EAP-AKA authentication vectors.

7.	If the EPC UDR is used, the HSS updates the SQN in the EPC-UDR.

8.	The HSS provides the authentication vectors to the UDM in the Nhss_UEAuthentication Response.

9.	The UDM proceeds with the authentication.

#### 6.2.3.4	Mobility Scenarios

##### 6.2.3.4.1	Mobility from 5GC to EPC, single registration, with N26 Interface

In a single registration scenario, the attachment in an MME triggers a cancellation of the AMF address in the UDM/UDR.

Figure 6.2.3.4.1-1shows the interaction in an interworking scenario with N26 interface, when the UE attaches to the EPC.

Figure 6.2.3.4.1-1: Mobility from 5GC to EPC

1.	The HSS receives an S6a ULR request containing the IMSI of the subscriber.

2.	If the EPC UDR is used, the HSS reads the subscription information related to the IMSI from the EPC-UDR. In particular, it receives the 5GC subscription information.

3.	If the EPC-UDR is used, the HSS updates the EPC-UDR with the new MME address.

4.	The HSS responds to the MME with an S6a-ULA.

5.	If the 5GC subscription indication is received in step 2, i.e. if there is an 5GC subscription for the user, then the HSS uses the Nudm_UECM_Deregister service of the HSS for the IMSI of the user. This does not include the AMF address as when a AMF triggers that hence this is marked with a *.

6.	The UDM responds to the HSS.

7.	The UDM uses the Nudr_DM_Query service to retrieve the AMF address previously assigned to the UE, if any. If the UE is registered at an AMF, steps 8 and 9 are executed.

8.	If there is an AMF address found for the user, the UDM cancels the attachment by sending a Nudm_UECM_DeregistrationNotification to the AMF.

9.	In this case, the UDM also updates the UDR entry using the Nudr_DM_Update service.

##### 6.2.3.4.2	Mobility from EPC to 5GC, single registration, with N26 Interface

In a single registration scenario, the attachment in an AMF triggers a cancellation of the MME address in the HSS.

Figure 6.2.3.4.2-1 shows the interaction in an interworking scenario with N26 interface, when the UE attaches to the 5GC.

Figure 6.2.3.4.2-1: Mobility from EPC to 5GC

1.	The UDM receives an Nudm_UECM_Registration request containing the SUPI of the subscriber.

2.	The UDM queries the UDR with an Nudr_DM_query for the old AMF address and the EPC subscription indication belonging to the SUPI.

3.	The UDR provides the requested data to the UDM.

4.	The UDM updates the UDR with the new AMF registration data using the Nudr_DM_Update service.

5.	If the EPC subscription indication is received in step 3, i.e. if there is an EPC subscription for the user, then the UDM uses the Nhss_UECM_Deregister service of the HSS for the IMSI of the user.

6.	The HSS responds to the UDM.

7.	If the EPC UDR is used, The HSS reads the MME mobility information in the EPC-UDR. If the UE is registered at an MME, steps 8 and 9 are executed.

8.	If there is an MME address found for the user, the HSS cancels the attachment of the IMSI in the MME by sending a CLR via the S6a interface.

9.	IF step 8 was executed, and EPC UDR is used, the HSS also updates the EPC-UDR entry to remove the MME assignment.

##### 6.2.3.4.3	Interworking without N26 Interface

For interworking without the N26 interface in Rel-15, IP address preservation is provided to UEs on inter-system mobility by storing and fetching PGW-C+SMF and corresponding APN/DDN information via the HSS+UDM, see TS 23.501 [6] clause 5.17.2.3.1. In an architecture with Nxx interface between HSS and UDM, this information must be exchanged via the Nxx interface.

For mobility from 5GC to EPC, the HSS can use the Nudm_SubscriberDataManagement (SDM) service for "UE context in SMF" data.

For mobility from EPC to 5GC, the HSS Nhss_SubscriberDataManagement service for the "UE context in PGW" provides the PDN-GW address per subscriber and APN. The UDM can use this service.

This service is also applicable for interworking with non-3GPP access if IP address preservation is requested.

#### 6.2.3.5	T-ADS

Figure 6.2.3.5-1 shows the interaction when the SCC-AS sends a T-ADS query to the HSS, and the user is subscribed to 5GC as well.

Figure 6.2.3.5-1: Terminating Access Domain Selection

1.	The HSS receives a UDR T-ADS query from the SCC-AS over the Sh interface.

2.	If EPC-UDR is used, the HSS reads the MME, SGSN, T-ADS related information and the 5G subscription indication from the EPC-UDR.

3.	Depending on registration state, the HSS queries the MME and/or SGSN for IMS over PS session support.

4.	If the user has a 5G subscription, the HSS requests 5GC-related T-ADS information from the UDM, using the Nudm_MT_ProvideDomainSelectionInfo service provided by the UDM. The HSS includes an indication, whether a time stamp of last attachment is required.

5.	On receipt of the request, the UDM queries the UDR for the AMF registration state and the stored T-ADS related information (e.g. homogenous support).

6.	If the user is registered in an AMF and there is no information regarding homogenous support or non-support, or if a time stamp has been requested by the HSS, the UDM sends an Namf_MT_ProvideDomainSelectionInfo service request to the AMF.

7.	The AMF responds with the T-ADS information.

8.	The UDM passes the T-ADS information to the HSS - in the response to the Nudm_MT_ProvideDomainSelectionInfo request.

9.	The HSS combines the information related to SGSN, MME and AMF and provides it to the SCC-AS in the Sh-UDA message.

#### 6.2.3.6	P-CSCF Restoration

Figure 6.2.3.6-1 shows the interaction when the S-CSCF initiates a P-CSCF restoration at the HSS, and the user is attached to 5GC. In this example the AMF based restoration is shown. The procedure for the SMF follows the same approach and same method across the Nxx interface.

Figure 6.2.3.6-1: P-CSCF Restoration

1.	The HSS receives a Cx-SAR (Server Assignment Request) message from the S-CSCF, which includes the P-CSCF restoration indication.

2.	If the EPC-UDR is used, the HSS reads the subscriber data from the EPC-UDR.

3.	If applicable, the HSS sends a P-CSCF restoration information towards the supporting nodes (SGSN, MME) and performs the unregistration/deregistration of the UE in IMS, as described in step 5 of TS 23.380 [8], clause 5.4.2.1. This includes the necessary updates in the EPC-UDR.

4.	The HSS replies with a Cx SAA (Server Assignment Answer) to the S-CSCF.

5.	In case of 5GC subscription indication, the HSS uses the enhanced Nudm_UECM_PcscfRestoration service of the UDM to trigger the P-CSCF restoration indication in the 5GC.

6.	The UDM reads the subscriber profile in the UDR to retrieve the relevant AMF address and callback-URI.

7.	(copy of step 6 in TS 23.380 [8], clause 5.8.4.3) The UDM sends Nudm_UECM_PcscfRestoration notification to the AMF serving the UE, using the received callback URI for P-CSCF restoration notifications. The AMF accepts the Nudm message and sends a response message to the UDM.

8.	The UDM responds to the HSS. Note: Message 8 may be sent at the same time like message 7 and need not wait for the response to message 7.

### 6.2.4	Impacts on existing services and interfaces

Following impacts are identified:

a)	Impacts in HSS:

New SBA service at least with one operation Request/Response, at described in figure 6.2.3-2.

This SBA service has to follow registration/discovery/selection procedures (by means of interaction with NRF or by configuration) as defined for Rel-15 or its evolution/adaptation to Rel-16.

No impacts to existing interfaces or procedures.

b)	Impacts in UDM

New SBA service at least with one operation Request/Response, at described in figure 6.2.3-1.

Editor's note:	Work in progress, further impacts could be identified.

### 6.2.5	Evaluation

Editor's note:	This clause provides an evaluation of the solution.

## 6.3	Solution #3: UDM/HSS FE interaction reusing legacy protocol

### 6.3.1	Introduction

This solution addresses Key Issue #1: Interaction between UDM and HSS with separate repositories.

The assumption of this solution is that 5GS UDR and EPS UDR are separate, between which there is no interface for direct interactions. The EPS profile is stored in EPS UDR and the 5GS profile is stored in 5GS UDR.

UDM and HSS FE interacts to retrieve or update the required user data for the following procedures:

-	Authentication procedure, i.e., initial attach/registration.

-	Network Provided Location Information (NPLI).

-	Procedure on IP address continuity of interworking without N26 from 5GS to EPS.

-	T-ADS query for IMS voice

Editor's note:	It is FFS to identify IMS related procedures on UDM and HSS FE interacts.

To avoid the impact on legacy HSS, the interactions for retrieving or updating user data are initiated by UDM through reusing existing HSS supported protocol and procedures, i.e. this solution only requires UDM to retrieve or update user data stored in EPS UDR via HSS FE. The interface between HSS FE and EPS UDR is also not impacted.

### 6.3.2	Solution on Authentication Procedure

#### 6.3.2.1	Architecture

A new interface, Shu, between UDM and HSS FE is introduced, by which UDM initiates interactions with HSS FE to retrieve or update user data stored in EPS UDR. Shu is a Point-to-Point reference point.

UDM is the only access point both to 5GC and EPC.

Figure 6.3.2.1.1: Solution architecture

In some 5G deployment, when a 4G subscriber is updated to 5G subscriber, the 4G subscription profile keeps stored in EPS UDR while the new 5G subscription data is provisioned in 5GS UDR. In this case it is assumed that the authentication data is only stored in EPS UDR and is reused by 5GS authentication.

Editor's note:	It is FFS if the authentication data is fully valid for 5GS.

For new 5G subscriber, all the subscription data is stored in 5GS UDR. Therefore, the solution described in this clause does not apply.

In this scenario, when a UE with 5GS subscription registers on 5GS, the UDM needs to interact with HSS FE to retrieve authentication information data stored in EPS UDR.

NOTE 1:	The UDM is a combo node which simultaneously supports UDM and HSS FE functionality.

In case of single registration in 5G, in order to cancel the registration in the other RAT, the UDM supports to perform a fake 4G registration in the HSS, as shown in Figure 6.3.2.2-2. This trigger the HSS to cancel the 4G registration. When the UE moves back to 4G, the HSS sends cancel location to UDM which trigger the UDM to deregister the UE in 5G.

#### 6.3.2.2	Illustrated procedure

The procedure of the interaction between UDM and HSS FE for Authentication Information Retrieval is shown in Figure 6.3.2.2.1.

Figure 6.3.2.2-1: Authentication Info Retrieval Procedure between UDM and HSS FE

1a.	When the UE attaches from EPS, MME sends Authentication-Information-Request to UDM to get authentication information for the UE. The MME message is sent to UDM by DRA routing (i.e., based on IMSI range).

1b.	When the UE registers from 5GS, AMF invokes Nausf_UEAuthentication_authenticate service operation to AUSF to get authentication information for the UE.

1b'. AUSF invokes Nudm_UEAuthentication_Get service operation.

2.	When UDM receives Nudm_UEAuthentication_Get, it sends S6a Authentication-Information-Request message to HSS-FE, if the authentication information is in EPS UDR.

Editor's note:	Whether there are HSS logics in UDM and how UDM identifies the need to rout to HSS-FE is FFS.

3.	HSS FE interacts with EPS UDR to retrieve authentication information of the UE via legacy UDC architecture.

4.	HSS FE returns Authentication-Information-Answer with authentication vectors to UDM.

5a.	UDM forwards Authentication-Information-Answer with authentication vectors to MME.

5b'.	UDM generates the respond to Nudm_UEAuthentication_Get service operation invocation with authentication vectors received in Authentication-Information-Answer, and sends the respond to AUSF.

5b.	AUSF response the AMF invoked Nausf_UEAuthentication_authenticate service.

Editor's note:	This procedure may be modified based on SA WG3 LS response.

The procedure of the interaction between UDM and HSS FE for single Registration is shown in Figure 6.3.2.2-2.

Figure 6.3.2.2-2: Single Registration Procedure between UDM and HSS FE

1.	The UE is registered in 5G. The AMF perform UE registration in UDM. The AMF indicates whether it requires single registration or dual registration.

2.	If the AMF requires single registration then the following steps are performed. Otherwise the procedure stops.

3.	The UDM selects the HSS based on the SUPI and sends Update Location Request to HSS. In this message a dedicated MME Identity is included. This dedicated MME Identity is pointing to the UDM. From HSS point of view, the UDM is now acted as a MME. This message also includes an indication to indicate single registration and an indication to indicate the skip of subscription data so the HSS will not send the UE 4G subscription data.

4.	The HSS stores the dedicated MME Identity into EPS UDR via Ud interface.

5.	The HSS sends Update Location Response to UDM.

6.	If the UE has been registered in an old MME, the HSS sends Cancel Location Request towards this old MME. This message will be routed to MME directly based on the old MME Identity.

7.	The old MME deregisters the UE and sends Cancel Location Response to the HSS.

8.	After the UE reselects 4G and perform Tracking Area Update procedure in 4G, the new MME sends Update Location Request towards the HSS. As the UDM is the only access point both to 5GC and EPC, this message is sent to UDM and the UDM relays this message to HSS.

9.	The HSS stores the new MME information into EPS UDR via Ud interface.

10.	The HSS sends Update Location Response to new MME via UDM including the UE 4G subscription.

11.	Because the UE has been registered in 5G and a dedicated MME Identity pointing to the UDM is stored in the EPS UDR, the EPS UDR triggers the HSS to send Cancel Location Request towards the UDM. This message will be routed to UDM based on the dedicated MME Identity.

12.	UDM sends Cancel Location Response to HSS.

13.	Upon receiving Cancel Location Request from HSS targeting to deregister the UE in 5G, the UDM initiated Deregistration procedure to deregister the UE in 5G.

#### 6.3.2.3	Impacts on existing services and interfaces

UDM supports:

-	generating Diameter S6a Authentication-Information-Request message based on Nudm_UEAuthentication_Get service operation;

-	forwarding Diameter S6a Authentication-Information-Answer message with authentication vectors to MME;

-	generating the respond to Nudm_UEAuthentication_Get service operation invocation with authentication vectors received in Authentication-Information-Answer, and sends the respond to AMF;

-	generating and sending Location Update Request message to perform fake 4G registration in HSS when the UE is registered in 5G;

-	de-registering the UE in 5G when it receives Cancel Location request with dedicated MME Identity from the HSS.

HSS FE: No impact.

5GS UDR: No impact.

EPS UDR: No impact.

#### 6.3.2.4	Evaluation

Editor's note:	This clause provides an evaluation of the solution.

### 6.3.3	Solution on IP address continuity of interworking without N26 from 5GS to EPS

#### 6.3.3.1	Architecture

Shu interface defined in clause 6.3.2.1 is reused.

UDM is the only access point both to 5GC and EPC.

Figure 6.3.3.1.1: Solution architecture

When a UE initially registers in 5GC, the FQDN for the S5/S8 interface of the PGW-C+SMF is registered into UDM via Nudm as specified in clause 4.11.2 of TS 23.502 [2].

When UE attaches in EPS due to interworking without N26, the procedure specified in clause 4.11.2 of TS 23.502 [2] applies. When MME sends Update Location Request to UDM, after forwarding the request to HSS, UDM needs to retrieve the stored PGW-C+SMF FQDN and add it into the Update Location Ack message returned from HSS.

The MME use PGW-C+SMF FQDN received from the Update Location Ack message to determine the address of PGW-C+SMF.

#### 6.3.3.2	Illustrated procedure

The procedure of the interaction between UDM and HSS FE is shown in Figure 6.3.3.2.1.

Figure 6.3.3.2-1: IP continuity procedure for interworking without N26 from 5GS to EPS

0.	PGW-C+SMF registers its FQDN in UDM during registration procedure. UDM stores the FQDN in 5GS UDR.

1.	When the UE attaches in EPS due to interworking without N26 from 5GS to EPS, during the procedure, MME sends Update Location Request to UDM.

2.	UDM forwards Update Location Request to HSS for further handling.

3.	HSS FE interacts with EPS UDR to handle Update Location Request.

4.	HSS FE returns Update Location Ack to UDM.

5.	after receiving Update Location Ack, UDM retrieve PGW-C+SMF FQDN from 5GS UDR.

6.	UDM generates a new Update Location Ack to add PGW-C+SMF FQDN into the message and forwards the message to MME.

NOTE:	the other signalling interactions not related to the solution are not present in this figure for simplification.

#### 6.3.3.3	Impacts on existing services and interfaces

UDM supports:

-	forwarding Diameter S6a Update Location Request message to HSS FE;

-	retrieving PGW-C+SMF FQDN from 5GS UDR after receiving Update Location Ack from HSS FE.

-	generating new Update Location Ack to add PGW-C+SMF FQDN into the message and forwarding the message to MME.

HSS FE: No impact.

5GS UDR: No impact.

EPS UDR: No impact.

#### 6.3.3.4	Evaluation

Editor's note:	This clause provides an evaluation of the solution.

### 6.3.4	Solution on supporting T-ADS for IMS voice

#### 6.3.4.1	Architecture

Shu interface defined in clause 6.3.2.1 is reused.

UDM is the only access point both to 5GC, EPC and IMS, i.e. UDM supports S6a and Sh interfaces along with Nudm.

Figure 6.3.4.1.1: Solution architecture

SCC AS initiates T-ADS query towards UDM via Sh interface as specified in TS 29.328 [11].

To handle T-ADS query, the UDM queries T-ADS information towards HSS FE to get T-ADS information related to EPS and UMTS. HSS FE may query MME and SGSN to get information as specified in TS 23.401 [16] and TS 23.060 [15].

After receiving response of T-ADS query from HSS FE, UDM responds T-ADS query to SCC AS based on the T-ADS information received from HSS and T-ADS information in UDM. UDM may query AMF to get necessary information as specified in TS 23.501 [6].

#### 6.3.4.2	Illustrated procedure

The procedure of the interaction between UDM and HSS FE is shown in Figure 6.3.4.2.1.

Figure 6.3.4.2-1: T-ADS Procedure between UDM and HSS FE

1.	SCC AS initiates T-ADS query towards UDM by sending a User-Data-Request message via Sh interface.

2.	UDM forwards this User-Data-Request message to get T-ADS information related to EPS and UMTS.

3.	HSS FE may query MME to get more T-ADS information as specified in TS 23.401 [16].

4.	HSS FE may query SGSN to get more T-ADS information as specified in TS 23.060 [15].

5.	HSS FE returns T-ADS information based on the information it derives by sending a User-Data-Answer message.

6.	after receiving T-ADS information from HSS FE, UDM may query AMF for more T-ADS information as specified in TS 23.501 [6].

7.	UDM determines T-ADS result based on T-ADS information received from HSS FE and AMF and responds to SCC AS with the T-ADS information by sending a User-Data-Answer message.

NOTE:	the other signalling interactions not related to the solution are not present in this figure for simplification.

#### 6.3.4.3	Impacts on existing services and interfaces

UDM supports:

-	forwarding User-Data-Answer message to HSS FE;

-	determine T-ADS result based on T-ADS information received from HSS FE and AMF.

HSS FE: No impact.

5GS UDR: No impact.

EPS UDR: No impact.

#### 6.3.4.4	Evaluation

Editor's note:	This clause provides an evaluation of the solution.

## 6.4	Solution #4: Use of a Data Access Layer for interaction between HSS FE and UDM with common repository

### 6.4.1	Introduction

This solution applies to Key Issue 2 and describes how HSS FE and UDM interact when data repositories are common, and N26 interface is used.

### 6.4.2	High-level Description

#### 6.4.2.1	Assumptions

It is assumed that:

-	UE 4G and 5G profiles in the common repository may be a common record for the UE or be split into different records.

-	The solution leverages EPS UDR to become the common repository for 4G and 5G subscription profiles to be accessed by means of the Ud reference point.

-	Technologies and protocols to access the data stored in the common repository are assumed to be database specific or proprietary, but exposing Ud operations to query, create, delete and update of data, as well as subscriptions/notifications to modifications of such data.

-	5G UEs are assumed to be provisioned in the repository and able to use 4G networks (with the corresponding limitations). 4G UEs are provisioned in the repository, as of today, but not assumed to be able to use 5G core services.

-	From an authentication perspective, this proposal assumes UDM to be UDM/ARPF as specified in TS 33.501 [12]) and it does not assume separation of authentication logic from the UDM. For EPS authentication, the security architecture specified in TS 33.401 [13] is assumed without changes. This implies execution of any cryptographic algorithms that use the long-term security credentials for authentication in each system to be separate procedures, while the security-related part of the subscriber profiles is assumed to be stored in the common repository.

Editor's note:	The final procedure is subject to confirmation from SA WG3.

-	The IMS data for the UEs is assumed to be stored in the same common repository.

-	From an IMS interaction perspective, it is assumed that IMS HSS/SLF is a logical entity that can act as an application front end making use of the common repository, following the specification TS 23.335 [3] and implementing Cx/Dx and Sh/Dh interfaces that remain untouched. The colocation of IMS HSS/SLF and HSS FE is an operator option.

NOTE:	This is considered as a temporary solution in order to cope with the transition of legacy IMS towards a service based IMS, work in progress in TR 23.794 [14].

#### 6.4.2.2	Architectural proposal

This solution makes use of a layer that separates the applications front end logic from the database technology/protocol used, leveraging the authorisation and policing capabilities from Rel-15 data storage architecture, and incorporating additional features to serve interworking with legacy systems.

Hence, in addition to access control for application front ends, policing and offering a common view of the information irrespective of where it is stored, the solution adds protocol conversion between application front ends and the repository, as well as a notifications engine to mimic the relevant notifications that will trigger specific procedures at the different front end applications and related to handovers between one system and the other.

The solution facilitates keeping existing deployments of application front ends and their repositories without modification, and provides a flexible model to introduce new application front ends without having to impact existing deployments and procedures.

With few updates compared to what is reflected in the next clauses, the solution also facilitates migration of 4G users to be able to receive services from the 5G core, with the limitations imposed by 4G subscriptions, as when e.g. accessing the 5GC from a E-UTRAN radio access (option 5 type of deployment).

Figure 6.4.2.2-1 High level architecture for interaction of HSS FE and UDM with common repository

Figure 6.4.2.2-2 High level architecture for interaction of HSS FE, IMS HSS/SLF and UDM with common repository

### 6.4.3	Services and Illustrated Procedures

#### 6.4.3.1	General description

The proposed data access layer will subscribe to changes performed in the repository, issuing some notifications towards the UDM or the HSS FE for specific UEs that may maintain profiles for 4G and 5G.

Figure 6.4.3.1-1

#### 6.4.3.2	5GS procedures

The procedures specified in TS 23.502 [2] for Registration (4.2.2.2), Deregistration (4.2.2.3), UE Configuration Update (4.2.4), Reachability procedures (4.2.5), Session Management (4.3), and User profile Management (4.2.5) do not change when introducing this solution.

Authentication procedures at registration (step 9 in 4.2.2.2 of TS 23.502 [2]) does not change either when introducing this solution.

Any potential query of information related to the 4G subscription profile of the UE, would imply a mapping/translation of the NUdr_DM_Query in the Data Access Layer to be converted into a Ud query.

#### 6.4.3.3	EPS procedures

EPS procedures, including authentication ones, are not expected to be affected from the point of view of network signalling.

The Data Access Layer will subscribe to changes in the common repository related to 5G subscribers having also 4G profiles. When any action is performed at the level of 4G data for the particular UE (like attachment, detachment, etc), the Data Access Layer will issue a notification towards the UDM. UDM will perform any action needed depending on the status of the UE, interworking needed, etc

This is intended to facilitate the tracking of the subscriber in either system, in order to execute the relevant deregistration in case the subscriber moves from one to the other.

Figure 6.4.3.3-1

The Ud Request expected to trigger a notification is Update Data as specified by TS 23.335 [3]. There is no need to notify any query of data performed by HSS FE.

Any potential query of information related to the 5G subscription profile of the UE, would imply a mapping/translation of the Ud Query in the Data Access Layer to be converted into a query to the common repository.

As the technologies and protocols used to access the data stored in the common repository are operator deployment choices, the mapping of the response depicted in the previous chart to the Nudr_DM_Notify operation towards the UDM is left as an operator deployment choice in agreement with its vendors.

#### 6.4.3.4	Handover procedures

##### 6.4.3.4.1	5GS to EPS

In 5GS to EPS handovers, the attach in the EPS will generate a Ud Request to modify data by HSS FE followed by a Response by the repository and a notification to the Data Access Layer. This will trigger a notification to the UDM to indicate the UE is now under EPS control. UDM should in turn modify the location data in the 5G profile and will proceed to deregister the AMF the UE moved from.

Figure 6.4.3.4.1-1

No other action is expected to be performed by the UDM, and the control by the EPS will proceed as specified in the TS 23.502 [2], clause 4.11.1.2.1, with the difference that the HSS FE will not cancel the location of the AMF during the TAU procedure. Instead, the HSS FE will acknowledge the location update to the new MME and proceed as normal.

##### 6.4.3.4.2	EPS to 5GS

In EPS to 5GS handovers, the registration in the 5GS will generate a Nudr request and subsequent notification when data is stored in the common repository. The Data Access Layer will capture that and generate a notification towards the HSS FE indicating the UE has left the EPS.

Figure 6.4.3.4.2-1

NOTE:	HSS FE subscribed to changes in the common repository when UE data was stored.

The procedure follows the specification in TS 23.502 [2] clause 4.11.1.2.2 with the modification that it is the HSS FE the one initiating the relevant cancel location towards the MME node the UE has moved from.

#### 6.4.3.5	IMS interaction

##### 6.4.3.5.1	Access to IMS data

Access to IMS data for creation, deletion, query and modification follows the specification in TS 23.228 [7] and TS 23.335 [3] by IMS making use of the IMS HSS/SLF functionality (either as a stand alone application front end or collocated with HSS FE).

It is not expected that any of the IMS entities need to interact with UDM, with the exception of the P-CSCF restoration mechanism invoked by S-CSCF, and the T-ADS information retrieval by the IMS AS.

For these specific cases, it is proposed to use the SLF functionality for UE identity to UDM identity or HSS FE identity resolution, according to TS 23.228 [7] clause 5.8.

NOTE 1:	It is proposed to introduce SLF functionality since the split of UDM and HSS FE will imply multiple addressable location servers for the 5G UEs and these IMS procedures.

NOTE 2:	Since the I-CSCF, S-CSCF and IMS AS will always interrogate the SLF to find the location server serving the UE, the SLF needs to be configured to return the IMS HSS address in first place since it is the front end able to return the IMS profile for the UE.

Any Cx or Sh operation triggered by CSCF or IMS AS, except restoration and T-ADS information retrieval, will be handled by the IMS HSS front end (either stand alone or collocated with the HSS FE).

When UE is registered to IMS, the common repository will contain the indication regarding the UE being served by 5GS or EPS. This is proposed to be done as per the description in the common procedures, regardless the UE re-registers or not to IMS when moving between the systems (as per clause 5.2.2.4 of TS 23.228 [7]). This indication is proposed to be checked when either P-CSCF restoration or T-ADS information retrieval is received by UDM or HSS FE (see next clauses).

In principle, both UDM and HSS FE would not require any additional action than what is described before for the general procedures.

##### 6.4.3.5.2	P-CSCF Restoration procedures

The P-CSCF restoration procedure via UDM is an optional mechanism according to TS 23.380 [8].

Option 1: When UDM and HSS FE are split, the preferred P-CSCF restoration procedure would be that performed by detection of the failure by SMF/UPF according to TS 23.380 [8] clause 5.8.3, or using the PCF based P-CSCF restoration as specified in clause 5.8.5 of TS 23.380 [8].

NOTE 1:	The adaptation of IMS to service based architecture, making use of proper virtualisation/cloud mechanisms for e.g.self-healing, high availability, redundancy, etc, would most likely make unnecessary the use of the current restoration procedures.

Option 2: To enable the P-CSCF restoration via either the UDM or the HSS FE, the SLF functionality is invoked in order to perform a user identity to HSS FE identity or UDM identity resolution.

The SLF will return the list of different servers (UDMs and HSS FEs) that may be in charge of the UE at that point in time, for the S-CSCF to request the relevant P-CSCF restoration mechanism to the relevant entity. This procedure is according to TS 29.228 [10] clause 6.4.

6.4.3.5.2-1

NOTE 2:	This assumes that UDM implements the Cx operations SAR/SAA to trigger the P-CSCF restoration. Potential conversion of Diameter Cx operation to an Nudm service operation is out of the scope of this study.

NOTE 3:	It is assumed that S-CSCF is aware of the IMS registration status of the UE prior to request P-CSCF restoration.

Both HSS FE and UDM will check the UE registration status in the corresponding network (either 5GS or EPS) in order to either execute the procedure (according to TS 23.380 [8]) or reject it.

No other impact is expected for restoration.

##### 6.4.3.5.3	Terminating domain selection information for IMS voice

According to TS 23.501 [6], clause 5.16.3.6, the UDM/HSS shall be able to query the serving AMF for T-ADS related information, and AMF shall respond to the query with the relevant information unless the UE is detached:

-	whether or not IMS voice over PS Session is supported in the registration area(s) where the UE is currently registered;

-	whether or not IMS voice over PS Session Supported Indication over non-3GPP access is supported in the WLAN where the UE is currently registered;

-	the time of the last radio contact with the UE; and

-	the current Access Type and RAT type.

Retrieval of T-ADS information is initiated by the IMS AS.

To retrieve the T-ADS information from either the UDM or the HSS FE, the SLF functionality is invoked in order to perform a user identity to HSS FE identity or UDM identity resolution.

The SLF will return the list of different servers (UDMs and HSS FEs) that may be in charge of the UE at that point in time, for the IMS AS to request the information for access domain selection. This procedure is according to TS 29.328 [11] clause 6.5.

If the UE is attached to IMS via 5GS, the T-ADS information request to the IMS HSS/SLF will end up in a Sh request (UDR/UDA) to UDM, which in turn will invoke the AMF service Namf_MT_ ProvideDomainSelectionInfo as specified in TS 23.501 [6] clause 5.16.3.6 and TS 23.228 [7] annex Y.

The request from IMS AS, when the UE is attached to the EPS, should follow the same procedures as defined in TS 23.228 [7] clause 4.2.4a.

NOTE:	It is assumed that IMS AS is aware of the IMS registration status of the UE prior to request T-ADS information.

Both UDM and HSS FE will check the registration status in the corresponding network in order to either execute the procedure (according to TS 23.228 [7]) or reject it.

Figure 6.4.3.5.3-1

NOTE:	This assumes that UDM implements the Sh operations UDR/UDA to trigger the T-ADS retrieval procedure. Potential conversion of Diameter Sh operation to an Nudm service operation is out of the scope of this study.

### 6.4.4	Impacts on existing services and interfaces

#### 6.4.4.1	General

This solution addresses scenarios in which the operator deployment is already a layered architecture according to TS 23.335 [3] or when user data are stored in an external entity via non standard interfaces, according to TS 23.002 [5], clause 4.1.1. Purely monolithic scenarios, in which the user data are stored within the HSS itself, would need to migrate towards a layered architecture.

The solution is basically an operator choice restricted to the introduction of a Data Access Layer with the capabilities described in clauses 6.4.3.1 to 6.4.3.4, and does not impact the specified interfaces with new operations.

The solution is also based on network configuration, by including SLF functionality for every IMS operation directed towards a location server (either HSS FE, IMS HSS or UDM). The SLF is configured as the contact point for I-CSCF, S-CSCF and IMS AS, and needs to be provisioned with the list of location servers serving the UEs in the network.

The solution proposes to add information regarding the system registration status, i.e. the system serving the UE:

-	For 5G profile, this extension can be handled as the resources Amf3GppAccessRegistration and AmfNon3GppAccessRegistration in the Nudr_DM_Update service operation as specified in TS 29.505 [9].

-	For 4G profile, this extension can be mapped to the repository field where MME reference is held. Since the organisation of data in the EPS UDR is not standardised, this mapping would be left for implementation.

Finally, the solution proposes adding the Sh pair UDR/UDA (Diameter) to UDM as a temporary solution until IMS is adapted to service based architecture or any other solution is specified for T-ADS information retrieval by IMS.

#### 6.4.4.2	Summary of impacts on UDM

-	For T-ADS information retrieval: Add Sh pair UDR/UDA

-	For P-CSCF restoration:

For Option 1, no impact (i.e. make use of PCF based restoration or detection of failure by SMF/UPF)

For Option 2: Add Cx pair SAR/SAA for P-CSCF restoration

#### 6.4.4.3	Summary of impacts on HSS FE

No impact

#### 6.4.4.4	Summary of impacts on EPS UDR

The impacts on the EPS UDR are left for implementation, since the protocols to access and the organisation of data is implementation specific.

### 6.4.5	Evaluation

Editor's note:	This clause provides an evaluation of the solution.

## 6.5	Solution #5: Use of a Data Access Layer for interaction between HSS FE and UDM with separate repositories

### 6.5.1	Introduction

This solution applies to Key Issue 1 and describes how HSS FE and UDM interact when data repositories are separate, and N26 interface is used.

### 6.5.2	High-level Description

#### 6.5.2.1	Assumptions

It is assumed that:

-	Technologies and protocols to access the data stored in the 5G repository are assumed to be database specific or proprietary, but exposing operations to query, create, delete and update of data, as well as subscriptions/notifications to modifications of such data..

-	5G UEs are assumed to be provisioned in the 5G repository and able to use 4G networks (with the corresponding limitations). 4G UEs are provisioned in the EPS UDR, as of today, but not assumed to be able to use 5G core services.

-	From an authentication perspective, this proposal assumes UDM to be UDM/ARPF as specified in TS 33.501 [12]) and it does not assume separation of authentication logic from the UDM. For EPS authentication, the security architecture specified in TS 33.401 [13] is assumed without changes. This implies execution of any cryptographic algorithms that use the long-term security credentials for authentication in each system to be separate procedures, while the security-related part of the subscriber profiles is assumed to be stored in the 5G repository and the EPS UDR.

Editor's note: The final procedure is subject to confirmation from SA WG3.

-	The IMS data for the UEs is assumed to be stored in the EPS UDR.

-	From an IMS interaction perspective, it is assumed that IMS HSS/SLF is a logical entity that can act as an application front end making use of the EPS UDR, following the specification TS 23.335 [3] and implementing Cx/Dx and Sh/Dh interfaces that remain untouched. The colocation of IMS HSS/SLF and HSS FE is an operator option.

NOTE:	This is considered as a temporary solution in order to cope with the transition of legacy IMS towards a service based IMS, work in progress in TR 23.794 [14].

#### 6.5.2.2	Architectural proposal

This solution makes use of a layer that separates the applications front end logic from the database technology/protocol used, leveraging the authorisation and policing capabilities from Rel-15 data storage architecture, and incorporating additional features to serve interworking with legacy systems.

Hence, in addition to access control for application front ends, policing and offering a common view of the information irrespective of where it is stored, the solution adds protocol conversion between application front ends and the repository, as well as a notifications engine to mimic the relevant notifications that will trigger specific procedures at the different front end applications and related to handovers between one system and the other.

The solution uses similar concepts as the current Solution #1 ("Retrieving subscription data from separate UDR repositories for EPS and 5GS subscription data using a UDR Translation Function (UTF)"), but makes use of different procedures for IMS interactions.

Figure 6.5.2.2-1: High level architecture for interaction of HSS FE and UDM with separate repositories

Figure 6.5.2.2-2: High level architecture for interaction of HSS FE, IMS HSS/SLF and UDM with separate repositories

### 6.5.3	Services and Illustrated Procedures

#### 6.5.3.1	General description

The proposed data access layer will subscribe to changes performed in the EPS UDR and 5G repository, issuing some notifications towards the UDM or the HSS FE for specific UEs that may maintain profiles for 4G and 5G. When needed, the Data Access layer translates Nudr operations issued by UDM into Ud requests towards the EPS UDR and vice-versa.

Figure 6.5.3.1-1

#### 6.5.3.2	5GS procedures

The procedures specified in TS 23.502 [2] for Registration (4.2.2.2), Deregistration (4.2.2.3), UE Configuration Update (4.2.4), Reachability procedures (4.2.5), Session Management (4.3), and User profile Management (4.2.5) do not change when introducing this solution.

Authentication procedures at registration (step 9 in 4.2.2.2 of TS 23.502 [2]) does not change either when introducing this solution.

Any potential query of information related to the 4G subscription profile of the UE, would imply a mapping/translation of the NUdr_DM_Query in the Data Access Layer to be converted into a Ud query to the EPS UDR.

#### 6.5.3.3	EPS procedures

EPS procedures, including authentication ones, are not expected to be affected from the point of view of network signalling.

The Data Access Layer will subscribe to changes in the EPS UDR related to 5G subscribers having also 4G profiles in the EPS UDR. When any action is performed at the level of 4G data for the particular UE (like attachment, detachment, etc), the Data Access Layer will issue a notification towards the UDM. UDM will perform any action needed depending on the status of the UE, interworking needed, etc

This is intended to facilitate the tracking of the subscriber in either system, in order to execute the relevant deregistration in case the subscriber moves from one to the other.

Figure 6.5.3.3-1

The Ud Request expected to trigger a notification is Update Data as specified by TS 23.335 [3]. There is no need to notify any query of data performed by HSS FE.

Any potential query of information related to the 5G subscription profile of the UE, would imply a mapping/translation of the Ud Query in the Data Access Layer to be converted into a query to the 5G repository.

As the technologies and protocols used to access the data stored in the repositories are operator deployment choices, the mapping of the Ud Notify depicted in the previous chart to the Nudr_DM_Notify operation towards the UDM is left as an operator deployment choice in agreement with its vendors.

#### 6.5.3.4	Handover procedures

##### 6.5.3.4.1	5GS to EPS

In 5GS to EPS handovers, the attach in the EPS will generate a Ud Request to modify data by HSS FE followed by a Ud Response by the EPS UDR and a Ud notification to the Data Access Layer. This will trigger a Nudr_DM_Notify to the UDM to indicate the UE is now under EPS control. UDM should in turn modify the location data in the 5G profile and will proceed to deregister the AMF the UE moved from.

Figure 6.5.3.4.1-1

No other action is expected to be performed by the UDM, and the control by the EPS will proceed as specified in the TS 23.502 [2], clause 4.11.1.2.1, with the difference that the HSS FE will not cancel the location of the AMF during the TAU procedure.

Instead, the HSS FE will acknowledge the location update to the new MME and proceed as normal and it will be the UDM the one deregistering the AMF following the flow shown before.

##### 6.5.3.4.2	EPS to 5GS

In EPS to 5GS handovers, the registration in the 5GS will generate a Nudr request and subsequent notification when data is stored in the 5G repository. The Data Access Layer will capture that and generate a notification towards the HSS FE indicating the UE has left the EPS.

Figure 6.5.3.4.2-1

NOTE 1:	HSS FE subscribed to changes regarding UEs having 4G and 5G profiles.

The procedure follows the specification in TS 23.502 [2] clause 4.11.1.2.2 with the modification that it is the HSS FE the one initiating the relevant cancel location towards the MME node the UE has moved from.

NOTE 2:	The mapping of the acknowledgement from the 5G repository to the Ud Notify towards the HSS FE depicted in the previous chart is left for implementation based on operator deployment.

As an alternative to the previous flow, the Data Access Layer can modify the UE data in the EPS UDR, which in turn would trigger a notification towards the HSS FE (HSS FE would have subscribed to changes in the EPS UDR). This alternative would save impacts in the HSS FE that would only need to subscribe to changes in the EPS UDR and be notified when the data is updated.

#### 6.5.3.5	IMS interaction

##### 6.5.3.5.1	Access to IMS data

Access to IMS data for creation, deletion, query and modification follows the specification in TS 23.228 [7] and TS 23.335 [3] by IMS making use of the IMS HSS/SLF functionality (either as a stand alone application front end or collocated with HSS FE).

It is not expected that any of the IMS entities need to interact with UDM, with the exception of the P-CSCF restoration mechanism invoked by S-CSCF, and the T-ADS information retrieval by the IMS AS.

For these specific cases, it is proposed to use the SLF functionality for UE identity to UDM identity or HSS FE identity resolution, according to TS 23.228 [7] clause 5.8.

NOTE 1:	It is proposed to introduce SLF functionality since the split of UDM and HSS FE will imply multiple addressable location servers for the 5G UEs and these IMS procedures.

NOTE 2:	Since the I-CSCF, S-CSCF and IMS AS will always interrogate the SLF to find the location server serving the UE, the SLF needs to be configured to return the IMS HSS address in first place since it is the front end able to return the IMS profile for the UE.

NOTE 3:	The evolution of IMS can also imply the IMS profile of the 5G UEs to be moved into the 5G repository. This can impact the way some procedures are executed for the different UEs.

Any Cx or Sh operation triggered by CSCF or IMS AS, except restoration and T-ADS information retrieval, will be handled by the IMS HSS front end (either stand alone or collocated with the HSS FE).

When UE is registered to IMS, EPS UDR will contain the indication regarding the UE being served or not by EPS, and 5G repository will contain the indication regarding the UE being served by 5GS. This is proposed to be done as per the description in the common procedures, regardless the UE re-registers or not to IMS when moving between the systems (as per clause 5.2.2.4 of TS 23.228 [7]).

These indications are proposed to be checked when either P-CSCF restoration or T-ADS information retrieval is received by UDM or HSS FE (see next clauses).

In principle, both UDM and HSS FE would not require any additional action than what is described before for the general procedures.

##### 6.5.3.5.2	P-CSCF Restoration procedures

The P-CSCF restoration procedure via UDM is an optional mechanism according to TS 23.380 [8].

Option 1: When UDM and HSS FE are split, the preferred P-CSCF restoration procedure would be that performed by detection of the failure by SMF/UPF according to TS 23.380 [8] clause 5.8.3, or using the PCF based P-CSCF restoration as specified in clause 5.8.5 of TS 23.380 [8].

NOTE 1:	The adaptation of IMS to service based architecture, making use of proper virtualisation/cloud mechanisms for e.g.self-healing, high availability, redundancy, etc, would most likely make unnecessary the use of the current restoration procedures.

Option 2: To enable the P-CSCF restoration via either the UDM or the HSS FE, the SLF functionality is invoked in order to perform a user identity to HSS FE identity or UDM identity resolution.

The SLF will return the list of different servers (UDMs and HSS FEs) that may be in charge of the UE at that point in time, for the S-CSCF to request the relevant P-CSCF restoration mechanism to the relevant entity. This procedure is according to TS 29.228 [10] clause 6.4.

Figure 6.5.3.5.2-1

NOTE 2:	This assumes that UDM implements the Cx operations SAR/SAA to trigger the P-CSCF restoration. Potential conversion of Diameter Cx operation to an Nudm service operation is out of the scope of this study.

NOTE 3:	It is assumed that S-CSCF is aware of the IMS registration status of the UE prior to request P-CSCF restoration.

Both HSS FE and UDM will check the UE registration status in the corresponding network in order to either execute the procedure (according to TS 23.380 [8]) or reject it.

No other impact is expected for restoration.

##### 6.5.3.5.3	Terminating domain selection information for IMS voice

According to TS 23.501 [6], clause 5.16.3.6, the UDM/HSS shall be able to query the serving AMF for T-ADS related information, and AMF shall respond to the query with the relevant information unless the UE is detached:

-	whether or not IMS voice over PS Session is supported in the registration area(s) where the UE is currently registered;

-	whether or not IMS voice over PS Session Supported Indication over non-3GPP access is supported in the WLAN where the UE is currently registered;

-	the time of the last radio contact with the UE; and

-	the current Access Type and RAT type.

Retrieval of T-ADS information is initiated by the IMS AS.

To retrieve the T-ADS information from either the UDM or the HSS FE, the SLF functionality is invoked in order to perform a user identity to HSS FE identity or UDM identity resolution.

The SLF will return the list of different servers (UDMs and HSS FEs) that may be in charge of the UE at that point in time, for the IMS AS to request the information for access domain selection. This procedure is according to TS 29.328 [11] clause 6.5.

If the UE is attached to IMS via 5GS, the T-ADS information request to the IMS HSS/SLF will end up in a Sh request (UDR/UDA) to UDM, which in turn will invoke the AMF service Namf_MT_ ProvideDomainSelectionInfo as specified in TS 23.501 [6] clause 5.16.3.6 and TS 23.228 [7] annex Y.

The request from IMS AS, when the UE is attached to the EPS, should follow the same procedures as defined in TS 23.228 [7] clause 4.2.4a.

NOTE 1:	It is assumed that IMS AS is aware of the IMS registration status of the UE prior to request T-ADS information.

Both UDM and HSS FE will check the registration status in the corresponding network in order to either execute the procedure (according to TS 23.228 [7]) or reject it.

Figure 6.5.3.5.3-1

NOTE 2:	This assumes that UDM implements the Sh operations UDR/UDA to trigger the T-ADS information retrieval procedure. Potential conversion of Diameter Sh operation to an Nudm service operation is out of the scope of this study.

### 6.5.4	Impacts on existing services and interfaces

#### 6.5.4.1	General

This solution addresses scenarios in which the operator deployment is already a layered architecture according to TS 23.335 [3]. Non-layered scenarios would need to migrate towards a layered architecture prior to deploying this solution.

The solution is basically an operator choice restricted to the introduction of a Data Access Layer with the capabilities described in clauses 6.x.3.1 to 6.x.3.4, and does not impact the specified interfaces with new operations.

The solution is also based on network configuration, by including SLF functionality for every IMS operation directed towards a location server (either HSS FE, IMS HSS or UDM). The SLF is configured as the contact point for I-CSCF, S-CSCF and IMS AS, and needs to be provisioned with the list of location servers serving the UEs in the network.

The solution proposes to add information regarding the system registration status, i.e. the system serving the UE:

-	For 5G profile, this extension can be handled as the resources Amf3GppAccessRegistration and AmfNon3GppAccessRegistration in the Nudr_DM_Update service operation as specified in TS 29.505 [9].

-	For 4G profile, this extension can be mapped to the repository field where MME reference is held. Since the organisation of data in the EPS UDR is not standardised, this mapping would be left for implementation.

Finally, the solution proposes adding the Sh pair UDR/UDA (Diameter) to UDM as a temporary solution until IMS is adapted to service based architecture or any other solution is specified for T-ADS information retrieval by IMS.

#### 6.5.4.2	Summary of impacts on UDM

-	For T-ADS information retrieval: Add Sh pair UDR/UDA

-	For P-CSCF restoration:

For Option 1, no impact (i.e. make use of PCF based restoration or detection of failure by SMF/UPF)

For Option 2: Add Cx pair SAR/SAA for P-CSCF restoration

#### 6.5.4.3	Summary of impacts on HSS FE

No impact on network procedures.

Depending on operator choice either:

-	A new subscription/notification interaction between HSS FE and the Data Access Layer in order to be informed when a 5G UE that camped in the EPS has moved back to 5GS system (in order to trigger a cancel location procedure), OR

-	No impact expected, if alternative depicted in clause 6.x.3.4.2 is decided. 6.x.4.4	Summary of impacts on EPS UDR or 5G repository

The impacts on the EPS UDR or 5G repository are left for implementation, since the protocols and technologies to access their data is implementation specific.

### 6.5.5	Evaluation

Editor's note:	This clause provides an evaluation of the solution.

# 7	Evaluation

Editor's note:	This clause will provide a general evaluation of the solutions.

# 8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2018-07 | SA2#128 | S2-186363 | - | - | - | Table of contents | 0.0.0 |
| 2018-07 | SA2#128 | S2-187479 S2-187481 S2-187483 |  |  |  | Inclusion of documents approved in SA2#128: S2-187479, Scope S2-187481, Architectural assumptions S2-187483, Key Issue S2-187484, Key Issue | 0.1.0 |
| 2018-08 | SA2#128b | S2-188571 S2-188924 S2-188925 S2-189022 |  |  |  | Inclusion of documents approved in SA2#128b S2-188571, IMS-5GS Interworking in key issues S2-188924, HSS FE-UDM interworking using UTF S2-188925, UDM-HSS Interworking over new reference point S2-189022, Interaction reusing legacy protocol | 0.2.0 |
| 2018-09 | - | - | - | - | - | MCC Update as wrong version was included in the zip file for version 0.2.0 | 0.2.1 |
| 2018-10 | SA2#129 | S2-1810689 S2-1810690 S2-1811602 S2-1811006 S2-1811007 S2-1811009 S2-1811010 | - | - | - | Inclusion of documents approved in SA2#129 S2-1810689, Update of solution #3 for IP address continuity S2-1810690, Update of solution #3 to support T-ADS S2-1811602, Update of solution #1 S2-1811006, Interaction HSS FE and UDM with common repository S2-1811007, Interaction HSS FE and UDM with different repositories S2-1811009, Updated solution#2: Direct SBA UDM-HSS Interworking S2-1811010, Updated solution 3: UDM/HSS FE interaction reusing legacy protocol | 0.3.0 |