3GPP TS 26.260 V0.0.2 (2017-10)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and Systems Aspects;

Objective Test Methodologies for the Evaluation of Immersive Audio Systems;

(Release 15	)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<keyword[, keyword, …]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	5

3.1	Definitions	5

3.2	Symbols	5

3.3	Abbreviations	6

4	Objective Test Methodologies for Immersive Audio Systems	6

4.1	Objective Test Methodologies for Assessment of Immersive Audio Sending Systems	6

4	.2	Objective Test Methodologies for Assessment of Immersive Audio Receiving Systems	6

4.3	Objective Test Methodologies for Assessment of End to End Immersive Audio Systems	6

Annex <X> (informative): Change history	7

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

# 1	Scope

The present document …

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

# 4	Objective Test Methodologies for Immersive Audio Systems

## 4.1	Objective Test Methodologies for Assessment of Immersive Audio Systems in the Sending Direction

Examples of subheadings for this section may include: 5.4.1 Spatial Microphone Self-Noise, 5.4.2 Spatial Microphone Frequency Response, 5.4.3 Spatial Microphone Directivity, etc. Objective methods for comparing different scene-based audio formats may also go here.

## 4.2		Objective Test Methodologies for Assessment of Immersive Audio Systems in the Receiving Direction

Examples of subheadings for this section may include: 5.4.1 Motion to Sound Latency in Dynamic Rendering Systems / 5.4.2 Predictive Models for Perceived Source Direction, etc.

## 4.3	Objective Test Methodologies for Assessment of End to End Immersive Audio Systems

Examples of subheadings for this section

may include: 5.4.1 e2e Delay.

-

Annex A

(informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 10/03/17 | SA4#95 | S4-170909 |  |  |  | Initial Skeleton Draft Input to SA4#95 | 0.0.1 |
| 10/12/17 | SA4#95 | S4-171042 |  |  |  | Output from SA4#95 | 0.0.2 |