| 3GPP TS 29.541 V0.3.0 (2019-11) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; 5G System; Network Exposure Function Services for Non-IP Data Delivery (NIDD); Stage 3 (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	6

2	References	6

3	Definitions, symbols and abbreviations	7

3.1	Definitions	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Overview	7

4.1	Introduction	7

5	Services offered by the NEF for NIDD	8

5.1	Introduction	8

5.2	Nnef_SMContext Service	8

5.2.1	Service Description	8

5.2.2	Service Operations	8

5.2.2.1	Introduction	8

5.2.2.2	Create Service Operation	8

5.2.2.2.1	General	8

5.2.2.3	Delete Service Operation	10

5.2.2.3.1	General	10

5.2.2.4	Status Notify Service Operation	10

5.2.2.4.1	General	10

5.2.2.4.2	Notify of Individual SM Context Release (Nnef_SMContext_DeleteNotify)	11

5.2.2.5	Update Service Operation	11

5.2.2.5.1	General	11

5.2.2.6	Deliver Service Operation	12

5.2.2.6.1	General	12

6	API Definitions	13

6.1	Nnef_SMContext Service API	13

6.1.1	Introduction	13

6.1.2	Usage of HTTP	13

6.1.2.1	General	13

6.1.2.2	HTTP standard headers	13

6.1.2.2.1	General	13

6.1.2.2.2	Content type	13

6.1.2.3	HTTP custom headers	13

6.1.3	Resources	14

6.1.3.1	Overview	14

6.1.3.2	Resource: SM Contexts Collection	14

6.1.3.2.1	Description	14

6.1.3.2.2	Resource Definition	15

6.1.3.2.3	Resource Standard Methods	15

6.1.3.2.3.1	POST	15

6.1.3.2.4	Resource Custom Operations	15

6.1.3.3	Resource: Individual SM Context	15

6.1.3.3.1	Description	15

6.1.3.3.2	Resource Definition	16

6.1.3.3.3	Resource Standard Methods	16

6.1.3.3.4	Resource Custom Operations	16

6.1.3.3.4.1	Overview	16

6.1.3.3.4.2	Operation: release	16

6.1.3.3.4.2.1	Description	16

6.1.3.3.4.2.2	Operation Definition	16

6.1.3.3.4.3	Operation: update	17

6.1.3.3.4.3.1	Description	17

6.1.3.3.4.3.2	Operation Definition	17

6.1.3.3.4.4	Operation: deliver	17

6.1.3.3.4.4.1	Description	17

6.1.3.3.4.4.2	Operation Definition	17

6.1.5	Notifications	18

6.1.5.1	General	18

6.1.5.2	Status Notification	18

6.1.5.2.1	Description	18

6.1.5.2.2	Target URI	18

6.1.5.2.3	Standard Methods	18

6.1.5.2.3.1	POST	18

6.1.6	Data Model	19

6.1.6.1	General	19

6.1.6.2	Structured data types	20

6.1.6.2.1	Introduction	20

6.1.6.2.2	Type: SmContextCreateData	21

6.1.6.2.3	Type: SmContextCreatedData	22

6.1.6.2.4	Type: SmContextReleaseData	22

6.1.6.2.5	Type: SmContextReleasedData	22

6.1.6.2.6	Type: SmContextStatusNotification	23

6.1.6.2.7	Type: NiddInformation	23

6.1.6.2.8	Type: SmContextConfiguration	23

6.1.6.2.9	Type: SmallDataRateControl	24

6.1.6.2.10	Type: SmContextUpdateData	24

6.1.6.2.11	Type: SmContextUpdatedData	24

6.1.6.2.12	Type: DeliverReqData	24

6.1.6.3	Simple data types and enumerations	24

6.1.6.3.1	Introduction	24

6.1.6.3.2	Simple data types	25

6.1.6.3.3	Enumeration: SmContextStatus	25

6.1.6.3.4	Enumeration: SmallDataRateControlTimeUnit	25

6.1.7	Error Handling	25

6.1.7.1	General	25

6.1.7.2	Protocol Errors	25

6.1.7.3	Application Errors	25

6.1.8	Feature negotiation	26

6.1.9	Security	26

Annex A (normative):	OpenAPI specification	27

A.1	General	27

A.2	Nnef_SMContext API	27

Annex B (informative):	Change history	34

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the stage 3 protocol and data model for the Nnef Service Based South-Bound Interfaces for NIDD. It provides stage 3 protocol definitions and message flows, and specifies the API for each service offered by the NEF.

The 5G System stage 2 architecture and procedures are specified in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3].

The Technical Realization of the Service Based Architecture and the Principles and Guidelines for Services Definition are specified in 3GPP TS 29.500 [4] and 3GPP TS 29.501 [5].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	OpenAPI: "OpenAPI 3.0.0 Specification", https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.0.md.

[7]	3GPP TR 21.900: "Technical Specification Group working methods".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[9]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[10]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[11]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[12]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[13]	IETF RFC 7807: "Problem Details for HTTP APIs".

[14]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[15]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[16]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

# 4	Overview

## 4.1	Introduction

Within the 5GC, the NEF offers NIDD services to the NF (e.g. SMF) via the Nnef service based southbound interface (see 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3]).

Figure 4.1-1 provides the reference model (in service based interface representation and in reference point representation), with focus on the NEF and the scope of the present specification.

Figure 4.1-1: Reference model – NEF

The functionalities supported by the NEF are listed in clause 6.2.5 of 3GPP TS 23.501 [2].

# 5	Services offered by the NEF for NIDD

## 5.1	Introduction

The table 5.1-1 shows the NEF Services and Service Operations for NIDD:

Table 5.1-1 List of NEF Services for NIDD

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) | Mapped Service Operation |
| --- | --- | --- | --- | --- |
| Nnef_SMContext | Create | Request/Response | SMF | Nnef_SMContext_Create |
|  | Delete | Request/Response | SMF | Nnef_SMContext_Delete |
|  | Status Notify | Subscribe/Notify | SMF | Nnef_SMContext_DeleteNotify (NOTE 1) |
|  | Update | Request/Response | SMF |  |
|  | Delivery | Request/Response | SMF | Nnef_NIDD_Delivery (NOTE 2) |
| NOTE 1: The Status Notify service operation models the Nnef_SMContext_DeleteNotify service operation specified in 3GPP TS 23.502 [3] (see clause 5.2.2.4.2). NOTE 2: On N29, the Deliver service operation in Nnef_SMContext service API models the Nnef_NIDD_Delivery service operation specified in 3GPP TS 23.502 [3]. |  |  |  |  |

## 5.2	Nnef_SMContext Service

### 5.2.1	Service Description

The service allows a NF to manage the SM Contexts on NEF for NIDD. A NF as service consumer (e.g. SMF) can create, update or release SM Contexts for NIDD on NEF. A created SM Context for NIDD may also be released by NEF.

### 5.2.2	Service Operations

#### 5.2.2.1	Introduction

The Nnef_SMContext service supports following service operations:

-	Create

-	Delete

-	StatusNotify

-	Update

-	Deliver

#### 5.2.2.2	Create Service Operation

##### 5.2.2.2.1	General

The Create service operation is used during the following procedure:

-	SMF-NEF Connection Establishment procedure (see 3GPP TS 23.502 [3], clause 4.25.2)

The Create service operation is invoked by a NF Service Consumer (e.g. a SMF) towards the NEF, when the SMF received a PDU Session establishment request from the UE with PDU Session type of "Unstructured", and the subscription information corresponding to the UE requested DNN includes the "NEF Identity for NIDD". There shall be only one individual SM context per PDU session.

The NF Service Consumer (e.g. the SMF) shall create the SM Context for NIDD on NEF by sending the HTTP POST request towards the SM Contexts Collection resource as shown in Figure 5.2.2.2.1-1.

Figure 5.2.2.2.1-1: Create Service Operation

1.	The NF Service Consumer shall send a POST request to the resource representing the SM Contexts Collection resource of the NEF with a "SmContextCreateData" object in request body, including:

-	SUPI of the UE;

-	PDU session ID;

-	S-NSSAI associated with the PDU session;

-	DNN of the PDU session;

-	NIDD information, such as GPSI, AF ID, etc.;

-	NEF ID, indicating the provisioned identity for NIDD service;

-	URI of the resource to receive downlink data delivery for NIDD;

-	Notification URI to receive the SM Context notifications;

-	optionally the indication of UE capability to support Reliable Data Service (RDS);

-	optionally the configuration parameters, e.g. serving PLMN rate control, small data rate control, etc.;

-	optionally small data rate control status, if small data rate control is previously enabled and to be resumed

2a.	On success, "201 Created" shall be returned and the "Location" header shall be present and shall contain the URI of the created Individual SM Context resource.

The payload body of the POST response shall contain a "SmContextCreatedData" object, including:

-	SUPI of the UE;

-	PDU session ID;

-	S-NSSAI associated with the PDU session;

-	DNN of the PDU session;

-	NEF ID, indicating the provisioned identity for NIDD service;

-	optionally the indication of NEF capability to support Reliable Data Service (RDS)

2b.	On failure, one of the HTTP status code listed in Table 6.1.3.2.3.1-3 shall be returned, the response body should contain a "ProblemDetails" object with "cause" attribute set to one of the application errors listed in Table 6.1.3.2.3.1-3.

#### 5.2.2.3	Delete Service Operation

##### 5.2.2.3.1	General

The Delete service operation is used during the following procedure:

-	SMF Initiated SMF-NEF Connection Release procedure (see 3GPP TS 23.502 [3], clause 4.25.7)

The Delete service operation is invoked by a NF Service Consumer (e.g. a SMF) towards the NEF, when the PDU Session Release is initiated and a SM context for NIDD has been previously created on NEF for the PDU session.

The NF Service Consumer (e.g. the SMF) shall delete the SM Context for NIDD on NEF by invoking the "release" custom operation on the Individual SM Context resource as shown in Figure 5.2.2.3.1-1.

Figure 5.2.2.3.1-1: Delete Service Operation

1.	The NF Service Consumer shall send a HTTP POST request towards the URI of "release" custom operation on the Individual SM Context resource received from the "Location" header during a successful Create service operation invocation (See clause 5.2.2.2). The request body shall contain a "SmContextReleaseData" object.

On success, "204 No Content" shall be returned if no information is to be returned to the NF service consumer; otherwise "200 OK" shall be returned with a "SmContextReleasedData" object in response body including necessary information to the NF service consumer, e.g.:

-	Small Data Rate Control status, if Small Data Rate Control is enforced

2b.	On failure, one of the HTTP status code listed in Table 6.1.3.3.4.2-2 shall be returned, the response body should contain a "ProblemDetails" object with "cause" attribute set to one of the application errors listed in Table 6.1.3.3.4.2.2-2.

#### 5.2.2.4	Status Notify Service Operation

##### 5.2.2.4.1	General

The Status Notify service operation is used during the following procedure:

-	NEF Initiated SMF-NEF Connection Release procedure (see 3GPP TS 23.502 [3], clause 4.25.8)

The Status Notify service operation is invoked by the NEF to inform a NF Service Consumer (e.g. a SMF), when the status of the Individual SM Context has changed.

The NEF shall inform the status change of the Individual SM Context resource by sending the HTTP POST method towards the Notification URI as shown in Figure 5.2.2.4.1-1.

Figure 5.2.2.4.1-1: Status Notify Service Operation

1.	The NEF shall send a POST request towards the Notification URI received in the Create service operation request (See clause 5.2.2.2). The request body shall contain a "SmContextStatusNotify" object indicating the changed status of the Individual SM Context resource. The "smContextId" attribute shall contain the URI of the SM Context resource that triggers the notification.

2a.	On success, "204 No content" shall be returned without response body.

2b.	On failure, one of the HTTP status code listed in Table 6.1.5.2.3.1-2 shall be returned, the response body should contain a "ProblemDetails" object.

##### 5.2.2.4.2	Notify of Individual SM Context Release (Nnef_SMContext_DeleteNotify)

During NEF initiated SMF-NEF connection release procedure (see 3GPP TS 23.502 [3], clause 4.25.8), the NEF shall send Status Notification to inform the NF service consumer that the Individual SM Context is released.

The requirements in clause 5.2.2.4.1 shall be applied, with following additions:

1.	Same as step 1 of Figure 5.2.2.4.1-1, the NEF shall set the value of "status" attribute in the request body to "RELEASED". If Small Data Rate Control is enforced, the response body should include the Small Data Rate Control status.

#### 5.2.2.5	Update Service Operation

##### 5.2.2.5.1	General

The Update service operation is invoked by a NF Service Consumer, e.g. a SMF, towards the NEF, when the SMF detects that some of the configurations of the PDU session has changed and the related SM Context for NIDD needs to be updated accordingly.

The NF Service Consumer (e.g. the SMF) shall update the SM Context for NIDD on NEF by invoking the "update" custom operation of the Individual SM Context resource as shown in Figure 5.2.2.5.1-1.

Figure 5.2.2.5.1-1: Update Service Operation

1.	The NF Service Consumer shall send a POST request to the URI of "update" custom operation on an Individual SM Context resource, with a "SmContextUpdateData" object in request body containing the attributes to be updated, e.g.:

-	URI of the resource to receive downlink data delivery for NIDD;

-	Notification URI to receive the SM Context notifications;

-	modified configuration parameters, e.g. serving PLMN rate control, small data rate control, etc.

2a.	On success, "204 No Content" shall be returned if no information is to be returned to the NF service consumer; otherwise "200 OK" shall be returned with a "SmContextUpdatedData" object in response body.

2b.	On failure, one of the HTTP status code listed in Table 6.1.3.3.4.3.2-2 shall be returned, the response body should contain a ProblemDetails object with "cause" attribute set to one of the application errors listed in Table 6.1.3.3.4.3.2-2.

#### 5.2.2.6	Deliver Service Operation

##### 5.2.2.6.1	General

The Deliver service operation is invoked by a NF Service Consumer, e.g. a SMF, to transport Mobile Originated data packet via NEF.

The NF Service Consumer (e.g. the SMF) shall deliver Mobile Originate data via NEF by invoking the "deliver" custom operation of the Individual SM Context resource as shown in Figure 5.2.2.6.1-1.

Figure 5.2.2.5.6-1: Deliver Service Operation

1.	The NF Service Consumer shall send a POST request to the URI of "deliver" custom operation on an Individual SM Context resource, with the request body containing:

-	the MO data as binary body part with content type set as "application/octet-stream"; and

-	a "DeliverReqData" object as another body part with "data" attribute refer to the MO data binary part.

2a.	On success, "204 No Content" shall be returned.

2b.	On failure, one of the HTTP status code listed in Table 6.1.3.3.4.4.2-2 shall be returned. The response body should contain a ProblemDetails object with "cause" attribute set to one of the application errors listed in Table 6.1.3.3.4.4.2-2.

# 6	API Definitions

## 6.1	Nnef_SMContext Service API

### 6.1.1	Introduction

The  Nnef_SMContext service shall use the Nnef_SMContext service API.

The request URI used in HTTP request from the NF service consumer towards the NF service producer shall have the structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "nnef-smcontext".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.1.3.

### 6.1.2	Usage of HTTP

#### 6.1.2.1	General

HTTP/2, IETF RFC 7540 [11], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

The OpenAPI [6] specification of HTTP messages and content bodies for the Nnef_SMContext API is contained in Annex A.

#### 6.1.2.2	HTTP standard headers

##### 6.1.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [4] for the usage of HTTP standard headers.

##### 6.1.2.2.2	Content type

JSON, IETF RFC 8259 [12], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [4]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 7807 [13].

#### 6.1.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [4] shall be applicable. In this release of the specification, no specific custom headers are defined for the Nnef_SMContext service.

### 6.1.3	Resources

#### 6.1.3.1	Overview

Figure 6.1.3.1-1 describes the resource URI structure of the Nnef_SMContext API.

Figure 6.1.3.1-1: Resource URI structure of the Nnef_SMContext API

Table 6.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| SM Contexts Collection | {apiRoot}/nnef-smcontext/<apiVersion>/sm-contexts | POST | Create |
| Individual SM Context | {apiRoot}/nnef-smcontext/<apiVersion>/sm-contexts/{smContextId}/release | release (POST) | Delete |
|  | {apiRoot}/nnef-smcontext/<apiVersion>/sm-contexts/{smContextId}/update | update (POST) | Update |
|  | {apiRoot}/nnef-smcontext/<apiVersion>/sm-contexts/{smContextId}/deliver | deliver (POST) | Deliver |

#### 6.1.3.2	Resource: SM Contexts Collection

##### 6.1.3.2.1	Description

This resource represents the collection of the Individual SM Context resources created in the NEF.

This resource is modelled with the Collection resource archetype (see clause C.2 of 3GPP TS 29.501 [5]).

##### 6.1.3.2.2	Resource Definition

Resource URI: {apiRoot}/nnef-smcontext/<apiVersion>/sm-contexts

This resource shall support the resource URI variables defined in table 6.1.3.2.2-1.

Table 6.1.3.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.1.1 |
| apiVersion | See clause 6.1.1 |

##### 6.1.3.2.3	Resource Standard Methods

###### 6.1.3.2.3.1	POST

This method creates an Individual SM Context resource in the SM Context Collection.

This method shall support the URI query parameters specified in table 6.1.3.2.3.1-1.

Table 6.1.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.2.3.1-3.

Table 6.1.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SmContextCreateData | M | 1 | Representation of the Individual SM context to be created. |

Table 6.1.3.2.3.1-3: Data structures supported by the <method 1> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SmContextCreatedData | M | 1 | 201 Created | Successful creation of an Individual SM context. |
| ProblemDetails | O | 1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application errors: - USER_UNKNOWN - NIDD_CONFIGURATION_NOT_AVAILABLE See table 6.1.7.3-1 for the description of these errors. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with response body containing an object of ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). |  |  |  |  |

##### 6.1.3.2.4	Resource Custom Operations

None.

#### 6.1.3.3	Resource: Individual SM Context

##### 6.1.3.3.1	Description

This resource represents an Individual SM Context resource in the NEF.

This resource is modelled with the Document resource archetype (see clause C.2 of 3GPP TS 29.501 [5]).

##### 6.1.3.3.2	Resource Definition

Resource URI: {apiRoot}/nnef-smcontext/<apiVersion>/sm-contexts/{smContextId}

This resource shall support the resource URI variables defined in table 6.1.3.3.2-1.

Table 6.1.3.3.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.1.1 |
| apiVersion | See clause 6.1.1 |
| smContextId | SM context identifier assigned by the NEF during the Create service operation. |

##### 6.1.3.3.3	Resource Standard Methods

There is no standard HTTP method supported by this resource.

##### 6.1.3.3.4	Resource Custom Operations

###### 6.1.3.3.4.1	Overview

This resource supports custom operation(s) as specified in table 6.1.3.3.4.1-1.

Table 6.1.3.3.4.1-1: Custom operations

| Custom operaration URI | Mapped HTTP method | Description |
| --- | --- | --- |
| {resourceUri}/release | POST | Delete service operation. |
| {resourceUri}/update | POST | Update service operation. |

###### 6.1.3.3.4.2	Operation: release

6.1.3.3.4.2.1	Description

This custom operation releases an Individual SM Context resource previously created in the NEF.

6.1.3.3.4.2.2	Operation Definition

This operation shall support the request data structures specified in table 6.1.3.2.4.2.2-1 and the response data structure and response codes specified in table 6.1.3.2.4.2.2-2.

Table 6.1.3.3.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SmContextReleaseData | M | 1 | Representation of the information to release the Individual SM context. |

Table 6.1.3.3.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SmContextReleasedData | M | 1 | 200 OK | Successful release of an Individual SM context with information sent to the NF service consumer. |
| n/a |  |  | 204 No Content | Successful release of an Individual SM context without information sent to the NF service consumer. |
| ProblemDetails | O | 1 | 404 Not Found | The "cause" attribute may be used to indicate the following application errors: - CONTEXT_NOT_FOUND See table 6.1.7.3-1 for the description of these errors. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with response body containing an object of ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). |  |  |  |  |

###### 6.1.3.3.4.3	Operation: update

6.1.3.3.4.3.1	Description

This custom operation updates an individual SM Context resource.

6.1.3.3.4.3.2	Operation Definition

This operation shall support the request data structures specified in table 6.1.3.3.4.3.2-1 and the response data structure and response codes specified in table 6.1.3.3.4.3.2-2.

Table 6.1.3.3.4.3.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SmContextUpdateData | M | 1 | Representation of the updates to apply to the Individual SM context. |

Table 6.1.3.3.4.3.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SmContextUpdatedData | M | 1 | 200 OK | Successful update of the Individual SM context, when the NEF needs to return information in the response. |
| n/a |  |  | 204 No Content | Successful update of the Individual SM context, when the NEF does not need to return information in the response. |
| ProblemDetails | O | 1 | 404 Not Found | The "cause" attribute may be used to indicate the following application errors: - CONTEXT_NOT_FOUND See table 6.1.7.3-1 for the description of these errors. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with response body containing an object of ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). |  |  |  |  |

###### 6.1.3.3.4.4	Operation: deliver

6.1.3.3.4.4.1	Description

This custom operation updates an individual SM Context resource.

6.1.3.3.4.4.2	Operation Definition

This operation shall support the request data structures specified in table 6.1.3.3.4.4.2-1 and the response data structure and response codes specified in table 6.1.3.3.4.4.2-2.

Table 6.1.3.3.4.4.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DeliverReqData | M | 1 | The data for Deliver service request, including the Mobile Originated data to be delivered via NEF. |

Table 6.1.3.3.4.4.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful delivery of Mobile Originate data via NEF. |
| ProblemDetails | O | 1 | 404 Not Found | The "cause" attribute may be used to indicate the following application errors: - CONTEXT_NOT_FOUND See table 6.1.7.3-1 for the description of these errors. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with response body containing an object of ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). |  |  |  |  |

### 6.1.5	Notifications

#### 6.1.5.1	General

This clause specifies the notifications provided by the Nnef_SMContext service.

Notifications shall comply to clause 6.2 of 3GPP TS 29.500 [4] and clause 4.6.2.3 of 3GPP TS 29.501 [5].

#### 6.1.5.2	Status Notification

##### 6.1.5.2.1	Description

If the NF Service Consumer (e.g. the SMF) has provided the Notification URI for getting notified about change of SM Context status, the NEF shall notify the NF Service Consumer when the SM Context status is updated.

##### 6.1.5.2.2	Target URI

The Notification URI "{notificationUri}" shall be used with the resource URI variables defined in table 6.1.5.2.2-1.

Table 6.1.5.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| notificationUri | String formatted as URI with the Notification Uri |

##### 6.1.5.2.3	Standard Methods

###### 6.1.5.2.3.1	POST

This method shall support the request data structures specified in table 6.1.5.2.3.1-1 and the response data structures and response codes specified in table 6.1.5.2.3.1-2.

Table 6.1.5.2.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SmContextStatusNotification | M | 1 | Representation of the SM Context status notification. |

Table 6.1.5.2.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful notification of the SM context status change |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply, with response body containing an object of ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). |  |  |  |  |

### 6.1.6	Data Model

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nnef_SMContext service based interface protocol.

Table 6.1.6.1-1: Nnef_SMContext specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| SmContextCreateData | 6.1.6.2.2 | Information within Create SM Context Request |  |
| SmContextCreatedData | 6.1.6.2.3 | Information within Create SM Context Response |  |
| SmContextReleaseData | 6.1.6.2.4 |  |  |
| SmContextReleasedData | 6.1.6.2.5 |  |  |
| SmContextStatusNotification | 6.1.6.2.6 | Information within Notify SM Context Status Request |  |
| NiddInformation | 6.1.6.2.7 | NIDD information associated to the SM Context |  |
| SmContextConfiguration | 6.1.6.2.8 | NIDD related configurations that should be applied for the SM Context on NEF |  |
| SmallDataRateControl | 6.1.6.2.9 | Data rate control information |  |
| SmContextUpdateData | 6.1.6.2.10 | Information within Update SM Context Request |  |
| SmContextUpdatedData | 6.1.6.2.11 | Information within Update SM Context Response |  |
| DeliverReqData | 6.1.6.2.12 | Information within Deliver Service Operation Request |  |
| SmContextStatus | 6.1.6.3.3 | Enumeration of the status for an Individual SM Context |  |
| SmallDataRateControlTimeUnit | 6.1.6.3.4 | Enumeration of the time units that are applied to data rate control |  |

Table 6.1.6.1-2 specifies data types re-used by the Nnef_SMContext service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the N<NF> service based interface.

Table 6.1.6.1-2: Nnef_SMContext re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Uri | 3GPP TS 29.571 [14] | Uniform Resource Identifier |  |
| Supi | 3GPP TS 29.571 [14] | Subscription Permanent Identifier |  |
| Gpsi | 3GPP TS 29.571 [14] | General Public Subscription Identifier |  |
| SupportedFeatures | 3GPP TS 29.571 [14] | Supported features |  |
| PduSessionId | 3GPP TS 29.571 [14] | PDU Session Identifier |  |
| Dnn | 3GPP TS 29.571 [14] | Data Network Name |  |
| Snssai | 3GPP TS 29.571 [14] | Single Network Slice Selection Assistance Information |  |
| ProblemDetails | 3GPP TS 29.571 [14] | Error description |  |
| SmallDataRateStatus | 3GPP TS 29.571 [14] | Small Data Rate Control Status |  |
| GroupId | 3GPP TS 29.571 [14] | External Group Identifier |  |
| RefToBinaryData | 3GPP TS 29.571 [14] | Reference to binary data part |  |

#### 6.1.6.2	Structured data types

##### 6.1.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 6.1.6.2.2	Type: SmContextCreateData

Table 6.1.6.2.2-1: Definition of type SmContextCreateData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supi | Supi | M | 1 | This IE shall contain the subscriber permanent identify of the UE. |  |
| pduSessionId | PduSessionId | M | 1 | This IE shall contain the PDU Session ID indicating the PDU session associated with the SM Context for NIDD to be created. |  |
| dnn | Dnn | M | 1 | This IE shall contain the requested DNN. |  |
| snssai | Snssai | M | 1 | This IE shall contain the requested S-NSSAI for the home PLMN. |  |
| nefId | string | M | 1 | This IE shall contain the NEF ID of the target NEF (see clause 6.1.6.2.48 of 3GPP TS 29.510 [10]). |  |
| dlNiddEndPoint | Uri | M | 1 | This IE shall contain the URI the resource provided by the NF service consumer to handle downlink NIDD data delivery. |  |
| notificationUri | Uri | M | 1 | This IE shall contain the URI to receive SM Context Status Notifications sent by the NEF. |  |
| niddInfo | NiddInformation | O | 0..1 | When present, this IE shall contain the information used for the SM Context. |  |
| rdsSupport | boolean | O | 0..1 | When present, this IE shall indicate the UE capability to support RDS. The value of this IE shall be set as following: - true: UE supports RDS - false (default): UE does not support RDS |  |
| smContextConfig | SmContextConfiguration | O | 0..1 | When present, this IE shall contain the configuration for the NIDD. |  |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one optional feature defined in clause 6.1.8 is supported. |  |

##### 6.1.6.2.3	Type: SmContextCreatedData

Table 6.1.6.2.3-1: Definition of type SmContextCreatedData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supi | Supi | M | 1 | This IE shall contain the subscriber permanent identify of the UE. |  |
| pduSessionId | PduSessionId | M | 1 | This IE shall contain the PDU Session ID indicating the PDU session associated with the SM Context for NIDD to be created. |  |
| dnn | Dnn | M | 1 | This IE shall contain the requested DNN. |  |
| snssai | Snssai | M | 1 | This IE shall contain the requested S-NSSAI for the home PLMN. |  |
| nefId | string | M | 1 | This IE shall contain the NEF ID of the target NEF (see clause 6.1.6.2.48 of 3GPP TS 29.510 [10]). |  |
| rdsSupport | boolean | O | 0..1 | When present, this IE shall indicate the NEF capability to support RDS. The value of this IE shall be set as following: - true: NEF supports RDS - false (default): NEF does not support RDS |  |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one optional feature defined in clause 6.1.8 is supported. |  |

##### 6.1.6.2.4	Type: SmContextReleaseData

Table 6.1.6.2.4-1: Definition of type SmContextReleaseData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| Cause (FFS) | string | M | 1 | The cause to release the SM Context |  |

##### 6.1.6.2.5	Type: SmContextReleasedData

Table 6.1.6.2.5-1: Definition of type SmContextReleasedData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| smallDataRateStatus | SmallDataRateStatus | C | 0..1 | This IE shall be present if the Small Data Rate Control is enabled for the SM Context. When present, this IE shall contain the Small Data Rate Control Status, as specified in clause 5.31.14.3 of 3GPP TS 23.501 [2]. |  |

##### 6.1.6.2.6	Type: SmContextStatusNotification

Table 6.1.6.2.6-1: Definition of type SmContextStatusNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| status | SmContextStatus | M | 1 | This IE shall contain the current status of the SM Context |  |
| smContextId | Uri | M | 1 | This IE shall contain the URI of the SM Context resource that triggers the notification. |  |
| smallDataRateStatus | SmallDataRateStatus | C | 0..1 | This IE shall be present if the SM Context is released and Small Data Rate Control is enabled for the SM Context. When present, this IE shall contain the Small Data Rate Control Status, as specified in clause 5.31.14.3 of 3GPP TS 23.501 [2]. |  |

##### 6.1.6.2.7	Type: NiddInformation

Table 6.1.6.2.7-1: Definition of type NiddInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| groupId | GroupId | O | 0..1 | When present, this IE shall contain the External group Id of the UE. |  |
| gpsi | Gpsi | O | 0..1 | When present, this IE shall contain the GPSI of the UE. |  |
| afId | string | O | 0..1 | The string identifying the AF as the owner of associated NIDD Configuration, which is received from Nidd Information in UDM (See clause 6.1.6.2.35 of 3GPP TS 29.503 [16]). |  |
| NOTE: At least one of the attributes in the table shall be present. |  |  |  |  |  |

##### 6.1.6.2.8	Type: SmContextConfiguration

Table 6.1.6.2.8-1: Definition of type SmContextConfiguration

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| smalDataRateControl | SmallDataRateControl | O | 0..1 | When present, this IE shall contain the configured Small Data Rate Control for downlink data, as specified in clause 5.31.14.3 of 3GPP TS 23.501 [2]. |  |
| servPlmnDataRateCtl | integer | O | 0..1 | When present, this IE shall contain the maximum allowed number of Downlink NAS Data PDUs per deci hour of the serving PLMN, as specified in clause 5.31.14.2 of 3GPP TS 23.501 [2]. Minimum: 10 |  |
| NOTE: At least one of the attributes in the table shall be present. |  |  |  |  |  |

##### 6.1.6.2.9	Type: SmallDataRateControl

Table 6.1.6.2.8-1: Definition of type SmallDataRateControl

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| allowDataPackets | integer | M | 1 | This IE shall contain the maximum allowed number of user data packets per time unit. |  |
| timeUnit | SmallDataRateControlTimeUnit | M | 1 | This IE shall indicate the time unit for which the data rate control is applied. |  |

##### 6.1.6.2.10	Type: SmContextUpdateData

Table 6.1.6.2.10-1: Definition of type SmContextUpdateData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dlNiddEndPoint | Uri | O | 0..1 | When present, this IE shall contain the URI of another resource handling downlink NIDD data delivery. The NEF shall send downlink data to this new resource after the update is completed. |  |
| notificationUri | Uri | O | 0..1 | When present, this IE shall contain a new URI to receive SM Context Status Notifications sent by the NEF. The NEF shall send SM Contact Status Notification to this new URI after the update is completed. |  |
| smContextConfig | SmContextConfiguration | O | 0..1 | When present, this IE shall contain the configuration for the NIDD. |  |
| NOTE: At least one of the attributes in the table shall be present. |  |  |  |  |  |

##### 6.1.6.2.11	Type: SmContextUpdatedData

Table 6.1.6.2.11-1: Definition of type SmContextUpdatedData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| FFS |  |  |  |  |  |

##### 6.1.6.2.12	Type: DeliverReqData

Table 6.1.6.2.12-1: Definition of type DeliverReqData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| data | RefToBinaryData | M | 1 | This IE shall contain the reference to Mobile Originated data to be delivered via NEF. |

#### 6.1.6.3	Simple data types and enumerations

##### 6.1.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.1.6.3.2	Simple data types

The simple data types defined in table 6.1.6.3.2-1 shall be supported.

Table 6.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 6.1.6.3.3	Enumeration: SmContextStatus

The enumeration SmContextStatus represents status of Individual SM Context in the NEF. It shall comply with the provisions defined in table 6.1.5.3.3-1.

Table 6.1.6.3.3-1: Enumeration SmContextStatus

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "RELEASED" | Indicates that the Individual SM Context for NIDD is released. |  |

##### 6.1.6.3.4	Enumeration: SmallDataRateControlTimeUnit

The enumeration SmallDataRateControlTimeUnit represents the allowed time unit. It shall comply with the provisions defined in table 6.1.5.3.3-1.

Table 6.1.6.3.4-1: Enumeration SmallDataRateControlTimeUnit

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "MINUTE" | Indicates the rate control is applied per minute. |  |
| "HOUR" | Indicates the rate control is applied per hour. |  |
| "DAY" | Indicates the rate control is applied per day. |  |
| "WEEK" | Indicates the rate control is applied per week. |  |

### 6.1.7	Error Handling

#### 6.1.7.1	General

For the Nnef_SMContext API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [5]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [4] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [4].

In addition, the requirements in the following clauses are applicable for the Nnef_SMContext API.

#### 6.1.7.2	Protocol Errors

Protocol errors handling shall be supported as specified in clause 5.2.7 of 3GPP TS 29.500 [4]. No specific procedures for the Nnef_SMContext service are specified.

#### 6.1.7.3	Application Errors

The application errors defined for the Nnef_SMContext service are listed in Table 6.1.7.3-1.

Table 6.1.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| "USER_UNKNOWN" | 403 | This application error indicates that the User Identity does not exist in the NEF. |
| "NIDD_CONFIGURATION_NOT_AVAILABLE" | 403 | This application error indicates that there is no valid NIDD configuration exists for the requested SM Context in the NEF, and NIDD configuration triggered by the NEF (see clause 4.4.12.2 of 3GPP TS 29.522 [15]) is not supported or has failed. |
| "CONTEXT_NOT_FOUND" | 404 | This application error indicates that the SM Context referred by the requested resource URI does not exist in the NEF. |

## 6.1.8	Feature negotiation

The optional features in table 6.1.8-1 are defined for the Nnef_SMContext API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.1.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

## 6.1.9	Security

As indicated in 3GPP TS 33.501 [8] and 3GPP TS 29.500 [4], the access to the Nnef_SMContext API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [9]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [10]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Nnef_SMContext API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [10], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nnef_SMContext service.

The Nnef_SMContext API defines a single scope "nnef_smcontext" for the entire service, and it does not define any additional scopes at resource or operation level.

Annex A (normative):
OpenAPI specification

## A.1	General

This Annex specifies the formal definition of the <Service Name> API(s). It consists of OpenAPI 3.0.0 specifications in YAML format.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on the public 3GPP file server in the following locations (see clause 5B of the 3GPP TS 21.900 [7] for further information):

-	https://www.3gpp.org/ftp/Specs/archive/OpenAPI/<Release>/, and

-	https://www.3gpp.org/ftp/Specs/<Plenary>/<Release>/OpenAPI/.

## A.2	Nnef_SMContext API

openapi: 3.0.0

info:

title: Nnef_SMContext

version: 1.0.0.alpha-1

description: |

Nnef SMContext Service.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.541 V0.2.0; 5G System; Session Management Services for Non-IP Data Delivery (NIDD).

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.541/

servers:

- url: '{apiRoot}/nnef-smcontext/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- nnef-smcontext

paths:

/sm-contexts:

post:

summary: Create SM Context

operationId: Create

tags:

- SM Contexts Collection (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SmContextCreateData'

responses:

'201':

description: Success

content:

application/json:

schema:

$ref: '#/components/schemas/SmContextCreatedData'

headers:

Location:

description: >

'Contains the URI of the newly created Individual SM Context resource, according to the structure:

{apiRoot}/nnef-smcontext/<version>/sm-contexts/{smContextId}'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

StatusNotify:

'{$request.body#/notificationUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SmContextStatusNotification'

responses:

'204':

description: No Content, Notification was succesfull

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/sm-contexts/{smContextId}/release:

post:

summary: Delete SM Context

operationId: Delete

tags:

- Individual SM Context (Document)

parameters:

- name: smContextId

in: path

description: SM Context Resource ID

required: true

schema:

type: string

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SmContextReleaseData'

responses:

'200':

description: OK. Resource representation is returned

content:

application/json:

schema:

$ref: '#/components/schemas/SmContextReleasedData'

'204':

description: No Content.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/sm-contexts/{smContextId}/update:

post:

summary: Update SM Context

operationId: Update

tags:

- Individual SM Context (Document)

parameters:

- name: smContextId

in: path

description: SM Context Resource ID

required: true

schema:

type: string

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SmContextUpdateData'

responses:

'200':

description: OK. Resource representation is returned

content:

application/json:

schema:

$ref: '#/components/schemas/SmContextUpdatedData'

'204':

description: No Content.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/sm-contexts/{smContextId}/deliver:

post:

summary: Deliver Uplink MO Data

operationId: Deliver

tags:

- Individual SM Context (Document)

parameters:

- name: smContextId

in: path

description: SM Context Resource ID

required: true

schema:

type: string

requestBody:

required: true

content:

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/DeliverReqData'

binaryMoData:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryMoData:

contentType:  application/octet-stream

headers:

Content-Id:

schema:

type: string

responses:

'204':

description: No Content.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nnef-smcontext: Access to the Nnef_SMContext API

schemas:

#

#  Structured Data Types

#

SmContextCreateData:

type: object

properties:

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

pduSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

nefId:

type: string

description: This IE shall contain the NEF ID of the target NEF.

dlNiddEndPoint:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notificationUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

niddInfo:

$ref: '#/components/schemas/NiddInformation'

rdsSupport:

type: boolean

description: |

When present, this IE shall indicate the UE capability to support RDS.

The value of this IE shall be set as following

- true  UE supports RDS

- false (default)  UE does not support RDS

smContextConfig:

$ref: '#/components/schemas/SmContextConfiguration'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- supi

- pduSessionId

- dnn

- snssai

- nefId

- dlNiddEndPoint

- notificationUri

SmContextCreatedData:

type: object

properties:

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

pduSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

nefId:

type: string

description: This IE shall contain the NEF ID of the target NEF.

rdsSupport:

type: boolean

description: |

When present, this IE shall indicate the NEF capability to support RDS.

The value of this IE shall be set as following

- true  NEF supports RDS

- false (default)  NEF does not support RDS

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- supi

- pduSessionId

- dnn

- snssai

- nefId

SmContextReleaseData:

type: object

properties:

cause:

type: string

description: The cause to release the SM Context

required:

- cause

SmContextReleasedData:

type: object

properties:

smallDataRateStatus:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SmallDataRateStatus'

SmContextStatusNotification:

type: object

properties:

status:

$ref: '#/components/schemas/SmContextStatus'

smContextId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

smallDataRateStatus:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SmallDataRateStatus'

required:

- status

- smContextId

NiddInformation:

type: object

properties:

groupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

afId:

type: string

description: When present, this IE shall contain the AF Id used for the SM Context.

SmContextConfiguration:

type: object

properties:

smalDataRateControl:

$ref: '#/components/schemas/SmallDataRateControl'

servPlmnDataRateCtl:

type: integer

description: >

When present, this IE shall contain the maximum allowed number of

Downlink NAS Data PDUs per deci hour of the serving PLMN, as specified

in subclause 5.31.14.2 of 3GPP TS 23.501 [2].

Minimum  10

SmallDataRateControl:

type: object

properties:

allowDataPackets:

type: integer

description: This IE shall contain the maximum allowed number of user data packets per time unit.

timeUnit:

$ref: '#/components/schemas/SmallDataRateControlTimeUnit'

required:

- allowDataPackets

- timeUnit

SmContextUpdateData:

type: object

properties:

dlNiddEndPoint:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notificationUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

smContextConfig:

$ref: '#/components/schemas/SmContextConfiguration'

SmContextUpdatedData:

type: object

#      properties:

#        FFS:

#          $ref: '#/components/schemas/'

DeliverReqData:

type: object

properties:

data:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

required:

- data

#

#  Simple Data Types

#

#

#  Enumeration Data Types

#

SmContextStatus:

anyOf:

- type: string

enum:

- "RELEASED"

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- "RELEASED": Indicates that the Individual SM Context for NIDD is released.

SmallDataRateControlTimeUnit:

anyOf:

- type: string

enum:

- "MINUTE"

- "HOUR"

- "DAY"

- "WEEK"

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- "MINUTE": Indicates the rate control is applied per minute.

- "HOUR": Indicates the rate control is applied per hour.

- "DAY": Indicates the rate control is applied per day.

- "WEEK": Indicates the rate control is applied per week.

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-09 | CT4#93 | C4-193889 |  |  |  | Initial Draft. | 0.1.0 |
| 2019-10 | CT4#94 | C4-194527 |  |  |  | Incorporated pCRs agreed on CT4#94, including: C4-194208, C4-194440, C4-194441, C4-194442, C4-194443. | 0.2.0 |
| 2019-11 | CT4#95 | C4-195642 |  |  |  | Incorporated pCRs agreed on CT4#95, including: C4-195283, C4-195284, C4-195295. | 0.3.0 |