# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document describes the physical channels for evolved UTRA.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.201: "Evolved Universal Terrestrial Radio Access (E-UTRA); LTE physical layer; General description".

[3]	3GPP TS 36.212: "Evolved Universal Terrestrial Radio Access (E-UTRA); Multiplexing and channel coding".

[4]	3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures".

[5]	3GPP TS 36.214: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer; Measurements".

[6]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception".

[7]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[8]	3GPP TS 36.321, "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification".

[9]	3GPP TS 36.331, "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC) Protocol specification"

[10]	3GPP TS 36.304, "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode"

# 3	Symbols and abbreviations

## 3.1	Symbols

For the purposes of the present document, the following symbols apply:

Resource element with frequency-domain index  and time-domain index

Value of resource element  [for antenna port]

Matrix for supporting cyclic delay diversity

Density of random access opportunities per radio frame

Carrier frequency

PRACH resource frequency index within the considered time-domain location

PRACH frequency hopping offset, expressed as a number of resource blocks

Start symbol in slot 0 for NPDCCH

Start symbol in slot 0 for NPDSCH

Bandwidth for PSBCH transmission, expressed as a number of subcarriers

Bandwidth for PSBCH transmission, expressed as a number of resource blocks

Bandwidth for PSCCH transmission, expressed as a number of subcarriers

Bandwidth for PSCCH transmission, expressed as a number of resource blocks

Bandwidth for PSDCH transmission, expressed as a number of subcarriers

Bandwidth for PSDCH transmission, expressed as a number of resource blocks

Scheduled bandwidth for PSSCH transmission, expressed as a number of subcarriers

Scheduled bandwidth for PSSCH transmission, expressed as a number of resource blocks

Scheduled bandwidth for uplink transmission, expressed as a number of subcarriers

Scheduled bandwidth for uplink transmission, expressed as a number of resource blocks

Scheduled number of repetitions of a NPUSCH transmission

Scheduled number of repetitions of a NPDSCH transmission

Scheduled bandwidth for uplink NPUSCH transmission, expressed as a number of subcarriers

Number of repetitions of identical slots for NPUSCH

Number of coded bits to transmit on a physical channel [for codeword ]

Number of modulation symbols to transmit on a physical channel [for codeword ]

Number of modulation symbols to transmit per layer for a physical channel

Number of modulation symbols to transmit per antenna port for a physical channel

A constant equal to 2048 for , 4096 for  and 8192 for

Downlink cyclic prefix length for OFDM symbol  in a slot

Cyclic shift value used for random access preamble generation

Number of cyclic shifts used for PUCCH formats 1/1a/1b in a resource block with a mix of formats 1/1a/1b and 2/2a/2b

Bandwidth available for use by PUCCH formats 2/2a/2b, expressed in multiples of

The offset used for PUSCH frequency hopping, expressed in number of resource blocks (set by higher layers)

Physical layer cell identity

Narrowband physical layer cell identity

MBSFN area identity

Physical layer sidelink synchronization identity

Positioning reference signal identity

Downlink bandwidth configuration, expressed in multiples of

Smallest downlink bandwidth configuration, expressed in multiples of

Largest downlink bandwidth configuration, expressed in multiples of

Uplink bandwidth configuration, expressed in multiples of

Smallest uplink bandwidth configuration, expressed in multiples of

Largest uplink bandwidth configuration, expressed in multiples of

Sidelink bandwidth configuration, expressed in multiples of

Number of scheduled subframes for NPDSCH transmission

Number of symbols for NPSS in a subframe

Number of symbols for NSSS in a subframe

Number of consecutive subcarriers in an UL resource unit for NB-IoT

Number of reference signal sequences available for the UL resource unit size

Number of scheduled UL resource units for NB-IoT

Total number of uplink narrowbands

Total number of uplink widebands

Number of subcarriers in the frequency domain for NB-IoT

Number of consecutive absolute subframes over which the scrambling sequence stays the same

Total number of absolute subframes a PUSCH with repetition spans , expressed as a number of absolute subframes

Number of repetititions of a PUSCH transmission

Number of consecutive absolute subframes over which PUCCH or PUSCH stays at the same narrowband before hopping to another narrowband, expressed as a number of absolute subframes

Narrowband offset between one narrowband and the next narrowband a PUSCH hops to, expressed as a number of uplink narrowbands

Total number of absolute subframes a PUCCH with repetition spans, expressed as a number of absolute subframes

Number of repetititions of a PUCCH transmission

Number of PRACH repetitions per preamble transmission attempt

Number of subframes allowed for preamble transmission within a 1024-frame interval

PRACH starting subframe periodicity

Number of NPRACH repetitions per preamble transmission attempt

NPRACH resource periodicity

Frequency location of the first sub-carrier allocated to NPRACH

Number of sub-carriers allocated to NPRACH

Number of starting sub-carriers allocated for UE initiated random access

NPRACH starting subframe

Fraction for starting subcarrier index for UE support for multi-tone msg3 transmission

Periodicity for NPDSCH/NPDCCH gaps

Duration for NPDSCH/NPDCCH gaps

Threshold for applying NPDDCH/NPDCCH gaps

Total number of downlink narrowbands

Total number of downlink widebands

Total number of absolute subframes a PDSCH with repetition spans, expressed as a number of absolute subframes

Number of repetititions of a PDSCH transmission

Number of consecutive absolute subframes over which MPDCCH or PDSCH stays at the same narrowband before hopping to another narrowband, expressed as a number of absolute subframes

Number of narrowbands over which MPDCCH or PDSCH frequency hops

Narrowband offset between one narrowband and the next narrowband an MPDCCH or PDSCH hops to, expressed as a number of downlink narrowbands

Number of times a PDSCH carrying SIB1-BR is transmitted over 8 radio frames

Total number of absolute subframes a MPDCCH with repetition spans , expressed as a number of absolute subframes

Number of repetitions of a MPDCCH transmission

Total number of absolute subframes a MPDCCH search space with maximum repetition level spans, expressed as a number of absolute subframes

Maximum repetition level of a MPDCCH search space

Number of ECCEs in a subframe for one MPDCCH

Number of OFDM symbols in a downlink slot

Number of SC-FDMA symbols in an uplink slot

Number of symbols in a guard period for narrowband or wideband retuning

Number of consecutive slots in an UL resource unit for NB-IoT

Number of SC-FDMA symbols in a sidelink slot

Resource block size in the frequency domain, expressed as a number of subcarriers

Number of sub-bands for PUSCH frequency-hopping with predefined hopping pattern

Size of each sub-band for PUSCH frequency-hopping with predefined hopping pattern, expressed as a number of resource blocks

Size of narrow-band random-access resource in number of subcarriers

Number of downlink to uplink switch points within the radio frame

Number of reference symbols per slot for PUCCH

Timing offset between uplink and downlink radio frames at the UE, expressed in units of

Fixed timing advance offset, expressed in units of

Timing offset between sidelink and timing reference frames at the UE, expressed in units of

Resource index for PUCCH formats 1/1a/1b

Resource index for PUCCH formats 2/2a/2b

Resource index for PUCCH formats 3

Number of PDCCHs present in a subframe

Physical resource block number

First physical resource block occupied by PRACH resource considered

First physical resource block available for PRACH

Subcarrier occupied by NPRACH resource considered

Virtual resource block number

Radio network temporary identifier

Sidelink group destination identity

System frame number

Slot number within a radio frame

Absolute subframe number

Index for subframes allowed for preamble transmission

Number of antenna ports used for transmission of a channel

Antenna port number

Codeword number

Index for PRACH versions with same preamble format and PRACH density

Qm	Modulation order: 2 for QPSK, 4 for 16QAM, 6 for 64QAM and 8 for 256QAM transmissions

Time-continuous baseband signal for antenna port  and OFDM symbol  in a slot

Radio frame indicator index of PRACH opportunity

Half frame index of PRACH opportunity within the radio frame

Uplink subframe number for start of PRACH opportunity within the half frame

Radio frame duration

Basic time unit

Slot duration

Precoding matrix for downlink spatial multiplexing

Amplitude scaling for PRACH

Amplitude scaling for NPRACH

Amplitude scaling for PUCCH

Amplitude scaling for PUSCH

Amplitude scaling for NPUSCH

Amplitude scaling for sounding reference symbols

Subcarrier spacing

Subcarrier spacing for the random access preamble

Number of transmission layers

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

CCE	Control Channel Element

CDD	Cyclic Delay Diversity

CRS	Cell-specific Reference Signal

CSI	Channel-State Information

DCI	Downlink Control Information

DM-RS	Demodulation Reference Signal

ECCE	Enhanced Control Channel Element

EPDCCH	Enhanced Physical Downlink Control CHannel

EREG	Enhanced Resource-Element Group

MPDCCH	MTC Physical Downlink Control Channel

NCCE	Narrowband Control Channel Element

NPBCH	Narrowband Physical Broadcast CHannel

NPDCCH	Narrowband Physical Downlink Control CHannel

NPDSCH	Narrowband Physical Downlink Shared CHannel

NPRACH	Narrowband Physical Random Access CHannel

NPUSCH	Narrowband Physical Uplink Shared CHannel

NPRS	Narrowband Positioning Reference Signal

NPSS	Narrowband Primary Synchronization Signal

NSSS 	Narrowband Secondary Synchronization Signal

NRS	Narrowband Reference Signal PBCH	Physical Broadcast CHannel

PCFICH	Physical Control Format Indicator CHannel

PDCCH	Physical Downlink Control CHannel

PDSCH	Physical Downlink Shared CHannel

PHICH	Physical Hybrid-ARQ Indicator CHannel

PMCH	Physical Multicast CHannel

PRACH	Physical Random Access CHannel

PRB	Physical Resource Block

PRS	Positioning Reference Signal

PSBCH	Physical Sidelink Broadcast CHannel

PSCCH	Physical Sidelink Control CHannel

PSDCH	Physical Sidelink Discovery CHannel

PSSCH	Physical Sidelink Shared CHannel

PUCCH	Physical Uplink Control CHannel

PUSCH	Physical Uplink Shared CHannel

REG	Resource-Element Group

SCG	Secondary Cell Group

SRS	Sounding Reference Signal

VRB	Virtual Resource Block

# 4	Frame structure

Throughout this specification, unless otherwise noted, the size of various fields in the time domain is expressed as a number of time units  seconds.

Downlink, uplink and sidelink transmissions are organized into radio frames with  duration. 
Three radio frame structures are supported:

-	Type 1, applicable to FDD only,

-	Type 2, applicable to TDD only,

-	Type 3, applicable to LAA secondary cell operation only.

NOTE: LAA secondary cell operation only applies to frame structure type 3.

Transmissions in multiple cells can be aggregated where up to 31 secondary cells can be used in addition to the primary cell. Unless otherwise noted, the description in this specification applies to each of the up to 32 serving cells. In case of multi-cell aggregation, different frame structures can be used in the different serving cells.

## 4.1	Frame structure type 1

Frame structure type 1 is applicable to both full duplex and half duplex FDD only. Each radio frame is  long and consists of 10 subframes of length , numbered from 0 to 9. Subframe  in frame  has an absolute subframe number  where  is the system frame number.

For subframes using  or , subframe  is defined as two slots,  and , of length  each.

For subframes using , subframe  is defined as one slot, , of length .

For FDD, 10 subframes are available for downlink transmission and 10 subframes are available for uplink transmissions in each 10 ms interval. Uplink and downlink transmissions are separated in the frequency domain. In half-duplex FDD operation, the UE cannot transmit and receive at the same time while there are no such restrictions in full-duplex FDD.

Figure 4.1-1: Frame structure type 1

## 4.2	Frame structure type 2

Frame structure type 2 is applicable to TDD only. Each radio frame of length  consists of two half-frames of length each. Each half-frame consists of five subframes of length. Each subframe is defined as two slots, and, of length  each. Subframe  in frame  has an absolute subframe number  where  is the system frame number.

The uplink-downlink configuration in a cell may vary between frames and controls in which subframes uplink or downlink transmissions may take place in the current frame. The uplink-downlink configuration in the current frame is obtained according to Clause 13 in [4].

The supported uplink-downlink configurations are listed in Table 4.2-2 where, for each subframe in a radio frame, "D" denotes a downlink subframe reserved for downlink transmissions, "U" denotes an uplink subframe reserved for uplink transmissions and "S" denotes a special subframe with the three fields DwPTS, GP and UpPTS. The length of DwPTS and UpPTS is given by Table 4.2-1 subject to the total length of DwPTS, GP and UpPTS being equal to where X is the number of additional SC-FDMA symbols in UpPTS provided by the higher layer parameter srs-UpPtsAdd if configured otherwise X is equal to 0. The UE is not expected to be configured with 2 additional UpPTS SC-FDMA symbols for special subframe configurations {3, 4, 7, 8} for normal cyclic prefix in downlink and special subframe configurations {2, 3, 5, 6} for extended cyclic prefix in downlink and 4 additional UpPTS SC-FDMA symbols for special subframe configurations {1, 2, 3, 4, 6, 7, 8} for normal cyclic prefix in downlink and special subframe configurations {1, 2, 3, 5, 6} for extended cyclic prefix in downlink.

Uplink-downlink configurations with both 5 ms and 10 ms downlink-to-uplink switch-point periodicity are supported.

-	In case of 5 ms downlink-to-uplink switch-point periodicity, the special subframe exists in both half-frames.

-	In case of 10 ms downlink-to-uplink switch-point periodicity, the special subframe exists in the first half-frame only.

Subframes 0 and 5 and DwPTS are always reserved for downlink transmission. UpPTS and the subframe immediately following the special subframe are always reserved for uplink transmission.

In case multiple cells are aggregated, the UE may assume that the guard period of the special subframe in the cells using frame structure type 2 have an overlap of at least .

In case multiple cells with different uplink-downlink configurations in the current radio frame are aggregated and the UE is not capable of simultaneous reception and transmission in the aggregated cells, the following constraints apply:

-	if the subframe in the primary cell is a downlink subframe, the UE shall not transmit any signal or channel on a secondary cell in the same subframe

-	if the subframe in the primary cell is an uplink subframe, the UE is not expected to receive any downlink transmissions on a secondary cell in the same subframe

-	if the subframe in the primary cell is a special subframe and the same subframe in a secondary cell is a downlink subframe, the UE is not expected to receive PDSCH/EPDCCH/PMCH/PRS transmissions in the secondary cell in the same subframe, and the UE is not expected to receive any other signals on the secondary cell in OFDM symbols that overlaps with the guard period or UpPTS in the primary cell.

Figure 4.2-1: Frame structure type 2 (for 5 ms switch-point periodicity)

Table 4.2-1: Configuration of special subframe (lengths of DwPTS/GP/UpPTS)

| Special subframe configuration | Normal cyclic prefix in downlink |  |  | Extended cyclic prefix in downlink |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DwPTS | UpPTS |  | DwPTS | UpPTS |  |
|  |  | Normal cyclic prefix in uplink | Extended cyclic prefix in uplink |  | Normal cyclic prefix in uplink | Extended cyclic prefix in uplink |
| 0 |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |
| 8 |  |  |  | - | - | - |
| 9 |  |  |  | - | - | - |
| 10 |  |  |  | - | - | - |

Table 4.2-2: Uplink-downlink configurations

| Uplink-downlink configuration | Downlink-to-Uplink Switch-point periodicity | Subframe number |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | 5 ms | D | S | U | U | U | D | S | U | U | U |
| 1 | 5 ms | D | S | U | U | D | D | S | U | U | D |
| 2 | 5 ms | D | S | U | D | D | D | S | U | D | D |
| 3 | 10 ms | D | S | U | U | U | D | D | D | D | D |
| 4 | 10 ms | D | S | U | U | D | D | D | D | D | D |
| 5 | 10 ms | D | S | U | D | D | D | D | D | D | D |
| 6 | 5 ms | D | S | U | U | U | D | S | U | U | D |

## 4.3	Frame structure type 3

Frame structure type 3 is applicable to LAA secondary cell operation with normal cyclic prefix only. Each radio frame is  long and consists of 20 slots of length, numbered from 0 to 19. A subframe is defined as two consecutive slots where subframe  consists of slots and.

The 10 subframes within a radio frame are available for downlink or uplink transmissions. Downlink transmissions occupy one or more consecutive subframes, starting anywhere within a subframe and ending with the last subframe either fully occupied or following one of the DwPTS durations in Table 4.2-1. Uplink transmisisons occupy one or more consecutive subframes.

# 5	Uplink

## 5.1	Overview

The smallest resource unit for uplink transmissions is denoted a resource element and is defined in clause 5.2.2.

### 5.1.1	Physical channels

An uplink physical channel corresponds to a set of resource elements carrying information originating from higher layers and is the interface defined between 3GPP TS 36.212 [3] and the present document 3GPP TS 36.211. 
The following uplink physical channels are defined:

-	Physical Uplink Shared Channel, PUSCH

-	Physical Uplink Control Channel, PUCCH

-	Physical Random Access Channel, PRACH

### 5.1.2	Physical signals

An uplink physical signal is used by the physical layer but does not carry information originating from higher layers. The following uplink physical signals are defined:

-	Reference signal

## 5.2	Slot structure and physical resources

### 5.2.1	Resource grid

The transmitted signal in each slot is described by one or several resource grids of  subcarriers and  SC-FDMA symbols. The resource grid is illustrated in Figure 5.2.1-1. The quantity  depends on the uplink transmission bandwidth configured in the cell and shall fulfil

where  and  are the smallest and largest uplink bandwidths, respectively, supported by the current version of this specification. The set of allowed values for  is given by 3GPP TS 36.101 [7].

The number of SC-FDMA symbols in a slot depends on the cyclic prefix length configured by the higher layer parameter UL-CyclicPrefixLength and is given in Table 5.2.3-1.

An antenna port is defined such that the channel over which a symbol on the antenna port is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed. There is one resource grid per antenna port. The antenna ports used for transmission of a physical channel or signal depends on the number of antenna ports configured for the physical channel or signal as shown in Table 5.2.1-1. The index  is used throughout clause 5 when a sequential numbering of the antenna ports is necessary.

Figure 5.2.1-1: Uplink resource grid

Table 5.2.1-1: Antenna ports used for different physical channels and signals

| Physical channel or signal | Index | Antenna port number as a function of the number of antenna ports configured for the respective physical channel/signal |  |  |
| --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 4 |
| PUSCH | 0 | 10 | 20 | 40 |
|  | 1 | - | 21 | 41 |
|  | 2 | - | - | 42 |
|  | 3 | - | - | 43 |
| SRS | 0 | 10 | 20 | 40 |
|  | 1 | - | 21 | 41 |
|  | 2 | - | - | 42 |
|  | 3 | - | - | 43 |
| PUCCH | 0 | 100 | 200 | - |
|  | 1 | - | 201 | - |

### 5.2.2	Resource elements

Each element in the resource grid is called a resource element and is uniquely defined by the index pair  in a slot where  and  are the indices in the frequency and time domains, respectively. Resource element  on antenna port  corresponds to the complex value . 
When there is no risk for confusion, or no particular antenna port is specified, the index  may be dropped. 
Quantities  corresponding to resource elements not used for transmission of a physical channel or a physical signal in a slot shall be set to zero.

### 5.2.3	Resource blocks

A physical resource block is defined as consecutive SC-FDMA symbols in the time domain and consecutive subcarriers in the frequency domain, where  and  are given by Table 5.2.3-1. 
A physical resource block in the uplink thus consists of  resource elements, corresponding to one slot in the time domain and 180 kHz in the frequency domain.

Table 5.2.3-1: Resource block parameters

| Configuration |  |  |
| --- | --- | --- |
| Normal cyclic prefix | 12 | 7 |
| Extended cyclic prefix | 12 | 6 |

The relation between the physical resource block number  in the frequency domain and resource elements  in a slot is given by

### 5.2.4	Narrowbands and widebands

A narrowband is defined as six non-overlapping consecutive physical resource blocks in the frequency domain. The total number of uplink narrowbands in the uplink transmission bandwidth configured in the cell is given by

The narrowbands are numbered  in order of increasing physical resource-block number where narrowband is composed of physical resource-block indices

where

If , a wideband is defined as four non-overlapping narrowbands in the frequency domain. The total number of uplink widebands in the uplink transmission bandwidth configured in the cell is given by

and the widebands are numbered  in order of increasing narrowband number where wideband  is composed of narrowband indices  where .

If , then  and the single wideband is composed of the  non-overlapping narrowband(s).

### 5.2.5	Guard period for narrowband and wideband retuning

For BL/CE UEs, a guard period of at most  SC-FDMA symbols is created for Tx-to-Tx frequency retuning between two consecutive subframes. If the higher layer parameter ce-RetuningSymbols is set, then  equals ce-RetuningSymbols, otherwise . If the higher layer parameter ce-pusch-maxBandwidth-config is set to 5 MHz, then the rules for guard period creation defined in the remainder of this clause do not apply for retuning between narrowbands but for retuning between widebands and for transmissions involving multiple widebands.

-	If the UE retunes from a first narrowband carrying PUSCH to a second narrowband carrying PUSCH, or if the UE retunes from a first narrowband carrying PUCCH to a second narrowband carrying PUCCH,

-	if , a guard period is created by the UE not transmitting the last SC-FDMA symbol in the first subframe;

-	if , a guard period is created by the UE not transmitting the last SC-FDMA symbol in the first subframe and the first SC-FDMA symbol in the second subframe.

-	If the UE retunes from a first narrowband carrying PUCCH to a second narrowband carrying PUSCH,

-	if the PUCCH uses a shortened PUCCH format and , a guard period is created by the UE not transmitting the last SC-FDMA symbol in the first subframe;

-	if the PUCCH uses a shortened PUCCH format and , a guard period is created by the UE not transmitting the last SC-FDMA symbol in the first subframe and the first SC-FDMA symbol in the second subframe;

-	if the PUCCH uses a normal PUCCH format, a guard period is created by the UE not transmitting the first  SC-FDMA symbols in the second subframe.

-	If the UE retunes from a first narrowband carrying PUSCH to a second narrowband carrying PUCCH,

-	a guard period is created by the UE not transmitting the last  SC-FDMA symbols in the first subframe.

-	For CEModeA, if the PUSCH is associated with C-RNTI or SPS C-RNTI and the higher layer parameter ce-pusch-maxBandwidth-config is set to 5 MHz,

-	If the PUSCH resource allocation is within a 5 MHz wideband, the center frequency of the transmission bandwidth is the center frequency of the wideband;

-	If the PUSCH resource allocation spans two 5 MHz widebands, the center frequency of transmission bandwidth is in the center of PUSCH resource allocation.

Furthermore, for BL/CE UEs configured with the higher layer parameter srs-UpPtsAdd, a guard period of at most  SC-FDMA symbols is created for Tx-to-Tx frequency retuning between a first special subframe and a second uplink subframe for frame structure type 2 according to:

-	If the UE retunes from a first narrowband carrying SRS in the last UpPTS symbol to a second narrowband carrying PUSCH,

-	a guard period is created by the UE not transmitting the first  SC-FDMA symbols in the second subframe.

-	If the UE retunes from a first narrowband carrying SRS in the last but one UpPTS symbol, but not in the last UpPTS symbol, to a second narrowband carrying PUSCH,

-	if , a guard period is created by the UE not transmitting the last UpPTS symbol in the first subframe;

-	if , a guard period is created by the UE not transmitting the last UpPTS symbol in the first subframe and the first SC-FDMA symbol in the second subframe.

-	If the UE retunes from a first narrowband carrying SRS to a second narrowband carrying PUCCH,

-	if , a guard period is created by the UE not transmitting the last UpPTS symbol in the first subframe;

-	if , a guard period is created by the UE not transmitting the last UpPTS symbol in the first subframe and the first SC-FDMA symbol in the second subframe.

For , and for SRS transmission in a special subframe, a BL/CE UE is not expected to be configured with a first SRS transmission in symbol l and a second SRS transmission in any of symbols if the first SRS transmission and the second SRS transmission use different narrowbands.

## 5.3	Physical uplink shared channel

The baseband signal representing the physical uplink shared channel is defined in terms of the following steps:

-	scrambling

-	modulation of scrambled bits to generate complex-valued symbols

-	mapping of the complex-valued modulation symbols onto one or several transmission layers

-	transform precoding to generate complex-valued symbols

-	precoding of the complex-valued symbols

-	mapping of precoded complex-valued symbols to resource elements

-	generation of complex-valued time-domain SC-FDMA signal for each antenna port

Figure 5.3-1: Overview of uplink physical channel processing

### 5.3.1	Scrambling

For each codeword , the block of bits , where  is the number of bits transmitted in codeword  on the physical uplink shared channel in one subframe, shall be scrambled with a UE-specific scrambling sequence prior to modulation, resulting in a block of scrambled bits  according to the following pseudo code

Set i = 0

while

if 	// ACK/NACK or Rank Indication placeholder bits

else

if 	// ACK/NACK or Rank Indication repetition placeholder bits

else	// Data or channel quality coded bits, Rank Indication coded bits or ACK/NACK coded bits

end if

end if

i = i + 1

end while

where x and y are tags defined in 3GPP TS 36.212 [3] clause 5.2.2.6 and where the scrambling sequence  is given by clause 7.2. The scrambling sequence generator shall be initialised with  at the start of each subframe where  corresponds to the RNTI associated with the PUSCH transmission as described in clause 8 in 3GPP TS 36.213 [4].

For BL/CE UEs, the same scrambling sequence is applied per subframe to PUSCH for a given block of  subframes. The subframe number of the first subframe in each block of  consecutive subframes, denoted as , satisfies . For the block of  subframes, the scrambling sequence generator shall be initialised with

where

and  is the absolute subframe number of the first uplink subframe intended for PUSCH. The PUSCH transmission spans  consecutive subframes including non-BL/CE UL subframes where the UE postpones the PUSCH transmission. For a BL/CE UE configured in CEModeA, . For a BL/CE UE configured with CEModeB,  for frame structure type 1 and  for frame structure type 2.

Up to two codewords can be transmitted in one subframe, i.e., . In the case of single-codeword transmission, .

### 5.3.2	Modulation

For each codeword, the block of scrambled bits  shall be modulated as described in clause 7.1, resulting in a block of complex-valued symbols . Table 5.3.2-1 specifies the modulation mappings applicable for the physical uplink shared channel.

Table 5.3.2-1: Uplink modulation schemes

| Physical channel | Modulation schemes |
| --- | --- |
| PUSCH | QPSK, 16QAM, 64QAM, 256QAM |

### 5.3.2A	Layer mapping

The complex-valued modulation symbols for each of the codewords to be transmitted are mapped onto one or two layers. Complex-valued modulation symbols  for codeword  shall be mapped onto the layers ,  where  is the number of layers and  is the number of modulation symbols per layer.

#### 5.3.2A.1	Layer mapping for transmission on a single antenna port

For transmission on a single antenna port, a single layer is used, , and the mapping is defined by

with .

#### 5.3.2A.2	Layer mapping for spatial multiplexing

For spatial multiplexing, the layer mapping shall be done according to Table 5.3.2A.2-1. The number of layers  is less than or equal to the number of antenna ports  used for transmission of the physical uplink shared channel. 
The case of a single codeword mapped to multiple layers is only applicable when the number of antenna ports used for PUSCH is four.

Table 5.3.2A.2-1: Codeword-to-layer mapping for spatial multiplexing

| Number of layers | Number of codewords | Codeword-to-layer mapping |  |
| --- | --- | --- | --- |
| 1 | 1 |  |  |
| 2 | 1 |  |  |
| 2 | 2 |  |  |
| 3 | 2 |  |  |
| 4 | 2 |  |  |

### 5.3.3	Transform precoding

For each layer  the block of complex-valued symbols  is divided into  sets, each corresponding to one SC-FDMA symbol. Transform precoding shall be applied according to

resulting in a block of complex-valued symbols . The variable, where  represents the bandwidth of the PUSCH in terms of resource blocks, and shall fulfil

where  is a set of non-negative integers.

### 5.3.3A	Precoding

The precoder takes as input a block of vectors ,  from the transform precoder and generates a block of vectors ,  to be mapped onto resource elements.

#### 5.3.3A.1	Precoding for transmission on a single antenna port

For transmission on a single antenna port, precoding is defined by

where , .

#### 5.3.3A.2	Precoding for spatial multiplexing

Precoding for spatial multiplexing is only used in combination with layer mapping for spatial multiplexing as described in clause 5.3.2A.2. Spatial multiplexing supports  or  antenna ports where the set of antenna ports used for spatial multiplexing is  and , respectively.

Precoding for spatial multiplexing is defined by

where , .

The precoding matrix  of size  is given by one of the entries in Table 5.3.3A.2-1 for  and by Tables 5.3.3A.2-2 through 5.3.3A.2-5 for  where the entries in each row are ordered from left to right in increasing order of codebook indices.

Table 5.3.3A.2-1: Codebook for transmission on antenna ports

| Codebook index | Number of layers |  |
| --- | --- | --- |
| 0 |  |  |
| 1 |  | - |
| 2 |  | - |
| 3 |  | - |
| 4 |  | - |
| 5 |  | - |

Table 5.3.3A.2-2: Codebook for transmission on antenna ports  with

| Codebook index | Number of layers |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 – 7 |  |  |  |  |  |  |  |  |
| 8 – 15 |  |  |  |  |  |  |  |  |
| 16 – 23 |  |  |  |  |  |  |  |  |

Table 5.3.3A.2-3: Codebook for transmission on antenna ports  with

| Codebook index | Number of layers |  |  |  |
| --- | --- | --- | --- | --- |
| 0 – 3 |  |  |  |  |
| 4 – 7 |  |  |  |  |
| 8 – 11 |  |  |  |  |
| 12 – 15 |  |  |  |  |

Table 5.3.3A.2-4: Codebook for transmission on antenna ports  with

| Codebook index | Number of layers |  |  |  |
| --- | --- | --- | --- | --- |
| 0 – 3 |  |  |  |  |
| 4 – 7 |  |  |  |  |
| 8 – 11 |  |  |  |  |

Table 5.3.3A.2-5: Codebook for transmission on antenna ports  with

| Codebook index | Number of layers |
| --- | --- |
| 0 |  |

### 5.3.4	Mapping to physical resources

For each antenna port  used for transmission of the PUSCH in a subframe the block of complex-valued symbols  shall be multiplied with the amplitude scaling factor  in order to conform to the transmit power specified in clause 5.1.1.1 in 3GPP TS 36.213 [4], and mapped in sequence starting with  to physical resource blocks on antenna port  and assigned for transmission of PUSCH. The relation between the index  and the antenna port number  is given by Table 5.2.1-1. The mapping to resource elements  corresponding to the physical resource blocks assigned for transmission and

-	not used for transmission of reference signals, and

-	not part of the last SC-FDMA symbol in a subframe, if the UE transmits SRS in the same subframe in the same serving cell, and

-	not part of the last SC-FDMA symbol in a subframe configured with cell-specific SRS for non-BL/CE UEs and BL/CE UEs in CEModeA, if the PUSCH transmission partly or fully overlaps with the cell-specific SRS bandwidth, and

-	not part of an SC-FDMA symbol reserved for possible SRS transmission in a UE-specific aperiodic SRS subframe in the same serving cell, and

-	not part of an SC-FDMA symbol reserved for possible SRS transmission in a UE-specific periodic SRS subframe in the same serving cell when the UE is configured with multiple TAGs

-	not part of the first SC-FDMA symbol in a subframe if the associated DCI indicates PUSCH starting position '01', '10', or '11'

-	not part of the last SC-FDMA symbol in a subframe if the associated DCI indicates PUSCH ending symbol '1'

shall be in increasing order of first the index , then the index, starting with the first slot in an uplink subframe. For the UpPTS, the mapping shall start at symbol  and if dmrsLess-UpPts is set to true the mapping shall end at symbol  in the second slot of a special subframe, otherwise, the mapping shall end at symbol  in the second slot of a special subframe.

For BL/CE UEs, the PUSCH transmission is restricted as follows:

-	For CEModeA, if the PUSCH is associated with C-RNTI or SPS C-RNTI and the higher layer parameter ce-pusch-maxBandwidth-config is set to 5 MHz, the maximum number of allocatable PRBs for PUSCH is 24 PRBs. The allocatable PRBs include the PRBs belonging to the narrowbands defined in clause 5.2.4 and the odd PRB at the center of the uplink system bandwidth in case of odd total number of uplink PRBs. If a resource assignment or frequency hopping would result in a PUSCH resource allocation outside the allocatable PRBs then the PUSCH transmission in that subframe is dropped.

-	For all other cases, the maximum number of allocatable PRBs for PUSCH is 6 PRBs restricted to one of the narrowbands defined in clause 5.2.4.

For BL/CE UEs in CEModeB, resource elements in the last SC-FDMA symbol in a subframe configured with cell-specific SRS shall be counted in the PUSCH mapping but not used for transmission of the PUSCH.

For BL/CE UEs, if one or more SC-FDMA symbol(s) are left empty due to guard period for narrowband or wideband retuning, the affected SC-FDMA symbol(s) shall be counted in the PUSCH mapping but not used for transmission of the PUSCH.

For a UE configured with SRS carrier switching, if the first symbol in a subframe overlaps with an SRS transmission (including any interruption due to uplink or downlink RF retuning time) in a carrier without PUSCH/PUCCH, the resource elements in the first SC-FDMA symbol shall be counted in the PUSCH mapping but not used for transmission of PUSCH.

For a UE configured with SRS carrier switching, if the last symbol in a subframe is counted in the PUSCH mapping and the last symbol in the subframe overlaps with an SRS transmission (including any interruption due to uplink or downlink RF retuning time) in a carrier without PUSCH/PUCCH, the resource elements in the last SC-FDMA symbol shall be counted in the PUSCH mapping but not used for transmission of PUSCH.

For a UE configured with SRS carrier switching, if the last symbol in a subframe is not counted in the PUSCH mapping and the second-to-last symbol in the subframe overlaps with an SRS transmission (including any interruption due to uplink or downlink RF retuning time) in a carrier without PUSCH/PUCCH, the resource elements in the second-to-last SC-FDMA symbol shall be counted in the PUSCH mapping but not used for transmission of PUSCH.

If uplink frequency-hopping is disabled or the resource blocks allocated for PUSCH transmission are not contiguous in frequency, the set of physical resource blocks to be used for transmission is given by  where  is obtained from the uplink scheduling grant as described in clause 8.1 in 3GPP TS 36.213 [4].

If uplink frequency-hopping with type 1 PUSCH hopping is enabled, the set of physical resource blocks to be used for transmission is given by clause 8.4.1 in 3GPP TS 36.213 [4].

If uplink frequency-hopping with predefined hopping pattern is enabled, the set of physical resource blocks to be used for transmission in slot  is given by the scheduling grant together with a predefined pattern according to

where  is obtained from the scheduling grant as described in clause 8.1 in 3GPP TS 36.213 [4]. The parameter pusch-HoppingOffset,, is provided by higher layers. The size  of each sub-band is given by,

where the number of sub-bands  is given by higher layers. The function  determines whether mirroring is used or not. The parameter Hopping-mode provided by higher layers determines if hopping is "inter-subframe" or "intra and inter-subframe".

The hopping function and the function are given by

where  and the pseudo-random sequence  is given by clause 7.2 and CURRENT_TX_NB indicates the transmission number for the transport block transmitted in slot as defined in [8]. The pseudo-random sequence generator shall be initialised with  for frame structure type 1 and  for frame structure type 2 at the start of each frame.

For BL/CE UEs, the PRB resources for PUSCH transmission in the first subframe are obtained from the DCI as described in clauses 5.3.3.1.10 and 5.3.3.1.11 in [3]. The PUSCH is transmitted with repetitions. The PUSCH transmission spans  consecutive subframes, including non-BL/CE UL subframes where the UE postpones the PUSCH transmission if . For BL/CE UE in CEModeA, PUSCH frequency hopping is enabled when the higher-layer parameter pusch-HoppingConfig is set and the frequency hopping flag in DCI format 6-0A indicates frequency hopping, otherwise frequency hopping is disabled. For BL/CE UE in CEModeB, PUSCH frequency hopping is enabled when the higher-layer parameter pusch-HoppingConfig is set, otherwise frequency hopping is disabled. If frequency hopping is not enabled for PUSCH, all PUSCH repetitions are located at the same PRB resources. If frequency hopping is enabled for PUSCH, PUSCH is transmitted in subframe  within the  consecutive uplink subframes using the same number of consecutive PRBs as in the previous subframe starting from the same starting PRB resource within narrowband

where  is the absolute subframe number of the first UL subframe intended for carrying the PUSCH and  and  are cell-specific higher-layer parameters. For the  consecutive subframes, the UE shall not transmit PUSCH in subframe  if it is not a BL/CE UL subframe.

For BL/CE UEs, for PUSCH transmission corresponding to the random access response grant and its retransmission, frequency hopping of the PUSCH is enabled when higher layer parameter rar-HoppingConfig is set. Further

-	if PRACH CE level 0 or 1 is used for the last PRACH attempt,  is set to the higher layer parameter interval-UlHoppingConfigCommonModeA;

-	if PRACH CE level 2 or 3 is used for the last PRACH attempt,  is set to the higher layer parameter interval-UlHoppingConfigCommonModeB.

For BL/CE UEs in CEModeB, for PUSCH transmission not associated with Temporary C-RNTI, for frame structure type 1, after a transmission duration of  time units (which may include non-BL/CE UL subframes), a gap of  time units shall be inserted, according to the UE capability ue-CE-NeedULGaps, as specified in TS 36.331 [9]. BL/CE UL subframes within the gap of  time units shall be counted for the PUSCH resource mapping but not used for transmission of the PUSCH.

For BL/CE UEs, for PUSCH transmission associated with Temporary C-RNTI for frame structure type 1, and if PRACH CE level 2 or 3 is used for the last PRACH attempt, after a transmission duration of  time units (which may include non-BL/CE UL subframes), a gap of  time units shall be inserted. BL/CE UL subframes within the gap of  time units shall be counted for the PUSCH resource mapping but not used for transmission of the PUSCH.

For UEs configured with PUSCH-EnhancementsConfig, the number of PUSCH subframe repetitions  and the PRB resources for PUSCH transmission in the first subframe are obtained from the DCI as described in clause 5.3.3.1.1C in [3]. PUSCH frequency hopping is enabled when the higher-layer parameters pusch-HoppingOffsetPUSCH-Enh and interval-ULHoppingPUSCH-Enh are set and the frequency hopping flag in DCI format 0C indicates frequency hopping, otherwise frequency hopping is disabled. If frequency hopping is not enabled for PUSCH, the PUSCH repetitions are located at the same PRB resources as in the first subframe. If frequency hopping is enabled for PUSCH, PUSCH is transmitted in subframe  within the  consecutive uplink subframes using the PRB resources starting at PRB index

where  is the absolute subframe number of the first UL subframe carrying the PUSCH and  is given by the higher-layer parameter interval-ULHoppingPUSCH-Enh and  is given by the higher-layer parameter pusch-HoppingOffsetPUSCH-Enh.

## 5.4	Physical uplink control channel

The physical uplink control channel, PUCCH, carries uplink control information. Simultaneous transmission of PUCCH and PUSCH from the same UE is supported if enabled by higher layers. For frame structure type 2, the PUCCH is not transmitted in the UpPTS field.

The physical uplink control channel supports multiple formats as shown in Table 5.4-1 with different number of bits per subframe, where  represents the bandwidth of the PUCCH format 4 as defined by clause 5.4.2B, and  and  are defined in Table 5.4.2C-1.

Formats 2a and 2b are supported for normal cyclic prefix only.

Table 5.4-1: Supported PUCCH formats

| PUCCH format | Modulation scheme | Number of bits per subframe, |
| --- | --- | --- |
| 1 | N/A | N/A |
| 1a | BPSK | 1 |
| 1b | QPSK | 2 |
| 2 | QPSK | 20 |
| 2a | QPSK+BPSK | 21 |
| 2b | QPSK+QPSK | 22 |
| 3 | QPSK | 48 |
| 4 | QPSK |  |
| 5 | QPSK |  |

All PUCCH formats use a cyclic shift, , which varies with the symbol number  and the slot number  according to

where the pseudo-random sequence  is defined by clause 7.2. The pseudo-random sequence generator shall be initialized with , where  is given by clause 5.5.1.5 with  corresponding to the primary cell, at the beginning of each radio frame.

The physical resources used for PUCCH depends on two parameters,  and , given by higher layers. 
The variable  denotes the bandwidth in terms of resource blocks that are available for use by PUCCH formats 2/2a/2b transmission in each slot. The variable  denotes the number of cyclic shift used for PUCCH formats 1/1a/1b in a resource block used for a mix of formats 1/1a/1b and 2/2a/2b. The value of  is an integer multiple of  within the range of {0, 1, …, 7}, where  is provided by higher layers. No mixed resource block is present if . At most one resource block in each slot supports a mix of formats 1/1a/1b and 2/2a/2b. 
Resources used for transmission of PUCCH formats 1/1a/1b, 2/2a/2b, 3, 4, and 5 are represented by the non-negative indices , , ,  and , respectively.

### 5.4.1	PUCCH formats 1, 1a and 1b

For PUCCH format 1, information is carried by the presence/absence of transmission of PUCCH from the UE. 
In the remainder of this clause,  shall be assumed for PUCCH format 1.

For PUCCH formats 1a and 1b, one or two explicit bits are transmitted, respectively. The block of bits  shall be modulated as described in Table 5.4.1-1, resulting in a complex-valued symbol. 
The modulation schemes for the different PUCCH formats are given by Table 5.4-1.

The complex-valued symbol  shall be multiplied with a cyclically shifted length  sequence  for each of the  antenna ports used for PUCCH transmission according to

where  is defined by clause 5.5.1 with and . The antenna-port specific cyclic shift  varies between symbols and slots as defined below.

The block of complex-valued symbols  shall be scrambled by  and block-wise spread with the antenna-port specific orthogonal sequence  according to

where

and

with  for the two slots in a subframe given by Table 5.4.1-1a. The sequence  is given by Table 5.4.1-2 and Table 5.4.1-3 and  is defined below.

Resources used for transmission of PUCCH format 1, 1a and 1b are identified by a resource index  from which the orthogonal sequence index  and the cyclic shift  are determined according to

where

The resource indices within the two resource blocks in the two slots of a subframe to which the PUCCH is mapped are given by

for  and by

for , where , with for normal CP and for extended CP.

The parameter deltaPUCCH-Shift  is provided by higher layers.

Table 5.4.1-1: Modulation symbol  for PUCCH formats 1a and 1b

| PUCCH format |  |  |
| --- | --- | --- |
| 1a | 0 |  |
|  | 1 |  |
| 1b | 00 |  |
|  | 01 |  |
|  | 10 |  |
|  | 11 |  |

Table 5.4.1-1a: The quantity  for PUCCH formats 1a and 1b

| PUCCH format |  |  |
| --- | --- | --- |
|  | first slot | second slot |
| normal 1/1a/1b | 4 | 4 |
| shortened 1/1a/1b | 4 | 3 |

Table 5.4.1-2: Orthogonal sequences  for

| Sequence index | Orthogonal sequences |
| --- | --- |
| 0 |  |
| 1 |  |
| 2 |  |

Table 5.4.1-3: Orthogonal sequences  for

| Sequence index | Orthogonal sequences |
| --- | --- |
| 0 |  |
| 1 |  |
| 2 |  |

### 5.4.2	PUCCH formats 2, 2a and 2b

The block of bits  shall be scrambled with a UE-specific scrambling sequence, resulting in a block of scrambled bits  according to

where the scrambling sequence  is given by clause 7.2. The scrambling sequence generator shall be initialised with  at the start of each subframe where  is C-RNTI.

The block of scrambled bits  shall be QPSK modulated as described in clause 7.1, resulting in a block of complex-valued modulation symbols .

Each complex-valued symbol  shall be multiplied with a cyclically shifted length  sequence  for each of the  antenna ports used for PUCCH transmission according to

where  is defined by clause 5.5.1 with and .

Resources used for transmission of PUCCH formats 2/2a/2b are identified by a resource index  from which the cyclic shift  is determined according to

where

and

for  and by

for .

For PUCCH formats 2a and 2b, supported for normal cyclic prefix only, the bit(s)  shall be modulated as described in Table 5.4.2-1 resulting in a single modulation symbol  used in the generation of the reference-signal for PUCCH format 2a and 2b as described in clause 5.5.2.2.1.

Table 5.4.2-1: Modulation symbol  for PUCCH formats 2a and 2b

| PUCCH format |  |  |
| --- | --- | --- |
| 2a | 0 |  |
|  | 1 |  |
| 2b | 00 |  |
|  | 01 |  |
|  | 10 |  |
|  | 11 |  |

### 5.4.2A	PUCCH format 3

The block of bits  shall be scrambled with a UE-specific scrambling sequence, resulting in a block of scrambled bits  according to

where the scrambling sequence  is given by clause 7.2. The scrambling sequence generator shall be initialised with  at the start of each subframe where  is the C-RNTI.

The block of scrambled bits  shall be QPSK modulated as described in Subclause 7.1, resulting in a block of complex-valued modulation symbols  where .

The complex-valued symbols  shall be block-wise spread with the orthogonal sequences  and  resulting in  sets of  values each according to

where  for both slots in a subframe using normal PUCCH format 3 and , holds for the first and second slot, respectively, in a subframe using shortened PUCCH format 3. The orthogonal sequences  and  are given by Table 5.4.2A-1. Resources used for transmission of PUCCH formats 3 are identified by a resource index  from which the quantities  and  are derived according to

Each set of complex-valued symbols shall be cyclically shifted according to

where  is given by Subclause 5.4,  is the slot number within a radio frame and  is the SC-FDMA symbol number within a slot.

The shifted sets of complex-valued symbols shall be transform precoded according to

where  is the number of antenna ports used for PUCCH transmission, resulting in a block of complex-valued symbols .

Table 5.4.2A-1: The orthogonal sequence

| Sequence index | Orthogonal sequence |  |
| --- | --- | --- |
| 0 |  |  |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  | - |

### 5.4.2B	PUCCH format 4

The block of bits  shall be scrambled with a UE-specific scrambling sequence, resulting in a block of scrambled bits  according to

where the scrambling sequence  is given by clause 7.2. The scrambling sequence generator shall be initialised with  at the start of each subframe where  is the C-RNTI.

The block of scrambled bits  shall be QPSK modulated as described in Subclause 7.1, resulting in a block of complex-valued modulation symbols  where .

The block of complex-valued symbols  is divided into  sets, each corresponding to one SC-FDMA symbol. Transform precoding shall be applied according to

where ,  and  are given by Table 5.4.2C-1 for normal PUCCH format 4 and shortened PUCCH format 4, resulting in a block of complex-valued symbols . The variable, where  represents the bandwidth of the PUCCH format 4 in terms of resource blocks, shall fulfil

where  is a set of non-negative integers.

### 5.4.2C	PUCCH format 5

The block of bits  shall be scrambled with a UE-specific scrambling sequence, resulting in a block of scrambled bits  according to

where the scrambling sequence  is given by clause 7.2. The scrambling sequence generator shall be initialised with  at the start of each subframe where  is the C-RNTI.

The block of scrambled bits  shall be QPSK modulated as described in Subclause 7.1, resulting in a block of complex-valued modulation symbols  where .

The complex-valued symbols  shall be divided into  sets, each corresponding to one SC-FDMA symbol. Block-wise spreading shall be applied according to

where ,  and  are given by Table 5.4.2C-1 for normal PUCCH format 5 and shortened PUCCH format 5, and  is given by Table 5.4.2C-2 with  provided by higher layers.

The block-wise spread complex-valued symbols shall be transform precoded according to

where , resulting in a block of complex-valued symbols .

Table 5.4.2C-1: The quantities  and

| PUCCH format type | Normal cyclic prefix |  | Extended cyclic prefix |  |
| --- | --- | --- | --- | --- |
| Normal PUCCH format | 6 | 6 | 5 | 5 |
| Shortened PUCCH format | 6 | 5 | 5 | 4 |

Table 5.4.2C-2: Orthogonal sequences

|  | Orthogonal sequences |
| --- | --- |
| 0 |  |
| 1 |  |

### 5.4.3	Mapping to physical resources

The block of complex-valued symbols  shall be multiplied with the amplitude scaling factor  in order to conform to the transmit power  specified in Subclause 5.1.2.1 in 3GPP TS 36.213 [4], and mapped in sequence starting with  to resource elements. PUCCH uses one or more resource block in each of the two slots in a subframe. Within the physical resource block(s) used for transmission, the mapping of  to resource elements  on antenna port  and not used for transmission of reference signals shall be in increasing order of first , then  and finally the slot number, starting with the first slot in the subframe. The relation between the index  and the antenna port number  is given by Table 5.2.1-1.

For non-BL/CE UEs, except for PUCCH format 4, the physical resource blocks to be used for transmission of PUCCH in slot  are given by

For BL/CE UEs, PUCCH is transmitted with repetitions. The PUCCH transmission spans  consecutive subframes, including non-BL/CE UL subframes where the UE postpones the PUCCH transmission if . The quantity  is given

-	by the higher layer parameter pucch-NumRepetitionCE-Format1 for PUCCH format 1/1a and pucch-NumRepetitionCE-Format2 for PUCCH format 2/2a/2b, if configured. Otherwise

-	by the higher-layer parameter pucch-NumRepetitionCE-Msg4-Level0-r13, pucch-NumRepetitionCE-Msg4-Level1-r13, pucch-NumRepetitionCE-Msg4-Level2-r13 or pucch-NumRepetitionCE-Msg4-Level3-r13.

The physical resource blocks to be used for transmission of PUCCH in subframe  within the  consecutive subframes are given by

where  is the absolute subframe number of the first uplink subframe intended for PUCCH.

The variable  depends on the PUCCH format.

-	Formats 1, 1a and 1b:

-	Formats 2, 2a and 2b:

-	Format 3:

-	Format 5 (non-BL/CE UEs only):

For non-BL/CE UEs, for PUCCH format 4, the physical resource blocks to be used for transmission of PUCCH in slot  are given by

where  is obtained from [4].

Mapping of modulation symbols for the physical uplink control channel for PUCCH formats 1 – 3 is illustrated in Figure 5.4.3-1.

In case of simultaneous transmission of sounding reference signal and PUCCH format 1, 1a, 1b, 3, 4 or 5 when there is one serving cell configured, the shortened PUCCH format shall be used where the last SC-FDMA symbol in the second slot of a subframe shall be left empty.

In case of guard period for narrowband or wideband retuning for BL/CE UEs, if an SC-FDMA symbol is left empty due to guard period, the SC-FDMA symbol shall be counted in the PUCCH mapping but not used for transmission of the PUCCH. The SC-FDMA symbol affected by the guard period can be the first SC-FDMA symbol in the first slot of a subframe and/or the last SC-FDMA symbol in the second slot of a subframe.

Figure 5.4.3-1: Mapping to physical resource blocks for PUCCH formats 1 – 3 for non-BL/CE UEs.

## 5.5	Reference signals

Two types of uplink reference signals are supported:

-	Demodulation reference signal, associated with transmission of PUSCH or PUCCH

-	Sounding reference signal, not associated with transmission of PUSCH or PUCCH

The same set of base sequences is used for demodulation and sounding reference signals.

### 5.5.1	Generation of the reference signal sequence

Reference signal sequence  is defined by a cyclic shift  of a base sequence  according to

where  is the length of the reference signal sequence, ,  is defined in subclause 5.5.2.1.2, and the quantity  when the higher-layer parameter ul-DMRS-IFDMA is set and the most recent uplink-related DCI contains the Cyclic Shift Field mapping table for DMRS bit field which indicates the use of Table 5.5.2.1.1-3, and  otherwise. Multiple reference signal sequences are defined from a single base sequence through different values of .

Base sequences  are divided into groups, where  is the group number and  is the base sequence number within the group, such that each group contains one base sequence () of each length ,  and two base sequences () of each length , . The sequence group number  and the number  within the group may vary in time as described in clauses 5.5.1.3 and 5.5.1.4, respectively. The definition of the base sequence  depends on the sequence length.

#### 5.5.1.1	Base sequences of length  or larger

For, the base sequence  is given by

where the  root Zadoff-Chu sequence is defined by

with  given by

The length  of the Zadoff-Chu sequence is given by the largest prime number such that.

#### 5.5.1.2	Base sequences of length less than

For , , , and , the base sequence is given by

where the value of  is given by Table 5.5.1.2-1, Table 5.5.1.2-2, Table 5.5.1.2-3, and Table 5.5.1.2-4 for , , , and , respectively. For , the base sequence  is given by

Table 5.5.1.2-1: Definition of  for .

| 0 | -1 | 1 | 3 | -3 | 3 | 3 | 1 | 1 | 3 | 1 | -3 | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 3 | 3 | 3 | -1 | 1 | -3 | -3 | 1 | -3 | 3 |
| 2 | 1 | 1 | -3 | -3 | -3 | -1 | -3 | -3 | 1 | -3 | 1 | -1 |
| 3 | -1 | 1 | 1 | 1 | 1 | -1 | -3 | -3 | 1 | -3 | 3 | -1 |
| 4 | -1 | 3 | 1 | -1 | 1 | -1 | -3 | -1 | 1 | -1 | 1 | 3 |
| 5 | 1 | -3 | 3 | -1 | -1 | 1 | 1 | -1 | -1 | 3 | -3 | 1 |
| 6 | -1 | 3 | -3 | -3 | -3 | 3 | 1 | -1 | 3 | 3 | -3 | 1 |
| 7 | -3 | -1 | -1 | -1 | 1 | -3 | 3 | -1 | 1 | -3 | 3 | 1 |
| 8 | 1 | -3 | 3 | 1 | -1 | -1 | -1 | 1 | 1 | 3 | -1 | 1 |
| 9 | 1 | -3 | -1 | 3 | 3 | -1 | -3 | 1 | 1 | 1 | 1 | 1 |
| 10 | -1 | 3 | -1 | 1 | 1 | -3 | -3 | -1 | -3 | -3 | 3 | -1 |
| 11 | 3 | 1 | -1 | -1 | 3 | 3 | -3 | 1 | 3 | 1 | 3 | 3 |
| 12 | 1 | -3 | 1 | 1 | -3 | 1 | 1 | 1 | -3 | -3 | -3 | 1 |
| 13 | 3 | 3 | -3 | 3 | -3 | 1 | 1 | 3 | -1 | -3 | 3 | 3 |
| 14 | -3 | 1 | -1 | -3 | -1 | 3 | 1 | 3 | 3 | 3 | -1 | 1 |
| 15 | 3 | -1 | 1 | -3 | -1 | -1 | 1 | 1 | 3 | 1 | -1 | -3 |
| 16 | 1 | 3 | 1 | -1 | 1 | 3 | 3 | 3 | -1 | -1 | 3 | -1 |
| 17 | -3 | 1 | 1 | 3 | -3 | 3 | -3 | -3 | 3 | 1 | 3 | -1 |
| 18 | -3 | 3 | 1 | 1 | -3 | 1 | -3 | -3 | -1 | -1 | 1 | -3 |
| 19 | -1 | 3 | 1 | 3 | 1 | -1 | -1 | 3 | -3 | -1 | -3 | -1 |
| 20 | -1 | -3 | 1 | 1 | 1 | 1 | 3 | 1 | -1 | 1 | -3 | -1 |
| 21 | -1 | 3 | -1 | 1 | -3 | -3 | -3 | -3 | -3 | 1 | -1 | -3 |
| 22 | 1 | 1 | -3 | -3 | -3 | -3 | -1 | 3 | -3 | 1 | -3 | 3 |
| 23 | 1 | 1 | -1 | -3 | -1 | -3 | 1 | -1 | 1 | 3 | -1 | 1 |
| 24 | 1 | 1 | 3 | 1 | 3 | 3 | -1 | 1 | -1 | -3 | -3 | 1 |
| 25 | 1 | -3 | 3 | 3 | 1 | 3 | 3 | 1 | -3 | -1 | -1 | 3 |
| 26 | 1 | 3 | -3 | -3 | 3 | -3 | 1 | -1 | -1 | 3 | -1 | -3 |
| 27 | -3 | -1 | -3 | -1 | -3 | 3 | 1 | -1 | 1 | 3 | -3 | -3 |
| 28 | -1 | 3 | -3 | 3 | -1 | 3 | 3 | -3 | 3 | 3 | -1 | -1 |
| 29 | 3 | -3 | -3 | -1 | -1 | -3 | -1 | 3 | -3 | 3 | 1 | -1 |

Table 5.5.1.2-2: Definition of  for

| 0 | -1 | 3 | 1 | -3 | 3 | -1 | 1 | 3 | -3 | 3 | 1 | 3 | -3 | 3 | 1 | 1 | -1 | 1 | 3 | -3 | 3 | -3 | -1 | -3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | -3 | 3 | -3 | -3 | -3 | 1 | -3 | -3 | 3 | -1 | 1 | 1 | 1 | 3 | 1 | -1 | 3 | -3 | -3 | 1 | 3 | 1 | 1 | -3 |
| 2 | 3 | -1 | 3 | 3 | 1 | 1 | -3 | 3 | 3 | 3 | 3 | 1 | -1 | 3 | -1 | 1 | 1 | -1 | -3 | -1 | -1 | 1 | 3 | 3 |
| 3 | -1 | -3 | 1 | 1 | 3 | -3 | 1 | 1 | -3 | -1 | -1 | 1 | 3 | 1 | 3 | 1 | -1 | 3 | 1 | 1 | -3 | -1 | -3 | -1 |
| 4 | -1 | -1 | -1 | -3 | -3 | -1 | 1 | 1 | 3 | 3 | -1 | 3 | -1 | 1 | -1 | -3 | 1 | -1 | -3 | -3 | 1 | -3 | -1 | -1 |
| 5 | -3 | 1 | 1 | 3 | -1 | 1 | 3 | 1 | -3 | 1 | -3 | 1 | 1 | -1 | -1 | 3 | -1 | -3 | 3 | -3 | -3 | -3 | 1 | 1 |
| 6 | 1 | 1 | -1 | -1 | 3 | -3 | -3 | 3 | -3 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | -3 | -1 | 1 | -1 | 3 | -1 | -3 |
| 7 | -3 | 3 | 3 | -1 | -1 | -3 | -1 | 3 | 1 | 3 | 1 | 3 | 1 | 1 | -1 | 3 | 1 | -1 | 1 | 3 | -3 | -1 | -1 | 1 |
| 8 | -3 | 1 | 3 | -3 | 1 | -1 | -3 | 3 | -3 | 3 | -1 | -1 | -1 | -1 | 1 | -3 | -3 | -3 | 1 | -3 | -3 | -3 | 1 | -3 |
| 9 | 1 | 1 | -3 | 3 | 3 | -1 | -3 | -1 | 3 | -3 | 3 | 3 | 3 | -1 | 1 | 1 | -3 | 1 | -1 | 1 | 1 | -3 | 1 | 1 |
| 10 | -1 | 1 | -3 | -3 | 3 | -1 | 3 | -1 | -1 | -3 | -3 | -3 | -1 | -3 | -3 | 1 | -1 | 1 | 3 | 3 | -1 | 1 | -1 | 3 |
| 11 | 1 | 3 | 3 | -3 | -3 | 1 | 3 | 1 | -1 | -3 | -3 | -3 | 3 | 3 | -3 | 3 | 3 | -1 | -3 | 3 | -1 | 1 | -3 | 1 |
| 12 | 1 | 3 | 3 | 1 | 1 | 1 | -1 | -1 | 1 | -3 | 3 | -1 | 1 | 1 | -3 | 3 | 3 | -1 | -3 | 3 | -3 | -1 | -3 | -1 |
| 13 | 3 | -1 | -1 | -1 | -1 | -3 | -1 | 3 | 3 | 1 | -1 | 1 | 3 | 3 | 3 | -1 | 1 | 1 | -3 | 1 | 3 | -1 | -3 | 3 |
| 14 | -3 | -3 | 3 | 1 | 3 | 1 | -3 | 3 | 1 | 3 | 1 | 1 | 3 | 3 | -1 | -1 | -3 | 1 | -3 | -1 | 3 | 1 | 1 | 3 |
| 15 | -1 | -1 | 1 | -3 | 1 | 3 | -3 | 1 | -1 | -3 | -1 | 3 | 1 | 3 | 1 | -1 | -3 | -3 | -1 | -1 | -3 | -3 | -3 | -1 |
| 16 | -1 | -3 | 3 | -1 | -1 | -1 | -1 | 1 | 1 | -3 | 3 | 1 | 3 | 3 | 1 | -1 | 1 | -3 | 1 | -3 | 1 | 1 | -3 | -1 |
| 17 | 1 | 3 | -1 | 3 | 3 | -1 | -3 | 1 | -1 | -3 | 3 | 3 | 3 | -1 | 1 | 1 | 3 | -1 | -3 | -1 | 3 | -1 | -1 | -1 |
| 18 | 1 | 1 | 1 | 1 | 1 | -1 | 3 | -1 | -3 | 1 | 1 | 3 | -3 | 1 | -3 | -1 | 1 | 1 | -3 | -3 | 3 | 1 | 1 | -3 |
| 19 | 1 | 3 | 3 | 1 | -1 | -3 | 3 | -1 | 3 | 3 | 3 | -3 | 1 | -1 | 1 | -1 | -3 | -1 | 1 | 3 | -1 | 3 | -3 | -3 |
| 20 | -1 | -3 | 3 | -3 | -3 | -3 | -1 | -1 | -3 | -1 | -3 | 3 | 1 | 3 | -3 | -1 | 3 | -1 | 1 | -1 | 3 | -3 | 1 | -1 |
| 21 | -3 | -3 | 1 | 1 | -1 | 1 | -1 | 1 | -1 | 3 | 1 | -3 | -1 | 1 | -1 | 1 | -1 | -1 | 3 | 3 | -3 | -1 | 1 | -3 |
| 22 | -3 | -1 | -3 | 3 | 1 | -1 | -3 | -1 | -3 | -3 | 3 | -3 | 3 | -3 | -1 | 1 | 3 | 1 | -3 | 1 | 3 | 3 | -1 | -3 |
| 23 | -1 | -1 | -1 | -1 | 3 | 3 | 3 | 1 | 3 | 3 | -3 | 1 | 3 | -1 | 3 | -1 | 3 | 3 | -3 | 3 | 1 | -1 | 3 | 3 |
| 24 | 1 | -1 | 3 | 3 | -1 | -3 | 3 | -3 | -1 | -1 | 3 | -1 | 3 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -3 | -1 | 3 |
| 25 | 1 | -1 | 1 | -1 | 3 | -1 | 3 | 1 | 1 | -1 | -1 | -3 | 1 | 1 | -3 | 1 | 3 | -3 | 1 | 1 | -3 | -3 | -1 | -1 |
| 26 | -3 | -1 | 1 | 3 | 1 | 1 | -3 | -1 | -1 | -3 | 3 | -3 | 3 | 1 | -3 | 3 | -3 | 1 | -1 | 1 | -3 | 1 | 1 | 1 |
| 27 | -1 | -3 | 3 | 3 | 1 | 1 | 3 | -1 | -3 | -1 | -1 | -1 | 3 | 1 | -3 | -3 | -1 | 3 | -3 | -1 | -3 | -1 | -3 | -1 |
| 28 | -1 | -3 | -1 | -1 | 1 | -3 | -1 | -1 | 1 | -1 | -3 | 1 | 1 | -3 | 1 | -3 | -3 | 3 | 1 | 1 | -1 | 3 | -1 | -1 |
| 29 | 1 | 1 | -1 | -1 | -3 | -1 | 3 | -1 | 3 | -1 | 1 | 3 | 1 | -1 | 3 | 1 | 3 | -3 | -3 | 1 | -1 | -1 | 1 | 3 |

Table 5.5.1.2-3: Definition of  for

| 0 | -1 | -3 | 3 | -3 | 3 | -3 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | -1 | 3 | -1 | 1 | 1 | 1 |
| 2 | 3 | -1 | -3 | -3 | 1 | 3 |
| 3 | 3 | -1 | -1 | 1 | -1 | -1 |
| 4 | -1 | -1 | -3 | 1 | -3 | -1 |
| 5 | 1 | 3 | -3 | -1 | -3 | 3 |
| 6 | -3 | 3 | -1 | -1 | 1 | -3 |
| 7 | -1 | -3 | -3 | 1 | 3 | 3 |
| 8 | 3 | -1 | -1 | 3 | 1 | 3 |
| 9 | 3 | -3 | 3 | 1 | -1 | 1 |
| 10 | -3 | 1 | -3 | -3 | -3 | -3 |
| 11 | -3 | -3 | -3 | 1 | -3 | -3 |
| 12 | 3 | -3 | 1 | -1 | -3 | -3 |
| 13 | 3 | -3 | 3 | -1 | -1 | -3 |
| 14 | 3 | -1 | 1 | 3 | 3 | 1 |
| 15 | -1 | 1 | -1 | -3 | 1 | 1 |
| 16 | -3 | -1 | -3 | -1 | 3 | 3 |
| 17 | 1 | -1 | 3 | -3 | 3 | 3 |
| 18 | 1 | 3 | 1 | 1 | -3 | 3 |
| 19 | -1 | -3 | -1 | -1 | 3 | -3 |
| 20 | 3 | -1 | -3 | -1 | -1 | -3 |
| 21 | 3 | 1 | 3 | -3 | -3 | 1 |
| 22 | 1 | 3 | -1 | -1 | 1 | -1 |
| 23 | -3 | 1 | -3 | 3 | 3 | 3 |
| 24 | 1 | 3 | -3 | 3 | -3 | 3 |
| 25 | -1 | -1 | 1 | -3 | 1 | -1 |
| 26 | 1 | -3 | -1 | -1 | 3 | 1 |
| 27 | -3 | -1 | -1 | 3 | 1 | 1 |
| 28 | -1 | 3 | -3 | -3 | -3 | 3 |
| 29 | 3 | 1 | -1 | 1 | 3 | 1 |

Table 5.5.1.2-4: Definition of  for

| 0 | -3 | -3 | -3 | -3 | -3 | -1 | 1 | -1 | -3 | 3 | -1 | 3 | -1 | 3 | -3 | -1 | -1 | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | -3 | -3 | -3 | -3 | -3 | -1 | 1 | -1 | 1 | -3 | -3 | -3 | 1 | -1 | 3 | -3 | -3 | 1 |
| 2 | -3 | -3 | -3 | -3 | -3 | -1 | 1 | 1 | 3 | -3 | 1 | 1 | -3 | 1 | -3 | 3 | 1 | -1 |
| 3 | -3 | -3 | -3 | -3 | -3 | -1 | 1 | 3 | -3 | -1 | 3 | -1 | 3 | 1 | -1 | -3 | 3 | -3 |
| 4 | -3 | -3 | -3 | -3 | -3 | -1 | 3 | -3 | -1 | 1 | -1 | -3 | 3 | 3 | 1 | -3 | 1 | -1 |
| 5 | -3 | -3 | -3 | -3 | -3 | 1 | -3 | -3 | -3 | -3 | 1 | 1 | 1 | -3 | 1 | 1 | -3 | -3 |
| 6 | -3 | -3 | -3 | -3 | -3 | 1 | -3 | -3 | 1 | 1 | -3 | -3 | -3 | 1 | -1 | 3 | -1 | 3 |
| 7 | -3 | -3 | -3 | -3 | -3 | 1 | -3 | -1 | 3 | -1 | 3 | 3 | -1 | -1 | 1 | 3 | 3 | -1 |
| 8 | -3 | -3 | -3 | -3 | -3 | 1 | -1 | -1 | -1 | -3 | 3 | -1 | 3 | -3 | 3 | -1 | 1 | 3 |
| 9 | -3 | -3 | -3 | -3 | -3 | 3 | -3 | 1 | -1 | 3 | -3 | 3 | 3 | -1 | -3 | 1 | 1 | -3 |
| 10 | -3 | -3 | -3 | -3 | -3 | 3 | -1 | -3 | -3 | 1 | 1 | 3 | -3 | -1 | 3 | -1 | 3 | 1 |
| 11 | -3 | -3 | -3 | -3 | -3 | 3 | 3 | -1 | -1 | -1 | 3 | 1 | -3 | 3 | -1 | 1 | -3 | 1 |
| 12 | -3 | -3 | -3 | -3 | -1 | -3 | -3 | -3 | 1 | 3 | 1 | -1 | 3 | -3 | -1 | -3 | 1 | 1 |
| 13 | -3 | -3 | -3 | -3 | -1 | -3 | -3 | 1 | -1 | -1 | 3 | -3 | -3 | 1 | 3 | 1 | -3 | 1 |
| 14 | -3 | -3 | -3 | -3 | -1 | -3 | -3 | 1 | 3 | -3 | -1 | 3 | 1 | 3 | -1 | 3 | -1 | -3 |
| 15 | -3 | -3 | -3 | -3 | -1 | -3 | -1 | 3 | -3 | 1 | -3 | 1 | -1 | -3 | -3 | 1 | 1 | 3 |
| 16 | -3 | -3 | -3 | -3 | -1 | -1 | 3 | -3 | 3 | -1 | -3 | 1 | 1 | -1 | -3 | -1 | 3 | -3 |
| 17 | -3 | -3 | -3 | -3 | -1 | -1 | 3 | -1 | -3 | 1 | 3 | -1 | -3 | -3 | 1 | 3 | -1 | 1 |
| 18 | -3 | -3 | -3 | -3 | -1 | 3 | -1 | -1 | 3 | 3 | -1 | -3 | 1 | 1 | 1 | -1 | -3 | -1 |
| 19 | -3 | -3 | -3 | -3 | -1 | 3 | 1 | -3 | -1 | -3 | 3 | 1 | -1 | 3 | -1 | 1 | 3 | -1 |
| 20 | -3 | -3 | -3 | -3 | 1 | -3 | -3 | 3 | 1 | 1 | -3 | -1 | 1 | 3 | 3 | -1 | 3 | -1 |
| 21 | -3 | -3 | -3 | -3 | 1 | -3 | 1 | 3 | 1 | -1 | -1 | 3 | 3 | -1 | 1 | 1 | -3 | 3 |
| 22 | -3 | -3 | -3 | -3 | 1 | -3 | 3 | -3 | -1 | 3 | 1 | 1 | -1 | -1 | 3 | 3 | -1 | 3 |
| 23 | -3 | -3 | -3 | -3 | 1 | -3 | 3 | -1 | 3 | -3 | -1 | -1 | -1 | 1 | -3 | -3 | 3 | 1 |
| 24 | -3 | -3 | -3 | -3 | 1 | 1 | 3 | 1 | 1 | -1 | 3 | 1 | 1 | 3 | -1 | -3 | 1 | 3 |
| 25 | -3 | -3 | -3 | -3 | 1 | 3 | 3 | 3 | 1 | -3 | 1 | -3 | -3 | 3 | -3 | 1 | -1 | -3 |
| 26 | -3 | -3 | -3 | -3 | 3 | 1 | 3 | 3 | -1 | 3 | -3 | -3 | -1 | 3 | -1 | -1 | -3 | 1 |
| 27 | -3 | -3 | -3 | -1 | -3 | -3 | -1 | -1 | -3 | 3 | 3 | 1 | -3 | -1 | -1 | 3 | 1 | -3 |
| 28 | -3 | -3 | -3 | -1 | -3 | 1 | -1 | 1 | -3 | 3 | 1 | -3 | -1 | 1 | 3 | 1 | -1 | -1 |
| 29 | -3 | -3 | -3 | -1 | -3 | 3 | 1 | 1 | -1 | -1 | 1 | 3 | 1 | -3 | 1 | -3 | -1 | 1 |

#### 5.5.1.3	Group hopping

The sequence-group number  in slot  is defined by a group hopping pattern  and a sequence-shift pattern  according to

There are 17 different hopping patterns and 30 different sequence-shift patterns. Sequence-group hopping can be enabled or disabled by means of the cell-specific parameter Group-hopping-enabled provided by higher layers. Sequence-group hopping for PUSCH can be disabled for a certain UE through the higher-layer parameter Disable-sequence-group-hopping despite being enabled on a cell basis unless the PUSCH transmission corresponds to a Random Access Response Grant or a retransmission of the same transport block as part of the contention based random access procedure.

The group-hopping pattern  may be different for PUSCH, PUCCH and SRS and is given by

where the pseudo-random sequence  is defined by clause 7.2. The pseudo-random sequence generator shall be initialized with  at the beginning of each radio frame where  is given by clause 5.5.1.5.

The sequence-shift pattern  definition differs between PUCCH, PUSCH and SRS.

For PUCCH, the sequence-shift pattern  is given by  where  is given by clause 5.5.1.5.

For PUSCH, the sequence-shift pattern  is given by , where  is configured by higher layers, if no value for  is provided by higher layers or if the PUSCH transmission corresponds to a Random Access Response Grant or a retransmission of the same transport block as part of the contention based random access procedure, otherwise it is given by  with  given by clause 5.5.1.5.

For SRS, the sequence-shift pattern  is given by where  is given by clause 5.5.1.5.

#### 5.5.1.4	Sequence hopping

Sequence hopping only applies for reference-signals of length .

For reference-signals of length , the base sequence number  within the base sequence group is given by .

For reference-signals of length , the base sequence number  within the base sequence group in slot  is defined by

where the pseudo-random sequence  is given by clause 7.2. The parameter Sequence-hopping-enabled provided by higher layers determines if sequence hopping is enabled or not. Sequence hopping for PUSCH can be disabled for a certain UE through the higher-layer parameter Disable-sequence-group-hopping despite being enabled on a cell basis unless the PUSCH transmission corresponds to a Random Access Response Grant or a retransmission of the same transport block as part of the contention based random access procedure.

For PUSCH or PUCCH format 4 transmission with ≥ 6 RBs, the pseudo-random sequence generator shall be initialized with  at the beginning of each radio frame where  is given by clause 5.5.1.5.

For SRS, the pseudo-random sequence generator shall be initialized with  at the beginning of each radio frame where  is given by clause 5.5.1.5 and  is given by clause 5.5.1.3.

#### 5.5.1.5	Determining virtual cell identity for sequence generation

The definition of  depends on the type of transmission.

Transmissions associated with PUSCH:

-	 if no value for  is configured by higher layers or if the PUSCH transmission corresponds to a Random Access Response Grant or a retransmission of the same transport block as part of the contention based random access procedure,

-	 otherwise.

Transmissions associated with PUCCH:

-	 if no value for  is configured by higher layers,

-	 otherwise.

Sounding reference signals:

-	.

### 5.5.2	Demodulation reference signal

#### 5.5.2.1	Demodulation reference signal for PUSCH

##### 5.5.2.1.1	Reference signal sequence

The PUSCH demodulation reference signal sequence  associated with layer  is defined by

where

and  if the higher-layer parameter ul-DMRS-IFDMA is set and the most recent uplink-related DCI contains the Cyclic Shift Field mapping table for DMRS bit field which indicates the use of Table 5.5.2.1.1-3 and  otherwise.

Subclause 5.5.1 defines the sequence  where, for PUSCH demodulation reference signal sequence,  when the higher-layer parameter ul-DMRS-IFDMA is set and the most recent uplink-related DCI contains the Cyclic Shift Field mapping table for DMRS bit field which indicates the use of Table 5.5.2.1.1-3, and  otherwise. The orthogonal sequence  is given by  for DCI format 0 if the higher-layer parameter Activate-DMRS-with OCC is not set or if the temporary C-RNTI was used to transmit the most recent uplink-related DCI for the transport block associated with the corresponding PUSCH transmission. Otherwise,

-	if higher-layer parameter ul-DMRS-IFDMA is not set,  is given by Table 5.5.2.1.1-1 using the cyclic shift field in the most recent uplink-related DCI [3],

-	if higher-layer parameter ul-DMRS-IFDMA is set and the Cyclic Shift Field mapping table for DMRS bit field is not present in the most recent uplink-related DCI,  is given by Table 5.5.2.1.1-1 using the cyclic shift field in the most recent uplink-related DCI,

-	if higher-layer parameter ul-DMRS-IFDMA is set and the Cyclic Shift Field mapping table for DMRS bit field is present in the most recent uplink-related DCI,  is given by Table 5.5.2.1.1-1 using the cyclic shift field in the most recent uplink-related DCI when the Cyclic Shift Field mapping table for DMRS bit field indicates the use of Table 5.5.2.1.1-1, and

-	if higher-layer parameter ul-DMRS-IFDMA is set and the Cyclic Shift Field mapping table for DMRS bit field is present in the most recent uplink-related DCI,  is given by Table 5.5.2.1.1-3 using the cyclic shift field in the most recent uplink-related DCI when the Cyclic Shift Field mapping table for DMRS bit field indicates the use of Table 5.5.2.1.1-3.

The cyclic shift  in a slot  is given as if the ul-V-SPS-RNTI-r14 was used to transmit the most recent uplink-related DCI for the transport block associated with the corresponding PUSCH transmission. Otherwise, the cyclic shift  in a slot  is given as  with

where the value of  is given by Table 5.5.2.1.1-2 according to the parameter cyclicShift provided by higher layers. For non-BL/CE UEs  is given using the most recent uplink-related DCI 3GPP TS 36.212 [3] for the transport block associated with the corresponding PUSCH transmission as follows:

-	if the higher-layer parameter ul-DMRS-IFDMA is not set,  is given by Table 5.5.2.1.1-1 using the cyclic shift field in the most recent uplink-related DCI,

-	if higher-layer parameter ul-DMRS-IFDMA is set and the Cyclic Shift Field mapping table for DMRS bit field is not present in the most recent uplink-related DCI,  is given by Table 5.5.2.1.1-1 using the cyclic shift field in the most recent uplink-related DCI,

-	if higher-layer parameter ul-DMRS-IFDMA is set and the Cyclic Shift Field mapping table for DMRS bit field is present in the most recent uplink-related DCI,  is given by Table 5.5.2.1.1-1 using the cyclic shift field in the most recent uplink-related DCI when the Cyclic Shift Field mapping table for DMRS bit field indicates the use of Table 5.5.2.1.1-1, and

-	if higher-layer parameter ul-DMRS-IFDMA is set and the Cyclic Shift Field mapping table for DMRS bit field is present in the most recent uplink-related DCI,  is given by Table 5.5.2.1.1-3 using the cyclic shift field in the most recent uplink-related DCI when the Cyclic Shift Field mapping table for DMRS bit field indicates the use of Table 5.5.2.1.1-3.

For BL/CE UEs, a cyclic shift field of '000' shall be assumed when determining  from Table 5.5.2.1.1-1.

The first row of Table 5.5.2.1.1-1 shall be used to obtain  and  if there is no uplink-related DCI for the same transport block associated with the corresponding PUSCH transmission, and

-	if the initial PUSCH for the same transport block is semi-persistently scheduled, or

-	if the initial PUSCH for the same transport block is scheduled by the random access response grant.

The quantity  is given by

where the pseudo-random sequence  is defined by clause 7.2. The application of  is cell-specific. The pseudo-random sequence generator shall be initialized with  at the beginning of each radio frame. The quantity  is given by  if no value for  is configured by higher layers for PUSCH/PUCCH format 4/PUCCH format 5 or the PUSCH transmission corresponds to a Random Access Response Grant or a retransmission of the same transport block as part of the contention based random access procedure, otherwise it is given by .

The vector of reference signals shall be precoded according to

where  is the number of antenna ports used for PUSCH transmission.

For PUSCH transmission using a single antenna port, ,  and .

For spatial multiplexing,  or  and the precoding matrix  shall be identical to the precoding matrix used in clause 5.3.3A.2 for precoding of the PUSCH in the same subframe.

Table 5.5.2.1.1-1: Mapping of Cyclic Shift Field in uplink-related DCI format to  and

| Cyclic Shift Field in uplink-related DCI format [3] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 000 | 0 | 6 | 3 | 9 |  |  |  |  |
| 001 | 6 | 0 | 9 | 3 |  |  |  |  |
| 010 | 3 | 9 | 6 | 0 |  |  |  |  |
| 011 | 4 | 10 | 7 | 1 |  |  |  |  |
| 100 | 2 | 8 | 5 | 11 |  |  |  |  |
| 101 | 8 | 2 | 11 | 5 |  |  |  |  |
| 110 | 10 | 4 | 1 | 7 |  |  |  |  |
| 111 | 9 | 3 | 0 | 6 |  |  |  |  |

Table 5.5.2.1.1-2: Mapping of cyclicShift to values

| cyclicShift |  |
| --- | --- |
| 0 | 0 |
| 1 | 2 |
| 2 | 3 |
| 3 | 4 |
| 4 | 6 |
| 5 | 8 |
| 6 | 9 |
| 7 | 10 |

Table 5.5.2.1.1-3: Mapping of Cyclic Shift Field in uplink-related DCI format to , , and

| Cyclic Shift Field in uplink-related DCI format [3] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 000 | 1 | 0 | 6 | 3 | 9 |  |  |  |  |
| 001 | 1 | 6 | 0 | 9 | 3 |  |  |  |  |
| 010 | 1 | 3 | 9 | 6 | 0 |  |  |  |  |
| 011 | 0 | 4 | 10 | 7 | 1 |  |  |  |  |
| 100 | 0 | 2 | 8 | 5 | 11 |  |  |  |  |
| 101 | 0 | 8 | 2 | 11 | 5 |  |  |  |  |
| 110 | 0 | 10 | 4 | 1 | 7 |  |  |  |  |
| 111 | 1 | 9 | 3 | 0 | 6 |  |  |  |  |

##### 5.5.2.1.2	Mapping to physical resources

For each antenna port used for transmission of the PUSCH, the sequence  shall be multiplied with the amplitude scaling factor  and mapped in sequence starting with  to the resource blocks.  when the higher-layer parameter ul-DMRS-IFDMA is set and the most recent uplink-related DCI contains the Cyclic Shift Field mapping table for DMRS bit field which indicates the use of Table 5.5.2.1.1-3, and  otherwise.

If higher-layer parameter ul-DMRS-IFDMA is set and the most recent uplink-related DCI contains the Cyclic Shift Field mapping table for DMRS bit field which indicates the use of Table 5.5.2.1.1-3, the mapping to resource elements , with  for normal cyclic prefix and  for extended cyclic prefix, in the subframe shall be in increasing order of first  for all values of  satisfying , then the slot number. The quantity  is given by Table 5.5.2.1.1-3 using the cyclic shift field in the most recent uplink-related DCI.

For all other cases, the set of physical resource blocks used in the mapping process and the relation between the index  and the antenna port number  shall be identical to the corresponding PUSCH transmission as defined in clause 5.3.4.

The mapping to resource elements , with  for normal cyclic prefix and  for extended cyclic prefix, in the subframe shall be in increasing order of first, then the slot number. No DM-RS shall be transmitted in UpPTS if dmrsLess-UpPts is set to true.

#### 5.5.2.2	Demodulation reference signal for PUCCH

##### 5.5.2.2.1	Reference signal sequence

The PUCCH demodulation reference signal sequence  for PUCCH formats 1, 1a, 1b, 2, 2a, 2b, and 3 is defined by

where

and  is the number of antenna ports used for PUCCH transmission. For PUCCH formats 2a and 2b,  equals  for , where  is defined in clause 5.4.2. For all other cases,

The sequence is given by clause 5.5.1 with  and  where the expression for the cyclic shift  is determined by the PUCCH format.

For PUCCH formats 1, 1a and 1b,  is given by

where , ,  and  are defined by clause 5.4.1. The number of reference symbols per slot  and the sequence  are given by Table 5.5.2.2.1-1 and 5.5.2.2.1-2, respectively.

For PUCCH formats 2, 2a and 2b,  is defined by clause 5.4.2. The number of reference symbols per slot  and the sequence  are given by Table 5.5.2.2.1-1 and 5.5.2.2.1-3, respectively.

For PUCCH format 3,  is given by

where  is given by Table 5.5.2.2.1-4 and  and  for the first and second slot in a subframe, respectively, are obtained from clause 5.4.2A. The number of reference symbols per slot  and the sequence  are given by Table 5.5.2.2.1-1 and 5.5.2.2.1-3, respectively.

Table 5.5.2.2.1-1: Number of PUCCH demodulation reference symbols per slot

| PUCCH format | Normal cyclic prefix | Extended cyclic prefix |
| --- | --- | --- |
| 1, 1a, 1b | 3 | 2 |
| 2, 3 | 2 | 1 |
| 2a, 2b | 2 | N/A |

Table 5.5.2.2.1-2: Orthogonal sequences  for PUCCH formats 1, 1a and 1b

| Sequence index | Normal cyclic prefix | Extended cyclic prefix |
| --- | --- | --- |
| 0 |  |  |
| 1 |  |  |
| 2 |  | N/A |

Table 5.5.2.2.1-3: Orthogonal sequences  for PUCCH formats 2, 2a, 2b and 3.

| Normal cyclic prefix | Extended cyclic prefix |
| --- | --- |

Table 5.5.2.2.1-4: Relation between  and  for PUCCH format 3.

| 0 | 0 | 0 |
| --- | --- | --- |
| 1 | 3 | 3 |
| 2 | 6 | 6 |
| 3 | 8 | 9 |
| 4 | 10 | N/A |

The PUCCH demodulation reference signal sequence  for PUCCH formats 4 and 5 is defined by

where

and

Subclause 5.5.1 defines the sequence  where .

The cyclic shift  in a slot  is given as  with

where the values of and  are given by Subclause 5.5.2.1.1 and

with  obtained as described in clause 5.4.2C.

##### 5.5.2.2.2	Mapping to physical resources

The sequence  shall be multiplied with the amplitude scaling factor  and mapped in sequence starting with  to resource elements on antenna port . The mapping shall be in increasing order of first, then  and finally the slot number. The set of values for  and the relation between the index  and the antenna port number  shall be identical to the values used for the corresponding PUCCH transmission. The values of the symbol index  in a slot are given by Table 5.5.2.2.2-1.

Table 5.5.2.2.2-1: Demodulation reference signal location for different PUCCH formats.

| PUCCH format | Set of values for |  |
| --- | --- | --- |
|  | Normal cyclic prefix | Extended cyclic prefix |
| 1, 1a, 1b | 2, 3, 4 | 2, 3 |
| 2, 3 | 1, 5 | 3 |
| 2a, 2b | 1, 5 | N/A |
| 4,5 | 3 | 2 |

### 5.5.3	Sounding reference signal

#### 5.5.3.1	Sequence generation

The sounding reference signal sequence  is defined by clause 5.5.1, where  is the sequence-group number defined in clause 5.5.1.3,  is the base sequence number defined in clause 5.5.1.4, and . The cyclic shift  of the sounding reference signal is given as

,

where  is configured separately for periodic and each configuration of aperiodic sounding by the higher-layer parameters cyclicShift and cyclicShift-ap, respectively, for each UE and  is the number of antenna ports used for sounding reference signal transmission. The parameter  if  , otherwise . The parameter  is given by the higher layer parameter transmissionCombNum if configured, otherwise .

#### 5.5.3.2	Mapping to physical resources

The sequence shall be multiplied with the amplitude scaling factor  in order to conform to the transmit power  specified in clause 5.1.3.1 in 3GPP TS 36.213 [4], and mapped in sequence starting with  to resource elements  on antenna port  according to

where  is the number of antenna ports used for sounding reference signal transmission and the relation between the index  and the antenna port  is given by Table 5.2.1-1. The set of antenna ports used for sounding reference signal transmission is configured independently for periodic and each configuration of aperiodic sounding. The quantity  is the frequency-domain starting position of the sounding reference signal and for  and  is the length of the sounding reference signal sequence defined as

where is given by Table 5.5.3.2-1 through Table 5.5.3.2-4 for each uplink bandwidth . The cell-specific parameter srs-BandwidthConfig,  and the UE-specific parameter srs-Bandwidth, are given by higher layers. For UpPTS,  shall be reconfigured to  if this reconfiguration is enabled by the cell-specific parameter srsMaxUpPts given by higher layers, otherwise if the reconfiguration is disabled ,where  is a SRS BW configuration and  is the set of SRS BW configurations from the Tables 5.5.3.2-1 to 5.5.3.2-4 for each uplink bandwidth ,  is the number of format 4 PRACH in the addressed UpPTS and derived from Table 5.7.1-4.

The frequency-domain starting position  is defined by

where for normal uplink subframes  is defined by

and for UpPTS by

The quantity  is given by

where the relation between the index  and the antenna port  is given by Table 5.2.1-1,  is given by the UE-specific parameter transmissionComb or transmissionComb-ap for periodic and each configuration of aperiodic transmission, respectively, provided by higher layers for the UE, and  is frequency position index. The variable  is equal to 0 for UpPTS in the first half frame and equal to 1 for UpPTS in the second half frame of a radio frame.

The frequency hopping of the sounding reference signal is configured by the parameter , provided by higher-layer parameter srs-HoppingBandwidth. Frequency hopping is not supported for aperiodic transmission. If frequency hopping of the sounding reference signal is not enabled (i.e., ), the frequency position index  remains constant (unless re-configured) and is defined by  where the parameter  is given by higher-layer parameters freqDomainPosition and freqDomainPosition-ap for periodic and each configuration of aperiodic transmission, respectively. If frequency hopping of the sounding reference signal is enabled (i.e., ), the frequency position indexes  are defined by

where  is given by Table 5.5.3.2-1 through Table 5.5.3.2-4 for each uplink bandwidth ,

where  regardless of the  value on Table 5.5.3.2-1 through Table 5.5.3.2-4, and

counts the number of UE-specific SRS transmissions, where  is UE-specific periodicity of SRS transmission defined in clause 8.2 of 3GPP TS 36.213 [4],  is SRS subframe offset defined in Table 8.2-2 of 3GPP TS 36.213 [4] and  is the maximum value of  for a certain configuration of SRS subframe offset.

The sounding reference signal shall be transmitted in the last symbol of the uplink subframe.

Table 5.5.3.2-1:  and , , values for the uplink bandwidth of

| SRS bandwidth configuration | SRS-Bandwidth |  | SRS-Bandwidth |  | SRS-Bandwidth |  | SRS-Bandwidth |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 36 | 1 | 12 | 3 | 4 | 3 | 4 | 1 |
| 1 | 32 | 1 | 16 | 2 | 8 | 2 | 4 | 2 |
| 2 | 24 | 1 | 4 | 6 | 4 | 1 | 4 | 1 |
| 3 | 20 | 1 | 4 | 5 | 4 | 1 | 4 | 1 |
| 4 | 16 | 1 | 4 | 4 | 4 | 1 | 4 | 1 |
| 5 | 12 | 1 | 4 | 3 | 4 | 1 | 4 | 1 |
| 6 | 8 | 1 | 4 | 2 | 4 | 1 | 4 | 1 |
| 7 | 4 | 1 | 4 | 1 | 4 | 1 | 4 | 1 |

Table 5.5.3.2-2:  and , , values for the uplink bandwidth of

| SRS bandwidth configuration | SRS-Bandwidth |  | SRS-Bandwidth |  | SRS-Bandwidth |  | SRS-Bandwidth |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 48 | 1 | 24 | 2 | 12 | 2 | 4 | 3 |
| 1 | 48 | 1 | 16 | 3 | 8 | 2 | 4 | 2 |
| 2 | 40 | 1 | 20 | 2 | 4 | 5 | 4 | 1 |
| 3 | 36 | 1 | 12 | 3 | 4 | 3 | 4 | 1 |
| 4 | 32 | 1 | 16 | 2 | 8 | 2 | 4 | 2 |
| 5 | 24 | 1 | 4 | 6 | 4 | 1 | 4 | 1 |
| 6 | 20 | 1 | 4 | 5 | 4 | 1 | 4 | 1 |
| 7 | 16 | 1 | 4 | 4 | 4 | 1 | 4 | 1 |

Table 5.5.3.2-3:  and , , values for the uplink bandwidth of

| SRS bandwidth configuration | SRS-Bandwidth |  | SRS-Bandwidth |  | SRS-Bandwidth |  | SRS-Bandwidth |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 72 | 1 | 24 | 3 | 12 | 2 | 4 | 3 |
| 1 | 64 | 1 | 32 | 2 | 16 | 2 | 4 | 4 |
| 2 | 60 | 1 | 20 | 3 | 4 | 5 | 4 | 1 |
| 3 | 48 | 1 | 24 | 2 | 12 | 2 | 4 | 3 |
| 4 | 48 | 1 | 16 | 3 | 8 | 2 | 4 | 2 |
| 5 | 40 | 1 | 20 | 2 | 4 | 5 | 4 | 1 |
| 6 | 36 | 1 | 12 | 3 | 4 | 3 | 4 | 1 |
| 7 | 32 | 1 | 16 | 2 | 8 | 2 | 4 | 2 |

Table 5.5.3.2-4: and, , values for the uplink bandwidth of

| SRS bandwidth configuration | SRS-Bandwidth |  | SRS-Bandwidth |  | SRS-Bandwidth |  | SRS-Bandwidth |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 96 | 1 | 48 | 2 | 24 | 2 | 4 | 6 |
| 1 | 96 | 1 | 32 | 3 | 16 | 2 | 4 | 4 |
| 2 | 80 | 1 | 40 | 2 | 20 | 2 | 4 | 5 |
| 3 | 72 | 1 | 24 | 3 | 12 | 2 | 4 | 3 |
| 4 | 64 | 1 | 32 | 2 | 16 | 2 | 4 | 4 |
| 5 | 60 | 1 | 20 | 3 | 4 | 5 | 4 | 1 |
| 6 | 48 | 1 | 24 | 2 | 12 | 2 | 4 | 3 |
| 7 | 48 | 1 | 16 | 3 | 8 | 2 | 4 | 2 |

#### 5.5.3.3	Sounding reference signal subframe configuration

The cell-specific subframe configuration period  and the cell-specific subframe offset  for the transmission of sounding reference signals are listed in Tables 5.5.3.3-1 and 5.5.3.3-2, for frame structures type 1 and 2 respectively, where the parameter srs-SubframeConfig is provided by higher layers. Sounding reference signal subframes are the subframes satisfying. For frame structure type 2, a sounding reference signal is transmitted only in uplink subframes or UpPTS.

Table 5.5.3.3-1: Frame structure type 1 sounding reference signal subframe configuration

| srs-SubframeConfig | Binary | Configuration Period (subframes) | Transmission offset (subframes) |
| --- | --- | --- | --- |
| 0 | 0000 | 1 | {0} |
| 1 | 0001 | 2 | {0} |
| 2 | 0010 | 2 | {1} |
| 3 | 0011 | 5 | {0} |
| 4 | 0100 | 5 | {1} |
| 5 | 0101 | 5 | {2} |
| 6 | 0110 | 5 | {3} |
| 7 | 0111 | 5 | {0,1} |
| 8 | 1000 | 5 | {2,3} |
| 9 | 1001 | 10 | {0} |
| 10 | 1010 | 10 | {1} |
| 11 | 1011 | 10 | {2} |
| 12 | 1100 | 10 | {3} |
| 13 | 1101 | 10 | {0,1,2,3,4,6,8} |
| 14 | 1110 | 10 | {0,1,2,3,4,5,6,8} |
| 15 | 1111 | reserved | reserved |

Table 5.5.3.3-2: Frame structure type 2 sounding reference signal subframe configuration

| srs-SubframeConfig | Binary | Configuration Period (subframes) | Transmission offset (subframes) |
| --- | --- | --- | --- |
| 0 | 0000 | 5 | {1} |
| 1 | 0001 | 5 | {1, 2} |
| 2 | 0010 | 5 | {1, 3} |
| 3 | 0011 | 5 | {1, 4} |
| 4 | 0100 | 5 | {1, 2, 3} |
| 5 | 0101 | 5 | {1, 2, 4} |
| 6 | 0110 | 5 | {1, 3, 4} |
| 7 | 0111 | 5 | {1, 2, 3, 4} |
| 8 | 1000 | 10 | {1, 2, 6} |
| 9 | 1001 | 10 | {1, 3, 6} |
| 10 | 1010 | 10 | {1, 6, 7} |
| 11 | 1011 | 10 | {1, 2, 6, 8} |
| 12 | 1100 | 10 | {1, 3, 6, 9} |
| 13 | 1101 | 10 | {1, 4, 6, 7} |
| 14 | 1110 | reserved | reserved |
| 15 | 1111 | reserved | reserved |

## 5.6	SC-FDMA baseband signal generation

This clause applies to all uplink physical signals and uplink physical channels except the physical random access channel.

The time-continuous signal  for antenna port  in SC-FDMA symbol  in an uplink slot is defined by

for where , ,  and  is the content of resource element  on antenna port .

For frame structure type 3, if the associated DCI indicates PUSCH starting position other than '00',  is given by

where

and  is given by clause 8.1. The UE behaviour if  is undefined.

The SC-FDMA symbols in a slot shall be transmitted in increasing order of , starting with , where SC-FDMA symbol starts at time  within the slot.

Table 5.6-1 lists the values of that shall be used.

Table 5.6-1: SC-FDMA parameters

| Configuration | Cyclic prefix length |
| --- | --- |
| Normal cyclic prefix |  |
| Extended cyclic prefix |  |

## 5.7	Physical random access channel

### 5.7.1	Time and frequency structure

The physical layer random access preamble, illustrated in Figure 5.7.1-1, consists of a cyclic prefix of length and a sequence part of length. The parameter values are listed in Table 5.7.1-1 and depend on the frame structure and the random access configuration. Higher layers control the preamble format.

Figure 5.7.1-1: Random access preamble format

Table 5.7.1-1: Random access preamble parameters

| Preamble format |  |  |
| --- | --- | --- |
| 0 |  |  |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 (see Note) |  |  |
| NOTE: Frame structure type 2 and special subframe configurations with UpPTS lengths and only assuming that the number of additional SC-FDMA symbols in UpPTS X in Table 4.2-1 is 0. |  |  |

The transmission of a random access preamble, if triggered by the MAC layer, is restricted to certain time and frequency resources. These resources are enumerated in increasing order of the subframe number within the radio frame and the physical resource blocks in the frequency domain such that index 0 correspond to the lowest numbered physical resource block and subframe within the radio frame. PRACH resources within the radio frame are indicated by a PRACH configuration index, where the indexing is in the order of appearance in Table 5.7.1-2 and Table 5.7.1-4.

For non-BL/CE UEs there are up to two PRACH configurations in a cell. The first PRACH configuration is configured by higher layers with a PRACH configuration index (prach-ConfigurationIndex) and a PRACH frequency offset  (prach-FrequencyOffset). The second PRACH configuration (if any) is configured by higher layers with a PRACH configuration index (prach-ConfigurationIndexHighSpeed) and a PRACH frequency offset  (prach-FrequencyOffsetHighSpeed).

For BL/CE UEs, for each PRACH coverage enhancement level, there is a PRACH configuration configured by higher layers with a PRACH configuration index (prach-ConfigurationIndex), a PRACH frequency offset  (prach-FrequencyOffset), a number of PRACH repetitions per attempt  (numRepetitionPerPreambleAttempt) and optionally a PRACH starting subframe periodicity  (prach-StartingSubframe). PRACH of preamble format 0-3 is transmitted  times, whereas PRACH of preamble format 4 is transmitted one time only.

For BL/CE UEs and for each PRACH coverage enhancement level, if frequency hopping is enabled for a PRACH configuration by the higher-layer parameter prach-HoppingConfig, the value of the parameter  depends on the SFN and the PRACH configuration index and is given by

-	In case the PRACH configuration index is such that a PRACH resource occurs in every radio frame when calculated as below from Table 5.7.1-2 or Table 5.7.1-4,

-	otherwise

where  is the system frame number corresponding to the first subframe for each PRACH repetition,  corresponds to a cell-specific higher-layer parameter prach-HoppingOffset. If frequency hopping is not enabled for the PRACH configuration then .

For frame structure type 1 with preamble format 0-3, for each of the PRACH configurations there is at most one random access resource per subframe. 
Table 5.7.1-2 lists the preamble formats according to Table 5.7.1-1 and the subframes in which random access preamble transmission is allowed for a given configuration in frame structure type 1. The start of the random access preamble shall be aligned with the start of the corresponding uplink subframe at the UE assuming , where  is defined in clause 8.1. For PRACH configurations 0, 1, 2, 15, 16, 17, 18, 31, 32, 33, 34, 47, 48, 49, 50 and 63 the UE may for handover purposes assume an absolute value of the relative time difference between radio frame  in the current cell and the target cell of less than . 
The first physical resource block  allocated to the PRACH opportunity considered for preamble formats 0, 1, 2 and 3 is defined as .

Table 5.7.1-2: Frame structure type 1 random access configuration for preamble formats 0-3

| PRACH Configuration Index | Preamble Format | System frame number | Subframe number | PRACH Configuration Index | Preamble Format | System frame number | Subframe number |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | Even | 1 | 32 | 2 | Even | 1 |
| 1 | 0 | Even | 4 | 33 | 2 | Even | 4 |
| 2 | 0 | Even | 7 | 34 | 2 | Even | 7 |
| 3 | 0 | Any | 1 | 35 | 2 | Any | 1 |
| 4 | 0 | Any | 4 | 36 | 2 | Any | 4 |
| 5 | 0 | Any | 7 | 37 | 2 | Any | 7 |
| 6 | 0 | Any | 1, 6 | 38 | 2 | Any | 1, 6 |
| 7 | 0 | Any | 2 ,7 | 39 | 2 | Any | 2 ,7 |
| 8 | 0 | Any | 3, 8 | 40 | 2 | Any | 3, 8 |
| 9 | 0 | Any | 1, 4, 7 | 41 | 2 | Any | 1, 4, 7 |
| 10 | 0 | Any | 2, 5, 8 | 42 | 2 | Any | 2, 5, 8 |
| 11 | 0 | Any | 3, 6, 9 | 43 | 2 | Any | 3, 6, 9 |
| 12 | 0 | Any | 0, 2, 4, 6, 8 | 44 | 2 | Any | 0, 2, 4, 6, 8 |
| 13 | 0 | Any | 1, 3, 5, 7, 9 | 45 | 2 | Any | 1, 3, 5, 7, 9 |
| 14 | 0 | Any | 0, 1, 2, 3, 4, 5, 6, 7, 8, 9 | 46 | N/A | N/A | N/A |
| 15 | 0 | Even | 9 | 47 | 2 | Even | 9 |
| 16 | 1 | Even | 1 | 48 | 3 | Even | 1 |
| 17 | 1 | Even | 4 | 49 | 3 | Even | 4 |
| 18 | 1 | Even | 7 | 50 | 3 | Even | 7 |
| 19 | 1 | Any | 1 | 51 | 3 | Any | 1 |
| 20 | 1 | Any | 4 | 52 | 3 | Any | 4 |
| 21 | 1 | Any | 7 | 53 | 3 | Any | 7 |
| 22 | 1 | Any | 1, 6 | 54 | 3 | Any | 1, 6 |
| 23 | 1 | Any | 2 ,7 | 55 | 3 | Any | 2 ,7 |
| 24 | 1 | Any | 3, 8 | 56 | 3 | Any | 3, 8 |
| 25 | 1 | Any | 1, 4, 7 | 57 | 3 | Any | 1, 4, 7 |
| 26 | 1 | Any | 2, 5, 8 | 58 | 3 | Any | 2, 5, 8 |
| 27 | 1 | Any | 3, 6, 9 | 59 | 3 | Any | 3, 6, 9 |
| 28 | 1 | Any | 0, 2, 4, 6, 8 | 60 | N/A | N/A | N/A |
| 29 | 1 | Any | 1, 3, 5, 7, 9 | 61 | N/A | N/A | N/A |
| 30 | N/A | N/A | N/A | 62 | N/A | N/A | N/A |
| 31 | 1 | Even | 9 | 63 | 3 | Even | 9 |

For frame structure type 2 with preamble formats 0-4, for each of the PRACH configurations there might be multiple random access resources in an UL subframe (or UpPTS for preamble format 4) depending on the UL/DL configuration [see table 4.2-2]. Table 5.7.1-3 lists PRACH configurations allowed for frame structure type 2 where the configuration index corresponds to a certain combination of preamble format, PRACH density value,  and version index, . 
For frame structure type 2 with PRACH configuration indices 0, 1, 2, 20, 21, 22, 30, 31, 32, 40, 41, 42, 48, 49, 50, or with PRACH configuration indices 51, 53, 54, 55, 56, 57 in UL/DL configuration 3, 4, 5, the UE may for handover purposes assume an absolute value of the relative time difference between radio frame in the current cell and the target cell is less than .

Table 5.7.1-3: Frame structure type 2 random access configurations for preamble formats 0-4

| PRACH configuration Index | Preamble Format | Density Per 10 ms | Version | PRACH configuration Index | Preamble Format | Density Per 10 ms | Version |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0.5 | 0 | 32 | 2 | 0.5 | 2 |
| 1 | 0 | 0.5 | 1 | 33 | 2 | 1 | 0 |
| 2 | 0 | 0.5 | 2 | 34 | 2 | 1 | 1 |
| 3 | 0 | 1 | 0 | 35 | 2 | 2 | 0 |
| 4 | 0 | 1 | 1 | 36 | 2 | 3 | 0 |
| 5 | 0 | 1 | 2 | 37 | 2 | 4 | 0 |
| 6 | 0 | 2 | 0 | 38 | 2 | 5 | 0 |
| 7 | 0 | 2 | 1 | 39 | 2 | 6 | 0 |
| 8 | 0 | 2 | 2 | 40 | 3 | 0.5 | 0 |
| 9 | 0 | 3 | 0 | 41 | 3 | 0.5 | 1 |
| 10 | 0 | 3 | 1 | 42 | 3 | 0.5 | 2 |
| 11 | 0 | 3 | 2 | 43 | 3 | 1 | 0 |
| 12 | 0 | 4 | 0 | 44 | 3 | 1 | 1 |
| 13 | 0 | 4 | 1 | 45 | 3 | 2 | 0 |
| 14 | 0 | 4 | 2 | 46 | 3 | 3 | 0 |
| 15 | 0 | 5 | 0 | 47 | 3 | 4 | 0 |
| 16 | 0 | 5 | 1 | 48 | 4 | 0.5 | 0 |
| 17 | 0 | 5 | 2 | 49 | 4 | 0.5 | 1 |
| 18 | 0 | 6 | 0 | 50 | 4 | 0.5 | 2 |
| 19 | 0 | 6 | 1 | 51 | 4 | 1 | 0 |
| 20 | 1 | 0.5 | 0 | 52 | 4 | 1 | 1 |
| 21 | 1 | 0.5 | 1 | 53 | 4 | 2 | 0 |
| 22 | 1 | 0.5 | 2 | 54 | 4 | 3 | 0 |
| 23 | 1 | 1 | 0 | 55 | 4 | 4 | 0 |
| 24 | 1 | 1 | 1 | 56 | 4 | 5 | 0 |
| 25 | 1 | 2 | 0 | 57 | 4 | 6 | 0 |
| 26 | 1 | 3 | 0 | 58 | N/A | N/A | N/A |
| 27 | 1 | 4 | 0 | 59 | N/A | N/A | N/A |
| 28 | 1 | 5 | 0 | 60 | N/A | N/A | N/A |
| 29 | 1 | 6 | 0 | 61 | N/A | N/A | N/A |
| 30 | 2 | 0.5 | 0 | 62 | N/A | N/A | N/A |
| 31 | 2 | 0.5 | 1 | 63 | N/A | N/A | N/A |

Table 5.7.1-4 lists the mapping to physical resources for the different random access opportunities needed for a certain PRACH density value, . Each quadruple of the format  indicates the location of a specific random access resource, where  is a frequency resource index within the considered time instance,  indicates whether the resource is reoccurring in all radio frames, in even radio frames, or in odd radio frames, respectively,  indicates whether the random access resource is located in first half frame or in second half frame, respectively, and where  is the uplink subframe number where the preamble starts, counting from 0 at the first uplink subframe between 2 consecutive downlink-to-uplink switch points, with the exception of preamble format 4 where  is denoted as (*). The start of the random access preamble formats 0-3 shall be aligned with the start of the corresponding uplink subframe at the UE assuming  and the random access preamble format 4 shall start  before the end of the UpPTS at the UE, where the UpPTS is referenced to the UE's uplink frame timing assuming.

The random access opportunities for each PRACH configuration shall be allocated in time first and then in frequency if and only if time multiplexing is not sufficient to hold all opportunities of a PRACH configuration needed for a certain density value  without overlap in time. For preamble format 0-3, the frequency multiplexing shall be done according to

where  is the number of uplink resource blocks, is the first physical resource block allocated to the PRACH opportunity considered and where  is the first physical resource block available for PRACH.

For preamble format 4, the frequency multiplexing shall be done according to

whereis the system frame number and whereis the number of DL to UL switch points within the radio frame.

For BL/CE UEs, only a subset of the subframes allowed for preamble transmission are allowed as starting subframes for the  repetitions. The allowed starting subframes for a PRACH configuration are determined as follows:

-	Enumerate the subframes that are allowed for preamble transmission for the PRACH configuration as  where  and  correspond to the two subframes allowed for preamble transmission with the smallest and the largest absolute subframe number  , respectively.

-	If a PRACH starting subframe periodicity  is not provided by higher layers, the periodicity of the allowed starting subframes in terms of subframes allowed for preamble transmission is . The allowed starting subframes defined over  are given by  where

-	If a PRACH starting subframe periodicity  is provided by higher layers, it indicates the periodicity of the allowed starting subframes in terms of subframes allowed for preamble transmission. The allowed starting subframes defined over  are given by  where

-	No starting subframe defined over  such that  is allowed.

Each random access preamble occupies a bandwidth corresponding to 6 consecutive resource blocks for both frame structures.

Table 5.7.1-4: Frame structure type 2 random access preamble mapping in time and frequency

| PRACH configuration Index (See Table 5.7.1-3) | UL/DL configuration (See Table 4.2-2) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| 0 | (0,1,0,2) | (0,1,0,1) | (0,1,0,0) | (0,1,0,2) | (0,1,0,1) | (0,1,0,0) | (0,1,0,2) |
| 1 | (0,2,0,2) | (0,2,0,1) | (0,2,0,0) | (0,2,0,2) | (0,2,0,1) | (0,2,0,0) | (0,2,0,2) |
| 2 | (0,1,1,2) | (0,1,1,1) | (0,1,1,0) | (0,1,0,1) | (0,1,0,0) | N/A | (0,1,1,1) |
| 3 | (0,0,0,2) | (0,0,0,1) | (0,0,0,0) | (0,0,0,2) | (0,0,0,1) | (0,0,0,0) | (0,0,0,2) |
| 4 | (0,0,1,2) | (0,0,1,1) | (0,0,1,0) | (0,0,0,1) | (0,0,0,0) | N/A | (0,0,1,1) |
| 5 | (0,0,0,1) | (0,0,0,0) | N/A | (0,0,0,0) | N/A | N/A | (0,0,0,1) |
| 6 | (0,0,0,2) (0,0,1,2) | (0,0,0,1) (0,0,1,1) | (0,0,0,0) (0,0,1,0) | (0,0,0,1) (0,0,0,2) | (0,0,0,0) (0,0,0,1) | (0,0,0,0) (1,0,0,0) | (0,0,0,2) (0,0,1,1) |
| 7 | (0,0,0,1) (0,0,1,1) | (0,0,0,0) (0,0,1,0) | N/A | (0,0,0,0) (0,0,0,2) | N/A | N/A | (0,0,0,1) (0,0,1,0) |
| 8 | (0,0,0,0) (0,0,1,0) | N/A | N/A | (0,0,0,0) (0,0,0,1) | N/A | N/A | (0,0,0,0) (0,0,1,1) |
| 9 | (0,0,0,1) (0,0,0,2) (0,0,1,2) | (0,0,0,0) (0,0,0,1) (0,0,1,1) | (0,0,0,0) (0,0,1,0) (1,0,0,0) | (0,0,0,0) (0,0,0,1) (0,0,0,2) | (0,0,0,0) (0,0,0,1) (1,0,0,1) | (0,0,0,0) (1,0,0,0) (2,0,0,0) | (0,0,0,1) (0,0,0,2) (0,0,1,1) |
| 10 | (0,0,0,0) (0,0,1,0) (0,0,1,1) | (0,0,0,1) (0,0,1,0) (0,0,1,1) | (0,0,0,0) (0,0,1,0) (1,0,1,0) | N/A | (0,0,0,0) (0,0,0,1) (1,0,0,0) | N/A | (0,0,0,0) (0,0,0,2) (0,0,1,0) |
| 11 | N/A | (0,0,0,0) (0,0,0,1) (0,0,1,0) | N/A | N/A | N/A | N/A | (0,0,0,1) (0,0,1,0) (0,0,1,1) |
| 12 | (0,0,0,1) (0,0,0,2) (0,0,1,1) (0,0,1,2) | (0,0,0,0) (0,0,0,1) (0,0,1,0) (0,0,1,1) | (0,0,0,0) (0,0,1,0) (1,0,0,0) (1,0,1,0) | (0,0,0,0) (0,0,0,1) (0,0,0,2) (1,0,0,2) | (0,0,0,0) (0,0,0,1) (1,0,0,0) (1,0,0,1) | (0,0,0,0) (1,0,0,0) (2,0,0,0) (3,0,0,0) | (0,0,0,1) (0,0,0,2) (0,0,1,0) (0,0,1,1) |
| 13 | (0,0,0,0) (0,0,0,2) (0,0,1,0) (0,0,1,2) | N/A | N/A | (0,0,0,0) (0,0,0,1) (0,0,0,2) (1,0,0,1) | N/A | N/A | (0,0,0,0) (0,0,0,1) (0,0,0,2) (0,0,1,1) |
| 14 | (0,0,0,0) (0,0,0,1) (0,0,1,0) (0,0,1,1) | N/A | N/A | (0,0,0,0) (0,0,0,1) (0,0,0,2) (1,0,0,0) | N/A | N/A | (0,0,0,0) (0,0,0,2) (0,0,1,0) (0,0,1,1) |
| 15 | (0,0,0,0) (0,0,0,1) (0,0,0,2) (0,0,1,1) (0,0,1,2) | (0,0,0,0) (0,0,0,1) (0,0,1,0) (0,0,1,1) (1,0,0,1) | (0,0,0,0) (0,0,1,0) (1,0,0,0) (1,0,1,0) (2,0,0,0) | (0,0,0,0) (0,0,0,1) (0,0,0,2) (1,0,0,1) (1,0,0,2) | (0,0,0,0) (0,0,0,1) (1,0,0,0) (1,0,0,1) (2,0,0,1) | (0,0,0,0) (1,0,0,0) (2,0,0,0) (3,0,0,0) (4,0,0,0) | (0,0,0,0) (0,0,0,1) (0,0,0,2) (0,0,1,0) (0,0,1,1) |
| 16 | (0,0,0,1) (0,0,0,2) (0,0,1,0) (0,0,1,1) (0,0,1,2) | (0,0,0,0) (0,0,0,1) (0,0,1,0) (0,0,1,1) (1,0,1,1) | (0,0,0,0) (0,0,1,0) (1,0,0,0) (1,0,1,0) (2,0,1,0) | (0,0,0,0) (0,0,0,1) (0,0,0,2) (1,0,0,0) (1,0,0,2) | (0,0,0,0) (0,0,0,1) (1,0,0,0) (1,0,0,1) (2,0,0,0) | N/A | N/A |
| 17 | (0,0,0,0) (0,0,0,1) (0,0,0,2) (0,0,1,0) (0,0,1,2) | (0,0,0,0) (0,0,0,1) (0,0,1,0) (0,0,1,1) (1,0,0,0) | N/A | (0,0,0,0) (0,0,0,1) (0,0,0,2) (1,0,0,0) (1,0,0,1) | N/A | N/A | N/A |
| 18 | (0,0,0,0) (0,0,0,1) (0,0,0,2) (0,0,1,0) (0,0,1,1) (0,0,1,2) | (0,0,0,0) (0,0,0,1) (0,0,1,0) (0,0,1,1) (1,0,0,1) (1,0,1,1) | (0,0,0,0) (0,0,1,0) (1,0,0,0) (1,0,1,0) (2,0,0,0) (2,0,1,0) | (0,0,0,0) (0,0,0,1) (0,0,0,2) (1,0,0,0) (1,0,0,1) (1,0,0,2) | (0,0,0,0) (0,0,0,1) (1,0,0,0) (1,0,0,1) (2,0,0,0) (2,0,0,1) | (0,0,0,0) (1,0,0,0) (2,0,0,0) (3,0,0,0) (4,0,0,0) (5,0,0,0) | (0,0,0,0) (0,0,0,1) (0,0,0,2) (0,0,1,0) (0,0,1,1) (1,0,0,2) |
| 19 | N/A | (0,0,0,0) (0,0,0,1) (0,0,1,0) (0,0,1,1) (1,0,0,0) (1,0,1,0) | N/A | N/A | N/A | N/A | (0,0,0,0) (0,0,0,1) (0,0,0,2) (0,0,1,0) (0,0,1,1) (1,0,1,1) |
| 20 / 30 | (0,1,0,1) | (0,1,0,0) | N/A | (0,1,0,1) | (0,1,0,0) | N/A | (0,1,0,1) |
| 21 / 31 | (0,2,0,1) | (0,2,0,0) | N/A | (0,2,0,1) | (0,2,0,0) | N/A | (0,2,0,1) |
| 22 / 32 | (0,1,1,1) | (0,1,1,0) | N/A | N/A | N/A | N/A | (0,1,1,0) |
| 23 / 33 | (0,0,0,1) | (0,0,0,0) | N/A | (0,0,0,1) | (0,0,0,0) | N/A | (0,0,0,1) |
| 24 / 34 | (0,0,1,1) | (0,0,1,0) | N/A | N/A | N/A | N/A | (0,0,1,0) |
| 25 / 35 | (0,0,0,1) (0,0,1,1) | (0,0,0,0) (0,0,1,0) | N/A | (0,0,0,1) (1,0,0,1) | (0,0,0,0) (1,0,0,0) | N/A | (0,0,0,1) (0,0,1,0) |
| 26 / 36 | (0,0,0,1) (0,0,1,1) (1,0,0,1) | (0,0,0,0) (0,0,1,0) (1,0,0,0) | N/A | (0,0,0,1) (1,0,0,1) (2,0,0,1) | (0,0,0,0) (1,0,0,0) (2,0,0,0) | N/A | (0,0,0,1) (0,0,1,0) (1,0,0,1) |
| 27 / 37 | (0,0,0,1) (0,0,1,1) (1,0,0,1) (1,0,1,1) | (0,0,0,0) (0,0,1,0) (1,0,0,0) (1,0,1,0) | N/A | (0,0,0,1) (1,0,0,1) (2,0,0,1) (3,0,0,1) | (0,0,0,0) (1,0,0,0) (2,0,0,0) (3,0,0,0) | N/A | (0,0,0,1) (0,0,1,0) (1,0,0,1) (1,0,1,0) |
| 28 / 38 | (0,0,0,1) (0,0,1,1) (1,0,0,1) (1,0,1,1) (2,0,0,1) | (0,0,0,0) (0,0,1,0) (1,0,0,0) (1,0,1,0) (2,0,0,0) | N/A | (0,0,0,1) (1,0,0,1) (2,0,0,1) (3,0,0,1) (4,0,0,1) | (0,0,0,0) (1,0,0,0) (2,0,0,0) (3,0,0,0) (4,0,0,0) | N/A | (0,0,0,1) (0,0,1,0) (1,0,0,1) (1,0,1,0) (2,0,0,1) |
| 29 /39 | (0,0,0,1) (0,0,1,1) (1,0,0,1) (1,0,1,1) (2,0,0,1) (2,0,1,1) | (0,0,0,0) (0,0,1,0) (1,0,0,0) (1,0,1,0) (2,0,0,0) (2,0,1,0) | N/A | (0,0,0,1) (1,0,0,1) (2,0,0,1) (3,0,0,1) (4,0,0,1) (5,0,0,1) | (0,0,0,0) (1,0,0,0) (2,0,0,0) (3,0,0,0) (4,0,0,0) (5,0,0,0) | N/A | (0,0,0,1) (0,0,1,0) (1,0,0,1) (1,0,1,0) (2,0,0,1) (2,0,1,0) |
| 40 | (0,1,0,0) | N/A | N/A | (0,1,0,0) | N/A | N/A | (0,1,0,0) |
| 41 | (0,2,0,0) | N/A | N/A | (0,2,0,0) | N/A | N/A | (0,2,0,0) |
| 42 | (0,1,1,0) | N/A | N/A | N/A | N/A | N/A | N/A |
| 43 | (0,0,0,0) | N/A | N/A | (0,0,0,0) | N/A | N/A | (0,0,0,0) |
| 44 | (0,0,1,0) | N/A | N/A | N/A | N/A | N/A | N/A |
| 45 | (0,0,0,0) (0,0,1,0) | N/A | N/A | (0,0,0,0) (1,0,0,0) | N/A | N/A | (0,0,0,0) (1,0,0,0) |
| 46 | (0,0,0,0) (0,0,1,0) (1,0,0,0) | N/A | N/A | (0,0,0,0) (1,0,0,0) (2,0,0,0) | N/A | N/A | (0,0,0,0) (1,0,0,0) (2,0,0,0) |
| 47 | (0,0,0,0) (0,0,1,0) (1,0,0,0) (1,0,1,0) | N/A | N/A | (0,0,0,0) (1,0,0,0) (2,0,0,0) (3,0,0,0) | N/A | N/A | (0,0,0,0) (1,0,0,0) (2,0,0,0) (3,0,0,0) |
| 48 | (0,1,0,*) | (0,1,0,*) | (0,1,0,*) | (0,1,0,*) | (0,1,0,*) | (0,1,0,*) | (0,1,0,*) |
| 49 | (0,2,0,*) | (0,2,0,*) | (0,2,0,*) | (0,2,0,*) | (0,2,0,*) | (0,2,0,*) | (0,2,0,*) |
| 50 | (0,1,1,*) | (0,1,1,*) | (0,1,1,*) | N/A | N/A | N/A | (0,1,1,*) |
| 51 | (0,0,0,*) | (0,0,0,*) | (0,0,0,*) | (0,0,0,*) | (0,0,0,*) | (0,0,0,*) | (0,0,0,*) |
| 52 | (0,0,1,*) | (0,0,1,*) | (0,0,1,*) | N/A | N/A | N/A | (0,0,1,*) |
| 53 | (0,0,0,*) (0,0,1,*) | (0,0,0,*) (0,0,1,*) | (0,0,0,*) (0,0,1,*) | (0,0,0,*) (1,0,0,*) | (0,0,0,*) (1,0,0,*) | (0,0,0,*) (1,0,0,*) | (0,0,0,*) (0,0,1,*) |
| 54 | (0,0,0,*) (0,0,1,*) (1,0,0,*) | (0,0,0,*) (0,0,1,*) (1,0,0,*) | (0,0,0,*) (0,0,1,*) (1,0,0,*) | (0,0,0,*) (1,0,0,*) (2,0,0,*) | (0,0,0,*) (1,0,0,*) (2,0,0,*) | (0,0,0,*) (1,0,0,*) (2,0,0,*) | (0,0,0,*) (0,0,1,*) (1,0,0,*) |
| 55 | (0,0,0,*) (0,0,1,*) (1,0,0,*) (1,0,1,*) | (0,0,0,*) (0,0,1,*) (1,0,0,*) (1,0,1,*) | (0,0,0,*) (0,0,1,*) (1,0,0,*) (1,0,1,*) | (0,0,0,*) (1,0,0,*) (2,0,0,*) (3,0,0,*) | (0,0,0,*) (1,0,0,*) (2,0,0,*) (3,0,0,*) | (0,0,0,*) (1,0,0,*) (2,0,0,*) (3,0,0,*) | (0,0,0,*) (0,0,1,*) (1,0,0,*) (1,0,1,*) |
| 56 | (0,0,0,*) (0,0,1,*) (1,0,0,*) (1,0,1,*) (2,0,0,*) | (0,0,0,*) (0,0,1,*) (1,0,0,*) (1,0,1,*) (2,0,0,*) | (0,0,0,*) (0,0,1,*) (1,0,0,*) (1,0,1,*) (2,0,0,*) | (0,0,0,*) (1,0,0,*) (2,0,0,*) (3,0,0,*) (4,0,0,*) | (0,0,0,*) (1,0,0,*) (2,0,0,*) (3,0,0,*) (4,0,0,*) | (0,0,0,*) (1,0,0,*) (2,0,0,*) (3,0,0,*) (4,0,0,*) | (0,0,0,*) (0,0,1,*) (1,0,0,*) (1,0,1,*) (2,0,0,*) |
| 57 | (0,0,0,*) (0,0,1,*) (1,0,0,*) (1,0,1,*) (2,0,0,*) (2,0,1,*) | (0,0,0,*) (0,0,1,*) (1,0,0,*) (1,0,1,*) (2,0,0,*) (2,0,1,*) | (0,0,0,*) (0,0,1,*) (1,0,0,*) (1,0,1,*) (2,0,0,*) (2,0,1,*) | (0,0,0,*) (1,0,0,*) (2,0,0,*) (3,0,0,*) (4,0,0,*) (5,0,0,*) | (0,0,0,*) (1,0,0,*) (2,0,0,*) (3,0,0,*) (4,0,0,*) (5,0,0,*) | (0,0,0,*) (1,0,0,*) (2,0,0,*) (3,0,0,*) (4,0,0,*) (5,0,0,*) | (0,0,0,*) (0,0,1,*) (1,0,0,*) (1,0,1,*) (2,0,0,*) (2,0,1,*) |
| 58 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| 59 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| 60 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| 61 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| 62 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| 63 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| NOTE: * UpPTS |  |  |  |  |  |  |  |

### 5.7.2	Preamble sequence generation

The random access preambles are generated from Zadoff-Chu sequences with zero correlation zone, generated from one or several root Zadoff-Chu sequences. The network configures the set of preamble sequences the UE is allowed to use.

There are up to two sets of 64 preambles available in a cell where Set 1 corresponds to higher layer PRACH configuration using prach-ConfigurationIndex and prach-FrequencyOffset and Set 2, if configured, corresponds to higher layer PRACH configuration using prach-ConfigurationIndexHighSpeed and prach-FrequencyOffsetHighSpeed. The set of 64 preamble sequences in a cell is found by including first, in the order of increasing cyclic shift, all the available cyclic shifts of a root Zadoff-Chu sequence with the logical index rootSequenceIndexHighSpeed (for Set 2, if configured) or with the logical index RACH_ROOT_SEQUENCE (for Set 1), where both rootSequenceIndexHighSpeed (if configured) and RACH_ROOT_SEQUENCE are broadcasted as part of the System Information. Additional preamble sequences, in case 64 preambles cannot be generated from a single root Zadoff-Chu sequence, are obtained from the root sequences with the consecutive logical indexes until all the 64 sequences are found. 
The logical root sequence order is cyclic: the logical index 0 is consecutive to 837. The relation between a logical root sequence index and physical root sequence index  is given by Tables 5.7.2-4 and 5.7.2-5 for preamble formats 0 – 3 and 4, respectively.

The  root Zadoff-Chu sequence is defined by

where the length  of the Zadoff-Chu sequence is given by Table 5.7.2-1. From the  root Zadoff-Chu sequence, random access preambles with zero correlation zones of length are defined by cyclic shifts according to

where the cyclic shift is given by

and  is given by Tables 5.7.2-2 and 5.7.2-3 for preamble formats 0-3 and 4, respectively, where the higher-layer parameters zeroCorrelationZoneConfig and zeroCorrelationZoneConfigHighSpeed shall be used for PRACH preamble Set 1 and Set 2 (if configured), respectively. Restricted set type B shall be used for PRACH preamble Set 2 (if configured), and the parameter High-speed-flag provided by higher layers determines if unrestricted set or restricted set type A shall be used for PRACH preamble Set 1.

The variable  is the cyclic shift corresponding to a Doppler shift of magnitude  and is given by

where  is the smallest non-negative integer that fulfils . The parameters for restricted sets of cyclic shifts depend on .

For restricted set type A and , the parameters are given by

For restricted set type A and , the parameters are given by

For restricted set type B and , the parameters are given by

For restricted set type B and , the parameters are given by

For restricted set type B and , the parameters are given by

For restricted set type B and , the parameters are given by

For restricted set type B and , the parameters are given by

For restricted set type B and , the parameters are given by

For all other values of , there are no cyclic shifts in the restricted set.

Table 5.7.2-1: Random access preamble sequence length

| Preamble format |  |
| --- | --- |
| 0 – 3 | 839 |
| 4 | 139 |

Table 5.7.2-2:  for preamble generation (preamble formats 0-3)

| zeroCorrelationZoneConfig, zeroCorrelationZoneConfigHighSpeed | value |  |  |
| --- | --- | --- | --- |
|  | Unrestricted set | Restricted set type A | Restricted set type B |
| 0 | 0 | 15 | 15 |
| 1 | 13 | 18 | 18 |
| 2 | 15 | 22 | 22 |
| 3 | 18 | 26 | 26 |
| 4 | 22 | 32 | 32 |
| 5 | 26 | 38 | 38 |
| 6 | 32 | 46 | 46 |
| 7 | 38 | 55 | 55 |
| 8 | 46 | 68 | 68 |
| 9 | 59 | 82 | 82 |
| 10 | 76 | 100 | 100 |
| 11 | 93 | 128 | 118 |
| 12 | 119 | 158 | 137 |
| 13 | 167 | 202 | - |
| 14 | 279 | 237 | - |
| 15 | 419 | - | - |

Table 5.7.2-3:  for preamble generation (preamble format 4)

| zeroCorrelationZoneConfig | value |
| --- | --- |
| 0 | 2 |
| 1 | 4 |
| 2 | 6 |
| 3 | 8 |
| 4 | 10 |
| 5 | 12 |
| 6 | 15 |
| 7 | N/A |
| 8 | N/A |
| 9 | N/A |
| 10 | N/A |
| 11 | N/A |
| 12 | N/A |
| 13 | N/A |
| 14 | N/A |
| 15 | N/A |

Table 5.7.2-4: Root Zadoff-Chu sequence order for preamble formats 0 – 3

| Logical root sequence number | Physical root sequence number (in increasing order of the corresponding logical sequence number) |
| --- | --- |
| 0–23 | 129, 710, 140, 699, 120, 719, 210, 629, 168, 671, 84, 755, 105, 734, 93, 746, 70, 769, 60, 779 2, 837, 1, 838 |
| 24–29 | 56, 783, 112, 727, 148, 691 |
| 30–35 | 80, 759, 42, 797, 40, 799 |
| 36–41 | 35, 804, 73, 766, 146, 693 |
| 42–51 | 31, 808, 28, 811, 30, 809, 27, 812, 29, 810 |
| 52–63 | 24, 815, 48, 791, 68, 771, 74, 765, 178, 661, 136, 703 |
| 64–75 | 86, 753, 78, 761, 43, 796, 39, 800, 20, 819, 21, 818 |
| 76–89 | 95, 744, 202, 637, 190, 649, 181, 658, 137, 702, 125, 714, 151, 688 |
| 90–115 | 217, 622, 128, 711, 142, 697, 122, 717, 203, 636, 118, 721, 110, 729, 89, 750, 103, 736, 61, 778, 55, 784, 15, 824, 14, 825 |
| 116–135 | 12, 827, 23, 816, 34, 805, 37, 802, 46, 793, 207, 632, 179, 660, 145, 694, 130, 709, 223, 616 |
| 136–167 | 228, 611, 227, 612, 132, 707, 133, 706, 143, 696, 135, 704, 161, 678, 201, 638, 173, 666, 106, 733, 83, 756, 91, 748, 66, 773, 53, 786, 10, 829, 9, 830 |
| 168–203 | 7, 832, 8, 831, 16, 823, 47, 792, 64, 775, 57, 782, 104, 735, 101, 738, 108, 731, 208, 631, 184, 655, 197, 642, 191, 648, 121, 718, 141, 698, 149, 690, 216, 623, 218, 621 |
| 204–263 | 152, 687, 144, 695, 134, 705, 138, 701, 199, 640, 162, 677, 176, 663, 119, 720, 158, 681, 164, 675, 174, 665, 171, 668, 170, 669, 87, 752, 169, 670, 88, 751, 107, 732, 81, 758, 82, 757, 100, 739, 98, 741, 71, 768, 59, 780, 65, 774, 50, 789, 49, 790, 26, 813, 17, 822, 13, 826, 6, 833 |
| 264–327 | 5, 834, 33, 806, 51, 788, 75, 764, 99, 740, 96, 743, 97, 742, 166, 673, 172, 667, 175, 664, 187, 652, 163, 676, 185, 654, 200, 639, 114, 725, 189, 650, 115, 724, 194, 645, 195, 644, 192, 647, 182, 657, 157, 682, 156, 683, 211, 628, 154, 685, 123, 716, 139, 700, 212, 627, 153, 686, 213, 626, 215, 624, 150, 689 |
| 328–383 | 225, 614, 224, 615, 221, 618, 220, 619, 127, 712, 147, 692, 124, 715, 193, 646, 205, 634, 206, 633, 116, 723, 160, 679, 186, 653, 167, 672, 79, 760, 85, 754, 77, 762, 92, 747, 58, 781, 62, 777, 69, 770, 54, 785, 36, 803, 32, 807, 25, 814, 18, 821, 11, 828, 4, 835 |
| 384–455 | 3, 836, 19, 820, 22, 817, 41, 798, 38, 801, 44, 795, 52, 787, 45, 794, 63, 776, 67, 772, 72 767, 76, 763, 94, 745, 102, 737, 90, 749, 109, 730, 165, 674, 111, 728, 209, 630, 204, 635, 117, 722, 188, 651, 159, 680, 198, 641, 113, 726, 183, 656, 180, 659, 177, 662, 196, 643, 155, 684, 214, 625, 126, 713, 131, 708, 219, 620, 222, 617, 226, 613 |
| 456–513 | 230, 609, 232, 607, 262, 577, 252, 587, 418, 421, 416, 423, 413, 426, 411, 428, 376, 463, 395, 444, 283, 556, 285, 554, 379, 460, 390, 449, 363, 476, 384, 455, 388, 451, 386, 453, 361, 478, 387, 452, 360, 479, 310, 529, 354, 485, 328, 511, 315, 524, 337, 502, 349, 490, 335, 504, 324, 515 |
| 514–561 | 323, 516, 320, 519, 334, 505, 359, 480, 295, 544, 385, 454, 292, 547, 291, 548, 381, 458, 399, 440, 380, 459, 397, 442, 369, 470, 377, 462, 410, 429, 407, 432, 281, 558, 414, 425, 247, 592, 277, 562, 271, 568, 272, 567, 264, 575, 259, 580 |
| 562–629 | 237, 602, 239, 600, 244, 595, 243, 596, 275, 564, 278, 561, 250, 589, 246, 593, 417, 422, 248, 591, 394, 445, 393, 446, 370, 469, 365, 474, 300, 539, 299, 540, 364, 475, 362, 477, 298, 541, 312, 527, 313, 526, 314, 525, 353, 486, 352, 487, 343, 496, 327, 512, 350, 489, 326, 513, 319, 520, 332, 507, 333, 506, 348, 491, 347, 492, 322, 517 |
| 630–659 | 330, 509, 338, 501, 341, 498, 340, 499, 342, 497, 301, 538, 366, 473, 401, 438, 371, 468, 408, 431, 375, 464, 249, 590, 269, 570, 238, 601, 234, 605 |
| 660–707 | 257, 582, 273, 566, 255, 584, 254, 585, 245, 594, 251, 588, 412, 427, 372, 467, 282, 557, 403, 436, 396, 443, 392, 447, 391, 448, 382, 457, 389, 450, 294, 545, 297, 542, 311, 528, 344, 495, 345, 494, 318, 521, 331, 508, 325, 514, 321, 518 |
| 708–729 | 346, 493, 339, 500, 351, 488, 306, 533, 289, 550, 400, 439, 378, 461, 374, 465, 415, 424, 270, 569, 241, 598 |
| 730–751 | 231, 608, 260, 579, 268, 571, 276, 563, 409, 430, 398, 441, 290, 549, 304, 535, 308, 531, 358, 481, 316, 523 |
| 752–765 | 293, 546, 288, 551, 284, 555, 368, 471, 253, 586, 256, 583, 263, 576 |
| 766–777 | 242, 597, 274, 565, 402, 437, 383, 456, 357, 482, 329, 510 |
| 778–789 | 317, 522, 307, 532, 286, 553, 287, 552, 266, 573, 261, 578 |
| 790–795 | 236, 603, 303, 536, 356, 483 |
| 796–803 | 355, 484, 405, 434, 404, 435, 406, 433 |
| 804–809 | 235, 604, 267, 572, 302, 537 |
| 810–815 | 309, 530, 265, 574, 233, 606 |
| 816–819 | 367, 472, 296, 543 |
| 820–837 | 336, 503, 305, 534, 373, 466, 280, 559, 279, 560, 419, 420, 240, 599, 258, 581, 229, 610 |

Table 5.7.2-5: Root Zadoff-Chu sequence order for preamble format 4

| Logical root sequence number | Physical root sequence number (in increasing order of the corresponding logical sequence number) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 – 19 | 1 | 138 | 2 | 137 | 3 | 136 | 4 | 135 | 5 | 134 | 6 | 133 | 7 | 132 | 8 | 131 | 9 | 130 | 10 | 129 |
| 20 – 39 | 11 | 128 | 12 | 127 | 13 | 126 | 14 | 125 | 15 | 124 | 16 | 123 | 17 | 122 | 18 | 121 | 19 | 120 | 20 | 119 |
| 40 – 59 | 21 | 118 | 22 | 117 | 23 | 116 | 24 | 115 | 25 | 114 | 26 | 113 | 27 | 112 | 28 | 111 | 29 | 110 | 30 | 109 |
| 60 – 79 | 31 | 108 | 32 | 107 | 33 | 106 | 34 | 105 | 35 | 104 | 36 | 103 | 37 | 102 | 38 | 101 | 39 | 100 | 40 | 99 |
| 80 – 99 | 41 | 98 | 42 | 97 | 43 | 96 | 44 | 95 | 45 | 94 | 46 | 93 | 47 | 92 | 48 | 91 | 49 | 90 | 50 | 89 |
| 100 – 119 | 51 | 88 | 52 | 87 | 53 | 86 | 54 | 85 | 55 | 84 | 56 | 83 | 57 | 82 | 58 | 81 | 59 | 80 | 60 | 79 |
| 120 – 137 | 61 | 78 | 62 | 77 | 63 | 76 | 64 | 75 | 65 | 74 | 66 | 73 | 67 | 72 | 68 | 71 | 69 | 70 | - | - |
| 138 – 837 | N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.7.3	Baseband signal generation

The time-continuous random access signal  is defined by

where,  is an amplitude scaling factor in order to conform to the transmit power  specified in clause 6.1 in 3GPP TS 36.213 [4], and . The location in the frequency domain is controlled by the parameter  is derived from clause 5.7.1. The factor  accounts for the difference in subcarrier spacing between the random access preamble and uplink data transmission. The variable, the subcarrier spacing for the random access preamble, and the variable, a fixed offset determining the frequency-domain location of the random access preamble within the physical resource blocks, are both given by Table 5.7.3-1.

Table 5.7.3-1: Random access baseband parameters

| Preamble format |  |  |
| --- | --- | --- |
| 0 – 3 | 1250 Hz | 7 |
| 4 | 7500 Hz | 2 |

## 5.8	Modulation and upconversion

Modulation and upconversion to the carrier frequency of the complex-valued SC-FDMA baseband signal for each antenna port or the complex-valued PRACH baseband signal is shown in Figure 5.8-1. The filtering required prior to transmission is defined by the requirements in 3GPP TS 36.101 [7].

Figure 5.8-1: Uplink modulation