3GPP TR 37.863-02-01 V15.1.0 (2018-12)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

E-UTRA (Evolved Universal Terrestrial Radio Access) - NR Dual Connectivity (EN-DC) of LTE 2 Down Link (DL) / 1 Up Link (UL) and 1 NR band

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2018, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	16

1	Scope	17

2	References	22

3	Definitions, symbols and abbreviations	22

3.1	Definitions	22

3.2	Symbols	22

3.3	Abbreviations	22

4	Background	23

4.1	TR Maintenance	23

5	DC band combination of LTE 2DL/1UL + one NR band: General Part	23

5.1	Single Tx Switched UL	23

6	DC band combination of LTE 2DL/1UL + one NR band: Specific Band Combination Part	24

6.1	DC_3A-38A-n78_BCS0	24

6.1.1	Operating bands for DC	24

6.1.2	Channel bandwidths per operating band for DC	24

6.1.3	Co-existence studies	24

6.2	DC_1A-7A_n78A	25

6.2.1	Operating bands for DC	25

6.2.2	Channel bandwidths per operating band for DC	26

6.2.3	Co-existence studies	26

6.2.4	∆TIB and ∆RIB values	29

6.2.5	MSD	30

6.3	DC_1A-20A_n78A	30

6.3.1	Operating bands for DC	30

6.3.2	Channel bandwidths per operating band for DC	31

6.3.3	Co-existence studies	31

6.4	DC_3A- 7A_n78A	35

6.4.1	Operating bands for DC	35

6.4.2	Channel bandwidths per operating band for DC	36

6.4.3	Co-existence studies	36

6.5	DC_3A- 20A_n78	40

6.5.1	Operating bands for DC	40

6.5.2	Channel bandwidths per operating band for DC	41

6.5.3	Co-existence studies	41

6.6	DC_7A- 20A_n78A	45

6.6.1	Operating bands for DC	45

6.6.2	Channel bandwidths per operating band for DC	46

6.6.3	Co-existence studies	46

6.7	DC_5A-7A- n78A	50

6.8	DC_1A-3A-n78A	56

6.8.1	Operating bands for DC	56

6.8.2	Channel bandwidths per operating band for DC	57

6.8.3	Co-existence studies	57

6.8.4	∆TIB and ∆RIB values	57

6.8.5	MSD	57

6.9	DC_1A-5A_n78A_BCS0	58

6.9.1	Operating bands for DC	58

6.9.2	Channel bandwidths per operating band for DC	58

6.9.3	Co-existence studies	58

6.9.4	∆TIB and ∆RIB values	62

6.9.5	MSD	63

6.10	DC_1A-5A-n257A	63

6.10.1	Operating bands for DC	63

6.10.2	Channel bandwidths per operating band for DC	63

6.10.3	Co-existence studies	64

6.10.4	∆TIB and ∆RIB values	67

6.10.5	MSD	67

6.11	DC_1A-7A-n257A	67

6.11.1	Operating bands for DC	67

6.11.2	Channel bandwidths per operating band for DC	68

6.11.3	Co-existence studies	68

6.11.4	∆TIB and ∆RIB values	72

6.11.5	MSD	72

6.12	DC_3A-5A-n257A	72

6.12.1	Operating bands for DC	72

6.12.2	Channel bandwidths per operating band for DC	73

6.12.3	Co-existence studies	73

6.12.4	∆TIB and ∆RIB values	77

6.12.5	MSD	77

6.13	DC_3A-7A-n257A	77

6.13.1	Operating bands for DC	77

6.13.2	Channel bandwidths per operating band for DC	78

6.13.3	Co-existence studies	78

6.13.4	∆TIB and ∆RIB values	82

6.13.5	MSD	82

6.14	DC_5A-7A-n257A	82

6.14.1	Operating bands for DC	82

6.14.2	Channel bandwidths per operating band for DC	83

6.14.3	Co-existence studies	83

6.14.4	∆TIB and ∆RIB values	87

6.15	DC_7A-7A-n257A	87

6.15.1	Operating bands for DC	87

6.15.2	Channel bandwidths per operating band for DC	88

6.15.3	Co-existence studies	88

6.15.4	∆TIB and ∆RIB values	89

6.15.5	MSD	90

6.16	DC_3A-5A-n78A	90

6.16.1	Operating bands for DC	90

6.16.2	Channel bandwidths per operating band for DC	90

6.16.3	Co-existence studies	90

6.16.4	∆TIB and ∆RIB values	94

6.16.5	MSD	94

6.17	DC_7A-7A-n78A	96

6.17.1	Operating bands for DC	96

6.17.2	Channel bandwidths per operating band for DC	96

6.17.3	Co-existence studies	96

6.17.4	∆TIB and ∆RIB values	98

6.17.5	MSD	99

6.18	DC_1A-3A-n78	99

6.18.1	Operating bands for DC	99

6.18.2	Channel bandwidths per operating band for DC	99

6.18.3	Co-existence studies	99

6.18.4	∆TIB and ∆RIB values	100

6.19	DC_1A-19A-n78A	100

6.19.1	Operating bands for DC	100

6.19.2	Channel bandwidths per operating band for DC	101

6.19.3	Co-existence studies	101

6.19.4	∆TIB and ∆RIB values	101

6.19.5	MSD	101

6.20	DC_1A-21A-n78	102

6.20.1	Operating bands for DC	102

6.20.2	Channel bandwidths per operating band for DC	102

6.20.3	Co-existence studies	102

6.20.4	∆TIB and ∆RIB values	103

6.20.5	MSD	103

6.21	DC_3A-19A-n78	103

6.21.1	Operating bands for DC	103

6.21.2	Channel bandwidths per operating band for DC	104

6.21.3	Co-existence studies	104

6.21.4	∆TIB and ∆RIB values	104

6.21.5	MSD	104

6.22	DC_3A-21A-n78	105

6.22.1	Operating bands for DC	105

6.22.2	Channel bandwidths per operating band for DC	105

6.22.3	Co-existence studies	105

6.22.4	∆TIB and ∆RIB values	106

6.22.5	MSD	106

6.23	DC_19A-21A-n78	106

6.23.1	Operating bands for DC	106

6.23.2	Channel bandwidths per operating band for DC	107

6.23.3	Co-existence studies	107

6.23.4	∆TIB and ∆RIB values	107

6.23.5	MSD	107

6.24	DC_1A-3A-n79	108

6.24.1	Operating bands for DC	108

6.24.2	Channel bandwidths per operating band for DC	108

6.24.3	Co-existence studies	108

6.24.4	∆TIB and ∆RIB values	108

6.24.5	MSD	109

6.25	DC_1A-19A-n79	109

6.25.1	Operating bands for DC	109

6.25.2	Channel bandwidths per operating band for DC	110

6.25.3	Co-existence studies	110

6.25.4	∆TIB and ∆RIB values	110

6.25.5	MSD	110

6.26	DC_1A-21A-n79	111

6.26.1	Operating bands for DC	111

6.26.2	Channel bandwidths per operating band for DC	111

6.26.3	Co-existence studies	111

6.26.4	∆TIB and ∆RIB values	112

6.26.5	MSD	112

6.27	DC_3A-19A-n79	112

6.27.1	Operating bands for DC	112

6.27.2	Channel bandwidths per operating band for DC	113

6.27.3	Co-existence studies	113

6.27.4	∆TIB and ∆RIB values	113

6.27.5	MSD	113

6.28	DC_3A-21A-n79	114

6.28.1	Operating bands for DC	114

6.28.2	Channel bandwidths per operating band for DC	114

6.28.3	Co-existence studies	114

6.28.4	∆TIB and ∆RIB values	115

6.28.5	MSD	115

6.29	DC_19A-21A-n79	115

6.29.1	Operating bands for DC	115

6.29.2	Channel bandwidths per operating band for DC	116

6.29.3	Co-existence studies	116

6.29.4	∆TIB and ∆RIB values	116

6.29.5	MSD	116

6.30	DC_1A-3A-n257	117

6.30.1	Operating bands for DC	117

6.30.2	Channel bandwidths per operating band for DC	117

6.30.3	Co-existence studies	117

6.30.4	∆TIB and ∆RIB values	117

6.30.5	MSD	118

6.31	DC_1A-19A-n257	118

6.31.1	Operating bands for DC	118

6.31.2	Channel bandwidths per operating band for DC	118

6.31.3	Co-existence studies	119

6.31.4	∆TIB and ∆RIB values	119

6.31.5	MSD	119

6.32	DC_1A-21A-n257	119

6.32.1	Operating bands for DC	119

6.32.2	Channel bandwidths per operating band for DC	120

6.32.3	Co-existence studies	120

6.32.4	∆TIB and ∆RIB values	120

6.32.5	MSD	120

6.33	DC_3A-19A-n257	121

6.33.1	Operating bands for DC	121

6.33.2	Channel bandwidths per operating band for DC	121

6.33.3	Co-existence studies	121

6.33.4	∆TIB and ∆RIB values	121

6.33.5	MSD	122

6.34	DC_3A-21A-n257	122

6.34.1	Operating bands for DC	122

6.34.2	Channel bandwidths per operating band for DC	122

6.34.3	Co-existence studies	122

6.34.4	∆TIB and ∆RIB values	123

6.34.5	MSD	123

6.35	DC_1A-3A-n77	123

6.35.1	Operating bands for DC	123

6.35.2	Channel bandwidths per operating band for DC	123

6.35.3	Co-existence studies	124

6.35.4	∆TIB and ∆RIB values	124

6.35.5	MSD	124

6.36	(Void)	125

6.37	DC_1A-19A-n77	125

6.37.1	Operating bands for DC	125

6.37.2	Channel bandwidths per operating band for DC	125

6.37.3	Co-existence studies	125

6.37.4	∆TIB and ∆RIB values	126

6.37.5	MSD	126

6.38	DC_1A-21A-n77	126

6.38.1	Operating bands for DC	126

6.38.2	Channel bandwidths per operating band for DC	127

6.38.3	Co-existence studies	127

6.38.4	∆TIB and ∆RIB values	127

6.38.5	MSD	127

6.39	DC_3A-19A-n77	128

6.39.1	Operating bands for DC	128

6.39.2	Channel bandwidths per operating band for DC	128

6.39.3	Co-existence studies	128

6.39.4	∆TIB and ∆RIB values	129

6.39.5	MSD	129

6.40	DC_3A-21A-n77	129

6.40.1	Operating bands for DC	129

6.40.2	Channel bandwidths per operating band for DC	130

6.40.3	Co-existence studies	130

6.40.4	∆TIB and ∆RIB values	130

6.40.5	MSD	130

6.41	DC_19A-21A-n77	131

6.41.1	Operating bands for DC	131

6.41.2	Channel bandwidths per operating band for DC	131

6.41.3	Co-existence studies	131

6.41.4	∆TIB and ∆RIB values	132

6.41.5	MSD	132

6.42	DC_1A-3A_n28	132

6.42.1	Operating bands for DC	132

6.42.2	Channel bandwidths per operating band for DC	133

6.42.3	Co-existence studies	133

6.42.4	∆TIB and ∆RIB values	136

6.42.5	MSD	137

6.43	DC_1A-7A_n28A	137

6.43.1	Operating bands for DC	137

6.43.3	Co-existence studies	138

6.43.4	∆TIB and ∆RIB values	140

6.44	DC_1A-20A_n28	141

6.44.1	Operating bands for DC	141

6.44.3	Co-existence studies	142

6.44.4	∆TIB and ∆RIB values	145

6.44.5	MSD	145

6.45	DC_3A-7A_n28	145

6.45.1	Operating bands for DC	145

6.45.2	Channel bandwidths per operating band for DC	145

6.45.3	Co-existence studies	146

6.45.4	∆TIB and ∆RIB values	148

6.46	DC_3A-20A_n28	149

6.46.1	Operating bands for DC	149

6.46.2	Channel bandwidths per operating band for DC	149

6.46.3	Co-existence studies	150

6.46.4	∆TIB and ∆RIB values	152

6.47  DC_7A-20A_n28	153

6.47.1	Operating bands for DC	153

6.47.2	Channel bandwidths per operating band for DC	154

6.47.3	Co-existence studies	154

6.47.4	∆TIB and ∆RIB values	156

6.48	DC_19A-21A-n257	157

6.48.1	Operating bands for DC	157

6.48.2	Channel bandwidths per operating band for DC	158

6.48.3	Co-existence studies	158

6.48.4	∆TIB and ∆RIB values	158

6.48.5	MSD	158

6.49	DC_41C_n41A_BCS0	159

6.50	DC_2A-2A-n257A	162

6.51	DC_2A-5A-n257A	165

6.52	DC_2A-13A-n257A	170

6.53	DC_2A-66A-n257A	176

6.53.4	∆TIB and ∆RIB values	181

6.53.5	MSD	181

6.54	DC_2C-n257A	181

6.55	DC_5B-n257A	184

6.56	DC_5A-5A-n257A	188

6.57	DC_5A-66A-n257A	191

6.57.1	Operating bands for DC	191

6.57.2	Channel bandwidths per operating band for DC	191

6.57.3	Co-existence studies	192

6.57.4	∆TIB and ∆RIB values	195

6.57.5	MSD	196

6.58	DC_13A-66A-n257A	196

6.59	DC_48A-48A-n257A	201

6.60	DC_48C-n257A	203

6.61	DC_66A-66A_n257A	206

6.62	DC_66C-n257A	209

6.63	DC_66A-66A-n260A	212

6.64	DC_66C-n260A	215

6.65	DC_48A-48A-n260A	217

6.66	DC_2A-2A-n260A	220

6.67	DC_2A-13A-n260A	224

6.68	DC_2C-n260A	230

6.69	DC_5A-5A-n260A	233

6.70	DC_5B-n260A	236

6.71	DC_13A-66A-n260A	239

6.72	DC_48C-n260A	244

6.73	DC_2A-(n)71B	246

6.74	DC_19A-42A_n77	250

6.74.1	Operating bands for DC	250

6.74.2	Channel bandwidths per operating band for DC	251

6.74.3	Co-existence studies	251

6.74.4	∆TIB and ∆RIB values	251

6.74.5	MSD	251

6.75	DC_19A-42A_n78	252

6.75.1	Operating bands for DC	252

6.75.2	Channel bandwidths per operating band for DC	252

6.75.3	Co-existence studies	252

6.75.4	∆TIB and ∆RIB values	252

6.75.5	MSD	253

6.76	DC_19A-42A_n79	253

6.76.1	Operating bands for DC	253

6.76.2	Channel bandwidths per operating band for DC	253

6.76.3	Co-existence studies	253

6.76.4	∆TIB and ∆RIB values	254

6.76.5	MSD	254

6.77	DC_19A-42A_n257	254

6.77.1	Operating bands for DC	254

6.77.2	Channel bandwidths per operating band for DC	255

6.77.3	Co-existence studies	255

6.77.4	∆TIB and ∆RIB values	255

6.77.5	MSD	255

6.78	DC_1A-42A_n77	256

6.78.1	Operating bands for DC	256

6.78.2	Channel bandwidths per operating band for DC	256

6.78.3	Co-existence studies	256

6.78.4	∆TIB and ∆RIB values	256

6.78.5	MSD	257

6.79	DC_1A-42A_n78	257

6.79.1	Operating bands for DC	257

6.79.2	Channel bandwidths per operating band for DC	257

6.79.3	Co-existence studies	257

6.79.4	∆TIB and ∆RIB values	258

6.79.5	MSD	258

6.80	DC_1A-42A_n79	258

6.80.1	Operating bands for DC	258

6.80.2	Channel bandwidths per operating band for DC	259

6.80.3	Co-existence studies	259

6.80.4	∆TIB and ∆RIB values	259

6.80.5	MSD	259

6.81	DC_1A-42A_n257	260

6.81.1	Operating bands for DC	260

6.81.2	Channel bandwidths per operating band for DC	260

6.81.3	Co-existence studies	261

6.81.4	∆TIB and ∆RIB values	261

6.81.5	MSD	261

6.82 DC_42C_n77	261

6.82.1	Operating bands for DC	261

6.82.2	Channel bandwidths per operating band for DC	262

6.82.3	Co-existence studies	262

6.82.4	∆TIB and ∆RIB values	262

6.82.5	MSD	262

6.83	DC_42C_n78	263

6.83.1	Operating bands for DC	263

6.83.2	Channel bandwidths per operating band for DC	263

6.83.3	Co-existence studies	263

6.83.4	∆TIB and ∆RIB values	264

6.83.5	MSD	264

6.84	DC_42C_n79	264

6.84.1	Operating bands for DC	264

6.84.2	Channel bandwidths per operating band for DC	264

6.84.3	Co-existence studies	265

6.84.4	∆TIB and ∆RIB values	265

6.84.5	MSD	265

6.85	DC_42C_n257	265

6.85.1	Operating bands for DC	265

6.85.2	Channel bandwidths per operating band for DC	266

6.85.3	Co-existence studies	266

6.85.4	∆TIB and ∆RIB values	266

6.85.5	MSD	266

6.86	DC_3A-41A_n78A	267

6.86.1	Operating bands for DC	267

6.86.2	Channel bandwidths per operating band for DC	267

6.86.3	Co-existence studies	267

6.86.4	∆TIB and ∆RIB values	268

6.87	DC_3C-41A_n78A	269

6.87.1	Operating bands for DC	269

6.87.2	Channel bandwidths per operating band for DC	269

6.87.3	Co-existence studies	270

6.87.4	∆TIB and ∆RIB values	270

6.87.5	MSD	270

6.88	DC_7A-46A_n78A	271

6.88.1	Operating bands for DC	271

6.88.2	Channel bandwidths per operating band for DC	271

6.88.3	Co-existence studies	271

6.88.4	∆TIB and ∆RIB values	271

6.88.5	MSD	272

6.89	DC_46C_n78A	272

6.89.1	Operating bands for DC	272

6.89.2	Channel bandwidths per operating band for DC	273

6.90	DC_3C_n78A	273

6.90.1	Operating bands for DC	273

6.90.2	Channel bandwidths per operating band for DC	273

6.90.3	Co-existence studies	273

6.90.4	∆TIB and ∆RIB values	273

6.90.5	MSD	274

6.91	DC_7C_n78A	275

6.91.1	Operating bands for DC	275

6.91.2	Channel bandwidths per operating band for DC	276

6.91.3	Co-existence studies	276

6.91.4	∆TIB and ∆RIB values	276

6.91.5	MSD	276

6.92	DC_1A-18A-n78A	276

6.92.1	Operating bands for DC	276

6.92.2	Channel bandwidths per operating band for DC	277

6.92.3	Co-existence studies	277

6.92.4	∆TIB and ∆RIB values	277

6.92.5	MSD	277

6.93	DC_1A-28A-n78	278

6.93.1	Operating bands for DC	278

6.93.2	Channel bandwidths per operating band for DC	278

6.93.3	Co-existence studies	278

6.93.4	∆TIB and ∆RIB values	279

6.93.5	MSD	279

6.94	DC_18A-28A-n78A	279

6.94.1	Operating bands for DC	279

6.94.2	Channel bandwidths per operating band for DC	280

6.94.3	Co-existence studies	280

6.94.4	∆TIB and ∆RIB values	280

6.94.5	MSD	280

6.95	DC_1A-18A-n77A	281

6.95.1	Operating bands for DC	281

6.95.2	Channel bandwidths per operating band for DC	281

6.95.3	Co-existence studies	282

6.95.4	∆TIB and ∆RIB values	282

6.95.5	MSD	282

6.96	DC_1A-28A-n77	283

6.96.1	Operating bands for DC	283

6.96.2	Channel bandwidths per operating band for DC	283

6.96.3	Co-existence studies	283

6.96.4	∆TIB and ∆RIB values	283

6.96.5	MSD	284

6.97	DC_18A-28A-n77A	284

6.97.1	Operating bands for DC	284

6.97.2	Channel bandwidths per operating band for DC	285

6.97.3	Co-existence studies	285

6.97.4	∆TIB and ∆RIB values	285

6.97.5	MSD	285

6.98	DC_1A-41A-n79A	286

6.98.1	Operating bands for DC	286

6.98.2	Channel bandwidths per operating band for DC	286

6.98.3	Co-existence studies	286

6.98.4	∆TIB and ∆RIB values	287

6.98.5	MSD	287

6.99	DC_41A-42A-n79A	287

6.99.1	Operating bands for DC	287

6.99.2	Channel bandwidths per operating band for DC	288

6.99.3	Co-existence studies	288

6.99.4	∆TIB and ∆RIB values	288

6.99.5	MSD	289

6.100	DC_42C-n79	289

6.100.1	Operating bands for DC	289

6.100.2	Channel bandwidths per operating band for DC	289

6.100.3	Co-existence studies	289

6.100.4	∆TIB and ∆RIB values	290

6.100.5	MSD	290

6.101	DC_1A-41A-n257A	290

6.101.1	Operating bands for DC	290

6.101.2	Channel bandwidths per operating band for DC	291

6.101.3	Co-existence studies	291

6.101.4	∆TIB and ∆RIB values	291

6.101.5	MSD	291

6.102	DC_41A-42A-n257A	292

6.102.1	Operating bands for DC	292

6.102.2	Channel bandwidths per operating band for DC	292

6.102.3	Co-existence studies	292

6.102.4	∆TIB and ∆RIB values	292

6.102.5	MSD	293

6.103	DC_21A-42A_n77	293

6.103.1	Operating bands for DC	293

6.103.2	Channel bandwidths per operating band for DC	293

6.103.3	Co-existence studies	293

6.103.4	∆TIB and ∆RIB values	294

6.103.5	MSD	294

6.104	DC_21A-42A_n78	294

6.104.1	Operating bands for DC	294

6.104.2	Channel bandwidths per operating band for DC	294

6.104.3	Co-existence studies	295

6.104.4	∆TIB and ∆RIB values	295

6.104.5	MSD	295

6.105	DC_21A-42A_n79	295

6.105.1	Operating bands for DC	295

6.105.2	Channel bandwidths per operating band for DC	296

6.105.3	Co-existence studies	296

6.105.4	∆TIB and ∆RIB values	296

6.105.5	MSD	296

6.106	DC_21A-42A_n257	297

6.106.1	Operating bands for DC	297

6.106.2	Channel bandwidths per operating band for DC	297

6.106.3	Co-existence studies	297

6.106.4	∆TIB and ∆RIB values	297

6.106.5	MSD	298

6.107	DC_66A_(n)71B	298

6.107.1	Operating bands for DC	298

6.107.2	Channel bandwidths per operating band for DC	298

6.107.3	Co-existence studies	298

6.107.4	∆TIB and ∆RIB values	300

6.107.5	MSD	301

6.108	DC_3A-28A_n78_BCS0	301

6.108.1	Operating bands for DC	301

6.108.2	Channel bandwidths per operating band for DC	301

6.108.3	Co-existence studies	302

6.109	DC_7A-28A_n78A _BCS0	306

6.109.1	Operating bands for DC	306

6.109.2	Channel bandwidths per operating band for DC	306

6.109.3	Co-existence studies	306

6.110	 DC_7C_n78A	311

6.110.1	Operating bands for DC	311

6.110.2	Channel bandwidths per operating band for DC	311

6.110.3	Co-existence studies	311

6.110.4	∆TIB and ∆RIB values	314

6.110.5	MSD	314

6.111	DC_3A-42A_n78	315

6.111.1	Operating bands for DC	315

6.111.2	Channel bandwidths per operating band for DC	315

6.111.3	Co-existence studies	315

6.111.4	∆TIB and ∆RIB values	315

6.111.5	MSD	316

6.112	DC_3A-42A-n79	316

6.112.1	Operating bands for DC	316

6.112.2	Channel bandwidths per operating band for DC	316

6.112.3	Co-existence studies	316

6.112.4	∆TIB and ∆RIB values	317

6.112.5	MSD	317

6.113	DC_3A-42A-n257	317

6.113.1	Operating bands for DC	317

6.113.2	Channel bandwidths per operating band for DC	318

6.113.3	Co-existence studies	318

6.113.4	∆TIB and ∆RIB values	318

6.113.5	MSD	318

6.114	DC_1A-41A-n78A	319

6.114.1	Operating bands for DC	319

6.114.2	Channel bandwidths per operating band for DC	319

6.114.3	Co-existence studies	319

6.114.4	∆TIB and ∆RIB values	320

6.114.5	MSD	320

6.115	DC_5A-41A-n78A	320

6.115.1	Operating bands for DC	320

6.115.2	Channel bandwidths per operating band for DC	320

6.115.3	Co-existence studies	321

6.115.4	∆TIB and ∆RIB values	321

6.115.5	MSD	322

6.116	DC_1A-41A-n77A	322

6.116.1	Operating bands for DC	322

6.116.2	Channel bandwidths per operating band for DC	322

6.116.3	Co-existence studies	322

6.116.4	∆TIB and ∆RIB values	323

6.116.5	MSD	323

6.117	DC_41C-n79A	324

6.117.1	Operating bands for DC	324

6.117.2	Channel bandwidths per operating band for DC	324

6.117.3	Co-existence studies	324

6.117.4	∆TIB and ∆RIB values	324

6.117.5	MSD	325

6.118	DC_2A-66A_n71A	325

6.118.1	Operating bands for DC	325

6.118.2	Channel bandwidths per operating band for DC	325

6.118.3	Co-e118istence studies	325

6.118.4	∆TIB and ∆RIB values	329

6.118.5	MSD requirements	329

6.119	DC_1A-18A-n79A	330

6.119.1	Operating bands for DC	330

6.119.2	Channel bandwidths per operating band for DC	331

6.119.3	Co-existence studies	331

6.119.4	∆TIB and ∆RIB values	331

6.119.5	MSD	331

6.120	DC_1A-28A-n79	332

6.120.1	Operating bands for DC	332

6.120.2	Channel bandwidths per operating band for DC	333

6.120.3	Co-existence studies	333

6.120.4	∆TIB and ∆RIB values	333

6.120.5	MSD	333

6.121	DC_1A-28A-n257	334

6.121.1	Operating bands for DC	334

6.121.2	Channel bandwidths per operating band for DC	335

6.121.3	Co-existence studies	335

6.121.4	∆TIB and ∆RIB values	335

6.121.5	MSD	335

6.122	DC_1A-28A_n79	336

6.122.1	Operating bands for DC	336

6.122.2	Channel bandwidths per operating band for DC	336

6.122.3	Co-e122istence studies	336

6.122.4	∆TIB and ∆RIB values	336

6.122.5	MSD	337

6.123	DC_3A-28A_n77	337

6.123.1	Operating bands for DC	337

6.123.2	Channel bandwidths per operating band for DC	337

6.123.3	Co-existence studies	337

6.123.4	∆TIB and ∆RIB values	337

6.123.5	MSD	338

6.124	 DC_3A-28A_n79	339

6.124.1	Operating bands for DC	339

6.124.2	Channel bandwidths per operating band for DC	339

6.124.3	Co-existence studies	339

6.124.4	∆TIB and ∆RIB values	339

6.124.5	MSD	340

6.125	 DC_21A-28A_n77	340

6.125.1	Operating bands for DC	340

6.125.2	Channel bandwidths per operating band for DC	341

6.125.3	Co-existence studies	341

6.125.4	∆TIB and ∆RIB values	341

6.125.5	MSD	341

6.126	DC_21A-28A_n78	342

6.126.1	Operating bands for DC	342

6.126.2	Channel bandwidths per operating band for DC	343

6.126.3	Co-existence studies	343

6.126.4	∆TIB and ∆RIB values	343

6.126.5	MSD	343

6.127	DC_21A-28A_n79	344

6.127.1	Operating bands for DC	344

6.127.2	Channel bandwidths per operating band for DC	344

6.127.3	Co-existence studies	344

6.127.4	∆TIB and ∆RIB values	344

6.127.5	MSD	345

6.128	DC_28A-42A_n79	345

6.128.1	Operating bands for DC	345

6.128.2	Channel bandwidths per operating band for DC	345

6.128.3	Co-existence studies	346

6.128.4	∆TIB and ∆RIB values	346

6.128.5	MSD	346

6.129	DC_1A-8A_n78	347

6.129.1	Operating bands for DC	347

6.129.2	Channel bandwidths per operating band for DC	347

6.129.3	Co-existence studies	347

6.129.4	∆TIB and ∆RIB values	347

6.129.5	MSD	348

6.130	DC_3A-8A_n78	348

6.130.1	Operating bands for DC	348

6.130.2	Channel bandwidths per operating band for DC	348

6.130.3	Co-existence studies	348

6.130.4	∆TIB and ∆RIB values	349

6.130.5	MSD	349

6.131	DC_1A-18A-n257A	349

6.131.1	Operating bands for DC	349

6.131.2	Channel bandwidths per operating band for DC	349

6.131.3	Co-existence studies	350

6.131.4	∆TIB and ∆RIB values	350

6.131.5	MSD	350

6.132	DC_1A-28A_n257	350

6.132.1	Operating bands for DC	350

6.132.2	Channel bandwidths per operating band for DC	351

6.132.3	Co-existence studies	351

6.132.4	∆TIB and ∆RIB values	351

6.132.5	MSD	351

6.133	DC_3A-28A_n257	352

6.133.1	Operating bands for DC	352

6.133.2	Channel bandwidths per operating band for DC	352

6.133.3	Co-existence studies	352

6.133.4	∆TIB and ∆RIB values	352

6.133.5	MSD	353

6.134	DC_21A-28A_n257	353

6.134.1	Operating bands for DC	353

6.134.2	Channel bandwidths per operating band for DC	353

6.134.3	Co-existence studies	353

6.134.4	∆TIB and ∆RIB values	354

6.134.5	MSD	354

6.135	DC_28A-42A_n77	354

6.135.1	Operating bands for DC	354

6.135.2	Channel bandwidths per operating band for DC	354

6.135.3	Co-existence studies	355

6.135.4	∆TIB and ∆RIB values	355

6.135.5	MSD	355

6.136	DC_28A-42A_n78	355

6.136.1	Operating bands for DC	355

6.136.2	Channel bandwidths per operating band for DC	356

6.136.3	Co-existence studies	356

6.136.4	∆TIB and ∆RIB values	356

6.136.5	MSD	356

6.137	DC_28A-42A_n257	357

6.137.1	Operating bands for DC	357

6.137.2	Channel bandwidths per operating band for DC	357

6.137.3	Co-existence studies	357

6.137.4	∆TIB and ∆RIB values	357

6.137.5	MSD	358

6.138	DC_18A-28A-n79A	358

6.138.1	Operating bands for DC	358

6.138.2	Channel bandwidths per operating band for DC	358

6.138.3	Co-existence studies	358

6.138.4	∆TIB and ∆RIB values	358

6.138.5	MSD	359

6.139	DC_18A-28A-n257A	359

6.139.1	Operating bands for DC	359

6.139.3	Co-existence studies	359

6.139.4	∆TIB and ∆RIB values	360

6.139.5	MSD	360

6.140	DC_41A-42A-n77A	360

6.140.1	Operating bands for DC	360

6.140.2	Channel bandwidths per operating band for DC	361

6.140.3	Co-existence studies	361

6.140.4	∆TIB and ∆RIB values	361

6.140.5	MSD	361

6.141	DC_41A-42A-n78A	362

6.141.1	Operating bands for DC	362

6.141.2	Channel bandwidths per operating band for DC	362

6.141.3	Co-existence studies	362

6.141.4	∆TIB and ∆RIB values	362

6.141.5	MSD	363

6.142	DC_41C-n77A	363

6.142.1	Operating bands for DC	363

6.142.2	Channel bandwidths per operating band for DC	363

6.142.3	Co-existence studies	363

6.142.4	∆TIB and ∆RIB values	363

6.142.5	MSD	364

6.143	DC_41C-n78A	364

6.143.1	Operating bands for DC	364

6.143.2	Channel bandwidths per operating band for DC	364

6.143.3	Co-existence studies	364

6.143.4	∆TIB and ∆RIB values	365

6.143.5	MSD	365

6.144	DC_41C-n257A	365

6.144.1	Operating bands for DC	365

6.144.2	Channel bandwidths per operating band for DC	365

6.144.3	Co-existence studies	365

6.144.4	∆TIB and ∆RIB values	366

6.144.5	MSD	366

6.145	DC_42C-n77	366

6.145.1	Operating bands for DC	366

6.145.2	Channel bandwidths per operating band for DC	366

6.145.3	Co-existence studies	367

6.145.4	∆TIB and ∆RIB values	367

6.145.5	MSD	367

6.146	DC_42C-n78	367

6.146.1	Operating bands for DC	367

6.146.2	Channel bandwidths per operating band for DC	368

6.146.3	Co-existence studies	368

6.146.4	∆TIB and ∆RIB values	368

6.146.5	MSD	368

6.147	DC_12A-30A_n66A	369

6.148	DC_2A-12A_n66A	374

6.149	 DC_1A-8A_n257A	379

6.149.1	Operating bands for DC	379

6.149.2	Channel bandwidths per operating band for DC	379

6.149.3	Co-existence studies	379

6.149.4	∆TIB and ∆RIB values	379

6.149.5	MSD	380

6.150	DC_3A-41A_n257A	380

6.150.1	Operating bands for DC	380

6.150.2	Channel bandwidths per operating band for DC	380

6.150.3	Co-existence studies	380

6.150.4	∆TIB and ∆RIB values	381

6.150.5	MSD	381

6.151	DC_41C_n78A	381

6.151.1	Operating bands for DC	381

6.151.2	Channel bandwidths per operating band for DC	382

6.151.3	Co-existence studies	382

6.151.4	∆TIB and ∆RIB values	382

6.151.5	MSD	382

6.152	DC_5A-30A_n66A	382

6.153	DC_2A-30A_n66A	388

6.154	DC_2A-5A_n66A	393

6.155	DC_1A-41A-n78A	398

6.156	DC_30A-66A_n260A	399

6.157	DC_12A-66A_n260A	404

6.158	DC_12A-30A_n260A	409

6.159	DC_5A-66A_n260A	414

6.160	DC_5A-30A_n260A	419

6.161	DC_2A-66A_n260A	425

6.162	DC_2A-30A_n260A	430

6.163	DC_2A-12A_n260A	435

6.164	DC_2A-5A-n260A	440

Annex A:	Change history	447

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document is a technical report for Dual connectivity (DC) band combinations of LTE 2DL/1UL + one NR band under Rel-15 time frame. The purpose is to gather the relevant background information and studies in order to address Dual connectivity (DC) band combinations of LTE 2DL/1UL + one NR band for the Rel-15 band combinations in Table 1-1.

Table 1-1: Release 15 DC band combination of LTE 2DL/1UL + one NR band

| DC combination | REL independent from | UL Band in LTE | Difficult for Simultaneous UL(*) |
| --- | --- | --- | --- |
| DC_1A-3A-n28 | Rel-15 | 1 | No |
|  |  | 3 | No |
| DC_1A-7A-n28 | Rel-15 | 1 | No |
|  |  | 7 | No |
| DC_1A-20A-n28 | Rel-15 | 1 | No |
|  |  | 20 | No |
| DC_3A-7A-n28 | Rel-15 | 3 | No |
|  |  | 7 | No |
| DC_3A-20A-n28 | Rel-15 | 3 | No |
|  |  | 20 | No |
| DC_7A-20A-n28 | Rel-15 | 7 | No |
|  |  | 20 | No |
| DC_41C-n41 | Rel-15 |  | No |
| DC_1A-3A-n50 | Rel-15 |  |  |
| DC_1A-20A-n50 | Rel-15 |  |  |
| DC_1A-28A-n50 | Rel-15 |  |  |
| DC_3A-20A-n50 | Rel-15 |  |  |
| DC_3A-28A-n50 | Rel-15 |  |  |
| DC_7C-n50 | Rel-15 |  |  |
| DC_1A-7A-n50 | Rel-15 |  |  |
| DC_3A-7A-n50 | Rel-15 |  |  |
| DC_20A-28A-n50 | Rel-15 |  |  |
| DC_42C-n50 | Rel-15 |  |  |
| DC_1A-3A-n51 | Rel-15 |  |  |
| DC_1A-20A-n51 | Rel-15 |  |  |
| DC_1A-28A-n51 | Rel-15 |  |  |
| DC_3A-20A-n51 | Rel-15 |  |  |
| DC_3A-28A-n51 | Rel-15 |  |  |
| DC_7C-n51 | Rel-15 |  |  |
| DC_1A-7A-n51 | Rel-15 |  |  |
| DC_3A-7A-n51 | Rel-15 |  |  |
| DC_20A-28A-n51 | Rel-15 |  |  |
| DC_42C-n51 | Rel-15 |  |  |
| DC_66A-71A-n71 | Rel-15 |  |  |
| DC_2A-66A-n71 | Rel-15 |  |  |
| DC_2A-(n)71B | Rel-15 | 2 | No |
|  |  | 71 | No |
| DC_1A-18A-n77 | Rel-15 |  |  |
| DC_1A-28A-n77 | Rel-15 |  |  |
| DC_18A-28A-n77 | Rel-15 |  |  |
| DC_1A-3A-n77 | Rel-15 | 1 | Yes |
|  |  | 3 | Yes |
| DC_1A-19A-n77 | Rel-15 | 1 | Yes |
|  |  | 19 | No |
| DC_1A-21A-n77 | Rel-15 | 1 | Yes |
|  |  | 21 | No |
| DC_3A-19A-n77 | Rel-15 | 3 | Yes |
|  |  | 19 | No |
| DC_3A-21A-n77 | Rel-15 | 3 | Yes |
|  |  | 21 | No |
| DC_19A-21A-n77 | Rel-15 | 19 | No |
|  |  | 21 | No |
| DC_3A-7A-n78 | Rel-15 | 3 | Yes |
|  |  | 7 | No |
| DC_3A-28A-n78 | Rel-15 |  |  |
| DC_7A-28A-n78 | Rel-15 |  |  |
| DC_7C-n78 | Rel-15 |  |  |
| DC_1A-18A-n78 | Rel-15 |  |  |
| DC_1A-28A-n78 | Rel-15 |  |  |
| DC_18A-28A-n78 | Rel-15 |  |  |
| DC_1A-5A-n78 | Rel-15 | 1 | No |
|  |  | 5 | No |
| DC_5A-7A-n78 | Rel-15 |  |  |
| DC_1A-19A-n78 | Rel-15 | 1 | No |
|  |  | 19 | No |
| DC_1A-21A-n78 | Rel-15 | 1 | No |
|  |  | 21 | No |
| DC_3A-19A-n78 | Rel-15 | 3 | Yes |
|  |  | 19 | No |
| DC_3A-21A-n78 | Rel-15 | 3 | Yes |
|  |  | 21 | No |
| DC_19A-21A-n78 | Rel-15 | 19 | No |
|  |  | 21 | No |
| DC_1A-3A-n78 | Rel-15 | 1 | No |
|  |  | 3 | Yes |
| DC_1A-5A-n78 | Rel-15 |  |  |
| DC-3C-n78 | Rel-15 |  |  |
| DC_7C-n78 | Rel-15 |  |  |
| DC_1A-7A-n78 | Rel-15 | 1 | No |
|  |  | 7 | No |
| DC_3A-5A-n78 | Rel-15 | 3 | Yes |
|  |  | 5 | No |
| DC_5A-7A-n78 | Rel-15 | 5 | No |
|  |  | 7 | No |
| DC_7A-7A-n78 | Rel-15 | 7 | No |
|  |  | 7 | No |
| DC_1A-20A-n78 | Rel-15 | 1 | No |
|  |  | 20 | No |
| DC_3A-20A-n78 | Rel-15 | 3 | Yes |
|  |  | 20 | No |
| DC_7A-20A-n78 | Rel-15 | 7 | No |
|  |  | 20 | No |
| DC_3C-n78 | Rel-15 |  |  |
| DC_3A-38A-n78 | Rel-15 | 3 | Yes |
|  |  | 38 | No |
| DC_1A-41A-n79 | Rel-15 |  |  |
| DC_1A-42A-n79 | Rel-15 |  |  |
| DC_41A-42A-n79 | Rel-15 |  |  |
| DC_42C-n79 | Rel-15 |  |  |
| DC_1A-3A-n79 | Rel-15 | 1 | No |
|  |  | 3 | No |
| DC_1A-19A-n79 | Rel-15 | 1 | No |
|  |  | 19 | No |
| DC_1A-21A-n79 | Rel-15 | 1 | No |
|  |  | 21 | No |
| DC_3A-19A-n79 | Rel-15 | 3 | No |
|  |  | 19 | No |
| DC_3A-21A-n79 | Rel-15 | 3 | No |
|  |  | 21 | No |
| DC_19A-21A-n79 | Rel-15 | 19 | No |
|  |  | 21 | No |
| DC_1A-41A-n257 | Rel-15 |  |  |
| DC_1A-42A-n257 | Rel-15 |  |  |
| DC_41A-42A-n257 | Rel-15 |  |  |
| DC_42C-n257 | Rel-15 |  |  |
| DC_1A-3A-n257 | Rel-15 | 1 | No |
|  |  | 3 | No |
| DC_1A-19A-n257 | Rel-15 | 1 | No |
|  |  | 19 | No |
| DC_1A-21A-n257 | Rel-15 | 1 | No |
|  |  | 21 | No |
| DC_3A-19A-n257 | Rel-15 | 3 | No |
|  |  | 19 | No |
| DC_3A-21A-n257 | Rel-15 | 3 | No |
|  |  | 21 | No |
| DC_19A-21A-n257 | Rel-15 | 19 | No |
|  |  | 21 | No |
| DC_1A-3A-n257 | Rel-15 |  |  |
| DC_1A-5A-n257 | Rel-15 | 1 | No |
|  |  | 5 | No |
| DC_1A-7A-n257 | Rel-15 | 1 | No |
|  |  | 7 | No |
| DC_3A-5A-n257 | Rel-15 | 3 | No |
|  |  | 5 | No |
| DC_3A-7A-n257 | Rel-15 | 3 | No |
|  |  | 7 | No |
| DC_5A-7A-n257 | Rel-15 | 5 | No |
|  |  | 7 | No |
| DC_7A-7A-n257 | Rel-15 | 7 | No |
|  |  | 7 | No |
| DC_1A-20A_n78 | Rel-15 |  |  |
| DC_3A-20A_n78 | Rel-15 |  |  |
| DC_7A-20A_n78 | Rel-15 |  |  |
| DC_3C_n78 | Rel-15 |  |  |
| DC_3A-38A_n78 | Rel-15 |  |  |
| DC_1A-3A_n50 | Rel-15 |  |  |
| DC_1A-20A_n50 | Rel-15 |  |  |
| DC_1A-28A_n50 | Rel-15 |  |  |
| DC_3A-20A_n50 | Rel-15 |  |  |
| DC_3A-28A_n50 | Rel-15 |  |  |
| DC_7C_n50 | Rel-15 |  |  |
| DC_1A-7A_n50 | Rel-15 |  |  |
| DC_3A-7A_n50 | Rel-15 |  |  |
| DC_20A-28A_n50 | Rel-15 |  |  |
| DC_42C_n50 | Rel-15 |  |  |
| DC_1A-3A_n51 | Rel-15 |  |  |
| DC_1A-20A_n51 | Rel-15 |  |  |
| DC_1A-28A_n51 | Rel-15 |  |  |
| DC_3A-20A_n51 | Rel-15 |  |  |
| DC_3A-28A_n51 | Rel-15 |  |  |
| DC_7C_n51 | Rel-15 |  |  |
| DC_1A-7A_n51 | Rel-15 |  |  |
| DC_3A-7A_n51 | Rel-15 |  |  |
| DC_20A-28A_n51 | Rel-15 |  |  |
| DC_42C_n51 | Rel-15 |  |  |
| DC_3A-41A_n78 | Rel-15 |  |  |
| DC_1A-8A_n78 | Rel-15 |  |  |
| DC_3A-8A_n78 | Rel-15 |  |  |
| DC_2A-12A_n260 | Rel-15 |  |  |
| DC_2A-5A_n260 | Rel-15 |  |  |
| DC_2A-66A_n260 | Rel-15 |  |  |
| DC_2A-30A_n260 | Rel-15 |  |  |
| DC_12A-66A_n260 | Rel-15 |  |  |
| DC_12A-30A_n260 | Rel-15 |  |  |
| DC_30A-66A_n260 | Rel-15 |  |  |
| DC_66A-66A_n260 | Rel-15 |  |  |
| DC_5A-30A_n260 | Rel-15 |  |  |
| DC_5A-66A_n260 | Rel-15 |  |  |
| DC_2A-12A_n5 | Rel-15 |  |  |
| DC_2A-66A_n5 | Rel-15 |  |  |
| DC_2A-30A_n5 | Rel-15 |  |  |
| DC_12A-30A_n5 | Rel-15 |  |  |
| DC_12A-66A_n5 | Rel-15 |  |  |
| DC_30A-66A_n5 | Rel-15 |  |  |
| DC_66A-66A_n5 | Rel-15 |  |  |
| DC_2A-12A_n66 | Rel-15 |  |  |
| DC_2A-5A_n66 | Rel-15 |  |  |
| DC_2A-30A_n66 | Rel-15 |  |  |
| DC_12A-30A_n66 | Rel-15 |  |  |
| DC_5A-30A_n66 | Rel-15 |  |  |
| DC_1A-42A_n77 | Rel-15 |  |  |
| DC_3A-42A_n77 | Rel-15 |  |  |
| DC_19A-42A_n77 | Rel-15 |  |  |
| DC_21A-42A_n77 | Rel-15 |  |  |
| DC_42C_n77 | Rel-15 |  |  |
| DC_1A-42A_n78 | Rel-15 |  |  |
| DC_3A-42A_n78 | Rel-15 |  |  |
| DC_19A-42A_n78 | Rel-15 |  |  |
| DC_21A-42A_n78 | Rel-15 |  |  |
| DC_42C_n78 | Rel-15 |  |  |
| DC_1A-42A_n79 | Rel-15 |  |  |
| DC_3A-42A_n79 | Rel-15 |  |  |
| DC_19A-42A_n79 | Rel-15 |  |  |
| DC_21A-42A_n79 | Rel-15 |  |  |
| DC_42C_n79 | Rel-15 |  |  |
| DC_1A-42A_n257 | Rel-15 |  |  |
| DC_3A-42A_n257 | Rel-15 |  |  |
| DC_19A-42A_n257 | Rel-15 |  |  |
| DC_21A-42A_n257 | Rel-15 |  |  |
| DC_42C_n257 | Rel-15 |  |  |
| DC_(n)41D | Rel-15 |  | No |
| DC_42C_n258 | Rel-15 |  |  |
| DC_42A-42A_n258 | Rel-15 |  |  |
| DC_2A-2A_n257A | Rel-15 |  |  |
| DC_2C_n257A | Rel-15 |  |  |
| DC_2A-5A_n257A | Rel-15 |  |  |
| DC_2A-13A_n257A | Rel-15 |  |  |
| DC_2A-66A_n257A | Rel-15 |  |  |
| DC_5A-5A_n257A | Rel-15 |  |  |
| DC_5B_n257A | Rel-15 |  |  |
| DC_5A-66A_n257A | Rel-15 |  |  |
| DC_13A-66A_n257A | Rel-15 |  |  |
| DC_48A-48A_n257A | Rel-15 |  |  |
| DC_48C_n257A | Rel-15 |  |  |
| DC_66A-66A_n257A | Rel-15 |  |  |
| DC_66C_n257A | Rel-15 |  |  |
| DC_2A-2A_n260A | Rel-15 |  |  |
| DC_2C_n260A | Rel-15 |  |  |
| DC_2A-13A_n260A | Rel-15 |  |  |
| DC_5A-5A_n260A | Rel-15 |  |  |
| DC_5B_n260A | Rel-15 |  |  |
| DC_13A-66A_n260A | Rel-15 |  |  |
| DC_48A-48A_n260A | Rel-15 |  |  |
| DC_48C_n260A | Rel-15 |  |  |
| DC_66A-66A_n260A | Rel-15 |  |  |
| DC_66C_n260A | Rel-15 |  |  |
| DC_1A-20A_n78 | Rel-15 |  |  |
| DC_3A-20A_n78 | Rel-15 |  |  |
| DC_7A-20A_n78 | Rel-15 |  |  |
| DC_3C_n78 | Rel-15 |  |  |
| DC_3A-38A_n78 | Rel-15 |  |  |
| DC_1A-3A_n50 | Rel-15 |  |  |
| DC_1A-20A_n50 | Rel-15 |  |  |
| DC_1A-28A_n50 | Rel-15 |  |  |

Note(*):	Details on how to support it can be found in Section 5.1.

This TR contains a general part and band specific combination part. The actual requirements are added to the corresponding technical specifications.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]		3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[3]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[4]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[5]	3GPP TR 37.863-01-01: "E-UTRA (Evolved Universal Terrestrial Radio Access) - NR Dual Connectivity (EN-DC) of LTE 1 Down Link (DL) / 1 Up Link (UL) and 1 NR band".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c.

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

BS	Base Station

CA	Carrier Aggregation

CA_nX-nY	Inter-band CA of component carrier(s) in one sub-block within Band X and component carrier(s) in one sub-block within Band Y where X and Y are the applicable NR operating band

CC	Component Carriers

DL	DownLink

FDD	Frequency Division Duplex

IMD	Inter-modulation

MSD	Maximum Sensitivity Deduction

NR	New RAT

SCS	Subcarrier spacing

TDD	Time Division Duplex

UE	User Equipment

UL	UpLink

# 4	Background

The present document is a technical report for Dual connectivity (DC) band combinations of LTE 2DL/1UL + one NR band under Rel-15 time frame. It covers both the UE and BS side. The document is divided in two different parts:

-	General part: this part covers BS and UE specific which is band combination independent.

-	Specific band combination part:  this part covers each band combination and its specific issues independently from each other (i.e. one subclause is defined per band combination)

The specific band combination parts are independent and therefore, the working speed also differs.

## 4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

# 5	DC band combination of LTE 2DL/1UL + one NR band: General Part

## 5.1	Single Tx Switched UL

In some LTE-NR inter-band combinations, as indicated in the “Difficult for simultaneous UL” column in Table 1-1, it is difficult for UE to perform simultaneous LTE+NR UL transmission. The band combinations in which the 2nd order IMD of the simultaneous UL bands fall into its own Rx band are categorized as difficult. And the band combinations for L-L (<1GHz) and H-H (1.695GHz-2.695GHz) in which the 3rd order IMD of the simultaneous UL bands fall into its own Rx band are categorized as difficult. For non-difficult band combinations, simultaneous UL transmission is mandatory.

The following formulas define which channel combinations within difficult band combinations are not affected by IM2 and IM3. Simultaneous UL transmission is mandatory for the unaffected channel combinations in difficult band combinations.

Interference bandwidth: IBW = |a| x CBW1 + |b| x CBW2

|a| + |b| = 2 (or 3)

CBW1 and CBW2 are UL channels

Center frequency of IBW:  fIBW = a x f1 + b x f2

f1 and f2 are center frequency of each UL channel

The range of IMD 2 (or 3):  [fIBW – IBW/2, fIBW + IBW/2]

Note1: The DL channel and UL channels are the transmission bandwidth, on which UE shall be able to apply operations which are configured by RRC reconfiguration and corresponding HARQ timing.

Note 2: For identified difficult band combination, during two adjacent RRC reconfiguration, the changing of transmission bandwidth should not introduce IM2 and IM3, which will result in UE changing from 2Tx to 1Tx. Otherwise, UE behavior is not specified.

# 6	DC band combination of LTE 2DL/1UL + one NR band: Specific Band Combination Part

## 6.1	DC_3A-38A-n78_BCS0

### 6.1.1	Operating bands for DC

Table 6.1.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-38A-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz |  |

### 6.1.2	Channel bandwidths per operating band for DC

Table 6.1.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| DC_3A-38A-n78 | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 | 0 |
|  | 38 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |

### 6.1.3	Co-existence studies

Based on co-existence studies of Band 3 + Band n78 captured in TR 37.863-01-01 [5], no harmonics or IMD fall into own Rx of Band 38. Since simultaneous transmission of Band 3 + Band n78 is not supported, at least in Rel-15, no additional co-existence studies are not needed.

### 6.1.4	∆TIB and ∆RIB values

For DC_3A-38A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.1.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_38A-n78A | 3 | 0.6 |
|  | 38 | 0 |
|  | n78 | 0.8 |

Table 6.1.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_38A-n78A | 3 | 0.2 |
|  | 38 | 0.4 |
|  | n78 | 0.5 |

### 6.1.5	MSD

MSD of this 3DL/2UL DC configuration is already covered by 2DL/2UL DC_3A_n78A, since for DC_38A_n78A:

-	LTE Band 38 and NR Band n78 are synchronised

-	Simultaneous transmission of LTE Band 38 and NR Band n78 are not supported, at least in Rel-15 timeframe

There are no additional MSD requirements.

## 6.2	DC_1A-7A_n78A

### 6.2.1	Operating bands for DC

Table 6.2.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-7A_n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.2.2	Channel bandwidths per operating band for DC

Table 6.2.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| DC_1A-7A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 | 0 |
|  | 7 | 15 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |

### 6.2.3	Co-existence studies

Tables 6.2.3-1 and 6.2.3-2 list Band 1A-7A + Band n78A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.2.3-1: Band 1 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 |  | 3300 | 3800 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 3840 – 3960 |  |  | 6600 – 7600 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 5760 – 5940 |  |  | 9900 – 11400 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1320 – 1880 |  |  | 5220 – 5870 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 40 – 660 |  |  | 4620 – 5680 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 7140 – 7760 |  |  | 8520 – 9580 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 1820 – 2080 |  |  | 3280 – 3820 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1960 – 2640 |  |  | 7920 – 9480 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 2640 – 3760 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9060 – 9740 |  |  | 11820 – 13380 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 10440 – 11560 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11220 – 13280 |  |  | 3880 – 4620 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5940 – 7560 |  |  | 660 – 1840 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15120 – 17180 |  |  | 10980 – 11720 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740 – 15360 |  |  | 12360 – 13540 |  |  |

Based on Table 6.2.3-1,

-	2nd order harmonics may fall into Rx frequencies of 3.3-4.2 GHz Band (Z)

-	3rd order harmonics may fall into Rx frequencies of Band 46, and 47

-	2nd order IMD may fall into Rx frequencies of Band 3, 9, 11, 21, 24, 32, 35, 45, and 46

-	3rd order IMD may fall into Rx frequencies of Band 2, 3, 9, 22, 25, 31, 33, 34, 35, 36, 37, 39, 42, 43, 46, 48, n77 and n78

-	4th order IMD may fall into Rx frequencies of Band 1, 2, 4, 7, 10, 22, 23, 25, 30, 34, 36, 38, 40, 41, 42, 43, 48, 70, n77 and n78

-	5th order IMD may fall into Rx frequencies Band 3, 5, 6, 8, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 26, 27, 28, 29, 32, 44, 45, 67, 68, n78

-	It should be noted that IMD will not be an issue for Band n78 (no self-interference for the TDD band)  even through the IMD products may fall into the concerning band

Table 6.2.3-2: Band 7 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 |  | 3300 | 3800 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 5000 – 5140 |  |  | 6600 – 7600 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 7500 – 7710 |  |  | 9900 – 11400 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 730 – 1300 |  |  | 5800 – 6370 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 1200 – 1840 |  |  | 4030 – 5100 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 8300 – 8940 |  |  | 9100 – 10170 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 2400 – 2670 |  |  | 3280 – 3820 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3700 – 4410 |  |  | 7330 – 8900 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 1460 – 2600 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10800 – 11510 |  |  | 12400 – 13970 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 11600 – 12740 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 10630 – 12700 |  |  | 6200 – 6980 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4760 – 6400 |  |  | 100 – 1110 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15700 – 17770 |  |  | 13300 – 14080 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14900 – 16540 |  |  | 14100 – 15310 |  |  |

Based on Table 6.2.3-2,

-	2nd order IMD may fall into Rx frequencies of Band 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 44, 46, 47, 67, and 68

-	3rd order IMD may fall into Rx frequencies of Band 3, 7, 11, 21, 22, 24, 32, 38, 41, 42, 43, 45, 48 n77 and n78

-	4th order IMD may fall into Rx frequencies of Band 1, 2, 3, 4, 9, 10, 11, 21, 23, 24, 25, 30, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 43, 45, 65, 66, 69, 70, n77 and n78

-	5th order IMD may fall into Rx frequencies of Band 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 31, 44, 46, 47, 67, 68

-	It should be noted that IMD will not be an issue for Band n78 (no self-interference for the TDD band)  even through the IMD products may fall into the concerning band

When 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS; the harmonics and intermodulation products can have impact on these systems. Table 6.2.3-3 and 6.2.3-4 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.2.3-3: 2UL Band 1 + Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD2, 5 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD2, 5 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD2, 5 |
| GPS | 1563 | - | 1587 | Yes |  | IMD2, 5 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD4 |
|  | 2400 | - | 2494 | Yes | Asia | IMD4 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | Harmonics3. IM2, 3 |
|  | 5150 | - | 5350 | Yes | Europe | IM2, 3 |
|  | 5470 | - | 5725 | Yes |  | IM2, 3 |
|  | 5150 | - | 5825 | Yes | Asia | IM2, 3 |

Table 6.2.3-4: 2UL Band 7 + Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD3, 4 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD3. 4 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD3, 4 |
| GPS | 1563 | - | 1587 | Yes |  | IMD3, 4 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD3, 4 |
|  | 2400 | - | 2494 | Yes | Asia | IMD3, 4 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IM2, 5 |
|  | 5150 | - | 5350 | Yes | Europe | IM5 |
|  | 5470 | - | 5725 | Yes |  | IM5 |
|  | 5150 | - | 5825 | No | Asia | IM2, 5 |

The protected bands for DC_1A-7A_n78A are covered in the constituent fall-back modes.

### 6.2.4	∆TIB and ∆RIB values

For DC_1A-7A-n78, the TIB,c and RIB values are given in the tables below.

Table 6.2.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-7A-n78 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |

Table 6.2.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-7A-n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |

### 6.2.5	MSD

Table 6.2.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.10.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_1A-7A_n78A | 1 | 1930 | 5 | 25 | 2120 | 8.3 | FDD | IMD4 ¦fB78-3*fB1¦ |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n78 | 3670 | 10 | 52 | 3670 | N/A | TDD | N/A |
| DC_1A-7A_n78A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | FDD | N/A |
|  | 7 | 2507.5 | 5 | 25 | 2627.5 | 9.1 | FDD | IMD4 ¦fB78-3*fB1¦ |
|  | n78 | 3305 | 10 | 52 | 3305 | N/A | TDD | N/A |
| DC_1A-7A_n78A | 1 | 1950 | 5 | 25 | 2140 | 8.7 | FDD | IMD4 ¦2*fB78-2*fB7¦ |
|  | 7 | 2510 | 10 | 50 | 2630 | N/A | FDD | N/A |
|  | n78 | 3310 | 10 | 52 | 3310 | N/A | TDD | N/A |

## 6.3	DC_1A-20A_n78A

### 6.3.1	Operating bands for DC

Table 6.3.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-20A_n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.3.2	Channel bandwidths per operating band for DC

Table 6.3.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| DC_1A-20A-n78 | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 | 0 |
|  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |

### 6.3.3	Co-existence studies

Tables 6.3.3-1 and 6.3.3-2 list Band 1A-20A + Band n78A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.3.3-1: Band 1 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 |  | 3300 | 3800 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 3840 – 3960 |  |  | 6600 – 7600 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 5760 – 5940 |  |  | 9900 – 11400 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1320 – 1880 |  |  | 5220 – 5870 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 40 – 660 |  |  | 4620 – 5680 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 7140 – 7760 |  |  | 8520 – 9580 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 1820 – 2080 |  |  | 3280 – 3820 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1960 – 2640 |  |  | 7920 – 9480 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 2640 – 3760 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9060 – 9740 |  |  | 11820 – 13380 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 10440 – 11560 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11220 – 13280 |  |  | 3880 – 4620 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5940 – 7560 |  |  | 660 – 1840 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15120 – 17180 |  |  | 10980 – 11720 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740 – 15360 |  |  | 12360 – 13540 |  |  |

Based on Table 6.3.3-1,

-	2nd order harmonics may fall into Rx frequencies of 3.3-4.2 GHz Band (Z)

-	3rd order harmonics may fall into Rx frequencies of Band 46, and 47

-	2nd order IMD may fall into Rx frequencies of Band 3, 9, 11, 21, 24, 32, 35, 45, and 46

-	3rd order IMD may fall into Rx frequencies of Band 2, 3, 9, 22, 25, 31, 33, 34, 35, 36, 37, 39, 42, 43, 46, 48, n77 and n78

-	4th order IMD may fall into Rx frequencies of Band 1, 2, 4, 7, 10, 22, 23, 25, 30, 34, 36, 38, 40, 41, 42, 43, 48, 70, n77 and n78

-	5th order IMD may fall into Rx frequencies Band 3, 5, 6, 8, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 26, 27, 28, 29, 32, 44, 45, 67, 68, n78

-	It should be noted that IMD will not be an issue for Band n78 (no self-interference for the TDD band)  even through the IMD products may fall into the concerning band

Table 6.3.3-2: Band 20 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 |  | 3300 | 3800 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1664 – 1724 |  |  | 6600 – 7600 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2496 – 2596 |  |  | 9900 – 11400 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 2438 – 2968 |  |  | 4132 – 4662 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 1576 – 2136 |  |  | 5738 – 6768 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 4964 – 5524 |  |  | 7432 – 8462 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 732 – 962 |  |  | 3280 – 3820 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 714 – 1304 |  |  | 9038 – 10568 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 4876 – 5936 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5796 – 6386 |  |  | 10732 – 12262 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 8264 – 9324 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12338 – 14368 |  |  | 148 – 472 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 8176 – 9736 |  |  | 4014 – 5104 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14032 – 16062 |  |  | 6628 – 7248 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11564 – 13124 |  |  | 9096 – 10186 |  |  |

Based on Table 6.3.3-2,

-	3rd order harmonics may fall into Rx frequencies of Band 38, 41, 69

-	2nd order IMD may fall into Rx frequencies of Band 7, 38, 41, 69 and n77

-	3rd order IMD may fall into Rx frequencies of Band 1, 2, 3, 4, 5, 6, 8, 9, 10, 12, 13, 14, 17, 18, 19, 20, 22, 25, 26, 27, 28, 33, 34, 35, 36, 37, 39, 42, 43, 44, 46, 47, 48, 65, 66, 67, 68, 70, n77 and n78

-	4th order IMD may fall into Rx frequencies of Band 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 44, 46, 47

-	5th order IMD may fall into Rx frequencies of Band 31 and 77

-	It should be noted that IMD will not be an issue for Band n78 (no self-interference for the TDD band)  even through the IMD products may fall into the concerning band

When 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS; the harmonics and intermodulation products can have impact on these systems. Table 6.X.3-3 and 6.X.3-4 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.3.3-3: 2UL Band 1 +Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD2, 5 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD2, 5 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD2, 5 |
| GPS | 1563 | - | 1587 | Yes |  | IMD2, 5 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD4 |
|  | 2400 | - | 2494 | Yes | Asia | IMD4 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | Harmonics3. IMD2, 3 |
|  | 5150 | - | 5350 | Yes | Europe | IMD2, 3 |
|  | 5470 | - | 5725 | Yes |  | IMD2, 3 |
|  | 5150 | - | 5825 | Yes | Asia | IMD2, 3 |

Table 6.3.3-4: 2UL Band 20 +Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD3 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD3 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD3 |
| GPS | 1563 | - | 1587 | Yes |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD4 |
|  | 2400 | - | 2494 | Yes | Asia | IMD4 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3, 4 |
|  | 5150 | - | 5350 | Yes | Europe | IMD3, 4 |
|  | 5470 | - | 5725 | Yes |  | IMD3, 4 |
|  | 5150 | - | 5825 | Yes | Asia | IMD3, 4 |

The protected bands for DC_1A-20A_n78A are covered in the constituent fall-back modes.

### 6.3.4	∆TIB and ∆RIB values

For DC_1A-20A-n78, the TIB,c and RIB values are given in the tables below.

Table 6.3.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-20A-n78 | 1 | 0.3 |
|  | 20 | 0.3 |
|  | n78 | 0.8 |

Table 6.3.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-20A-n78 | 1 | 0 |
|  | 20 | 0 |
|  | n78 | 0.5 |

### 6.2.5	MSD

Table 6.2.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.2.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_1A-20A_n78A | 1 | 1930 | 5 | 25 | 2120 | 20.3 | FDD | IMD3 ¦fB78-2*fB20¦ |
|  | 20 | 835 | 5 | 25 | 794 | N/A |  | N/A |
|  | n78 | 3790 | 10 | 52 | 3790 | N/A | TDD | N/A |
| DC_1A-20A_n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 3 |  | IMD5 ¦2*fB78-3*fB1¦ |
|  | n78 | 3330 | 10 | 52 | 3330 | N/A | TDD | N/A |

## 6.4	DC_3A- 7A_n78A

### 6.4.1	Operating bands for DC

Table 6.4.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-7A_n78A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.4.2	Channel bandwidths per operating band for DC

Table 6.4.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| DC_3A-7A-n78 | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 | 0 |
|  | 7 | 15 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |

### 6.4.3	Co-existence studies

Tables 6.4.3-1 and 6.4.3-2 list Band 3A-7A + Band n78A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.4.3-1: Band 3 and Band 78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 |  | 3300 | 3800 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 3420 – 3570 |  |  | 6600 – 7600 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 5130 – 5355 |  |  | 9900 – 11400 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1515 – 2090 |  |  | 5010 – 5585 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 270 – 380 |  |  | 4815 – 5890 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 6720 – 7370 |  |  | 8310 – 9385 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 1610 – 1885 |  |  | 3280 – 3820 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1330 – 2055 |  |  | 8115 – 9690 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 3030 – 4180 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8430 – 9155 |  |  | 11610 – 13185 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 10020 – 11170 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11415 – 13490 |  |  | 3040 – 3840 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6330 – 7980 |  |  | 1245 – 2470 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 – 16985 |  |  | 10140 – 10940 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 – 14970 |  |  | 11730 – 12955 |  |  |

Based on Table 6.4.3-1,

-	2nd order harmonics may fall into Rx frequencies of Band 22, 42, 48, n77 and n78

-	3rd order harmonics may fall into Rx frequencies of Band 46

-	2nd order IMD may fall into Rx frequencies of Band 2, 3, 9, 24, 25, 33, 34, 35, 36, 37, 39, 46, and 70

-	3rd order IMD may fall into Rx frequencies of Band 3, 9, 22, 35, 39, 42, 43, 46, 47, 48, n77 and n78

-	4th order IMD may fall into Rx frequencies of Band 2, 3, 9, 11, 21, 22, 24, 25, 32, 33, 34, 35, 36, 37, 39, 42, 43, 45, 48, 70, n77 and n78

-	and 5th order IMD may fall into Rx frequencies of Band 1, 2, 3, 4, 9, 10, 11, 21, 22, 23, 24, 25,30, 32, 33, 34, 35, 36, 37, 39, 40, 42, 43, 45, 48, 70, n77 and n78

-	It should be noted that IMD will not be an issue for Band n78 (no self-interference for the TDD band)  even through the IMD products may fall into the concerning band

Table 6.4.3-2: Band 7 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 |  | 3300 | 3800 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 5000 – 5140 |  |  | 6600 – 7600 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 7500 – 7710 |  |  | 9900 – 11400 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 730 – 1300 |  |  | 5800 – 6370 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 1200 – 1840 |  |  | 4030 – 5100 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 8300 – 8940 |  |  | 9100 – 10170 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 2400 – 2670 |  |  | 3280 – 3820 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3700 – 4410 |  |  | 7330 – 8900 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 1460 – 2600 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10800 – 11510 |  |  | 12400 – 13970 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 11600 – 12740 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 10630 – 12700 |  |  | 6200 – 6980 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4760 – 6400 |  |  | 100 – 1110 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15700 – 17770 |  |  | 13300 – 14080 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14900 – 16540 |  |  | 14100 – 15310 |  |  |

Based on Table 6.4.3-2,

-	2nd order IMD may fall into Rx frequencies of Band 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 44, 46, 47, 67, and 68

-	3rd order IMD may fall into Rx frequencies of Band 3, 7, 11, 21, 22, 24, 32, 38, 41, 42, 43, 45, 48 n77 and n78

-	4th order IMD may fall into Rx frequencies of Band 1, 2, 3, 4, 9, 10, 11, 21, 23, 24, 25, 30, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 43, 45, 65, 66, 69, 70, n77 and n78

-	5th order IMD may fall into Rx frequencies of Band 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 31, 44, 46, 47, 67, 68

-	It should be noted that IMD will not be an issue for Band n78 (no self-interference for the TDD band)  even through the IMD products may fall into the concerning band

When 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS; the harmonics and intermodulation products can have impact on these systems. Table 6.4.3-3 and 6.4.3-4 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.4.3-3: 2UL Band 3 + Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD2, 4, 5 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD2, 4, 5 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD2, 4, 5 |
| GPS | 1563 | - | 1587 | Yes |  | IMD2, 4, 5 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD5 |
|  | 2400 | - | 2494 | Yes | Asia | IMD5 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | Harmonics3, IMD2, 3 |
|  | 5150 | - | 5350 | Yes | Europe | Harmonics3, IMD2, 3 |
|  | 5470 | - | 5725 | Yes |  | IMD2, 3 |
|  | 5150 | - | 5825 | Yes | Asia | Harmonics3, IMD2, 3 |

Table 6.4.3-4: 2UL Band 7 + Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD3, 4 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD3. 4 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD3, 4 |
| GPS | 1563 | - | 1587 | Yes |  | IMD3, 4 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD3, 4 |
|  | 2400 | - | 2494 | Yes | Asia | IMD3, 4 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD2, 5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD5 |
|  | 5470 | - | 5725 | Yes |  | IMD5 |
|  | 5150 | - | 5825 | Yes | Asia | IMD2, 5 |

The protected bands for the DC_3A-7A_n78A will be covered in the constituent fallback modes.

### 6.4.4	∆TIB and ∆RIB values

For DC_3A-7A-n78, the TIB,c and RIB values are given in the tables below.

Table 6.4.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-7A-n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |

Table 6.4.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-7A-n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |

### 6.2.5	MSD

Table 6.2.5-1 lists the MSD required for the dual connectivity configurationfor the cases that IMD interference fall into the own 3rd Rx frequency band. The cases that harmonic issue and IMD interference fall into the own Rx frequency of interfering band have been discussed and specified in the constituent fallback mode. The detailed analysis for MSD can be found in the contribution of R4-1801626.

Table 6.4.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_3A-7A_n78A | 3 | 1725 | 5 | 25 | 1820 | 17.6 | FDD | IMD3 ¦fB78-2*fB7¦ |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n78 | 3310 | 10 | 52 | 3310 | N/A | TDD | N/A |
| DC_3A-7A_n78A | 3 | 1725 | 5 | 25 | 1820 | 8.6 | FDD | IMD4 ¦2*fB78-2*fB7¦ |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n78 | 3475 | 10 | 52 | 3475 | N/A | TDD | N/A |

## 6.5	DC_3A- 20A_n78

### 6.5.1	Operating bands for DC

Table 6.5.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-20A_n78A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.5.2	Channel bandwidths per operating band for DC

Table 6.5.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| DC_3A-20A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 | 0 |
|  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |

### 6.5.3	Co-existence studies

Tables 6.5.3-1 and 6.5.3-2 list Band 3A-20A + Band n78A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.5.3-1: Band 3 and Band 78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 |  | 3300 | 3800 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 3420 – 3570 |  |  | 6600 – 7600 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 5130 – 5355 |  |  | 9900 – 11400 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1515 – 2090 |  |  | 5010 – 5585 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 270 – 380 |  |  | 4815 – 5890 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 6720 – 7370 |  |  | 8310 – 9385 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 1610 – 1885 |  |  | 3280 – 3820 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1330 – 2055 |  |  | 8115 – 9690 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 3030 – 4180 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8430 – 9155 |  |  | 11610 – 13185 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 10020 – 11170 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11415 – 13490 |  |  | 3040 – 3840 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6330 – 7980 |  |  | 1245 – 2470 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 – 16985 |  |  | 10140 – 10940 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 – 14970 |  |  | 11730 – 12955 |  |  |

Based on Table 6.5.3-1,

-	2nd order harmonics may fall into Rx frequencies of Band 22, 42, 48, n77 and n78

-	3rd order harmonics may fall into Rx frequencies of Band 46

-	2nd order IMD may fall into Rx frequencies of Band 2, 3, 9, 24, 25, 33, 34, 35, 36, 37, 39, 46, and 70

-	3rd order IMD may fall into Rx frequencies of Band 3, 9, 22, 35, 39, 42, 43, 46, 47, 48, n77 and n78

-	4th order IMD may fall into Rx frequencies of Band 2, 3, 9, 11, 21, 22, 24, 25, 32, 33, 34, 35, 36, 37, 39, 42, 43, 45, 48, 70, n77 and n78

-	and 5th order IMD may fall into Rx frequencies of Band 1, 2, 3, 4, 9, 10, 11, 21, 22, 23, 24, 25,30, 32, 33, 34, 35, 36, 37, 39, 40, 42, 43, 45, 48, 70, n77 and n78

-	It should be noted that IMD will not be an issue for Band n78 (no self-interference for the TDD band)  even through the IMD products may fall into the concerning band

Table 6.5.3-2: Band 20 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 |  | 3300 | 3800 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1664 – 1724 |  |  | 6600 – 7600 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2496 – 2596 |  |  | 9900 – 11400 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 2438 – 2968 |  |  | 4132 – 4662 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 1576 – 2136 |  |  | 5738 – 6768 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 4964 – 5524 |  |  | 7432 – 8462 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 732 – 962 |  |  | 3280 – 3820 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 714 – 1304 |  |  | 9038 – 10568 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 4876 – 5936 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5796 – 6386 |  |  | 10732 – 12262 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 8264 – 9324 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12338 – 14368 |  |  | 148 – 472 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 8176 – 9736 |  |  | 4014 – 5104 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14032 – 16062 |  |  | 6628 – 7248 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11564 – 13124 |  |  | 9096 – 10186 |  |  |

Based on Table 6.5.3-2,

-	3rd order harmonics may fall into Rx frequencies of Band 38, 41, 69

-	2nd order IMD may fall into Rx frequencies of Band 7, 38, 41, 69 and n77

-	3rd order IMD may fall into Rx frequencies of Band 1, 2, 3, 4, 5, 6, 8, 9, 10, 12, 13, 14, 17, 18, 19, 20, 22, 25, 26, 27, 28, 33, 34, 35, 36, 37, 39, 42, 43, 44, 46, 47, 48, 65, 66, 67, 68, 70, n77 and n78

-	4th order IMD may fall into Rx frequencies of Band 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 44, 46, 47

-	5th order IMD may fall into Rx frequencies of Band 31 and 77

-	It should be noted that IMD will not be an issue for Band n78 (no self-interference for the TDD band)  even through the IMD products may fall into the concerning band

When 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS; the harmonics and intermodulation products can have impact on these systems. Table 6.5.3-3 and 6.5.3-4 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.5.3-3: 2UL Band 3 + Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD2, 4, 5 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD2, 4, 5 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD2, 4, 5 |
| GPS | 1563 | - | 1587 | Yes |  | IMD2, 4, 5 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD5 |
|  | 2400 | - | 2494 | Yes | Asia | IMD5 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | Harmonics3, IMD2, 3 |
|  | 5150 | - | 5350 | Yes | Europe | Harmonics3, IMD2, 3 |
|  | 5470 | - | 5725 | Yes |  | IMD2, 3 |
|  | 5150 | - | 5825 | Yes | Asia | Harmonics3, IMD2, 3 |

Table 6.5.3-4: 2UL Band 20 + Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD3 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD3 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD3 |
| GPS | 1563 | - | 1587 | Yes |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD4 |
|  | 2400 | - | 2494 | Yes | Asia | IMD4 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3, 4 |
|  | 5150 | - | 5350 | Yes | Europe | IMD3, 4 |
|  | 5470 | - | 5725 | Yes |  | IMD3, 4 |
|  | 5150 | - | 5825 | Yes | Asia | IMD3, 4 |

The protected bands for the DC_3A-20A_n78A will be covered in the constituent fallback modes.

### 6.5.4	∆TIB and ∆RIB values

For DC_3A-20A_n78A, the TIB,c and RIB values are given in the tables below.

Table 6.5.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-20A_n78A | 3 | 0.5 |
|  | 20 | 0.3 |
|  | n78 | 0.8 |

Table 6.5.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-20A_n78A | 3 | 0.2 |
|  | 20 | 0 |
|  | n78 | 0.5 |

### 6.5.5	MSD

Table 6.5.5-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The cases that harmonic issue and IMD interference fall into the own Rx frequency of interfering band have been discussed and specified in the constituent fallback mode. The detailed analysis for MSD can be found in the contribution of R4-1801626.

Table 6.5.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_3A-20A_n78A | 3 | 1725 | 5 | 25 | 1820 | 17.3 | FDD | IMD3 ¦fB78-2*fB20¦ |
|  | 20 | 845 | 5 | 25 | 804 | N/A | FDD | N/A |
|  | n78 | 3510 | 10 | 52 | 3510 | N/A | TDD | N/A |

## 6.6	DC_7A- 20A_n78A

### 6.6.1	Operating bands for DC

Table 6.6.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7A-20A_n78A | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.6.2	Channel bandwidths per operating band for DC

Table 6.6.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| DC_7A-20A_n78A | 7 | 15 |  | Yes | Yes | Yes |  |  |  |  |  | 140 | 0 |
|  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |

### 6.6.3	Co-existence studies

Tables 6.6.3-1 and 6.6.3-2 list Band 7A-20A + Band n78A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.6.3-1: Band 7 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 |  | 3300 | 3800 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 5000 – 5140 |  |  | 6600 – 7600 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 7500 – 7710 |  |  | 9900 – 11400 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 730 – 1300 |  |  | 5800 – 6370 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 1200 – 1840 |  |  | 4030 – 5100 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 8300 – 8940 |  |  | 9100 – 10170 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 2400 – 2670 |  |  | 3280 – 3820 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3700 – 4410 |  |  | 7330 – 8900 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 1460 – 2600 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10800 – 11510 |  |  | 12400 – 13970 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 11600 – 12740 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 10630 – 12700 |  |  | 6200 – 6980 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4760 – 6400 |  |  | 100 – 1110 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15700 – 17770 |  |  | 13300 – 14080 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14900 – 16540 |  |  | 14100 – 15310 |  |  |

Based on Table 6.6.3-1,

-	2nd order IMD may fall into Rx frequencies of Band 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 44, 46, 47, 67, and 68

-	3rd order IMD may fall into Rx frequencies of Band 3, 7, 11, 21, 22, 24, 32, 38, 41, 42, 43, 45, 48 n77 and n78

-	4th order IMD may fall into Rx frequencies of Band 1, 2, 3, 4, 9, 10, 11, 21, 23, 24, 25, 30, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 43, 45, 65, 66, 69, 70, n77 and n78

-	5th order IMD may fall into Rx frequencies of Band 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 31, 44, 46, 47, 67, 68

-	It should be noted that IMD will not be an issue for Band n78 (no self-interference for the TDD band)  even through the IMD products may fall into the concerning band

Table 6.6.3-2: Band 20 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 |  | 3300 | 3800 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1664 – 1724 |  |  | 6600 – 7600 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2496 – 2596 |  |  | 9900 – 11400 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 2438 – 2968 |  |  | 4132 – 4662 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 1576 – 2136 |  |  | 5738 – 6768 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 4964 – 5524 |  |  | 7432 – 8462 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 732 – 962 |  |  | 3280 – 3820 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 714 – 1304 |  |  | 9038 – 10568 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 4876 – 5936 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5796 – 6386 |  |  | 10732 – 12262 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 8264 – 9324 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12338 – 14368 |  |  | 148 – 472 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 8176 – 9736 |  |  | 4014 – 5104 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14032 – 16062 |  |  | 6628 – 7248 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11564 – 13124 |  |  | 9096 – 10186 |  |  |

Based on Table 6.6.3-2,

-	3rd order harmonics may fall into Rx frequencies of Band 38, 41, 69

-	2nd order IMD may fall into Rx frequencies of Band 7, 38, 41, 69 and n77

-	3rd order IMD may fall into Rx frequencies of Band 1, 2, 3, 4, 5, 6, 8, 9, 10, 12, 13, 14, 17, 18, 19, 20, 22, 25, 26, 27, 28, 33, 34, 35, 36, 37, 39, 42, 43, 44, 46, 47, 48, 65, 66, 67, 68, 70, n77 and n78

-	4th order IMD may fall into Rx frequencies of Band 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 44, 46, 47

-	5th order IMD may fall into Rx frequencies of Band 31 and 77

-	It should be noted that IMD will not be an issue for Band n78 (no self-interference for the TDD band)  even through the IMD products may fall into the concerning band

When 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS; the harmonics and intermodulation products can have impact on these systems. Table 6.6.3-3 and 6.6.3-4 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.6.3-3: 2UL Band 7 + Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD3, 4 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD3. 4 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD3, 4 |
| GPS | 1563 | - | 1587 | Yes |  | IMD3, 4 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD3, 4 |
|  | 2400 | - | 2494 | Yes | Asia | IMD3, 4 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD2, 5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD5 |
|  | 5470 | - | 5725 | Yes |  | IMD5 |
|  | 5150 | - | 5825 | Yes | Asia | IMD2, 5 |

Table 6.6.3-4: 2UL Band 20 +Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD3 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD3 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD3 |
| GPS | 1563 | - | 1587 | Yes |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD4 |
|  | 2400 | - | 2494 | Yes | Asia | IMD4 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3, 4 |
|  | 5150 | - | 5350 | Yes | Europe | IMD3, 4 |
|  | 5470 | - | 5725 | Yes |  | IMD3, 4 |
|  | 5150 | - | 5825 | Yes | Asia | IMD3, 4 |

The protected bands for the DC_7A-20A_n78A will be covered in the constituent fallback modes.

### 6.6.4	∆TIB and ∆RIB values

For DC_7A-20A_n78A, the TIB,c and RIB values are given in the tables below.

Table 6.6.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-20A_n78A | 7 | 0.3 |
|  | 20 | 0.3 |
|  | n78 | 0.8 |

Table 6.6.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7A-20A_n78A | 7 | 0 |
|  | 20 | 0 |
|  | n78 | 0.5 |

### 6.6.5	MSD

Table 6.6.5-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The cases that harmonic issue and IMD interference fall into the own Rx frequency of interfering band have been discussed and specified in the constituent fallback mode. The detailed analysis for MSD can be found in the contribution of R4-1801626.

Table 6.6.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_7A-20A_n78A | 7 | 2560 | 5 | 25 | 2680 | N/A | FDD | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 30.5 | FDD | IMD2 ¦fB78-fB7¦ |
|  | n78 | 3370 | 10 | 52 | 3370 | N/A | TDD | N/A |
| DC_7A-20A_n78A | 7 | 2560 | 5 | 25 | 2680 | N/A | FDD | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 3.0 | FDD | IMD5 ¦2*fB78-3*fB7¦ |
|  | n78 | 3435 | 10 | 52 | 3435 | N/A | TDD | N/A |
| DC_7A-20A_n78A | 7 | 2555 | 5 | 25 | 2675 | 30.8 | FDD | IMD2 ¦fB78-fB20¦ |
|  | 20 | 845 | 5 | 25 | 804 | N/A | FDD | N/A |
|  | n78 | 3520 | 10 | 52 | 3520 | N/A | TDD | N/A |

## 6.7	DC_5A-7A- n78A

### 6.7.1	Operating bands for DC

Table 6.7.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5A-7A_n78A | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.7.2	Channel bandwidths per operating band for DC

Table 6.7.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_5A-7_n78A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  | 130 |
|  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.7.3	Co-existence studies

Table 6.7.3-1 and 6.7.3-2 lists Band 5A-7A +Band n78 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.7.3-1: Band 5 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 |  | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 |  | 1698 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 |  | 2547 | 9900 | 11400 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2451 | 2976 |  | 4124 | 4649 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2152 |  | 1602 | 5751 | 6776 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4948 |  | 5498 | 7424 | 8449 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 724 |  | 949 | 3290 | 3810 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1328 | 753 |  | 9051 | 10576 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 5952 |  | 4902 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5772 |  | 6347 | 10724 | 12249 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8248 |  | 9298 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14376 |  | 12351 | 96 | 504 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9752 |  | 8202 | 4053 | 5128 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14024 |  | 16049 | 6596 | 7196 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11548 |  | 13098 | 9072 | 10147 |

Based on Table 6.7.3-1;

-	3rd order harmonic may fall into Rx frequencies of band 41

-	2nd order IMD may also fall into own Rx of band n78 and Rx frequencies of bands 7, 38, 41, 69 and n79

-	3rd order IMD may also fall into own Rx frequencies of bands 1, 2, 3, 4, 9, 10, 25, 33, 34, 35, 36, 37, 39, 46, 47, 65, 66, 70, and n79

-	4th order IMD may also fall into own Rx of band 5 and Rx frequencies of bands 6, 8, 13, 14, 18, 19, 20, 26, 27, 28, 44, 46, 47, 67, 68 and n79

-	5th order IMD may also fall into Rx frequencies of bands 31, n77 and n79

Table 6.7.3-2: Band 7 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 |  | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 |  | 5140 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 |  | 7710 | 9900 | 11400 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 730 | 1300 |  | 5800 | 6370 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1200 |  | 1840 | 4030 | 5100 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8300 |  | 8940 | 9100 | 10170 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 2400 |  | 2670 | 3280 | 3820 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3700 | 4410 |  | 7330 | 8900 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 2600 |  | 1460 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10800 |  | 11510 | 12400 | 13970 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 11600 |  | 12740 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12700 |  | 10630 | 6980 | 6200 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6400 |  | 4760 | 1110 | 100 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15700 |  | 17770 | 13300 | 14080 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14900 |  | 16540 | 14100 | 15310 |

Based on Table 6.7.3-2;

-	2nd order IMD may also fall into Rx frequencies of band 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 44, 46, 47, 67, and 68

-	3rd order IMD may also fall into Rx frequencies of bands 3, 11, 21, 24, 32, 45, n77 and n79

-	4th order IMD may also fall into own Rx of band n78 and Rx frequencies of bands 1, 2, 3, 4, 9, 10, 11, 21, 23, 24, 25, 30, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 43, 45, 65, 66, 69, 70, n77 and n79

-	5th order IMD may also fall into Rx frequencies of bands 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 31, 44, 46, 47, 67, 68 and n79

It should be noted that IMD will not be an issue for band n78 (no self-interference for the TDD band) even through the IMD products may fall into the concerning band. When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.7.3-3 and 6.7.3-4 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.7.3-3: 2UL Band 5 +Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD2 |
|  | 2400 | - | 2494 | Yes | Asia | IMD2 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3, 4 |
|  | 5150 | - | 5350 | Yes | Europe | IMD3, 4 |
|  | 5470 | - | 5725 | Yes |  | IMD3, 4 |
|  | 5150 | - | 5825 | Yes | Asia | IMD3 |

Table 6.7.3-4: 2UL Band 7 +Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD3, 4 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD3, 4 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD3, 4 |
| GPS | 1563 | - | 1587 | Yes |  | IMD3, 4 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD3, 4 |
|  | 2400 | - | 2494 | Yes | Asia | IMD3, 4 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD2, 5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD5 |
|  | 5470 | - | 5725 | Yes |  | IMD5 |
|  | 5150 | - | 5825 | Yes | Asia | IMD2, 5 |

Table 6.7.3-5 lists the protected bands required for the dual connectivity configuration.

Table 6.7.3-5: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_1A-5A-7A_n78A | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 20, 22, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 67, 68, 70, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 3800 | - | 3805 | +1.6 | 5 | 1, 2, 3 |
|  | Frequency range | 3805 | - | 3825 | -15.5 | 5 | 1, 2, 3 |
|  | Frequency range | 3825 | - | 3850 | -40 | 1 | 1, 3 |
|  | Frequency range | 3850 | - | 4200 | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 4, 5 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 6 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 | 5 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 7, 8, 9, 10 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 7, 8, 9, 10 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 7, 10 |
| NOTE 1: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 2: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 3: The requirement is applicable for any channel bandwidths within the range 3300 - 3800 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range [TBD – 3792.5] MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range [TBD – 3790] MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to [TBD] RB. NOTE 4: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 5: Applicable only when the assigned E-UTRA carrier is confined within 824 MHz and 849 MHz for UE category M1, M2 and UE category NB1 and NB2. NOTE 6: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 7: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 8: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 9: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 10: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. |  |  |  |  |  |  |  |

### 6.7.4	MSD

Table 6.7.4-1 lists the MSD required for the dual connectivity configuration.

Table 6.7.4-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_5A-7A_n78A | 5 | 844 | 5 | 25 | 889 | 8.3 | FDD | IMD4 ¦fB78-3*fB5¦ |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n78 | 3421 | 10 | 52 | 3421 | N/A | TDD | N/A |
| DC_5A-7A_n78A | 5 | 844 | 5 | 25 | 889 | N/A | FDD | N/A |
|  | 7 | 2525 | 5 | 25 | 2645 | 30.1 | FDD | IMD2 ¦fB78-fB5¦ |
|  | n78 | 3489 | 10 | 52 | 3489 | N/A | TDD | N/A |
| DC_5A-7A_n78A | 5 | 834 | 5 | 25 | 879 | 30.2 | FDD | IMD2 ¦fB78-fB7¦ |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n78 | 3429 | 10 | 52 | 3429 | N/A | TDD | N/A |
| DC_5A-7A_n78A | 5 | 830 | 5 | 25 | 875 | 3.3 | FDD | IMD5 ¦2*fB78-3fB7¦ |
|  | 7 | 2525 | 5 | 25 | 2645 | N/A | FDD | N/A |
|  | n78 | 3350 | 10 | 52 | 3350 | N/A | TDD | N/A |

### 6.7.5	∆TIB and ∆RIB values

For DC_5A-7A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.7.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5A-7A-n78A | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |

Table 6.7.5-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5A-7A-n78A | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |

## 6.8	DC_1A-3A-n78A

### 6.8.1	Operating bands for DC

Table 6.8.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.8.2	Channel bandwidths per operating band for DC

Table 6.8.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A-n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.8.3	Co-existence studies

Based on co-existence studies of Band 1 + Band n78 and Band 3 + Band n78 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	2nd, 3rd and 5th order IMD generated by dual uplink of Band 1 + Band n78 may also fall into own Rx of band 3.

-	5th order IMD generated by dual uplink of Band 3 + Band n78 may also fall into own Rx of band 1.

The protected bands for DC_1A-3A_n78A are covered in the constituent fall-back modes.

### 6.8.4	∆TIB and ∆RIB values

For DC_1A-3A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.8.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-3A-n78A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n78 | 0.8 |

Table 6.8.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-3A-n78A | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n78 | 0.5 |

### 6.8.5	MSD

Table 6.8.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.8.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_1A-3A-n78A | 1 | 1950 | 5 | 25 | 2140 | N/A |  | N/A |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | [31.2] |  | IMD2 |
|  | n78 | 3757.5 | 10 | 52 | 3757.5 | N/A |  | N/A |
|  | 1 | 1935 | 5 | 25 | 2125 | [2.8] | FDD | IMD5 |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A |  | N/A |
|  | n78 | 3725 | 10 | 52 | 3725 | N/A | TDD | N/A |

## 6.9	DC_1A-5A_n78A_BCS0

### 6.9.1	Operating bands for DC

Table 6.9.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-5A_n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.9.2	Channel bandwidths per operating band for DC

Table 6.9.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| DC_1A-5A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 130 | 0 |
|  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |

### 6.9.3	Co-existence studies

Tables 6.9.3-1 and 6.9.3-2 list Band 1A-5A + Band n78A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.9.3-1: Band 1 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 |  | 3300 | 3800 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 3840 – 3960 |  |  | 6600 – 7600 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 5760 – 5940 |  |  | 9900 – 11400 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1320 – 1880 |  |  | 5220 – 5780 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 40 – 660 |  |  | 4620 – 5680 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 7140 – 7760 |  |  | 8520 – 9580 |  |  |
| Two-tone 3rd order IMD products | ¦fx_low – max BW fn¦ |  | ¦fx_high + max BW fn¦ | ¦fn_low – max BW fx¦ |  | ¦fn_high + max BW fx¦ |
| IMD frequency limits (MHz) | 1820 – 2080 |  |  | 3280 – 3820 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1960 – 2640 |  |  | 7920 – 9480 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 2640 – 3760 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9060 – 9740 |  |  | 11820 – 13380 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 10440 – 11560 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11220 – 13280 |  |  | 3880 – 4620 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5940 – 7560 |  |  | 660 – 1840 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15120 – 17180 |  |  | 10980 – 11720 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740 – 15360 |  |  | 12360 – 13540 |  |  |

Based on Table 6.9.3-1,

-	2nd order harmonics may fall into Rx frequencies of Band n77

-	3rd order harmonics may fall into Rx frequencies of Band 46, and 47

-	2nd order IMD may fall into Rx frequencies of Band 3, 9, 11, 21, 24, 32, 35, 45, 46, 50, 51, 74, 75 and 76

-	3rd order IMD may fall into own Rx of Band n78 and Rx frequencies of Band 2, 3, 9, 22, 25, 31, 33, 34, 35, 36, 37, 39, 42, 43, 46, 48, 70, 71, 72,  n77, and n79

-	4th order IMD may fall into own Rx of Band 1 and Band n78 and Rx frequencies of Band 2, 4, 7, 10, 22, 23, 25, 30, 34, 36, 38, 40, 41, 42, 43, 48, 65, 66, 69, 70 and n77

-	5th order IMD may fall into Rx frequencies Band 3, 5, 6, 8, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 26, 27, 28, 29, 32, 44, 45, 50, 51, 67, 68, 74, 75, 76, n77 and n79

-	It should be noted that IMD will not be an issue for Band n78 (no self-interference for the TDD band)  even through the IMD products may fall into the concerning band

Table 6.9.3-2: Band 5 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fn_low | fn_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fn_low | 2* fn_high |
| 2nd harmonics frequency limits (MHz) | 1648－1698 |  | 6600－7600 |  |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fn_low | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2472－2547 |  | 9900－11400 |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |
| IMD frequency limits (MHz) | 2451－2976 |  | 4124－4649 |  |
| Two-tone 3rd order IMD products | ¦fn_low – 2*fx_high¦ | ¦fn_high – 2*fx_low¦ | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |
| IMD frequency limits (MHz) | 1602－2152 |  | 5751－6776 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |
| IMD frequency limits (MHz) | 4948－5498 |  | 7424－8449 |  |
| Two-tone 3rd order IMD products | ¦fx_low – max BW fn¦ | ¦fx_high + max BW fn¦ | ¦fn_low – max BW fx¦ | ¦fn_high + max BW fx¦ |
| IMD frequency limits (MHz) | 724－949 |  | 3290－3810 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 753－1328 |  | 9051－10576 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ | ¦2*fx_high –2* fn_low¦ |  |  |
| IMD frequency limits (MHz) | 4902－5952 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5772－6347 |  | 10724－12249 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ | ¦2*fx_high +2* fn_high¦ |  |  |
| IMD frequency limits (MHz) | 8248－9298 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12351－14376 |  | 96－504 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 8202－9752 |  | 4053－5128 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14024－16049 |  | 6596－7196 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11548－13098 |  | 9072－10147 |  |

Based on Table 6.9.3-1;

-	3rd order harmonic may fall into Rx frequencies of Band 41

-	2nd order IMD may also fall into Rx frequencies of Band 7, 38, 41, 69, n77  and n79

-	3rd order IMD may also fall into own Rx of Band 5 and n78 and Rx frequencies of Band 1, 2, 3, 4, 6, 8, 9, 10, 12, 13, 14, 17, 18, 19, 20, 22, 25, 26, 27, 28, 29, 33, 34, 35, 36, 37, 39, 42, 43, 44, 46, 47, 48, 65, 66, 67, 68, 70, n77 and n79

-	4th order IMD may also fall into own Rx of Band 5 and Rx frequencies of Band 6, 8, 13, 14, 18, 19, 20, 26, 27, 28, 44, 46, 47, 67, 68 and n79

-	5th order IMD may also fall into Rx frequencies of Band 31, 72, n77 and n79

-	It should be noted that IMD will not be an issue for Band n78 (no self-interference for the TDD band)  even through the IMD products may fall into the concerning band

When 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS; the harmonics and intermodulation products can have impact on these systems. Table 6.9.3-3 and 6.9.3-4 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.9.3-3: 2UL Band 1 + Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD2, 5 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD2, 5 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD2, 5 |
| GPS | 1563 | - | 1587 | Yes |  | IMD2, 5 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD4 |
|  | 2400 | - | 2494 | Yes | Asia | IMD4 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | Harmonics3, IMD2, 3 |
|  | 5150 | - | 5350 | Yes | Europe | IMD2, 3 |
|  | 5470 | - | 5725 | Yes |  | IMD2, 3 |
|  | 5150 | - | 5825 | Yes | Asia | Harmonics3, IMD2, 3 |

Table 6.9.3-4: 2UL Band 5 + Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD3 |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | Harmonics3, IMD2 |
|  | 2400 | - | 2494 | Yes | Asia | Harmonics3, IMD2 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3, 4 |
|  | 5150 | - | 5350 | Yes | Europe | IMD3, 4 |
|  | 5470 | - | 5725 | Yes |  | IMD3, 4 |
|  | 5150 | - | 5825 | Yes | Asia | IMD3, 4 |

Table 6.9.3-5 lists the protected bands required for the dual connectivity configuration.

Table 6.9.3-5: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_1A-5A_n78A | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 20, 22, 24, 25, 26, 27, 28, 29, 30, 31, 32, 34, 38, 40, 42, 43, 44, 45, 48, 50, 51, 65, 66, 67, 68, 69, 70, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 3800 | - | 3805 | +1.6 | 5 | 1, 2, 3 |
|  | Frequency range | 3805 | - | 3825 | -15.5 | 5 | 1, 2, 3 |
|  | Frequency range | 3825 | - | 3850 | -40 | 1 | 1, 3 |
|  | Frequency range | 3850 | - | 4200 | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 4, 5 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 6 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 | 5 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 7 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 7, 8 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 7, 8, 9 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 7, 8, 9 |
| NOTE 1: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 2: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 3: The requirement is applicable for any channel bandwidths within the range 3300 - 3800 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range [TBD – 3792.5] MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range [TBD – 3790] MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to [TBD] RB. NOTE 4: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 5: Applicable only when the assigned E-UTRA carrier is confined within 824 MHz and 849 MHz for UE category M1, M2 and UE category NB1 and NB2. NOTE 6: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 7: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 8: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 9: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |  |

### 6.9.4	∆TIB and ∆RIB values

For DC_1A-5A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.9.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-5A-n78A | 1 | 0.3 |
|  | 5 | 0.6 |
|  | n78 | 0.8 |

Table 6.9.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-5A-n78A | 1 | 0.2 |
|  | 5 | 0.2 |
|  | n78 | 0.5 |

### 6.9.5	MSD

Table 6.9.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.9.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_1A-5A_n78A | 1 | 1930 | 5 | 25 | 2120 | 8.3 | FDD | IMD4 ¦fB78-3*fB1¦ |
|  | 5 | 844 | 5 | 25 | 889 | N/A | FDD | N/A |
|  | n78 | 3670 | 10 | 52 | 3670 | N/A | TDD | N/A |
| DC_1A-5A_n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | 5 | 844 | 5 | 25 | 889 | 8.3 | FDD | IMD4 ¦fB78-3*fB5¦ |
|  | n78 | 3421 | 10 | 52 | 3421 | N/A | TDD | N/A |
| DC_1A-5A_n78A | 1 | 1932 | 5 | 25 | 2122 | 18.1 | FDD | IMD3 ¦fB78-2*fB5¦ |
|  | 5 | 829 | 5 | 25 | 874 | N/A | FDD | N/A |
|  | n78 | 3780 | 10 | 52 | 3780 | N/A | TDD | N/A |
| DC_1A-5A_n78A | 1 | 1975 | 5 | 25 | 2165 | N/A | FDD | N/A |
|  | 5 | 840 | 5 | 25 | 885 | 3.1 | FDD | IMD5 ¦2*fB78-3*fB1¦ |
|  | n78 | 3405 | 10 | 52 | 3405 | N/A | TDD | N/A |

## 6.10	DC_1A-5A-n257A

### 6.10.1	Operating bands for DC

Table 6.10.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-5A-n257A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.10.2	Channel bandwidths per operating band for DC

Table 6.10.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-5A-n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 430 |
|  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |

### 6.10.3	Co-existence studies

Tables 6.10.3-1 and 6.10.3-2 list Band 1A-5A + Band n257A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.10.3-1: Band 1 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 |  | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 |  | 3960 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 |  | 5940 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 24520 | 27580 |  | 28420 | 31480 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 22540 |  | 25660 | 51020 | 57080 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 30340 |  | 33460 | 54920 | 60980 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1520 |  | 2380 | 26480 | 29520 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 20560 | 23740 |  | 77520 | 86580 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 49040 |  | 55160 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 32260 |  | 35440 | 81420 | 90480 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 56840 |  | 62960 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 104020 |  | 116080 | 18580 | 21820 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 75540 |  | 84660 | 47060 | 53240 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 107920 |  | 119980 | 34180 | 37420 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 83340 |  | 92460 | 58760 | 64940 |

Based on Table 6.10.3-1;

-	2nd order harmonic may fall into Rx frequency of band n77

-	3rd order harmonic may fall into Rx frequencies of bands 46 and 47

-	2nd order IMD may also fall into Rx frequencies of bands n257 and n258

-	3rd order IMD may also fall into Rx frequencies of bands 1, 2, 3, 4, 9, 10, 23, 24, 25, 30, 33, 34, 35, 36, 37, 39, 40, 65, 66, 70, n257 and n258

Table 6.10.3-2: Band 5 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 |  | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 |  | 1698 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 |  | 2547 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 25651 | 28676 |  | 27324 | 30349 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 27852 |  | 24802 | 52151 | 58176 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 28148 |  | 31198 | 53824 | 59849 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 424 |  | 1249 | 26490 | 29510 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 27028 | 23953 |  | 78651 | 87676 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 57352 |  | 51302 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 28972 |  | 32047 | 80324 | 89349 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 54648 |  | 60698 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 117176 |  | 105151 | 23104 | 26204 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 86852 |  | 77802 | 50453 | 56528 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 106824 |  | 118849 | 29796 | 32896 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 81148 |  | 90198 | 55472 | 61547 |

Based on Table 6.10.3-2;

-	3rd order harmonic may fall into Rx frequencies of band 41

-	2nd order IMD may also fall into own Rx of band n257 and Rx frequencies of band n258

-	3rd order IMD may also fall into own Rx of bands 5 and n257 and Rx frequencies of bands 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 31, 44, 67, 68 and n258

-	4th order IMD may also fall into own Rx of band n257 and Rx frequencies of bands n258 and n259

-	5th order IMD may also fall into Rx frequencies of bands n258 and n259

When 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.10.3-3 and 6.10.3-4 list if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.10.3-3: 2UL Band 1 + Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD3 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD3 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD3 |
| GPS | 1563 | - | 1587 | Yes |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | Harmonic3 |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | Harmonic3 |

Table 6.10.3-4: 2UL Band 5 + Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.10.3-5 lists the protected bands required for the dual connectivity configuration.

Table 6.10.3-5: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_1A-n257A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 42, 65, n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |

Table 6.10.3-6: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_5A-n257A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 42, 65, n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |

### 6.10.4	∆TIB and ∆RIB values

For DC_1A-5A-n257A, the TIB,c and RIB values are given in the tables below.

Table 6.10.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-5A-n257A | 1 | 0 |
|  | 5 | 0 |
|  | n257 | 0 |

Table 6.10.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-5A-n257A | 1 | 0 |
|  | 5 | 0 |
|  | n257 | 0 |

### 6.10.5	MSD

No IMD problems are expected at both LTE band and NR band at FR2. Hence, not necessary for study on IMD problems for LTE-NR DC UE between sub6GHz and FR2. And up to 7th order harmonics are not impact to Rx frequency of both LTE and NR bands.

## 6.11	DC_1A-7A-n257A

### 6.11.1	Operating bands for DC

Table 6.11.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-7A-n257A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.11.2	Channel bandwidths per operating band for DC

Table 6.11.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-7A-n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 440 |
|  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |

### 6.11.3	Co-existence studies

Tables 6.11.3-1 and 6.11.3-2 list Band 1A-7A + Band n257A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.11.3-1: Band 1 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 |  | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 |  | 3960 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 |  | 5940 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 24520 | 27580 |  | 28420 | 31480 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 22540 |  | 25660 | 51020 | 57080 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 30340 |  | 33460 | 54920 | 60980 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1520 |  | 2380 | 26480 | 29520 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 20560 | 23740 |  | 77520 | 86580 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 49040 |  | 55160 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 32260 |  | 35440 | 81420 | 90480 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 56840 |  | 62960 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 104020 |  | 116080 | 18580 | 21820 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 75540 |  | 84660 | 47060 | 53240 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 107920 |  | 119980 | 34180 | 37420 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 83340 |  | 92460 | 58760 | 64940 |

Based on Table 6.11.3-1;

-	2nd order harmonic may fall into Rx frequency of band n77

-	3rd order harmonic may fall into Rx frequencies of bands 46 and 47

-	2nd order IMD may also fall into Rx frequencies of bands n257 and n258

-	3rd order IMD may also fall into Rx frequencies of bands 1, 2, 3, 4, 9, 10, 23, 24, 25, 30, 33, 34, 35, 36, 37, 39, 40, 65, 66, 70, n257 and n258

Table 6.11.3-2: Band 7 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 |  | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 |  | 5140 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 |  | 7710 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 23930 | 27000 |  | 29000 | 32070 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 21360 |  | 24500 | 50430 | 56500 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 31500 |  | 34640 | 55500 | 61570 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 2100 |  | 2970 | 26480 | 29520 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 22000 | 18790 |  | 76930 | 86000 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 54000 |  | 47860 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 34000 |  | 37210 | 82000 | 91070 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 58000 |  | 64160 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 115500 |  | 103430 | 16220 | 19500 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 83500 |  | 74360 | 45290 | 51500 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 108500 |  | 120570 | 36500 | 39780 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 84500 |  | 93640 | 60500 | 66710 |

Based on Table 6.11.3-2;

-	3rd order IMD may fall into Rx frequencies of bands 1, 4, 7, 10, 23, 30, 34, 40, 41, 65, 66, 69, n257, n258 and n259.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.11.3-3 and 6.11.3-4  list if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.11.3-3: 2UL Band 1 + Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD3 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD3 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD3 |
| GPS | 1563 | - | 1587 | Yes |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | Harmonic3 |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | Harmonic3 |

Table 6.11.3-4: 2UL Band 7 + Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD3 |
|  | 2400 | - | 2494 | Yes | Asia | IMD3 |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.11.3-5 lists the protected bands required for the dual connectivity configuration.

Table 6.11.3-5: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_1A_n257A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 42, 65, n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |

Table 6.11.3-6: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_7A_n257A | E-UTRA Band 1, 3, 5, 7, 8, 26, 28, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 |  |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 |  |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 |  |

### 6.11.4	∆TIB and ∆RIB values

For DC_1A-7A-n257A, the TIB,c and RIB values are given in the tables below.

Table 6.11.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-7A-n257A | 1 | 0 |
|  | 7 | 0 |
|  | n257 | 0 |

Table 6.11.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-7A-n257A | 1 | 0 |
|  | 7 | 0 |
|  | n257 | 0 |

### 6.11.5	MSD

No IMD problems are expected at both LTE band and NR band at FR2. Hence, not necessary for study on IMD problems for LTE-NR DC UE between sub6GHz and FR2. And up to 7th order harmonics are not impact to Rx frequency of both LTE and NR bands.

## 6.12	DC_3A-5A-n257A

### 6.12.1	Operating bands for DC

Table 6.12.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-5A-n257A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 GHz | – | 29500 MHz | TDD |

### 6.12.2	Channel bandwidths per operating band for DC

Table 6.12.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-5A-n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 430 |
|  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |

### 6.12.3	Co-existence studies

Tables 6.12.3-1 and 6.12.3-2 list Band 3A-5A + Band n257A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.12.3-1: Band 3 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 |  | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 |  | 3570 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 |  | 5355 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 24715 | 27790 |  | 28210 | 31285 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 22930 |  | 26080 | 51215 | 57290 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 29920 |  | 33070 | 54710 | 60785 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1310 |  | 2185 | 26480 | 29520 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 21145 | 24370 |  | 77715 | 86790 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 49430 |  | 55580 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 31630 |  | 34855 | 81210 | 90285 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 56420 |  | 62570 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 104215 |  | 116290 | 19360 | 22660 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 75930 |  | 85080 | 47645 | 53870 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 107710 |  | 119785 | 33340 | 36640 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 82920 |  | 92070 | 58130 | 64355 |

Based on Table 6.12.3-1;

-	2nd order harmonic may fall into Rx frequency of band 22, 42, 48, n77 and n78

-	3rd order harmonic may fall into Rx frequencies of bands 46

-	2nd order IMD may also fall into Rx frequencies of bands n257 and n258

-	3rd order IMD may also fall into Rx frequencies of bands 1, 2, 3, 4, 9, 10, 11, 21, 23, 24, 25, 33, 34, 35, 36, 37, 39, 45, 65, 66, 70, n257 and n258

-	4th order IMD may also fall into Rx frequencies of bands n258

Table 6.12.3-2: Band 5 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 |  | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 |  | 1698 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 |  | 2547 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 25651 | 28676 |  | 27324 | 30349 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 27852 |  | 24802 | 52151 | 58176 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 28148 |  | 31198 | 53824 | 59849 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 424 |  | 1249 | 26490 | 29510 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 27028 | 23953 |  | 78651 | 87676 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 57352 |  | 51302 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 28972 |  | 32047 | 80324 | 89349 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 54648 |  | 60698 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 117176 |  | 105151 | 23104 | 26204 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 86852 |  | 77802 | 50453 | 56528 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 106824 |  | 118849 | 29796 | 32896 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 81148 |  | 90198 | 55472 | 61547 |

Based on Table 6.12.3-2;

-	3rd order harmonic may fall into Rx frequencies of band 41

-	2nd order IMD may also fall into own Rx of band n257 and Rx frequencies of band n258

-	3rd order IMD may also fall into own Rx of bands 5 and n257 and Rx frequencies of bands 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 31, 44, 67, 68 and n258

-	4th order IMD may also fall into own Rx of band n257 and Rx frequencies of bands n258 and n259

-	5th order IMD may also fall into Rx frequencies of bands n258 and n259

When 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.12.3-3 and 6.12.3-4 list if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.12.3-3: 2UL Band 3 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | Harmonic3 |
|  | 5150 | - | 5350 | Yes | Europe | Harmonic3 |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | Harmonic3 |

Table 6.12.3-4: 2UL Band 5 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.12.3-5 and 6.12.3-6 list the protected bands required for the dual connectivity configuration.

Table 6.12.3-5: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_3A-n257A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 65, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, n77, n78 | FDL_low | - | FDL_high | -50 | 1 | Harmonic exception |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |

Table 6.12.3-6: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_5A-n257A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 42, 65, n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |

### 6.12.4	∆TIB and ∆RIB values

For DC_3A-5A-n257A, the TIB,c and RIB values are given in the tables below.

Table 6.12.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-5A-n257A | 3 | 0 |
|  | 5 | 0 |
|  | n257 | 0 |

Table 6.12.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-5A-n257A | 3 | 0 |
|  | 5 | 0 |
|  | n257 | 0 |

### 6.12.5	MSD

No IMD problems are expected at both LTE band and NR band at FR2. Hence, not necessary for study on IMD problems for LTE-NR DC UE between sub6GHz and FR2. And up to 7th order harmonics are not impact to Rx frequency of both LTE and NR bands.

## 6.13	DC_3A-7A-n257A

### 6.13.1	Operating bands for DC

Table 6.13.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-7A-n257A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 GHz | – | 29500 MHz | TDD |

### 6.13.2	Channel bandwidths per operating band for DC

Table 6.13.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-7A-n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 440 |
|  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |

### 6.13.3	Co-existence studies

Tables 6.13.3-1 and 6.13.3-2 list Band 3A-7A + Band n257A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.13.3-1: Band 3 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 |  | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 |  | 3570 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 |  | 5355 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 24715 | 27790 |  | 28210 | 31285 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 22930 |  | 26080 | 51215 | 57290 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 29920 |  | 33070 | 54710 | 60785 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1310 |  | 2185 | 26480 | 29520 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 21145 | 24370 |  | 77715 | 86790 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 49430 |  | 55580 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 31630 |  | 34855 | 81210 | 90285 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 56420 |  | 62570 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 104215 |  | 116290 | 19360 | 22660 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 75930 |  | 85080 | 47645 | 53870 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 107710 |  | 119785 | 33340 | 36640 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 82920 |  | 92070 | 58130 | 64355 |

Based on Table 6.13.3-1;

-	2nd order harmonic may fall into Rx frequency of band 22, 42, 48, n77 and n78

-	3rd order harmonic may fall into Rx frequencies of bands 46

-	2nd order IMD may also fall into Rx frequencies of bands n257 and n258

-	3rd order IMD may also fall into Rx frequencies of bands 1, 2, 3, 4, 9, 10, 11, 21, 23, 24, 25, 33, 34, 35, 36, 37, 39, 45, 65, 66, 70, n257 and n258

-	4th order IMD may also fall into Rx frequencies of bands n258

Table 6.13.3-2: Band 7 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 |  | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 |  | 5140 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 |  | 7710 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 23930 | 27000 |  | 29000 | 32070 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 21360 |  | 24500 | 50430 | 56500 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 31500 |  | 34640 | 55500 | 61570 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 2100 |  | 2970 | 26480 | 29520 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 22000 | 18790 |  | 76930 | 86000 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 54000 |  | 47860 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 34000 |  | 37210 | 82000 | 91070 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 58000 |  | 64160 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 115500 |  | 103430 | 16220 | 19500 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 83500 |  | 74360 | 45290 | 51500 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 108500 |  | 120570 | 36500 | 39780 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 84500 |  | 93640 | 60500 | 66710 |

Based on Table 6.13.3-2;

-	3rd order IMD may fall into Rx frequencies of bands 1, 4, 7, 10, 23, 30, 34, 40, 41, 65, 66, 69, n257, n258 and n259.

When 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.13.3-3 and 6.X.3-4 list if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.13.3-3: 2UL Band 3 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | Harmonic3 |
|  | 5150 | - | 5350 | Yes | Europe | Harmonic3 |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | Harmonic3 |

Table 6.13.3-4: 2UL Band 7 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD3 |
|  | 2400 | - | 2494 | Yes | Asia | IMD3 |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.13.3-5 and 6.13.3-6 list the protected bands required for the dual connectivity configuration.

Table 6.13.3-5: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_3A-n257A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 65, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, n77, n78 | FDL_low | - | FDL_high | -50 | 1 | Harmonic exception |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |

Table 6.13.3-6: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_7A_n257A | E-UTRA Band 1, 3, 5, 7, 8, 26, 28, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 |  |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 |  |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 |  |

### 6.13.4	∆TIB and ∆RIB values

For DC_3A-7A-n257A, the TIB,c and RIB values are given in the tables below.

Table 6.13.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-7A-n257A | 3 | 0 |
|  | 7 |  |
|  | n257 |  |

Table 6.13.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-7A-n257A | 3 |  |
|  | 7 |  |
|  | n257 |  |

### 6.13.5	MSD

No IMD problems are expected at both LTE band and NR band at FR2. Hence, not necessary for study on IMD problems for LTE-NR DC UE between sub6GHz and FR2. And up to 7th order harmonics are not impact to Rx frequency of both LTE and NR bands.

## 6.14	DC_5A-7A-n257A

### 6.14.1	Operating bands for DC

Table 6.14.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5A-7A-n257A | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 GHz | – | 29500 MHz | TDD |

### 6.14.2	Channel bandwidths per operating band for DC

Table 6.14.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_5A-7A-n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  | 430 |
|  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |

### 6.14.3	Co-existence studies

Tables 6.14.3-1 and 6.14.3-2 list Band 5A-7A + Band n257A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.14.3-1: Band 5 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 |  | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 |  | 1698 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 |  | 2547 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 25651 | 28676 |  | 27324 | 30349 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 27852 |  | 24802 | 52151 | 58176 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 28148 |  | 31198 | 53824 | 59849 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 424 |  | 1249 | 26490 | 29510 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 27028 | 23953 |  | 78651 | 87676 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 57352 |  | 51302 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 28972 |  | 32047 | 80324 | 89349 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 54648 |  | 60698 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 117176 |  | 105151 | 23104 | 26204 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 86852 |  | 77802 | 50453 | 56528 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 106824 |  | 118849 | 29796 | 32896 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 81148 |  | 90198 | 55472 | 61547 |

Based on Table 6.14.3-1;

-	3rd order harmonic may fall into Rx frequencies of band 41

-	2nd order IMD may also fall into own Rx of band n257 and Rx frequencies of band n258

-	3rd order IMD may also fall into own Rx of bands 5 and n257 and Rx frequencies of bands 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 31, 44, 67, 68 and n258

-	4th order IMD may also fall into own Rx of band n257 and Rx frequencies of bands n258 and n259

-	5th order IMD may also fall into Rx frequencies of bands n258 and n259

Table 6.14.3-2: Band 7 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 |  | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 |  | 5140 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 |  | 7710 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 23930 | 27000 |  | 29000 | 32070 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 21360 |  | 24500 | 50430 | 56500 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 31500 |  | 34640 | 55500 | 61570 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 2100 |  | 2970 | 26480 | 29520 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 22000 | 18790 |  | 76930 | 86000 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 54000 |  | 47860 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 34000 |  | 37210 | 82000 | 91070 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 58000 |  | 64160 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 115500 |  | 103430 | 16220 | 19500 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 83500 |  | 74360 | 45290 | 51500 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 108500 |  | 120570 | 36500 | 39780 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 84500 |  | 93640 | 60500 | 66710 |

Based on Table 6.14.3-2;

-	3rd order IMD may fall into Rx frequencies of bands 1, 4, 7, 10, 23, 30, 34, 40, 41, 65, 66, 69, n257, n258 and n259.

When 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.14.3-3 and 6.14.3-4 list if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.14.3-3: 2UL Band 5 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.14.3-4: 2UL Band 7 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD3 |
|  | 2400 | - | 2494 | Yes | Asia | IMD3 |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.14.3-5 and 6.14.3-6 list the protected bands required for the dual connectivity configuration.

Table 6.14.3-5: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_5A-n257A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 42, 65, n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |

Table 6.14.3-6: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) | Maximum Level (dBm) | MBW (MHz) | NOTE |

### 6.14.4	∆TIB and ∆RIB values

For DC_5A-7A-n257A, the TIB,c and RIB values are given in the tables below.

Table 6.14.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5A-7A-n257A | 5 | 0 |
|  | 7 | 0 |
|  | n257 | 0 |

Table 6.14.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5A-7A-n257A | 5 | 0 |
|  | 7 | 0 |
|  | n257 | 0 |

### 6.14.5	MSD

No IMD problems are expected at both LTE band and NR band at FR2. Hence, not necessary for study on IMD problems for LTE-NR DC UE between sub6GHz and FR2. And up to 7th order harmonics are not impact to Rx frequency of both LTE and NR bands.

## 6.15	DC_7A-7A-n257A

### 6.15.1	Operating bands for DC

Table 6.15.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7A-7A-n257A | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.15.2	Channel bandwidths per operating band for DC

Table 6.15.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A-7A-n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 440 |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |

### 6.15.3	Co-existence studies

Table 6.15.3-1 lists Band 7A-7A +Band n257 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.15.3-1: Band 7 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 |  | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 |  | 5140 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 |  | 7710 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 23930 | 27000 |  | 29000 | 32070 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 21360 |  | 24500 | 50430 | 56500 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 31500 |  | 34640 | 55500 | 61570 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 2100 |  | 2970 | 26480 | 29520 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 22000 | 18790 |  | 76930 | 86000 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 54000 |  | 47860 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 34000 |  | 37210 | 82000 | 91070 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 58000 |  | 64160 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 115500 |  | 103430 | 16220 | 19500 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 83500 |  | 74360 | 45290 | 51500 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 108500 |  | 120570 | 36500 | 39780 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 84500 |  | 93640 | 60500 | 66710 |

Based on Table 6.15.3-1;

-	3rd order IMD may fall into Rx frequencies of bands 1, 4, 7, 10, 23, 30, 34, 40, 41, 65, 66, 69, n257, n258 and n259.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.15.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.15.3-2: 2UL Band 7 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD3 |
|  | 2400 | - | 2494 | Yes | Asia | IMD3 |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.15.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.15.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_7A_n257A | E-UTRA Band 1, 3, 5, 7, 8, 26, 28, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 |  |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 |  |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 |  |

### 6.15.4	∆TIB and ∆RIB values

For DC_7A-n257A, the TIB,c and RIB values are given in the tables below.

Table 6.15.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-n257A | 7 | 0 |
|  | n257 | 0 |

Table 6.15.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7A-n257A | 7 | 0 |
|  | n257 | 0 |

### 6.15.5	MSD

No IMD problems are expected at both LTE band and NR band at FR2. Hence, not necessary for study on IMD problems for LTE-NR DC UE between sub6GHz and FR2. And up to 7th order harmonics are not impact to Rx frequency of both LTE and NR bands.

## 6.16	DC_3A-5A-n78A

### 6.16.1	Operating bands for DC

Table 6.16.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-5A_n78A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.16.2	Channel bandwidths per operating band for DC

Table 6.16.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| DC_3A-5A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 130 | 0 |
|  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |

### 6.16.3	Co-existence studies

Tables 6.16.3-1 and 6.16.3-2 list Band 3A-5A + Band n78A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.16.3-1: Band 3 and Band 78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 |  | 3300 | 3800 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 3420 – 3570 |  |  | 6600 – 7600 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 5130 – 5355 |  |  | 9900 – 11400 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1515 – 2090 |  |  | 5010 – 5585 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 270 – 380 |  |  | 4815 – 5890 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 6720 – 7370 |  |  | 8310 – 9385 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 1610 – 1885 |  |  | 3280 – 3820 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1330 – 2055 |  |  | 8115 – 9690 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 3030 – 4180 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8430 – 9155 |  |  | 11610 – 13185 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 10020 – 11170 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11415 – 13490 |  |  | 3040 – 3840 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6330 – 7980 |  |  | 1245 – 2470 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 – 16985 |  |  | 10140 – 10940 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 – 14970 |  |  | 11730 – 12955 |  |  |

Based on Table 6.16.3-1,

-	2nd order harmonics may fall into Rx frequencies of Band 22, 42, 48, n77 and n78

-	3rd order harmonics may fall into Rx frequencies of Band 46

-	2nd order IMD may fall into Rx frequencies of Band 2, 3, 9, 24, 25, 33, 34, 35, 36, 37, 39, 46, and 70

-	3rd order IMD may fall into Rx frequencies of Band 3, 9, 22, 35, 39, 42, 43, 46, 47, 48, n77 and n78

-	4th order IMD may fall into Rx frequencies of Band 2, 3, 9, 11, 21, 22, 24, 25, 32, 33, 34, 35, 36, 37, 39, 42, 43, 45, 48, 70, n77 and n78

-	and 5th order IMD may fall into Rx frequencies of Band 1, 2, 3, 4, 9, 10, 11, 21, 22, 23, 24, 25,30, 32, 33, 34, 35, 36, 37, 39, 40, 42, 43, 45, 48, 70, n77 and n78

-	It should be noted that IMD will not be an issue for Band n78 (no self-interference for the TDD band)  even through the IMD products may fall into the concerning band

Table 6.16.3-2: Band 5 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 |  | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 |  | 1698 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 |  | 2547 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low |  | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3296 |  | 3396 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low |  | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4120 |  | 4245 | 16500 | 19000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2451 | 2976 |  | 4124 | 4649 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2152 |  | 1602 | 5751 | 6776 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4948 |  | 5498 | 7424 | 8449 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 724 |  | 949 | 3290 | 3810 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1328 | 753 |  | 9051 | 10576 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 5952 |  | 4902 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5772 |  | 6347 | 10724 | 12249 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8248 |  | 9298 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14376 |  | 12351 | 96 | 504 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9752 |  | 8202 | 4053 | 5128 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14024 |  | 16049 | 6596 | 7196 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11548 |  | 13098 | 9072 | 10147 |

Based on Table 6.16.3-2,

-	3rd order harmonic may fall into Rx frequencies of band 41

-	4th order harmonic may fall into Rx frequencies of band n78

-	2nd order IMD may also fall into own Rx of band n78 and Rx frequencies of bands 7, 38, 41, 69 and n79

-	3rd order IMD may also fall into own Rx of bands 5 and n78 and Rx frequencies of bands 1, 2, 3, 4, 6, 8, 9, 10, 12, 13, 14, 17, 18, 19, 20, 22, 25, 26, 27, 28, 29, 33, 34, 35, 36, 37, 39, 42, 43, 44, 46, 47, 48, 65, 66, 67, 68, 70, n77 and n79

-	4th order IMD may also fall into own Rx of band 5 and Rx frequencies of bands 6, 8, 13, 14, 18, 19, 20, 26, 27, 28, 44, 46, 47, 67, 68 and n79

-	5th order IMD may also fall into Rx frequencies of bands 31, n77 and n79

When 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS; the harmonics and intermodulation products can have impact on these systems. Table 6.16.3-3 and 6.16.3-4 list if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.16.3-3: 2UL Band 3 + Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD2, 4, 5 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD2, 4, 5 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD2, 4, 5 |
| GPS | 1563 | - | 1587 | Yes |  | IMD2, 4, 5 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD5 |
|  | 2400 | - | 2494 | Yes | Asia | IMD5 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | Harmonics3, IMD2, 3 |
|  | 5150 | - | 5350 | Yes | Europe | Harmonics3, IMD2, 3 |
|  | 5470 | - | 5725 | Yes |  | IMD2, 3 |
|  | 5150 | - | 5825 | Yes | Asia | Harmonics3, IMD2, 3 |

Table 6.16.3-4: 2UL Band 5 + Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD3 |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD2 |
|  | 2400 | - | 2494 | Yes | Asia | IMD2 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3, 4 |
|  | 5150 | - | 5350 | Yes | Europe | IMD3, 4 |
|  | 5470 | - | 5725 | Yes |  | IMD3, 4 |
|  | 5150 | - | 5825 | Yes | Asia | IMD3 |

Table 6.16.3-5 lists the protected bands required for the dual connectivity configuration.

Table 6.16.3-5: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_3A-5A_n78A | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 65, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 3800 | - | 3805 | +1.6 | 5 | 1, 2, 3 |
|  | Frequency range | 3805 | - | 3825 | -15.5 | 5 | 1, 2, 3 |
|  | Frequency range | 3825 | - | 3850 | -40 | 1 | 1, 3 |
|  | Frequency range | 3850 | - | 4200 | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 4, 5 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 6 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 | 5 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | NR Band n257 | 26500 | - | 29500 | [-25] | 1 |  |
| NOTE 1: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 2: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 3: The requirement is applicable for any channel bandwidths within the range 3300 - 3800 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range [TBD – 3792.5] MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range [TBD – 3790] MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to [TBD] RB. NOTE 4: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 5: Applicable only when the assigned E-UTRA carrier is confined within 824 MHz and 849 MHz for UE category M1, M2 and UE category NB1 and NB2. NOTE 6: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |

### 6.16.4	∆TIB and ∆RIB values

For DC_3A-5A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.16.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-5A-n78A | 3 | 0.6 |
|  | 5 | 0.6 |
|  | n78 | 0.8 |

Table 6.16.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-5A-n78A | 3 | 0.2 |
|  | 5 | 0.2 |
|  | n78 | 0.5 |

### 6.16.5	MSD

Table 6.16.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.16.5-1: MSD for the DC configuration for IMD

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_3A-5A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | 5 | 844 | 5 | 25 | 889 | 8.3 | FDD | IMD4 ¦fB78-3*fB5¦4 |
|  | n78 | 3421 | 10 | 52 | 3421 | N/A | TDD | N/A |
|  | 3 | 1740 | 5 | 25 | 1835 | [26] | FDD | IMD2 ¦fB78-fB3¦ |
|  |  |  |  |  |  | [28.75] |  |  |
|  | 5 | 840 | 5 | 25 | 885 | N/A | FDD | N/A |
|  | n78 | 3575 | 10 | 25 | 3575 | N/A | TDD | N/A |
|  | n78 | 3710 | 10 | 25 | 3710 | N/A | TDD | N/A |
|  | 3 | 1770 | 5 | 25 | 1865 | [8.0] | FDD | IMD4 ¦fB78-3*fB3¦4 |
|  |  |  |  |  |  | [10.75] |  |  |
|  | 5 | 840 | 5 | 25 | 885 | N/A | FDD | N/A |
|  | n78 | 3445 | 10 | 52 | 3445 | N/A | TDD | N/A |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in subclause 6.2.5A NOTE 2: RBSTART = 0 NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

MSD values for Band n78 due to 2nd harmonic of Band 3 in LTE-NR DC_3A_n78 are captured in Table 6.16.5-2.

Table 6.16.5-2: MSD due to harmonic issue for DC_3A_n78

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| 3 | n781,2 | 27.1 | 23.9 | 22.1 | 20.9 |  | 17.9 |  |  |  |  |
|  | n783 | 1.9 | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 2: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. |  |  |  |  |  |  |  |  |  |  |  |

Regarding the uplink configuration, 40MHz for n78 should be considered additionally. Therefore, 20MHz with 100RB allocation for Band 3 should be added.

Table 6.15.5-3 Uplink configuration for the low band (exceptions) for E-UTRA

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LTE-NR DC band combination | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 20 MHz | Duplex mode |
| DC_3A_n78 | 3 |  |  | 12 | 26 | 39 | 531 | 1002 | FDD |
| NOTE 1: The configuration is used for measurement of MSD for NR channel bandwidth of 20MHz. NOTE 2: The configuration is used for measurement of MSD for NR channel bandwidth of 40MHz. |  |  |  |  |  |  |  |  |  |

## 6.17	DC_7A-7A-n78A

### 6.17.1	Operating bands for DC

Table 6.17.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7A-7A_n78A | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.17.2	Channel bandwidths per operating band for DC

Table 6.17.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A-7A_n78A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.17.3	Co-existence studies

Tables 6.17.3-1 lists Band 7A-7A + Band n78A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.17.3-1: Band 7 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 9900 | 11400 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 730 | 1300 | 5800 | 6370 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1200 | 1840 | 4030 | 5100 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8300 | 8940 | 9100 | 10170 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3700 | 4410 | 7330 | 8900 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10800 | 11510 | 12400 | 13970 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2600 | 1460 | 11600 | 12740 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12700 | 10630 | 6980 | 6200 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15700 | 17770 | 13300 | 14080 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6400 | 4760 | 1110 | 100 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14900 | 16540 | 14100 | 15310 |

Based on Table 6.17.3-1;

-	There was no harmonic issues at legacy E-UTRA bands and NR new band 3.3~4.2GHz and 4.4~5GHz

-	2nd order IMD may also fall into B5, B8, B12, B13, B14, B17, B18, B19, B20, B26, B27, B28, B67, B68 and TDD B44, B46 and B47.

-	3rd order IMD may also fall into B3, B11, B21, B24, B32 and TDD B45, n77

-	4th order IMD may also fall into B1, B2, B3, B4, B9, B10, B11, B21, B23, B24, B25, B30, B32, B65, B66, TDD B33, B34, B35, B36, B37, B38, B39, B40, B41, B43, B45, n77, n78, n79

-	5th order IMD may also fall into B5, B8, B12, B13, B14, B17, B18, B19, B20, B26, B27, B28, B29, B31, B67, B68, TDD B44, B46, B47 and n79.

The 4th IMD will be impact to the own received band n78. However, the NR TDD band is not any impacts from dual uplink transmission since there was no simultaneous Tx/Rx transmission in the n78 NR band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.17.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.17.3-2: 2UL Band 7 +Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | 3rd IMD |
| Galileo | 1559 | - | 1591 | Yes |  | 3rd IMD |
| GLONASS | 1591 | - | 1610 | Yes |  | 3rd IMD |
| GPS | 1563 | - | 1587 | Yes |  | 3rd IMD |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | 4th IMD |
|  | 2400 | - | 2494 | Yes | Asia | 4th IMD |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd & 5th IMD |
|  | 5150 | - | 5350 | Yes | Europe | 5th IMD |
|  | 5470 | - | 5725 | Yes |  | 5th IMD |
|  | 5150 | - | 5825 | Yes | Asia | 2nd & 5th IMD |

Table 6.17.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.17.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_7A-n78A | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 40, 50, 51, 65, 66, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 1,2,3 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 1,2,3 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 1,2 |
|  | Frequency range | 3800 | - | 3805 | +1.6 | 5 | 1,3,4 |
|  | Frequency range | 3805 | - | 3825 | -15.5 | 5 | 1,3,4 |
|  | Frequency range | 3825 | - | 3850 | -40 | 1 | 1,4 |
|  | Frequency range | 3850 | - | 4200 | -50 | 1 |  |
| NOTE 1: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 2: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 3: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 4: This requirement is applicable for any channel bandwidths within the range 3300 - 3800 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range [TBD – 3792.5] MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range [TBD - 3790] MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to [TBD] RB. |  |  |  |  |  |  |  |

### 6.17.4	∆TIB and ∆RIB values

For DC_7A-7A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.17.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-7A-n78A | 7 | 0.5 |
|  | n78 | 0.8 |

Table 6.17.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7A-7A-n78A | 7 | 0.0 |
|  | n78 | 0.5 |

### 6.17.5	MSD

From the coexistence analysis 6.17.3, there was no harmonic/IMD problems into the own RX frequency bands for DC_7A-n78A UE.

## 6.18	DC_1A-3A-n78

### 6.18.1	Operating bands for DC

Table 6.18.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3A-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.18.2	Channel bandwidths per operating band for DC

Table 6.18.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A-n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-3A-n78C | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 | See CA_n78C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.18.3	Co-existence studies

Based on co-existence studies of Band 1 + Band n78 and Band 3 + Band n78 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	2nd, 3rd and 5th order IMD generated by dual uplink of Band 1 + Band n78 may also fall into own Rx of band 3

-	5th order IMD generated by dual uplink of Band 3 + Band n78 may also fall into own Rx of band 1

### 6.18.4	∆TIB and ∆RIB values

For DC_1A-3A-n78, the TIB,c and RIB values are given in the tables below.

Table 6.18.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-3A-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n78 | 0.8 |

Table 6.18.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-3A-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n78 | 0.5 |

### 6.18.5	MSD

Table 6.18.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.18.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_1A-3A-n78 | 1 | TBD | TBD | TBD | TBD | TBD | FDD | TBD |
|  | 3 | TBD | TBD | TBD | TBD | TBD |  | TBD |
|  | n78 | TBD | TBD | TBD | TBD | TBD | TDD | TBD |

## 6.19	DC_1A-19A-n78A

### 6.19.1	Operating bands for DC

Table 6.19.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-19A-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.19.2	Channel bandwidths per operating band for DC

Table 6.19.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-19A-n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 135 |
|  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-19A-n78C | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 235 |
|  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 | See CA_n78C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.19.3	Co-existence studies

Based on co-existence studies of Band 1 + Band n78 and Band 19 + Band n78 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	5th order IMD generated by dual uplink of Band 1 + Band n78 may also fall into own Rx of band 19

-	3rd order IMD generated by dual uplink of Band 19 + Band n78 may also fall into own Rx of band 1

Taking into account that Band 19 is currently operated only by a certain operator in Japan, the frequency range can be limited as Band 1 UL/DL = 1940-1960/2130-2150 MHz and Band n78 UL/DL = 3600-3800/3600-3800 MHz. Then own Rx impact can be simplified as below.

-	5th order IMD generated by dual uplink of Band 1 + Band n78 doesn’t fall into own Rx.

-	3rd order IMD generated by dual uplink of Band 19 + Band n78 may also fall into own Rx of band 1

### 6.19.4	∆TIB and ∆RIB values

For DC_1A-19A-n78, the TIB,c and RIB values are given in the tables below.

Table 6.19.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-19A-n78 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | n78 | 0.8 |

Table 6.19.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-19A-n78 | 1 | 0 |
|  | 19 | 0 |
|  | n78 | 0.5 |

### 6.19.5	MSD

Table 6.19.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.19.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_1A-19A-n78 | 1 | 1940 | 5 | 25 | 2130 | [17.8] | FDD | IMD3 |
|  | 19 | 832.5 | 5 | 25 | 877.5 | N/A |  | N/A |
|  | n78 | 3795 | 10 | 52 | 3795 | N/A | TDD | N/A |

## 6.20	DC_1A-21A-n78

### 6.20.1	Operating bands for DC

Table 6.20.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-21A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.20.2	Channel bandwidths per operating band for DC

Table 6.20.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-21A-n78 | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 135 |
|  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-21A-n78C | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 235 |
|  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 | See CA_n78C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.20.3	Co-existence studies

Based on co-existence studies of Band 1 + Band n78 and Band 21 + Band n78 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	2nd and 5th order IMD generated by dual uplink of Band 1 + Band n78 may also fall into own Rx of band 21

-	2nd and 5th  order IMD generated by dual uplink of Band 21 + Band n78 may also fall into own Rx of band 1

Taking into account that Band 21 is currently operated only by a certain operator in Japan, the frequency range can be limited as Band 1 UL/DL = 1940-1960/2130-2150 MHz and Band n78 UL/DL = 3600-3800/3600-3800 MHz. Then own Rx impact can be simplified as below.

-	5th order IMD generated by dual uplink of Band 1 + Band n78 may also fall into own Rx of band 21

-	2nd and 5th  order IMD generated by dual uplink of Band 21 + Band n78 may also fall into own Rx of band 1

### 6.20.4	∆TIB and ∆RIB values

For DC_1A-21A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.20.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-21A-n78 | 1 | 0.6 |
|  | 21 | 0.4 |
|  | n78 | 0.8 |

Table 6.20.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-21A-n78 | 1 | 0.2 |
|  | 21 | 0 |
|  | n78 | 0.5 |

### 6.20.5	MSD

Table 6.20.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.20.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_1A-21A-n78 | 1 | 1964.6 | 5 | 25 | 2154.6 | [30.6] | FDD | IMD2 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A |  | N/A |
|  | n78 | 3605 | 10 | 52 | 3605 | N/A | TDD | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | 21 | 1452 | 5 | 25 | 1500 | [2.9] |  | IMD5 |
|  | n78 | 3675 | 10 | 52 | 3675 | N/A | TDD | N/A |

## 6.21	DC_3A-19A-n78

### 6.21.1	Operating bands for DC

Table 6.21.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-19A-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.21.2	Channel bandwidths per operating band for DC

Table 6.21.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-19A-n78 | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 135 |
|  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-19A-n78C | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 235 |
|  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 | See CA_n78C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.21.3	Co-existence studies

Based on co-existence studies of Band 3 + Band n78 and Band 19 + Band n78 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	IMD generated by dual uplink of Band 3 + Band n78 doesn’t fall into own Rx of band 19

-	3rd order IMD generated by dual uplink of Band 19 + Band n78 may also fall into own Rx of band 3

Taking into account that Band 19 is currently operated only by a certain operator in Japan, the frequency range can be limited as Band n78 UL/DL = 3600-3800/3600-3800 MHz. Then own Rx impact can be simplified as below.

-	IMD generated by dual uplink of Band 3 + Band n78 doesn’t fall into own Rx of band 19

-	IMD generated by dual uplink of Band 19 + Band n78 doesn’t fall into own Rx of band 3

### 6.21.4	∆TIB and ∆RIB values

For DC_3A-19A-n78, the TIB,c and RIB values are given in the tables below.

Table 6.21.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-19A-n78 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | n78 | 0.8 |

Table 6.21.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-19A-n78 | 3 | 0.2 |
|  | 19 | 0 |
|  | n78 | 0.5 |

### 6.21.5	MSD

Table 6.21.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.21.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_3A-19A-n78 | 3 | TBD | TBD | TBD | TBD | TBD | FDD | TBD |
|  | 19 | TBD | TBD | TBD | TBD | TBD |  | TBD |
|  | n78 | TBD | TBD | TBD | TBD | TBD | TDD | TBD |

## 6.22	DC_3A-21A-n78

### 6.22.1	Operating bands for DC

Table 6.22.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-21A-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.22.2	Channel bandwidths per operating band for DC

Table 6.22.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-21A-n78 | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 135 |
|  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-21A-n78C | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 235 |
|  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 | See CA_n78C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.22.3	Co-existence studies

Based on co-existence studies of Band 3 + Band n78 and Band 21 + Band n78 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	4th and 5th order IMD generated by dual uplink of Band 3 + Band n78 may also fall into own Rx of band 21

-	2nd order IMD generated by dual uplink of Band 21 + Band n78 may also fall into own Rx of band 3

Taking into account that Band 21 is currently operated only by a certain operator in Japan, the frequency range can be limited as Band n78 UL/DL = 3600-3800/3600-3800 MHz. Then own Rx impact can be simplified as below.

-	4th order IMD generated by dual uplink of Band 3 + Band n78 may also fall into own Rx of band 21

-	IMD generated by dual uplink of Band 21 + Band n78 doesn’t fall into own Rx of band 3

### 6.22.4	∆TIB and ∆RIB values

For DC_3A-21A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.22.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-21A-n78 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n78 | 0.8 |

Table 6.22.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-21A-n78 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n78 | 0.5 |

### 6.22.5	MSD

Table 6.22.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.22.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_3A-21A-n78 | 3 | 1767.5 | 5 | 25 | 1862.5 | N/A | FDD | N/A |
|  | 21 | 1459.5 | 5 | 25 | 1507.5 | [8.8] |  | IMD4 |
|  | n78 | 3795 | 10 | 52 | 3795 | N/A | TDD | N/A |

## 6.23	DC_19A-21A-n78

### 6.23.1	Operating bands for DC

Table 6.23.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19A-21A-n78 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.23.2	Channel bandwidths per operating band for DC

Table 6.23.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_19A-21A-n78A | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 130 |
|  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_19A-21A-n78C | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 230 |
|  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 | See CA_n78C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.23.3	Co-existence studies

Based on co-existence studies of Band 19 + Band n78 and Band 21 + Band n78 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	IMD generated by dual uplink of Band 19 + Band n78 doesn’t fall into own Rx of band 21

-	3rd and 4th order IMD generated by dual uplink of Band 21 + Band n78 may also fall into own Rx of band 19

Taking into account that Band 19 and 21 are currently operated only by a certain operator in Japan, the frequency range can be limited as Band n78 UL/DL = 3600-3800/3600-3800 MHz. Then own Rx impact can be simplified as below.

-	3rd order IMD generated by dual uplink of Band 21 + Band n78 may also fall into own Rx of band 19

### 6.23.4	∆TIB and ∆RIB values

For DC_19A-21A-n78, the TIB,c and RIB values are given in the tables below.

Table 6.23.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19A-21A-n78 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | n78 | 0.8 |

Table 6.23.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19A-21A-n78 | 19 | 0 |
|  | 21 | 0 |
|  | n78 | 0.5 |

### 6.23.5	MSD

Table 6.23.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.23.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_19A-21A-n78 | 19 | 837.5 | 5 | 25 | 882.5 | [18.7] | FDD | IMD3 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A |  | N/A |
|  | n78 | 3783.3 | 10 | 52 | 3783.3 | N/A | TDD | N/A |

## 6.24	DC_1A-3A-n79

### 6.24.1	Operating bands for DC

Table 6.24.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3A-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz |  |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.24.2	Channel bandwidths per operating band for DC

Table 6.24.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A-n79 | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-3A-n79C | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 240 |
|  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n79 | See CA_n79C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.24.3	Co-existence studies

Based on co-existence studies of Band 1 + Band n79 and Band 3 + Band n79 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	3rd order IMD generated by dual uplink of Band 1 + Band n79 may also fall into own Rx of band 3

-	5th order IMD generated by dual uplink of Band 3 + Band n79 may also fall into own Rx of band 1

### 6.24.4	∆TIB and ∆RIB values

For DC_1A-3A-n79, the TIB,c and RIB values are given in the tables below.

Table 6.24.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-3A-n79 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | n79 | 0 |

Table 6.24.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-3A-n79 | 1 | 0 |
|  | 3 | 0 |
|  | n79 | 0 |

### 6.24.5	MSD

Table 6.24.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.24.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_1A-3A-n79 | 1 | 1950 | 5 | 25 | 2140 | [3.6] | FDD | IMD5 |
|  | 3 | 1750 | 5 | 25 | 1845 | N/A |  | N/A |
|  | n79 | 4860 | 40 | 216 | 4860 | N/A | TDD | N/A |

## 6.25	DC_1A-19A-n79

### 6.25.1	Operating bands for DC

Table 6.25.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-19A-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz |  |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.25.2	Channel bandwidths per operating band for DC

Table 6.25.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-19A-n79 | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 135 |
|  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-19A-n79C | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 235 |
|  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n79 | See CA_n79C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.25.3	Co-existence studies

Based on co-existence studies of Band 1 + Band n79 and Band 19 + Band n79 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	3rd and 4th order IMD generated by dual uplink of Band 1 + Band n79 may also fall into own Rx of band 19

-	4th order IMD generated by dual uplink of Band 19 + Band n79 may also fall into own Rx of band 1

Taking into account that Band 19 is currently operated only by a certain operator in Japan, the frequency range can be limited as Band 1 UL/DL = 1940-1960/2130-2150 MHz and Band n79 UL/DL = 4400-4900/4400-4900 MHz. Then own Rx impact can be simplified as below.

-	3rd order IMD generated by dual uplink of Band 1 + Band n79 may also fall into own Rx of band 19

-	4th order IMD generated by dual uplink of Band 19 + Band n79 may also fall into own Rx of band 1

### 6.25.4	∆TIB and ∆RIB values

For DC_1A-19A-n79, the TIB,c and RIB values are given in the tables below.

Table 6.25.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-19A-n79 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | n79 | 0 |

Table 6.25.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-19A-n79A | 1 | 0.3 |
|  | 19 | 0.3 |
|  | n79 | 0 |

### 6.25.5	MSD

Table 6.25.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.25.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_1A-19A-n79 | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | 19 | 837.5 | 5 | 25 | 882.5 | [18.3] |  | IMD3 |
|  | n79 | 4782.5 | 40 | 216 | 4782.5 | N/A | TDD | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | [8.1] | FDD | IMD4 |
|  | 19 | 837.5 | 5 | 25 | 882.5 | N/A |  | N/A |
|  | n79 | 4652.5 | 40 | 216 | 4652.5 | N/A | TDD | N/A |

## 6.26	DC_1A-21A-n79

### 6.26.1	Operating bands for DC

Table 6.26.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-21A-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz |  |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.26.2	Channel bandwidths per operating band for DC

Table 6.26.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-21A-n79 | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 135 |
|  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-21A-n79C | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 235 |
|  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n79 | See CA_n79C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.26.3	Co-existence studies

Based on co-existence studies of Band 1 + Band n79 and Band 21 + Band n79 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	4th order IMD generated by dual uplink of Band 1 + Band n79 may also fall into own Rx of band 21

-	IMD generated by dual uplink of Band 21 + Band n79 doesn’t fall into own Rx of band 1

Taking into account that Band 21 is currently operated only by a certain operator in Japan, the frequency range can be limited as Band 1 UL/DL = 1940-1960/2130-2150 MHz and Band n79 UL/DL = 4400-4900/4400-4900 MHz. Then own Rx impact can be simplified as below.

-	IMD generated by dual uplink of Band 1 + Band n79 doesn’t fall into own Rx of band 21

-	IMD generated by dual uplink of Band 21 + Band n79 doesn’t fall into own Rx of band 1

### 6.26.4	∆TIB and ∆RIB values

For DC_1A-21A-n79, the TIB,c and RIB values are given in the tables below.

Table 6.26.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-21A-n79 | 1 | 0.3 |
|  | 21 | 0.3 |
|  | n79 | 0 |

Table 6.26.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-21A-n79 | 1 | 0 |
|  | 21 | 0 |
|  | n79 | 0 |

### 6.26.5	MSD

Table 6.26.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.26.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_1A-21A-n79 | 1 | TBD | TBD | TBD | TBD | TBD | FDD | TBD |
|  | 21 | TBD | TBD | TBD | TBD | TBD |  | TBD |
|  | n79 | TBD | TBD | TBD | TBD | TBD | TDD | TBD |

## 6.27	DC_3A-19A-n79

### 6.27.1	Operating bands for DC

Table 6.27.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-19A-n79 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz |  |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.27.2	Channel bandwidths per operating band for DC

Table 6.27.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-19A-n79A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 135 |
|  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-19A-n79C | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 235 |
|  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n79 | See CA_n79C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.27.3	Co-existence studies

Based on co-existence studies of Band 3 + Band n79 and Band 19 + Band n79 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	3rd and 4th order IMD generated by dual uplink of Band 3 + Band n79 may also fall into own Rx of band 19

-	4th order IMD generated by dual uplink of Band 19 + Band n79 may also fall into own Rx of band 3

Taking into account that Band 19 is currently operated only by a certain operator in Japan, the frequency range can be limited as Band n79 UL/DL = 4400-4900/4400-4900 MHz. However, own Rx impact cannot be simplified in this case.

### 6.27.4	∆TIB and ∆RIB values

For DC_3A-19A-n79, the TIB,c and RIB values are given in the tables below.

Table 6.27.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-19A-n79 | 3 | 0.3 |
|  | 19 | 0.3 |
|  | n79 | 0 |

Table 6.27.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-19A-n79 | 3 | 0 |
|  | 19 | 0 |
|  | n79 | 0 |

### 6.27.5	MSD

Table 6.27.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.27.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_3A-19A-n79 | 3 | 1775 | 5 | 25 | 1870 | N/A | FDD | N/A |
|  | 19 | 840 | 5 | 25 | 885 | [18.5] |  | IMD3 |
|  | n79 | 4435 | 40 | 216 | 4435 | N/A | TDD | N/A |
|  | 3 | 1782.5 | 5 | 25 | 1877.5 | [0.2] | FDD | IMD4 |
|  | 19 | 842.5 | 5 | 25 | 887.5 | N/A |  | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |

## 6.28	DC_3A-21A-n79

### 6.28.1	Operating bands for DC

Table 6.28.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-21A-n79 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz |  |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.28.2	Channel bandwidths per operating band for DC

Table 6.28.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-21A-n79A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 135 |
|  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-21A-n79C | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 235 |
|  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n79 | See CA_n79C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.28.3	Co-existence studies

Based on co-existence studies of Band 3 + Band n79 and Band 21 + Band n79 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	3rd  order IMD generated by dual uplink of Band 3 + Band n79 may also fall into own Rx of band 21

-	3rd order IMD generated by dual uplink of Band 21 + Band n79 may also fall into own Rx of band 3

Taking into account that Band 21 is currently operated only by a certain operator in Japan, the frequency range can be limited as Band n79 UL/DL = 4400-4900/4400-4900 MHz. Then own Rx impact can be simplified as below.

-	IMD generated by dual uplink of Band 3 + Band n79 does not fall into own Rx of band 21

-	3rd order IMD generated by dual uplink of Band 21 + Band n79 may also fall into own Rx of band 3

### 6.28.4	∆TIB and ∆RIB values

For DC_3A-21A-n79, the TIB,c and RIB values are given in the tables below.

Table 6.28.4-2: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-21A-n79 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n79 | 0 |

Table 6.28.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-21A-n79 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n79 | 0 |

### 6.28.5	MSD

Table 6.28.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.28.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_3A-21A-n79 | 3 | 1774.2 | 5 | 25 | 1869.2 | [17.8] | FDD | IMD3 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A |  | N/A |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | TDD | N/A |

## 6.29	DC_19A-21A-n79

### 6.29.1	Operating bands for DC

Table 6.29.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19A-21A-n79 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz |  |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.29.2	Channel bandwidths per operating band for DC

Table 6.29.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_19A-21A-n79A | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 130 |
|  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
| DC_19A-21A-n79C | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 230 |
|  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n79 | See CA_n79C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.29.3	Co-existence studies

Based on co-existence studies of Band 19 + Band n79 and Band 21 + Band n79 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	5th order IMD generated by dual uplink of Band 19 + Band n79 may also fall into own Rx of band 21

-	5th order IMD generated by dual uplink of Band 21 + Band n79 may also fall into own Rx of band 19

Taking into account that Band 19 and 21 are currently operated only by a certain operator in Japan, the frequency range can be limited as Band n79 UL/DL = 4400-4900/4400-4900 MHz. Then own Rx impact can be simplified as below.

-	5th order IMD generated by dual uplink of Band 19 + Band n79 may also fall into own Rx of band 21

-	IMD generated by dual uplink of Band 21 + Band n79 doesn’t fall into own Rx of band 19

### 6.29.4	∆TIB and ∆RIB values

For DC_19A-21A-n79, the TIB,c and RIB values are given in the tables below.

Table 6.29.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19A-21A-n79 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | n79 | 0 |

Table 6.29.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19A-21A-n79 | 19 | 0 |
|  | 21 | 0 |
|  | n79 | 0 |

### 6.29.5	MSD

Table 6.29.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.29.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_19A-21A-n79 | 19 | 837.5 | 5 | 25 | 882.2 | N/A | FDD | N/A |
|  | 21 | 1452 | 5 | 25 | 1500 | [3.8] |  | IMD5 |
|  | n79 | 4850 | 40 | 216 | 4850 | N/A | TDD | N/A |

## 6.30	DC_1A-3A-n257

### 6.30.1	Operating bands for DC

Table 6.30.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3A-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz |  |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.30.2	Channel bandwidths per operating band for DC

Table 6.30.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-3A-n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 440 |
|  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |
| DC_1A-3A-n257D | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 440 |
|  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-n257E | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 640 |
|  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-n257F | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 840 |
|  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |

### 6.30.3	Co-existence studies

Based on co-existence studies of Band 1 + Band n257 and Band 3 + Band n257 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	3rd order IMD generated by dual uplink of Band 1 + Band n257 may also fall into own Rx of band 3

-	3rd order IMD generated by dual uplink of Band 3 + Band n257 may also fall into own Rx of band 1

### 6.30.4	∆TIB and ∆RIB values

For DC_1A-3A-n257, the TIB,c and RIB values are given in the tables below.

Table 6.30.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-3A-n257 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | n257 | 0 |

Table 6.30.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-3A-n257 | 1 | 0 |
|  | 3 | 0 |
|  | n257 | 0 |

### 6.30.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.31	DC_1A-19A-n257

### 6.31.1	Operating bands for DC

Table 6.31.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-19A-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz |  |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.31.2	Channel bandwidths per operating band for DC

Table 6.31.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-19A-n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 435 |
|  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |
| DC_1A-19A-n257D | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 435 |
|  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_1A-19A-n257E | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 635 |
|  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_1A-19A-n257F | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 835 |
|  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |

### 6.31.3	Co-existence studies

Based on co-existence studies of Band 1 + Band n257 and Band 19 + Band n257 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	IMD generated by dual uplink of Band 1 + Band n257 doesn’t fall into own Rx of band 19

-	IMD generated by dual uplink of Band 19 + Band n257 doesn’t fall into own Rx of band 1

### 6.31.4	∆TIB and ∆RIB values

For DC_1A-19A-n257, the TIB,c and RIB values are given in the tables below.

Table 6.31.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-19A-n257 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | n257 | 0 |

Table 6.31.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-19A-n257 | 1 | 0 |
|  | 19 | 0 |
|  | n257 | 0 |

### 6.31.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.32	DC_1A-21A-n257

### 6.32.1	Operating bands for DC

Table 6.32.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-21A-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz |  |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.32.2	Channel bandwidths per operating band for DC

Table 6.32.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-21A-n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 435 |
|  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |
| DC_1A-21A-n257D | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 435 |
|  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_1A-21A-n257E | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 635 |
|  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_1A-21A-n257F | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 835 |
|  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |

### 6.32.3	Co-existence studies

Based on co-existence studies of Band 1 + Band n257 and Band 21 + Band n257 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	IMD generated by dual uplink of Band 1 + Band n257 doesn’t fall into own Rx of band 21

-	IMD generated by dual uplink of Band 21 + Band n257 doesn’t fall into own Rx of band 1

### 6.32.4	∆TIB and ∆RIB values

For DC_1A-21A-n257, the TIB,c and RIB values are given in the tables below.

Table 6.32.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-21A-n257 | 1 | 0.3 |
|  | 21 | 0.3 |
|  | n257 | 0.0 |

Table 6.32.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-21A-n257 | 1 | 0.0 |
|  | 21 | 0.0 |
|  | n257 | 0.0 |

### 6.32.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.33	DC_3A-19A-n257

### 6.33.1	Operating bands for DC

Table 6.33.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-19A-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz |  |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.33.2	Channel bandwidths per operating band for DC

Table 6.X.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-19A-n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 435 |
|  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |
| DC_3A-19A-n257D | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 435 |
|  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_3A-19A-n257E | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 635 |
|  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_3A-19A-n257F | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 835 |
|  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |

### 6.33.3	Co-existence studies

Based on co-existence studies of Band 3 + Band n257 and Band 19 + Band n257 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	IMD generated by dual uplink of Band 3 + Band n257 doesn’t fall into own Rx of band 19

-	IMD generated by dual uplink of Band 19 + Band n257 doesn’t fall into own Rx of band 3

### 6.33.4	∆TIB and ∆RIB values

For DC_3A-19A-n257, the TIB,c and RIB values are given in the tables below.

Table 6.33.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-19A-n257 | 3 | 0.3 |
|  | 19 | 0.3 |
|  | n257 | 0 |

Table 6.33.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-19A-n257 | 3 | 0 |
|  | 19 | 0 |
|  | n257 | 0 |

### 6.33.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.34	DC_3A-21A-n257

### 6.34.1	Operating bands for DC

Table 6.34.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-21A-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz |  |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.34.2	Channel bandwidths per operating band for DC

Table 6.34.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-21A-n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 435 |
|  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |
| DC_3A-21A-n257D | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 435 |
|  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_3A-21A-n257E | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 635 |
|  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_3A-21A-n257F | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 835 |
|  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |

### 6.34.3	Co-existence studies

Based on co-existence studies of Band 3 + Band n257 and Band 21 + Band n257 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	3rd  order IMD generated by dual uplink of Band 3 + Band n257 may also fall into own Rx of band 21

-	3rd order IMD generated by dual uplink of Band 21 + Band n257 may also fall into own Rx of band 3

### 6.34.4	∆TIB and ∆RIB values

For DC_3A-21A-n257, the TIB,c and RIB values are given in the tables below.

Table 6.34.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-21A-n257 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n257 | 0 |

Table 6.34.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-21A-n257 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n257 | 0 |

### 6.34.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.35	DC_1A-3A-n77

### 6.35.1	Operating bands for DC

Table 6.35.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3A-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz |  |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.35.2	Channel bandwidths per operating band for DC

Table 6.35.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A-n77 | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A-n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-3A-n77C | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 240 |
|  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n77 | See CA_n77C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.35.3	Co-existence studies

Based on co-existence studies of Band 1 + Band n77 and Band 3 + Band n77 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	2nd, 3rd, 4th and 5th order IMD generated by dual uplink of Band 1 + Band n77 may also fall into own Rx of band 3

-	2nd and 5th order IMD generated by dual uplink of Band 3 + Band n77 may also fall into own Rx of band 1

### 6.35.4	∆TIB and ∆RIB values

For DC_1A-3A-n77A, the TIB,c and RIB values are given in the tables below.

Table 6.35.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-3A-n77A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n77 | 0.8 |

Table 6.35.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-3A-n77A | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n77 | 0.5 |

### 6.35.5	MSD

Table 6.35.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.36.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_1A-3A-n77 | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | [31.5] |  | IMD2 |
|  | n77 | 3757.5 | 10 | 52 | 3757.5 | N/A | TDD | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | 3 | 1775 | 5 | 25 | 1870 | [8.5] |  | IMD4 |
|  | n77 | 3980 | 10 | 52 | 3980 | N/A | TDD | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | [31.0] | FDD | IMD2 |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A |  | N/A |
|  | n77 | 3915 | 10 | 52 | 3915 | N/A | TDD | N/A |

## 6.36	(Void)

## 6.37	DC_1A-19A-n77

### 6.37.1	Operating bands for DC

Table 6.37.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-19A-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz |  |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.37.2	Channel bandwidths per operating band for DC

Table 6.37.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-19A-n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 135 |
|  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-19A-n77C | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 235 |
|  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n77 | See CA_n77C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.37.3	Co-existence studies

Based on co-existence studies of Band 1 + Band n77 and Band 19 + Band n77 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	5th order IMD generated by dual uplink of Band 1 + Band n77 may also fall into own Rx of band 19

-	3rd order IMD generated by dual uplink of Band 19 + Band n77 may also fall into own Rx of band 1

Taking into account that Band 19 is currently operated only by a certain operator in Japan, the frequency range can be limited as Band 1 UL/DL = 1940-1960/2130-2150 MHz and Band n77 UL/DL = 3600-4200/3600-4200 MHz. Then own Rx impact can be simplified as below.

-	5th order IMD generated by dual uplink of Band 1 + Band n77 doesn’t fall into own Rx.

-	3rd order IMD generated by dual uplink of Band 19 + Band n77 may also fall into own Rx of band 1

### 6.37.4	∆TIB and ∆RIB values

For DC_1A-19A-n77, the TIB,c and RIB values are given in the tables below.

Table 6.37.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-19A-n77 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | n77 | 0.8 |

Table 6.37.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-19A-n77 | 1 | 0 |
|  | 19 | 0 |
|  | n77 | 0.5 |

### 6.37.5	MSD

Table 6.37.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.37.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_1A-19A-n77 | 1 | 1940 | 5 | 25 | 2130 | [17.8] | FDD | IMD3 |
|  | 19 | 832.5 | 5 | 25 | 877.5 | N/A |  | N/A |
|  | n77 | 3795 | 10 | 52 | 3795 | N/A | TDD | N/A |

## 6.38	DC_1A-21A-n77

### 6.38.1	Operating bands for DC

Table 6.38.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-21A-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz |  |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.38.2	Channel bandwidths per operating band for DC

Table 6.38.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-21A-n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 135 |
|  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-21A-n77C | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 235 |
|  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n77 | See CA_n77C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.38.3	Co-existence studies

Based on co-existence studies of Band 1 + Band n77 and Band 21 + Band n77 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	2nd and 5th order IMD generated by dual uplink of Band 1 + Band n77 may also fall into own Rx of band 21

-	2nd and 5th  order IMD generated by dual uplink of Band 21 + Band n77 may also fall into own Rx of band 1

Taking into account that Band 21 is currently operated only by a certain operator in Japan, the frequency range can be limited as Band 1 UL/DL = 1940-1960/2130-2150 MHz and Band n77 UL/DL = 3600-4200/3600-4200 MHz. Then own Rx impact can be simplified as below.

-	5th order IMD generated by dual uplink of Band 1 + Band n77 may also fall into own Rx of band 21

-	2nd and 5th  order IMD generated by dual uplink of Band 21 + Band n77 may also fall into own Rx of band 1

### 6.38.4	∆TIB and ∆RIB values

For DC_1A-21A-n77, the TIB,c and RIB values are given in the tables below.

Table 6.38.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-21A-n77 | 1 | 0.3 |
|  | 21 | 0.3 |
|  | n77 | 0.8 |

Table 6.38.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-21A-n77 | 1 | 0 |
|  | 21 | 0 |
|  | n77 | 0.5 |

### 6.38.5	MSD

Table 6.38.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.38.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_1A-21A-n77 | 1 | 1964.6 | 5 | 25 | 2154.6 | [30.6] | FDD | IMD2 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A |  | N/A |
|  | n77 | 3605 | 10 | 52 | 3605 | N/A | TDD | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | 21 | 1452 | 5 | 25 | 1500 | [2.9] |  | IMD5 |
|  | n77 | 3675 | 10 | 52 | 3675 | N/A | TDD | N/A |

## 6.39	DC_3A-19A-n77

### 6.39.1	Operating bands for DC

Table 6.39.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-19A-n77 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz |  |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.39.2	Channel bandwidths per operating band for DC

Table 6.39.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-19A-n77A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 135 |
|  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-19A-n77C | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 235 |
|  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n77 | See CA_n77C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.39.3	Co-existence studies

Based on co-existence studies of Band 3 + Band n77 and Band 19 + Band n77 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	IMD generated by dual uplink of Band 3 + Band n77 doesn’t fall into own Rx of band 19

-	3rd order IMD generated by dual uplink of Band 19 + Band n77 may also fall into own Rx of band 3

Taking into account that Band 19 is currently operated only by a certain operator in Japan, the frequency range can be limited as Band n77 UL/DL = 3600-4200/3600-4200 MHz. Then own Rx impact can be simplified as below.

-	IMD generated by dual uplink of Band 3 + Band n77 doesn’t fall into own Rx of band 19

-	IMD generated by dual uplink of Band 19 + Band n77 doesn’t fall into own Rx of band 3

### 6.39.4	∆TIB and ∆RIB values

For DC_3A-19A-n77, the TIB,c and RIB values are given in the tables below.

Table 6.39.4-2: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-19A-n77 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | n77 | 0.8 |

Table 6.39.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-19A-n77A | 3 | 0.2 |
|  | 19 | 0 |
|  | n77 | 0.5 |

### 6.39.5	MSD

Table 6.39.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.39.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_3A-19A-n77 | 3 | TBD | TBD | TBD | TBD | TBD | FDD | TBD |
|  | 19 | TBD | TBD | TBD | TBD | TBD |  | TBD |
|  | n77 | TBD | TBD | TBD | TBD | TBD | TDD | TBD |

## 6.40	DC_3A-21A-n77

### 6.40.1	Operating bands for DC

Table 6.40.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-21A-n77 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz |  |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.40.2	Channel bandwidths per operating band for DC

Table 6.40.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-21A-n77A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 135 |
|  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-21A-n77C | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 235 |
|  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n77 | See CA_n77C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.40.3	Co-existence studies

Based on co-existence studies of Band 3 + Band n77 and Band 21 + Band n77 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	4th and 5th order IMD generated by dual uplink of Band 3 + Band n77 may also fall into own Rx of band 21

-	2nd and 5th order IMD generated by dual uplink of Band 21 + Band n77 may also fall into own Rx of band 3

Taking into account that Band 21 is currently operated only by a certain operator in Japan, the frequency range can be limited as Band n77 UL/DL = 3600-4200/3600-4200 MHz. Then own Rx impact can be simplified as below.

-	4th order IMD generated by dual uplink of Band 3 + Band n77 may also fall into own Rx of band 21

-	5th order IMD generated by dual uplink of Band 21 + Band n77 may also fall into own Rx of band 3

### 6.40.4	∆TIB and ∆RIB values

For DC_3A-21A-n77, the TIB,c and RIB values are given in the tables below.

Table 6.40.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-21A-n77 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n77 | 0.8 |

Table 6.40.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-21A-n77 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n77 | 0.5 |

### 6.40.5	MSD

Table 6.40.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.40.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_3A-21A-n77 | 3 | 1767.5 | 5 | 25 | 1862.5 | N/A | FDD | N/A |
|  | 21 | 1459.5 | 5 | 25 | 1507.5 | [8.8] |  | IMD4 |
|  | n77 | 3795 | 10 | 52 | 3795 | N/A | TDD | N/A |
|  | 3 | 1771.6 | 5 | 25 | 1866.6 | [3.4] | FDD | IMD5 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A |  | N/A |
|  | n77 | 3935 | 10 | 52 | 3935 | N/A | TDD | N/A |

## 6.41	DC_19A-21A-n77

### 6.41.1	Operating bands for DC

Table 6.41.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19A-21A-n77 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz |  |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.41.2	Channel bandwidths per operating band for DC

Table 6.41.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_19A-21A-n77A | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 130 |
|  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_19A-21A-n77C | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 230 |
|  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n77 | See CA_n77C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.41.3	Co-existence studies

Based on co-existence studies of Band 19 + Band n77 and Band 21 + Band n77 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	4th order IMD generated by dual uplink of Band 19 + Band n77 may also fall into own Rx of band 21

-	3rd and 4th order IMD generated by dual uplink of Band 21 + Band n77 may also fall into own Rx of band 19

Taking into account that Band 19 and 21 are currently operated only by a certain operator in Japan, the frequency range can be limited as Band n77 UL/DL = 3600-4200/3600-4200 MHz. Then own Rx impact can be simplified as below.

-	4th order IMD generated by dual uplink of Band 19 + Band n77 may also fall into own Rx of band 21

-	3rd order IMD generated by dual uplink of Band 21 + Band n77 may also fall into own Rx of band 19

### 6.41.4	∆TIB and ∆RIB values

For DC_19A-21A-n77, the TIB,c and RIB values are given in the tables below.

Table 6.41.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19A-21A-n77 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | n77 | 0.8 |

Table 6.41.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19A-21A-n77 | 19 | 0 |
|  | 21 | 0 |
|  | n77 | 0.5 |

### 6.41.5	MSD

Table 6.41.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.41.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_19A-21A-n77 | 19 | 837.5 | 5 | 25 | 882.5 | [18.7] | FDD | IMD3 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A |  | N/A |
|  | n77 | 3783.3 | 10 | 52 | 3783.3 | N/A | TDD | N/A |
|  | 19 | 837.5 | 5 | 25 | 882.5 | N/A | FDD | N/A |
|  | 21 | 1454.5 | 5 | 25 | 1502.5 | [9.0] |  | IMD4 |
|  | n77 | 4015 | 10 | 52 | 4015 | N/A | TDD | N/A |

## 6.42	DC_1A-3A_n28

### 6.42.1	Operating bands for DC

Table 6.42.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3A_n281 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz |  |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| Note 1: The frequency range in band 28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. |  |  |  |  |  |  |  |  |

### 6.42.2	Channel bandwidths per operating band for DC

Table 6.42.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A_n28 | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 60 |
|  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |

### 6.42.3	Co-existence studies

Tables 6.42.3-1 and 6.42.3-2 list 1A-3A+n28 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.42.3-1: Band 1 and Band n28 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 733 |  | 1920 | 1980 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1406 – 1466 |  |  | 3840 – 3960 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2109 – 2199 |  |  | 5760 – 5940 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1187 – 1277 |  |  | 2623 – 2713 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 454 – 574 |  |  | 3107 – 3257 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 3326 – 3446 |  |  | 4543 – 4693 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 683 – 753 |  |  | 1910 – 1990 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 129 – 279 |  |  | 5027 – 5237 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 2374 – 2554 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4029 – 4179 |  |  | 6463 – 6673 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 5246 – 5426 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6947 – 7217 |  |  | 832 – 1012 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4294 – 4534 |  |  | 1641 – 1851 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8383 – 8653 |  |  | 4732 – 4912 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7166 – 7406 |  |  | 5949 – 6159 |  |  |

Based on Table 6.42.3-1,

-	2nd order harmonics may fall into Rx frequencies of Band 32, 45, 50, 51, 75, 76, and n77

-	3rd order harmonics may fall into Rx frequencies of Band 1, 4, 10, 23, 46, 47, 65, and 66

-	2nd order IMD may fall into Rx frequencies of Band 7 and 41

-	3rd order IMD may fall into Rx frequencies of Band 2, 12, 13,  17, 25, 29, 31, 33, 35, 36, 37, 39, 42, 44, 67, 68,  n77, and n78

-	4th order IMD may fall into Rx frequencies of Band 30, 40, 41, 46, and n77

-	5th order IMD may fall into Rx frequencies Band 3, 5, 6, 8, 9, 18, 19, 26, 27, 35, and n79

Table 6.42.3-2: Band 3 and Band n28 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 733 |  | 1710 | 1785 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1406 – 1466 |  |  | 3420 – 3570 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2109 – 2199 |  |  | 5130 – 5355 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 977 – 1082 |  |  | 2413 – 2518 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 244 – 379 |  |  | 2687 – 2867 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 3116 – 3251 |  |  | 4123 – 4303 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 683 – 753 |  |  | 1700 – 1795 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 324 – 489 |  |  | 4397 – 4652 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 1954 – 2164 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3819 – 3984 |  |  | 5833 – 6088 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 4826 – 5036 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6107 – 6437 |  |  | 1027 – 1222 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3664 – 3949 |  |  | 1221 – 1461 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7543 – 7873 |  |  | 4522 – 4717 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6536 – 6821 |  |  | 5529 – 5769 |  |  |

Based on Table 6.42.3-2,

-	2nd order harmonics may fall into Rx frequencies of Band  22, 32, 42, 45, 48, 50, 51, 75, 76, n77, and n78

-	3rd order harmonics may fall into Rx frequencies of Band 1, 4, 10, 23, 46, 65, and 66

-	2nd order IMD may fall into Rx frequencies of Band 41

-	3rd order IMD may fall into Rx frequencies of Band 7, 12, 13, 17,  29, 41, 44, 67, 68, and n77

-	4th order IMD may fall into Rx frequencies of Band 1, 2, 4, 10, 25, 31, 36,  65, 66, 70, n77, and n79

-	and 5th order IMD may fall into Rx frequencies of Band 32, 43, 45, 46, 50, 51, 75, 76, n77, n78, and n79

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Tables 6.42.3-3 and 6.42.3-4 list if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.42.3-3: 2UL Band 1 + Band n28 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD4 |
|  | 2400 | - | 2494 | Yes | Asia | IMD4 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | H3, IMD4 |
|  | 5150 | - | 5350 | Yes | Europe | IMD4 |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | H3, IMD4 |

Table 6.42.3-4: 2UL Band 3 + Band n28 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD2 |
|  | 2400 | - | 2494 | Yes | Asia | IMD2 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | H3, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | H3 |
|  | 5470 | - | 5725 | Yes |  | IMD5 |
|  | 5150 | - | 5825 | Yes | Asia | H3, IMD5 |

The protected bands required for the dual connectivity configuration DC_1A-3A_n28 follow from those derived for the constituent fallback modes captured in TR 37.863-01-01 [5].

### 6.42.4	∆TIB and ∆RIB values

For DC_1A-3A_n28, the TIB,c and RIB values are given in the tables below.

Table 6.42.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-3A_n28A | 1 | 0.3 |
|  | 3 | 0.3 |
|  | n28 | 0.6 |

Table 6.42.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-3A_n28A | 1 | 0 |
|  | 3 | 0 |
|  | n28 | 0.2 |

### 6.42.5	MSD

Table 6.X.5-1 lists the MSD required for the dual connectivity configuration for the cases where IMD interference falls into the own 3rd Rx frequency band, reusing values specified in LTE for CA_1A-3A-28A.

The MSD related to 3rd harmonic of Band n28 has already been discussed and specified in the constituent fallback mode.

Table 6.42.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_1A-3A_n28A | 1 | 1975 | 5 | 25 | 2165 | N/A | FDD | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |
|  | 3 | 1723.5 | 5 | 25 | 1818.5 | 4.0 | FDD | IMD5 |
| DC_1A-3A_n28A | 3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |
|  | 1 | 1949 | 5 | 25 | 2139 | 11.0 | FDD | IMD4 |

## 6.43	DC_1A-7A_n28A

### 6.43.1	Operating bands for DC

Table 6.43.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-7A_n281 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz |  |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| Note 1: The frequency range in band 28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. |  |  |  |  |  |  |  |  |

### 6.43.2	Channel bandwidths per operating band for DC

Table 6.43.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-7A_n28 | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 60 |
|  | 7 | 15 |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |

### 6.43.3	Co-existence studies

Tables 6.43.3-1 and 6.43.3-2 list 1A-7A+n28 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.43.3-1: Band 1 and Band n28 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 733 |  | 1920 | 1980 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1406 – 1466 |  |  | 3840 – 3960 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2109 – 2199 |  |  | 5760 – 5940 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1187 – 1277 |  |  | 2623 – 2713 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 454 – 574 |  |  | 3107 – 3257 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 3326 – 3446 |  |  | 4543 – 4693 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 683 – 753 |  |  | 1900 – 2000 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 129 – 279 |  |  | 5027 – 5237 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 2374 – 2554 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4029 – 4179 |  |  | 6463 – 6673 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 5246 – 5426 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6947 – 7217 |  |  | 832 – 1012 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4294 – 4534 |  |  | 1641 – 1851 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8383 – 8653 |  |  | 4732 – 4912 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7166 – 7406 |  |  | 5949 – 6159 |  |  |

Based on Table 6.43.3-1,

-	2nd order harmonics may fall into Rx frequencies of Band 32, 45, 50, 51, 75, 76, and n77

-	3rd order harmonics may fall into Rx frequencies of Band 1, 4, 10, 23, 46, 47, 65, and 66

-	2nd order IMD may fall into Rx frequencies of Band 7 and 41

-	3rd order IMD may fall into Rx frequencies of Band 2, 12, 13,  17, 25, 29, 31, 33, 35, 36, 37, 39, 42, 44, 67, 68,  n77, and n78

-	4th order IMD may fall into Rx frequencies of Band 30, 40, 41, 46, and n77

-	5th order IMD may fall into Rx frequencies Band 3, 5, 6, 8, 9, 18, 19, 26, 27, 35, and n79

Table 6.43.3-2: Band 7 and Band n28 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 733 |  | 2500 | 2570 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1406 – 1466 |  |  | 5000 – 5140 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2109 – 2199 |  |  | 7500 – 7710 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1767 – 1867 |  |  | 3203 – 3303 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 1034 – 1164 |  |  | 4267 – 4437 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 3906 – 4036 |  |  | 5703 – 5873 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 683 – 753 |  |  | 2490 – 2580 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 301 – 461 |  |  | 6767 – 7007 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 3534 – 3734 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4609 – 4769 |  |  | 8203 – 8443 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 6406 – 6606 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9267 – 9577 |  |  | 242 – 432 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6034 – 6304 |  |  | 2801 – 3031 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10703 – 11013 |  |  | 5312 – 5502 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8906 – 9176 |  |  | 7109 – 7339 |  |  |

Based on Table 6.43.3-2,

-	2nd order harmonics may fall into Rx frequencies of Band  32, 45, 50, 51, 75, and 76

-	3rd order harmonics may fall into Rx frequencies of Band 1, 4, 10, 23, 65, and 66

-	2nd order IMD may fall into Rx frequencies of Band 3, 35, n77, and n78

-	3rd order IMD may fall into Rx frequencies of Band 12, 13, 17, 29, 38, 41, 44, 46, 47, 67, 68, 69, n77, and n79

-	4th order IMD may fall into Rx frequencies of Band 22, 42, 43, 48, n77, n78, and n79

-	5th order IMD may fall into Rx frequencies of Band  46

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Tables 6.43.3-3 and 6.43.3-4 list if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.43.3-3: 2UL Band 1 + Band n28 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD4 |
|  | 2400 | - | 2494 | Yes | Asia | IMD4 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | H3, IMD4 |
|  | 5150 | - | 5350 | Yes | Europe | IMD4 |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | H3, IMD4 |

Table 6.43.3-4: 2UL Band 7 + Band n28 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe | IMD3 |
|  | 2400 | - | 2494 | Yes | Asia | IMD3 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD5 |
|  | 5470 | - | 5725 | Yes |  | IMD5 |
|  | 5150 | - | 5825 | Yes | Asia | IMD5 |

The protected bands required for the dual connectivity configuration DC_1A-7A_n28 follow from those derived for the constituent fallback modes captured in TR 37.863-01-01 [5].

### 6.43.4	∆TIB and ∆RIB values

For DC_1A-7A_n28A, the TIB,c and RIB values are given in the tables below.

Table 6.43.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-7A_n28A | 1 | 0.5 |
|  | 7 | 0.6 |
|  | n28 | 0.6 |

Table 6.43.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-7A_n28A | 1 | 0 |
|  | 7 | 0 |
|  | n28 | 0.2 |

### 6.43.5	MSD

Table 6.43.5-1 lists the required MSD levels from interested companies and propose required MSD levels using the MSD principle for the dual connectivity configuration.

Table 6.43.5-1: MSD results from each companies

|  | E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- |
| DC Configuration |  | EUTRA and NR band | MSD (LGE) | MSD (HW) | Avg. MSD | IMD2 ¦fB1+fn28¦ |
| DC_1A-7A_n28A |  | 1 | 28.8 | 31.0 | 30.0 |  |
|  |  | n28 |  |  |  |  |
|  |  | 7 |  |  |  |  |

Table 6.43.5-2 propose the required MSD levels for the dual connectivity configuration as average manner.

Table 6.43.5-2: MSD for the DC configuration

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-7A_n28A | 1 | IMD2 | ¦fB1 +fn28¦ | 1935 | 5 | 25 | 2125 | 5 | 2.0 | N/A |
|  | n28 |  |  | 718 | 5 | 25 | 773 | 5 |  |  |
|  | 7 |  |  | 2533 | 10 | 52 | 2653 | 10 |  | 30.0 |

## 6.44	DC_1A-20A_n28

### 6.44.1	Operating bands for DC

Table 6.44.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-20A_n281 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz |  |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| Note 1: The frequency range in band 28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. |  |  |  |  |  |  |  |  |

### 6.44.2	Channel bandwidths per operating band for DC

Table 6.44.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-20A_n28 | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 60 |
|  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |

### 6.44.3	Co-existence studies

Tables 6.44.3-1 and 6.44.3-2 list 1A-20A+n28 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.44.3-1: Band 1 and Band n28 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 733 |  | 1920 | 1980 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1406 – 1466 |  |  | 3840 – 3960 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2109 – 2199 |  |  | 5760 – 5940 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1187 – 1277 |  |  | 2623 – 2713 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 454 – 574 |  |  | 3107 – 3257 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 3326 – 3446 |  |  | 4543 – 4693 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 683 – 753 |  |  | 1900 – 2000 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 129 – 279 |  |  | 5027 – 5237 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 2374 – 2554 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4029 – 4179 |  |  | 6463 – 6673 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 5246 – 5426 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6947 – 7217 |  |  | 832 – 1012 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4294 – 4534 |  |  | 1641 – 1851 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8383 – 8653 |  |  | 4732 – 4912 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7166 – 7406 |  |  | 5949 – 6159 |  |  |

Based on Table 6.44.3-1,

-	2nd order harmonics may fall into Rx frequencies of Band 32, 45, 50, 51, 75, 76, and n77

-	3rd order harmonics may fall into Rx frequencies of Band 1, 4, 10, 23, 46, 47, 65, and 66

-	2nd order IMD may fall into Rx frequencies of Band 7 and 41

-	3rd order IMD may fall into Rx frequencies of Band 2, 12, 13,  17, 25, 29, 31, 33, 35, 36, 37, 39, 42, 44, 67, 68,  n77, and n78

-	4th order IMD may fall into Rx frequencies of Band 30, 40, 41, 46, and n77

-	5th order IMD may fall into Rx frequencies Band 3, 5, 6, 8, 9, 18, 19, 26, 27, 35, and n79

Table 6.44.3-2: Band 20 and Band n28 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 733 |  | 832 | 862 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1406 – 1466 |  |  | 1664 – 1724 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2109 – 2199 |  |  | 2496 – 2586 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 99 – 159 |  |  | 1535 – 1595 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 544 – 634 |  |  | 931 – 1021 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 2238 – 2328 |  |  | 2367 – 2457 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 683 – 753 |  |  | 812 – 882 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1247 – 1367 |  |  | 1763 – 1883 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 198 – 318 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2941 – 3061 |  |  | 3199 – 3319 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 3070 – 3190 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2595 – 2745 |  |  | 1950 – 2100 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1030 – 1180 |  |  | 385 – 535 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4031 – 4181 |  |  | 3644 – 3794 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3902 – 4052 |  |  | 3773 – 3923 |  |  |

Based on Table 6.44.3-2,

-	2nd order harmonics may fall into Rx frequencies of Band  32, 45, 50, 51, 75, 76

-	3rd order harmonics may fall into Rx frequencies of Band 1, 4, 10, 23, 38, 41, 65, 66, and 69

-	2nd order IMD may fall into Rx frequencies of Band 24

-	3rd order IMD may fall into Rx frequencies of Band 5, 8, 12, 13, 17, 18, 20, 26, 27, 29, 30, 40, 44, 67 and 68

-	4th order IMD may fall into Rx frequencies of Band 3, 9, 35, 39, n77 and n78

-	5th order IMD may fall into Rx frequencies of Band  2, 7,, 25, 31, 34, 36, 38, 41, 43, 48,  69, 70, n77 and n78

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Tables 6.44.3-3 and 6.44.3-4 list if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.44.3-3: 2UL Band 1 + Band n28 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD4 |
|  | 2400 | - | 2494 | Yes | Asia | IMD4 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | H3, IMD4 |
|  | 5150 | - | 5350 | Yes | Europe | IMD4 |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | H3, IMD4 |

Table 6.44.3-4: 2UL Band 20 + Band n28 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD2 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD2 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD2 |
| GPS | 1563 | - | 1587 | Yes |  | IMD2 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD3 |
|  | 2400 | - | 2494 | Yes | Asia | IMD3 |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

The protected bands required for the dual connectivity configuration DC_1A-20A_n28 follow from those derived for the constituent fallback modes captured in TR 37.863-01-01 [5].

### 6.44.4	∆TIB and ∆RIB values

For DC_1A-20A_n28A, the TIB,c and RIB values are given in the tables below.

Table 6.44.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-20A_n28A | 1 | 0.3 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |

Table 6.44.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-20A_n28A | 1 | 0 |
|  | 20 | 0.2 |
|  | n28 | 0.2 |

### 6.44.5	MSD

The MSD related to 3rd harmonic of Band n28 falling into band 1 Rx has already been discussed and specified in the constituent fallback mode. No other MSD issue.

## 6.45	DC_3A-7A_n28

### 6.45.1	Operating bands for DC

Table 6.45.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-7A_n281 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz |  |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| Note 1: The frequency range in band 28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. |  |  |  |  |  |  |  |  |

### 6.45.2	Channel bandwidths per operating band for DC

Table 6.45.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-7A_n28 | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 60 |
|  | 7 | 15 |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |

### 6.45.3	Co-existence studies

Tables 6.45.3-1 and 6.45.3-2 list 3A-7A+n28 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.45.3-1: Band 3 and Band n28 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 733 |  | 1710 | 1785 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1406 – 1466 |  |  | 3420 – 3570 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2109 – 2199 |  |  | 5130 – 5355 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 977 – 1082 |  |  | 2413 – 2518 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 244 – 379 |  |  | 2687 – 2867 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 3116 – 3251 |  |  | 4123 – 4303 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 683 – 753 |  |  | 1690 – 1805 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 324 – 489 |  |  | 4397 – 4652 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 1954 – 2164 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3819 – 3984 |  |  | 5833 – 6088 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 4826 – 5036 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6107 – 6437 |  |  | 1027 – 1222 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3664 – 3949 |  |  | 1221 – 1461 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7543 – 7873 |  |  | 4522 – 4717 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6536 – 6821 |  |  | 5529 – 5769 |  |  |

Based on Table 6.45.3-1,

-	2nd order harmonics may fall into Rx frequencies of Band  22, 32, 42, 45, 48, 50, 51, 75, 76, n77, and n78

-	3rd order harmonics may fall into Rx frequencies of Band 1, 4, 10, 23, 46, 65, and 66

-	2nd order IMD may fall into Rx frequencies of Band 41

-	3rd order IMD may fall into Rx frequencies of Band 7, 12, 13, 17,  29, 41, 44, 67, 68, and n77

-	4th order IMD may fall into Rx frequencies of Band 1, 2, 4, 10, 25, 31, 36,  65, 66, 70, n77, and n79

-	and 5th order IMD may fall into Rx frequencies of Band 32, 43, 45, 46, 50, 51, 75, 76, n77, n78, and n79

Table 6.45.3-2: Band 7 and Band n28 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 733 |  | 2500 | 2570 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1406 – 1466 |  |  | 5000 – 5140 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2109 – 2199 |  |  | 7500 – 7710 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1767 – 1867 |  |  | 3203 – 3303 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 1034 – 1164 |  |  | 4267 – 4437 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 3906 – 4036 |  |  | 5703 – 5873 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 683 – 753 |  |  | 2480 – 2590 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 301 – 461 |  |  | 6767 – 7007 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 3534 – 3734 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4609 – 4769 |  |  | 8203 – 8443 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 6406 – 6606 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9267 – 9577 |  |  | 242 – 432 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6034 – 6304 |  |  | 2801 – 3031 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10703 – 11013 |  |  | 5312 – 5502 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8906 – 9176 |  |  | 7109 – 7339 |  |  |

Based on Table 6.45.3-2,

-	2nd order harmonics may fall into Rx frequencies of Band  32, 45, 50, 51, 75, and 76

-	3rd order harmonics may fall into Rx frequencies of Band 1, 4, 10, 23, 65, and 66

-	2nd order IMD may fall into Rx frequencies of Band 3, 35, n77, and n78

-	3rd order IMD may fall into Rx frequencies of Band 12, 13, 17, 29, 38, 41, 44, 46, 47, 67, 68, 69, n77, and n79

-	4th order IMD may fall into Rx frequencies of Band 22, 42, 43, 48, n77, n78, and n79

-	 5th order IMD may fall into Rx frequencies of Band  46

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Tables 6.45.3-3 and 6.45.3-4 list if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.45.3-3: 2UL Band 3 + Band n28 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD2 |
|  | 2400 | - | 2494 | Yes | Asia | IMD2 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | H3, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | H3 |
|  | 5470 | - | 5725 | Yes |  | IMD5 |
|  | 5150 | - | 5825 | Yes | Asia | H3, IMD5 |

Table 6.45.3-4: 2UL Band 7 + Band n28 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe | IMD3 |
|  | 2400 | - | 2494 | Yes | Asia | IMD3 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD5 |
|  | 5470 | - | 5725 | Yes |  | IMD5 |
|  | 5150 | - | 5825 | Yes | Asia | IMD5 |

The protected bands required for the dual connectivity configuration DC_3A-7A_n28 follow from those derived for the constituent fallback modes captured in TR 37.863-01-01 [5].

### 6.45.4	∆TIB and ∆RIB values

For DC_3A-7A_n28, the TIB,c and RIB values are given in the tables below.

Table 6.45.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-7A_n28A | 3 | 0.5 |
|  | 7 | 0.5 |
|  | n28 | 0.3 |

Table 6.45.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-7A_n28A | 3 | 0 |
|  | 7 | 0 |
|  | n28 | 0 |

### 6.45.5	MSD

Table 6.45.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.45.5-1: MSD for the DC configuration

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-7A_n28A | 3 | IMD3 | ¦2*fB3 -fn28¦ | 1712.5 | 5 | 25 | 1807.5 | 5 | N/A |
|  | n28 |  |  | 743 | 5 | 25 | 798 | 5 |  |
|  | 7 |  |  | 2562 | 10 | 52 | 2682 | 10 | 16.9 |
|  | 7 | IMD2 | ¦fB7 –fn28¦ | 2543 | 10 | 52 | 2663 | 10 | N/A |
|  | n28 |  |  | 710.5 | 5 | 25 | 765.5 | 5 |  |
|  | 3 |  |  | 1737.5 | 5 | 25 | 1832.5 | 5 | 26.0 |

## 6.46	DC_3A-20A_n28

### 6.46.1	Operating bands for DC

Table 6.46.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-20A_n281 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz |  |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| Note 1: The frequency range in band 28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. |  |  |  |  |  |  |  |  |

### 6.46.2	Channel bandwidths per operating band for DC

Table 6.46.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-20A_n28 | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 60 |
|  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |

### 6.46.3	Co-existence studies

Tables 6.46.3-1 and 6.46.3-2 list 3A-20A+n28 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.46.3-1: Band 3 and Band n28 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 733 |  | 1710 | 1785 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1406 – 1466 |  |  | 3420 – 3570 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2109 – 2199 |  |  | 5130 – 5355 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 977 – 1082 |  |  | 2413 – 2518 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 244 – 379 |  |  | 2687 – 2867 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 3116 – 3251 |  |  | 4123 – 4303 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 683 – 753 |  |  | 1690 – 1805 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 324 – 489 |  |  | 4397 – 4652 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 1954 – 2164 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3819 – 3984 |  |  | 5833 – 6088 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 4826 – 5036 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6107 – 6437 |  |  | 1027 – 1222 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3664 – 3949 |  |  | 1221 – 1461 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7543 – 7873 |  |  | 4522 – 4717 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6536 – 6821 |  |  | 5529 – 5769 |  |  |

Based on Table 6.46.3-1,

-	2nd order harmonics may fall into Rx frequencies of Band  22, 32, 42, 45, 48, 50, 51, 75, 76, n77, and n78

-	3rd order harmonics may fall into Rx frequencies of Band 1, 4, 10, 23, 46, 65, and 66

-	2nd order IMD may fall into Rx frequencies of Band 41

-	3rd order IMD may fall into Rx frequencies of Band 7, 12, 13, 17,  29, 41, 44, 67, 68, and n77

-	4th order IMD may fall into Rx frequencies of Band 1, 2, 4, 10, 25, 31, 36,  65, 66, 70, n77, and n79

-	and 5th order IMD may fall into Rx frequencies of Band 32, 43, 45, 46, 50, 51, 75, 76, n77, n78, and n79

Table 6.46.3-2: Band 20 and Band n28 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 733 |  | 832 | 862 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1406 – 1466 |  |  | 1664 – 1724 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2109 – 2199 |  |  | 2496 – 2586 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 99 – 159 |  |  | 1535 – 1595 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 544 – 634 |  |  | 931 – 1021 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 2238 – 2328 |  |  | 2367 – 2457 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 683 – 753 |  |  | 812 – 882 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1247 – 1367 |  |  | 1763 – 1883 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 198 – 318 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2941 – 3061 |  |  | 3199 – 3319 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 3070 – 3190 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2595 – 2745 |  |  | 1950 – 2100 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1030 – 1180 |  |  | 385 – 535 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4031 – 4181 |  |  | 3644 – 3794 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3902 – 4052 |  |  | 3773 – 3923 |  |  |

Based on Table 6.46.3-2,

-	2nd order harmonics may fall into Rx frequencies of Band  32, 45, 50, 51, 75, 76

-	3rd order harmonics may fall into Rx frequencies of Band 1, 4, 10, 23, 38, 41, 65, 66, and 69

-	2nd order IMD may fall into Rx frequencies of Band 24

-	3rd order IMD may fall into Rx frequencies of Band 5, 8, 12, 13, 17, 18, 20, 26, 27, 29, 30, 40, 44, 67 and 68

-	4th order IMD may fall into Rx frequencies of Band 3, 9, 35, 39, n77 and n78

-	5th order IMD may fall into Rx frequencies of Band  2, 7,, 25, 31, 34, 36, 38, 41, 43, 48,  69, 70, n77 and n78

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Tables 6.46.3-3 and 6.46.3-4 list if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.46.3-3: 2UL Band 3 + Band n28 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD2 |
|  | 2400 | - | 2494 | Yes | Asia | IMD2 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | H3, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | H3 |
|  | 5470 | - | 5725 | Yes |  | IMD5 |
|  | 5150 | - | 5825 | Yes | Asia | H3, IMD5 |

Table 6.46.3-4: 2UL Band 20 + Band n28 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD2 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD2 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD2 |
| GPS | 1563 | - | 1587 | Yes |  | IMD2 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD3 |
|  | 2400 | - | 2494 | Yes | Asia | IMD3 |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

The protected bands required for the dual connectivity configuration DC_3A-20A_n28 follow from those derived for the constituent fallback modes captured in TR 37.863-01-01 [5].

### 6.46.4	∆TIB and ∆RIB values

For DC_3A-20A_n28, the TIB,c and RIB values are given in the tables below.

Table 6.46.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-20A_n28A | 3 | 0.3 |
|  | 20 | 0.5 |
|  | n28 | 0.5 |

Table 6.46.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-20A_n28A | 3 | 0 |
|  | 20 | 0.1 |
|  | n28 | 0.1 |

### 6.46.5	MSD

Table 6.46.5-1 lists the required MSD levels from interested companies and propose required MSD levels using the MSD principle for the dual connectivity configuration.

Table 6.46.5-1: MSD results from each companies

|  | E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- |
| DC Configuration |  | EUTRA and NR band | MSD (LGE) | MSD (HW) | Avg. MSD | IMD4 ¦3*fB20-fB28¦ |
| DC_3A-20A_n28A |  | 20 | 10.3 | 8.5 | 9.4 |  |
|  |  | n28 |  |  |  |  |
|  |  | 3 |  |  |  |  |

Based on above MSD test results, we propose as below MSD test requirements in Table 6.46.5-2.

Table 6.46.5-2: Proposed MSD for the DC configuration

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-20A_n28A | 20 | IMD4 | ¦3*fB20 -fn28¦ | 852 | 5 | 25 | 811 | 5 | N/A |
|  | n28 |  |  | 738 | 5 | 25 | 793 | 5 |  |
|  | 3 |  |  | 1723 | 5 | 25 | 1818 | 5 | [9.4] |

## 6.47  DC_7A-20A_n28

### 6.47.1	Operating bands for DC

Table 6.47.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7A-20A_n281 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz |  |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| Note 1: The frequency range in band 28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. |  |  |  |  |  |  |  |  |

### 6.47.2	Channel bandwidths per operating band for DC

Table 6.47.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A-20A_n28 | 7 | 15 |  | Yes | Yes | Yes |  |  |  |  |  | 60 |
|  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |

### 6.47.3	Co-existence studies

Tables 6.47.3-1 and 6.47.3-2 list 7A-20A+n28 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.47.3-1: Band 7 and Band n28 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 733 |  | 2500 | 2570 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1406 – 1466 |  |  | 5000 – 5140 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2109 – 2199 |  |  | 7500 – 7710 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1767 – 1867 |  |  | 3203 – 3303 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 1034 – 1164 |  |  | 4267 – 4437 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 3906 – 4036 |  |  | 5703 – 5873 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 683 – 753 |  |  | 2480 – 2590 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 301 – 461 |  |  | 6767 – 7007 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 3534 – 3734 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4609 – 4769 |  |  | 8203 – 8443 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 6406 – 6606 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9267 – 9577 |  |  | 242 – 432 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6034 – 6304 |  |  | 2801 – 3031 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10703 – 11013 |  |  | 5312 – 5502 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8906 – 9176 |  |  | 7109 – 7339 |  |  |

Based on Table 6.47.3-1,

-	2nd order harmonics may fall into Rx frequencies of Band  32, 45, 50, 51, 75, and 76

-	3rd order harmonics may fall into Rx frequencies of Band 1, 4, 10, 23, 65, and 66

-	2nd order IMD may fall into Rx frequencies of Band 3, 35, n77, and n78

-	3rd order IMD may fall into Rx frequencies of Band 12, 13, 17, 29, 38, 41, 44, 46, 47, 67, 68, 69, n77, and n79

-	4th order IMD may fall into Rx frequencies of Band 22, 42, 43, 48, n77, n78, and n79

-	 5th order IMD may fall into Rx frequencies of Band  46

Table 6.47.3-2: Band 20 and Band n28 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 733 |  | 832 | 862 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1406 – 1466 |  |  | 1664 – 1724 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2109 – 2199 |  |  | 2496 – 2586 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 99 – 159 |  |  | 1535 – 1595 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 544 – 634 |  |  | 931 – 1021 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 2238 – 2328 |  |  | 2367 – 2457 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 683 – 753 |  |  | 812 – 882 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1247 – 1367 |  |  | 1763 – 1883 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 198 – 318 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2941 – 3061 |  |  | 3199 – 3319 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 3070 – 3190 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2595 – 2745 |  |  | 1950 – 2100 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1030 – 1180 |  |  | 385 – 535 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4031 – 4181 |  |  | 3644 – 3794 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3902 – 4052 |  |  | 3773 – 3923 |  |  |

Based on Table 6.47.3-2,

-	2nd order harmonics may fall into Rx frequencies of Band  32, 45, 50, 51, 75, 76

-	3rd order harmonics may fall into Rx frequencies of Band 1, 4, 10, 23, 38, 41, 65, 66, and 69

-	2nd order IMD may fall into Rx frequencies of Band 24

-	3rd order IMD may fall into Rx frequencies of Band 5, 8, 12, 13, 17, 18, 20, 26, 27, 29, 30, 40, 44, 67 and 68

-	4th order IMD may fall into Rx frequencies of Band 3, 9, 35, 39, n77 and n78

-	5th order IMD may fall into Rx frequencies of Band  2, 7,, 25, 31, 34, 36, 38, 41, 43, 48,  69, 70, n77 and n78

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Tables 6.47.3-3 and 6.47.3-4 list if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.47.3-3: 2UL Band 7 + Band n28 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe | IMD3 |
|  | 2400 | - | 2494 | Yes | Asia | IMD3 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD5 |
|  | 5470 | - | 5725 | Yes |  | IMD5 |
|  | 5150 | - | 5825 | Yes | Asia | IMD5 |

Table 6.47.3-4: 2UL Band 20 + Band n28 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD2 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD2 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD2 |
| GPS | 1563 | - | 1587 | Yes |  | IMD2 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD3 |
|  | 2400 | - | 2494 | Yes | Asia | IMD3 |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

The protected bands required for the dual connectivity configuration DC_7A-20A_n28 follow from those derived for the constituent fallback modes captured in TR 37.863-01-01 [5].

### 6.47.4	∆TIB and ∆RIB values

For DC_7A-20A_n28, the TIB,c and RIB values are given in the tables below.

Table 6.47.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-20A_n28A | 7 | 0.3 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |

Table 6.47.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7A-20A_n28A | 7 | 0 |
|  | 20 | 0.2 |
|  | n28 | 0.2 |

### 6.47.5	MSD

Table 6.47.5-1 lists the required MSD levels from interested companies and propose required MSD levels using the MSD principle for the dual connectivity configuration.

Table 6.47.5-1: MSD results from each companies

|  | E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- |
| DC Configuration |  | EUTRA and NR band | MSD (LGE) | MSD (HW) | Avg. MSD | IMD5 ¦4*fB20 –fn28¦ |
| DC_7A-20A_n28A |  | 20 | 4.2 | 7.6 | 5.9 |  |
|  |  | n28 |  |  |  |  |
|  |  | 7 |  |  |  |  |

Based on above MSD test results, we propose as below MSD test requirements in Table 6.47.5-2.

Table 6.47.5-2: MSD for the DC configuration

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A-20A_n28A | 20 | IMD5 | ¦4*fB20 –fn28¦ | 852 | 5 | 25 | 811 | 5 | N/A |
|  | n28 |  |  | 738 | 5 | 25 | 793 | 5 |  |
|  | 7 |  |  | 2550 | 10 | 52 | 2670 | 10 | [5.9] |

## 6.48	DC_19A-21A-n257

### 6.48.1	Operating bands for DC

Table 6.48.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19A-21A-n257 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz |  |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.48.2	Channel bandwidths per operating band for DC

Table 6.48.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_19A-21A-n257A | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  | 430 |
|  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |
| DC_19A-21A-n257D | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  | 430 |
|  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_19A-21A-n257E | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  | 630 |
|  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_19A-21A-n257F | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  | 830 |
|  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |

### 6.48.3	Co-existence studies

Based on co-existence studies of Band 19 + Band n257 and Band 21 + Band n257 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	IMD generated by dual uplink of Band 19 + Band n257 doesn’t fall into own Rx of band 21

-	IMD generated by dual uplink of Band 21 + Band n257 doesn’t fall into own Rx of band 19

### 6.48.4	∆TIB and ∆RIB values

For DC_19A-21A-n257, the TIB,c and RIB values are given in the tables below.

Table 6.48.4-2: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19A-21A-n257 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | n257 | 0 |

Table 6.48.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19A-21A-n257 | 19 | 0 |
|  | 21 | 0 |
|  | n257 | 0 |

### 6.48.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.49	DC_41C_n41A_BCS0

### 6.49.1	Operating bands for DC

Table 6.49.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_41C_n41A, DC_(n)41D | 41 | 2496 GHz | – | 2690 MHz | 2496 GHz | – | 2690 MHz | TDD |
|  | n41 | 2496 GHz | – | 2690 MHz | 2496 GHz | – | 2690 MHz | TDD |

### 6.49.2	Channel bandwidths per operating band for DC

Table 6.49.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |  | Maximum aggregated bandwidth [MHz] |
| DC_41C_n41A, , DC_(n)41D | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 140 |
|  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes |  |  |

### 6.49.3	Co-existence studies

Table 6.49.3-1 lists Band 41C + Band n41 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.49.3-1: Band 41 and Band n41 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 | 7488 | 8070 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 194 | 194 | 4992 | 5380 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2302 | 2884 | 2302 | 2884 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7488 | 8070 | 7488 | 8070 |
| 3rd order IMD products (max BW) | ¦fx_low – max BW fy¦ | ¦fx_high + max BW fy¦ | ¦fy_low – max BW fx¦ | ¦fy_high + max BW fx¦ |
| IMD frequency limits (MHz) | 2396 | 2790 | 2476 | 2710 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 388 | 388 | 9984 | 10760 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4798 | 5574 | 4798 | 5574 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9984 | 10760 | 9984 | 10760 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8264 | 7294 | 8264 | 7294 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 12480 | 13450 | 12480 | 13450 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3078 | 2108 | 3078 | 2108 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 12480 | 13450 | 12480 | 13450 |

Based on Table 6.49.3-1;

-	2nd order harmonics may fall into Rx frequencies of bands 46 and n79

-	2nd order IMD may fall into Rx frequencies of bands 46 and n79

-	3rd order IMD may fall into Rx frequencies of band 7, 30, 38, 40. 41 and 69

-	4th order IMD may fall into Rx frequencies of band 46 and 79

-	5th order IMD may fall into Rx frequencies of band 1, 4, 7, 10, 23, 30, 38, 40, 41, 65. 66 and 69

When 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.49.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.49.3-2: 2UL Band 41 +Band n41 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD3 |
|  | 2400 | - | 2494 | Yes | Asia | IMD3 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd Harmonic, IMD2, IMD4 |
|  | 5150 | - | 5350 | Yes | Europe | 2nd Harmonic, IMD2, IMD4 |
|  | 5470 | - | 5725 | No |  | IMD4 |
|  | 5150 | - | 5825 | Yes | Asia | 2nd Harmonic, IMD2, IMD4 |

Table 6.49.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.49.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_41C_n41A | TBD | FDL_low | - | FDL_high | TBD | TBD |  |

### 6.49.4	∆TIB and ∆RIB values

For DC_41C_n41A, the TIB,c and RIB values are given in the tables below.

Table 6.49.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41C_n41A | 41 | 0 |
|  | n41 | 0 |

Table 6.49.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_41C_n41A | 41 | 0 |
|  | n41 | 0 |

### 6.49.5	MSD

Since Band 41 and n41 are both TDD bands, no MSD is required for the dual connectivity configuration.

### 6.49.6	SEM for DC_41C_n41A

The spectral emission mask for DC_41C_n41A is a composite of the emission mask for each CC with the level set to the maximum value from each mask for each frequency outside of the transmission bandwidth of either carrier.

### 6.49.6a	SEM for DC_(n)41D

The spectral emission mask for DC_(n)41D is TBD.

### 6.49.7	A-MPR

A-MPR for DC_41C_n41A is TBD.

### 6.49.7a	A-MPR for DC_(n)41D

A-MPR for DC_(n)41D is TBD.

## 6.50	DC_2A-2A-n257A

### 6.50.1	Operating bands for DC

Table 6.50.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2A-2A-n257A | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.50.2	Channel bandwidths per operating band for DC

Table 6.50.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |  |
| DC_2A-2A-n257 | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 240 |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 440 |  |

### 6.50.3	Co-existence studies

Table 6.50.3-1 lists Band 2A-2A +Band n257A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.50.3-1: Band 2 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 27650 | 24590 | 28350 | 31410 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 25800 | 22680 | 51090 | 57150 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 30200 | 33320 | 54850 | 60910 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 1450 | 2310 | 26480 | 29520 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 55300 | 49180 | 56700 | 62820 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 23950 | 20770 | 77590 | 86650 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 32050 | 35230 | 81350 | 90410 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 116150 | 104090 | 18860 | 22100 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 107850 | 119910 | 33900 | 37140 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 84800 | 75680 | 47270 | 53450 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 83200 | 92320 | 58550 | 64730 |

Based on Table 6.50.3-1

-	2nd order harmonics may fall into LTE frequencies of Band 43

-	3rd order harmonic may fall into LTE frequencies of Band 46

-	2nd order IMD products may fall into Rx frequencies of band n257 and n258

-	3rd order IMD products may fall into Rx frequencies of Band 1, 3, 4, 9, 10, 11, 21, 23, 24, 25, 32, 33, 34, 35, 36, 37, 39, 40, 45, 50, 65, 66, 70, 74,75 and band n257, n258

-	3rd order IMD products may fall into own Rx frequencies of Band 2 and n257

-	5th order IMD products may fall into Rx frequencies of band n260

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.50.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.50.3-2: 2UL Band 2 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | IMD3 |
| Galileo | 1559 | - | 1591 | YES |  | IMD3 |
| GLONASS | 1591 | - | 1610 | YES |  | IMD3 |
| GPS | 1563 | - | 1587 | YES |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | 3rd order harmonic |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | YES |  | 3rd order harmonic |
|  | 5150 | - | 5825 | YES | Asia | 3rd order harmonic |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | YES | China | 5th IMD |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | YES | Europe | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 57050 | - | 64000 | YES | USA - Canada | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 57000 | - | 64000 | YES | South Korea | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 59000 | - | 66000 | YES | Japan | 3rd, 4th, 5th IMD |
|  | 59000 | - | 64000 | YES | China | 3rd, 4th, 5th IMD |
|  | 59400 | - | 62900 | YES | Australia | 3rd, 4th, 5th IMD |

Table 6.50.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.50.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_2A-2A-n257A | Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 50, 51, 66, 70, 71, 74, 74, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | x |
|  | Band 43 | FDL_low | - | FDL_high | -50 | 1 | 1* |
| * Table 6.5.3.2-1 of TS38.101 Note x: NOTE 15 in Table 6.6.3.2-1 of 36.101 [2] will be applied here. Additional requirements for the frequency ranges that are less than FOOB (MHz) in Table 6.50.3.1-1 need further study. |  |  |  |  |  |  |  |

### 6.50.4	∆TIB and ∆RIB values

For DC_2A-2A-n257A, the TIB,c and RIB values are given in the tables below.

Table 6.50.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2A-2A-n257A | 2 | 0 |
|  | n257 | 0 |

Table 6.50.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2A-2A-n257A | 2 | 0 |
|  | n257 | 0 |

### 6.50.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.51	DC_2A-5A-n257A

### 6.51.1	Operating bands for DC

Table 6.51.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2A-5A-n257A | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.51.2	Channel bandwidths per operating band for DC

Table 6.51.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |  |
| DC_2A-5A-n257A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 230 |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 430 |  |

### 6.51.3	Co-existence studies

Table 6.51.3-1 lists Band 2A-5A+Band n257A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.51.3-1: Band 2 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 27650 | 24590 | 28350 | 31410 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 25800 | 22680 | 51090 | 57150 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 30200 | 33320 | 54850 | 60910 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 1450 | 2310 | 26480 | 29520 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 55300 | 49180 | 56700 | 62820 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 23950 | 20770 | 77590 | 86650 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 32050 | 35230 | 81350 | 90410 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 116150 | 104090 | 18860 | 22100 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 107850 | 119910 | 33900 | 37140 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 84800 | 75680 | 47270 | 53450 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 83200 | 92320 | 58550 | 64730 |

Based on Table 6.51.3-1

-	2nd order harmonics may fall into LTE frequencies of Band 43

-	3rd order harmonic may fall into LTE frequencies of Band 46

-	2nd order IMD products may fall into Rx frequencies of band n257 and n258

-	3rd order IMD products may fall into Rx frequencies of Band 1, 3, 4, 9, 10, 11, 21, 23, 24, 25, 32, 33, 34, 35, 36, 37, 39, 40, 45, 50, 65, 66, 70, 74,75 and band n257, n258

-	3rd order IMD products may fall into own Rx frequencies of Band 2 and n257

-	5th order IMD products may fall into Rx frequencies of band n260

Table 6.51.3-2: Band 5 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 28676 | 25651 | 27324 | 30349 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 27852 | 24802 | 52151 | 58176 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 28148 | 31198 | 53824 | 59849 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 424 | 1249 | 26490 | 29510 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 57352 | 51302 | 54648 | 60698 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 27028 | 23953 | 78651 | 87676 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 28972 | 32047 | 80324 | 89349 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 117176 | 105151 | 23104 | 26204 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 106824 | 118849 | 29796 | 32896 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 86852 | 77802 | 50453 | 56528 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 81148 | 90198 | 55472 | 61547 |

Based on Table 6.51.3-2

-	3rd order harmonic may fall into frequencies of band n41

-	2nd order IMD products may fall into own Rx frequencies of band n257 and n258

-	3rd order IMD products may fall into Rx frequencies of Band 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27 , 28 , 29, 31, 44, 67, 68, 71, 72 and 73

-	3rd order IMD products may fall into Rx frequencies of band n257 and n258

-	4th order IMD products may fall into Rx frequencies of band n257 and n258

-	5th order IMD products may fall into Rx frequencies of band n258

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.51.3-3 and Table 6.51.3-4 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.51.3-3: 2UL Band 2 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | IMD3 |
| Galileo | 1559 | - | 1591 | YES |  | IMD3 |
| GLONASS | 1591 | - | 1610 | YES |  | IMD3 |
| GPS | 1563 | - | 1587 | YES |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | 3rd order harmonic |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | YES |  | 3rd order harmonic |
|  | 5150 | - | 5825 | YES | Asia | 3rd order harmonic |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | YES | China | 5th IMD |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | YES | Europe | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 57050 | - | 64000 | YES | USA - Canada | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 57000 | - | 64000 | YES | South Korea | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 59000 | - | 66000 | YES | Japan | 3rd, 4th, 5th IMD |
|  | 59000 | - | 64000 | YES | China | 3rd, 4th, 5th IMD |
|  | 59400 | - | 62900 | YES | Australia | 3rd, 4th, 5th IMD |

Table 6.51.3-4: 2UL Band 5 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | YES | US/Europe | 3rd order harmonic |
|  | 2400 | - | 2494 | YES | Asia | 3rd order harmonic |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | YES | Europe | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 57050 | - | 64000 | YES | USA - Canada | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 57000 | - | 64000 | YES | South Korea | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 59000 | - | 66000 | YES | Japan | 3rd, 4th, 5th IMD |
|  | 59000 | - | 64000 | YES | China | 3rd, 4th, 5th IMD |
|  | 59400 | - | 62900 | YES | Australia | 3rd, 4th, 5th IMD |

Table 6.51.3-5 and Table 6.51.3-6 list the protected bands required for the dual connectivity configuration.

Table 6.51.3-5: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_2A-n257A | Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 50, 51, 66, 70, 71, 74, 74, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | x |
|  | Band 43 | FDL_low | - | FDL_high | -50 | 1 | 1* |
| * Table 6.5.3.2-1 of TS38.101 Note x: NOTE 15 in Table 6.6.3.2-1 of 36.101 [2] will be applied here. Additional requirements for the frequency ranges that are less than FOOB (MHz) in Table 6.51.3.1-1 need further study. |  |  |  |  |  |  |  |

Table 6.51.3-6: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_5A-n257A | Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 74, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 26 | FDL_low | - | FDL_high | -27 | 1 |  |
|  | Band 41 | FDL_low | - | FDL_high | -50 | 1 | 1* |
| * Table 6.5.3.2-1 of TS38.101 |  |  |  |  |  |  |  |

### 6.51.4	∆TIB and ∆RIB values

For DC_2A-5A-n257A, the TIB,c and RIB values are given in the tables below.

Table 6.51.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2A-5A-n257A | 2 | 0.3 |
|  | 5 | 0.3 |
|  | n257 | 0 |

Table 6.51.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2A-5A-n257A | 2 | 0 |
|  | 5 | 0 |
|  | n257 | 0 |

### 6.51.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.52	DC_2A-13A-n257A

### 6.52.1	Operating bands for DC

Table 6.52.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2A-13A-n257A | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.52.2	Channel bandwidths per operating band for DC

Table 6.52.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |  |
| DC_2A-13A-n257A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 230 |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 430 |  |

### 6.52.3	Co-existence studies

Table 6.52.3-1 lists Band 2A-13A +Band n257A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.52.3-1: Band 2 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 27650 | 24590 | 28350 | 31410 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 25800 | 22680 | 51090 | 57150 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 30200 | 33320 | 54850 | 60910 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 1450 | 2310 | 26480 | 29520 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 55300 | 49180 | 56700 | 62820 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 23950 | 20770 | 77590 | 86650 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 32050 | 35230 | 81350 | 90410 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 116150 | 104090 | 18860 | 22100 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 107850 | 119910 | 33900 | 37140 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 84800 | 75680 | 47270 | 53450 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 83200 | 92320 | 58550 | 64730 |

Based on Table 6.52.3-1

-	2nd order harmonics may fall into LTE frequencies of Band 43

-	3rd order harmonic may fall into LTE frequencies of Band 46

-	2nd order IMD products may fall into Rx frequencies of band n257 and n258

-	3rd order IMD products may fall into Rx frequencies of Band 1, 3, 4, 9, 10, 11, 21, 23, 24, 25, 32, 33, 34, 35, 36, 37, 39, 40, 45, 50, 65, 66, 70, 74,75 and band n257, n258

-	3rd order IMD products may fall into own Rx frequencies of Band 2 and n257

-	5th order IMD products may fall into Rx frequencies of band n260

Table 6.52.3-2: Band 13 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 777 | 787 | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1554 | 1574 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2331 | 2361 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 28723 | 25713 | 27277 | 30287 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 27946 | 24926 | 52213 | 58223 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 28054 | 31074 | 53777 | 59787 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 377 | 1187 | 26490 | 29510 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | -57446 | -51426 | 54554 | 60574 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | -27169 | -24139 | 78713 | 87723 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 28831 | 31861 | 80277 | 89287 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | -117223 | -105213 | 23352 | 26392 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 106777 | 118787 | 29608 | 32648 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | -86946 | -77926 | 50639 | 56669 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 81054 | 90074 | 55331 | 61361 |

Based on Table 6.52.3-2

-	2nd order harmonics may fall into frequencies of band 24

-	3rd order harmonic may fall into frequencies of band 30 and 40

-	2nd order IMD products may fall into Rx frequencies of band n257 and n258

-	3rd order IMD products may fall into Rx frequencies of Band 5, 6, 8, 12, 14, 17, 18, 19, 20, 26, 27, 28, 29, 31, 44, 67, 68, 71, 72, 73 and band n258

-	3rd order IMD products may fall into own Rx frequencies of Band 13, band n257

-	4th order IMD products may fiall into Rx frequencies of band n257 and n258

-	5th order IMD products may fall into Rx frequencies of band n258

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.52.3-3 and Table 6.52.3-4 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.52.3-3: 2UL Band 2 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | IMD3 |
| Galileo | 1559 | - | 1591 | YES |  | IMD3 |
| GLONASS | 1591 | - | 1610 | YES |  | IMD3 |
| GPS | 1563 | - | 1587 | YES |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | 3rd order harmonic |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | YES |  | 3rd order harmonic |
|  | 5150 | - | 5825 | YES | Asia | 3rd order harmonic |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | YES | China | 5th IMD |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | YES | Europe | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 57050 | - | 64000 | YES | USA - Canada | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 57000 | - | 64000 | YES | South Korea | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 59000 | - | 66000 | YES | Japan | 3rd, 4th, 5th IMD |
|  | 59000 | - | 64000 | YES | China | 3rd, 4th, 5th IMD |
|  | 59400 | - | 62900 | YES | Australia | 3rd, 4th, 5th IMD |

Table 6.52.3-4: 2UL Band 13 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | 2nd harmonics |
| Galileo | 1559 | - | 1591 | YES |  | 2nd harmonics |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | YES |  | 2nd harmonics |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | YES | Europe | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 57050 | - | 64000 | YES | USA - Canada | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 57000 | - | 64000 | YES | South Korea | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 59000 | - | 66000 | YES | Japan | 3rd, 4th, 5th IMD |
|  | 59000 | - | 64000 | YES | China | 3rd, 4th, 5th IMD |
|  | 59400 | - | 62900 | YES | Australia | 3rd, 4th, 5th IMD |

Table 6.52.3-5 and Table 6.52.3-6 list the protected bands required for the dual connectivity configuration.

Table 6.52.3-5: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_2A-n257A | Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 50, 51, 66, 70, 71, 74, 74, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | x |
|  | Band 43 | FDL_low | - | FDL_high | -50 | 1 | 1* |
| * Table 6.5.3.2-1 of TS38.101 Note: NOTE 15 in Table 6.6.3.2-1 of 36.101 [2] will be applied here. Additional requirements for the frequency ranges that are less than FOOB (MHz) in Table 6.52.3.1-1 need further study. |  |  |  |  |  |  |  |

Table 6.52.3-6: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_13A-n257A | Band 2, 4, 5, 10, 12, 13, 17, 25, 26, 27, 29, 41, 48, 50, 51, 66, 70, 71, 74, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 14 | FDL_low | - | FDL_high | -50 | 1 | x |
|  | Band 24, 30 | FDL_low | - | FDL_high | -50 | 1 | 1* |
| * Table 6.5.3.2-1 of TS38.101 Note: NOTE 15 in Table 6.6.3.2-1 of 36.101 [2] will be applied here. Additional requirements for the frequency ranges that are less than FOOB (MHz) in Table 6.52.3.1-1 need further study. |  |  |  |  |  |  |  |

### 6.52.4	∆TIB and ∆RIB values

For DC_2A-13A-n257A, the TIB,c and RIB values are given in the tables below.

Table 6.52.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2A-13A-n257A | 2 | 0.3 |
|  | 13 | 0.3 |
|  | n257 | 0 |

Table 6.52.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2A-13A-n257A | 2 | 0 |
|  | 13 | 0 |
|  | n257 | 0 |

### 6.52.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.53	DC_2A-66A-n257A

### 6.53.1	Operating bands for DC

Table 6.53.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2A-66A-n257A | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 66 | 1710MHz |  | 1780MHz | 2110MHz |  | 2200MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.53.2	Channel bandwidths per operating band for DC

Table 6.53.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |  |
| DC_2A-66A-n257A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 240 |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 440 |  |

### 6.53.3	Co-existence studies

Table 6.53.3-1 lists Band 2A-66A+Band n257A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.53.3-1: Band 2 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 27650 | 24590 | 28350 | 31410 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 25800 | 22680 | 51090 | 57150 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 30200 | 33320 | 54850 | 60910 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 1450 | 2310 | 26480 | 29520 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 55300 | 49180 | 56700 | 62820 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 23950 | 20770 | 77590 | 86650 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 32050 | 35230 | 81350 | 90410 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 116150 | 104090 | 18860 | 22100 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 107850 | 119910 | 33900 | 37140 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 84800 | 75680 | 47270 | 53450 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 83200 | 92320 | 58550 | 64730 |

Based on Table 6.53.3-1

-	2nd order harmonics may fall into LTE frequencies of Band 43

-	3rd order harmonic may fall into LTE frequencies of Band 46

-	2nd order IMD products may fall into Rx frequencies of band n257 and n258

-	3rd order IMD products may fall into Rx frequencies of Band 1, 3, 4, 9, 10, 11, 21, 23, 24, 25, 32, 33, 34, 35, 36, 37, 39, 40, 45, 50, 65, 66, 70, 74,75 and band n257, n258

-	3rd order IMD products may fall into own Rx frequencies of Band 2 and n257

-	5th order IMD products may fall into Rx frequencies of band n260

Table 6.53.3-2: Band 66 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 27790 | 24720 | 28210 | 31280 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 26080 | 22940 | 51220 | 57290 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 29920 | 33060 | 54710 | 60780 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 1310 | 2180 | 26480 | 29520 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 55580 | 49440 | 56420 | 62560 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 24370 | 21160 | 77720 | 86790 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 31630 | 34840 | 81210 | 90280 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 116290 | -104220 | 19380 | 22660 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 107710 | 119780 | 33340 | 36620 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 85080 | 75940 | 47660 | 53870 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 82920 | 92060 | 58130 | 64340 |

Based on Table 6.53.3-2;

-	2nd order harmonics may fall into LTE frequencies of Band 22, 42 and 48

-	3rd order harmonic may fall into LTE frequencies of Band 46

-	2nd order IMD products may fall into own Rx frequencies of band n257 and n258

-	3rd order IMD products may fall into Rx frequencies of Band 1, 2, 3, 4, 9, 10, 11, 21, 23, 24, 25, 32, 33, 34, 35, 36, 37, 39, 45, 50, 51, 65, 66, 70, 74, 75 and 76

-	3rd order IMD products may fall into own Rx frequencies of  band n257 and n258

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.53.3-3 and Table 6.53.3-4 list if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.53.3-3: 2UL Band 2 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | IMD3 |
| Galileo | 1559 | - | 1591 | YES |  | IMD3 |
| GLONASS | 1591 | - | 1610 | YES |  | IMD3 |
| GPS | 1563 | - | 1587 | YES |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | 3rd order harmonic |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | YES |  | 3rd order harmonic |
|  | 5150 | - | 5825 | YES | Asia | 3rd order harmonic |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | YES | China | 5th IMD |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | YES | Europe | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 57050 | - | 64000 | YES | USA - Canada | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 57000 | - | 64000 | YES | South Korea | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 59000 | - | 66000 | YES | Japan | 3rd, 4th, 5th IMD |
|  | 59000 | - | 64000 | YES | China | 3rd, 4th, 5th IMD |
|  | 59400 | - | 62900 | YES | Australia | 3rd, 4th, 5th IMD |

Table 6.53.3-4: 2UL Band 66 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | IMD3 |
| Galileo | 1559 | - | 1591 | YES |  | IMD3 |
| GLONASS | 1591 | - | 1610 | YES |  | IMD3 |
| GPS | 1563 | - | 1587 | YES |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | 3rd order harmonic |
|  | 5150 | - | 5350 | YES | Europe | 3rd order harmonic |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | YES | Asia | 3rd order harmonic |

Table 6.53.3-5 and Table 6.53.3-6 list the protected bands required for the dual connectivity configuration.

Table 6.53.3-5: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_2A-n257A | Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 50, 51, 66, 70, 71, 74, 74, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | x |
|  | Band 43 | FDL_low | - | FDL_high | -50 | 1 | 1* |
| Note 1: * Table 6.5.3.2-1 of TS38.101 Note 2: NOTE 15 in Table 6.6.3.2-1 of 36.101 [2] will be applied here. Additional requirements for the frequency ranges that are less than FOOB (MHz) in Table 6.53.3.1-1 need further study. |  |  |  |  |  |  |  |

Table 6.53.3-6: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_66A-n257A | Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 42, 48 | FDL_low | - | FDL_high | -50 | 1 | 1* |
| Note : *Table 6.5.3.2-1 of TS38.101 |  |  |  |  |  |  |  |

### 6.53.4	∆TIB and ∆RIB values

For DC_2A-66A-n257A, the TIB,c and RIB values are given in the tables below

Table 6.53.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2A-66A-n257A | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n257 | 0 |

Table 6.53.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2A-66A-n257A | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n257 | 0 |

### 6.53.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.54	DC_2C-n257A

### 6.54.1	Operating bands for DC

Table 6.54.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2C-n257A | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.54.2	Channel bandwidths per operating band for DC

Table 6.54.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |  |
| DC_2C-n257 | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 240 |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 440 |  |

### 6.54.3	Co-existence studies

Table 6.54.3-1 lists Band 2C +Band n257A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.54.3-1: Band 2 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 27650 | 24590 | 28350 | 31410 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 25800 | 22680 | 51090 | 57150 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 30200 | 33320 | 54850 | 60910 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 1450 | 2310 | 26480 | 29520 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 55300 | 49180 | 56700 | 62820 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 23950 | 20770 | 77590 | 86650 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 32050 | 35230 | 81350 | 90410 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 116150 | 104090 | 18860 | 22100 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 107850 | 119910 | 33900 | 37140 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 84800 | 75680 | 47270 | 53450 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 83200 | 92320 | 58550 | 64730 |

Based on Table 6.54.3-1

-	2nd order harmonics may fall into LTE frequencies of Band 43

-	3rd order harmonic may fall into LTE frequencies of Band 46

-	2nd order IMD products may fall into Rx frequencies of band n257 and n258

-	3rd order IMD products may fall into Rx frequencies of Band 1, 3, 4, 9, 10, 11, 21, 23, 24, 25, 32, 33, 34, 35, 36, 37, 39, 40, 45, 50, 65, 66, 70, 74,75 and band n257, n258

-	3rd order IMD products may fall into own Rx frequencies of Band 2 and n257

-	5th order IMD products may fall into Rx frequencies of band n260

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.54.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.54.3-2: 2UL Band 2 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | IMD3 |
| Galileo | 1559 | - | 1591 | YES |  | IMD3 |
| GLONASS | 1591 | - | 1610 | YES |  | IMD3 |
| GPS | 1563 | - | 1587 | YES |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | 3rd order harmonic |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | YES |  | 3rd order harmonic |
|  | 5150 | - | 5825 | YES | Asia | 3rd order harmonic |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | YES | China | 5th IMD |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | YES | Europe | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 57050 | - | 64000 | YES | USA - Canada | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 57000 | - | 64000 | YES | South Korea | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 59000 | - | 66000 | YES | Japan | 3rd, 4th, 5th IMD |
|  | 59000 | - | 64000 | YES | China | 3rd, 4th, 5th IMD |
|  | 59400 | - | 62900 | YES | Australia | 3rd, 4th, 5th IMD |

Table 6.54.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.54.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_2C-n257A | Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 50, 51, 66, 70, 71, 74, 74, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | x |
|  | Band 43 | FDL_low | - | FDL_high | -50 | 1 | 1* |
| Note 1: * Table 6.5.3.2-1 of TS38.101 Note 2: NOTE 15 in Table 6.6.3.2-1 of 36.101 [2] will be applied here. Additional requirements for the frequency ranges that are less than FOOB (MHz) in Table 6.54.3.1-1 need further study. |  |  |  |  |  |  |  |

### 6.54.4	∆TIB and ∆RIB values

For DC_2C-n257A, the TIB,c and RIB values are given in the tables below.

Table 6.54.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2C-n257A | 2 | 0 |
|  | n257 | 0 |

Table 6.54.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2C-n257A | 2 | 0 |
|  | n257 | 0 |

### 6.54.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.55	DC_5B-n257A

### 6.55.1	Operating bands for DC

Table 6.55.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5B-n257A | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.55.2	Channel bandwidths per operating band for DC

Table 6.55.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |  |
| DC_5B-n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 220 |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 420 |  |

### 6.55.3	Co-existence studies

Table 6.55.3-1 lists Band 5B +Band n257A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.55.3-1: Band 5 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 28676 | 25651 | 27324 | 30349 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 27852 | 24802 | 52151 | 58176 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 28148 | 31198 | 53824 | 59849 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 424 | 1249 | 26490 | 29510 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 57352 | 51302 | 54648 | 60698 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 27028 | 23953 | 78651 | 87676 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 28972 | 32047 | 80324 | 89349 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 117176 | 105151 | 23104 | 26204 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 106824 | 118849 | 29796 | 32896 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 86852 | 77802 | 50453 | 56528 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 81148 | 90198 | 55472 | 61547 |

Based on Table 6.55.3-1

-	3rd order harmonic may fall into frequencies of band n41

-	2nd order IMD products may fall into own Rx frequencies of band n257 and n258

-	3rd order IMD products may fall into Rx frequencies of Band 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27 , 28 , 29, 31, 44, 67, 68, 71, 72 and 73

-	3rd order IMD products may fall into Rx frequencies of band n257 and n258

-	4th order IMD products may fall into Rx frequencies of band n257 and n258

-	5th order IMD products may fall into Rx frequencies of band n258

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.55.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.55.3-2: 2UL Band 5 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | YES | US/Europe | 3rd order harmonic |
|  | 2400 | - | 2494 | YES | Asia | 3rd order harmonic |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | YES | Europe | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 57050 | - | 64000 | YES | USA - Canada | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 57000 | - | 64000 | YES | South Korea | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 59000 | - | 66000 | YES | Japan | 3rd, 4th, 5th IMD |
|  | 59000 | - | 64000 | YES | China | 3rd, 4th, 5th IMD |
|  | 59400 | - | 62900 | YES | Australia | 3rd, 4th, 5th IMD |

Table 6.55.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.55.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_5B-n257A | Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 74, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 26 | FDL_low | - | FDL_high | -27 | 1 |  |
|  | Band 41 | FDL_low | - | FDL_high | -50 | 1 | 1* |
| Note: * Table 6.5.3.2-1 of TS38.101 |  |  |  |  |  |  |  |

### 6.55.4	∆TIB and ∆RIB values

For DC_5B-n257A, the TIB,c and RIB values are given in the tables below.

Table 6.55.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5B-n257A | 5 | 0 |
|  | n257 | 0 |

Table 6.55.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5B-n257A | 5 | 0 |
|  | n257 | 0 |

### 6.55.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.56	DC_5A-5A-n257A

### 6.56.1	Operating bands for DC

Table 6.56.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5A-5A-n257A | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.56.2	Channel bandwidths per operating band for DC

Table 6.56.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |  |
| DC_5A-5A-n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 220 |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 420 |  |

### 6.56.3	Co-existence studies

Table 6.56.3-1 lists Band 5A-5A +Band n257A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.56.3-1: Band 5 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 28676 | 25651 | 27324 | 30349 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 27852 | 24802 | 52151 | 58176 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 28148 | 31198 | 53824 | 59849 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 424 | 1249 | 26490 | 29510 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 57352 | 51302 | 54648 | 60698 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 27028 | 23953 | 78651 | 87676 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 28972 | 32047 | 80324 | 89349 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 117176 | 105151 | 23104 | 26204 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 106824 | 118849 | 29796 | 32896 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 86852 | 77802 | 50453 | 56528 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 81148 | 90198 | 55472 | 61547 |

Based on Table 6.56.3-1

-	3rd order harmonic may fall into frequencies of band n41

-	2nd order IMD products may fall into own Rx frequencies of band n257 and n258

-	3rd order IMD products may fall into Rx frequencies of Band 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27 , 28 , 29, 31, 44, 67, 68, 71, 72 and 73

-	3rd order IMD products may fall into own Rx frequencies of band n257 and n258

-	4th order IMD products may fall into own Rx frequencies of band n257 and n258

-	5th order IMD products may fall into own Rx frequencies of band n258

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.56.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.56.3-2: 2UL Band 5 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | YES | US/Europe | 3rd order harmonic |
|  | 2400 | - | 2494 | YES | Asia | 3rd order harmonic |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | YES | Europe | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 57050 | - | 64000 | YES | USA - Canada | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 57000 | - | 64000 | YES | South Korea | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 59000 | - | 66000 | YES | Japan | 3rd, 4th, 5th IMD |
|  | 59000 | - | 64000 | YES | China | 3rd, 4th, 5th IMD |
|  | 59400 | - | 62900 | YES | Australia | 3rd, 4th, 5th IMD |

Table 6.56.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.56.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_5A-5A-n257A | Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 74, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 26 | FDL_low | - | FDL_high | -27 | 1 |  |
|  | Band 41 | FDL_low | - | FDL_high | -50 | 1 | 1* |
| Note: * Table 6.5.3.2-1 of TS38.101 |  |  |  |  |  |  |  |

### 6.56.4	∆TIB and ∆RIB values

For DC_5A-5A-n257A, the TIB,c and RIB values are given in the tables below.

Table 6.56.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5A-5A-n257A | 5 | 0 |
|  | n257 | 0 |

Table 6.56.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5A-5A-n257A | 5 | 0 |
|  | n257 | 0 |

### 6.56.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.57	DC_5A-66A-n257A

### 6.57.1	Operating bands for DC

Table 6.57.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5A-66A-n257A | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 66 | 1710MHz |  | 1780MHz | 2110MHz |  | 2200MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.57.2	Channel bandwidths per operating band for DC

Table 6.57.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |  |
| DC_5A-66A-n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 230 |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 430 |  |

### 6.57.3	Co-existence studies

Table 6.57.3-1 lists Band 5A-66A+Band n257A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.57.3-1: Band 5 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 28676 | 25651 | 27324 | 30349 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 27852 | 24802 | 52151 | 58176 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 28148 | 31198 | 53824 | 59849 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 424 | 1249 | 26490 | 29510 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 57352 | 51302 | 54648 | 60698 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 27028 | 23953 | 78651 | 87676 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 28972 | 32047 | 80324 | 89349 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 117176 | 105151 | 23104 | 26204 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 106824 | 118849 | 29796 | 32896 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 86852 | 77802 | 50453 | 56528 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 81148 | 90198 | 55472 | 61547 |

Based on Table 6.57.3-1

-	3rd order harmonic may fall into frequencies of band n41

-	2nd order IMD products may fall into own Rx frequencies of band n257 and n258

-	3rd order IMD products may fall into Rx frequencies of Band 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27 , 28 , 29, 31, 44, 67, 68, 71, 72 and 73

-	3rd order IMD products may fall into own Rx frequencies of band n257 and n258

-	4th order IMD products may fall into own Rx frequencies of band n257 and n258

-	5th order IMD products may fall into own Rx frequencies of band n258

Table 6.57.3-2: Band 66 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 27790 | 24720 | 28210 | 31280 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 26080 | 22940 | 51220 | 57290 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 29920 | 33060 | 54710 | 60780 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 1310 | 2180 | 26480 | 29520 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 55580 | 49440 | 56420 | 62560 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 24370 | 21160 | 77720 | 86790 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 31630 | 34840 | 81210 | 90280 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 116290 | -104220 | 19380 | 22660 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 107710 | 119780 | 33340 | 36620 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 85080 | 75940 | 47660 | 53870 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 82920 | 92060 | 58130 | 64340 |

Based on Table 6.57.3-2;

-	2nd order harmonics may fall into LTE frequencies of Band 22, 42 and 48

-	3rd order harmonic may fall into LTE frequencies of Band 46

-	2nd order IMD products may fall into own Rx frequencies of band n257 and n258

-	3rd order IMD products may fall into Rx frequencies of Band 1, 2, 3, 4, 9, 10, 11, 21, 23, 24, 25, 32, 33, 34, 35, 36, 37, 39, 45, 50, 51, 65, 66, 70, 74, 75 and 76

-	3rd order IMD products may fall into own Rx frequencies of  band n257 and n258

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.57.3-3 and Table 6.57.3-4 list if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.57.3-3: 2UL Band 5 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | YES | US/Europe | 3rd order harmonic |
|  | 2400 | - | 2494 | YES | Asia | 3rd order harmonic |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | YES | Europe | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 57050 | - | 64000 | YES | USA - Canada | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 57000 | - | 64000 | YES | South Korea | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 59000 | - | 66000 | YES | Japan | 3rd, 4th, 5th IMD |
|  | 59000 | - | 64000 | YES | China | 3rd, 4th, 5th IMD |
|  | 59400 | - | 62900 | YES | Australia | 3rd, 4th, 5th IMD |

Table 6.57.3-4: 2UL Band 66 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | IMD3 |
| Galileo | 1559 | - | 1591 | YES |  | IMD3 |
| GLONASS | 1591 | - | 1610 | YES |  | IMD3 |
| GPS | 1563 | - | 1587 | YES |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | 3rd order harmonic |
|  | 5150 | - | 5350 | YES | Europe | 3rd order harmonic |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | YES | Asia | 3rd order harmonic |

Table 6.57.3-5 and Table 6.57.3-6 list the protected bands required for the dual connectivity configuration.

Table 6.57.3-5: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_5A-n257A | Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 74, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 26 | FDL_low | - | FDL_high | -27 | 1 |  |
|  | Band 41 | FDL_low | - | FDL_high | -50 | 1 | 1* |
| * Table 6.5.3.2-1 of TS38.101 |  |  |  |  |  |  |  |

Table 6.57.3-6: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_66A-n257A | Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |

### 6.57.4	∆TIB and ∆RIB values

For DC_5A-66A-n257A, the TIB,c and RIB values are given in the tables below.

Table 6.57.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5A-66A-n257A | 5 | 0.3 |
|  | 66 | 0.3 |
|  | n257 | 0 |

Table 6.57.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5A-66A-n257A | 5 | 0 |
|  | 66 | 0 |
|  | n257 | 0 |

### 6.57.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.58	DC_13A-66A-n257A

### 6.58.1	Operating bands for DC

Table 6.58.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_13A-66A-n257A | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | 66 | 1710MHz |  | 1780MHz | 2110MHz |  | 2200MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.58.2	Channel bandwidths per operating band for DC

Table 6.58.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |  |
| DC_13A-66A-n257A | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 230 |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 430 |  |

### 6.58.3	Co-existence studies

Table 6.58.3-1 lists Band 13A-66A+Band n257A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.58.3-1: Band 13 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 777 | 787 | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1554 | 1574 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2331 | 2361 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 28723 | 25713 | 27277 | 30287 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 27946 | 24926 | 52213 | 58223 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 28054 | 31074 | 53777 | 59787 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 377 | 1187 | 26490 | 29510 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | -57446 | -51426 | 54554 | 60574 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | -27169 | -24139 | 78713 | 87723 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 28831 | 31861 | 80277 | 89287 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | -117223 | -105213 | 23352 | 26392 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 106777 | 118787 | 29608 | 32648 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | -86946 | -77926 | 50639 | 56669 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 81054 | 90074 | 55331 | 61361 |

Based on Table 6.58.3-1

-	2nd order harmonics may fall into frequencies of band 24

-	3rd order harmonic may fall into frequencies of band 30 and 40

-	2nd order IMD products may fall into Rx frequencies of band n257 and n258

-	3rd order IMD products may fall into Rx frequencies of Band 5, 6, 8, 12, 14, 17, 18, 19, 20, 26, 27, 28, 29, 31, 44, 67, 68, 71, 72, 73 and band n258

-	3rd order IMD products may fall into own Rx frequencies of Band 13, band n257

-	4th order IMD products may fall into Rx frequencies of band n257 and n258

-	5th order IMD products may fall into Rx frequencies of band n258

Table 6.58.3-2: Band 66 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 27790 | 24720 | 28210 | 31280 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 26080 | 22940 | 51220 | 57290 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 29920 | 33060 | 54710 | 60780 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 1310 | 2180 | 26480 | 29520 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 55580 | 49440 | 56420 | 62560 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 24370 | 21160 | 77720 | 86790 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 31630 | 34840 | 81210 | 90280 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 116290 | -104220 | 19380 | 22660 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 107710 | 119780 | 33340 | 36620 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 85080 | 75940 | 47660 | 53870 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 82920 | 92060 | 58130 | 64340 |

Based on Table 6.58.3-2;

-	2nd order harmonics may fall into LTE frequencies of Band 22, 42 and 48

-	3rd order harmonic may fall into LTE frequencies of Band 46

-	2nd order IMD products may fall into own Rx frequencies of band n257 and n258

-	3rd order IMD products may fall into Rx frequencies of Band 1, 2, 3, 4, 9, 10, 11, 21, 23, 24, 25, 32, 33, 34, 35, 36, 37, 39, 45, 50, 51, 65, 66, 70, 74, 75 and 76

-	3rd order IMD products may fall into own Rx frequencies of  band n257 and n258

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.58.3-3 and Table 6.58.3-4 list if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.58.3-2: 2UL Band 13 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | 2nd harmonics |
| Galileo | 1559 | - | 1591 | YES |  | 2nd harmonics |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | YES |  | 2nd harmonics |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | YES | Europe | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 57050 | - | 64000 | YES | USA - Canada | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 57000 | - | 64000 | YES | South Korea | 2nd order harmonic 3rd, 4th, 5th IMD |
|  | 59000 | - | 66000 | YES | Japan | 3rd, 4th, 5th IMD |
|  | 59000 | - | 64000 | YES | China | 3rd, 4th, 5th IMD |
|  | 59400 | - | 62900 | YES | Australia | 3rd, 4th, 5th IMD |

Table 6.58.3-4: 2UL Band 66 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | IMD3 |
| Galileo | 1559 | - | 1591 | YES |  | IMD3 |
| GLONASS | 1591 | - | 1610 | YES |  | IMD3 |
| GPS | 1563 | - | 1587 | YES |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | 3rd order harmonic |
|  | 5150 | - | 5350 | YES | Europe | 3rd order harmonic |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | YES | Asia | 3rd order harmonic |

Table 6.58.3-5 and Table 6.58.3-6 list the protected bands required for the dual connectivity configuration.

Table 6.58.3-5: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_13A-n257A | Band 2, 4, 5, 10, 12, 13, 17, 25, 26, 27, 29, 41, 48, 50, 51, 66, 70, 71, 74, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 14 | FDL_low | - | FDL_high | -50 | 1 | x |
|  | Band 24, 30 | FDL_low | - | FDL_high | -50 | 1 | 1* |
| Note 1: * Table 6.5.3.2-1 of TS38.101 Note 2: NOTE 15 in Table 6.6.3.2-1 of 36.101 [2] will be applied here. Additional requirements for the frequency ranges that are less than FOOB (MHz) in Table 6.58.3.1-1 need further study. |  |  |  |  |  |  |  |

Table 6.58.3-6: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_66A-n257A | Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 42, 48 | FDL_low | - | FDL_high | -50 | 1 | 1* |
| Note : * Table 6.5.3.2-1 of TS38.101 |  |  |  |  |  |  |  |

### 6.58.4	∆TIB and ∆RIB values

For DC_13A-66A -n257A, the TIB,c and RIB values are given in the tables below.

Table 6.58.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13A-66A-n257A | 13 | 0 |
|  | 66 | 0 |
|  | n257 | 0 |

Table 6.58.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_13A-66A-n257A | 13 | 0 |
|  | 66 | 0 |
|  | n257 | 0 |

### 6.58.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.59	DC_48A-48A-n257A

### 6.59.1	Operating bands for DC

Table 6.59.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_48A-48A-n257A | 48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.59.2	Channel bandwidths per operating band for DC

Table 6.59.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_48A-48A-n257A | 48 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 240 |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 440 |

### 6.59.3	Co-existence studies

Table 6.59.3-1 lists Band 48A-48A +Band n257A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.59.3-1: Band 48 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 3550 | 3700 | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 7100 | 7400 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 10650 | 11100 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 25950 | 22800 | 30050 | 33200 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 22400 | 19100 | 49300 | 55450 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 33600 | 36900 | 56550 | 62700 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 3150 | 4100 | 26480 | 29520 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 51900 | 45600 | 60100 | 66400 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 18850 | 15400 | 75800 | 84950 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 37150 | 40600 | 83050 | 92200 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 114450 | 102300 | 11700 | 15300 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 109550 | 121700 | 40700 | 44300 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 81400 | 72100 | 41900 | 48350 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 86600 | 95900 | 63650 | 70100 |

Based on Table 6.59.3-1

-	2nd order IMD products may fall into frequencies of band n258

-	3rd order IMD products may fall into frequencies of band 22, 42, 43 and 48

-	3rd order IMD products may fall into own Rx frequencies of band n257 and n258

-	4th order IMD products may fall into frequencies of band n260

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.59.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.59.3-2: 2UL Band 48 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.59.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.59.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_48A-48A-n257A | Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |

### 6.59.4	∆TIB and ∆RIB values

For DC_48A-48A-n257A, the TIB,c and RIB values are given in the tables below.

Table 6.59.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_48A-48A-n257A | 48 | 0 |
|  | n257 | 0 |

Table 6.59.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_48A-48A-n257A | 48 | 0 |
|  | n257 | 0 |

### 6.59.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.60	DC_48C-n257A

### 6.60.1	Operating bands for DC

Table 6.60.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_48C-n257A | 48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.60.2	Channel bandwidths per operating band for DC

Table 6.60.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |  |
| DC_48C-n257A | 48 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 240 |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 440 |  |

### 6.60.3	Co-existence studies

Table 6.60.3-1 lists Band 48C +Band n257A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.60.3-1: Band 48 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 3550 | 3700 | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 7100 | 7400 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 10650 | 11100 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 25950 | 22800 | 30050 | 33200 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 22400 | 19100 | 49300 | 55450 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 33600 | 36900 | 56550 | 62700 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 3150 | 4100 | 26480 | 29520 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 51900 | 45600 | 60100 | 66400 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 18850 | 15400 | 75800 | 84950 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 37150 | 40600 | 83050 | 92200 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 114450 | 102300 | 11700 | 15300 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 109550 | 121700 | 40700 | 44300 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 81400 | 72100 | 41900 | 48350 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 86600 | 95900 | 63650 | 70100 |

Based on Table 6.60.3-1

-	2nd order IMD products may fall into frequencies of band n258

-	3rd order IMD products may fall into frequencies of band 22, 42, 43 and 48

-	3rd order IMD products may fall into own Rx frequencies of band n257 and n258

-	4th order IMD products may fall into frequencies of band n260

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.60.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.60.3-2: 2UL Band 48 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.60.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.60.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_48C-n257A | Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |

### 6.60.4	∆TIB and ∆RIB values

For DC_48C-n257A, the TIB,c and RIB values are given in the tables below.

Table 6.60.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_48C-n257A | 48 | 0 |
|  | n257 | 0 |

Table 6.60.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_48C-n257A | 48 | 0 |
|  | n257 | 0 |

### 6.60.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.61	DC_66A-66A_n257A

### 6.61.1	Operating bands for DC

Table 6.61.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66A-66A-n257A | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2220 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.61.2	Channel bandwidths per operating band for DC

Table 6.61.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |  |
| DC_66A-66A-n257A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 240 |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 440 |  |

### 6.61.3	Co-existence studies

Table 6.61.3-1 lists Band 66A+66A +Band n257A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.61.3-1: Band 66 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 27790 | 24720 | 28210 | 31280 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 26080 | 22940 | 51220 | 57290 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 29920 | 33060 | 54710 | 60780 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 1310 | 2180 | 26480 | 29520 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 55580 | 49440 | 56420 | 62560 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 24370 | 21160 | 77720 | 86790 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 31630 | 34840 | 81210 | 90280 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 116290 | -104220 | 19380 | 22660 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 107710 | 119780 | 33340 | 36620 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 85080 | 75940 | 47660 | 53870 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 82920 | 92060 | 58130 | 64340 |

Based on Table 6.61.3-1;

-	2nd order harmonics may fall into LTE frequencies of Band 22, 42 and 48

-	3rd order harmonic may fall into LTE frequencies of Band 46

-	2nd order IMD products may fall into own Rx frequencies of band n257 and n258

-	3rd order IMD products may fall into Rx frequencies of Band 1, 2, 3, 4, 9, 10, 11, 21, 23, 24, 25, 32, 33, 34, 35, 36, 37, 39, 45, 50, 51, 65, 66, 70, 74, 75 and 76

-	3rd order IMD products may fall into own Rx frequencies of  band n257 and n258

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.61.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.61.3-2: 2UL Band 66 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | IMD3 |
| Galileo | 1559 | - | 1591 | YES |  | IMD3 |
| GLONASS | 1591 | - | 1610 | YES |  | IMD3 |
| GPS | 1563 | - | 1587 | YES |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | 3rd order harmonic |
|  | 5150 | - | 5350 | YES | Europe | 3rd order harmonic |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | YES | Asia | 3rd order harmonic |

Table 6.61.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.61.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_66A-n257A | Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 42, 48 | FDL_low | - | FDL_high | -50 | 1 | 1* |
| * Table 6.5.3.2-1 of TS38.101 |  |  |  |  |  |  |  |

### 6.61.4	∆TIB and ∆RIB values

For DC_66A-66A-n257A, the TIB,c and RIB values are given in the tables below.

Table 6.61.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66A-66A-n257A | 66 | 0 |
|  | n257 | 0 |

Table 6.61.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66A-66A-n257A | 66 | 0 |
|  | n257 | 0 |

### 6.61.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.62	DC_66C-n257A

### 6.62.1	Operating bands for DC

Table 6.62.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66C-n257A | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2220 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.62.2	Channel bandwidths per operating band for DC

Table 6.62.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |  |
| DC_66C-n257A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 240 |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 440 |  |

### 6.62.3	Co-existence studies

Table 6.62.3-1 lists Band 66C +Band n257A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.62.3-1: Band 66 and Band n257 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 27790 | 24720 | 28210 | 31280 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 26080 | 22940 | 51220 | 57290 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 29920 | 33060 | 54710 | 60780 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 1310 | 2180 | 26480 | 29520 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 55580 | 49440 | 56420 | 62560 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 24370 | 21160 | 77720 | 86790 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 31630 | 34840 | 81210 | 90280 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 116290 | -104220 | 19380 | 22660 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 107710 | 119780 | 33340 | 36620 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 85080 | 75940 | 47660 | 53870 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 82920 | 92060 | 58130 | 64340 |

Based on Table 6.62.3-1;

-	2nd order harmonics may fall into LTE frequencies of Band 22, 42 and 48

-	3rd order harmonic may fall into LTE frequencies of Band 46

-	2nd order IMD products may fall into own Rx frequencies of band n257 and n258

-	3rd order IMD products may fall into Rx frequencies of Band 1, 2, 3, 4, 9, 10, 11, 21, 23, 24, 25, 32, 33, 34, 35, 36, 37, 39, 45, 50, 51, 65, 66, 70, 74, 75 and 76

-	3rd order IMD products may fall into own Rx frequencies of  band n257 and n258

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.62.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.62.3-2: 2UL Band 66 +Band n257 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | IMD3 |
| Galileo | 1559 | - | 1591 | YES |  | IMD3 |
| GLONASS | 1591 | - | 1610 | YES |  | IMD3 |
| GPS | 1563 | - | 1587 | YES |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | 3rd order harmonic |
|  | 5150 | - | 5350 | YES | Europe | 3rd order harmonic |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | YES | Asia | 3rd order harmonic |

Table 6.62.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.62.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_66A-n257A | Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 42, 48 | FDL_low | - | FDL_high | -50 | 1 | 1* |
| * Table 6.5.3.2-1 of TS38.101 |  |  |  |  |  |  |  |

### 6.62.4	∆TIB and ∆RIB values

For DC_66C-n257A, the TIB,c and RIB values are given in the tables below.

Table 6.62.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66C-n257A | 66 | 0 |
|  | n257 | 0 |

Table 6.62.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66C-n257A | 66 | 0 |
|  | n257 | 0 |

### 6.62.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.63	DC_66A-66A-n260A

### 6.63.1	Operating bands for DC

Table 6.63.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66A-66A-n260A | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

### 6.63.2	Channel bandwidths per operating band for DC

Table 6.63.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |  |
| DC_66A-66A-n260A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 240 |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 440 |  |
| DC_66A-66A_n260G | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 240 |  |
|  | n260 | 60 | See CA_n260G Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_66A-66A_n260H | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 340 |  |
|  | n260 | 60 | See CA_n260H Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_66A-66A_n260I | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 440 |  |
|  | n260 | 60 | See CA_n260I Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_66A-66A_n260J | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 540 |  |
|  | n260 | 60 | See CA_n260J Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_66A-66A_n260K | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 640 |  |
|  | n260 | 60 | See CA_n260K Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_66A-66A_n260L | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 740 |  |
|  | n260 | 60 | See CA_n260L Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_66A-66A_n260M | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 840 |  |
|  | n260 | 60 | See CA_n260M Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.63.3	Co-existence studies

Table 6.63.3-1 lists Band 66A-66A +Band n260A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.63.3-1: Band 66 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 111000 | 120000 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 38290 | 35220 | 38710 | 41780 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 36580 | 33440 | 72220 | 78290 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 40420 | 43560 | 75710 | 81780 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 1310 | 2180 | 36980 | 40020 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 76580 | 70440 | 77420 | 83560 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 34870 | 31660 | 109220 | 118290 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 42130 | 45340 | 112710 | 121780 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 158290 | 146220 | 29880 | 33160 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 149710 | 161780 | 43840 | 47120 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 116580 | 107440 | 68660 | 74870 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 114420 | 123560 | 79130 | 85340 |

Based on Table 6.63.3-1

-	2nd order harmonics may fall into LTE frequencies of Band 22, 42, 48

-	3rd order harmonic may fall into LTE frequencies of Band 46

-	2nd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into Rx frequencies of Band 1, 2, 3, 4, 9, 10, 11, 21, 23, 24, 25, 32, 33, 34, 35, 36, 37, 39, 45, 50, 51, 65, 70, 74, 75, and 76

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.63.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.63.3-2: 2UL Band 66+Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | IMD3 |
| Galileo | 1559 | - | 1591 | YES |  | IMD3 |
| GLONASS | 1591 | - | 1610 | YES |  | IMD3 |
| GPS | 1563 | - | 1587 | YES |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | 3rd harmonics |
|  | 5150 | - | 5350 | YES | Europe | 3rd harmonics |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | YES | Asia | 3rd harmonics |

Table 6.63.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.63.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_66A-66A-n260A | Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 42, 48 | FDL_low | - | FDL_high | -50 | 1 | 1* |
| * Table 6.5.3.2-1 of TS38.101 |  |  |  |  |  |  |  |

### 6.63.4	∆TIB and ∆RIB values

For DC_66A-66A-n260A, the TIB,c and RIB values are given in the tables below.

Table 6.63.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66A-66A-n260A | 66 | 0 |
|  | n260 | 0 |

Table 6.63.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66A-66A-n260A | 66 | 0 |
|  | n260 | 0 |

### 6.63.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.64	DC_66C-n260A

### 6.64.1	Operating bands for DC

Table 6.64.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66C-n260A | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

### 6.64.2	Channel bandwidths per operating band for DC

Table 6.64.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |  |
| DC_66C-n260A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 240 |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 440 |  |

### 6.64.3	Co-existence studies

Table 6.64.3-1 lists Band 66C +Band n260A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.64.3-1: Band 66 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 111000 | 120000 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 38290 | 35220 | 38710 | 41780 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 36580 | 33440 | 72220 | 78290 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 40420 | 43560 | 75710 | 81780 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 1310 | 2180 | 36980 | 40020 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 76580 | 70440 | 77420 | 83560 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 34870 | 31660 | 109220 | 118290 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 42130 | 45340 | 112710 | 121780 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 158290 | 146220 | 29880 | 33160 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 149710 | 161780 | 43840 | 47120 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 116580 | 107440 | 68660 | 74870 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 114420 | 123560 | 79130 | 85340 |

Based on Table 6.64.3-1

-	2nd order harmonics may fall into LTE frequencies of Band 22, 42, 48

-	3rd order harmonic may fall into LTE frequencies of Band 46

-	2nd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into Rx frequencies of Band 1, 2, 3, 4, 9, 10, 11, 21, 23, 24, 25, 32, 33, 34, 35, 36, 37, 39, 45, 50, 51, 65, 70, 74, 75, and 76

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.64.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.64.3-2: 2UL Band 66+Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | IMD3 |
| Galileo | 1559 | - | 1591 | YES |  | IMD3 |
| GLONASS | 1591 | - | 1610 | YES |  | IMD3 |
| GPS | 1563 | - | 1587 | YES |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | 3rd harmonics |
|  | 5150 | - | 5350 | YES | Europe | 3rd harmonics |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | YES | Asia | 3rd harmonics |

Table 6.64.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.64.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_66C-n260A | Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 42, 48 | FDL_low | - | FDL_high | -50 | 1 | 1* |
| * Table 6.5.3.2-1 of TS38.101 |  |  |  |  |  |  |  |

### 6.64.4	∆TIB and ∆RIB values

For DC_66C-n260A, the TIB,c and RIB values are given in the tables below.

Table 6.64.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66C-n260A | 66 | 0 |
|  | n260 | 0 |

Table 6.64.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66C-n260A | 66 | 0 |
|  | n260 | 0 |

### 6.64.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.65	DC_48A-48A-n260A

### 6.65.1	Operating bands for DC

Table 6.65.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_48A-48A-n260A | 48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

### 6.65.2	Channel bandwidths per operating band for DC

Table 6.65.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |  |
| DC_48A-48A-n260A | 48 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 240 |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 440 |  |

### 6.65.3	Co-existence studies

Table 6.65.3-1 lists Band 48A-48A +Band n260A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.65.3-1: Band 48 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 3550 | 3700 | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 7100 | 7400 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 10650 | 11100 | 111000 | 120000 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 36450 | 33300 | 40550 | 43700 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 32900 | 29600 | 70300 | 76450 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 44100 | 47400 | 77550 | 83700 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 3150 | 4100 | 36980 | 40020 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | -72900 | -66600 | 81100 | 87400 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | -29350 | -25900 | 107300 | 116450 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 47650 | 51100 | 114550 | 123700 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | -156450 | -144300 | 22200 | 25800 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 151550 | 163700 | 51200 | 54800 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | -112900 | -103600 | 62900 | 69350 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 118100 | 127400 | 84650 | 91100 |

Based on Table 6.65.3-1

-	3rd order IMD products may fall into frequencies of Band 22, 42, 43, 76 and n260

-	4th order IMD products may fall into frequencies of band n257 and n258

-	5th order IMD products may fall into frequencies of band n258

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.65.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.65.3-2: 2UL Band 48 +Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.65.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.65.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_48A-48A-n260A | Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |

### 6.65.4	∆TIB and ∆RIB values

For DC_48A-48A-n260A, the TIB,c and RIB values are given in the tables below.

Table 6.65.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_48A-48A-n260A | 48 | 0 |
|  | n260 | 0 |

Table 6.65.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_48A-48A-n260A | 48 | 0 |
|  | n260 | 0 |

### 6.65.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.66	DC_2A-2A-n260A

### 6.66.1	Operating bands for DC

Table 6.66.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2A-2A-n260A | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

### 6.66.2	Channel bandwidths per operating band for DC

Table 6.66.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz |  | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_2A-2A-n260A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 240 |
|  | n260 | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes |  | Yes | Yes | 440 |
| DC_2A-2A_n260G | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 240 |
|  | n260 | 60 | See CA_n260G Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-2A_n260H | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 340 |
|  | n260 | 60 | See CA_n260H Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-2A_n260I | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 440 |
|  | n260 | 60 | See CA_n260I Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-2A_n260J | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 540 |
|  | n260 | 60 | See CA_n260J Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-2A_n260K | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 640 |
|  | n260 | 60 | See CA_n260K Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-2A_n260L | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 740 |
|  | n260 | 60 | See CA_n260L Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-2A_n260M | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 840 |
|  | n260 | 60 | See CA_n260M Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.66.3	Co-existence studies

Table 6.66.3-1 lists Band 2A-2A +Band n260A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.66.3-1: Band 2 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 111000 | 120000 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 38150 | 35090 | 38850 | 41910 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 36300 | 33180 | 72090 | 78150 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 40700 | 43820 | 75850 | 81910 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 1450 | 2310 | 36980 | 40020 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 76300 | 70180 | 77700 | 83820 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 34450 | 31270 | 109090 | 118150 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 42550 | 45730 | 112850 | 121910 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 158150 | 146090 | 29360 | 32600 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 149850 | 161910 | 44400 | 47640 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 116300 | 107180 | 68270 | 74450 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 114700 | 123820 | 79550 | 85730 |

Based on Table 6.66.3-1

-	2nd order harmonics may fall into LTE frequencies of Band 43

-	3rd order harmonics may fall into LTE frequencies of Band 46

-	2nd order IMD products may fall into Rx frequencies of band n260

-	3rd order IMD products may fall into Rx frequencies of Band 1, 3, 4, 9, 10, 11, 21, 23, 24, 25, 32, 33, 34, 35, 36, 37, 39, 40, 45, 50, 65, 66, 70, 74, 75 and band 257

-	3rd order IMD products may fall into own Rx frequencies of Band 2 and n260

-	5th order IMD products may fall into Rx frequencies of band n257

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.66.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.66.3-2: 2UL Band 2 +Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | IMD3 |
| Galileo | 1559 | - | 1591 | YES |  | IMD3 |
| GLONASS | 1591 | - | 1610 | YES |  | IMD3 |
| GPS | 1563 | - | 1587 | YES |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | 3rd order harmonic |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | YES |  | 3rd order harmonic |
|  | 5150 | - | 5825 | YES | Asia | 3rd order harmonic |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | YES | China | 3rd order harmonic |
|  | 47200 | - | 48400 | YES | China | 5th IMD |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.66.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.66.3-5: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_2A-2A-n260A | Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 50, 51, 66, 70, 71, 74, 74, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | x |
|  | Band 43 | FDL_low | - | FDL_high | -50 | 1 | 1* |
| * Table 6.5.3.2-1 of TS38.101 Note x: NOTE 15 in Table 6.6.3.2-1 of 36.101 [2] will be applied here. Additional requirements for the frequency ranges that are less than FOOB (MHz) in Table 6.66.3.1-1 need further study. |  |  |  |  |  |  |  |

### 6.66.4	∆TIB and ∆RIB values

For DC_2A-2A-n260A, the TIB,c and RIB values are given in the tables below.

Table 6.66.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2A-2A-n260A | 2 | 0 |
|  | n260 | 0 |

Table 6.66.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2A-2A-n260A | 2 | 0 |
|  | n260 | 0 |

### 6.66.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.67	DC_2A-13A-n260A

### 6.67.1	Operating bands for DC

Table 6.67.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2A-13A-n260A | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

### 6.67.2	Channel bandwidths per operating band for DC

Table 6.67.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |  |
| DC_2A-13A-n260A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 230 |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 430 |  |

### 6.67.3	Co-existence studies

Table 6.67.3-1 lists Band 2A-13A +Band n260A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.67.3-1: Band 2 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 111000 | 120000 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 38150 | 35090 | 38850 | 41910 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 36300 | 33180 | 72090 | 78150 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 40700 | 43820 | 75850 | 81910 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 1450 | 2310 | 36980 | 40020 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 76300 | 70180 | 77700 | 83820 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 34450 | 31270 | 109090 | 118150 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 42550 | 45730 | 112850 | 121910 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 158150 | 146090 | 29360 | 32600 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 149850 | 161910 | 44400 | 47640 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 116300 | 107180 | 68270 | 74450 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 114700 | 123820 | 79550 | 85730 |

Based on Table 6.67.3-1

-	2nd order harmonics may fall into LTE frequencies of Band 43

-	3rd order harmonics may fall into LTE frequencies of Band 46

-	2nd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into Rx frequencies of Band 1,  3, 4, 9, 10, 11, 21, 23, 24, 25, 32, 33, 34, 35, 36, 37, 39, 40, 45, 50, 65, 66, 70, 74, 75 and band n257

-	3rd order IMD products may fall into own Rx frequencies of Band 2 and n260

-	5th order IMD products may fall into Rx frequencies of band n257

Table 6.67.3-2: Band 13 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 777 | 787 | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1554 | 1574 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2331 | 2361 | 111000 | 120000 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 39223 | 36213 | 37777 | 40787 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 38446 | 35426 | 73213 | 79223 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 38554 | 41574 | 74777 | 80787 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 377 | 1187 | 36990 | 40010 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 78446 | 72426 | 75554 | 81574 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 37669 | 34639 | 110213 | 119223 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 39331 | 42361 | 111777 | 120787 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 159223 | 147213 | 33852 | 36892 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 148777 | 160787 | 40108 | 43148 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 118446 | 109426 | 71639 | 77669 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 112554 | 121574 | 76331 | 82361 |

Based on Table 6.67.3-2

-	2nd order harmonics may fall into LTE frequencies of Band 24

-	3rd order harmonic may fall into LTE frequencies of Band 30 and 40

-	2nd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into Rx frequencies of Band 5, 6, 8, 12, 14, 17, 18, 19, 20, 26, 27, 28, 29, 31, 44, 67, 68, 71, 72

-	4th order IMD products may fall into own Rx frequencies of band n260

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.67.3-3 and Table 6.67.3-4 list if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.67.3-3: 2UL Band 2 +Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | IMD3 |
| Galileo | 1559 | - | 1591 | YES |  | IMD3 |
| GLONASS | 1591 | - | 1610 | YES |  | IMD3 |
| GPS | 1563 | - | 1587 | YES |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | 3rd order harmonic |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | YES |  | 3rd order harmonic |
|  | 5150 | - | 5825 | YES | Asia | 3rd order harmonic |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | YES | China | 3rd order harmonic |
|  | 47200 | - | 48400 | YES | China | 5th IMD |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.67.3-4: 2UL Band 13 +Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | 2nd harmonics |
| Galileo | 1559 | - | 1591 | YES |  | 2nd harmonics |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | YES |  | 2nd harmonics |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | YES | China | 4th, 5th IMD |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.67.3-5 and Table 6.67.3-6 list the protected bands required for the dual connectivity configuration.

Table 6.67.3-5: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_2A-n260A | Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 50, 51, 66, 70, 71, 74, 74, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | x |
|  | Band 43 | FDL_low | - | FDL_high | -50 | 1 | 1* |
| Note 1: * Table 6.5.3.2-1 of TS38.101 Note 2: NOTE 15 in Table 6.6.3.2-1 of 36.101 [2] will be applied here. Additional requirements for the frequency ranges that are less than FOOB (MHz) in Table 6.67.3.1-1 need further study. |  |  |  |  |  |  |  |

Table 6.67.3-6: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_13A-n260A | E-UTRA Band 2, 4, 5, 10, 12, 13, 17, 25, 26, 27, 29, 41, 48, 50, 51, 66, 70, 71, 74, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 14 | FDL_low | - | FDL_high | -50 | 1 | x |
|  | Band 24, 30 | FDL_low | - | FDL_high | -50 | 1 | 1* |
| Note 1: * Table 6.5.3.2-1 of TS38.101 Note 2: NOTE 15 in Table 6.6.3.2-1 of 36.101 [2] will be applied here. Additional requirements for the frequency ranges that are less than FOOB (MHz) in Table 6.67.3.1-1 need further study. |  |  |  |  |  |  |  |

### 6.67.4	∆TIB and ∆RIB values

For DC_2A-13A-n260A, the TIB,c and RIB values are given in the tables below.

Table 6.67.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
|  | 2 | 0.3 |
| DC_2A-13A-n260A | 13 | 0.3 |
|  | n260 | 0 |

Table 6.67.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
|  | 2 | 0 |
| DC_2A-13A-n260A | 13 | 0 |
|  | n260 | 0 |

### 6.67.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.68	DC_2C-n260A

### 6.68.1	Operating bands for DC

Table 6.68.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2C-n260A | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

### 6.68.2	Channel bandwidths per operating band for DC

Table 6.68.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |  |
| DC_2C-n260A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 240 |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 440 |  |

### 6.68.3	Co-existence studies

Table 6.68.3-1 lists Band 2C +Band n260A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.68.3-1: Band 2 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 111000 | 120000 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 38150 | 35090 | 38850 | 41910 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 36300 | 33180 | 72090 | 78150 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 40700 | 43820 | 75850 | 81910 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 1450 | 2310 | 36980 | 40020 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 76300 | 70180 | 77700 | 83820 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 34450 | 31270 | 109090 | 118150 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 42550 | 45730 | 112850 | 121910 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 158150 | 146090 | 29360 | 32600 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 149850 | 161910 | 44400 | 47640 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 116300 | 107180 | 68270 | 74450 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 114700 | 123820 | 79550 | 85730 |

Based on Table 6.68.3-1

-	2nd order harmonics may fall into LTE frequencies of Band 43

-	3rd order harmonics may fall into LTE frequencies of Band 46

-	2nd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into Rx frequencies of Band 1, 3, 4, 9, 10, 11, 21, 23, 24, 25, 32, 33, 34, 35, 36, 37, 39, 40, 45, 50, 65, 66, 70, 74, 75 and band n257

-	3rd order IMD products may fall into own Rx frequencies of Band 2 and n260

-	5th order IMD products may fall into Rx frequencies of band n257

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.68.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.68.3-2: 2UL Band 2 +Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | IMD3 |
| Galileo | 1559 | - | 1591 | YES |  | IMD3 |
| GLONASS | 1591 | - | 1610 | YES |  | IMD3 |
| GPS | 1563 | - | 1587 | YES |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | 3rd order harmonic |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | YES |  | 3rd order harmonic |
|  | 5150 | - | 5825 | YES | Asia | 3rd order harmonic |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | YES | China | 3rd order harmonic |
|  | 47200 | - | 48400 | YES | China | 5th IMD |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.68.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.68.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_2C-n260A | Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 50, 51, 66, 70, 71, 74, 74, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | x |
|  | Band 43 | FDL_low | - | FDL_high | -50 | 1 | 1* |
| Note 1: * Table 6.5.3.2-1 of TS38.101 Note 2: NOTE 15 in Table 6.6.3.2-1 of 36.101 [2] will be applied here. Additional requirements for the frequency ranges that are less than FOOB (MHz) in Table 6.68.3.1-1 need further study. |  |  |  |  |  |  |  |

### 6.68.4	∆TIB and ∆RIB values

For DC_2C-n260A, the TIB,c and RIB values are given in the tables below.

Table 6.68.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2C-n260A | 2 | 0 |
|  | n260 | 0 |

Table 6.68.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2C-n260A | 2 | 0 |
|  | n260 | 0 |

### 6.68.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.69	DC_5A-5A-n260A

### 6.69.1	Operating bands for DC

Table 6.69.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5A-5A-n260A | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

### 6.69.2	Channel bandwidths per operating band for DC

Table 6.69.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |  |
| DC_5A-5A-n260A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 220 |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 420 |  |

### 6.69.3	Co-existence studies

Table 6.69.3-1 lists Band 5A-5A +Band n260A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.69.3-1: Band 5 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 111000 | 120000 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 39176 | 36151 | 37824 | 40849 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 38352 | 35302 | 73151 | 79176 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 38648 | 41698 | 74824 | 80849 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 424 | 1249 | 36990 | 40010 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 78352 | 72302 | 75648 | 81698 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 37528 | 34453 | 110151 | 119176 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 39472 | 42547 | 111824 | 120849 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 159176 | 147151 | 33604 | 36704 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 148824 | 160849 | 40296 | 43396 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 118352 | 109302 | 71453 | 77528 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 112648 | 121698 | 76472 | 82547 |

Based on Table 6.69.3-1

-	3rd order harmonic may fall into frequencies of band n41

-	2nd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into frequencies of band 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 31, 44, 67, 68, 71, and 72

-	4th order IMD products may fall into own Rx frequencies of band n260

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.69.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.69.3-2: 2UL Band 5 +Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | YES | US/Europe | 3rd order harmonic |
|  | 2400 | - | 2494 | YES | Asia | 3rd order harmonic |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.69.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.69.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_5A-5A-n260A | Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 74, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 26 | FDL_low | - | FDL_high | -27 | 1 |  |
|  | Band 41 | FDL_low | - | FDL_high | -50 | 1 | 1* |
| * Table 6.5.3.2-1 of TS38.101 |  |  |  |  |  |  |  |

### 6.69.4	∆TIB and ∆RIB values

For DC_5A-5A-n260A, the TIB,c and RIB values are given in the tables below.

Table 6.69.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5A-5A-n260A | 5 | 0 |
|  | n260 | 0 |

Table 6.69.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5A-5A-n260A | 5 | 0 |
|  | n260 | 0 |

### 6.69.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.70	DC_5B-n260A

### 6.70.1	Operating bands for DC

Table 6.70.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5B-n260A | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

### 6.70.2	Channel bandwidths per operating band for DC

Table 6.70.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |  |
| DC_5B-n260A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 220 |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 420 |  |

### 6.70.3	Co-existence studies

Table 6.70.3-1 lists Band 5B +Band n260A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.70.3-1: Band 5 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 111000 | 120000 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 39176 | 36151 | 37824 | 40849 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 38352 | 35302 | 73151 | 79176 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 38648 | 41698 | 74824 | 80849 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 424 | 1249 | 36990 | 40010 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 78352 | 72302 | 75648 | 81698 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 37528 | 34453 | 110151 | 119176 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 39472 | 42547 | 111824 | 120849 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 159176 | 147151 | 33604 | 36704 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 148824 | 160849 | 40296 | 43396 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 118352 | 109302 | 71453 | 77528 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 112648 | 121698 | 76472 | 82547 |

Based on Table 6.70.3-1

-	3rd order harmonic may fall into frequencies of band n41

-	2nd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into frequencies of band 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 31, 44, 67, 68, 71, and 72

-	4th order IMD products may fall into own Rx frequencies of band n260

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.70.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.70.3-2: 2UL Band 5 +Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | YES | US/Europe | 3rd order harmonic |
|  | 2400 | - | 2494 | YES | Asia | 3rd order harmonic |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.70.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.70.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_5B-n260A | Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 74, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 26 | FDL_low | - | FDL_high | -27 | 1 |  |
|  | Band 41 | FDL_low | - | FDL_high | -50 | 1 | 1* |
| Note : * Table 6.5.3.2-1 of TS38.101 |  |  |  |  |  |  |  |

### 6.70.4	∆TIB and ∆RIB values

For DC_5B-n260A, the TIB,c and RIB values are given in the tables below.

Table 6.70.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5B-n260A | 5 | 0 |
|  | n260 | 0 |

Table 6.70.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5B-n260A | 5 | 0 |
|  | n260 | 0 |

### 6.70.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.71	DC_13A-66A-n260A

### 6.71.1	Operating bands for DC

Table 6.71.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC 13A-66A-n260A | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

### 6.71.2	Channel bandwidths per operating band for DC

Table 6.71.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |  |
| DC_13A-66A- n260 | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 230 |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 430 |  |

### 6.71.3	Co-existence studies

Table 6.71.3-1 lists Band 13A-66A +Band n260A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.71.3-1: Band 13 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 777 | 787 | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1554 | 1574 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2331 | 2361 | 111000 | 120000 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 39223 | 36213 | 37777 | 40787 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 38446 | 35426 | 73213 | 79223 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 38554 | 41574 | 74777 | 80787 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 377 | 1187 | 36990 | 40010 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 78446 | 72426 | 75554 | 81574 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 37669 | 34639 | 110213 | 119223 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 39331 | 42361 | 111777 | 120787 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 159223 | 147213 | 33852 | 36892 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 148777 | 160787 | 40108 | 43148 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 118446 | 109426 | 71639 | 77669 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 112554 | 121574 | 76331 | 82361 |

Based on Table 6.71.3-1

-	2nd order harmonics may fall into LTE frequencies of Band 24

-	3rd order harmonic may fall into LTE frequencies of Band 30 and 40

-	2nd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into Rx frequencies of Band 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 31, 44, 67, 68, 71 and 72

-	4th order IMD products may fiall into own Rx frequencies of band n260

Table 6.71.3-2: Band 66 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 111000 | 120000 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 38290 | 35220 | 35290 | 38220 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 36580 | 33440 | 72220 | 78290 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 40420 | 43560 | 75710 | 81780 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fx_high + Max BW-fy |
| IMD frequency limits (MHz) | 1310 | 2180 | 36990 | 40010 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 76580 | 70440 | 77420 | 83560 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 34870 | 31660 | 109220 | 118290 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 42130 | 45340 | 112710 | 121780 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 158290 | 146220 | 29880 | 33160 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 149710 | 161780 | 43840 | 47120 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 116580 | 107440 | 68660 | 74870 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 114420 | 123560 | 79130 | 85340 |

Based on Table 6.71.3-2

-	2nd order harmonics may fall into LTE frequencies of Band 22, 42, 48

-	3rd order harmonic may fall into LTE frequencies of Band 46

-	2nd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into Rx frequencies of Band 1, 2, 3, 4, 9, 10, 11, 21, 23, 24, 25, 32, 33, 34, 35, 36, 37, 39, 45, 50, 51, 65, 66, 70, 74, 75, and 76

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.71.3-3 and Table 6.71.3-4 list if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.71.3-3: 2UL Band 13 +Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | 2nd harmonics |
| Galileo | 1559 | - | 1591 | YES |  | 2nd harmonics |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | YES |  | 2nd harmonics |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | YES | China | 4th, 5th IMD |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.71.3-4: 2UL Band 66 +Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | IMD3 |
| Galileo | 1559 | - | 1591 | YES |  | IMD3 |
| GLONASS | 1591 | - | 1610 | YES |  | IMD3 |
| GPS | 1563 | - | 1587 | YES |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | 3rd harmonics |
|  | 5150 | - | 5350 | YES | Europe | 3rd harmonics |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | YES | Asia | 3rd harmonics |

Table 6.71.3-5 and Table 6.71.3-6  list the protected bands required for the dual connectivity configuration.

Table 6.71.3-5: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_13A-n260A | E-UTRA Band 2, 4, 5, 10, 12, 13, 17, 25, 26, 27, 29, 41, 48, 50, 51, 66, 70, 71, 74, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 14 | FDL_low | - | FDL_high | -50 | 1 | x |
|  | Band 24, 30 | FDL_low | - | FDL_high | -50 | 1 | 1* |
| Note 1: * Table 6.5.3.2-1 of TS38.101 Note 2: NOTE 15 in Table 6.6.3.2-1 of 36.101 [2] will be applied here. Additional requirements for the frequency ranges that are less than FOOB (MHz) in Table 6.71.3.1-1 need further study. |  |  |  |  |  |  |  |

Table 6.71.3-6: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_66A-n260A | Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 42, 48 | FDL_low | - | FDL_high | -50 | 1 | 1* |
| Note: * Table 6.5.3.2-1 of TS38.101 |  |  |  |  |  |  |  |

### 6.71.4	∆TIB and ∆RIB values

For DC_13A-66A-n260A, the TIB,c and RIB values are given in the tables below.

Table 6.71.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
|  | 13 | 0.3 |
| DC_13A-66A-n260A | 66 | 0.3 |
|  | n260 | 0 |

Table 6.71.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
|  | 13 | 0 |
| DC_13A-66A-n260A | 66 | 0 |
|  | n260 | 0 |

### 6.71.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.72	DC_48C-n260A

### 6.72.1	Operating bands for DC

Table 6.72.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_48C-n260A | 48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

### 6.72.2	Channel bandwidths per operating band for DC

Table 6.72.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |  |
| DC_48C-n260A | 48 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 240 |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 440 |  |

### 6.72.3	Co-existence studies

Table 6.72.3-1 lists Band 48C +Band n260A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.72.3-1: Band 48 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 3550 | 3700 | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 7100 | 7400 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 10650 | 11100 | 111000 | 120000 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 36450 | 33300 | 40550 | 43700 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 32900 | 29600 | 70300 | 76450 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 44100 | 47400 | 77550 | 83700 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 3150 | 4100 | 36980 | 40020 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | -72900 | -66600 | 81100 | 87400 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | -29350 | -25900 | 107300 | 116450 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 47650 | 51100 | 114550 | 123700 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | -156450 | -144300 | 22200 | 25800 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 151550 | 163700 | 51200 | 54800 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | -112900 | -103600 | 62900 | 69350 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 118100 | 127400 | 84650 | 91100 |

Based on Table 6.72.3-1

-	3rd order IMD products may fall into frequencies of Band 22, 42, 43, 76 and n260

-	4th order IMD products may fall into frequencies of band n257 and n258

-	5th order IMD products may fall into frequencies of band n258

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.72.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.72.3-2: 2UL Band 48 +Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.72.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.72.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_48C-n260A | Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |

### 6.72.4	∆TIB and ∆RIB values

For DC_48C-n260A, the TIB,c and RIB values are given in the tables below.

Table 6.72.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_48C-n260A | 48 | 0 |
|  | n260 | 0 |

Table 6.72.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_48C-n260A | 48 | 0 |
|  | n260 | 0 |

### 6.72.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.73	DC_2A-(n)71B

### 6.73.1	Operating bands for DC

Table 6.73.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band Combination | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2A-(n)71B | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 71 | 663 MHz – 698 MHz |  |  | 617 MHz – 652 MHz |  |  | FDD |
|  | n71 |  |  |  |  |  |  |  |

### 6.73.2	Channel bandwidths per operating band for DC

Table 6.73.2-1 Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

|  | DC operating / channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] |
| DC_2A-(n)71B | 2 | 15 | Yes | Yes | Yes | Yes | 40 |
|  | 71 | See DC_(n)71B Bandwidth combination Set 0 in below Table 6.73.2-2 |  |  |  |  |  |
|  | n71 |  |  |  |  |  |  |

Table 6.73.2-2 DC_(n)71B channel bandwidth combination

| EN-DC configuration / Bandwidth Combination Set |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for LTE carrier[MHz] | Channel bandwidths for NR carrier[MHz] |  |  |
| DC_(n)71B | DC_(n)71B | 15 | 5 | 20 | 0 |
|  |  | 10 | 5, 10 |  |  |
|  |  | 5 | 5, 10, 15 |  |  |

### 6.73.3	Co-existence studies

Table 6.73.3-1and 6.73.3-2 list Band 2-Band 71 + Band n71 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.73.3-1: Band 2 and Band n71 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 |  | 663 | 698 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 |  | 3820 | 1326 | 1396 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 |  | 5730 | 1989 | 2094 |
| 2nd order IMD products | ¦f2_low–f1_high¦ |  | ¦f2_high–f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency limits (MHz) | 1152 |  | 1247 | 2513 | 2608 |
| 3rd order IMD products | ¦f2_low–2*f1_high¦ |  | ¦f2_high–2*f1_low¦ | ¦2*f2_low–f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limits (MHz) | 3002 |  | 3157 | 454 | 584 |
| 3rd order IMD products | ¦2*f1_low + f2_low¦ |  | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
| IMD frequency limits (MHz) | 4363 |  | 4518 | 3176 | 3306 |
| 4rd order IMD products | ¦3*f1_low – f2_high¦ |  | ¦3*f1_high – f2_low¦ | ¦3*f2_low–f1_high¦ | ¦3*f2_high –f1_low¦ |
| IMD frequency limits (MHz) | 4852 |  | 5067 | 79 | 244 |
| 4rd order IMD products | ¦3*f1_low +f2_low¦ |  | ¦3*f1_high + f2_high¦ | ¦3*f2_low+f1_low¦ | ¦3*f2_high +f1_high¦ |
| IMD frequency limits (MHz) | 6213 |  | 6428 | 3839 | 4004 |
| 4rd order IMD products | ¦2*f1_low-2*f2_high ¦ |  | ¦2*f1_high –2*f2_low¦ | ¦2*f1_low+2*f2_low¦ | ¦2*f1_high +2*f2_high¦ |
| IMD frequency limits (MHz) | 2304 |  | 2494 | 5026 | 5216 |
| 5rd order IMD products | ¦f1_low –4*f2_high ¦ |  | ¦f1_high –4*f2_low¦ | ¦f2_low–4*f1_high¦ | ¦f2_high –4*f1_low¦ |
| IMD frequency limits (MHz) | 742 |  | 942 | 6702 | 6977 |
| 5rd order IMD products | ¦f1_low +4*f2_low¦ |  | ¦f1_high +4*f2_high¦ | ¦f2_low+4*f1_low¦ | ¦f2_high +4*f1_high¦ |
| IMD frequency limits (MHz) | 4502 |  | 4702 | 8063 | 8338 |
| 5rd order IMD products | ¦2*f1_low-3*f2_high¦ |  | ¦2*f1_high–3*f2_low¦ | ¦2*f2_low-3*f1_high¦ | ¦2*f2_high -3*f1_low¦ |
| IMD frequency limits (MHz) | 1606 |  | 1831 | 4154 | 4404 |
| 5rd order IMD products | ¦2*f1_low +3*f2_low¦ |  | ¦2*f1_high +3*f2_high¦ | ¦2*f2_low+3*f1_low¦ | ¦2*f2_high +3*f1_high¦ |
| IMD frequency limits (MHz) | 5689 |  | 5914 | 6876 | 7126 |

Based on Table 6.73.3-1, it can be seen that

-	2nd order harmonic may fall into Rx frequencies of bands 43, n77and n78.

-	3rd order harmonic may fall into Rx frequencies of bands 2, 25, 34, 36, 46 and 70.

-	2nd order IMD may fall into own Rx frequencies of bands 38, 41and 69.

-	3rd order IMD may fall into own Rx frequencies of bands 31, 72, 73, n77, n78 and n79.

-	4th order IMD may fall into own Rx frequencies of bands 30, 40, 46, n77 and n79.

-	5th order IMD may fall into own Rx frequencies of  bands 3, 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 44, 46, 47, 67, 68, n77 and n79.

Therefore it can be concluded that there is just 3rd order harmonic falling into own Rx frequencies of Band 2.

Table 6.73.3-2: Band 71 and Band n71 UL harmonics

| Uplink band |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 71/n71 | 663 | 698 | 1326 | 1396 | 1989 | 2094 |

Based on Table 6.73.3-2, it can be seen that 3rd harmonic overlaps with bands 2, 25, 34, 36 and 70. It should be noted that the third harmonic products of Band n71 fall within 1989 – 2094 MHz, which has 1MHz overlap with the downlink of Band 2 (1930 - 1990 MHz). Therefore, third harmonic interference from uplink in Band 71 to downlink in Band 2 could be observed.  However, the potential impact could be limited, because this third harmonic interference could happen only in the case where the lowest edge of Band 71 paired with simultaneous downlink in the uppermost edge of Band 2 is utilized. The guard bands defined for LTE and NR transmission bandwidth configuration could help to avoid the direct overlap while some effect due to spectral spreading of the third harmonic may still exist.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.73.3-3 lists up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.73.3-3: Band 2 and Band n71 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | Yes |  | 5th IMD |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | 4th IMD |
|  | 2400 | - | 2494 | Yes | Asia | 4th IMD |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 3rd harmonic, 4th, 5th IMD |
|  | 5150 | - | 5350 | Yes | Europe | 4th IMD |
|  | 5470 | - | 5725 | Yes |  | 3rd harmonic, 5th IMD |
|  | 5150 | - | 5825 | Yes | Asia | 3rd harmonic, 4th, 5th IMD |

### 6.73.4	∆TIB and ∆RIB values

For DC_2A-(n)71B, the TIB,c and RIB,c values are given in the tables below.

Table 6.73.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2A-(n)71B | 2 | 0.3 |
|  | 71 | 0.3 |
|  | n71 |  |

Table 6.73.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2A-(n)71B | 2 | 0 |
|  | 71 | 0 |
|  | n71 |  |

### 6.73.5	MSD requirements

The MSD requirements DC_2A-(n)71B was caused by the 3rd harmonic of uplink in Band n71 interfering with downlink in Band 2, Band71 and Band n71 Rx 3rd order harmonic mixing  and Band 71 or Band n71 Tx leakage to own Rx band. Table 6.73.5-1, table 6.73.5-2 and table 6.73.5-3 list the MSD required for the dual connectivity configuration.

Table 6.73.5-1: MSD for the DC_2A-(n)71B (exceptions due to harmonic issue)

| MSD due to harmonic issue exception for the DL band |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| n71 | 21 | 4.6 | 1 | 0.7 | 0.6 | FDD |
|  | 22 | 1.7 | 1 | 0.7 | 0.6 |  |
| NOTE1: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band n71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 2: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band n71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. |  |  |  |  |  |  |

Table 6.73.5-2: MSD for the DC_2A-(n)71B (exceptions due to harmonic mixing issue)

| MSD due to harmonic mixing issue exception for the DL band |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| 2 | 713,4 | 26.8 | 23.8 | 21.6 |  | FDD |
|  | n713,4 | 26.8 | 23.6 | 21.2 |  |  |
| NOTE 3: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel’s transmission bandwidth of an aggressor (higher) band. NOTE 4: The requirements should be verified for UL EARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |

Table 6.73.5-3: MSD for DC_2A-(n)71B (exceptions due to Tx leakage)

| MSD due to Tx leakage issue exception for the DL band |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA and NR DC Configuration | E-UTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL FC (MHz) | MSD (dB) | Duplex mode |
| DC_2A-(n)71B | 71 | 665.5 | 5 | 5 (RBend =24) | 619.5 | 0 | FDD |
|  | n71 | 675.5 | 15 | 15 (RBstart = 0) | 629.5 | 1.8 |  |
|  | 71 | 670.5 | 15 | 15 (RBend = 74) | 624.5 | 0 |  |
|  | n71 | 680.5 | 5 | 5 (RBstart = 0) | 634.5 | 1.6 |  |
|  | 71 | 668 | 10 | 10 (RBend = 49) | 622 | 0 |  |
|  | n71 | 678 | 10 | 10 (RBstart = 0) | 632 | 1.7 |  |
|  | 71 | 668 | 10 | 10 (RBstart = 0) | 622 | 17.2 |  |
|  | n71 | 678 | 10 | 10 (RBend = 51) | 632 | 29.4 |  |

## 6.74	DC_19A-42A_n77

### 6.74.1	Operating bands for DC

Table 6.74.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19A-42A_n77 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 42 | 3400MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz |  |

### 6.74.2	Channel bandwidths per operating band for DC

Table 6.74.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_19A-42A_n77A | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 135 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_19A-42A_n77C | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 235 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n77 | See CA_n77C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.74.3	Co-existence studies

The co-existence studies for the lower order 2DL/2UL DC configurations are already captured in TR 37.863-01-01 [5]. For the additional 3rd receiver band, there is no MSD issue for this DC configuration based on the following conditions.

-	LTE Band 42 and NR Band n77 are synchronized

-	Simultaneous transmission of LTE Band 42 and NR Band n77 are not supported at least in Rel-15 timeframe

### 6.74.4	∆TIB and ∆RIB values

For DC_19A-42A_n77, the TIB,c and RIB values are given in the tables below.

Table 6.74.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19A-42A_n77 | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |

Table 6.74.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19A-42A_n77 | 19 | 0 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |

### 6.74.5	MSD

As mentioned in 6.74.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 6.75	DC_19A-42A_n78

### 6.75.1	Operating bands for DC

Table 6.75.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19A-42A_n78 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 42 | 3400MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz |  |

### 6.75.2	Channel bandwidths per operating band for DC

Table 6.75.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_19A-42A_n78A | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 135 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_19A-42A_n78C | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 235 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 | See CA_n78C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.75.3	Co-existence studies

The co-existence studies for the lower order 2DL/2UL DC configurations are already captured in TR 37.863-01-01 [5]. For the additional 3rd receiver band, there is no MSD issue for this DC configuration based on the following conditions.

-	LTE Band 42 and NR Band n78 are synchronized

-	Simultaneous transmission of LTE Band 42 and NR Band n78 are not supported at least in Rel-15 timeframe

### 6.75.4	∆TIB and ∆RIB values

For DC_19A-42A_n78, the TIB,c and RIB values are given in the tables below.

Table 6.8.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19A-42A_n78 | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |

Table 6.8.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19A-42A_n78 | 19 | 0 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |

### 6.75.5	MSD

As mentioned in 6.75.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 6.76	DC_19A-42A_n79

### 6.76.1	Operating bands for DC

Table 6.76.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19A-42A_n79 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 42 | 3400MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz |  |

### 6.76.2	Channel bandwidths per operating band for DC

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_19A-42A_n79A | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 135 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
| DC_19A-42A_n79C | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 235 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n79 | See CA_n79C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.76.3	Co-existence studies

Based on co-existence studies of Band 19 + Band n79 and Band 42 + Band n79 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	2nd order IMD generated by dual uplink of Band 19 + Band n79 may also fall into own Rx of band 42. However, considering actual spectrum holdings, no IMD issue is expected.

-	2nd and 5th order IMD generated by dual uplink of Band 42 + Band n79 may also fall into own Rx of band 19.

### 6.76.4	∆TIB and ∆RIB values

For DC_19A-42A_n79, the TIB,c and RIB values are given in the tables below.

Table 6.76.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19A-42A_n79 | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n79 | 0 |

Table 6.76.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19A-42A_n79 | 19 | 0 |
|  | 42 | 0.5 |
|  | n79 | 0 |

### 6.76.5	MSD

Table 6.76.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.76.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_19A-42A_n79 | 19 | 842.5 | 5 | 25 | 887.5 | TBD | FDD | IMD2 |
|  | 42 | 3517.5 | 5 | 25 | 3517.5 | N/A | TDD | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A |  | N/A |

## 6.77	DC_19A-42A_n257

### 6.77.1	Operating bands for DC

Table 6.77.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19A-42A_n257 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 42 | 3400MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

### 6.77.2	Channel bandwidths per operating band for DC

Table 6.77.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_19A-42A_n257A | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  | 435 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |
| DC_19A-42A_n257D | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  | 435 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_19A-42A_n257E | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  | 635 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_19A-42A_n257F | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  | 835 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |

### 6.77.3	Co-existence studies

The co-existence studies for the lower order 2DL/2UL DC configurations are already captured in TR 37.863-01-01 [5]. For the additional 3rd receiver band, no IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

### 6.77.4	∆TIB and ∆RIB values

For DC_19A-42A_n257, the TIB,c and RIB values are given in the tables below.

Table 6.77.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19A-42A_n257 | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n257 | 0 |

Table 6.77.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19A-42A_n257 | 19 | 0 |
|  | 42 | 0.5 |
|  | n257 | 0 |

### 6.77.5	MSD

As mentioned in 6.77.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 6.78	DC_1A-42A_n77

### 6.78.1	Operating bands for DC

Table 6.78.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-42A_n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 42 | 3400MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz |  |

### 6.78.2	Channel bandwidths per operating band for DC

Table 6.78.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-42A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-42A_n77C | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 240 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n77 | See CA_n77C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.78.3	Co-existence studies

The co-existence studies for the lower order 2DL/2UL DC configurations are already captured in TR 37.863-01-01 [5]. For the additional 3rd receiver band, there is no MSD issue for this DC configuration based on the following conditions.

-	TDD LTE Band 42 and NR Band n77 are synchronized

-	Simultaneous transmission of LTE Band 42 and NR Band n77 are not supported at least in Rel-15 timeframe

### 6.78.4	∆TIB and ∆RIB values

For DC_1A-42A_n77, the TIB,c and RIB values are given in the tables below.

Table 6.78.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-42A_n77 | 1 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |

Table 6.78.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-42A_n77 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |

### 6.78.5	MSD

As mentioned in 6.78.3, there is no need to specify additional MSD requirement for this DC configuration.

## 6.79	DC_1A-42A_n78

### 6.79.1	Operating bands for DC

Table 6.79.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-42A_n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 42 | 3400MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz |  |

### 6.79.2	Channel bandwidths per operating band for DC

Table 6.79.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-42A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-42A_n78C | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 240 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 | See CA_n78C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.79.3	Co-existence studies

The co-existence studies for the lower order 2DL/2UL DC configurations are already captured in TR 37.863-01-01 [5]. For the additional 3rd receiver band, there is no MSD issue for this DC configuration based on the following conditions.

-	LTE Band 42 and NR Band n78 are synchronized

-	Simultaneous transmission of LTE Band 42 and NR Band n78 are not supported at least in Rel-15 timeframe

### 6.79.4	∆TIB and ∆RIB values

For DC_1A-42A_n78, the TIB,c and RIB values are given in the tables below.

Table 6.79.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-42A_n78 | 1 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |

Table 6.79.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-42A_n78 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |

### 6.79.5	MSD

As mentioned in 6.79.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 6.80	DC_1A-42A_n79

### 6.80.1	Operating bands for DC

Table 6.80.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-42A_n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 42 | 3400MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz |  |

### 6.80.2	Channel bandwidths per operating band for DC

Table 6.80.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-42A_n79A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-42A_n79C | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 240 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n79 | See CA_n79C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.80.3	Co-existence studies

Based on co-existence studies of Band 1 + Band n79 and Band 42 + Band n79 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	5th order IMD generated by dual uplink of Band 1 + Band n79 may also fall into own Rx of band 42.

-	3rd and 4th order IMD generated by dual uplink of Band 42 + Band n79 may also fall into own Rx of band 1.

### 6.80.4	∆TIB and ∆RIB values

For DC_1A-42A_n79, the TIB,c and RIB values are given in the tables below.

Table 6.80.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-42A_n79 | 1 | 0.3 |
|  | 42 | 0.8 |
|  | n79 | 0 |

Table 6.80.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-42A_n79 | 1 | 0 |
|  | 42 | 0.5 |
|  | n79 | 0 |

### 6.80.5	MSD

Table 6.8.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.8.5-1

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-42A_n79A | 1 | IMD5 | ¦4*fB1 –fn79¦ | 1977.5 | 5 | 25 | 2167.5 | 5 | 0.6 | N/A |
|  | n79 |  |  | 4420 | 40 | 216 | 4420 | 40 |  |  |
|  | 42 |  |  | 3490 | 5 | 25 | 3490 | 5 |  | 4.8 |
|  | 42 | IMD3 | ¦2*fB42 –fn79¦ | 3402.5 | 5 | 25 | 3402.5 | 5 | 1.6 | N/A |
|  | n79 |  |  | 4640 | 40 | 216 | 4640 | 40 |  |  |
|  | 1 |  |  | 1975 | 5 | 25 | 2165 | 5 |  | 15.5 |
|  | 42 | IMD4 | ¦2*fB42 –2*fn79¦ | 3450 | 5 | 25 | 3450 | 5 | 0.9 | N/A |
|  | n79 |  |  | 4520 | 40 | 216 | 4520 | 40 |  |  |
|  | 1 |  |  | 1950 | 5 | 25 | 2140 | 5 |  | 9.3 |

## 6.81	DC_1A-42A_n257

### 6.81.1	Operating bands for DC

Table 6.81.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-42A_n257A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 42 | 3400MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

### 6.81.2	Channel bandwidths per operating band for DC

Table 6.81.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-42A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 440 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |
| DC_1A-42A_n257D | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 440 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_1A-42A_n257E | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 640 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_1A-42A_n257F | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 840 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |

### 6.81.3	Co-existence studies

The co-existence studies for the lower order 2DL/2UL DC configurations are already captured in TR 37.863-01-01 [5]. For the additional 3rd receiver band, no IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

### 6.81.4	∆TIB and ∆RIB values

For DC_1A-42A_n257, the TIB,c and RIB values are given in the tables below.

Table 6.81.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-42A_n257 | 1 | 0.3 |
|  | 42 | 0.8 |
|  | n257 | 0 |

Table 6.81.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-42A_n257 | 1 | 0 |
|  | 42 | 0.5 |
|  | n257 | 0 |

### 6.81.5	MSD

As mentioned in 6.81.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 6.82 DC_42C_n77

### 6.82.1	Operating bands for DC

Table 6.82.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
|  | 42 | 3400MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz |  |

### 6.82.2	Channel bandwidths per operating band for DC

Table 6.82.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_42C_n77A | 42 | 15 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS 36.101 [2] |  |  |  |  |  |  |  |  | 140 |
|  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_42C_n77C | 42 | 15 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS 36.101 [2] |  |  |  |  |  |  |  |  | 240 |
|  | n77 | See CA_n77C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.82.3	Co-existence studies

The co-existence studies for the lower order 2DL/2UL DC configurations are already captured in TR 37.863-01-01 [5]. For the additional 3rd receiver band, there is no MSD issue for this DC configuration based on the following conditions.

-	LTE Band 42 and NR Band n77 are synchronized

-	Simultaneous transmission of LTE Band 42 and NR Band n77 are not supported at least in Rel-15 timeframe

### 6.82.4	∆TIB and ∆RIB values

For DC_42C_n77, the TIB,c and RIB values are given in the tables below.

Table 6.82.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_42C_n77 | 42 | 0 |
|  | n77 | 0 |

Table 6.82.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_42C_n77 | 42 | 0 |
|  | n77 | 0 |

### 6.82.5	MSD

As mentioned in 6.82.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 6.83	DC_42C_n78

### 6.83.1	Operating bands for DC

Table 6.83.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_42C_n78 | 42 | 3400MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz |  |

### 6.83.2	Channel bandwidths per operating band for DC

Table 6.83.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_42C_n78A | 42 | 15 | See CA_42C Bandwidth Combination Set 0 in Table5.6A.1-1 in TS 36.101 [2] |  |  |  |  |  |  |  |  | 140 |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_42C_n78A | 42 | 15 | See CA_42C Bandwidth Combination Set 0 in Table5.6A.1-1 in TS 36.101 [2] |  |  |  |  |  |  |  |  | 140 |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_42C_n78C | 42 | 15 | See CA_42C Bandwidth Combination Set 0 in Table5.6A.1-1 in TS 36.101 [2] |  |  |  |  |  |  |  |  | 240 |
|  | n78 | See CA_n78C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.83.3	Co-existence studies

The co-existence studies for the lower order 2DL/2UL DC configurations are already captured in TR 37.863-01-01 [5]. For the additional 3rd receiver band, there is no MSD issue for this DC configuration based on the following conditions.

-	LTE Band 42 and NR Band n78 are synchronized

-	Simultaneous transmission of LTE Band 42 and NR Band n78 are not supported at least in Rel-15 timeframe

### 6.83.4	∆TIB and ∆RIB values

For DC_42C_n78, the TIB,c and RIB values are given in the tables below.

Table 6.83.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_42C_n78 | 42 | 0 |
|  | n78 | 0 |

Table 6.83.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_42C_n78 | 42 | 0 |
|  | n78 | 0 |

### 6.83.5	MSD

As mentioned in 6.83.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 6.84	DC_42C_n79

### 6.84.1	Operating bands for DC

Table 6.84.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_42C_n79 | 42 | 3400MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz |  |

### 6.84.2	Channel bandwidths per operating band for DC

Table 6.84.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_42C_n79A | 42 | 15 | See CA_42C Bandwidth Combination Set 0 in Table5.6A.1-1 in TS 36.101 [2] |  |  |  |  |  |  |  |  | 140 |
|  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
| DC_42C_n79C | 42 | 15 | See CA_42C Bandwidth Combination Set 0 in Table5.6A.1-1 in TS 36.101 [2] |  |  |  |  |  |  |  |  | 240 |
|  | n79 | See CA_n79C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.84.3	Co-existence studies

The co-existence studies of Band 42 + Band n79 are already captured in TR 37.863-01-01 [5].

### 6.84.4	∆TIB and ∆RIB values

For DC_42C_n79, the TIB,c and RIB values are given in the tables below.

Table 6.84.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_42C_n79 | 42 | 0 |
|  | n79 | 0 |

Table 6.84.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_42C_n79 | 42 | 0 |
|  | n79 | 0 |

### 6.84.5	MSD

As mentioned in 6.84.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 6.85	DC_42C_n257

### 6.85.1	Operating bands for DC

Table 6.85.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_42C_n257 | 42 | 3400MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

### 6.85.2	Channel bandwidths per operating band for DC

Table 6.85.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_42C_n257A | 42 | 15 | See CA_42C Bandwidth Combination Set 0 in Table5.6A.1-1 in TS 36.101 [2] |  |  |  |  |  |  |  | 440 |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |
| DC_42C_n257D | 42 | 15 | See CA_42C Bandwidth Combination Set 0 in Table5.6A.1-1 in TS 36.101 [2] |  |  |  |  |  |  |  | 440 |
|  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_42C_n257E | 42 | 15 | See CA_42C Bandwidth Combination Set 0 in Table5.6A.1-1 in TS 36.101 [2] |  |  |  |  |  |  |  | 640 |
|  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_42C_n257F | 42 | 15 | See CA_42C Bandwidth Combination Set 0 in Table5.6A.1-1 in TS 36.101 [2] |  |  |  |  |  |  |  | 840 |
|  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |

### 6.85.3	Co-existence studies

The co-existence studies for the lower order 2DL/2UL DC configurations are already captured in TR 37.863-01-01 [5]. For the additional 3rd receiver band, no IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

### 6.85.4	∆TIB and ∆RIB values

For DC_42C_n257, the TIB,c and RIB values are given in the tables below.

Table 6.85.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_42C_n257 | 42 | 0 |
|  | n257 | 0 |

Table 6.85.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_42C_n257 | 42 | 0 |
|  | n257 | 0 |

### 6.85.5	MSD

As mentioned in 6.85.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 6.86	DC_3A-41A_n78A

### 6.86.1	Operating bands for DC

Table 6.86.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-41A_n78A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.86.2	Channel bandwidths per operating band for DC

Table 6.86.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-41A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.86.3	Co-existence studies

Based on co-existence studies of Band 3 + Band n78 and Band 41 + Band n78 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	IMD generated by dual uplink of Band 3 + Band n78 doesn’t fall into own Rx of band 41.

-	3rd and 4th order IMD generated by dual uplink of Band 41 + Band n78 may also fall into own Rx of band 3.

The IMD3 issue can be comprehended in Table 2-1. The IMD4 issue can be comprehended in Table 2-2.

Table 6.86.3-1 LTE DL and NR UL IMD3 relation for DC_3A_41A-n78A

|  | B41 UL | IMD3 of B41 and n78 | n78 UL |
| --- | --- | --- | --- |
| Range (MHz) | 2496 - 2690 | 1192 - 2080 | 3300 - 3800 |
|  | B3 DL |  | n78 DL |
| Range (MHz) | 1805 - 1880 |  | 3300 - 3800 |

Table 6.86.3-2 LTE DL and NR UL IMD4 relation for DC_3A_41A-n78A

|  | B41 UL | IMD4 of B41 and n78 | n78 UL |
| --- | --- | --- | --- |
| Range (MHz) | 2496 - 2690 | 1220 - 2608 | 3300 - 3800 |
|  | B3 DL |  | n78 DL |
| Range (MHz) | 1805 - 1880 |  | 3300 - 3800 |

The harmonic and IMD for ISM and GNSS bands, and the protected band for the DC_3A-41A-n78A will be covered in the constituent fallback modes.

### 6.86.4	∆TIB and ∆RIB values

For DC_3A-41A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6. 86.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-41A-n78A | 3 | 0.6 |
|  | 41 | 0.31 |
|  |  | 0.82 |
|  | n78 | 0.8 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 6. 86.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-41A-n78A | 3 | 0.2 |
|  | 41 | 01 |
|  |  | 0.52 |
|  | n78 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

### 6.86.5	MSD

Table 6.86.5-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band.

In addition, the cases that harmonic issue and IMD interference fall into the own Rx frequency of interfering band have been discussed and specified in the constituent fallback mode.

Table 6.86.5-1: MSD for the DC configuration

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA/NR DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode | Source of IMD | Applicable active UL band |
| DC_3A-41A-n78A | 3 | 1720 | 5 | 25 | 1815 | [17.6] | FDD | IMD3 ¦fB78-2*fB41¦ | 41, n78 |
|  | 41 | 2625 | 5 | 25 | 2625 | N/A | TDD | N/A |  |
|  | n78 | 3440 | 10 | 52 | 3440 | N/A | TDD | N/A |  |
| DC_3A-41A-n78A | 3 | 1755 | 5 | 25 | 1850 | [8.6] | FDD | IMD4 ¦2*fB78-2*fB41¦ | 41, n78 |
|  | 41 | 2565 | 5 | 25 | 2565 | N/A | TDD | N/A |  |
|  | n78 | 3490 | 10 | 52 | 3490 | N/A | TDD | N/A |  |

## 6.87	DC_3C-41A_n78A

### 6.87.1	Operating bands for DC

Table 6.87.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3C-41A_n78A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 6.87.2-2: Supported E-UTRA bandwidths of CA_3C configuration

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_3C | 5,10,15 | 20 | 40 | 0 |
|  | 20 | 5,10,15,20 |  |  |

### 6.87.2	Channel bandwidths per operating band for DC

Table 6.87.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3C-41A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.87.3	Co-existence studies

Based on co-existence studies of Band 3 + Band n78 and Band 41 + Band n78 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	IMD generated by dual uplink of Band 3 + Band n78 doesn’t fall into own Rx of band 41.

-	3rd and 4th order IMD generated by dual uplink of Band 41 + Band n78 may also fall into own Rx of band 3.

The IMD3 issue can be comprehended in Table 2-1. The IMD4 issue can be comprehended in Table 2-2.

The harmonic and IMD for ISM and GNSS bands, and the protected band for the DC_3C-41A-n78A will be covered in the constituent fallback modes.

### 6.87.4	∆TIB and ∆RIB values

For DC_3C-41A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6. 87.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3C-41A-n78A | 3 | 0.6 |
|  | 41 | 0.31 |
|  |  | 0.82 |
|  | n78 | 0.8 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 6. 87.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3C-41A-n78A | 3 | 0.2 |
|  | 41 | 01 |
|  |  | 0.52 |
|  | n78 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

### 6.87.5	MSD

The same MSD as DC_3A-41A-n78A can be applied to this combination.

## 6.88	DC_7A-46A_n78A

### 6.88.1	Operating bands for DC

Table 6.88.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7A-46A_n78A | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz |  |

### 6.88.2	Channel bandwidths per operating band for DC

Table 6.88.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A-46A_n78A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 140 |
|  | 46 | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes |  | Yes |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes |  | Yes |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes |  | Yes |  |  | Yes | Yes | Yes | Yes | Yes |  |

6.88.2-2: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_7A-46A_n78A | DC_7A_n78A | CA_7A-46A | n78 |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. |  |  |  |

### 6.88.3	Co-existence studies

The dual uplink bands for DC_7A-46A_n78A are fixed to be Band 7 + Band n78, as Band 46 is not intended to operate stand alone without combining with licensed E-UTRA band such as Band 7.

Based on co-existence studies for DC_7A_n78 captured in TR 37.863-01-01 [5], own Rx impact for the 3rd band is as follows.

-	2nd or 5th order IMD generated by dual uplink of Band 7 + Band n78 may also fall into own Rx of Band 46.

### 6.88.4	∆TIB and ∆RIB values

For DC_7A-46A_n78A, the TIB,c and RIB,c values are given in the tables below.

Table 6.88.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-46A_n78A | 7 | 0.5 |
|  | n78 | 0.8 |

Table 6.88.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_7A-46A_n78A | 7 | 0 |
|  | n78 | 0.5 |

### 6.88.5	MSD

For this DC configuration, 2nd or 5th order IMD generated by dual uplink of Band 7 + Band n78 falls into own Rx of Band 46. In this case, the frequency gap to avoid MSD in Band 46 and the UL configuration are specified as shown in table 6.88.5-1.

Table 6.88.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_7A-46A_n78A1 | 7 | N/A | N/A | N/A | N/A | N/A | FDD | N/A |
|  | 46 | N/A | N/A | N/A | N/A | N/A | TDD | IMD2, IMD5 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
| NOTE 1: No requirements apply when there is at least one individual RE within the intermodulation generated by the dual uplink is within the downlink transmission bandwidth of the Band 46. The reference sensitivity should only be verified when this is not the case (the requirements for Band 46 specified in the CA_7A-46A in clause 7.3.1 of 36.101 [2] apply). |  |  |  |  |  |  |  |  |

## 6.89	DC_46C_n78A

### 6.89.1	Operating bands for DC

Table 6.89.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_46C_n78A | 46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz |  |

### 6.89.2	Channel bandwidths per operating band for DC

Table 6.89.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_46C_n78A1 | 46 | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  | 140 |
|  | n78 | 15 |  | Yes |  | Yes |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes |  | Yes |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes |  | Yes |  |  | Yes | Yes | Yes | Yes | Yes |  |
| NOTE 1: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured Pcell. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 6.90	DC_3C_n78A

### 6.90.1	Operating bands for DC

Table 6.90.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3C_n78A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.90.2	Channel bandwidths per operating band for DC

Table 6.90.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3C_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 140 |
|  | n78 | 15 |  | Yes |  | Yes |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes |  | Yes |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes |  | Yes |  |  | Yes | Yes | Yes | Yes | Yes |  |

### 6.90.3	Co-existence studies

The co-existence studies for DC_3C_n78A can refer to the studies for DC_3A_n78A captured in TR 37.863-01-01 [5].

### 6.90.4	∆TIB and ∆RIB values

For DC_3C_n78A, the TIB,c and RIB,c values are given in the tables below.

Table 6.90.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3C_n78A | 3 | 0.6 |
|  | n78 | 0.8 |
| NOTE 1: HTF is assumed for Band 3. |  |  |

Table 6.90.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3C_n78A | 3 | 0.2 |
|  | n78 | 0.5 |
| NOTE 1: HTF is assumed for Band 3. |  |  |

### 6.90.5	MSD

Tables below lists the MSD required for the dual connectivity configuration due to IMD and 2nd harmonic.

Table 6.90.5-1: MSD for the DC configuration for IMD2 and 4

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA/NR DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_3C-n78A | 3 | 1740 | 5 | 25 | 1835 | [26] | FDD | IMD24 |
|  |  |  |  |  |  | [28.75] |  |  |
|  | n78 | 3575 | 10 | 25 | 3575 | N/A | TDD | N/A |
|  | n78 | 3710 | 10 | 25 | 3710 | N/A | TDD | N/A |
| DC_3C-n78A | 3 | 1765 | 5 | 25 | 1860 | [8.0] | FDD | IMD44 |
|  |  |  |  |  |  | [10.75] |  |  |
|  | n78 | 3435 | 10 | 25 | 3435 | N/A | TDD | N/A |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in subclause 6.2.5A NOTE 2: RBSTART = 0 NOTE4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

MSD values for Band n78 due to 2nd harmonic of Band 3 in LTE-NR DC_3C_n78A are captured in Table 6.90.5-2.

Table 6.90.5-2: MSD due to harmonic issue for DC_3C_n78A

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| 3 | n781,2 | 27.1 | 23.9 | 22.1 | 20.9 |  | 17.9 |  |  |  |  |
|  | n783 | 1.9 | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 2: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. |  |  |  |  |  |  |  |  |  |  |  |

Regarding the uplink configuration, 40MHz for n78 should be considered additionally. Therefore, 20MHz with 100RB allocation for Band 3 should be added.

Table 6.90.5-3 Uplink configuration for the low band (exceptions) for E-UTRA

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LTE-NR DC band combination | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 20 MHz | Duplex mode |
| DC_3C_n78A | 3 |  |  | 12 | 26 | 39 | 531 | 1002 | FDD |
| NOTE 1: The configuration is used for measurement of MSD for NR channel bandwidth of 20MHz. NOTE 2: The configuration is used for measurement of MSD for NR channel bandwidth of 40MHz. |  |  |  |  |  |  |  |  |  |

## 6.91	DC_7C_n78A

### 6.91.1	Operating bands for DC

Table 6.91.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7C_n78A | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.91.2	Channel bandwidths per operating band for DC

Table 6.91.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7C_n78A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 140 |
|  | n78 | 15 |  | Yes |  | Yes |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes |  | Yes |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes |  | Yes |  |  | Yes | Yes | Yes | Yes | Yes |  |

### 6.91.3	Co-existence studies

The co-existence studies for DC_7C_n78A can refer to the studies for DC_7A_n78A captured in TR 37.863-01-01 [5].

### 6.91.4	∆TIB and ∆RIB values

For DC_7C_n78A, the TIB,c and RIB,c values are given in the tables below.

Table 6.91.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7C_n78A | 7 | 0.5 |
|  | n78 | 0.8 |

Table 6.91.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_7C_n78A | 7 | 0 |
|  | n78 | 0.5 |

### 6.91.5	MSD

There is no harmonic/IMD problems into the own RX frequency bands for DC_7C-n78A UE.

## 6.92	DC_1A-18A-n78A

### 6.92.1	Operating bands for DC

Table 6.92.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-18A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.92.2	Channel bandwidths per operating band for DC

Table 6.92.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-18A-n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 135 |
|  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.92.3	Co-existence studies

Based on co-existence studies of Band 1 + Band n78 and Band 18 + Band n78 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	5th order IMD generated by dual uplink of Band 1 + Band n78 may also fall into own Rx of band 18

-	3rd order IMD generated by dual uplink of Band 18 + Band n78 may also fall into own Rx of band 1

Taking into account that Band 18 is currently operated only by a certain operator in Japan, the frequency range can be limited as Band 1 UL/DL = 1920-1940/2110-2130 MHz and Band n78 UL/DL = 3600-3800/3600-3800 MHz. Then own Rx impact can be simplified as below.

-	5th order IMD generated by dual uplink of Band 1 + Band n78 doesn’t fall into own Rx.

-	3rd order IMD generated by dual uplink of Band 18 + Band n78 may also fall into own Rx of band 1

### 6.92.4	∆TIB and ∆RIB values

For DC_1A-18A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.92.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-18A-n78A | 1 | 0.3 |
|  | 18 | 0.3 |
|  | n78 | 0.8 |

Table 6.92.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-18A-n78A | 1 | 0 |
|  | 18 | 0 |
|  | n78 | 0.5 |

### 6.92.5	MSD

Table 6.92.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.92.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_1A-18A-n78A | 1 | TBD | TBD | TBD | TBD | TBD | FDD | TBD |
|  | 18 | TBD | TBD | TBD | TBD | TBD |  | TBD |
|  | n78 | TBD | TBD | TBD | TBD | TBD | TDD | TBD |

## 6.93	DC_1A-28A-n78

### 6.93.1	Operating bands for DC

Table 6.93.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-28A-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.93.2	Channel bandwidths per operating band for DC

Table 6.93.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-28A-n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-28A-n78C | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 240 |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 | See CA_n78C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.93.3	Co-existence studies

Based on co-existence studies of Band 1 + Band n78 and Band 28 + Band n78 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	5th order IMD generated by dual uplink of Band 1 + Band n78 may also fall into own Rx of band 28

-	3rd order IMD generated by dual uplink of Band 28 + Band n78 may also fall into own Rx of band 1

-	3rd order harmonic of band 28 may fall into own Rx of band 1.

### 6.93.4	∆TIB and ∆RIB values

For DC_1A-28A-n78, the TIB,c and RIB values are given in the tables below.

Table 6.93.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-28A-n78 | 1 | 0.3 |
|  | 28 | 0.6 |
|  | n78 | 0.8 |

Table 6.93.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-28A-n78 | 1 | 0 |
|  | 28 | 0.2 |
|  | n78 | 0.5 |

### 6.93.5	MSD

Table 6.93.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.93.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_1A-28A-n78 | 1 | TBD | TBD | TBD | TBD | TBD | FDD | TBD |
|  | 28 | TBD | TBD | TBD | TBD | TBD |  | TBD |
|  | n78 | TBD | TBD | TBD | TBD | TBD | TDD | TBD |

MSD requirement of 3rd order harmonic is the same as defined in Table 7.3.1A-0a of TS36.101 [2]?

## 6.94	DC_18A-28A-n78A

### 6.94.1	Operating bands for DC

Table 6.94.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_18A-28A-n78A | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.94.2	Channel bandwidths per operating band for DC

Table 6.94.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_18A-28A-n78A | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 135 |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.94.3	Co-existence studies

Based on co-existence studies of Band 18 + Band n78 and Band 28 + Band n78 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	5th order IMD generated by dual uplink of Band 18 + Band n78 may also fall into own Rx of band 28

-	5th order IMD generated by dual uplink of Band 28 + Band n78 may also fall into own Rx of band 18

### 6.94.4	∆TIB and ∆RIB values

For DC_1A-28A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.94.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18A-28A-n78A | 18 | 0.5 |
|  | 28 | 0.5 |
|  | n78 | 0.8 |

Table 6.94.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_18A-28A-n78A | 18 | 0 |
|  | 28 | 0 |
|  | n78 | 0.5 |

### 6.94.5	MSD

Table 6.94.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.94.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_18A-28A-n78A | 18 | TBD | TBD | TBD | TBD | TBD | FDD | TBD |
|  | 28 | TBD | TBD | TBD | TBD | TBD |  | TBD |
|  | n78 | TBD | TBD | TBD | TBD | TBD | TDD | TBD |

## 6.95	DC_1A-18A-n77A

### 6.95.1	Operating bands for DC

Table 6.95.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-18A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz |  |
|  | n78 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.95.2	Channel bandwidths per operating band for DC

Table 6.95.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-18A-n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 135 |
|  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.95.3	Co-existence studies

Based on co-existence studies of Band 1 + Band n77 and Band 18 + Band n77 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	5th order IMD generated by dual uplink of Band 1 + Band n77 may also fall into own Rx of band 18

-	3rd order IMD generated by dual uplink of Band 18 + Band n77 may also fall into own Rx of band 1

Taking into account that Band 18 is currently operated only by a certain operator in Japan, the frequency range can be limited as Band 1 UL/DL = 1920-1940/2110-2130 MHz and Band n78 UL/DL = 3600-4200/3600-4200 MHz. Then own Rx impact can be simplified as below.

-	5th order IMD generated by dual uplink of Band 1 + Band n78 doesn’t fall into own Rx.

-	3rd order IMD generated by dual uplink of Band 18 + Band n78 may also fall into own Rx of band 1

### 6.95.4	∆TIB and ∆RIB values

For DC_1A-18A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.95.4-2: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-18A-n77A | 1 | 0.3 |
|  | 18 | 0.3 |
|  | n77 | 0.8 |

Table 6.95.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-18A-n77A | 1 | 0 |
|  | 18 | 0 |
|  | n77 | 0.5 |

### 6.95.5	MSD

Table 6.95.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.95.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_1A-18A-n77A | 1 | TBD | TBD | TBD | TBD | TBD | FDD | TBD |
|  | 18 | TBD | TBD | TBD | TBD | TBD |  | TBD |
|  | n77 | TBD | TBD | TBD | TBD | TBD | TDD | TBD |

## 6.96	DC_1A-28A-n77

### 6.96.1	Operating bands for DC

Table 6.96.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-28A-n77A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.96.2	Channel bandwidths per operating band for DC

Table 6.96.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-28A-n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-28A-n77C | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 240 |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n77 | See CA_n77C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.96.3	Co-existence studies

Based on co-existence studies of Band 1 + Band n77 and Band 28 + Band n77 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	5th order IMD generated by dual uplink of Band 1 + Band n77 may also fall into own Rx of band 28

-	3rd order IMD generated by dual uplink of Band 28 + Band n77 may also fall into own Rx of band 1

-	3rd order harmonic of band 28 may fall into own Rx of band 1.

### 6.96.4	∆TIB and ∆RIB values

For DC_1A-28A-n77, the TIB,c and RIB values are given in the tables below.

Table 6.96.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-28A-n77 | 1 | 0.3 |
|  | 28 | 0.6 |
|  | n77 | 0.8 |

Table 6.96.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-28A-n77 | 1 | 0 |
|  | 28 | 0.2 |
|  | n77 | 0.5 |

### 6.96.5	MSD

Table 6.96.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.96.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_1A-28A-n77 | 1 | TBD | TBD | TBD | TBD | TBD | FDD | TBD |
|  | 28 | TBD | TBD | TBD | TBD | TBD |  | TBD |
|  | n77 | TBD | TBD | TBD | TBD | TBD | TDD | TBD |

## 6.97	DC_18A-28A-n77A

### 6.97.1	Operating bands for DC

Table 6.97.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_18A-28A-n77A | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.97.2	Channel bandwidths per operating band for DC

Table 6.97.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_18A-28A-n77A | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 135 |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.97.3	Co-existence studies

Based on co-existence studies of Band 18 + Band n77and Band 28 + Band n77 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	5th order IMD generated by dual uplink of Band 18 + Band n77 may also fall into own Rx of band 28

-	5th order IMD generated by dual uplink of Band 28 + Band n77 may also fall into own Rx of band 18

### 6.97.4	∆TIB and ∆RIB values

For DC_1A-28A-n77A, the TIB,c and RIB values are given in the tables below.

Table 6.97.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18A-28A-n77A | 18 | 0.5 |
|  | 28 | 0.5 |
|  | n77 | 0.8 |

Table 6.97.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_18A-28A-n77A | 18 | 0 |
|  | 28 | 0 |
|  | n77 | 0.5 |

### 6.97.5	MSD

Table 6.97.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.97.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_18A-28A-n77A | 18 | TBD | TBD | TBD | TBD | TBD | FDD | TBD |
|  | 28 | TBD | TBD | TBD | TBD | TBD |  | TBD |
|  | n77 | TBD | TBD | TBD | TBD | TBD | TDD | TBD |

## 6.98	DC_1A-41A-n79A

### 6.98.1	Operating bands for DC

Table 6.98.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-41A-n79A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz |  |

### 6.98.2	Channel bandwidths per operating band for DC

Table 6.98.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-41A-n79A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |

### 6.98.3	Co-existence studies

Based on co-existence studies of Band 1 + Band n79 and Band 41 + Band n79 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	2nd and 5th order IMD generated by dual uplink of Band 1 + Band n79 may also fall into own Rx of band 41

-	2nd and 5th order IMD generated by dual uplink of Band 41 + Band n79 may also fall into own Rx of band 1

-	Since UL of Band 41 may impact Band 1 receiver, it will not be used together with Band 1. Then intermodulation and harmonic impact can be simplified as follow.

-	2nd and 5th order IMD generated by dual uplink of Band 1 + Band n79 may fall into own Rx of band 41

### 6.98.4	∆TIB and ∆RIB values

For DC_1A-41A-n79A, the TIB,c and RIB values are given in the tables below.

Table 6.98.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-41A-n79A | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n79 | 0 |

Table 6.98.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-41A-n79A | 1 | 0 |
|  | 41 | 0 |
|  | n79 | 0 |

### 6.98.5	MSD

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-41A_n79A | 1 | IMD2 | ¦fB1 -fn79¦ | 1970 | 5 | 25 | 2160 | 5 | 2.2 | N/A |
|  | n79 |  |  | 4500 | 40 | 216 | 4500 | 40 |  |  |
|  | 41 |  |  | 2530 | 5 | 25 | 2530 | 5 |  | 29.4 |
|  | 1 | IMD5 | ¦4*fB1 –fn79¦ | 1922.5 | 5 | 25 | 2112.5 | 5 | 0 | N/A |
|  | n79 |  |  | 4980 | 40 | 216 | 4980 | 40 |  |  |
|  | 41 |  |  | 2687.5 | 5 | 25 | 2687.5 | 5 |  | 0.0 |

## 6.99	DC_41A-42A-n79A

### 6.99.1	Operating bands for DC

Table 6.99.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_41A-42A-n79A | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz |  |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz |  |

### 6.99.2	Channel bandwidths per operating band for DC

Table 6.99.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_41A-42A-n79A | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |

### 6.99.3	Co-existence studies

Based on co-existence studies of Band 41 + Band n79 and Band 42 + Band n79 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	4th order IMD generated by dual uplink of Band 41 + Band n79 may also fall into own Rx of band 42

-	3rd and 4th order IMD generated by dual uplink of Band 42 + Band n79 may also fall into own Rx of band 41

Since Band 41 and Band 42 are assumed of synchronized operation, there is no IMD issue for this dual connectivity configuration.

### 6.99.4	∆TIB and ∆RIB values

For DC_41A-42A-n79A, the TIB,c and RIB values are given in the tables below.

Table 6.99.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41A-42A-n79A | 41 | 0.3 |
|  | 42 | 0.8 |
|  | n79 | 0 |

Table 6.99.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_41A-42A-n79A | 41 | 0 |
|  | 42 | 0.5 |
|  | n79 | 0 |

### 6.99.5	MSD

There is no MSD requirement for this dual connectivity configuration.

## 6.100	DC_42C-n79

### 6.100.1	Operating bands for DC

Table 6.100.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_42C-n79 | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz |  |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz |  |

### 6.100.2	Channel bandwidths per operating band for DC

Table 6.100.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_42C-n79A | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
| DC_42C-n79C | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 240 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n79 | See CA_n79C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.100.3	Co-existence studies

Based on co-existence studies of Band 42 + Band n79  captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	3rd order IMD generated by dual uplink of Band 42 + Band n79 may also fall into own Rx of Band 42

Taking into account of the synchronization operation of Band 42,  own Rx impact can be simplified as below.

-	3rd order IMD generated by dual uplink of Band 42 + Band n79 may not fall into own Rx of Band 42

### 6.100.4	∆TIB and ∆RIB values

For DC_42C-n79, the TIB,c and RIB values are given in the tables below.

Table 6.100.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_42C-n79 | 42 | 0 |
|  | 42 | 0 |
|  | n79 | 0 |

Table 6.100.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_42C-n79 | 42 | 0 |
|  | 42 | 0 |
|  | n79 | 0 |

### 6.100.5	MSD

There is no MSD issue for this DC configuration.

## 6.101	DC_1A-41A-n257A

### 6.101.1	Operating bands for DC

Table 6.101.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-41A-n257A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

### 6.101.2	Channel bandwidths per operating band for DC

Table 6.101.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100MHz | 200MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-41A-n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 440 |
|  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |

### 6.101.3	Co-existence studies

Based on co-existence studies of Band 1 + Band n257 and Band 41 + Band n257 captured in TR 37.863-01-01 [5], there is no own Rx impact of the 3rd band.

### 6.101.4	∆TIB and ∆RIB values

For DC_1A-41A-n257A, the TIB,c and RIB values are given in the tables below.

Table 6.101.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-41A-n257A | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n257 | 0 |

Table 6.101.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-41A-n79A | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n257 | 0 |

### 6.101.5	MSD

There is no MSD issue for this DC configuration.

## 6.102	DC_41A-42A-n257A

### 6.102.1	Operating bands for DC

Table 6.102.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_41A-42A-n257A | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz |  |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

### 6.102.2	Channel bandwidths per operating band for DC

Table 6.102.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100MHz | 200MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_41A-42A-n257A | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 440 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |

### 6.102.3	Co-existence studies

Based on co-existence studies of Band 41 + Band n257 and Band 42 + Band n257 captured in TR 37.863-01-01 [5], there is no own Rx impact of the 3rd band.

### 6.102.4	∆TIB and ∆RIB values

For DC_41A-42A-n257A, the TIB,c and RIB values are given in the tables below.

Table 6.102.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41A-42A-n257A | 41 | 0.3 |
|  | 42 | 0.8 |
|  | n257 | 0 |

Table 6.102.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_41A-42A-n257A | 41 | 0 |
|  | 42 | 0.5 |
|  | n257 | 0 |

### 6.102.5	MSD

There is no MSD issue for this DC configuration.

## 6.103	DC_21A-42A_n77

### 6.103.1	Operating bands for DC

Table 6.103.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21A-42A_n77 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz |  |

### 6.103.2	Channel bandwidths per operating band for DC

Table 6.103.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_21A-42A_n77A | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 135 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_21A-42A_n77C | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 235 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n77 | See CA_n77C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.103.3	Co-existence studies

Based on co-existence studies of Band 21 + Band n77 and Band 42 + Band n77 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	3rd and 5th order IMD generated by dual uplink of Band 21 + Band n77 may also fall into own Rx of band 42. However, since simultaneous TX-RX of Band 42 + Band n77 is not assumed, there is no IMD issue.

-	4th order IMD generated by dual uplink of Band 42 + Band n77 may also fall into own Rx of band 21. However, since simultaneous TX-RX of Band 42 + Band n77 is not assumed, there is no IMD issue.

### 6.103.4	∆TIB and ∆RIB values

For DC_21A-42A_n77, the TIB,c and RIB values are given in the tables below.

Table 6.103.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21A-42A_n77 | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |

Table 6.103.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_21A-42A_n77 | 21 | 0 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |

### 6.103.5	MSD

As described in 6.103.3, no IMD issue is expected and no MSD requirement is needed.

## 6.104	DC_21A-42A_n78

### 6.104.1	Operating bands for DC

Table 6.104.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21A-42A_n78 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz |  |

### 6.104.2	Channel bandwidths per operating band for DC

Table 6.104.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_21A-42A_n78A | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 135 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_21A-42A_n78C | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 235 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 | See CA_n78C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.104.3	Co-existence studies

Based on co-existence studies of Band 21 + Band n78 and Band 42 + Band n78 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	IMD generated by dual uplink of Band 21 + Band n78 doesn’t impact own Rx of band 42 since simultaneous TX-RX of Band 42 + Band n78 is not assumed.

-	IMD generated by dual uplink of Band 42 + Band n78 doesn’t fall into own Rx of band 21 since simultaneous TX-RX of Band 42 + Band n78 is not assumed.

### 6.104.4	∆TIB and ∆RIB values

For DC_21A-42A_n78A, the TIB,c and RIB values are given in the tables below.

Table 6.104.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21A-42A_n78 | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |

Table 6.104.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_21A-42A_n78 | 21 | 0 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |

### 6.104.5	MSD

As described in 6.104.3, there is no IMD issue for this DC configuration.

## 6.105	DC_21A-42A_n79

### 6.105.1	Operating bands for DC

Table 6.105.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21A-42A_n79 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz |  |

### 6.105.2	Channel bandwidths per operating band for DC

Table 6.105.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_21A-42A_n79A | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 135 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
| DC_21A-42A_n79C | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 235 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n79 | See CA_n79C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.105.3	Co-existence studies

Based on co-existence studies of Band 21 + Band n79 and Band 42 + Band n79 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	2nd order IMD generated by dual uplink of Band 21 + Band n79 may also fall into own Rx of band 42

-	2nd and 5th order IMD generated by dual uplink of Band 42 + Band n79 may also fall into own Rx of band 21

However, considering actual spectrum holdings, no IMD issue is expected.

### 6.105.4	∆TIB and ∆RIB values

For DC_21A-42A_n79, the TIB,c and RIB values are given in the tables below.

Table 6.105.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21A-42A_n79 | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n79 | 0 |

Table 6.105.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_21A-42A_n79 | 21 | 0 |
|  | 42 | 0.5 |
|  | n79 | 0 |

### 6.105.5	MSD

As described in 6.105.3, no IMD issue is expected considering actual spectrum holdings and no MSD requirement is needed._

## 6.106	DC_21A-42A_n257

### 6.106.1	Operating bands for DC

Table 6.106.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21A-42A_n257 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

### 6.106.2	Channel bandwidths per operating band for DC

Table 6.106.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_21A-42A_n257A | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  | 435 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |
| DC_21A-42A_n257D | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  | 435 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_21A-42A_n257E | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  | 635 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_21A-42A_n257F | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  | 835 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |

### 6.106.3	Co-existence studies

Based on co-existence studies of Band 21 + Band n257 and Band 42 + Band n257 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	IMD generated by dual uplink of Band 21 + Band n257 doesn’t fall into own Rx of band 42

-	IMD generated by dual uplink of Band 42 + Band n257 doesn’t fall into own Rx of band 21

### 6.106.4	∆TIB and ∆RIB values

For DC_21A-42A_n257, the TIB,c and RIB values are given in the tables below.

Table 6.106.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21A-42A_n257 | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n257 | 0 |

Table 6.106.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_21A-42A_n257 | 21 | 0 |
|  | 42 | 0.5 |
|  | n257 | 0 |

### 6.106.5	MSD

As described in 6.106.3, no IMD issue is expected.

## 6.107	DC_66A_(n)71B

### 6.107.1	Operating bands for DC

Table 6.107.1-1: DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66A_(n)71B | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

### 6.107.2	Channel bandwidths per operating band for DC

Table 6.107.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 50 MHz | 80 MHz | 100 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] | Total Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_(n)71B | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 20 | 40 |
|  | 71 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 20 |  |
|  | n71 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes |  |  |  |  |  |  |  |  |

### 6.107.3	Co-existence studies

The DC_71b_n71b part is a contiguous allocation thus only uplink harmonic analysis is needed.

Table 6.107.3-1: DC_71b_n71b UL harmonics

| Uplink band |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 71 | 663 | 698 | 1326 | 1396 | 1989 | 2094 |

From Table 6.107.3-1 it can be seen that uplink harmonics 3rd harmonic overlaps with bands 3, 25, 34 and 70.

Table 6.107.3-2 lists Band 66 +Band 71/n71 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.107.3-2: Band 66 and Band 71/n71 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 698 |  | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1326 |  | 1396 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 1989 |  | 2094 | 5130 | 5340 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
|  | 1117 | 1012 |  | 2373 | 2478 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
|  | 454 |  | 314 | 2722 | 2897 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
|  | 3036 |  | 3176 | 4083 | 4258 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
|  | 2234 |  | 2024 | 4746 | 4956 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
|  | 209 | 384 |  | 4432 | 4677 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
|  | 3699 |  | 3874 | 5793 | 6038 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ |  | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
|  | 6457 |  | 6142 | 1082 | 872 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
|  | 7503 |  | 7818 | 4362 | 4572 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ |  | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
|  | 4014 |  | 3734 | 1326 | 1571 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
|  | 6456 |  | 6736 | 5409 | 5654 |

Based on Table 6.107.3-2;

-	2nd order harmonic may fall into Rx frequencies of bands 22, 42, 48, 77 and 78.

-	3rd order harmonic may fall into Rx frequencies of bands 2, 25, 34, 36, 46 and 70.

-	2nd order IMD may fall into Rx frequencies of band 30 and 40.

-	3rd order IMD may fall into Rx frequencies of band 77.

-	4th order IMD may fall into own Rx frequencies of band 66 as well as Rx frequencies of band 1, 4, 10, 23, 34, 43, 47, 48, 65, 77, 79 and 79.

-	5th order IMD may fall into Rx frequencies of band 5, 6, 8, 11, 18, 19, 21, 24, 26, 32, 43, 45, 46, 50, 74, 75, 76, 77, 78 and 79.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.107.3-3 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.107.3-3: 2UL Band 66 +Band n71 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD5 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD5 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD5 |
| GPS | 1563 | - | 1587 | Yes |  | IMD5 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD4, IMD5 |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | Yes |  | IMD5 |
|  | 5150 | - | 5825 | Yes | Asia | IMD4, IMD5 |

Table 6.107.3-4 lists the protected bands required for the dual connectivity configuration.

Table 6.107.3-4: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_66A_(n)71B | E-UTRA Band 4, 5, 7,10, 13, 14, 17, 22, 24, 26, 27, 29, 30, 43, 50, 51, 66, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 42, 48, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 3 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

### 6.107.4	∆TIB and ∆RIB values

For DC_66A_(n)71B, the TIB,c and RIB values are given in the tables below.

Table 6.107.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66A_(n)71B | 66 | 0.3 |
|  | 71 | 0.3 |
|  | n71 | 0.3 |

Table 6.107.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66A_(n)71B | 66 | 0 |
|  | 71 | 0 |
|  | n71 | 0 |

### 6.107.5	MSD

A combination of Band 66 and band 71 have similarities to CA_1A-8A and CA_3A-5A. Both these are a low band plus a high band, and both have IMD4 affecting the high band. CA_3A-5A has 4 dB and CA_1A-8A has 6 dB as MSD, and for a combination of band 66 and band 71 the MSD is set to an average value of 5 dB.

Table 6.107.5-1 lists the MSD required for the dual connectivity configuration DC_66A_(n)71B.

Table 6.107.5-1: MSD for the DC_66A_(n)71B configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_66A_(n)71B | 66 | 1750 | 5 | 25 | 2150 | 5 | FDD | IMD4 |
|  | n71 | 678 | 10 | 10 (RBstart =0) | 632 | N/A |  | N/A |

## 6.108	DC_3A-28A_n78_BCS0

### 6.108.1	Operating bands for DC

Table 6.108.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-28A_n78A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.108.2	Channel bandwidths per operating band for DC

Table 6.108.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| DC_3A-28A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 | 0 |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |

### 6.108.3	Co-existence studies

Tables 6.108.3-1 and 6.108.3-2 list Band 3A-20A + Band n78A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.108.3-1: Band 3 and Band 78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 |  | 3300 | 3800 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 3420 – 3570 |  |  | 6600 – 7600 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 5130 – 5355 |  |  | 9900 – 11400 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1515 – 2090 |  |  | 5010 – 5585 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 270 – 380 |  |  | 4815 – 5890 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 6720 – 7370 |  |  | 8310 – 9385 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 1610 – 1885 |  |  | 3280 – 3820 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1330 – 2055 |  |  | 8115 – 9690 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 3030 – 4180 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8430 – 9155 |  |  | 11610 – 13185 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 10020 – 11170 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11415 – 13490 |  |  | 3040 – 3840 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6330 – 7980 |  |  | 1245 – 2470 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 – 16985 |  |  | 10140 – 10940 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 – 14970 |  |  | 11730 – 12955 |  |  |

Based on Table 6.108.3-1,

-	2nd order harmonics may fall into own Rx of band n78 and Rx frequencies of Band 22, 42, 48and n77

-	3rd order harmonics may fall into Rx frequencies of Band 46

-	2nd order IMD may fall into own Rx of band 3 and Rx frequencies of Band 2, 9, 24, 25, 33, 34, 35, 36, 37, 39, 46, and 70

-	3rd order IMD may fall into own Rx of band n78, and Rx frequencies of Band 9, 22, 35, 39, 42, 43, 46, 47, 48, n77 and n78

-	4th order IMD may fall into Rx frequencies of Band 2, 3, 9, 11, 21, 22, 24, 25, 32, 33, 34, 35, 36, 37, 39, 42, 43, 45, 48, 70and n77

-	and 5th order IMD may fall into own Rx of band 3 and n78, and also Rx frequencies of Band 1, 2, 3, 4, 9, 10, 11, 21, 22, 23, 24, 25,30, 32, 33, 34, 35, 36, 37, 39, 40, 42, 43, 45, 48, 70and n77

-	It should be noted that IMD will not be an issue for Band n78 (no self-interference for the TDD band) even through the IMD products may fall into the concerning band

Table 6.108.3-2: Band 28 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 3300 | 3800 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 6600 |  | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 9900 |  | 11400 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 3097 | 2552 | 4003 |  | 4548 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 2394 | 1804 | 5852 |  | 6897 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 4706 | 5296 | 7303 |  | 8348 |
| Two-tone 3rd order IMD products | ¦fx_low – max BW fy¦ | ¦fx_high + max BW fy¦ | ¦fy_low – max BW fx¦ |  | ¦fy_high + max BW fx¦ |
| IMD frequency limits (MHz) | 603 | 848 | 3280 |  | 3820 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 6194 | 5104 | 8006 |  | 9096 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ |  | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1691 | 1056 | 9152 |  | 10697 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ |  | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5409 | 6044 | 10603 |  | 12148 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ |  | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14497 | 12452 | 308 |  | 988 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ |  | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13903 | 15948 | 6112 |  | 6792 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ |  | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9994 | 8404 | 4356 |  | 5491 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ |  | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11306 | 12896 | 8709 |  | 9844 |

Based on Table 6.108.3-2,

-	2nd order harmonics may fall into Rx frequencies of Band 11, 21, 32, 45, 50, 51, 74, 75 and 76

-	3rd order harmonics may fall into Rx frequencies of Band 1, 4, 10, 23, 65 and 66

-	2nd order IMD may fall into Rx frequencies of Band 7, 38, 41, 69, 77 and 79

-	3rd order IMD may fall into own Rx of band 3 and Rx frequencies of Band 1, 2, 3, 4, 9, 10, 23, 25, 30, 33, 34, 35, 36, 37, 39, 40, 46, 47, 65, 66, 70 and 79

-	4th order IMD may fall into Rx frequencies of Band 11, 21, 24, 32, 45, 46, 47, 50, 51, 74, 75, 76

-	5th order IMD may fall into own Rx of band 28 and Rx frequencies of Band 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 29, 31, 44, 46, 67, 68, 71, 72 and 79

-	It should be noted that IMD will not be an issue for Band n78 (no self-interference for the TDD band) even though the IMD products may fall into the concerning band

When 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS; the harmonics and intermodulation products can have impact on these systems. Table 6.108.3-3 and 6.108.3-4 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.108.3-3: 2UL Band 3 + Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD2, 4, 5 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD2, 4, 5 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD2, 4, 5 |
| GPS | 1563 | - | 1587 | Yes |  | IMD2, 4, 5 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD5 |
|  | 2400 | - | 2494 | Yes | Asia | IMD5 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | Harmonics3, IMD2, 3 |
|  | 5150 | - | 5350 | Yes | Europe | Harmonics3, IMD2, 3 |
|  | 5470 | - | 5725 | Yes |  | IMD2, 3 |
|  | 5150 | - | 5825 | Yes | Asia | Harmonics3, IMD2, 3 |

Table 6.108.3-4: 2UL Band 28 + Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD4 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD4 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD4 |
| GPS | 1563 | - | 1587 | Yes |  | IMD4 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3, 4, 5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD4, 5 |
|  | 5470 | - | 5725 | Yes |  | IMD4, 5 |
|  | 5150 | - | 5825 | Yes | Asia | IMD3, 4, 5 |

Table 6.108.3-5 lists the protected bands required for the dual connectivity configuration.

Table 6.108.3-5: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_3A-28A_n78A | TBD | FDL_low | - | FDL_high | TBD | TBD |  |
| NOTE 1: |  |  |  |  |  |  |  |

### 6.108.4	∆TIB and ∆RIB values

For DC_3A-28A_n78A, the TIB,c and RIB values are given in the tables below.

Table 6.108.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-28A_n78A | 3 | 0.5 |
|  | 28 | 0.3 |
|  | n78 | 0.8 |

Table 6.108.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-28A_n78A | 3 | 0.2 |
|  | 28 | 0 |
|  | n78 | 0.5 |

### 6.108.5	MSD

Table 6.108.5-1 lists the MSD required for the dual connectivity configuration. Band 3 IMD2, IMD4 and IMD5 values are derived from Band 3 IMD2 and IMD4 for DC_3A-5A_n78A. Band 3 IMD3 value is derived from Band 3 IMD3 for DC_3A-20A_n78A. Band 28 IMD5 value is derived from Band 5 IMD4 DC_3A-5A_n78A

Table 6.108.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_3A-28A_n78A | 3 | 1750 | 5 | 25 | 1850 | [26] | FDD | IMD2 |
|  | 28 | 760 | 5 | 25 | 760 | N/A |  | N/A |
|  | n78 | 3600 | 10 | 25 | 3600 | N/A | TDD | N/A |
|  | 3 | 1775 | 5 | 25 | 1870 | 17.3 | FDD | IMD3 |
|  | 28 | 740 | 5 | 25 | 760 | N/A |  | N/A |
|  | n78 | 3350 | 10 | 25 | 3350 | N/A | TDD | N/A |
|  | 3 | 1775 | 5 | 25 | 1845 | [8.0] | FDD | IMD4 |
|  | 28 | 740 | 5 | 25 | 760 | N/A |  | N/A |
|  | n78 | 3480 | 10 | 25 | 3480 | N/A | TDD | N/A |
|  | 3 | 1775 | 5 | 25 | 1875 | [8.0] | FDD | IMD5 |
|  | 28 | 740 | 5 | 25 | 760.5 | N/A |  | N/A |
|  | n78 | 3600 | 10 | 25 | 3600 | N/A | TDD | N/A |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | FDD | N/A |
|  | 28 | 705 | 5 | 25 | 780 | 8.3 |  | IMD5 |
|  | n78 | 3600 | 10 | 25 | 3600 | N/A | TDD | N/A |

## 6.109	DC_7A-28A_n78A _BCS0

### 6.109.1	Operating bands for DC

Table 6.109.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7A-28A_n78A | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.109.2	Channel bandwidths per operating band for DC

Table 6.109.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| DC_7A-28A_n78A | 7 | 15 |  | Yes | Yes | Yes |  |  |  |  |  | 140 | 0 |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |

### 6.109.3	Co-existence studies

Tables 6.109.3-1 and 6.109.3-2 list Band 7A-28A + Band n78A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.109.3-1: Band 7 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 |  | 3300 | 3800 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 5000 – 5140 |  |  | 6600 – 7600 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 7500 – 7710 |  |  | 9900 – 11400 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 730 – 1300 |  |  | 5800 – 6370 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 1200 – 1840 |  |  | 4030 – 5100 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 8300 – 8940 |  |  | 9100 – 10170 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 2400 – 2670 |  |  | 3280 – 3820 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3700 – 4410 |  |  | 7330 – 8900 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 1460 – 2600 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10800 – 11510 |  |  | 12400 – 13970 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 11600 – 12740 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 10630 – 12700 |  |  | 6200 – 6980 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4760 – 6400 |  |  | 100 – 1110 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15700 – 17770 |  |  | 13300 – 14080 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14900 – 16540 |  |  | 14100 – 15310 |  |  |

Based on Table 6.109.3-1,

-	2nd order IMD may fall into own Rx of band 28 and Rx frequencies of Band 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 44, 46, 47, 67, and 68

-	3rd order IMD may fall into Rx frequencies of Band 3, 11, 21, 22, 24, 32, 38, 41, 42, 43, 45, 48 n77 and n78

-	4th order IMD may fall into Rx frequencies of Band 1, 2, 3, 4, 9, 10, 11, 21, 23, 24, 25, 30, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 43, 45, 65, 66, 69, 70, n77 and n78

-	5th order IMD may fall into own Rx of band 28 and Rx frequencies of Band 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 29, 31, 44, 46, 47, 67, 68

-	It should be noted that IMD will not be an issue for Band n78 (no self-interference for the TDD band)  even through the IMD products may fall into the concerning band

Table 6.109.3-2: Band 28 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 3300 | 3800 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 6600 |  | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 9900 |  | 11400 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 3097 | 2552 | 4003 |  | 4548 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 2394 | 1804 | 5852 |  | 6897 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 4706 | 5296 | 7303 |  | 8348 |
| Two-tone 3rd order IMD products | ¦fx_low – max BW fy¦ | ¦fx_high + max BW fy¦ | ¦fy_low – max BW fx¦ |  | ¦fy_high + max BW fx¦ |
| IMD frequency limits (MHz) | 603 | 848 | 3280 |  | 3820 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 6194 | 5104 | 8006 |  | 9096 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ |  | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1691 | 1056 | 9152 |  | 10697 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ |  | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5409 | 6044 | 10603 |  | 12148 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ |  | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14497 | 12452 | 308 |  | 988 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ |  | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13903 | 15948 | 6112 |  | 6792 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ |  | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9994 | 8404 | 4356 |  | 5491 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ |  | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11306 | 12896 | 8709 |  | 9844 |

Based on Table 6.x.3-2,

-	2nd order harmonics may fall into Rx frequencies of Band 11, 21, 32, 45, 50, 51, 74, 75 and 76

-	3rd order harmonics may fall into Rx frequencies of Band 1, 4, 10, 23, 65 and 66

-	2nd order IMD may fall into own Rx of band 7 and Rx frequencies of Band 7, 38, 41, 69, 77 and 79

-	3rd order IMD may fall into Rx frequencies of Band 1, 2, 3, 4, 9, 10, 23, 25, 30, 33, 34, 35, 36, 37, 39, 40, 46, 47, 65, 66, 70 and 79

-	4th order IMD may fall into Rx frequencies of Band 11, 21, 24, 32, 45, 46, 47, 50, 51, 74, 75, 76

-	5th order IMD may fall into own Rx of band 28 and Rx frequencies of Band 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 29, 31, 44, 46, 67, 68, 71, 72 and 79

-	It should be noted that IMD will not be an issue for Band n78 (no self-interference for the TDD band) even though the IMD products may fall into the concerning band

When 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS; the harmonics and intermodulation products can have impact on these systems. Table 6.109.3-3 and 6.109.3-4 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.109.3-3: 2UL Band 7 + Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD3, 4 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD3. 4 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD3, 4 |
| GPS | 1563 | - | 1587 | Yes |  | IMD3, 4 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD3, 4 |
|  | 2400 | - | 2494 | Yes | Asia | IMD3, 4 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD2, 5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD5 |
|  | 5470 | - | 5725 | Yes |  | IMD5 |
|  | 5150 | - | 5825 | Yes | Asia | IMD2, 5 |

Table 6.109.3-4: 2UL Band 28 + Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD4 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD4 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD4 |
| GPS | 1563 | - | 1587 | Yes |  | IMD4 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3, 4, 5 |
|  | 5150 | - | 5350 | Yes | Europe | IMD4, 5 |
|  | 5470 | - | 5725 | Yes |  | IMD4, 5 |
|  | 5150 | - | 5825 | Yes | Asia | IMD3, 4, 5 |

Table 6.109.3-5 lists the protected bands required for the dual connectivity configuration.

Table 6.109.3-5: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_7A-28A_n78A | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 39, 40, 41, 50, 51, 65, 66, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
|  | Frequency range | 3800 | - | 3805 | +1.6 | 5 | 5, 7, 8 |
|  | Frequency range | 3805 | - | 3825 | -15.5 | 5 | 5, 7, 8 |
|  | Frequency range | 3825 | - | 3850 | -40 | 1 | 5, 8 |

### 6.109.4	∆TIB and ∆RIB values

For DC_7A-28A_n78A, the TIB,c and RIB values are given in the tables below.

Table 6.109.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-28A_n78A | 7 | 0.3 |
|  | 28 | 0.3 |
|  | n78 | 0.8 |

Table 6.109.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7A-28A_n78A | 7 | 0 |
|  | 28 | 0 |
|  | n78 | 0.5 |

### 6.109.5	MSD

Table 6.109.5-1 lists the MSD required for the dual connectivity configuration.

Band 28 IMD2 MSD value as chosen from Band 5 in DC_5A-7A_n78A. Band 28 IMD5 MSD value as chosen from Band 20 in DC_7A-20A_n78A. Band 7 IMD2 MSD value as chosen from Band 7 in DC_7A-20A_n78A.

Table 6.109.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_7A-28A_n78A | 7 | 2570 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | 28 | 720 | 5 | 25 | 780 | 8.3 |  | IMD2 |
|  | n78 | 3350 | 10 | 52 | 3421 | N/A | TDD | N/A |
|  | 7 | 2570 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | 28 | 720 | 5 | 25 | 790 | 3.0 |  | IMD5 |
|  | n78 | 3460 | 10 | 52 | 3421 | N/A | TDD | N/A |
|  | 7 | 2570 | 5 | 25 | 2650 | 30.5 | FDD | IMD2 |
|  | 28 | 740 | 5 | 25 | 768 | N/A |  | N/A |
|  | n78 | 3390 | 10 | 52 | 3421 | N/A | TDD | N/A |

## 6.110		DC_7C_n78A

### 6.110.1	Operating bands for DC

Table 6.110.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7C_n78A | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.110.2	Channel bandwidths per operating band for DC

Table 6.110.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7C_n78A | 7 | 15 | See CA_7C Bandwidth Combination Set 0 in Table 5.6A.1-1 of TS 36.101 [2] |  |  |  |  |  |  |  |  | 140 |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.110.3	Co-existence studies

Tables 6.110.3-1 lists Band 7C + Band n78A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.110.3-1: Band 7 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 9900 | 11400 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 730 | 1300 | 5800 | 6370 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1200 | 1840 | 4030 | 5100 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8300 | 8940 | 9100 | 10170 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3700 | 4410 | 7330 | 8900 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10800 | 11510 | 12400 | 13970 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2600 | 1460 | 11600 | 12740 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12700 | 10630 | 6980 | 6200 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15700 | 17770 | 13300 | 14080 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6400 | 4760 | 1110 | 100 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14900 | 16540 | 14100 | 15310 |

Based on Table 6.110.3-1;

-	There was no harmonic issues at legacy E-UTRA bands and NR new band 3.3~4.2GHz and 4.4~5GHz

-	2nd order IMD may also fall into B5, B8, B12, B13, B14, B17, B18, B19, B20, B26, B27, B28, B67, B68 and TDD B44, B46 and B47.

-	3rd order IMD may also fall into B3, B11, B21, B24, B32 and TDD B45, n77

-	4th order IMD may also fall into B1, B2, B3, B4, B9, B10, B11, B21, B23, B24, B25, B30, B32, B65, B66, TDD B33, B34, B35, B36, B37, B38, B39, B40, B41, B43, B45, n77, n78, n79

-	5th order IMD may also fall into B5, B8, B12, B13, B14, B17, B18, B19, B20, B26, B27, B28, B29, B31, B67, B68, TDD B44, B46, B47 and n79.

The 4th IMD will be impact to the own received band n78. However, the NR TDD band is not any impacts from dual uplink transmission since there was no simultaneous Tx/Rx transmission in the n78 NR band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.110.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.110.3-2: 2UL Band 7 +Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | 3rd IMD |
| Galileo | 1559 | - | 1591 | Yes |  | 3rd IMD |
| GLONASS | 1591 | - | 1610 | Yes |  | 3rd IMD |
| GPS | 1563 | - | 1587 | Yes |  | 3rd IMD |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | 4th IMD |
|  | 2400 | - | 2494 | Yes | Asia | 4th IMD |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd & 5th IMD |
|  | 5150 | - | 5350 | Yes | Europe | 5th IMD |
|  | 5470 | - | 5725 | Yes |  | 5th IMD |
|  | 5150 | - | 5825 | Yes | Asia | 2nd & 5th IMD |

Table 6.110.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.110.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_7C_n78A | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 40, 50, 51, 65, 66, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 1,2,3 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 1,2,3 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 1,2 |
|  | Frequency range | 3800 | - | 3805 | +1.6 | 5 | 1,3,4 |
|  | Frequency range | 3805 | - | 3825 | -15.5 | 5 | 1,3,4 |
|  | Frequency range | 3825 | - | 3850 | -40 | 1 | 1,4 |
|  | Frequency range | 3850 | - | 4200 | -50 | 1 |  |
| NOTE 1: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 2: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 3: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 4: This requirement is applicable for any channel bandwidths within the range 3300 - 3800 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range [TBD – 3792.5] MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range [TBD - 3790] MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to [TBD] RB. |  |  |  |  |  |  |  |

### 6.110.4	∆TIB and ∆RIB values

For DC_7C_n78A, the TIB,c and RIB values are defined as given in the tables below.

Table 6.39.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7C_n78A | 7 | 0.5 |
|  | n78 | 0.8 |

Table 6.39.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7C_n78A | 7 | 0 |
|  | n78 | 0.5 |

### 6.110.5	MSD

From the coe110istence analysis Table 6.110.3-1, there was no harmonic/IMD problems into the own R110 frequency bands for DC_7C_n78A UE.

## 6.111	DC_3A-42A_n78

### 6.111.1	Operating bands for DC

Table 6.111.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-42A_n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz |  |

### 6.111.2	Channel bandwidths per operating band for DC

Table 6.111.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-42A-n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-42A-n78C | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 240 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 | See CA_n78C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.111.3	Co-existence studies

Based on co-existence studies of Band 3 + Band n78 and Band 42 + Band n78 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	2nd order harmonic and 3rd to 5th order IMDs generated by dual uplink of Band 3 + Band n78 may also fall into own Rx of band 42.

-	However, since simultaneous TX-RX of Band 42 + Band n78 is not assumed, there is no IMD issue.

### 6.111.4	∆TIB and ∆RIB values

For DC_3A-42A_n78, the TIB,c and RIB values are given in the tables below.

Table 6.111.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-42A_n78 | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |

Table 6.111.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-42A_n78 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |

### 6.111.5	MSD

As described in 6.111.3, no IMD issue is expected and no MSD requirement is needed.

## 6.112	DC_3A-42A-n79

### 6.112.1	Operating bands for DC

Table 6.112.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-42A-n79 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz |  |

### 6.112.2	Channel bandwidths per operating band for DC

Table 6.112.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-42A-n79A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-42A-n79C | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 240 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n79 | See CA_n79C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.112.3	Co-existence studies

Based on co-existence studies of Band 3 + Band n79 and Band 42 + Band n79 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	2nd order harmonic and 5th order IMD generated by dual uplink of Band 3 + Band n79 may also fall into own Rx of band 42

-	However, considering actual spectrum holdings, no IMD issue is expected.

### 6.112.4	∆TIB and ∆RIB values

For DC_3A-42A-n79, the TIB,c and RIB values are given in the tables below.

Table 6.112.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-42A-n79 | 3 | 0.0 |
|  | 42 | 0.0 |
|  | n79 | 0.0 |

Table 6.112.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-42A-n79 | 3 | 0.0 |
|  | 42 | 0.0 |
|  | n79 | 0.0 |

### 6.112.5	MSD

As described in 6.112.3, no IMD issue is expected considering actual spectrum holdings and no MSD requirement is needed.

## 6.113	DC_3A-42A-n257

### 6.113.1	Operating bands for DC

Table 6.113.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-42A-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

### 6.113.2	Channel bandwidths per operating band for DC

Table 6.113.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-42A-n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 440 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |
| DC_3A-42A-n257D | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 440 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_3A-42A-n257E | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 640 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_3A-42A-n257F | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 840 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |

### 6.113.3	Co-existence studies

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

### 6.113.4	∆TIB and ∆RIB values

For DC_3A-42A-n257, the TIB,c and RIB values are given in the tables below.

Table 6.113.4-2: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-42A-n257 | 3 | 0.0 |
|  | 42 | 0.0 |
|  | n257 | 0.0 |

Table 6.113.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-42A-n257 | 3 | 0.0 |
|  | 42 | 0.0 |
|  | n257 | 0.0 |

### 6.113.5	MSD

As described in 6.113.3, no IMD issue is expected.

## 6.114	DC_1A-41A-n78A

### 6.114.1	Operating bands for DC

Table 6.114.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-41A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Note :	Currently, for LTE UL carrier is only supported on Band 1 not Band 41 because the fall back mode 1UL/2DL LTE CA_1A-41A has the limitation that UL carrier is only supported on Band 1. This will be the case until RAN4 draw a conclusion to support B41 as Pcell for LTE CA_1A-41A.

### 6.114.2	Channel bandwidths per operating band for DC

Table 6.114.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-41A-n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.114.3	Co-existence studies

Note that currently for LTE UL carrier is only supported on Band 1 not Band 41 because the fall back mode LTE 1UL/2DL CA_1A-41A has the limitation that UL carrier is only supported on Band 1. This will be the case until RAN4 draw a conclusion to support B41 as Pcell for CA_1A-41A. So the Band 41 UL harmonics, Band 41 and Band n78 UL IMD are not issues for EN-DC 1A-41A-n78A for now.

Tables 6.114.3-1 list Band 1A + Band n78A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis. For the detailed harmonics and IMD frequency information of the corresponding combinations one can refer to TR37.863-01-01 [5].

Table 6.114.3-1: Band 1 and Band n78 UL harmonics and IMD products

| NR DC Configuration | E-UTRA/NR band | DL frequency range (MHz) | Rx Impact | Harmonic/IMD orders | Notes |
| --- | --- | --- | --- | --- | --- |
| DC_1A-n78A | 1 | 2110 MHz – 2170 MHz | Yes | IMD4 |  |
|  | n78 | 3300 MHz – 3800 MHz | No | IMD4 | IMD will not be an issue for TDD bands |

Based on co-existence studies of Band 1 + Band n78 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	4th order IMD generated by dual uplink of Band 1 + Band n78 may also fall into own Rx of band 41.

### 6.114.4	∆TIB and ∆RIB values

For DC_1A-41A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.114.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-41A-n78A | 1 | [0.5] |
|  | 41 | [0.5] |
|  | n78 | 0.8 |

Table 6.114.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-41A-n78A | 1 | [0] |
|  | 41 | [0] |
|  | n78 | 0.5 |

### 6.114.5	MSD

## 6.115	DC_5A-41A-n78A

### 6.115.1	Operating bands for DC

Table 6.115.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5A-41A-n78A | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.115.2	Channel bandwidths per operating band for DC

Table 6.115.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_5A-41A-n78A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  | 130 |
|  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.115.3	Co-existence studies

Tables 6.115.3-1 and 6.115.3-2 list Band 5A-41A + Band n78A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis. For the detailed harmonics and IMD frequency information of the corresponding combinations one can refer to TR37.863-01-01 [5].

Table 6. 115.3-1: Band 5 and Band n78 UL harmonics and IMD products

| NR DC Configuration | E-UTRA/NR band | DL frequency range (MHz) | Rx Impact | Harmonic/IMD orders | Notes |
| --- | --- | --- | --- | --- | --- |
| DC_5A-n78A | 5 | 824 MHz – 849 MHz | Yes | IMD4 |  |
|  | n78 | 3300 MHz – 3800 MHz | No |  |  |

Table 6.115.3-2: Band 41 and Band n78 UL harmonics and IMD products

| NR DC Configuration | E-UTRA/NR band | DL frequency range (MHz) | Rx Impact | Harmonic/IMD orders | Notes |
| --- | --- | --- | --- | --- | --- |
| DC_41A-n78A | 41 | 2496 MHz – 2690 MHz | No | IMD4 | IMD will not be an issue for TDD bands |
|  | n78 | 3300 MHz – 3800 MHz | No | IMD4 | IMD will not be an issue for TDD bands |

Based on co-existence studies of Band 5 + Band n78 and Band 41 + Band n78 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	3rd order Harmonic generated by uplink of Band 5 may also fall into own Rx of band 41

-	2nd order IMD generated by dual uplink of Band 5 + Band n78 may also fall into own Rx of band 41

-	2nd order and 5th order IMD generated by dual uplink of Band 41 + Band n78 may also fall into own Rx of band 5

### 6.115.4	∆TIB and ∆RIB values

For DC_5A-41A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.115.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5A-41A-n78A | 5 | [0.6] |
|  | 41 | [0.5] |
|  | n78 | 0.8 |

Table 6.115.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5A-41A-n78A | 5 | [0.2] |
|  | 41 | [0] |
|  | n78 | 0.5 |

### 6.115.5	MSD

Table 6.115.5-1 show the MSD of DC_5A_41A-n78A due to IMD2 and IMD5.

Table 6.115.5-1: B5 MSD due to DC_5A-41A_n78A IMD2 and IMD5 issues

## 6.116	DC_1A-41A-n77A

### 6.116.1	Operating bands for DC

Table 6.116.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-41A-n77A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz |  |

### 6.116.2	Channel bandwidths per operating band for DC

Table 6.116.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-41A-n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.116.3	Co-existence studies

Based on co-existence studies of Band 1 + Band n77 and Band 41 + Band n77 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	4th and 5th order IMD generated by dual uplink of Band 1 + Band n77 may also fall into own Rx of band 41

-	4th order IMD generated by dual uplink of Band 41 + Band n77 may also fall into own Rx of band 1

Since UL of Band 41 may impact Band 1 receiver, it will not be used together with Band 1. Then own Rx impact of the 3rd band can be simplified as follow.

-	4th and 5th order IMD generated by dual uplink of Band 1 + Band n77 may also fall into own Rx of band 41

### 6.116.4	∆TIB and ∆RIB values

For DC_1A-41A-n77A, the TIB,c and RIB values are given in the tables below.

Table 6.116.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-41A-n77A | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n77 | 0.8 |

Table 6.116.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-41A-n77A | 1 | 0 |
|  | 41 | 0 |
|  | n77 | 0.5 |

### 6.116.5	MSD

Table 6.116.5-1 provided the MSD levels from interested companies using the MSD principle for the dual connectivity configuration.

Table 6.116.5-1: MSD results from each companies

|  | E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- |
| DC Configuration |  | EUTRA and NR band | MSD (LGE) | MSD (MTK) | Avg. MSD |  |
| DC_1A-41A_n77A |  | 1 | 9.9 | 12.0 | 11.0 | IMD4 ¦fn77-3*fB1¦ |
|  |  | n77 |  |  |  |  |
|  |  | 41 |  |  |  |  |
|  |  | 1 | 4.0 | 3.1 | 3.6 | IMD5 ¦3*fB1 –2*fn77¦ |
|  |  | n77 |  |  |  |  |
|  |  | 41 |  |  |  |  |

Table 6.116.5-2 propose the required MSD levels for the dual connectivity configuration as average manner.

Table 6.116.5-2: MSD for the DC configuration

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-41A_n77A | 1 | IMD4 | ¦3*fB1 -fn77¦ | 1970 | 5 | 25 | 2160 | 5 | 0.7 | N/A |
|  | n77 |  |  | 3400 | 10 | 52 | 3400 | 10 |  |  |
|  | 41 |  |  | 2510 | 5 | 25 | 2510 | 5 |  | 11.0 |
|  | 1 | IMD5 | ¦3*fB1 –2*fn77¦ | 1930 | 5 | 25 | 2120 | 5 | 0.4 | N/A |
|  | n77 |  |  | 4150 | 10 | 52 | 4150 | 10 |  |  |
|  | 41 |  |  | 2510 | 5 | 25 | 2510 | 5 |  | 3.6 |

## 6.117	DC_41C-n79A

### 6.117.1	Operating bands for DC

Table 6.117.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz |  |

### 6.117.2	Channel bandwidths per operating band for DC

Table 6.117.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_41C-n79A | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |

### 6.117.3	Co-existence studies

Co-existence studies of this DC configuration can reuse the results of  DC_41A-n79A captured in TR 37.863-01-01 [5].

### 6.117.4	∆TIB and ∆RIB values

For DC_41C-n77A, the TIB,c and RIB values are given in the tables below.

Table 6.117.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41C-n77A | 41 | 0.3 |
|  | n77 | 0.8 |

Table 6.117.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_41C-n77A | 41 | 0 |
|  | n77 | 0.5 |

### 6.117.5	MSD

MSD requirement of DC_41A-n79A captured in TR 37.863-01-01 [5]is applied for this DC configuration.

## 6.118	DC_2A-66A_n71A

### 6.118.1	Operating bands for DC

Table 6.118.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band Combination | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duple118 mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2A-66A_n71A | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHZ | 2110 MHz | – | 2200MHz | FDD |
|  | n71 | 663 MHz – 698 MHz |  |  | 617 MHz – 652 MHz |  |  | FDD |

### 6.118.2	Channel bandwidths per operating band for DC

Table 6.118.2-1 Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

|  | DC operating / channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Ma118imum aggregated bandwidth [MHz] |
| DC_2A-66A_n71A | 2 | 15 | Yes | Yes | Yes | Yes | 55 |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |
|  | n71 | 15 | Yes | Yes | Yes |  |  |
|  |  | 30 |  | Yes | Yes |  |  |

### 6.118.3	Co-e118istence studies

Table 6.118.3-1, table 6.118.3-2 and table 6.118.3-3 list Band 2-Band66 + Band n71 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coe118istence analysis.

Table 6.118.3-1: Band 2 and Band n71 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 |  | 663 | 698 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 |  | 3820 | 1326 | 1396 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 |  | 5730 | 1989 | 2094 |
| 2nd order IMD products | ¦f2_low–f1_high¦ |  | ¦f2_high–f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency limits (MHz) | 1152 |  | 1247 | 2513 | 2608 |
| 3rd order IMD products | ¦f2_low–2*f1_high¦ |  | ¦f2_high–2*f1_low¦ | ¦2*f2_low–f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limits (MHz) | 3002 |  | 3157 | 454 | 584 |
| 3rd order IMD products | ¦2*f1_low + f2_low¦ |  | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
| IMD frequency limits (MHz) | 4363 |  | 4518 | 3176 | 3306 |
| 4rd order IMD products | ¦3*f1_low – f2_high¦ |  | ¦3*f1_high – f2_low¦ | ¦3*f2_low–f1_high¦ | ¦3*f2_high –f1_low¦ |
| IMD frequency limits (MHz) | 4852 |  | 5067 | 79 | 244 |
| 4rd order IMD products | ¦3*f1_low +f2_low¦ |  | ¦3*f1_high + f2_high¦ | ¦3*f2_low+f1_low¦ | ¦3*f2_high +f1_high¦ |
| IMD frequency limits (MHz) | 6213 |  | 6428 | 3839 | 4004 |
| 4rd order IMD products | ¦2*f1_low-2*f2_high ¦ |  | ¦2*f1_high –2*f2_low¦ | ¦2*f1_low+2*f2_low¦ | ¦2*f1_high +2*f2_high¦ |
| IMD frequency limits (MHz) | 2304 |  | 2494 | 5026 | 5216 |
| 5rd order IMD products | ¦f1_low –4*f2_high ¦ |  | ¦f1_high –4*f2_low¦ | ¦f2_low–4*f1_high¦ | ¦f2_high –4*f1_low¦ |
| IMD frequency limits (MHz) | 742 |  | 942 | 6702 | 6977 |
| 5rd order IMD products | ¦f1_low +4*f2_low¦ |  | ¦f1_high +4*f2_high¦ | ¦f2_low+4*f1_low¦ | ¦f2_high +4*f1_high¦ |
| IMD frequency limits (MHz) | 4502 |  | 4702 | 8063 | 8338 |
| 5rd order IMD products | ¦2*f1_low-3*f2_high¦ |  | ¦2*f1_high–3*f2_low¦ | ¦2*f2_low-3*f1_high¦ | ¦2*f2_high -3*f1_low¦ |
| IMD frequency limits (MHz) | 1606 |  | 1831 | 4154 | 4404 |
| 5rd order IMD products | ¦2*f1_low +3*f2_low¦ |  | ¦2*f1_high +3*f2_high¦ | ¦2*f2_low+3*f1_low¦ | ¦2*f2_high +3*f1_high¦ |
| IMD frequency limits (MHz) | 5689 |  | 5914 | 6876 | 7126 |

Based on Table 6.118.3-1, it can be seen that

-	2nd order harmonic may fall into Rx frequencies of bands 43, n77and n78.

-	3rd order harmonic may fall into Rx frequencies of bands 2, 25, 34, 36, 46 and 70.

-	2nd order IMD may fall into own Rx frequencies of bands 38, 41and 69.

-	3rd order IMD may fall into own Rx frequencies of bands 31, 72, 73, n77, n78 and n79.

-	4th order IMD may fall into own Rx frequencies of bands 30, 40, 46, n77 and n79.

-	5th order IMD may fall into own Rx frequencies of  bands 3, 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 44, 46, 47, 67, 68, n77 and n79.

Therefore it can be concluded that there is just 3rd order harmonic falling into own R118 frequencies of Band 2.

Table 6.118.3-2: Band 66 and Band n71 UL harmonics and IMD products

| UE UL carriers | fxfx_low | fxfx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 698 |  | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fxfx_low | 2*fxfx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1326 |  | 1396 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fxfx_low |  | 3*fxfx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 1989 |  | 2094 | 5130 | 5340 |
| 2nd order IMD products | ¦fy_high – fxfx_low¦ | ¦fy_low – fxfx_high¦ |  | ¦fy_low + fxfx_low¦ | ¦fy_high + fxfx_high¦ |
|  | 1117 | 1012 |  | 2373 | 2478 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fxfx_low¦ | ¦fy_low – 2*fxfx_high¦ |  | ¦2*fy_low – fxfx_high¦ | ¦2*fy_high – fxfx_low¦ |
|  | 454 |  | 314 | 2722 | 2897 |
| Two-tone 3rd order IMD products | ¦2*fxfx_low + fy_low¦ | ¦2*fxfx_high + fy_high¦ |  | ¦2*fy_low + fxfx_low¦ | ¦2*fy_high + fxfx_high¦ |
|  | 3036 |  | 3176 | 4083 | 4258 |
| Two-tone 4th order IMD products | ¦2*fxfx_low –2* fy_high¦ |  | ¦2*fxfx_high – 2*fy_low¦ | ¦2*fxfx_low +2* fy_low¦ | ¦2*fxfx_high +2* fy_high¦ |
|  | 2234 |  | 2024 | 4746 | 4956 |
| Two-tone 4th order IMD products | ¦3*fxfx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
|  | 209 | 384 |  | 4432 | 4677 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
|  | 3699 |  | 3874 | 5793 | 6038 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ |  | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
|  | 6457 |  | 6142 | 1082 | 872 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
|  | 7503 |  | 7818 | 4362 | 4572 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ |  | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
|  | 4014 |  | 3734 | 1326 | 1571 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
|  | 6456 |  | 6736 | 5409 | 5654 |

Based on Table 6.118.3-2, it can be seen that:

-	2nd order harmonic may fall into Rx frequencies of bands 22, 42, 48, 49, n77 and n78.

-	3rd order harmonic may fall into Rx frequencies of bands 2, 25, 34, 36, 46 and 70.

-	2nd order IMD may fall into Rx frequencies of band 40.

-	3rd order IMD may fall into Rx frequencies of band 44, 71, n77.

-	4th order IMD may fall into own Rxfrequencies of band 66 as well as Rx frequencies of band 1, 4, 10, 23, 34, 43, 46, 47, 48, 49, 65, 66, n77, n78 and n79.

-	5th order IMD may fall into Rx frequencies of band 5, 6, 8, 11, 18, 19, 21, 24, 26, 32, 43, 45, 46, 50, 51, 74, 75, 76, n77, n78 and n79.

Therefore it can be concluded that there is just 4th IMD falling into own Rx frequencies of Band 66.

Table 6.118.3-3: Band n71 UL harmonics

| Uplink band |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n71 | 663 | 698 | 1326 | 1396 | 1989 | 2094 |

Based on Table 6.118.3-3, it can be seen that 3rd harmonic overlaps with bands 2, 25, 34, 36 and 70. It should be noted that the third harmonic products of Band n71 fall within 1989 – 2094 MHz, which has 1MHz overlap with the downlink of Band 2 (1930 - 1990 MHz). Therefore, third harmonic interference from uplink in Band n71 to downlink in Band 2 could be observed.  However, the potential impact could be limited, because this third harmonic interference could happen only in the case where the lowest edge of Band n71 paired with simultaneous downlink in the uppermost edge of Band 2 is utilized. The guard bands defined for LTE and NR transmission bandwidth configuration could help to avoid the direct overlap while some effect due to spectral spreading of the third harmonic may still exist.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.118.3-4 and Table 6.118.3-5 list up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.118.3-4: Band 2 and Band n71 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | Yes |  | 5th IMD |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | 4th IMD |
|  | 2400 | - | 2494 | Yes | Asia | 4th IMD |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 3rd harmonic, 4th, 5th IMD |
|  | 5150 | - | 5350 | Yes | Europe | 4th IMD |
|  | 5470 | - | 5725 | Yes |  | 3rd harmonic, 5th IMD |
|  | 5150 | - | 5825 | Yes | Asia | 3rd harmonic, 4th, 5th IMD |

Table 6.118.3-5: Band 66 and Band n71 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | 5th IMD |
| Galileo | 1559 | - | 1591 | Yes |  | 5th IMD |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | Yes |  | 5th IMD |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | 2nd IMD |
|  | 2400 | - | 2494 | Yes | Asia | 2nd IMD |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 3rd harmonic, 4th, 5th IMD |
|  | 5150 | - | 5350 | Yes | Europe | 3rd harmonic |
|  | 5470 | - | 5725 | Yes |  | 5th IMD |
|  | 5150 | - | 5825 | Yes | Asia | 3rd harmonic, 4th, 5th IMD |

The protected bands for DC_2A-66A_n71A are covered in the constituent fall-back modes.

### 6.118.4	∆TIB and ∆RIB values

For DC_2A-66A_n71A, the TIB,c and RIB,c values are given in the tables below.

Table 6.118.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2A-66A_n71A | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n71 | 0.3 |

Table 6.118.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2A-66A_n71A | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n71 | 0 |

### 6.118.5	MSD requirements

The MSD requirements for DC_2A-66A_n71A was caused by the 3rd harmonic of uplink in Band n71 interfering with downlink in Band 2, 4th IMD interfering with downlink in Band 66 and Band n71 Rx 3rd order harmonic mixing. Table 6.118.5-1, table 6.118.5-2, and table 6.118.5-3 list the MSD required for the dual connectivity configuration.

Table 6.118.5-1: MSD for the DC_2A-66A_n71A (exceptions due to harmonic issue)

| MSD due to harmonic issue e118ception for the DL band |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duple118 mode |
| n71 | 21 | 4.6 | 1 | 0.7 | 0.6 | FDD |
|  | 22 | 1.2 | 1 | 0.7 | 0.6 |  |
| NOTE1: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band n71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 2: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band n71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. |  |  |  |  |  |  |

Table 6.118.5-2: MSD for the DC_2A-66A_n71A (exceptions due to harmonic mixing issue)

| MSD due to harmonic mi118ing issue exception for the DL band |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| 2 | n713,4 | 26.8 | 23.6 | 21.2 |  | FDD |
| NOTE 3: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel’s transmission bandwidth of an aggressor (higher) band. NOTE 4: The requirements should be verified for UL EARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |

Table 6.118.5-3: MSD for the DC_2A-66A_n71A (exceptions due to IMD issue)

| MSD due to IMD issue exception for the DL band |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA and NR DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplexmode | Source of IMD |
| DC_2A-66A_n71A | 66 | 1750 | 5 | 25 | 2150 | 5 | FDD | IMD4 |
|  | n71 | 675 | 5 | 25 | 629 | N/A |  | N/A |

## 6.119	DC_1A-18A-n79A

### 6.119.1	Operating bands for DC

Table 6.119.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-18A-n79A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz |  |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.119.2	Channel bandwidths per operating band for DC

Table 6.119.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-18A-n79A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 135 |
|  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |

### 6.119.3	Co-existence studies

Based on co-existence studies of Band 1 + Band n79 and Band 18 + Band n79 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	3rd and 4th order IMD generated by dual uplink of Band 1 + Band n79 may also fall into own Rx of band 18.

-	4th order IMD generated by dual uplink of Band 18 + Band n79 may also fall into own Rx of band 1

### 6.119.4	∆TIB and ∆RIB values

For DC_1A-18A-n79A, the TIB,c and RIB values are given in the tables below.

Table 6.119.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-18A-n79A | 1 | 0.3 |
|  | 18 | 0.3 |
|  | n79 | 0 |

Table 6.119.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-18A-n79A | 1 | 0.3 |
|  | 18 | 0.3 |
|  | n79 | 0 |

### 6.119.5	MSD

Table 6.119.5-1 lists the MSD required for the dual connectivity configuration. The same requirements of DC_1A-19A-n79 can be applied for this DC configuration.

Table 6.119.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_1A-18A-n79A | 1 | 1935 | 5 | 25 | 2125 | N/A | FDD | N/A |
|  | 18 | 822.5 | 5 | 25 | 867.5 | 18.3 |  | IMD3 |
|  | n79 | 4782.5 | 40 | 216 | 4782.5 | N/A | TDD | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | 18 | 820 | 5 | 25 | 865 | 8.9 |  | IMD4 |
|  | n79 | 4925 | 40 | 216 | 4925 | N/A | TDD | N/A |
|  | 1 | 1935 | 5 | 25 | 2125 | 8.1 | FDD | IMD4 |
|  | 18 | 822.5 | 5 | 25 | 867.5 | N/A |  | N/A |
|  | n79 | 4782.5 | 40 | 216 | 4782.5 | N/A | TDD | N/A |

## 6.120	DC_1A-28A-n79

### 6.120.1	Operating bands for DC

Table 6.120.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-28A-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.120.2	Channel bandwidths per operating band for DC

Table 6.120.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-28A-n79A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-28A-n79C | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 240 |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n79 | See CA_n79C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.120.3	Co-existence studies

Based on co-existence studies of Band 1 + Band n79 and Band 28 + Band n79 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	3rd and 4th order IMD generated by dual uplink of Band 1 + Band n79 may also fall into own Rx of band 28.

-	4th and 5th order IMD generated by dual uplink of Band 28 + Band n79 may also fall into own Rx of band 1.

-	3rd harmonic of Band 28 may fall into own Rx of band 1.

### 6.120.4	∆TIB and ∆RIB values

For DC_1A-28A-n79, the TIB,c and RIB values are given in the tables below.

Table 6.120.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-28A-n79 | 1 | 0.3 |
|  | 28 | 0.3 |
|  | n79 | 0 |

Table 6.120.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-28A-n79 | 1 | 0.3 |
|  | 28 | 0.3 |
|  | n79 | 0 |

### 6.120.5	MSD

Table 6.120.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.120.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_1A-28A-n79 | 1 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | 28 | 733 | 5 | 25 | 788 | 15.2 |  | IMD3 |
|  | n79 | 4648 | 40 | 216 | 4648 | N/A | TDD | N/A |
|  | 1 | 1925 | 5 | 25 | 2115 | N/A | FDD | N/A |
|  | 28 | 740 | 5 | 25 | 795 | 10.0 |  | IMD4 |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | TDD | N/A |
|  | 1 | 1977.5 | 5 | 25 | 2167.5 | 1.2 | FDD | IMD4 |
|  | 28 | 745.5 | 5 | 25 | 800.5 | N/A |  | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |
|  | 1 | 1935 | 5 | 25 | 2125 | 4.5 | FDD | IMD5 |
|  | 28 | 718 | 5 | 25 | 773 | N/A |  | N/A |
|  | n79 | 4807 | 40 | 216 | 4807 | N/A | TDD | N/A |

MSD requirement for 3rd harmonic of LTE CA_1A-28A specified in TS36.101 [2] is applied for this DC configuration.

## 6.121	DC_1A-28A-n257

### 6.121.1	Operating bands for DC

Table 6.121.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-28A-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.121.2	Channel bandwidths per operating band for DC

Table 6.121.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [50 MHz] | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-28A-n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 28 | 10 | Yes | Yes | Yes | Yes |  |  |  |  | 440 |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |
| DC_1A-28A-n257D | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 440 |
|  | 28 | 10 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_1A-28A-n257E | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 640 |
|  | 28 | 10 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_1A-28A-n257F | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 840 |
|  | 28 | 10 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |

### 6.121.3	Co-existence studies

Based on co-existence studies of Band 1 + Band n257 and Band 28 + Band n257 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	3rd harmonic of Band 28 may fall into own Rx of band 1.

### 6.121.4	∆TIB and ∆RIB values

For DC_1A-28A-n257, the TIB,c and RIB values are given in the tables below.

Table 6.121.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-28A-n257 | 1 | 0.3 |
|  | 28 | 0.3 |
|  | n257 | 0 |

Table 6.121.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-28A-n257 | 1 | 0.3 |
|  | 28 | 0.3 |
|  | n257 | 0 |

### 6.121.5	MSD

MSD requirement for 3rd harmonic of LTE CA_1A-28A specified in TS36.101 [2] is applied for this DC configuration.

## 6.122	DC_1A-28A_n79

### 6.122.1	Operating bands for DC

Table 6.122.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duple122 mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-28A_n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.122.2	Channel bandwidths per operating band for DC

Table 6.122.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Ma122imum aggregated bandwidth [MHz] |
| DC_1A-28A_n79A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-28A_n79C | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 240 |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n79 | See CA_n79C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.122.3	Co-e122istence studies

Based on co-existence studies of Band 1 + Band n79 and Band 28 + Band n79 captured in TR 37.863-01-01 [5], own R122 impact of the 3rd band is the followings.

-	3rd and 4th order IMD generated by dual uplink of Band 1 + Band n79 may also fall into own Rx of band 28.

-	4th and 5th order IMD generated by dual uplink of Band 28 + Band n79 may also fall into own Rx of band 1.

### 6.122.4	∆TIB and ∆RIB values

For DC_1A-28A_n79, the TIB,c and RIB,c values are given in the tables below.

Table 6.122.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-28A_n79 | 1 | 0.3 |
|  | 28 | 0.6 |
|  | n79 | 0 |

Table 6.122.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1A-28A_n79 | 1 | 0 |
|  | 28 | 0.2 |
|  | n79 | 0 |

### 6.122.5	MSD

## 6.123	DC_3A-28A_n77

### 6.123.1	Operating bands for DC

Table 6.123.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-28A_n77 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.123.2	Channel bandwidths per operating band for DC

Table 6.123.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-28A_n77A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-28A_n77C | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 240 |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n77 | See CA_n77C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.123.3	Co-existence studies

Based on co-existence studies of Band 3 + Band n77 and Band 28 + Band n77 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	3rd order IMD generated by dual uplink of Band 3 + Band n77 may also fall into own Rx of band 28.

-	3rd and 4th order IMD generated by dual uplink of Band 28 + Band n77 may also fall into own Rx of band 3.

### 6.123.4	∆TIB and ∆RIB values

For DC_3A-28A_n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.123.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-28A_n77 | 3 | 0.6 |
|  | 28 | 0.5 |
|  | n77 | 0.8 |

Table 6.123.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3A-28A_n77 | 3 | 0.2 |
|  | 28 | 0.2 |
|  | n77 | 0.5 |

### 6.123.5	MSD

Table 6.123.5-1 lists the required MSD levels from interested companies and propose required MSD levels using the MSD principle for the dual connectivity configuration.

Table 6.123.5-1: MSD results from each companies

|  | E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- |
| DC Configuration |  | EUTRA and NR band | MSD (LGE) | MSD (MTK) | Avg. MSD |  |
| DC_3A-28A_n77A |  | 3 | 16.2 | 14.4 | 15.3 | IMD3 ¦fn77-2*fB3¦ |
|  |  | n77 |  |  |  |  |
|  |  | 28 |  |  |  |  |
|  |  | 28 | 16.7 | 17.3 | 17.0 | IMD3 ¦fn77-2*fB28¦ |
|  |  | n77 |  |  |  |  |
|  |  | 3 |  |  |  |  |

Table 6.123.5-2 propose the required MSD levels for the dual connectivity configuration as average manner.

Table 6.123.5-2: MSD for the DC configuration

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA/NR DC |  | EUTRA/NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode |  |
| DL Configuration | UL Configuration |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| DC_3A-28A_n77A | DC_3A_n77A | 3 | 1712.5 | 5 | 25 | 1807.5 | - | FDD | - |
|  |  | 28 | 715 | 5 | 25 | 770 | 15.3 |  | IMD3 |
|  |  | n77 | 4195 | 10 | 52 | 4195 | - | TDD | - |
|  | DC_28A_n77A | 3 | 1755 | 5 | 25 | 1850 | 17.0 | FDD | IMD3 |
|  |  | 28 | 735 | 5 | 25 | 790 | - |  | - |
|  |  | n77 | 3320 | 10 | 52 | 3320 | - | TDD | - |

## 6.124		DC_3A-28A_n79

### 6.124.1	Operating bands for DC

Table 6.124.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-28A_n79 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.124.2	Channel bandwidths per operating band for DC

Table 6.124.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-28A_n79A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-28A_n79C | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 240 |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n79 | See CA_n79C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.124.3	Co-existence studies

Based on co-existence studies of Band 3 + Band n79 and Band 28 + Band n79 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	4th order IMD generated by dual uplink of Band 3 + Band n79 may also fall into own Rx of band 28.

-	5th order IMD generated by dual uplink of Band 28 + Band n79 may also fall into own Rx of band 3.

### 6.124.4	∆TIB and ∆RIB values

For DC_3A-28A_n79, the TIB,c and RIB,c values are given in the tables below.

Table 6.124.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-28A_n79 | 3 | 0.3 |
|  | 28 | 0.3 |
|  | n79 | 0 |

Table 6.124.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3A-28A_n79 | 3 | 0 |
|  | 28 | 0 |
|  | n79 | 0 |

### 6.124.5	MSD

Table 6.124.5-1 lists the required MSD levels for the dual connectivity configuration.

Table 6.124.5-1: MSD for the DC configuration

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA/NR DC |  | EUTRA/NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode |  |
| DL Configuration | UL Configuration |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| DC_3A-28A_n79A | DC_3A_n79A | 3 | 1770 | 5 | 25 | 1865 | - | FDD | - |
|  |  | 28 | 725 | 5 | 25 | 780 | 10.3 |  | IMD4 |
|  |  | n79 | 4530 | 40 | 216 | 4530 | - | TDD | - |
|  | DC_28A_n79A | 3 | 1775 | 5 | 25 | 1870 | 5.7 | FDD | IMD5 |
|  |  | 28 | 725 | 5 | 25 | 780 | - |  | - |
|  |  | n79 | 4770 | 40 | 216 | 4770 | - | TDD | - |

## 6.125		DC_21A-28A_n77

### 6.125.1	Operating bands for DC

Table 6.125.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21A-28A_n77 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.125.2	Channel bandwidths per operating band for DC

Table 6.125.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_21A-28A_n77A | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 135 |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_21A-28A_n77C | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 235 |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n77 | See CA_n77C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.125.3	Co-existence studies

Based on co-existence studies of Band 21 + Band n77 and Band 28 + Band n77 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	3rd order IMD generated by dual uplink of Band 21 + Band n77 may also fall into own Rx of band 28.

-	4th order IMD generated by dual uplink of Band 28 + Band n77 may also fall into own Rx of band 21.

### 6.125.4	∆TIB and ∆RIB values

For DC_21A-28A_n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.125.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21A-28A_n77 | 21 | 0.4 |
|  | 28 | 0.5 |
|  | n77 | 0.8 |

Table 6.125.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_21A-28A_n77 | 21 | 0.5 |
|  | 28 | 0.2 |
|  | n77 | 0.5 |

### 6.125.5	MSD

Table 6.125.5-1 lists the required MSD levels from interested companies and propose required MSD levels using the MSD principle for the dual connectivity configuration.

Table 6.125.5-1: MSD results from each companies

|  | E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- |
| DC Configuration |  | EUTRA and NR band | MSD (LGE) | MSD (MTK) | Avg. MSD |  |
| DC_21A-28A_n77 |  | 21 | 15.8 | 18.0 | 16.9 | IMD3 ¦fn77-2*fB21¦ |
|  |  | n77 |  |  |  |  |
|  |  | 28 |  |  |  |  |
|  |  | 28 | 11.0 | 8.8 | 9.9 | IMD4 ¦fn77-3*fB28¦ |
|  |  | n77 |  |  |  |  |
|  |  | 21 |  |  |  |  |

Table 6.125.5-2 propose the required MSD levels for the dual connectivity configuration as average manner.

Table 6.125.5-2: MSD for the DC configuration

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA/NR DC |  | EUTRA/NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode |  |
| DL Configuration | UL Configuration |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| DC_21A-28A_n77 | DC_21A_n77 | 21 | 1452 | 5 | 25 | 1500 | - | FDD | - |
|  |  | 28 | 730.5 | 5 | 25 | 785.5 | 16.9 |  | IMD3 |
|  |  | n77 | 3689.5 | 10 | 52 | 3689.5 | - | TDD | - |
|  | DC_28A_n77 | 21 | 1450.5 | 5 | 25 | 1498.5 | 9.9 | FDD | IMD4 |
|  |  | 28 | 730.5 | 5 | 25 | 785.5 | - |  | - |
|  |  | n77 | 3690 | 10 | 52 | 3690 | - | TDD | - |

## 6.126	DC_21A-28A_n78

### 6.126.1	Operating bands for DC

Table 6.126.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21A-28A_n78 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.126.2	Channel bandwidths per operating band for DC

Table 6.126.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_21A-28A_n78A | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 135 |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_21A-28A_n78C | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 235 |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 | See CA_n78C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.126.3	Co-existence studies

Based on co-existence studies of Band 21 + Band n78 and Band 28 + Band n78 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	3rd order IMD generated by dual uplink of Band 21 + Band n78 may also fall into own Rx of band 28.

-	4th order IMD generated by dual uplink of Band 28 + Band n78 may also fall into own Rx of band 21.

### 6.126.4	∆TIB and ∆RIB values

For DC_21A-28A_n78, the TIB,c and RIB,c values are given in the tables below.

Table 6.126.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21A-28A_n78 | 21 | 0.4 |
|  | 28 | 0.5 |
|  | n78 | 0.8 |

Table 6.126.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_21A-28A_n78 | 21 | 0 |
|  | 28 | 0.2 |
|  | n78 | 0.5 |

### 6.126.5	MSD

## 6.127	DC_21A-28A_n79

### 6.127.1	Operating bands for DC

Table 6.127.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21A-28A_n79 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.127.2	Channel bandwidths per operating band for DC

Table 6.127.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_21A-28A_n79A | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 135 |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
| DC_21A-28A_n79C | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 235 |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n79 | See CA_n79C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.127.3	Co-existence studies

Based on co-existence studies of Band 21 + Band n79 and Band 28 + Band n79 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	5th order IMD generated by dual uplink of Band 28 + Band n79 may also fall into own Rx of band 21.

### 6.127.4	∆TIB and ∆RIB values

For DC_21A-28A_n79, the TIB,c and RIB,c values are given in the tables below.

Table 6.127.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21A-28A_n79 | 21 | 0.4 |
|  | 28 | 0.3 |
|  | n79 | 0 |

Table 6.127.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_21A-28A_n79 | 21 | 0 |
|  | 28 | 0 |
|  | n79 | 0 |

### 6.127.5	MSD

Table 6.127.5-1 lists the required MSD levels for the dual connectivity configuration.

Table 6.127.5-1: MSD for the DC configuration

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA/NR DC |  | EUTRA/NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode |  |
| DL Configuration | UL Configuration |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| DC_21A-28A_n79A | DC_28A_n79A | 21 | 1450 | 5 | 25 | 1498 | 5.2 | FDD | IMD5 |
|  |  | 28 | 730.5 | 5 | 25 | 785.5 | - |  | - |
|  |  | n79 | 4420 | 40 | 216 | 4420 | - | TDD | - |

## 6.128	DC_28A-42A_n79

### 6.128.1	Operating bands for DC

Table 6.128.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28A-42A_n79 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz |  |

### 6.128.2	Channel bandwidths per operating band for DC

Table 6.128.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_28A-42A_n79A | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
| DC_28A-42A_n79C | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 240 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n79 | See CA_n79C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.128.3	Co-existence studies

Based on co-existence studies of Band 28 + Band n79 and Band 42 + Band n79 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	3rd order IMD generated by dual uplink of Band 28 + Band n79 may also fall into own Rx of band 42.

-	2nd and 5th order IMD generated by dual uplink of Band 42 + Band n79 may also fall into own Rx of band 28.

### 6.128.4	∆TIB and ∆RIB values

For DC_28A-42A_n79, the TIB,c and RIB,c values are given in the tables below.

Table 6.128.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28A-42A_n79 | 28 | 0.5 |
|  | 42 | 0.8 |
|  | n79 | 0 |

Table 6.128.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_28A-42A_n79 | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n79 | 0 |

### 6.128.5	MSD

Table 6.128.5-1 lists the required MSD levels for the dual connectivity configuration.

Table 6.128.5-1: MSD for the DC configuration

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA/NR DC |  | EUTRA/NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode |  |
| DL Configuration | UL Configuration |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| DC_28A-42A_n79A | DC_28A_n79A | 28 | 730 | 5 | 25 | 785 | - | FDD | - |
|  |  | 42 | 3420 | 5 | 25 | 3420 | 15.3 | TDD | IMD3 |
|  |  | n79 | 4880 | 40 | 216 | 4880 | - |  | - |
|  | DC_42A_n79A | 28 | 745 | 5 | 25 | 800 | 16.2 | FDD | IMD2 |
|  |  | 42 | 3597.5 | 5 | 25 | 3597.5 | - | TDD | - |
|  |  | n79 | 4420 | 40 | 216 | 4420 | - |  | - |

## 6.129	DC_1A-8A_n78

### 6.129.1	Operating bands for DC

Table 6.129.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-8A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915MHz | 925 MHz | – | 960 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.129.2	Channel bandwidths per operating band for DC

Table 6.129.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-8A-n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 130 |
|  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.129.3	Co-existence studies

Based on co-existence studies of Band 1 + Band n78 and Band 8 + Band n78 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	5th order IMD generated by dual uplink of Band 1 + Band n78 may also fall into own Rx of band 8

-	IMD generated by dual uplink of Band 8 + Band n78 doesn’t fall into own Rx of band 1

The harmonic and IMD for ISM and GNSS bands, and the protected band for the DC_1A-8A-n78A will be covered in the constituent fallback modes.

### 6.129.4	∆TIB and ∆RIB values

For DC_1A-8A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.129.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-8A-n78A | 1 | 0.3 |
|  | 8 | 0.6 |
|  | n78 | 0.8 |

Table 6.129.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-8A-n78A | 1 | 0 |
|  | 8 | 0.2 |
|  | n78 | 0.5 |

### 6.129.5	MSD

According the co-existing studies, although 5th order IMD generated by dual uplink of Band 1 + Band n78 may also fall into own Rx of band 8, based on the analysis of R4-18XXXX,  it is no need to specific additional MSD for Band 8 caused by IMD5 of Band 3 and Band n78 for DC_1A-8A_n78A.

## 6.130	DC_3A-8A_n78

### 6.130.1	Operating bands for DC

Table 6.130.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-8A-n78A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915MHz | 925 MHz | – | 960 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.130.2	Channel bandwidths per operating band for DC

Table 6.130.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-8A-n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.130.3	Co-existence studies

Based on co-existence studies of Band 3 + Band n78 and Band 8 + Band n78 captured in TR 37.863-01-01 [5], own Rx impact of the 3rd band is the followings.

-	IMD generated by dual uplink of Band 3 + Band n78 doesn’t fall into own Rx of band 8

-	2nd harmonic, 3rd order IMD generated by dual uplink of Band 8 + Band n78 may also fall into own Rx of band 3.   However, when considering the maximum supported channel bandwidth of Band 8 and Band n78, then the 3rd  order IMD doesn’t fall into own Rx of band 3. Therefore, only 2nd harmonic generated by dual uplink of Band 8 + Band n78 may also fall into own Rx of band 3.

The harmonic and IMD for ISM and GNSS bands, and the protected band for the DC_3A-8A-n78A will be covered in the constituent fallback modes.

### 6.130.4	∆TIB and ∆RIB values

For DC_3A-8A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.130.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-8A-n78A | 3 | 0.6 |
|  | 8 | 0.6 |
|  | n78 | 0.8 |

Table 6.130.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-8A-n78A | 3 | 0.2 |
|  | 8 | 0.2 |
|  | n78 | 0.5 |

### 6.130.5	MSD

According the co-existing studies, although 2nd harmonic generated by dual uplink of Band 8 + Band n78 may also fall into own Rx of band 3, based on the analysis of R4-18XXXX, there is significant opportunity to avoid the harmonic interference from the transmitter into the receiver. Therefore,  it is no need to specific additional MSD for Band 3 caused by 2nd order harmonic of Band 8 for DC_3A-8A_n78A.

## 6.131	DC_1A-18A-n257A

### 6.131.1	Operating bands for DC

Table 6.131.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-18A-n257A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz |  |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.131.2	Channel bandwidths per operating band for DC

Table 6.131.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [50 MHz] | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-18A-n257 | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  | 435 |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |

### 6.131.3	Co-existence studies

Based on co-existence studies of Band 1 + Band n257 and Band 18 + Band n257 captured in TR 37.863-01-01 [5], there is no IMD issue for this DC configuration.

### 6.131.4	∆TIB and ∆RIB values

For DC_1A-18A-n257A, the TIB,c and RIB values are given in the tables below.

Table 6.131.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-18A-n257A | 1 | 0.3 |
|  | 18 | 0.3 |
|  | n257 | 0 |

Table 6.131.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-18A-n257A | 1 | 0.3 |
|  | 18 | 0.3 |
|  | n257 | 0 |

### 6.131.5	MSD

There is no MSD issue for this DC configuration.

## 6.132	DC_1A-28A_n257

### 6.132.1	Operating bands for DC

Table 6.132.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-28A_n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.132.2	Channel bandwidths per operating band for DC

Table 6.132.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-28A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 440 |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |
| DC_1A-28A_n257D | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 440 |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_1A-28A_n257E | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 640 |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_1A-28A_n257F | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 840 |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |

### 6.132.3	Co-existence studies

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

### 6.132.4	∆TIB and ∆RIB values

For DC_1A-28A_n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.132.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-28A_n257 | 1 | 0.3 |
|  | 28 | 0.6 |
|  | n257 | 0 |

Table 6.132.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1A-28A_n257 | 1 | 0 |
|  | 28 | 0.2 |
|  | n257 | 0 |

### 6.132.5	MSD

As mentioned in 6.132.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 6.133	DC_3A-28A_n257

### 6.133.1	Operating bands for DC

Table 6.133.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-28A_n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.133.2	Channel bandwidths per operating band for DC

Table 6.133.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-28A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 440 |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |
| DC_3A-28A_n257D | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 440 |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_3A-28A_n257E | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 640 |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_3A-28A_n257F | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 840 |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |

### 6.133.3	Co-existence studies

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

### 6.133.4	∆TIB and ∆RIB values

For DC_3A-28A_n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.133.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-28A_n257 | 3 | 0.3 |
|  | 28 | 0.3 |
|  | n257 | 0 |

Table 6.133.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3A-28A_n257 | 3 | 0 |
|  | 28 | 0 |
|  | n257 | 0.5 |

### 6.133.5	MSD

As mentioned in 6.133.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 6.134	DC_21A-28A_n257

### 6.134.1	Operating bands for DC

Table 6.134.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21A-28A_n257 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.134.2	Channel bandwidths per operating band for DC

Table 6.134.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_21A-28A_n257A | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  | 435 |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |
| DC_21A-28A_n257D | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  | 435 |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_21A-28A_n257E | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  | 635 |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_21A-28A_n257F | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  | 835 |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |

### 6.134.3	Co-existence studies

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

### 6.134.4	∆TIB and ∆RIB values

For DC_21A-28A_n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.134.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21A-28A_n257 | 21 | 0.4 |
|  | 28 | 0.3 |
|  | n257 | 0 |

Table 6.134.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_21A-28A_n257 | 21 | 0 |
|  | 28 | 0 |
|  | n257 | 0 |

### 6.134.5	MSD

As mentioned in 6.134.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 6.135	DC_28A-42A_n77

### 6.135.1	Operating bands for DC

Table 6.135.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28A-42A_n77 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz |  |

### 6.135.2	Channel bandwidths per operating band for DC

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_28A-42A_n77A | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_28A-42A_n77C | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 240 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n77 | See CA_n77C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.135.3	Co-existence studies

There is no MSD issue for this DC configuration based on the following conditions.

-	LTE Band 42 and NR Band n77 are synchronized

-	Simultaneous transmission of LTE Band 42 and NR Band n77 are not supported at least in Rel-15 timeframe

### 6.135.4	∆TIB and ∆RIB values

For DC_28A-42A_n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.135.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28A-42A_n77 | 28 | 0.5 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |

Table 6.135.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_28A-42A_n77 | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |

### 6.135.5	MSD

As mentioned in 6.135.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 6.136	DC_28A-42A_n78

### 6.136.1	Operating bands for DC

Table 6.136.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28A-42A_n78 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz |  |

### 6.136.2	Channel bandwidths per operating band for DC

Table 6.136.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_28A-42A_n78A | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_28A-42A_n78C | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 240 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 | See CA_n78C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.136.3	Co-existence studies

There is no MSD issue for this DC configuration based on the following conditions.

-	LTE Band 42 and NR Band n78 are synchronized

-	Simultaneous transmission of LTE Band 42 and NR Band n78 are not supported at least in Rel-15 timeframe

### 6.136.4	∆TIB and ∆RIB values

For DC_28A-42A_n78, the TIB,c and RIB,c values are given in the tables below.

Table 6.136.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28A-42A_n78 | 28 | 0.5 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |

Table 6.136.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_28A-42A_n78 | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |

### 6.136.5	MSD

As mentioned in 6.136.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 6.137	DC_28A-42A_n257

### 6.137.1	Operating bands for DC

Table 6.137.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28A-42A_n257 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

### 6.137.2	Channel bandwidths per operating band for DC

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_28A-42A_n257A | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 440 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |
| DC_28A-42A_n257D | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 440 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_28A-42A_n257E | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 640 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |
| DC_28A-42A_n257F | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 840 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 [4] |  |  |  |  |  |  |  |  |  |

### 6.137.3	Co-existence studies

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

### 6.137.4	∆TIB and ∆RIB values

For DC_28A-42A_n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.137.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28A-42A_n257 | 28 | 0.5 |
|  | 42 | 0.8 |
|  | n257 | 0 |

Table 6.137.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_28A-42A_n257 | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n257 | 0 |

### 6.137.5	MSD

As mentioned in 6.137.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 6.138	DC_18A-28A-n79A

### 6.138.1	Operating bands for DC

Table 6.138.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_18A-28A-n79A | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.138.2	Channel bandwidths per operating band for DC

Table 6.138.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_18A-28A-n79A | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  | 135 |
|  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |

### 6.138.3	Co-existence studies

Based on co-existence studies of Band 18 + Band n79 and Band 28 + Band n79 captured in TR 37.863-01-01 [5], there is no own Rx impact of the 3rd band for this DC configuration .

### 6.138.4	∆TIB and ∆RIB values

For DC_18A-28A-n79A, the TIB,c and RIB values are given in the tables below.

Table 6.138.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18A-28A-n79A | 18 | 0.5 |
|  | 28 | 0.5 |
|  | n79 | 0 |

Table 6.138.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_18A-28A-n79A | 18 | 0 |
|  | 28 | 0 |
|  | n79 | 0 |

### 6.138.5	MSD

There is no MSD requirement for the dual connectivity configuration.

## 6.139	DC_18A-28A-n257A

### 6.139.1	Operating bands for DC

Table 6.139.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_18A-28A-n257A | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.139.2	Channel bandwidths per operating band for DC

Table 6.139.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [50 MHz] | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_18A-28A-n257 | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |
|  | 28 | 10 | Yes | Yes | Yes | Yes |  |  |  |  | 435 |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |

### 6.139.3	Co-existence studies

Based on co-existence studies of Band 18 + Band n257 and Band 28 + Band n257 captured in TR 37.863-01-01 [5], there is no own Rx impact of the 3rd band for this DC configuration .

### 6.139.4	∆TIB and ∆RIB values

For DC_18A-28A-n257A, the TIB,c and RIB values are given in the tables below.

Table 6.139.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18A-28A-n257A | 18 | 0.5 |
|  | 28 | 0.5 |
|  | n257 | 0 |

Table 6.139.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_18A-28A-n257A | 18 | 0 |
|  | 28 | 0 |
|  | n257 | 0 |

### 6.139.5	MSD

There is no MSD issue for this DC configuration.

## 6.140	DC_41A-42A-n77A

### 6.140.1	Operating bands for DC

Table 6.140.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_41A-42A-n77A | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz |  |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz |  |

### 6.140.2	Channel bandwidths per operating band for DC

Table 6.140.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_41A-42A-n77A | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.140.3	Co-existence studies

Based on co-existence studies of Band 41 + Band n77 captured in TR 37.863-01-01 [5], there is no harmonic issue for this DC configuration.

There is no IMD issue for this DC configuration based on the following conditions.

-	LTE Band 42 and NR Band n77 are synchronized

-	Simultaneous transmission of LTE Band 42 and NR Band n77 are not supported at least in Rel-15 timeframe

### 6.140.4	∆TIB and ∆RIB values

For DC_41A-42A-n77A, the TIB,c and RIB values are given in the tables below.

Table 6.140.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41A-42A-n77A | 41 | 0.3 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |

Table 6.140.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_41A-42A-n77A | 41 | 0 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |

### 6.140.5	MSD

There is no MSD issue for this DC configuration.

## 6.141	DC_41A-42A-n78A

### 6.141.1	Operating bands for DC

Table 6.141.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_41A-42A-n78A | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz |  |

### 6.141.2	Channel bandwidths per operating band for DC

Table 6.141.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_41A-42A-n78A | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.141.3	Co-existence studies

Based on co-existence studies of Band 41 + Band n78 captured in TR 37.863-01-01 [5], there is no harmonic issue for this DC configuration.

There is no IMD issue for this DC configuration based on the following conditions.

-	LTE Band 42 and NR Band n78 are synchronized

-	Simultaneous transmission of LTE Band 42 and NR Band n78 are not supported at least in Rel-15 timeframe

### 6.141.4	∆TIB and ∆RIB values

For DC_41A-42A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.141.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41A-42A-n78A | 41 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |

Table 6.141.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_41A-42A-n78A | 41 | 0 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |

### 6.141.5	MSD

There is no MSD issue for this DC configuration.

## 6.142	DC_41C-n77A

### 6.142.1	Operating bands for DC

Table 6.142.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz |  |

### 6.142.2	Channel bandwidths per operating band for DC

Table 6.142.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_41C-n77A | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.142.3	Co-existence studies

Co-existence studies of this DC configuration are the same as DC_41A-n77A captured in TR 37.863-01-01 [5].

### 6.142.4	∆TIB and ∆RIB values

For DC_41C-n77A, the TIB,c and RIB values are given in the tables below.

Table 6.142.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41C-n77A | 41 | 0.3 |
|  | n77 | 0.8 |

Table 6.142.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_41C-n77A | 41 | 0 |
|  | n77 | 0.5 |

### 6.142.5	MSD

There is no MSD requirement for this DC configuration.

## 6.143	DC_41C-n78A

### 6.143.1	Operating bands for DC

Table 6.143.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz |  |

### 6.143.2	Channel bandwidths per operating band for DC

Table 6.143.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_41C-n78A | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.143.3	Co-existence studies

Based on co-existence studies of DC_41A-n78A captured in TR 37.863-01-01 [5], there is no own Rx impact of the 3rd band for this DC configuration.

### 6.143.4	∆TIB and ∆RIB values

For DC_41C-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.143.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41C-n78A | 41 | 0.3 |
|  | n78 | 0.8 |

Table 6.143.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_41C-n78A | 41 | 0 |
|  | n78 | 0.5 |

### 6.143.5	MSD

There is no MSD issue for this DC configuration.

## 6.144	DC_41C-n257A

### 6.144.1	Operating bands for DC

Table 6.144.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

### 6.144.2	Channel bandwidths per operating band for DC

Table 6.144.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [50 MHz] | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_41C_n257 | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 440 |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |

### 6.144.3	Co-existence studies

There is no own Rx impact of the 3rd band for this DC configuration  based on co-existence studies of DC_41A-n257A captured in TR 37.863-01-01 [5].

### 6.144.4	∆TIB and ∆RIB values

For DC_41C-n257A, the TIB,c and RIB values are given in the tables below.

Table 6.144.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41C-n257A | 41 | 0 |
|  | n257 | 0 |

Table 6.144.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_41C-n257A | 41 | 0 |
|  | n257 | 0 |

### 6.144.5	MSD

There is no MSD issue for this DC configuration.

## 6.145	DC_42C-n77

### 6.145.1	Operating bands for DC

Table 6.145.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz |  |

### 6.145.2	Channel bandwidths per operating band for DC

Table 6.145.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_42C-n77A | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_42C-n77C | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 240 |
|  | n77 | See CA_n77C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.145.3	Co-existence studies

Since simultaneous transmission of LTE Band 42 and NR Band n77 are not supported at least in Rel-15 timeframe, no co-existence studies are needed at this moment.

### 6.145.4	∆TIB and ∆RIB values

For DC_42C-n77, the TIB,c and RIB values are given in the tables below.

Table 6.145.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_42C-n77 | 42 | 0 |
|  | n77 | 0 |

Table 6.145.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_42C-n77 | 42 | 0 |
|  | n77 | 0 |

### 6.145.5	MSD

There is no MSD issue for this DC configuration based on the following conditions.

-	LTE Band 42 and NR Band n77 are synchronized

-	Simultaneous transmission of LTE Band 42 and NR Band n77 are not supported at least in Rel-15 timeframe

## 6.146	DC_42C-n78

### 6.146.1	Operating bands for DC

Table 6.146.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz |  |

### 6.146.2	Channel bandwidths per operating band for DC

Table 6.146.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_42C-n78A | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_42C-n78C | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 240 |
|  | n78 | See CA_n78C in Table 5.5A.1-1 in TS 38.101-1 [3] |  |  |  |  |  |  |  |  |  |  |

### 6.146.3	Co-existence studies

Since simultaneous transmission of LTE Band 42 and NR Band n78 are not supported at least in Rel-15 timeframe, no co-existence studies are needed at this moment.

### 6.146.4	∆TIB and ∆RIB values

For DC_42C-n77, the TIB,c and RIB values are given in the tables below.

Table 6.146.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_42C-n78 | 42 | 0 |
|  | n78 | 0 |

Table 6.146.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_42C-n78 | 42 | 0 |
|  | n78 | 0 |

### 6.146.5	MSD

There is no MSD issue for this DC configuration based on the following conditions.

-	LTE Band 42 and NR Band n78 are synchronized

-	Simultaneous transmission of LTE Band 42 and NR Band n78 are not supported at least in Rel-15 timeframe

## 6.147	DC_12A-30A_n66A

### 6.147.1	Operating bands for DC

Table 6.147.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band Combination | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_12A-30A_n66A | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | 30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 6.147.2	Channel bandwidths per operating band for DC

Table 6.147.2-1 Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 101,2 MHz | 152 MHz | 202 MHz | 252 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_12A-30A_n66A | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 60 |
|  | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | n66 | 15 | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
| NOTE 1: 90% spectrum utilization may not be achieved for 30kHz SCS. NOTE 2: 90% spectrum utilization may not be achieved for 60kHz SCS. NOTE 3: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.147.3	Co-existence studies

Table 6.147.3-1 and 6.147.3-2 list up to 7th harmonic and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis for Band 12-30+Band n66 2UL DC.

Table 6.147.3-1: Band 12 and Band n66 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 |  | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1398 |  | 1432 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2097 |  | 2148 | 5130 | 5340 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high |  | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2796 | 2864 |  | 6840 | 7120 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high |  | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3495 |  | 3580 | 8550 | 8900 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high |  | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 4194 |  | 4296 | 10260 | 10680 |
| 7th harmonics frequency limits | 7*fx_low |  | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 4893 |  | 5012 | 11970 | 12460 |
| 2nd order IMD products | ¦fy_high – fx_low¦ |  | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1081 |  | 994 | 2409 | 2496 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ |  | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 382 |  | 278 | 2704 | 2861 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ |  | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3108 |  | 3212 | 4119 | 4276 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2162 |  | 1988 | 4818 | 4992 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 317 |  | 438 | 4414 | 4641 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3807 |  | 3928 | 5829 | 6056 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6421 |  | 6124 | 1154 | 1016 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7539 |  | 7836 | 4506 | 4644 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ |  | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3942 |  | 3698 | 1272 | 1463 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6528 |  | 6772 | 5517 | 5708 |

Based on Table 6.147.3-1, it can be seen that

-	2nd order harmonic may fall into Rx frequencies of bands 22, 42, 48, 49, 50, 51, 75, 76, 77, 78 and 79.

-	3rd order harmonic may fall into Rx frequencies of bands 1, 4, 10, 46 and 65 or its own RX frequency in band 66.

-	2nd order IMD products may fall into Rx frequencies of band 41.

-	3rd order IMD products may fall into Rx frequencies of band 77.

-	4th order IMD products may fall into Rx frequencies of bands 1, 2, 4, 10, 25, 34, 36, 46, 47, 65, 70, 77 and 79 or its own RX frequency in band 66.

-	5th order IMD products may fall into Rx frequencies of bands 32, 43, 45, 46, 48, 49, 50, 51, 75, 76, 77, 78 and 79.

Table 6.147.3-2: Band 30 and Band n66 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 |  | 2305 | 2315 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high |  | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 3420 |  | 3560 | 4610 | 4630 |
| 3rd harmonics frequency limits | 3* fy_low |  | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 5130 |  | 5340 | 6915 | 6945 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high |  | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7120 |  | 9220 | 9260 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high |  | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 8550 |  | 8900 | 11525 | 11575 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high |  | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 10260 |  | 10680 | 13830 | 13890 |
| 7th harmonics frequency limits | 7* fy_low |  | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 11970 |  | 12460 | 16135 | 16205 |
| 2nd order IMD products | ¦fy_high – fx_low¦ |  | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 605 |  | 525 | 4015 | 4095 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ |  | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1105 |  | 1255 | 2830 | 2920 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ |  | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5725 |  | 5875 | 6320 | 6410 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1210 |  | 1050 | 8030 | 8190 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2815 |  | 3035 | 5135 | 5235 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7435 |  | 7655 | 8625 | 8725 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7550 |  | 7440 | 4815 | 4525 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10930 |  | 11040 | 9145 | 9435 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ |  | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3525 |  | 3355 | 730 | 500 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10335 |  | 10505 | 9740 | 9970 |

Based on Table 6.147.3-1, it can be seen that

-	2nd order harmonic may fall into Rx frequencies of bands 22, 42, 48, 49, 77, 78 or 79.

-	3rd order harmonic may fall into Rx frequencies of band 46.

-	2nd order IMD products may fall into Rx frequencies of band 77

-	3rd order IMD products may fall into Rx frequencies of bands 46 or 47

-	4th order IMD products may fall into Rx frequencies of band 46.

-	5th order IMD products may fall into Rx frequencies of bands 12, 22, 29, 42, 44, 71, 77, 78 or 79.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.147.3-3 and Table 6.147.3-4 list up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.147.3-3: Band 12 and Band n66 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | 2nd IMD |
|  | 2400 | - | 2494 | Yes | Asia | 2nd IMD |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 4th, 5th IMD |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | Yes |  | 5th IMD |
|  | 5150 | - | 5825 | Yes | Asia | 5th IMD |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.147.3-4: Band 30 and Band n66 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 3rd, 4th IMD 3rd harmonic |
|  | 5150 | - | 5350 | Yes | Europe | 4th IMD 3rd harmonic |
|  | 5470 | - | 5725 | Yes |  | 3rd IMD |
|  | 5150 | - | 5825 | Yes | Asia | 3rd, 4th IMD 3rd harmonic |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

The protected bands for DC_12A-30A_n66A are covered in the constituent fall-back modes.

### 6.147.4	∆TIB and ∆RIB values

For DC_12A-30A_n66A, the TIB,c and RIB,c values are derived from LTE combination CA_12-30-66 and are given in the tables below.

Table 6.147.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12A-30A_n66A | 2 | 0.8 |
|  | 12 | 0.3 |
|  | n66 | 0.5 |

Table 6.147.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_12A_30A_n66A | 2 | 0.5 |
|  | 12 | 0.5 |
|  | n66 | 0.4 |

### 6.147.5	REFSENs requirements

Table 6.147.5-1 lists the MSD required for the dual connectivity configuration for the cases where IMD interference falls into the own 3rd Rx frequency band.

Table 6.147.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_12A-30A_n66A | 12 | TBD | TBD | TBD | TBD | TBD | FDD | IMD5 ¦2*fB30-3*fB66¦ |
|  | 30 | TBD | TBD | TBD | TBD | TBD |  | N/A |
|  | n66 | TBD | TBD | TBD | TBD | TBD |  | N/A |

## 6.148	DC_2A-12A_n66A

### 6.148.1	Operating bands for DC

Table 6.148.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band Combination | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2A-12A_n66A | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 6.148.2	Channel bandwidths per operating band for DC

Table 6.148.2-1 Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 101,2 MHz | 152 MHz | 202 MHz | 252 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_2A-12A_n66A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 70 |
|  | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | n66 | 15 | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
| NOTE 1: 90% spectrum utilization may not be achieved for 30kHz SCS. NOTE 2: 90% spectrum utilization may not be achieved for 60kHz SCS. NOTE 3: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.148.3	Co-existence studies

Table 6.148.3-1 and 6.148.3-2 list up to 7th harmonic and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis for Band 2-12+Band n66 2UL DC.

Table 6.148.3-1: Band 2 and Band n66 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 |  | 1850 | 1910 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high |  | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 3420 |  | 3560 | 3700 | 3820 |
| 3rd harmonics frequency limits | 3* fy_low |  | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 5130 |  | 5340 | 5550 | 5730 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high |  | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7120 |  | 7400 | 7640 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high |  | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 8550 |  | 8900 | 9250 | 9550 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high |  | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 10260 |  | 10680 | 11100 | 11460 |
| 7th harmonics frequency limits | 7* fy_low |  | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 11970 |  | 12460 | 12950 | 13370 |
| 2nd order IMD products | ¦fy_high – fx_low¦ |  | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 200 |  | 70 | 3560 | 3690 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ |  | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1510 |  | 1710 | 1920 | 2110 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ |  | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5270 |  | 5470 | 5410 | 5600 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 400 |  | 140 | 7120 | 7380 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3220 |  | 3490 | 3770 | 4020 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6980 |  | 7250 | 7260 | 7510 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5930 |  | 5620 | 5270 | 4930 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9110 |  | 9420 | 8690 | 9030 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ |  | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2310 |  | 1990 | 1640 | 1310 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 |  | 9290 | 8830 | 9160 |

Based on Table 6.148.3-1, it can be seen that

-	2nd order harmonic may fall into Rx frequencies of bands 22, 42, 43, 48, 49, 77 and 78.

-	3rd order harmonic may fall into Rx frequencies of band 46.

-	2nd order IMD products may fall into Rx frequencies of bands 22, 42, 43, 48, 49, 77 and 78.

-	3rd order IMD products may fall into Rx frequencies of bands 1, 4, 10, 21, 24, 25, 33, 34, 36, 37, 39, 46, 50, 65, 70, 74, 75, or its own Rx frequencies of band 2.

-	4th order IMD products may fall into Rx frequencies of bands 42, 43, 77 and 78.

-	5th order IMD products may fall into Rx frequencies of bands 1, 4, 10, 11, 21, 23, 24, 25, 32, 34, 36, 40, 45, 46, 47, 50, 51, 65, 70, 74, 75, 76, 79, or its own Rx frequencies of band 66.

Table 6.148.3-2: Band 12 and Band n66 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 |  | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1398 |  | 1432 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2097 |  | 2148 | 5130 | 5340 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high |  | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2796 | 2864 |  | 6840 | 7120 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high |  | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3495 |  | 3580 | 8550 | 8900 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high |  | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 4194 |  | 4296 | 10260 | 10680 |
| 7th harmonics frequency limits | 7*fx_low |  | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 4893 |  | 5012 | 11970 | 12460 |
| 2nd order IMD products | ¦fy_high – fx_low¦ |  | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1081 |  | 994 | 2409 | 2496 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ |  | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 382 |  | 278 | 2704 | 2861 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ |  | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3108 |  | 3212 | 4119 | 4276 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2162 |  | 1988 | 4818 | 4992 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 317 |  | 438 | 4414 | 4641 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3807 |  | 3928 | 5829 | 6056 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6421 |  | 6124 | 1154 | 1016 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7539 |  | 7836 | 4506 | 4644 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ |  | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3942 |  | 3698 | 1272 | 1463 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6528 |  | 6772 | 5517 | 5708 |

Based on Table 6.148.3-2, it can be seen that

-	2nd order harmonic may fall into Rx frequencies of bands 22, 42, 48, 49, 50, 51, 75, 76, 77, 78 and 79.

-	3rd order harmonic may fall into Rx frequencies of bands 46 and 65 or its own RX frequency in band 66.

-	2nd order IMD products may fall into Rx frequencies of band 41.

-	3rd order IMD products may fall into Rx frequencies of band 77.

-	4th order IMD products may fall into Rx frequencies of bands 1, 2, 4, 10, 25, 34, 36, 46, 47, 65, 70, 77 and 79 or its own RX frequency in band 66.

-	5th order IMD products may fall into Rx frequencies of bands 32, 43, 45, 46, 48, 49, 50, 51, 75, 76, 77, 78 and 79.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.148.3-3 and Table 6.148.3-4 list 48up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.148.3-3: Band 2 and Band n66 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | 3rd, 5th IMD |
| Galileo | 1559 | - | 1591 | Yes |  | 3rd, 5th IMD |
| GLONASS | 1591 | - | 1610 | Yes |  | 3rd, 5th IMD |
| GPS | 1563 | - | 1587 | Yes |  | 3rd, 5th IMD |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 3rd, 5th IMD 3rd harmonic |
|  | 5150 | - | 5350 | Yes | Europe | 3rd, 5th IMD 3rd harmonic |
|  | 5470 | - | 5725 | Yes |  | 3rd, 5th IMD 3rd harmonic |
|  | 5150 | - | 5825 | Yes | Asia | 3rd, 5th IMD 3rd harmonic |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.148.3-4: Band 12 and Band n66 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | 2nd IMD |
|  | 2400 | - | 2494 | Yes | Asia | 2nd IMD |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 4th, 5th IMD |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | Yes |  | 5th IMD |
|  | 5150 | - | 5825 | Yes | Asia | 5th IMD |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

The protected bands for DC_2A-12A_n66A are covered in the constituent fall-back modes.

### 6.148.4	∆TIB and ∆RIB values

For DC_2A-12A_n66A, the TIB,c and RIB,c values are derived from LTE combination CA_2-12-66 and are given in the tables below.

Table 6.148.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2A-12A_n66A | 2 | 0.5 |
|  | 12 | 0.8 |
|  | n66 | 0.5 |

Table 6.148.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2A_12A_n66A | 2 | 0.3 |
|  | 12 | 0.5 |
|  | n66 | 0.3 |

### 6.148.5	REFSENs requirements

Table 6.148.5-1 lists the MSD required for the dual connectivity configuration for the cases where IMD interference falls into the own 3rd Rx frequency band.

Table 6.148.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_2A-12A_n66A | 2 | TBD | TBD | TBD | TBD | TBD | FDD | IMD4 ¦2*fB12-2*fB66¦ |
|  | 12 | TBD | TBD | TBD | TBD | TBD |  | N/A |
|  | n66 | TBD | TBD | TBD | TBD | TBD |  | N/A |

## 6.149		DC_1A-8A_n257A

### 6.149.1	Operating bands for DC

Table 6.149.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-8A-n257A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915MHz | 925 MHz | – | 960 MHz |  |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.149.2	Channel bandwidths per operating band for DC

Table 6.149.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-8A-n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 430 |
|  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |

### 6.149.3	Co-existence studies

The relevant UE-to-UE co-existence studies have already addressed in constituent DC LTE 1DL/1UL + one NR band combination and no new study is needed for this LTE 2DL/1UL + one NR band.

### 6.149.4	∆TIB and ∆RIB values

For DC_1A-8A-n257A, the TIB,c and RIB values are given in the tables below.

Table 6.149.4-2: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-8A-n257A | 1 | 0.3 |
|  | 8 | 0.3 |
|  | n257 | 0 |

Table 6.149.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-8A-n257A | 1 | 0 |
|  | 8 | 0 |
|  | n257 | 0 |

### 6.149.5	MSD

Since 2UL configurations required for this combination are FR1+FR2, there is no need to address MSD.

## 6.150	DC_3A-41A_n257A

### 6.150.1	Operating bands for DC

Table 6.150.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-41A-n257A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

### 6.150.2	Channel bandwidths per operating band for DC

Table 6.150.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-41A-n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | 440 |
|  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  | Yes | Yes | Yes | Yes |  |

### 6.150.3	Co-existence studies

The relevant UE-to-UE co-existence studies have already addressed in constituent DC LTE 1DL/1UL + one NR band combination and no new study is needed for this LTE 2DL/1UL + one NR band.

### 6.150.4	∆TIB and ∆RIB values

For DC_3A-41A-n257A, the TIB,c and RIB values are given in the tables below.

Table 6.150.4-2: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-41A-n257A | 3 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | n257 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 6.150.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-41A-n257A | 3 | 0 |
|  | 41 | 01/0.52 |
|  | n257 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

### 6.150.5	MSD

Since 2UL configurations required for this combination are FR1+FR2, there is no need to address MSD.

## 6.151	DC_41C_n78A

### 6.151.1	Operating bands for DC

Table 6.151.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_41C-n78A | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz |  |

### 6.151.2	Channel bandwidths per operating band for DC

Table 6.151.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_41C-n78A | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.151.3	Co-existence studies

The relevant UE-to-UE co-existence studies have already addressed in constituent DC LTE 1DL/1UL + one NR band combination and no new study is needed for this LTE 2DL/1UL + one NR band.

### 6.151.4	∆TIB and ∆RIB values

The same TIB,c and RIB values as of DC_41A-n78A can be reused to this combination since no architectural change in RF front-end is required.

### 6.151.5	MSD

The same MSD scenarios and values as of DC_41A-n78A can be reused to this combination since there is no new band to be added.

## 6.152	DC_5A-30A_n66A

### 6.152.1	Operating bands for DC

Table 6.152.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band Combination | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5A-30A_n66A | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 6.152.2	Channel bandwidths per operating band for DC

Table 6.152.2-1 Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 101,2 MHz | 152 MHz | 202 MHz | 252 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_5A-30A_n66A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 60 |
|  | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | n66 | 15 | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
| NOTE 1: 90% spectrum utilization may not be achieved for 30kHz SCS. NOTE 2: 90% spectrum utilization may not be achieved for 60kHz SCS. NOTE 3: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.152.3	Co-existence studies

Table 6.152.3-1 and 6.152.3-2 list up to 7th harmonic and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis for Band 5-30+Band n66 2UL DC.

Table 6.152.3-1: Band 5 and Band n66 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 |  | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 |  | 1698 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 |  | 2547 | 5130 | 5340 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high |  | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 |  | 6840 | 7120 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high |  | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4120 |  | 4245 | 8550 | 8900 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high |  | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 4944 |  | 5094 | 10260 | 10680 |
| 7th harmonics frequency limits | 7*fx_low |  | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 5768 |  | 5943 | 11970 | 12460 |
| 2nd order IMD products | ¦fy_high – fx_low¦ |  | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 956 |  | 861 | 2534 | 2629 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ |  | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 132 |  | 12 | 2571 | 2736 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ |  | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3358 |  | 3478 | 4244 | 4409 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1912 |  | 1722 | 5068 | 5258 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 692 |  | 837 | 4281 | 4516 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4182 |  | 4327 | 5954 | 6189 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6296 |  | 5991 | 1686 | 1516 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7664 |  | 7969 | 5006 | 5176 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ |  | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3692 |  | 3432 | 873 | 1088 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6778 |  | 7038 | 5892 | 6107 |

Based on Table 6.152.3-1, it can be seen that

-	2nd order harmonic may fall into Rx frequencies of bands 22, 42, 48, 49, 77 and 78.

-	3rd order harmonic may fall into Rx frequencies of bands 41 and 46.

-	2nd order IMD products may fall into Rx frequencies of bands 6, 7, 8, 18, 19, 26, 27, 38, 41 and 69, or its own RX frequency in band 5.

-	3rd order IMD products may fall into Rx frequencies of bands 7, 38, 41, 42, 69, 77, 78 and 79.

-	4th order IMD products may fall into Rx frequencies of bands 3, 9, 12, 13, 17, 20, 28, 29, 33, 35, 37, 39, 44, 46, 67, 68, 77 and 79.

-	5th order IMD products may fall into Rx frequencies of bands 6, 8, 18, 19, 22, 24, 26, 42, 43, 46, 47, 48, 49, 50, 74, 75, 77 and 78, or its own RX frequency in band 5.

Table 6.152.3-2: Band 30 and Band n66 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 |  | 2305 | 2315 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high |  | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 3420 |  | 3560 | 4610 | 4630 |
| 3rd harmonics frequency limits | 3* fy_low |  | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 5130 |  | 5340 | 6915 | 6945 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high |  | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7120 |  | 9220 | 9260 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high |  | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 8550 |  | 8900 | 11525 | 11575 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high |  | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 10260 |  | 10680 | 13830 | 13890 |
| 7th harmonics frequency limits | 7* fy_low |  | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 11970 |  | 12460 | 16135 | 16205 |
| 2nd order IMD products | ¦fy_high – fx_low¦ |  | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 605 |  | 525 | 4015 | 4095 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ |  | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1105 |  | 1255 | 2830 | 2920 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ |  | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5725 |  | 5875 | 6320 | 6410 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1210 |  | 1050 | 8030 | 8190 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2815 |  | 3035 | 5135 | 5235 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7435 |  | 7655 | 8625 | 8725 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7550 |  | 7440 | 4815 | 4525 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10930 |  | 11040 | 9145 | 9435 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ |  | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3525 |  | 3355 | 730 | 500 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10335 |  | 10505 | 9740 | 9970 |

Based on Table 6.152.3-1, it can be seen that

-	2nd order harmonic may fall into Rx frequencies of bands 22, 42, 48, 49, 77, 78 or 79.

-	3rd order harmonic may fall into Rx frequencies of band 46.

-	2nd order IMD products may fall into Rx frequencies of band 77

-	3rd order IMD products may fall into Rx frequencies of bands 46 or 47

-	4th order IMD products may fall into Rx frequencies of band 46.

-	5th order IMD products may fall into Rx frequencies of bands 12, 22, 29, 42, 44, 71, 77, 78 or 79.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.152.3-3 and Table 6.152.3-4 list up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.152.3-3: Band 5 and Band n66 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | 5th IMD |
| Galileo | 1559 | - | 1591 | Yes |  | 5th IMD |
| GLONASS | 1591 | - | 1610 | Yes |  | 5th IMD |
| GPS | 1563 | - | 1587 | Yes |  | 5th IMD |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | 3rd harmonic |
|  | 2400 | - | 2494 | Yes | Asia | 3rd harmonic |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 4th, 5th IMD 3rd harmonic |
|  | 5150 | - | 5350 | Yes | Europe | 4th, 5th IMD 3rd harmonic |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | 4th, 5th IMD 3rd harmonic |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.152.3-4: Band 30 and Band n66 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 3rd, 4th IMD 3rd harmonic |
|  | 5150 | - | 5350 | Yes | Europe | 4th IMD 3rd harmonic |
|  | 5470 | - | 5725 | Yes |  | 3rd IMD |
|  | 5150 | - | 5825 | Yes | Asia | 3rd, 4th IMD 3rd harmonic |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

The protected bands for DC_2A-30A_n66A are covered in the constituent fall-back modes.

### 6.152.4	∆TIB and ∆RIB values

For DC_5A-30A_n66A, the TIB,c and RIB,c values are derived from LTE combination CA_5-30-66 and are given in the tables below.

Table 6.152.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5A-30A_n66A | 5 | 0.3 |
|  | 30 | 0.3 |
|  | n66 | 0.5 |

Table 6.152.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_5A_30A_n66A | 2 | 0 |
|  | 30 | 0.5 |
|  | n66 | 0.4 |

### 6.152.5	REFSENs requirements

There is no MSD required for the dual connectivity configuration for the cases where IMD interference falls into the own 3rd Rx frequency band.

## 6.153	DC_2A-30A_n66A

### 6.153.1	Operating bands for DC

Table 6.153.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band Combination | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2A-30A_n66A | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 6.153.2	Channel bandwidths per operating band for DC

Table 6.153.2-1 Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 101,2 MHz | 152 MHz | 202 MHz | 252 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_2A-30A_n66A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 70 |
|  | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | n66 | 15 | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
| NOTE 1: 90% spectrum utilization may not be achieved for 30kHz SCS. NOTE 2: 90% spectrum utilization may not be achieved for 60kHz SCS. NOTE 3: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.153.3	Co-existence studies

Table 6.153.3-1 and 6.153.3-2 list up to 7th harmonic and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis for Band 2-30+Band n66 2UL DC.

Table 6.153.3-1: Band 2 and Band n66 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 |  | 1850 | 1910 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high |  | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 3420 |  | 3560 | 3700 | 3820 |
| 3rd harmonics frequency limits | 3* fy_low |  | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 5130 |  | 5340 | 5550 | 5730 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high |  | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7120 |  | 7400 | 7640 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high |  | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 8550 |  | 8900 | 9250 | 9550 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high |  | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 10260 |  | 10680 | 11100 | 11460 |
| 7th harmonics frequency limits | 7* fy_low |  | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 11970 |  | 12460 | 12950 | 13370 |
| 2nd order IMD products | ¦fy_high – fx_low¦ |  | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 200 |  | 70 | 3560 | 3690 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ |  | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1510 |  | 1710 | 1920 | 2110 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ |  | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5270 |  | 5470 | 5410 | 5600 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 400 |  | 140 | 7120 | 7380 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3220 |  | 3490 | 3770 | 4020 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6980 |  | 7250 | 7260 | 7510 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5930 |  | 5620 | 5270 | 4930 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9110 |  | 9420 | 8690 | 9030 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ |  | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2310 |  | 1990 | 1640 | 1310 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 |  | 9290 | 8830 | 9160 |

Based on Table 6.153.3-1, it can be seen that

-	2nd order harmonic may fall into Rx frequencies of bands 22, 42, 43, 48, 49, 77 and 78.

-	3rd order harmonic may fall into Rx frequencies of band 46.

-	2nd order IMD products may fall into Rx frequencies of bands 22, 42, 43, 48, 49, 77 and 78.

-	3rd order IMD products may fall into Rx frequencies of bands 1, 4, 10, 21, 24, 25, 33, 34, 36, 37, 39, 46, 50, 65, 70, 74, 75, or its own Rx frequencies of band 2.

-	4th order IMD products may fall into Rx frequencies of bands 42, 43, 77 and 78.

-	5th order IMD products may fall into Rx frequencies of bands 1, 4, 10, 11, 21, 23, 24, 25, 32, 34, 36, 40, 45, 46, 47, 50, 51, 65, 70, 74, 75, 76, 79, or its own Rx frequencies of band 66.

Table 6.153.3-2: Band 30 and Band n66 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 |  | 2305 | 2315 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high |  | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 3420 |  | 3560 | 4610 | 4630 |
| 3rd harmonics frequency limits | 3* fy_low |  | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 5130 |  | 5340 | 6915 | 6945 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high |  | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7120 |  | 9220 | 9260 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high |  | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 8550 |  | 8900 | 11525 | 11575 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high |  | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 10260 |  | 10680 | 13830 | 13890 |
| 7th harmonics frequency limits | 7* fy_low |  | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 11970 |  | 12460 | 16135 | 16205 |
| 2nd order IMD products | ¦fy_high – fx_low¦ |  | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 605 |  | 525 | 4015 | 4095 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ |  | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1105 |  | 1255 | 2830 | 2920 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ |  | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5725 |  | 5875 | 6320 | 6410 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1210 |  | 1050 | 8030 | 8190 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2815 |  | 3035 | 5135 | 5235 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7435 |  | 7655 | 8625 | 8725 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7550 |  | 7440 | 4815 | 4525 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10930 |  | 11040 | 9145 | 9435 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ |  | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3525 |  | 3355 | 730 | 500 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10335 |  | 10505 | 9740 | 9970 |

Based on Table 6.153.3-1, it can be seen that

-	2nd order harmonic may fall into Rx frequencies of bands 22, 42, 48, 49, 77, 78 or 79.

-	3rd order harmonic may fall into Rx frequencies of band 46.

-	2nd order IMD products may fall into Rx frequencies of band 77

-	3rd order IMD products may fall into Rx frequencies of bands 46 or 47

-	4th order IMD products may fall into Rx frequencies of band 46.

-	5th order IMD products may fall into Rx frequencies of bands 12, 22, 29, 42, 44, 71, 77, 78 or 79.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.153.3-3 and Table 6.153.3-4 list up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.153.3-3: Band 2 and Band n66 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | 3rd, 5th IMD |
| Galileo | 1559 | - | 1591 | Yes |  | 3rd, 5th IMD |
| GLONASS | 1591 | - | 1610 | Yes |  | 3rd, 5th IMD |
| GPS | 1563 | - | 1587 | Yes |  | 3rd, 5th IMD |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 3rd, 5th IMD 3rd harmonic |
|  | 5150 | - | 5350 | Yes | Europe | 3rd, 5th IMD 3rd harmonic |
|  | 5470 | - | 5725 | Yes |  | 3rd, 5th IMD 3rd harmonic |
|  | 5150 | - | 5825 | Yes | Asia | 3rd, 5th IMD 3rd harmonic |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.153.3-4: Band 30 and Band n66 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 3rd, 4th IMD 3rd harmonic |
|  | 5150 | - | 5350 | Yes | Europe | 4th IMD 3rd harmonic |
|  | 5470 | - | 5725 | Yes |  | 3rd IMD |
|  | 5150 | - | 5825 | Yes | Asia | 3rd, 4th IMD 3rd harmonic |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

The protected bands for DC_2A-30A_n66A are covered in the constituent fall-back modes.

### 6.153.4	∆TIB and ∆RIB values

For DC_2A-30A_n66A, the TIB,c and RIB,c values are derived from LTE combination CA_2-30-66 and are given in the tables below.

Table 6.153.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2A-30A_n66A | 2 | 0.5 |
|  | 30 | 0.3 |
|  | n66 | 0.5 |

Table 6.153.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2A_30A_n66A | 2 | 0.4 |
|  | 30 | 0.5 |
|  | n66 | 0.4 |

### 6.153.5	REFSENs requirements

There is no MSD required for the dual connectivity configuration for the cases where IMD interference falls into the own 3rd Rx frequency band.

## 6.154	DC_2A-5A_n66A

### 6.154.1	Operating bands for DC

Table 6.154.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band Combination | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2A-5A_n66A | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 6.154.2	Channel bandwidths per operating band for DC

Table 6.154.2-1 Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 101,2 MHz | 152 MHz | 202 MHz | 252 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_2A-5A_n66A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 70 |
|  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | n66 | 15 | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |
| NOTE 1: 90% spectrum utilization may not be achieved for 30kHz SCS. NOTE 2: 90% spectrum utilization may not be achieved for 60kHz SCS. NOTE 3: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.154.3	Co-existence studies

Table 6.154.3-1 and 6.154.3-2 list up to 7th harmonic and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis for Band 2-5+Band n66 2UL DC.

Table 6.154.3-1: Band 2 and Band n66 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 |  | 1850 | 1910 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high |  | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 3420 |  | 3560 | 3700 | 3820 |
| 3rd harmonics frequency limits | 3* fy_low |  | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 5130 |  | 5340 | 5550 | 5730 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high |  | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7120 |  | 7400 | 7640 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high |  | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 8550 |  | 8900 | 9250 | 9550 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high |  | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 10260 |  | 10680 | 11100 | 11460 |
| 7th harmonics frequency limits | 7* fy_low |  | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 11970 |  | 12460 | 12950 | 13370 |
| 2nd order IMD products | ¦fy_high – fx_low¦ |  | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 200 |  | 70 | 3560 | 3690 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ |  | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1510 |  | 1710 | 1920 | 2110 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ |  | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5270 |  | 5470 | 5410 | 5600 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 400 |  | 140 | 7120 | 7380 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3220 |  | 3490 | 3770 | 4020 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6980 |  | 7250 | 7260 | 7510 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5930 |  | 5620 | 5270 | 4930 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9110 |  | 9420 | 8690 | 9030 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ |  | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2310 |  | 1990 | 1640 | 1310 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 |  | 9290 | 8830 | 9160 |

Based on Table 6.154.3-1, it can be seen that

-	2nd order harmonic may fall into Rx frequencies of bands 22, 42, 43, 48, 49, 77 and 78.

-	3rd order harmonic may fall into Rx frequencies of band 46.

-	2nd order IMD products may fall into Rx frequencies of bands 22, 42, 43, 48, 49, 77 and 78.

-	3rd order IMD products may fall into Rx frequencies of bands 1, 4, 10, 21, 24, 25, 33, 34, 36, 37, 39, 46, 50, 65, 70, 74, 75, or its own Rx frequencies of band 2.

-	4th order IMD products may fall into Rx frequencies of bands 42, 43, 77 and 78.

-	5th order IMD products may fall into Rx frequencies of bands 1, 4, 10, 11, 21, 23, 24, 25, 32, 34, 36, 40, 45, 46, 47, 50, 51, 65, 70, 74, 75, 76, 79, or its own Rx frequencies of band 66.

Table 6.154.3-2: Band 5 and Band n66 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 |  | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 |  | 1698 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 |  | 2547 | 5130 | 5340 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high |  | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 |  | 6840 | 7120 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high |  | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4120 |  | 4245 | 8550 | 8900 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high |  | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 4944 |  | 5094 | 10260 | 10680 |
| 7th harmonics frequency limits | 7*fx_low |  | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 5768 |  | 5943 | 11970 | 12460 |
| 2nd order IMD products | ¦fy_high – fx_low¦ |  | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 956 |  | 861 | 2534 | 2629 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ |  | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 132 |  | 12 | 2571 | 2736 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ |  | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3358 |  | 3478 | 4244 | 4409 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1912 |  | 1722 | 5068 | 5258 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 692 |  | 837 | 4281 | 4516 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4182 |  | 4327 | 5954 | 6189 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6296 |  | 5991 | 1686 | 1516 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7664 |  | 7969 | 5006 | 5176 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ |  | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3692 |  | 3432 | 873 | 1088 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6778 |  | 7038 | 5892 | 6107 |

Based on Table 6.154.3-1, it can be seen that

-	2nd order harmonic may fall into Rx frequencies of bands 22, 42, 48, 49, 77 and 78.

-	3rd order harmonic may fall into Rx frequencies of bands 41 and 46.

-	2nd order IMD products may fall into Rx frequencies of bands 6, 7, 8, 18, 19, 26, 27, 38, 41 and 69, or its own RX frequency in band 5.

-	3rd order IMD products may fall into Rx frequencies of bands 7, 38, 41, 42, 69, 77, 78 and 79.

-	4th order IMD products may fall into Rx frequencies of bands 3, 9, 12, 13, 17, 20, 28, 29, 33, 35, 37, 39, 44, 46, 67, 68, 77 and 79.

-	5th order IMD products may fall into Rx frequencies of bands 6, 8, 18, 19, 22, 24, 26, 42, 43, 46, 47, 48, 49, 50, 74, 75, 77 and 78, or its own RX frequency in band 5.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.154.3-3 and Table 6.154.3-4 list up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.154.3-3: Band 2 and Band n66 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | 3rd, 5th IMD |
| Galileo | 1559 | - | 1591 | Yes |  | 3rd, 5th IMD |
| GLONASS | 1591 | - | 1610 | Yes |  | 3rd, 5th IMD |
| GPS | 1563 | - | 1587 | Yes |  | 3rd, 5th IMD |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 3rd, 5th IMD 3rd harmonic |
|  | 5150 | - | 5350 | Yes | Europe | 3rd, 5th IMD 3rd harmonic |
|  | 5470 | - | 5725 | Yes |  | 3rd, 5th IMD 3rd harmonic |
|  | 5150 | - | 5825 | Yes | Asia | 3rd, 5th IMD 3rd harmonic |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.154.3-4: Band 5 and Band n66 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | 5th IMD |
| Galileo | 1559 | - | 1591 | Yes |  | 5th IMD |
| GLONASS | 1591 | - | 1610 | Yes |  | 5th IMD |
| GPS | 1563 | - | 1587 | Yes |  | 5th IMD |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | 3rd harmonic |
|  | 2400 | - | 2494 | Yes | Asia | 3rd harmonic |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 4th, 5th IMD 3rd harmonic |
|  | 5150 | - | 5350 | Yes | Europe | 4th, 5th IMD 3rd harmonic |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | 4th, 5th IMD 3rd harmonic |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

The protected bands for DC_2A-5A_n66A are covered in the constituent fall-back modes.

### 6.154.4	∆TIB and ∆RIB values

For DC_2A-5A_n66A, the TIB,c and RIB,c values are derived from LTE combination CA_2-5-66 and are given in the tables below.

Table 6.154.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2A-5A_n66A | 2 | 0.5 |
|  | 5 | 0.3 |
|  | n66 | 0.5 |

Table 6.154.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2A_5A_n66A | 2 | 0.3 |
|  | 5 | 0 |
|  | n66 | 0.3 |

### 6.154.5	REFSENs requirements

There is no MSD required for the dual connectivity configuration for the cases where IMD interference falls into the own 3rd Rx frequency band.

## 6.155	DC_1A-41A-n78A

### 6.155.1	Operating bands for DC

Table 6.155.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-41A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Note 1: Currently, for LTE UL carrier is only supported on Band 1 not Band 41 because the fall back mode 1UL/2DL LTE CA_1A-41A has the limitation that UL carrier is only supported on Band 1. This will be the case until RAN4 draw a conclusion to support B41 as Pcell for LTE CA_1A-41A.

### 6.155.2	Channel bandwidths per operating band for DC

Table 6. 155.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-41A-n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 140 |
|  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.155.3	Co-existence studies

Note that currently for LTE UL carrier is only supported on Band 1 not Band 41 because the fall back mode LTE 1UL/2DL CA_1A-41A has the limitation that UL carrier is only supported on Band 1. This will be the case until RAN4 draw a conclusion to support B41 as Pcell for CA_1A-41A. So the Band 41 UL harmonics, Band 41 and Band n78 UL IMD are not issues for EN-DC 1A-41A-n78A for now.

Tables 6.155.3-1 list Band 1A + Band n78A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis. For the detailed harmonics and IMD frequency information of the corresponding combinations one can refer to TR37.863-01-01 [5].

Table 6. 155.3-1: Band 1 and Band n78 UL harmonics and IMD products

| NR DC Configuration | E-UTRA/NR band | DL frequency range (MHz) | Rx Impact | Harmonic/IMD orders | Notes |
| --- | --- | --- | --- | --- | --- |
| DC_1A-n78A | 1 | 2110 MHz – 2170 MHz | Yes | IMD4 |  |
|  | n78 | 3300 MHz – 3800 MHz | No | IMD4 | IMD will not be an issue for TDD bands |

For the additional 3rd receiver Band 41, there is no MSD issue for this DC configuration based on the following condition.

-	LTE Band 41 and NR Band n78 are synchronized.

### 6.155.4	∆TIB and ∆RIB values

For DC_1A-41A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6. 155.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-41A-n78A | 1 | [0.5] |
|  | 41 | [0.5] |
|  | n78 | 0.8 |

Table 6. 155.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-41A-n78A | 1 | [0] |
|  | 41 | [0] |
|  | n78 | 0.5 |

### 6.155.5	MSD

As mentioned in 6.155.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 6.156	DC_30A-66A_n260A

### 6.156.1	Operating bands for DC

Table 6.156.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_30A-66A_n260A | 30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

### 6.156.2	Channel bandwidths per operating band for DC

Table 6.156.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_30A-66A_n260A | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 230 |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | n260 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 430 |
| DC_30A-66A_n260G | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 230 |
|  | 60 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260G Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_30A-66A_n260H | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 330 |
|  | 60 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260H Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_30A-66A_n260I | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 430 |
|  | 60 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260I Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_30A-66A_n260J | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 530 |
|  | 60 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260J Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_30A-66A_n260K | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 630 |
|  | 60 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260K Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_30A-66A_n260L | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 730 |
|  | 60 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260L Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_30A-66A_n260M | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 830 |
|  | 60 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260M Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |

### 6.156.3	Co-existence studies

Table 6.156.3-1 lists Band 30A-66A +Band n260A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.156.3-1: Band 30 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2305 | 2315 | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4610 | 4630 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 6915 | 6945 | 111000 | 120000 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 34685 | 37695 | 39305 | 42315 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 35390 | 32370 | 71685 | 77695 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 41610 | 44630 | 76305 | 82315 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 33085 | 30055 | 108685 | 117695 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 43915 | 46945 | 113305 | 122315 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 75390 | 69370 | 78610 | 84630 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 157695 | 145685 | 27740 | 30780 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 150305 | 162315 | 46220 | 49260 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 115390 | 106370 | 67055 | 73085 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 115610 | 124630 | 80915 | 86945 |

Based on Table 6.156.3-1

-	2rd order harmonic may fall into frequencies of band n79

-	2nd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into own Rx frequencies of band n260

Table 6.156.3-2: Band 66 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 111000 | 120000 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 38290 | 35220 | 38710 | 41780 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 36580 | 33440 | 72220 | 78290 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 40420 | 43560 | 75710 | 81780 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 1310 | 2180 | 36980 | 40020 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 76580 | 70440 | 77420 | 83560 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 34870 | 31660 | 109220 | 118290 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 42130 | 45340 | 112710 | 121780 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 158290 | 146220 | 29880 | 33160 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 149710 | 161780 | 43840 | 47120 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 116580 | 107440 | 68660 | 74870 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 114420 | 123560 | 79130 | 85340 |

Based on Table 6.156.3-2

-	2nd order harmonics may fall into LTE frequencies of Band 22, 42, 48

-	3rd order harmonic may fall into LTE frequencies of Band 46

-	2nd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into Rx frequencies of Band 1, 2, 3, 4, 9, 10, 11, 21, 23, 24, 25, 32, 33, 34, 35, 36, 37, 39, 45, 50, 51, 65, 70, 74, 75, and 76

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.156.3-3 and Table 6.156.3-4 list if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.156.3-3: 2UL Band 30 + Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.156.3-4: 2UL Band 66 + Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | IMD3 |
| Galileo | 1559 | - | 1591 | YES |  | IMD3 |
| GLONASS | 1591 | - | 1610 | YES |  | IMD3 |
| GPS | 1563 | - | 1587 | YES |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | 3rd harmonics |
|  | 5150 | - | 5350 | YES | Europe | 3rd harmonics |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | YES | Asia | 3rd harmonics |

Table 6.156.3-5 and Table 6.156.3-6 list the protected bands required for the dual connectivity configuration.

Table 6.156.3-5: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_30A_n260A | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 38, 41, 48, 66, 70, 71, 85, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
| Note : Table 6.5.3.2-1 of TS38.101 |  |  |  |  |  |  |  |

Table 6.156.3-6: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_66A_n260A | Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 42, 48 | FDL_low | - | FDL_high | -50 | 1 | 1* |
| Note : Table 6.5.3.2-1 of TS38.101 |  |  |  |  |  |  |  |

### 6.156.4	∆TIB and ∆RIB values

For DC_30A-66A_n260A, the TIB,c and RIB values are given in the tables below.

Table 6.156.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_30A-66A_n260A | 30 | 0.5 |
|  | 66 | 0.5 |
|  | n260 | 0 |

Table 6.156.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_30A-66A_n260A | 30 | 0 |
|  | 66 | 0 |
|  | n260 | 0 |

### 6.156.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.157	DC_12A-66A_n260A

### 6.157.1	Operating bands for DC

Table 6.157.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_12A-66A_n260A | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

### 6.157.2	Channel bandwidths per operating band for DC

Table 6.157.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_12A-66A_n260A | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 230 |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | n260 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 430 |
| DC_12A-66A_n260G | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 230 |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260G Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_12A-66A_n260H | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 330 |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260H Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_12A-66A_n260I | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 430 |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260I Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_12A-66A_n260J | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 530 |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260J Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_12A-66A_n260K | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 630 |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260K Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_12A-66A_n260L | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 730 |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260L Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_12A-66A_n260M | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 830 |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260M Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.157.3	Co-existence studies

Table 6.157.3-1 and Table 6.157.3-2  lists Band 12A-66A + Band n260A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.157.3-1: Band 12 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1398 | 1432 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2097 | 2148 | 111000 | 120000 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 36284 | 39301 | 37699 | 40716 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 38602 | 35568 | 73284 | 79301 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 38398 | 41432 | 74699 | 80716 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 37903 | 34852 | 110284 | 119301 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 39097 | 42148 | 111699 | 120716 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 78602 | 72568 | 75398 | 81432 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 159301 | 147284 | 34136 | 37204 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 148699 | 160716 | 39796 | 42864 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 118602 | 109568 | 71852 | 77903 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 112398 | 121432 | 76097 | 82148 |

Based on Table 6.157.3-1

-	2nd order harmonics may fall into LTE frequencies of Band 51, 76

-	3rd order harmonics may fall into LTE frequencies of Band 1, 4, 10, 65, 66, 70

Table 6.157.3-2: Band 66 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 111000 | 120000 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 38290 | 35220 | 38710 | 41780 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 36580 | 33440 | 72220 | 78290 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 40420 | 43560 | 75710 | 81780 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 1310 | 2180 | 36980 | 40020 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 76580 | 70440 | 77420 | 83560 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 34870 | 31660 | 109220 | 118290 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 42130 | 45340 | 112710 | 121780 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 158290 | 146220 | 29880 | 33160 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 149710 | 161780 | 43840 | 47120 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 116580 | 107440 | 68660 | 74870 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 114420 | 123560 | 79130 | 85340 |

Based on Table 6.157.3-2

-	2nd order harmonics may fall into LTE frequencies of Band 22, 42, 48

-	3rd order harmonic may fall into LTE frequencies of Band 46

-	2nd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into Rx frequencies of Band 1, 2, 3, 4, 9, 10, 11, 21, 23, 24, 25, 32, 33, 34, 35, 36, 37, 39, 45, 50, 51, 65, 70, 74, 75, and 76

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.157.3-3 and Table 6.157.3-4 list if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.157.3-3: 2UL Band 12 + Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.157.3-4: 2UL Band 66 + Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | IMD3 |
| Galileo | 1559 | - | 1591 | YES |  | IMD3 |
| GLONASS | 1591 | - | 1610 | YES |  | IMD3 |
| GPS | 1563 | - | 1587 | YES |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | 3rd harmonics |
|  | 5150 | - | 5350 | YES | Europe | 3rd harmonics |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | YES | Asia | 3rd harmonics |

Table 6.157.3-5 and Table 6.157.3-6 list the protected bands required for the dual connectivity configuration.

Table 6.157.3-5: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_12A_n260A | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 48, 50, 51, 71, 74, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 10, 66, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| Note 1: Table 6.5.3.2-1 of TS38.101 Note 2: NOTE 15 in Table 6.6.3.2-1 of 36.101 [2] will be applied here. Additional requirements for the frequency ranges that are less than FOOB (MHz) in Table 6.67.3.1-1 need further study. |  |  |  |  |  |  |  |

Table 6.157.3-6: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_12A_n260A | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 48, 50, 51, 71, 74, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 10, 66, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | NR Band 257 | FDL_low | - | FDL_high | [-5] | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | [-5] | 100 |  |
|  | Frequency range | 23600 | - | 24000 | [TBD] | 200 |  |
|  | Frequency range | 57000 | - | 66000 | [2] | 100 |  |
| Note 1: Table 6.5.3.2-1 of TS38.101 Note 2: NOTE 15 in Table 6.6.3.2-1 of 36.101 [2] will be applied here. Additional requirements for the frequency ranges that are less than FOOB (MHz) in Table 6.67.3.1-1 need further study. |  |  |  |  |  |  |  |

### 6.157.4	∆TIB and ∆RIB values

For DC_2A-66A_n260A, the TIB,c and RIB values are given in the tables below.

Table 6.157.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
|  | 12 | 0.3 |
| DC_12A-66A_n260A | 66 | 0.3 |
|  | n260 | 0 |

Table 6.157.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
|  | 12 | 0 |
| DC_12A-66A_n260A | 66 | 0 |
|  | n260 | 0 |

### 6.157.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.158	DC_12A-30A_n260A

### 6.158.1	Operating bands for DC

Table 6.158.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_12A-30A_n260A | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | 30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

### 6.158.2	Channel bandwidths per operating band for DC

Table 6.158.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  | [40 MHz] | 50 MHz |  | [60 MHz] |  | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_12A-30A_n260A | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  | 220 |
|  | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes | Yes | 420 |
| DC_12A-30A_n260G | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  | 220 |
|  | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260G Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_12A-30A_n260H | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  | 320 |
|  | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260H Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_12A-30A_n260I | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  | 420 |
|  | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260I Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_12A-30A_n260J | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  | 520 |
|  | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260J Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_12A-30A_n260K | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  | 620 |
|  | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260K Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_12A-30A_n260L | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  | 720 |
|  | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260L Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_12A-30A_n260M | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  | 820 |
|  | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260M Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.158.3	Co-existence studies

Table 6.158.3-1 lists Band 12A-30A +Band n260A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.158.3-1: Band 12 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1398 | 1432 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2097 | 2148 | 111000 | 120000 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 36284 | 39301 | 37699 | 40716 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 38602 | 35568 | 73284 | 79301 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 38398 | 41432 | 74699 | 80716 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 37903 | 34852 | 110284 | 119301 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 39097 | 42148 | 111699 | 120716 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 78602 | 72568 | 75398 | 81432 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 159301 | 147284 | 34136 | 37204 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 148699 | 160716 | 39796 | 42864 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 118602 | 109568 | 71852 | 77903 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 112398 | 121432 | 76097 | 82148 |

Based on Table 6.158.3-1

-	2nd order harmonics may fall into LTE frequencies of Band 51, 76

-	3rd order harmonics may fall into LTE frequencies of Band 1, 4, 10, 65, 66, 70

Table 6.158.3-2: Band 30 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2305 | 2315 | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4610 | 4630 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 6915 | 6945 | 111000 | 120000 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 34685 | 37695 | 39305 | 42315 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 35390 | 32370 | 71685 | 77695 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 41610 | 44630 | 76305 | 82315 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 33085 | 30055 | 108685 | 117695 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 43915 | 46945 | 113305 | 122315 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 75390 | 69370 | 78610 | 84630 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 157695 | 145685 | 27740 | 30780 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 150305 | 162315 | 46220 | 49260 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 115390 | 106370 | 67055 | 73085 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 115610 | 124630 | 80915 | 86945 |

Based on Table 6.158.3-2

-	2rd order harmonic may fall into frequencies of band n79

-	2nd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into own Rx frequencies of band n260

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.158.3-3 and Table 6.158.3-4 list if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.158.3-3: 2UL Band 12 + Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.158.3-4: 2UL Band 30 + Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.158.3-5 and Table 6.158.3-6 list the protected bands required for the dual connectivity configuration.

Table 6.158.3-5: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_12A_n260A | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 48, 50, 51, 71, 74, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 10, 66, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | NR Band 257 | FDL_low | - | FDL_high | [-5] | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | [-5] | 100 |  |
|  | Frequency range | 23600 | - | 24000 | [TBD] | 200 |  |
|  | Frequency range | 57000 | - | 66000 | [2] | 100 |  |
| Note 1: Table 6.5.3.2-1 of TS38.101 Note 2: NOTE 15 in Table 6.6.3.2-1 of 36.101 [2] will be applied here. Additional requirements for the frequency ranges that are less than FOOB (MHz) in Table 6.67.3.1-1 need further study. |  |  |  |  |  |  |  |

Table 6.158.3-6: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_30A_n260A | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 38, 41, 48, 66, 70, 71, 85, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band 257 | FDL_low | - | FDL_high | [-5] | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | [-5] | 100 |  |
|  | Frequency range | 23600 | - | 24000 | [TBD] | 200 |  |
|  | Frequency range | 57000 | - | 66000 | [2] | 100 |  |
| Note: Table 6.5.3.2-1 of TS38.101 |  |  |  |  |  |  |  |

### 6.158.4	∆TIB and ∆RIB values

For DC_12A-30A_n260A, the TIB,c and RIB values are given in the tables below.

Table 6.158.4-1: ΔTIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12A-30A_n260A | 12 | 0.5 |
|  | 30 | 0.5 |
|  | n260 | 0 |

Table 6.158.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_12A-30A_n260A | 12 | 0 |
|  | 30 | 0 |
|  | n260 | 0 |

### 6.158.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.159	DC_5A-66A_n260A

### 6.159.1	Operating bands for DC

Table 6.159.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5A-66A_n260A | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

### 6.159.2	Channel bandwidths per operating band for DC

Table 6.159.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  | [40 MHz] |  | 50 MHz |  | [60 MHz] |  | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_5A-66A_n260A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | 230 |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes | Yes | 430 |
| DC_5A-66A_n260G | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | 230 |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260G Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-66A_n260H | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | 330 |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260H Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-66A_n260I | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | 430 |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260I Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-66A_n260J | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | 530 |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260J Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-66A_n260K | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | 630 |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260K Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-66A_n260L | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | 730 |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260L Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-66A_n260M | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | 830 |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260M Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.159.3	Co-existence studies

Table 6.159.3-1 and Table 6.159.3-2  lists Band 5A-66A + Band n260A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.159.3-1: Band 5 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 824 | 849 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 1648 | 1698 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 2472 | 2547 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1086 | 1001 | 2674 | 2759 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2851 | 2996 | 262 | 152 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4524 | 4669 | 3498 | 3608 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 4701 | 4906 | 562 | 697 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6374 | 6579 | 4322 | 4457 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2002 | 2172 | 5348 | 5518 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1546 | 1386 | 6816 | 6551 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5146 | 5306 | 8224 | 8489 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1153 | 1348 | 4082 | 3852 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6172 | 6367 | 7198 | 7428 |

Based on Table 6.159.3-1

-	3rd order harmonic may fall into frequencies of band n41

-	2nd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into frequencies of band 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 31, 44, 67, 68, 71, and 72

-	4th order IMD products may fall into own Rx frequencies of band n260

Table 6.159.3-2: Band 66 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 111000 | 120000 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 38290 | 35220 | 38710 | 41780 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 36580 | 33440 | 72220 | 78290 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 40420 | 43560 | 75710 | 81780 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 1310 | 2180 | 36980 | 40020 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 76580 | 70440 | 77420 | 83560 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 34870 | 31660 | 109220 | 118290 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 42130 | 45340 | 112710 | 121780 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 158290 | 146220 | 29880 | 33160 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 149710 | 161780 | 43840 | 47120 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 116580 | 107440 | 68660 | 74870 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 114420 | 123560 | 79130 | 85340 |

Based on Table 6.159.3-2

-	2nd order harmonics may fall into LTE frequencies of Band 22, 42, 48

-	3rd order harmonic may fall into LTE frequencies of Band 46

-	2nd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into Rx frequencies of Band 1, 2, 3, 4, 9, 10, 11, 21, 23, 24, 25, 32, 33, 34, 35, 36, 37, 39, 45, 50, 51, 65, 70, 74, 75, and 76

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.159.3-3 and Table 6.159.3-4 list if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.159.3-3: 2UL Band 5 + Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | YES | US/Europe | 3rd order harmonic |
|  | 2400 | - | 2494 | YES | Asia | 3rd order harmonic |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.159.3-4: 2UL Band 66 + Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | IMD3 |
| Galileo | 1559 | - | 1591 | YES |  | IMD3 |
| GLONASS | 1591 | - | 1610 | YES |  | IMD3 |
| GPS | 1563 | - | 1587 | YES |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | 3rd harmonics |
|  | 5150 | - | 5350 | YES | Europe | 3rd harmonics |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | YES | Asia | 3rd harmonics |

Table 6.159.3-5 and Table 6.159.3-6 list the protected bands required for the dual connectivity configuration.

Table 6.159.3-5: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_5A_n260A | Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 74, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 26 | FDL_low | - | FDL_high | -27 | 1 |  |
|  | Band 41 | FDL_low | - | FDL_high | -50 | 1 | 1 (Note) |
| Note: Table 6.5.3.2-1 of TS38.101 |  |  |  |  |  |  |  |

Table 6.159.3-6: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_66A_n260A | Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 42, 48 | FDL_low | - | FDL_high | -50 | 1 | 1 (Note) |
| Note: Table 6.5.3.2-1 of TS38.101 |  |  |  |  |  |  |  |

### 6.159.4	∆TIB and ∆RIB values

For DC_5A-66A-n260A, the TIB,c and RIB values are given in the tables below.

Table 6.159.4-1: ΔTIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5A-66A_n260A | 5 | 0.3 |
|  | 66 | 0.3 |
|  | n260 | 0 |

Table 6.159.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5A-66A_n260A | 5 | 0 |
|  | 66 | 0 |
|  | n260 | 0 |

### 6.159.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.160	DC_5A-30A_n260A

### 6.160.1	Operating bands for DC

Table 6.160.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5A-30A_n260A | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

### 6.160.2	Channel bandwidths per operating band for DC

Table 6.160.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  | [40 MHz] |  | 50 MHz |  | [60 MHz] |  | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_5A-30A_n260A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | 220 |
|  | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes | Yes | 420 |
| DC_5A-30A_n260G | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | 220 |
|  | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260G Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-30A_n260H | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | 320 |
|  | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260H Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-30A_n260I | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | 420 |
|  | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260I Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-30A_n260J | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | 520 |
|  | 66 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260J Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-30A_n260K | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | 620 |
|  | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260K Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-30A_n260L | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | 720 |
|  | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260L Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-30A_n260M | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | 820 |
|  | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260M Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.160.3	Co-existence studies

Table 6.x.3-1 lists Band 5A-30A +Band n260A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.160.3-1: Band 5 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 824 | 849 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 1648 | 1698 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 2472 | 2547 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1086 | 1001 | 2674 | 2759 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2851 | 2996 | 262 | 152 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4524 | 4669 | 3498 | 3608 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 4701 | 4906 | 562 | 697 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6374 | 6579 | 4322 | 4457 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2002 | 2172 | 5348 | 5518 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1546 | 1386 | 6816 | 6551 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5146 | 5306 | 8224 | 8489 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1153 | 1348 | 4082 | 3852 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6172 | 6367 | 7198 | 7428 |

Based on Table 6.160.3-1

-	3rd order harmonic may fall into frequencies of band n41

-	2nd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into frequencies of band 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 31, 44, 67, 68, 71, and 72

-	4th order IMD products may fall into own Rx frequencies of band n260

Table 6.160.3-2: Band 30 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2305 | 2315 | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4610 | 4630 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 6915 | 6945 | 111000 | 120000 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 34685 | 37695 | 39305 | 42315 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 35390 | 32370 | 71685 | 77695 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 41610 | 44630 | 76305 | 82315 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 33085 | 30055 | 108685 | 117695 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 43915 | 46945 | 113305 | 122315 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 75390 | 69370 | 78610 | 84630 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 157695 | 145685 | 27740 | 30780 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 150305 | 162315 | 46220 | 49260 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 115390 | 106370 | 67055 | 73085 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 115610 | 124630 | 80915 | 86945 |

Based on Table 6.160.3-2

-	2rd order harmonic may fall into frequencies of band n79

-	2nd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into own Rx frequencies of band n260

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.160.3-3 and Table 6.160.3-4 list if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.160.3-3: 2UL Band 5 + Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | YES | US/Europe | 3rd order harmonic |
|  | 2400 | - | 2494 | YES | Asia | 3rd order harmonic |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.160.3-4: 2UL Band 30 + Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.160.3-5 and Table 6.160.3-6 list the protected bands required for the dual connectivity configuration.

Table 6.160.3-5: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_5A_n260A | Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 74, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 26 | FDL_low | - | FDL_high | -27 | 1 |  |
|  | Band 41 | FDL_low | - | FDL_high | -50 | 1 | 1 (Note) |
| Note: Table 6.5.3.2-1 of TS38.101 |  |  |  |  |  |  |  |

Table 6.160.3-6: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_30A_n260A | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 38, 41, 48, 66, 70, 71, 85, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band 257 | FDL_low | - | FDL_high | [-5] | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | [-5] | 100 |  |
|  | Frequency range | 23600 | - | 24000 | [TBD] | 200 |  |
|  | Frequency range | 57000 | - | 66000 | [2] | 100 |  |
| Note: Table 6.5.3.2-1 of TS38.101 |  |  |  |  |  |  |  |

### 6.160.4	∆TIB and ∆RIB values

For DC_5A-30A_n260A, the TIB,c and RIB values are given in the tables below.

Table 6.160.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
|  | 5 | 0.5 |
| DC_5A-30A_n260A | 30 | 0.5 |
|  | n260 | 0 |

Table 6.160.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
|  | 5 | 0 |
| DC_5A-30A_n260A | 30 | 0 |
|  | n260 | 0 |

### 6.160.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.161	DC_2A-66A_n260A

### 6.161.1	Operating bands for DC

Table 6.161.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2A-66A_n260A | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

### 6.161.2	Channel bandwidths per operating band for DC

Table 6.161.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  | [40 MHz] |  | 50 MHz |  | [60 MHz] |  | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_2A-66A_n260A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 240 |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes | Yes | 440 |
| DC_2A-66A_n260G | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 240 |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260G Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-66A_n260H | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 340 |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260H Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-66A_n260I | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 440 |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260I Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-66A_n260J | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 540 |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260J Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-66A_n260K | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 640 |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260K Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-66A_n260L | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 740 |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260L Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-66A_n260M | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 840 |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260M Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.161.3	Co-existence studies

Table 6.161.3-1 and Table 6.161.3-2  lists Band 2A-66A + Band n260A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.161.3-1: Band 2 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 111000 | 120000 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 38150 | 35090 | 38850 | 41910 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 36300 | 33180 | 72090 | 78150 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 40700 | 43820 | 75850 | 81910 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 1450 | 2310 | 36980 | 40020 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 76300 | 70180 | 77700 | 83820 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 34450 | 31270 | 109090 | 118150 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 42550 | 45730 | 112850 | 121910 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 158150 | 146090 | 29360 | 32600 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 149850 | 161910 | 44400 | 47640 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 116300 | 107180 | 68270 | 74450 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 114700 | 123820 | 79550 | 85730 |

Based on Table 6.161.3-1

-	2nd order harmonics may fall into LTE frequencies of Band 43

-	3rd order harmonics may fall into LTE frequencies of Band 46

-	2nd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into Rx frequencies of Band 1,  3, 4, 9, 10, 11, 21, 23, 24, 25, 32, 33, 34, 35, 36, 37, 39, 40, 45, 50, 65, 66, 70, 74, 75 and band n257

-	3rd order IMD products may fall into own Rx frequencies of Band 2 and n260

-	5th order IMD products may fall into Rx frequencies of band n257

Table 6.161.3-2: Band 66 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 111000 | 120000 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 38290 | 35220 | 38710 | 41780 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 36580 | 33440 | 72220 | 78290 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 40420 | 43560 | 75710 | 81780 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 1310 | 2180 | 36980 | 40020 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 76580 | 70440 | 77420 | 83560 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 34870 | 31660 | 109220 | 118290 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 42130 | 45340 | 112710 | 121780 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 158290 | 146220 | 29880 | 33160 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 149710 | 161780 | 43840 | 47120 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 116580 | 107440 | 68660 | 74870 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 114420 | 123560 | 79130 | 85340 |

Based on Table 6.161.3-2

-	2nd order harmonics may fall into LTE frequencies of Band 22, 42, 48

-	3rd order harmonic may fall into LTE frequencies of Band 46

-	2nd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into Rx frequencies of Band 1, 2, 3, 4, 9, 10, 11, 21, 23, 24, 25, 32, 33, 34, 35, 36, 37, 39, 45, 50, 51, 65, 70, 74, 75, and 76

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.161.3-3 and Table 6.161.3-4 list if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.161.3-3: 2UL Band 2 + Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | IMD3 |
| Galileo | 1559 | - | 1591 | YES |  | IMD3 |
| GLONASS | 1591 | - | 1610 | YES |  | IMD3 |
| GPS | 1563 | - | 1587 | YES |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | 3rd order harmonic |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | YES |  | 3rd order harmonic |
|  | 5150 | - | 5825 | YES | Asia | 3rd order harmonic |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | YES | China | 3rd order harmonic |
|  | 47200 | - | 48400 | YES | China | 5th IMD |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.161.3-4: 2UL Band 66 + Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | IMD3 |
| Galileo | 1559 | - | 1591 | YES |  | IMD3 |
| GLONASS | 1591 | - | 1610 | YES |  | IMD3 |
| GPS | 1563 | - | 1587 | YES |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | 3rd harmonics |
|  | 5150 | - | 5350 | YES | Europe | 3rd harmonics |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | YES | Asia | 3rd harmonics |

Table 6.161.3-5 and Table 6.161.3-6 list the protected bands required for the dual connectivity configuration.

Table 6.161.3-5: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_2A_n260A | Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 50, 51, 66, 70, 71, 74, 85, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | x |
|  | Band 43 | FDL_low | - | FDL_high | -50 | 1 | 1* |
| * Table 6.5.3.2-1 of TS38.101 Note x: NOTE 15 in Table 6.6.3.2-1 of 36.101 [2] will be applied here. Additional requirements for the frequency ranges that are less than FOOB (MHz) in Table 6.X.3.1-1 need further study. |  |  |  |  |  |  |  |

Table 6.161.3-6: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_66A_n260A | Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 42, 48 | FDL_low | - | FDL_high | -50 | 1 | 1* |
| * Table 6.5.3.2-1 of TS38.101 |  |  |  |  |  |  |  |

### 6.161.4	∆TIB and ∆RIB values

For DC_2A-66A-n260A, the TIB,c and RIB values are given in the tables below.

Table 6.161.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2A-66A_n260A | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n260 | 0 |

Table 6.161.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2A-66A_n260A | 2 | 0 |
|  | 66 | 0 |
|  | n260 | 0 |

### 6.161.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.162	DC_2A-30A_n260A

### 6.162.1	Operating bands for DC

Table 6.162.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2A-30A_n260A | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

### 6.162.2	Channel bandwidths per operating band for DC

Table 6.162.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  | [40 MHz] |  | 50 MHz |  | [60 MHz] |  | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_2A-30A_n260A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 230 |
|  | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes | Yes | 430 |
| DC_2A-30A_n260G | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 230 |
|  | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260G Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-30A_n260H | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 330 |
|  | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260H Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-30A_n260I | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 430 |
|  | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260I Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-30A_n260J | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 530 |
|  | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260J Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-30A_n260K | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 630 |
|  | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260K Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-30A_n260L | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 730 |
|  | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260L Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-30A_n260M | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 830 |
|  | 30 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260M Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.162.3	Co-existence studies

Table 6.162.3-1 lists Band 2A-30A +Band n260A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.162.3-1: Band 2 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 111000 | 120000 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 38150 | 35090 | 38850 | 41910 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 36300 | 33180 | 72090 | 78150 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 40700 | 43820 | 75850 | 81910 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 1450 | 2310 | 36980 | 40020 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 76300 | 70180 | 77700 | 83820 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 34450 | 31270 | 109090 | 118150 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 42550 | 45730 | 112850 | 121910 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 158150 | 146090 | 29360 | 32600 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 149850 | 161910 | 44400 | 47640 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 116300 | 107180 | 68270 | 74450 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 114700 | 123820 | 79550 | 85730 |

Based on Table 6.162.3-1

-	2nd order harmonics may fall into LTE frequencies of Band 43

-	3rd order harmonics may fall into LTE frequencies of Band 46

-	2nd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into Rx frequencies of Band 1,  3, 4, 9, 10, 11, 21, 23, 24, 25, 32, 33, 34, 35, 36, 37, 39, 40, 45, 50, 65, 66, 70, 74, 75 and band n257

-	3rd order IMD products may fall into own Rx frequencies of Band 2 and n260

-	5th order IMD products may fall into Rx frequencies of band n257

Table 6.162.3-2: Band 30 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2305 | 2315 | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4610 | 4630 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 6915 | 6945 | 111000 | 120000 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 34685 | 37695 | 39305 | 42315 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 35390 | 32370 | 71685 | 77695 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 41610 | 44630 | 76305 | 82315 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 33085 | 30055 | 108685 | 117695 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 43915 | 46945 | 113305 | 122315 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 75390 | 69370 | 78610 | 84630 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 157695 | 145685 | 27740 | 30780 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 150305 | 162315 | 46220 | 49260 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 115390 | 106370 | 67055 | 73085 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 115610 | 124630 | 80915 | 86945 |

Based on Table 6.162.3-2

-	2rd order harmonic may fall into frequencies of band n79

-	2nd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into own Rx frequencies of band n260

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.162.3-3 and Table 6.162.3-4 list if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.162.3-3: 2UL Band 2 + Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | IMD3 |
| Galileo | 1559 | - | 1591 | YES |  | IMD3 |
| GLONASS | 1591 | - | 1610 | YES |  | IMD3 |
| GPS | 1563 | - | 1587 | YES |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | 3rd order harmonic |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | YES |  | 3rd order harmonic |
|  | 5150 | - | 5825 | YES | Asia | 3rd order harmonic |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | YES | China | 3rd order harmonic |
|  | 47200 | - | 48400 | YES | China | 5th IMD |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.162.3-4: 2UL Band 30 + Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.162.3-5 and Table 6.162.3-6 list the protected bands required for the dual connectivity configuration.

Table 6.162.3-5: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_2A-n260A | Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 50, 51, 66, 70, 71, 74, 85, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | x |
|  | Band 43 | FDL_low | - | FDL_high | -50 | 1 | 1 (Note 1) |
| Note 1: Table 6.5.3.2-1 of TS38.101 Note 2: NOTE 15 in Table 6.6.3.2-1 of 36.101 [2] will be applied here. Additional requirements for the frequency ranges that are less than FOOB (MHz) in Table 6.x.3.1-1 need further study. |  |  |  |  |  |  |  |

Table 6.162.3-6: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_30A_n260A | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 38, 41, 48, 66, 70, 71, 85, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
| Note : Table 6.5.3.2-1 of TS38.101 |  |  |  |  |  |  |  |

### 6.162.4	∆TIB and ∆RIB values

For DC_2A-30A_n260A, the TIB,c and RIB values are given in the tables below.

Table 6.162.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2A-30A_n260A | 2 | 0.5 |
|  | 30 | 0.5 |
|  | n260 | 0 |

Table 6.162.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2A-30A_n260A | 2 | 0 |
|  | 30 | 0 |
|  | n260 | 0 |

### 6.162.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.163	DC_2A-12A_n260A

### 6.163.1	Operating bands for DC

Table 6.163.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2A-12A_n260A | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

### 6.163.2	Channel bandwidths per operating band for DC

Table 6.163.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  | [40 MHz] |  | 50 MHz |  | [60 MHz] |  | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_2A-12A_n260A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 230 |
|  | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes | Yes | 430 |
| DC_2A-12A_n260G | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 230 |
|  | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260G Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-12A_n260H | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 330 |
|  | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260H Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-12A_n260I | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 430 |
|  | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260I Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-12A_n260J | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 530 |
|  | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260J Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-12A_n260K | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 630 |
|  | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260K Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-12A_n260L | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 730 |
|  | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260L Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-12A_n260L | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 830 |
|  | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260m Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.163.3	Co-existence studies

Table 6.163.3-1 lists Band 2A-12A +Band n260A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.163.3-1: Band 2 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 111000 | 120000 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 38150 | 35090 | 38850 | 41910 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 36300 | 33180 | 72090 | 78150 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 40700 | 43820 | 75850 | 81910 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 1450 | 2310 | 36980 | 40020 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 76300 | 70180 | 77700 | 83820 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 34450 | 31270 | 109090 | 118150 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 42550 | 45730 | 112850 | 121910 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 158150 | 146090 | 29360 | 32600 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 149850 | 161910 | 44400 | 47640 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 116300 | 107180 | 68270 | 74450 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 114700 | 123820 | 79550 | 85730 |

Based on Table 6.163.3-1

-	2nd order harmonics may fall into LTE frequencies of Band 43

-	3rd order harmonics may fall into LTE frequencies of Band 46

-	2nd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into Rx frequencies of Band 1,  3, 4, 9, 10, 11, 21, 23, 24, 25, 32, 33, 34, 35, 36, 37, 39, 40, 45, 50, 65, 66, 70, 74, 75 and band n257

-	3rd order IMD products may fall into own Rx frequencies of Band 2 and n260

-	5th order IMD products may fall into Rx frequencies of band n257

Table 6.163.3-2: Band 12 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1398 | 1432 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2097 | 2148 | 111000 | 120000 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 36284 | 39301 | 37699 | 40716 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 38602 | 35568 | 73284 | 79301 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 38398 | 41432 | 74699 | 80716 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 37903 | 34852 | 110284 | 119301 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 39097 | 42148 | 111699 | 120716 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 78602 | 72568 | 75398 | 81432 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 159301 | 147284 | 34136 | 37204 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 148699 | 160716 | 39796 | 42864 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 118602 | 109568 | 71852 | 77903 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 112398 | 121432 | 76097 | 82148 |

Based on Table 6.163.3-2

-	2nd order harmonics may fall into LTE frequencies of Band 51, 76

-	3rd order harmonics may fall into LTE frequencies of Band 1, 4, 10, 65, 66, 70

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.163.3-3 and Table 6.163.3-4 list if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.163.3-3: 2UL Band 2 + Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | IMD3 |
| Galileo | 1559 | - | 1591 | YES |  | IMD3 |
| GLONASS | 1591 | - | 1610 | YES |  | IMD3 |
| GPS | 1563 | - | 1587 | YES |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | 3rd order harmonic |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | YES |  | 3rd order harmonic |
|  | 5150 | - | 5825 | YES | Asia | 3rd order harmonic |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | YES | China | 3rd order harmonic |
|  | 47200 | - | 48400 | YES | China | 5th IMD |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.163.3-4: 2UL Band 12 + Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.163.3-5 and Table 6.163.3-6 list the protected bands required for the dual connectivity configuration.

Table 6.163.3-5: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_2A_n260A | Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 50, 51, 66, 70, 71, 74, 85, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | x |
|  | Band 43 | FDL_low | - | FDL_high | -50 | 1 | 1 (Note 1) |
| Note 1: Table 6.5.3.2-1 of TS38.101 Note 2: NOTE 15 in Table 6.6.3.2-1 of 36.101 [2] will be applied here. Additional requirements for the frequency ranges that are less than FOOB (MHz) in Table 6.1633.1-1 need further study. |  |  |  |  |  |  |  |

Table 6.163.3-6: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_12A_n260A | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 48, 50, 51, 71, 74, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 10, 66, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | NR Band 257 | FDL_low | - | FDL_high | [-5] | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | [-5] | 100 |  |
|  | Frequency range | 23600 | - | 24000 | [TBD] | 200 |  |
|  | Frequency range | 57000 | - | 66000 | [2] | 100 |  |
| Note 1: Table 6.5.3.2-1 of TS38.101 Note 2: NOTE 15 in Table 6.6.3.2-1 of 36.101 [2] will be applied here. Additional requirements for the frequency ranges that are less than FOOB (MHz) in Table 6.67.3.1-1 need further study. |  |  |  |  |  |  |  |

### 6.163.4	∆TIB and ∆RIB values

For DC_2A-12A_n260A, the TIB,c and RIB values are given in the tables below.

Table 6.163.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2A-12A_n260A | 2 | 0.3 |
|  | 12 | 0.3 |
|  | n260 | 0 |

Table 6.163.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2A-12A_n260A | 2 | 0 |
|  | 12 | 0 |
|  | n260 | 0 |

### 6.163.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

## 6.164	DC_2A-5A-n260A

### 6.164.1	Operating bands for DC

Table 6.164.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2A-5A_n260A | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

### 6.164.2	Channel bandwidths per operating band for DC

Table 6.164.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  | [40 MHz] |  | 50 MHz |  | [60 MHz] |  | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_2A-5A_n260A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 230 |
|  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |
|  |  | 120 |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes | Yes | 430 |
| DC_2A-5A_n260G | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 230 |
|  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260G Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-5A_n260H | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 330 |
|  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260H Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-5A_n260I | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 430 |
|  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260I Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-5A_n260J | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 530 |
|  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260J Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-5A_n260K | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 630 |
|  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260K Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-5A_n260L | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 730 |
|  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260L Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-5A_n260M | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 830 |
|  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260M Bandwidth Combination Fall-back Group 3 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.164.3	Co-existence studies

Table 6.164.3-1 lists Band 2A-5A +Band n260A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.164.3-1: Band 2 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 37000 | 40000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 74000 | 80000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 111000 | 120000 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 38150 | 35090 | 38850 | 41910 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 36300 | 33180 | 72090 | 78150 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 40700 | 43820 | 75850 | 81910 |
| Two-tone 3rd order IMD products | fx_low – Max BW-fy | fx_high + Max BW-fy | fy_low – Max BW-fx | fy_high + Max BW-fx |
| IMD frequency limits (MHz) | 1450 | 2310 | 36980 | 40020 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 76300 | 70180 | 77700 | 83820 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 34450 | 31270 | 109090 | 118150 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 42550 | 45730 | 112850 | 121910 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 158150 | 146090 | 29360 | 32600 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 149850 | 161910 | 44400 | 47640 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 116300 | 107180 | 68270 | 74450 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 114700 | 123820 | 79550 | 85730 |

Based on Table 6.164.3-1

-	2nd order harmonics may fall into LTE frequencies of Band 43

-	3rd order harmonics may fall into LTE frequencies of Band 46

-	2nd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into Rx frequencies of Band 1,  3, 4, 9, 10, 11, 21, 23, 24, 25, 32, 33, 34, 35, 36, 37, 39, 40, 45, 50, 65, 66, 70, 74, 75 and band n257

-	3rd order IMD products may fall into own Rx frequencies of Band 2 and n260

-	5th order IMD products may fall into Rx frequencies of band n257

Table 6.164.3-2: Band 5 and Band n260 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 824 | 849 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 1648 | 1698 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 2472 | 2547 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1086 | 1001 | 2674 | 2759 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2851 | 2996 | 262 | 152 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4524 | 4669 | 3498 | 3608 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 4701 | 4906 | 562 | 697 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6374 | 6579 | 4322 | 4457 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2002 | 2172 | 5348 | 5518 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1546 | 1386 | 6816 | 6551 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5146 | 5306 | 8224 | 8489 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1153 | 1348 | 4082 | 3852 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6172 | 6367 | 7198 | 7428 |

Based on Table 6.164.3-2

-	3rd order harmonic may fall into frequencies of band n41

-	2nd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into own Rx frequencies of band n260

-	3rd order IMD products may fall into frequencies of band 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 31, 44, 67, 68, 71, and 72

-	4th order IMD products may fall into own Rx frequencies of band n260

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.164.3-3 and Table 6.164.3-4 list if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.164.3-3: 2UL Band 2 + Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | IMD3 |
| Galileo | 1559 | - | 1591 | YES |  | IMD3 |
| GLONASS | 1591 | - | 1610 | YES |  | IMD3 |
| GPS | 1563 | - | 1587 | YES |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | 3rd order harmonic |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | YES |  | 3rd order harmonic |
|  | 5150 | - | 5825 | YES | Asia | 3rd order harmonic |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | YES | China | 3rd order harmonic |
|  | 47200 | - | 48400 | YES | China | 5th IMD |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA - Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

Table 6.164.3-4: 2UL Band 5 + Band n260 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | YES | US/Europe | 3rd order harmonic |
|  | 2400 | - | 2494 | YES | Asia | 3rd order harmonic |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

Table 6.164.3-5 and Table 6.164.3-6 list the protected bands required for the dual connectivity configuration.

Table 6.164.3-5: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_2A-n260A | Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 50, 51, 66, 70, 71, 74, 85, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | x |
|  | Band 43 | FDL_low | - | FDL_high | -50 | 1 | 1(Note 1) |
| Note 1: Table 6.5.3.2-1 of TS38.101 Note 2: NOTE 15 in Table 6.6.3.2-1 of 36.101 [2] will be applied here. Additional requirements for the frequency ranges that are less than FOOB (MHz) in Table 6.164.3.1-1 need further study. |  |  |  |  |  |  |  |

Table 6.164.3-6: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_5A_n260A | Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 74, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 26 | FDL_low | - | FDL_high | -27 | 1 |  |
|  | Band 41 | FDL_low | - | FDL_high | -50 | 1 | 1 (Note) |
| Note: Table 6.5.3.2-1 of TS38.101 |  |  |  |  |  |  |  |

### 6.164.4	∆TIB and ∆RIB values

For DC_2A-5A_n260A, the TIB,c and RIB values are given in the tables below.

Table 6.164.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2A-5A_n260A | 2 | 0.3 |
|  | 5 | 0.3 |
|  | n260 | 0 |

Table 6.164.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2A-5A_n260A | 2 | 0 |
|  | 5 | 0 |
|  | n260 | 0 |

### 6.164.5	MSD

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |  |
| 2017-08 | RAN4#84 | R4-1708985 |  |  |  | Initial TR skeleton | 0.0.1 |  |
| 2017-09 | RAN4 NR adhoc#3 | R4-1709578 |  |  |  | R4-1708984, TP for TR 37.863-02-01: DC band combination of LTE 3A-38A + NR Band n78 | 0.1.0 |  |
| 201710 | RAN4#84bis | R4-1710236 |  |  |  | R4-1709595, TP for TR 37.863-02-01: DC band combination of LTE 1A-7A + NR Band n78 R4-1709597, TP for TR 37.863-02-01: DC band combination of LTE 1A-20A and NR Band n78 R4-1709612, TP for TR 37.863-02-01: DC band combination of LTE 3A-7A and NR Band 78 R4-1709616, TP for TR 37.863-02-01: DC band combination of LTE 3A-20A and NR Band 78 R4-1709650, TP for TR 37.863-02-01: DC band combination of LTE 7A-20A and NR Band 78 R4-1709681, TP for TR 37.863-02-01 DC_5A-7A-n78A R4-1710033, TP for TR 37.863-02-01: DC band combination of LTE 3A-38A and NR Band n78 | 0.2.0 |  |
| 2017-11 | RAN4#85 | R4-1713446 |  |  |  | R4-1711838, TP for TR 37.863-02-01 DC_1A-3A-n78A R4-1710630, TP for TR 37.863-02-01 DC_1A-5A-n78A R4-1710678, TP for TR 37.863-02-01 DC_1A-5A_n257A R4-1710679, TP for TR 37.863-02-01 DC_1A-7A_n257A R4-1710680, TP for TR 37.863-02-01 DC_3A-5A_n257A R4-1710681, TP for TR 37.863-02-01 DC_3A-7A_n257A R4-1710682, TP for TR 37.863-02-01 DC_5A-7A_n257A R4-1710683, TP for TR 37.863-02-01 DC_7A-7A_n257A R4-1710697, TP for TR 37.863-02-01 DC_3A-5A_n78A R4-1710698, TP for TR 37.863-02-01 DC_7A-7A_n78A R4-1710964, TP for TR 37.863-02-01 3DL/2UL DC_1A-3A-n78A R4-1710965, TP for TR 37.863-02-01 3DL/2UL DC_1A-19A-n78A R4-1710966, TP for TR 37.863-02-01 3DL/2UL DC_1A-21A-n78A R4-1710967, TP for TR 37.863-02-01 3DL/2UL DC_3A-19A-n78A R4-1710968, TP for TR 37.863-02-01 3DL/2UL DC_3A-21A-n78A R4-1710969, TP for TR 37.863-02-01 3DL/2UL DC_19A-21A-n78A R4-1710978, TP for TR 37.863-02-01 3DL/2UL DC_1A-3A-n79A R4-1710979, TP for TR 37.863-02-01 3DL/2UL DC_1A-19A-n79A R4-1710980, TP for TR 37.863-02-01 3DL/2UL DC_1A-21A-n79A R4-1710981, TP for TR 37.863-02-01 3DL/2UL DC_3A-19A-n79A R4-1710982, TP for TR 37.863-02-01 3DL/2UL DC_3A-21A-n79A R4-1710983, TP for TR 37.863-02-01 3DL/2UL DC_19A-21A-n79A R4-1711050, TP for TR 37.863-02-01 DC_1A-3A-n257A R4-1711051, TP for TR 37.863-02-01 DC_1A-19A-n257A R4-1711052, TP for TR 37.863-02-01 DC_1A-21A-n257A R4-1711053, TP for TR 37.863-02-01 DC_3A-19A-n257A R4-1711054, TP for TR 37.863-02-01 DC_3A-21A-n257A R4-1711055, TP for TR 37.863-02-01 DC_19A-21A-n257A R4-1711077, TP for TR 37.863-02-01 DC_1A-3A-n77A R4-1711078, TP for TR 37.863-02-01 DC_1A-19A-n77A R4-1711079, TP for TR 37.863-02-01 DC_1A-21A-n77A R4-1711080, TP for TR 37.863-02-01 DC_3A-19A-n77A R4-1711081, TP for TR 37.863-02-01 DC_3A-21A-n77A R4-1711082, TP for TR 37.863-02-01 DC_19A-21A-n77A R4-1711380, TP for TR 37.863-02-01: DC_1A-3A_n28 R4-1711381, TP for TR 37.863-02-01: DC_1A-7A_n28 R4-1711382, TP for TR 37.863-02-01: DC_1A-20A_n28 R4-1711383, TP for TR 37.863-02-01: DC_3A-7A_n28 R4-1711384, TP for TR 37.863-02-01: DC_3A-20A_n28 R4-1711385, TP for TR 37.863-02-01: DC_7A-20A_n28 | 0.3.0 |  |
| 2018-01 | RAN4#AH1801 | R4-1800448 |  |  |  | R4-1712217, TP for TR 37.863-02-01 DC_1A-5A-n257A R4-1712218, TP for TR 37.863-02-01 DC_1A-7A-n257A R4-1712219, TP for TR 37.863-02-01 DC_3A_5A-n257A R4-1712220, TP for TR 37.863-02-01 DC_3A-7A-n257A R4-1712221, TP for TR 37.863-02-01 DC_5A-7A-n257A R4-1712222, TP for TR 37.863-02-01 DC_7A-7A-n257A R4-1712288, TP for TR 37.863-02-01 DC_41C-n41A R4-1712750, TP for TR 37.863-02-01: DC band combination of LTE 1A-20A and NR Band n78 R4-1712751, TP for TR 37.863-02-01: DC band combination of LTE 3A-7A and NR Band n78 R4-1712752, TP for TR 37.863-02-01: DC band combination of LTE 3A-20A and NR Band n78 R4-1712787, TP for TR 37.863-02-01: DC band combination of LTE 7A-20A and NR Band n78 R4-1712822, TP for TR 37.863-02-01 MSD for DC including Band n77, n78 and n79 R4-1712823, TP for TR 37.863-02-01 Delta values for DC including Band n77, n78 and n79 R4-1713187, TP for TR 37.863-02-01 Delta values for DC combinations including Band n257 R4-1713551, TP for TR 37.863-02-01: DC_1A-3A_n28 R4-1713553, TP for TR 37.863-02-01: DC_1A-7A_n28 R4-1713554, TP for TR 37.863-02-01: DC_1A-20A_n28 R4-1713555, TP for TR 37.863-02-01: DC_3A-7A_n28 R4-1713567, TP for TR 37.863-02-01: DC_3A-20A_n28 R4-1713568, TP for TR 37.863-02-01: DC_7A-20A_n28 R4-1713598, TP on protected bands, additional ILs and MSD level for TR 37.863-02-01 DC_5A-7A_n78A R4-1714044, TP for TR 37.863-02-01 DC_7A-7A-n78A R4-1714087, TP on protected bands, additional ILs and MSD level for TR 37.863-02-01 DC_1A-7A_n78A R4-1714174, TP for TR 37.863-02-01 DC_3A-5A-n78A R4-1714332, TP on protected bands, additional ILs and MSD level for TR 37.863-02-01 DC_1A-5A_n78A R4-1714350 TP on TR 37.863-02-01 for Single Tx Switched UL | 0.4.0 |  |
| 2018-02 | RAN4#86 | R4-1801521 |  |  |  | R4-1801146 TP for TR 37.863 DC_2A-2A-n257A R4-1801147 TP for TR 37.863 DC_2A-5A-n257A R4-1801148 TP for TR 37.863 DC_2A-13A-n257A R4-1801149 TP for TR 37.863 DC_2A-66A-n257A R4-1801150 TP for TR 37.863 DC_2C-n257A R4-1801152 TP for TR 37.863 DC_5B-n257A R4-1801153 TP for TR 37.863 DC_5A-5A-n257A R4-1801154 TP for TR 37.863 DC_5A-66A-n257A R4-1801156 TP for TR 37.863 DC_13A-66A-n257A R4-1801158 TP for TR 37.863 DC_48A-48A-n257A R4-1801159 TP for TR 37.863 DC_48C-n257A R4-1801160 TP for TR 37.863 DC_66A-66A-n257A R4-1801161 TP for TR 37.863 DC_66C-n257A R4-1801162 TP for TR 37.863 DC_66A-66A-n260A R4-1801163 TP for TR 37.863 DC_66C-n260A R4-1801164 TP for TR 37.863 DC_48A-48A-n260A R4-1801165 TP for TR 37.863 DC_2A-2A-n260A R4-1801166 TP for TR 37.863 DC_2A-13A-n260A R4-1801167 TP for TR 37.863 DC_2C-n260A R4-1801168 TP for TR 37.863 DC_5A-5A-n260A R4-1801169 TP for TR 37.863 DC_5B-n260A R4-1801172 TP for TR 37.863 DC_13A-66A-n260A R4-1801173 TP for TR 37.863 DC_48C-n260A R4-1801131 DC_2A-(n)71B TP for TR 37 863-02-01. R4-1800219 TP for TR 37.863-02-01 DC_19A-42A_n78A R4-1800220 TP for TR 37.863-02-01 DC_19A-42A_n79A R4-1800221 TP for TR 37.863-02-01 DC_19A-42A_n257A R4-1800222 TP for TR 37.863-02-01 DC_1A-42A_n77A R4-1800223 TP for TR 37.863-02-01 DC_1A-42A_n78A R4-1800224 TP for TR 37.863-02-01 DC_1A-42A_n79A R4-1800225 TP for TR 37.863-02-01 DC_1A-42A_n257A R4-1800226 TP for TR 37.863-02-01 DC_42C_n77A R4-1800227 TP for TR 37.863-02-01 DC_42C_n78A R4-1800228 TP for TR 37.863-02-01 DC_42C_n79A R4-1800229 TP for TR 37.863-02-01 DC-42C_n257A R4-1800458 TP for TR 37.863-02-01: Updated scope R4-1800465 TP for TR 37.863-02-01 DC_3A-41A-n78A R4-1800466 TP for TR 37.863-02-01 DC_3C-41A-n78A R4-1800502 TP for TR 37.863-02-01 DC_7A-46A_n78A R4-1800503 TP for TR 37.863-02-01 DC_46C_n78A R4-1800504 TP for TR 37.863-02-01 DC_3C_n78A R4-1800505 TP for TR 37.863-02-01 DC_7C_n78A R4-1801116 TP for TR 37.863-02-01 for DC combinations of LTE bands 1, 18, 28, 41, 42 and NR bands n77, n78, n79, n257 R4-1800677 TP for TR 37.863-02-01 3DL/2UL DC_21A-42A_n77A R4-1800678 TP for TR 37.863-02-01 3DL/2UL DC_21A-42A_n78A R4-1800679 TP for TR 37.863-02-01 3DL/2UL DC_21A-42A_n79A R4-1800680 TP for TR 37.863-02-01 3DL/2UL DC_21A-42A_n257A R4-1801184 DC_66A_(n)71B R4-1800725 DC_3A-28A_n78 R4-1800726 DC_7A-28A_n78 R4-1800727 DC_7C_n78 R4-1800815 TP for TR 37.863-02-01 DC_3A-42A_n78A R4-1800816 TP for TR 37.863-02-01 DC_3A-42A_n79A R4-1800817 TP for TR 37.863-02-01 DC_3A-42A_n257A | 0.5.0 |  |
| 2018-04 | RAN4#86bis | R4-1803876 |  |  |  | R4-1803266 TP for TR 37.863-02-01: MSD for DC_3A-7A_n78A R4-1803267 TP for TR 37.863-02-01: MSD for DC_3A-20A_n78A R4-1803272 TP for TR 37.863-02-01: MSD for DC_7A-20A_n78A R4-1803194 TP for TR 37.863-02-01: DC_7A-20A_n78A R4-1803193 TP for TR 37.863-02-01: DC_3A-20A_n78A R4-1803192 TP for TR 37.863-02-01: DC_3A-7A_n78A R4-1803191 TP for TR 37.863-02-01: DC_1A-20A_n78A R4-1802200 TP for TR 37.863-02-01: DC_41C_n41A SEM R4-1802265 TP for TR 37.863-02-01: Introduction of DC_(n)41D R4-1802172 TP for TR 37.863-02-01: DC_1A-3A_n28 R4-1802173 TP for TR 37.863-02-01: DC_1A-7A_n28 R4-1802175 TP for TR 37.863-02-01: DC_1A-20A_n28 R4-1802176 TP for TR 37.863-02-01: DC_3A-7A_n28 R4-1802177 TP for TR 37.863-02-01: DC_3A-20A_n28 R4-1802178 TP for TR 37.863-02-01: DC_7A-20A_n28 | 0.6.0 |  |
| 2018-05 | RAN4#87 | R4-1807134 |  |  |  | R4-1805594 TP for TR 37.863-02-01 DC_1A-41A-n78A R4-1805119 TP for TR 37.863-02-01: DC_1A-3A_n78A R4-1805053 TP for TR 37.863-02-01 DC_5A-41A-n78A R4-1805125 TP for TR 37.863-02-01: DC_1A-7A_n78A R4-1805715 MSD analysis results for remaining LTE(2DL/1UL) + NR(1DL/1UL) DC UE R4-1805574 TP for TR 37.863-02-01 for DC combinations of DC_1A-41A-n77A R4-1804893 Insertion loss and MSD values for DC_7C_n78 R4-1805051 TP for TR 37.863-02-01 for DC combinations of DC_41C-n79A R4-1803833 TP for TR 37.863-02-01: DC_2A-66A-n71A R4-1803943 TP for TR 37.863-02-01: DC_1A-20A_n78A R4-1804068 TP for TR 37.863-02-01: MSD requirements for DC_1A-3A_n28 R4-1804490 TP for TR 37.863-02-01 for DC combinations of DC_1A-18A-n79A R4-1804495 TP for TR 37.863-02-01 for DC combinations of DC_1A-28A-n79A R4-1804527 TP for TR 37.863-02-01 for DC combinations of DC_1A-28A-n257A R4-1804532 TP for TR 37.863-02-01 DC_1A-28A_n79A R4-1804534 TP for TR 37.863-02-01 DC_3A-28A_n77A R4-1804535 TP for TR 37.863-02-01 DC_3A-28A_n79A R4-1804537 TP for TR 37.863-02-01 DC_21A-28A_n77A R4-1804538 TP for TR 37.863-02-01 DC_21A-28A_n78A R4-1804539 TP for TR 37.863-02-01 DC_21A-28A_n79A R4-1804543 TP for TR 37.863-02-01 DC_28A-42A_n79A R4-1804671 TP for TR 37.863-02-01 for DC combinations of DC_1A-41A-n79 R4-1804890 MSD values for DC_66A_(n)71B R4-1804891 Protected bands and MSD values for DC_3A-28A_n78 R4-1804892 Protected bands and MSD values for DC_7A-28A_n78 R4-1803777 TP for TR 37.863-02-01: DC_1A-8A_n78 R4-1803778 TP for TR 37.863-02-01: DC_3A-8A_n78 R4-1804493 TP for TR 37.863-02-01 for DC combinations of DC_1A-18A-n257A R4-1804533 TP for TR 37.863-02-01 DC_1A-28A_n257A R4-1804536 TP for TR 37.863-02-01 DC_3A-28A_n257A R4-1804540 TP for TR 37.863-02-01 DC_21A-28A_n257A R4-1804541 TP for TR 37.863-02-01 DC_28A-42A_n77A R4-1804542 TP for TR 37.863-02-01 DC_28A-42A_n78A R4-1804544 TP for TR 37.863-02-01 DC_28A-42A_n257A R4-1804904 TP for TR 37.863-02-01 for DC combinations of DC_18A-28A-n79A R4-1804954 TP for TR 37.863-02-01 for DC combinations of DC_18A-28A-n257A R4-1804964 TP for TR 37.863-02-01 for DC combinations of DC_41A-42A-n77A R4-1804965 TP for TR 37.863-02-01 for DC combinations of DC_41A-42A-n78A R4-1805046 TP for TR 37.863-02-01 for DC combinations of DC_41C-n77A R4-1805049 TP for TR 37.863-02-01 for DC combinations of DC_41C-n78A R4-1805052 TP for TR 37.863-02-01 for DC combinations of DC_41C-n257A R4-1805054 TP for TR 37.863-02-01 for DC combinations of DC_42C-n77A R4-1805058 TP for TR 37.863-02-01 for DC combinations of DC_42C-n78A | 0.7.0 |  |
| 2018-06 | RAN#80 | RP-181123 |  |  |  | R4-1808038 TP for TR 37.863-02-01 DC_3A-41A-n78A R4-1806611 TP for 37.863-02-01: DC_3A-38A_n78A R4-1808039 TP for TR 37.863-02-01 DC_3C-41A-n78A R4-1806414 TP for TR 37.863-02-01: to updated MSD for DC_2A-66A-n71A R4-1806415 TP for TR 37.863-02-01: to update requirements of DC_2A-(n)71B R4-1808194 TP for TR 37.863-02-01: to update requirements of DC_2A-(n)71B R4-1806477 TP for TR 37.863-02-01: DC_41A-42A-n79 R4-1808114 MSD analysis results for remaining LTE(2DL/1UL) + NR(1DL/1UL) DC UE R4-1806798 TP for TR 37.863-02-01: DC_12A-30A_n66A R4-1806802 TP for TR 37.863-02-01: DC_2A-12A_n66A R4-1806963 TP for TR 37.863-02-01: MSD requirements for DC_1A-20A_n28A R4-1807043 TP for MSD analysis for DC_1A_41A-n77A due to IMD4 and IMD5 R4-1807045 TP for MSD analysis for DC_5A_41A-n78A due to IMD2 and IMD5 R4-1807047 TP for MSD analysis for DC_3A-28A_n77A due to IMD3 R4-1806513 TP for TR 37.863-02-01 to add n77C, n78C, n79C and n257DEF for EN-DC including B1, 3, 19, 21, 28 and 42 R4-1806770 TP on TR 37.863-02-01: DC_1A-8A-n257A R4-1806771 TP on TR 37.863-02-01: DC_3A-41A-n257A R4-1806772 TP on TR 37.863-02-01: DC_41C-n78A R4-1806799 TP for TR 37.863-02-01: DC_5A-30A_n66A R4-1806800 TP for TR 37.863-02-01: DC_2A-30A_n66A R4-1806801 TP for TR 37.863-02-01: DC_2A-5A_n66A R4-1806836 TP for TR 37.863-02-01 DC_1A-41A-n78A R4-1806988 TP to TR37.863-02-01: DC_30A-66A_n260A R4-1806989 TP to TR37.863-02-01: DC_12A-66A_n260A R4-1806990 TP to TR37.863-02-01: DC_12A-30A_n260A R4-1806991 TP to TR37.863-02-01: DC_5A-66A_n260A R4-1806992 TP to TR37.863-02-01: DC_5A-30A_n260A R4-1806993 TP to TR37.863-02-01: DC_2A-66A_n260A R4-1806994 TP to TR37.863-02-01: DC_2A-30A_n260A R4-1806995 TP to TR37.863-02-01: DC_2A-12A_n260A R4-1807061 TP to TR37.863-02-01: DC_2A-5A_n260A v1.0.0 submitted for plenary approval | 1.0.0 |  |
| 2018-06 | RAN#80 |  |  |  |  | Approved by plenary – Rel-15 spec under change control | 15.0.0 |  |
| 2018-12 | RAN#82 | RP-182359 | 0001 | 1 | F | Correction on IMD source of DC_5A-7A_n78A in MSD table to TR 37.863-02-01 | 15.1.0 |  |
| 2018-12 | RAN#82 | RP-182359 | 0002 |  | F | CR to correct n260 BW classes | 15.1.0 |  |