3GPP TSG SA Meeting #78	TD SP-170835

## Lisbon, PT, 20-22 December 2017	Agenda Item: 18

3GPP WG-S4 Meeting #96 	S4-171356

Albuquerque, NM, USA, 13-17 November 2017 	(revision of S4-171347)

# Source:	Sony Mobile Communications, ORANGE, Qualcomm Incorporated, Apple (UK) Limited

# Title:	New WID on Receive acoustic output test in the presence of background noise

# Document for:	Approval

Agenda Item:	20

3GPP™ Work Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.
Comprehensive instructions can be found at http://www.3gpp.org/Work-Items

# Title: 	Receive acoustic output test in the presence of background noise

## Acronym: RAOT

## Unique identifier: 	{A number to be provided by MCC at the plenary}

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | x |  |  |  |
| No | x |  | x | x | x |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a

| x | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent and child Work Items

| Parent and child Work Items |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

## 3	Justification

Some UEs have a feature which automatically boosts the receive signal when in noisy environments, to maintain a sufficient signal-to-noise ratio at the user’s ear. Currently, the maximum acoustic output of the earpiece/earphone in handset and headset modes is specified in TS 26.131 in terms of receive loudness rating for maximum volume control setting (RLR ≤ -13 dB). However, this test is performed under silent lab conditions. Hence, there is currently no test to determine the maximum output (or minimum RLR) in case of background noise conditions.

CTIA has provided a liaison statement S4-171243 on the topic, indicating a need in the field for such new test.

## 4	Objective

Develop and specify a test method for receive acoustic output, or minimum receive loudness rating RLR, in the presence of background noise.

Analyse potential acoustic safety implications, considering the expected exposure times and levels.

Based on the safety analysis, specify recommendations and/or requirements.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | Series | Title | For info at TSG# | For approval at TSG# | Remarks |
| - | - | - | - | - | - |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |
| --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# |
| 26.131 | Add recommendations/requirements | SA#79 |
| 26.132 | Add test method | SA#79 |

## 6	Work item Rapporteur(s)

ISBERG, Peter, Sony Mobile Communications, peter.isberg [at] sony.com

## 7	Work item leadership

SA4

## 8	Aspects that involve other WGs

None

## 9	Supporting Individual Members

Supporting IM name

Sony Mobile Communications

ORANGE

Qualcomm Incorporated

Apple (UK) Limited