3GPP TR 23.715 V0.4.0 (2018-05)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Study on System enhancements for Provision of Access to

Restricted Local Operator Services by Unauthenticated UEs

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

3GPP, access, authentication, enhancements, services

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2018, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	6

2	References	6

3	Definitions, symbols and abbreviations	6

3.1	Definitions	6

3.2	Symbols	7

3.3	Abbreviations	7

4	Architectural Assumptions and Requirements	7

4.1	Architectural Assumptions	7

4.2	Architectural Requirements	8

5	Key Issues for EPC	8

5.1	Key Issue #EPC-1: Network indicating support for Restricted Local Operator Services and related UE behaviour	8

5.2	Key Issue #EPC-2: RLOS request indication	8

5.3	Key Issue #EPC-3: Support of unauthenticated UEs access to RLOS	8

5.4	Key Issue #EPC-4: Support of authenticated UEs access to RLOS	8

5.5	Void	9

5.6	Key Issue #EPC-6: Collection of charging information	9

5.7	Key Issue #EPC-7: Level of security	9

6	Key Issues for IMS	9

6.1	Key issue #IMS-1: Support for unauthenticated and authenticated user	9

6.2	Key issue #IMS-2: Identification of Restricted Local Operator Services at IMS layer	9

6.3	Key issue #IMS-3: Handling IMS session for Restricted Local Operator Service	9

6.4	Key issue #IMS-4: Support of emergency services by UEs attached for RLOS	10

7	Solutions	10

7.1	Solution #1: New SIB indicating support for Restricted Local Operator Services	10

7.1.1	Functional Description	10

7.1.2	Procedures	10

7.1.3	Impact on existing entities and interfaces	10

7.2	Solution #2: EPC attach/PDN connection for RLOS from unauthenticated/authenticated UE	10

7.2.1	Functional Description	10

7.2.2	Procedures	11

7.2.2.2	Detach procedure	11

7.2.3	Impact on existing entities and interfaces	12

7.3	Solution #3: IMS procedures for RLOS	12

7.3.1	Functional Description	12

7.3.2	Procedures	12

7.3.3	Impact on existing entities and interfaces	13

7.4	Solution #4: EPC solution using dedicated RLOS-APN	13

7.4.1	Functional Description	13

7.4.2	Procedures	13

7.4.2.1	Attach procedure	15

7.4.2.2	UE-initiated Detach procedure	16

7.4.2.3	MME-initiated Detach procedure	17

7.4.2.4	S1-based handover procedure	17

7.4.2.5	UE requested PDN connectivity	17

7.4.3	Impact on existing entities and interfaces	17

7.5	Solution #5: IMS solution with dedicated RLOS-APN and RLOS-specific P/S-CSCF	17

7.5.1	Functional Description	17

7.5.2	Procedures	18

7.5.2.1	IMS registration	18

7.5.2.2	Location Information retrieval	18

7.5.2.3	UE initiated RLOS IMS session establishment	19

7.5.3	Impact on existing entities and interfaces	19

7.6	Solution #6: Solution to key issue #EPC-3 & #EPC-4 to enable VoLTE calls after initial attachment	19

7.6.1	Functional Description	19

7.6.2	Procedures	20

7.6.3	Impact on existing entities and interfaces	20

7.7	Solution #7 to key issues #IMS-1, #IMS-2, and #IMS-3: IMS support for RLOS users	20

7.7.1	Functional Description	20

7.7.2	Procedures	21

7.7.3	Impact on existing entities and interfaces	21

7.8	Solution #8: Solution to key issues #IMS-4: Support of emergency services by UEs attached for RLOS	22

7.8.1	Functional Description	22

7.8.2	Procedures	22

7.8.2.1	UE performs detach from RLOS and then do emergency Attach	22

7.8.2.2	UE stays RLOS attached, and establishes emergency PDN connection	23

7.8.3	Impact on existing entities and interfaces	24

7.9	Solution #9: PLMN selection and radio access for RLOS	24

7.9.1	Functional Description	24

7.9.2	Procedures	25

7.9.3	Impact on existing entities and interfaces	25

7.X	Solution #X: <Solution Title>	25

7.X.1	Functional Description	25

7.X.2	Procedures	25

7.X.3	Impact on existing entities and interfaces	25

8	Evaluation	26

9	Conclusions	26

Annex <A>:	Change history	27

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

Y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The objective of this Technical Report is to identify and evaluate potential architecture enhancements of EPS and IMS needed to support Provision of Access to Restricted Local Operator Services by Unauthenticated UEs as identified in TS 22.101 [2], TS 22.115 [3] and TS 22.228 [4], and determine which of the solutions can proceed to normative specifications.

This feature is only applicable to EPS 3GPP access. Access to local operator services does not affect the UICC.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.101: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; Service aspects; Service principles".

[3]	3GPP TS 22.115: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Service aspects; Charging and billing".

[4]	3GPP TS 22.228: "3rd Generation Partnership Project; Service requirements for the IP multimedia core network subsystem".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[6]	3GPP TS 23.203: "Policies and Charging control architecture; Stage 2".

[7]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[8]	3GPP TS 23.167: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; IP Multimedia Subsystem (IMS) emergency sessions".

[9]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Restricted local operator services: communication services provided by an operator that involve either automated or human assistance (e.g. credit card billing, directory assistance, customer care) for which successful authentication is not necessary.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

RLOS	Restricted Local Operator Services

# 4	Architectural Assumptions and Requirements

## 4.1	Architectural Assumptions

The goal of the study is to enable access to those Restricted Local Operator Services (RLOS), however the definition of such restricted local operator services offered by an operator is out of scope of 3GPP.

Architectural assumptions are the following:

-	Access to RLOS is only possible for UEs when using EPC via E-UTRAN as IPCAN.

-	Both unauthenticated and authenticated UEs can access RLOS via the same architecture.

An authenticated UE allowed to access EPC may access the same services that are provided as RLOS but in this case, it is not within the scope of RLOS.

-	The UE shall indicate to the EPC and the IMS network that the request is a request for RLOS.

-	The standard shall support IMS emergency services for UEs attached for RLOS.

-	Allowing access to RLOS is completely under the local operator's control.

-	The solution shall support both non-IMS and IMS RLOS services.

-	When RLOS are accessed via IMS sessions:

1)	they do not require any specific support for location over and above what is defined by IMS already;

2)	they do not require any specific support regarding call back to the user that has initiated the session;

3)	the IMS RLOS are securely isolated to avoid e.g. DOS attacks to IMS entities offering regular IMS services.

-	Only UE-originated RLOS requests are supported.

-	No support of multiple PDN connections for RLOS.

-	No support of mobile terminated services.

-	This feature is only applicable to EPS 3GPP access.

-	Inter-RAT handovers and handover between 3GPP and non-3GPP accesses are not supported.

-	The use of the RLOS feature does not impact the local service provider's ability to support LI.

## 4.2	Architectural Requirements

Editor's note:	This clause will define the architectural requirements based on the normative stage-1 requirements defined in TS 22.101 [2], TS 22.115 [3] and TS 22.228 [4] with regards to Provision of Access to Restricted Local Operator Services by Unauthenticated UEs.

# 5	Key Issues for EPC

Editor's note:	This clause will describe the key issues for EPC.

## 5.1	Key Issue #EPC-1: Network indicating support for Restricted Local Operator Services and related UE behaviour

TS 22.101 [2] specifies: "When a UE recognizes an origination attempt to a restricted local operator service and has not received an indication from the serving system that restricted local operator services are available, the UE shall block the origination attempt."

This key issue addresses:

-	how the PLMN announces its support of RLOS to all UEs;

-	the behaviour of a supporting UE when it detects the network support of RLOS;

-	the behaviour of a supporting UE when it does not detect the network support of RLOS.

NOTE:	This key issue does not address the mechanisms for rejecting UE requests in case of an unauthorized attempt to access RLOS.

## 5.2	Key Issue #EPC-2: RLOS request indication

The solution shall address how unauthenticated and authenticated UEs indicate to the EPC that a request is for RLOS (e.g. at Attach).

## 5.3	Key Issue #EPC-3: Support of unauthenticated UEs access to RLOS

The solution shall address the EPC mechanisms required to support unauthenticated UEs. In particular, the following aspects are required to be studied:

-	how to allow unauthenticated UEs to access EPC network for RLOS only if the EPC network supports RLOS;

-	how to prevent unauthenticated UEs to access services which are not RLOS.

## 5.4	Key Issue #EPC-4: Support of authenticated UEs access to RLOS

The solution shall address the EPC mechanisms required to support authenticated UEs, in particular:

-	how to allow authenticated UEs to access the same RLOS as unauthenticated UEs. This applies to UEs that the MME has successfully authenticated but the UE is not able to receive normal service (e.g. is in a forbidden area).

NOTE:	Regularly attached UEs able to receive normal service may access the same services that are provided as RLOS but in this case, it is not within the scope of RLOS.

## 5.5	Void

## 5.6	Key Issue #EPC-6: Collection of charging information

The solution shall address how to collect charging information regarding the use of RLOS.

## 5.7	Key Issue #EPC-7: Level of security

The solution shall have a level of security, which should not be less than that which is currently applied to existing equivalent network access methods (e.g., unauthenticated emergency calling).

# 6	Key Issues for IMS

Editor's note:	This clause will describe the key issues for IMS.

## 6.1	Key issue #IMS-1: Support for unauthenticated and authenticated user

The key issue here is that the network enables any UE access to a requested service through RLOS. This applies to all UEs, authenticated and non-authenticated.

The solution should use the same mechanisms for both unauthenticated and authenticated UEs as much as possible.

## 6.2	Key issue #IMS-2: Identification of Restricted Local Operator Services at IMS layer

The key issue here is that it is beneficial if IMS services allow clear service identification. For access to restricted local operator service this means that:

-	the UE need to be able to determine the case and start session setup without requiring IMS registration for RLOS services. Further the UE will need to indicate to the IMS network that its request is for RLOS via appropriate information in the SIP request; and

-	the core network functional entities need to be able to detect the specific information for an RLOS request.

The study will determine what to configure in the IMS network (e.g., phone number, captive portal), in which IMS entity(ies), and what to signal to the UE (e.g. specific CSCF address);

## 6.3	Key issue #IMS-3: Handling IMS session for Restricted Local Operator Service

At IMS layer, RLOS are always provided in the local PLMN without any involvement of the Home PLMN.

The key issue for the UE is to support initiating a session setup for RLOS without requiring IMS registration for RLOS services.

NOTE:	UE only initiates IMS sessions for RLOS if the network has indicated support via signalling in the underlying layers.

The key issue for the IMS core network functional entities is to support session setup for RLOS from users without requiring IMS registration for RLOS services. Sessions for RLOS from authenticated and unauthenticated users are handled in the PLMN that provides the entry point into the IMS. This means that the IMS core network functional entities of the PLMN that provides the P-CSCF to which the UE is attached will handle the IMS session request locally.

## 6.4	Key issue #IMS-4: Support of emergency services by UEs attached for RLOS

The solution shall describe how a UE attached for RLOS can obtain emergency services.

# 7	Solutions

Editor's note:	This clause is intended to document the agreed architecture solutions. Each solution should clearly describe which of the key issues it covers and how.

## 7.1	Solution #1: New SIB indicating support for Restricted Local Operator Services

### 7.1.1	Functional Description

This is a solution to key issue EPC-1.

A new SIB provided by E-UTRAN indicates that the PLMN supports Restricted Local Operator Services. The PLMNs where RLOS is supported may be stored in UICC (or ME) for the UE.

### 7.1.2	Procedures

UE sees through SIB that PLMN supports Restricted Local Operator Services. The PLMNs where RLOS is supported may be configured from HPLMN stored in UICC (or ME) for the UE.

### 7.1.3	Impact on existing entities and interfaces

E-UTRAN and UE need to support a new SIB that indicates support for Restricted Local Operator Services.

The PLMNs where RLOS is supported may be configured from HPLMN and stored in UICC (or ME) for the UE.

Editor's note:	Whether the PLMN list where RLOS is supported is the same Preferred PLMN list that is used for "normal" PLMN selection (in which case no impacts in UICC) or a new list (in which case there are impacts in UICC) is FFS.

## 7.2	Solution #2: EPC attach/PDN connection for RLOS from unauthenticated/authenticated UE

### 7.2.1	Functional Description

This is a solution to key issue EPC-2, EPC-3, EPC-4 that defines Attach/Detach and PDN connectivity procedure for RLOS. The solution considers only the scenario where the UE requesting RLOS is unauthenticated, or the UE requesting RLOS is authenticated but in limited state.

### 7.2.2	Procedures

Figure 7.2.2-1: Attach/PDN connectivity procedure for RLOS

0.	A new SIB provided by E-UTRAN indicates that PLMN supports Restricted Local Operator Services. See clause 7.1.

1.	The UE sends an indication in Attach Request message that the Attach is for Restricted Local Operator Services (this is similar to the Emergency Attach indication that is used for "unauthenticated" UEs for emergency calls) which triggers the MME to select a locally configured APN that is used for RLOS.

In case of the authenticated UE (i.e. in limited state), the UE shall perform the detach procedure before step 1.

2.	The UE does not need to be authenticated by the MME, IMSI (if available) and IMEI(SV) are retrieved from the UE.

3.	MME sends a Create Session Request towards the PGW including the indication that is for RLOS (indication or APN) the IMSI (if available), and the IMEI(SV) as specified in TS 23.401 [5].

4.	PGW establishes an IP-CAN session with the PCRF as described in TS 23.401 [5] and TS 23.203 [6]. The IP-CAN session is identified with UE's IPv4 address or IPv6 prefix associated with the PDN connection for RLOS. The IMSI (if available), and the IMEI(SV) are passed to the PCRF as part of the IP-CAN session establishment.

Available RLOS service amount (e.g. allowed service time, allowed service volume, allowed inactivity period) is configured in PGW. If dynamic PCC is deployed, PCRF may provide PGW with Available RLOS service amount. When the Available RLOS service amount expires, PGW initiates deletion of the default bearer.

Editor's note:	What the UE would do with the Available RLOS service amount, and whether there is a need to deliver it to the UE is for FFS.

If needed the PGW returns back to the UE in a PCO the P-CSCF address used for RLOS using procedures defined in TS 23.228 [7] for P-CSCF discovery. The PGW shall block any traffic that is not from or to addresses of network entities (e.g. P-CSCF, captive portal) providing Restricted Local Operator Service.

5.	UE completes the Attach or UE requested PDN connection procedure.

#### 7.2.2.2	Detach procedure

When the Available RLOS service amount expires, PGW initiates release of the default bearer. When the PGW initiates to delete the default bearer for RLOS, if emergency PDN connection is not established, the MME performs detach procedure as specified in TS 23.401 [5].

When the RLOS-attached UE has no emergency PDN connection, if UE is transferred to ECM-IDLE state, UE and Network shall implicitly detach without NAS signalling transactions. i.e. if the RRC connection is released after allowed inactivity period configured in E-UTRAN expires, the UE shall detach itself implicitly without sending detach request, and if S1-AP is released, the MME shall detach the UE without sending the detach request.

### 7.2.3	Impact on existing entities and interfaces

Impacts in UE, MME, SGW/PGW and PCRF to implement the procedures defined in clause 7.2.2.

-	UE: UE includes RLOS indication in Attach request for RLOS.

When RLOS-attached UE becomes ECM-IDLE, UE detaches implicitly without NAS transaction with MME.

-	MME: MME selects the RLOS-dedicated APN to setup PDN connection, and may include RLOS indication to SGW/PGW.

When the UE attaches for RLOS, MME skips mutual Authentication procedure and location update to HSS.

When RLOS-attached UE becomes in ECM-IDLE state, MME detaches without NAS transaction with UE.

-	SGW/PGW: When the Available RLOS service amount (e.g. allowed inactivity period) expires, PGW initiates deletion of the default bearer of the RLOS PDN connection.

-	PCRF: provides the Available RLOS service amount (e.g. allowed inactivity period) of the RLOS PDN connection to PGW.

## 7.3	Solution #3: IMS procedures for RLOS

### 7.3.1	Functional Description

This is a solution to key issue IMS-1, IMS-2, IMS-3 that defines IMS procedure for RLOS.

### 7.3.2	Procedures

Figure 7.3.2-1: IMS procedure for RLOS

1.	If the UE is unauthenticated in IMS it initiates IMS registration by sending a SIP REGISTER (UserID-1) message indicating that is IMS Registration for RLOS. The UserID-1 parameter is an IMPI and optionally an IMPU.

2.	Upon reception of the SIP REGISTER message from the indication that the SIP REGISTER is for RLOS the P-CSCF determines that it is for RLOS. The P-CSCF may optionally request from the PCRF the EPS-level identities (e.g. IMSI, IMEI(SV)) in the Rx session establishment request. The PCRF performs session binding based on the UE's IP address/prefix (as defined in TS 23.203 [6] clause 6.1.1.2) and provides one or more EPS-level identities to the P-CSCF.

3.	Based on operator configuration for RLOS, the GIBA procedure over Gm as defined in TS 24.229 [9] is performed, the P-CSCF responds with a 420 response with sec-agree value listed in the unsupported header field.

4.	UE according to TS 24.229 [9], performs a new initial registration by sending a SIP REGISTER (UserID-2, IMEI) message and without inclusion of the Authorization header field. UserID-2 is an a public user identity derived from IMSI. P-CSCF may verify the IMSI/IMEI provided by the PCRF in step 7 against the IMSI/IMEI derived from the public user identity provided by the UE, prior to accepting the SIP REGISTER message.

5.	P-CSCF accepts the registration with 200 OK. From the UE point of view, the procedure is the same as specified for GIBA (GPRS-IMS bundled authentication) procedures in TS 24.229 [9].

6.	UE then attempts an RLOS session by sending a SIP INVITE (UserID-3) message. UserID-3 is set to UE's public identity.

7.	The P-CSCF verifies whether the UserID-3 indicated in the SIP INVITE message. If compliant, P-CSCF forwards the SIP INVITE towards the Call Centre that is providing the RLOS.

Editor's note:	It is FFS whether the P/S-CSCF or AS will restrict access only to the Restricted Local Operator Services.

### 7.3.3	Impact on existing entities and interfaces

Impacts in UE, P-CSCF and PCRF to implement the procedures in clause 7.3.2.

## 7.4	Solution #4: EPC solution using dedicated RLOS-APN

### 7.4.1	Functional Description

Editor's note:	This clause will describe the general description, assumption, and principles of the solution. The related key issues it solves will be mentioned.

### 7.4.2	Procedures

During the attach procedure:

-	If the UE is not attached to the network and detects that the user is requesting RLOS, then the UE shall check whether the PLMN is advertising its support of RLOS to all UEs. If the PLMN does not advertise its support of RLOS, the UE shall block the origination attempt (according to TS 22.101 [2] clause 34.2). If the PLMN advertises its support of RLOS, the UE shall indicate in the Attach Request that the attachment is for RLOS. In such case, and if the network supports RLOS, the network shall skip the authentication, establish a default PDN connection to a specific RLOS APN as part of the Attach procedure, and accept the Attach.

Session Management:

-	The solution assumes that a specific RLOS-APN, unique for the PLMN and configured in the MME, is used.

-	During attach, based on the presence of the RLOS indication in the NAS Attach Request, the MME may either select a specific RLOS PGW in the same PLMN, preconfigured in the MME RLOS Configuration Data or a dynamically allocated PGW in the same PLMN, based on a specific RLOS-APN.

-	During PDN connection establishment, MME send RLOS APN to the SGW and the PGW. The PCRF derives that the PDN connection is for RLOS via the RLOS-APN.

-	For IMS RLOS sessions, the PCRF may establish dedicated bearers.

-	Because at the end of RLOS session, the UE should be detached, any of the following procedures triggers a UE Detach procedure:

-	PDN GW initiated bearer deactivation, as described in clause 5.4.4.1.

-	UE initiated Detach procedure as described in clause 5.3.8.2.1.

Idle mode mobility and handovers:

-	During idle mobility involving MME change, as in emergency service, if the UE's IMSI is unauthenticated, the unauthenticated IMSI indication will be sent from the old/source MME to the new/target MME, and if the UE does not have a USIM, IMSI will not be included in Context Response as specified in TS 23.401 [5] clause 5.3.1.

-	During handover involving MME change, as in emergency service, if the UE is RLOS attached and the UE does not have a USIM, IMSI cannot be included in the MME UE context in Forward Relocation Request message, and if the IMSI cannot be authenticated, then the IMSI shall be marked as unauthenticated as specified in TS 23.401 [5] clause 5.5.1.2.

-	No additional impact is expected.

Charging:

-	Charging of RLOS PDN connections can be performed by OFCS and OCS as for regular PDN connections. The rating group provided by PCRF should just be a RLOS specific rating group. No changes are needed as the APN is known by the PCRF.

Location procedures:

-	Location services are not invoked.

#### 7.4.2.1	Attach procedure

Figure 7.4.2.1-1: Attach procedure

Attach procedure from Figure 5.3.2.1-1 applies, except for the following aspects:

Step 1	If the UE has detected that the user is requesting RLOS, the UE shall check whether the PLMN is advertising its support of RLOS to all UEs.  If the PLMN does not advertise its support of RLOS, the UE shall block the origination attempt (according to TS 22.101 [2] clause 34.2). If the PLMN announces its support of RLOS, the UE proceeds to attach by sending an Attach Request in which it indicates that the attachment is for RLOS (via e.g. Attach Type and/or Request Type) and it provides its IMSI (or its IMEI if the UE has no IMSI). The UE shall not identify itself with a temporary identity.

Step 5a	If the MME is configured to support RLOS for unauthenticated IMSIs and the UE has indicated that the attach request is for RLOS (via e.g. Attach Type "RLOS"), the MME skips the authentication and security setup.

Editor's note:	It is FFS whether, instead or in addition to skipping the authentication and security setup, the MME may also proceed with authentication, waiting for an authentication/authorization failure before continuing the attach procedure.

Step 5b	This step may be performed based on operator's policy.

Step 8	For an RLOS Attach in which the UE was not successfully authenticated, the MME shall not send an Update Location Request to the HSS.

Step 11	For an RLOS Attach, the MME shall not check for access restrictions, regional restrictions or subscription restrictions (e.g. CSG restrictions) and shall continue with the Attach procedure.

Step 12	For an RLOS Attach, the MME applies the parameters from MME RLOS Configuration Data for the RLOS bearer establishment performed in this step and any potentially stored IMSI related subscription data are ignored by the MME. For initial RLOS Attach, the MME selects either a specific RLOS PGW in the same PLMN, preconfigured in the MME RLOS Configuration Data, or a dynamically allocated PGW in the same PLMN, based on a specific RLOS-APN. For RLOS attached UEs, IMSI is included if available and, if the IMSI cannot be authenticated, then the IMSI shall be marked as unauthenticated. The RLOS characteristics of the default PDN connection (i.e. APN-AMBR, MBR, ARP) are pre-configured in the MME.

Step 13	For RLOS attached UEs, IMSI is included if available and if the IMSI cannot be authenticated then the IMSI shall be marked as unauthenticated.

Step 14	The P-GW and the PCRF determines that RLOS are requested based on the RLOS APN received in Create Session Request message. For RLOS attached UEs which are unauthenticated, the PDN GW provides the IMEI as the UE Identity instead of IMSI, to the PCRF. If the PCC is configured to support RLOS and if dynamic PCC is deployed, the PCRF, based on the RLOS APN, sets the ARP of the PCC rules to a value that is reserved for RLOS and the authorization of dynamic PCC rules. If dynamic PCC is not deployed, the PDN GW uses the ARP of the default RLOS EPS bearer for any potentially initiated dedicated RLOS EPS bearer.

Step 17	The new MME sends an Attach Accept with RLOS indication. For an RLOS attached UE, i.e. for UEs that have only RLOS EPS bearers established, there is no AS security context information included in the S1 control messages and there is no NAS level security when the UE cannot be authenticated. The RLOS Service Support indicator informs the UE that RLOS bearer services are supported, i.e. the UE is allowed to request PDN connectivity for RLOS.

Editor's note:	Whether AS or NAS security is needed is to be assessed by SA WG3.

Step 18	Manual CSG selection is not supported when an RLOS has been initiated.

Step 25	For an unauthenticated or roaming UE, if the UE has indicated that the request of for RLOS (via e.g. Request Type set to "RLOS"), the MME shall not send any Notify Request to an HSS.

#### 7.4.2.2	UE-initiated Detach procedure

UE-initiated Detach procedure from Figure 5.3.8.2-1 applies, except for the following aspects:

Step 1:	Security procedures that may be invoked if the NAS message is used to establish the S1 connection are not performed in the case of RLOS attached UEs that were not successfully authenticated.

#### 7.4.2.3	MME-initiated Detach procedure

MME-initiated Detach procedure from Figure 5.3.8.3-1 applies, except for the following aspects:

Step 1:	For RLOS attached UEs, MME initiated implicit detach procedures are based on an inactivity timeout specific to RLOS.

#### 7.4.2.4	S1-based handover procedure

S1-based handover procedure from Figure 5.5.1.2.2-1 applies, except for the following aspects:

Step 3:	For RLOS attached UEs, if the IMSI cannot be authenticated, then the IMSI shall be marked as unauthenticated. Also, in this case, security parameters are included only if available.

#### 7.4.2.5	UE requested PDN connectivity

An RLOS attached UE shall not initiate any PDN Connectivity Request procedure.

If the MME receives a PDN Connectivity Request from an RLOS attached UE, the MME shall reject this request.

### 7.4.3	Impact on existing entities and interfaces

Editor's note:	This clause will describe the impacts to existing nodes or functionality and interfaces.

## 7.5	Solution #5: IMS solution with dedicated RLOS-APN and RLOS-specific P/S-CSCF

### 7.5.1	Functional Description

This solution is intended to solve key issues #IMS-1, #IMS-2, #IMS-3 and #IMS-4.

During the EPC attach procedure, based on the RLOS indication provided by the UE, the EPC establishes a PDN connection to a specific APN dedicated to RLOS (see solutions #2 and #4). As RLOS are always provided in the PLMN the UE is accessing, the UE needs to select a P-CSCF suitable for RLOS sessions in that PLMN. For that, the address of the suitable P-CSCF is provided in the PCO by the PGW during Attach procedure. This dedicated address enables the use of either a general-purpose P-CSCF or a RLOS-specific P-CSCF. Whether a general-purpose P-CSCF or a RLOS-specific P-CSCF is used depends on the operator wishes about IMS isolation.

The UE, whatever it is unauthenticated or authenticated in limited state, performs an IMS registration for RLOS by adding an RLOS indication in the SIP Register message. The P-CSCF does not authenticate the UE if the UE has indicated RLOS, but it verifies that the UE is EPC attached via RLOS APN by querying the PCRF on whether the PDN connection was established for RLOS. If the above verification fails, the IMS registration is rejected. The P-CSCF creates a P-CSCF UE context.

Editor's note:	It is FFS how IP UE address spoofing is solved in case of an unauthenticated UE.

When the UE is successfully IMS registered for RLOS, it can request RLOS IMS services. On receiving the SIP Invite for an UE that is IMS registered UE for RLOS, the P-CSCF shall route the SIP request to a specific RLOS-CSCF to satisfy the IMS network isolation requirement (similar principle as for emergency IMS calls, which uses a specific E-CSCF). Emergency calls are detected the same way as in a regular P-CSCF and, in this case, the SIP request is routed to the E-CSCF. If the P-CSCF has determined that it is not for an emergency service and if the RLOS-CSCF has determined that the UE SIP request if not for RLOS, the UE SIP request is rejected.

The UE may request RLOS IMS services as long as it is IMS registered for RLOS. If the IMS registration timer expires, UE can re-register as long as it is EPC attached to the RLOS APN. However, when the UE is detached from the EPC, the PCRF shall indicate it to the P-CSCF, which then deregisters the UE from IMS i.e. it removes the P-CSCF UE context.

When provided, the network provided location information conveyed from the EPC to the P-CSCF via PCRF shall be forwarded to the RLOS-CSCF.

Charging of RLOS IMS sessions is performed by the IMS network as for a regular IMS sessions.

P-CSCF additional features for RLOS

-	The P-CSCF can be a RLOS-specific P-CSCF or a general-purpose P-CSCF.

-	The P-CSCF detects whether the IMS registration request from an UE is for RLOS from the RLOS indication provided by the UE. The P-CSCF verifies that the UE is EPC RLOS attached by querying the PCRF on whether the PDN connection used for IMS signalling was established for RLOS (i.e. to RLOS APN).

-	The P-CSCF selects an E-CSCF if it detects that the UE has initiated an emergency session request, per TS 23.167 [8] clause 7.3 (Emergency Session Establishment in the Serving IMS network). The P-CSCF shall also be able to detect non-UE detectable emergency sessions as specified in TS 23.167 [8] clause 7.2.

Editor's note:	Details for non UE-detectable emergency calls are FFS.

-	If available, the P-CSCF shall provide RLOS-CSCF with the NPLI received from the PGW via PCRF.

-	When the P-CSCF receives an indication from the PCRF that the UE has been RLOS detached from EPC, it shall deregister the UE from IMS i.e. it removes the P-CSCF UE context

PCRF additional features for RLOS

-	The PCRF shall reply to the request from P-CSCF asking whether the PDN connection was established for RLOS (i.e. to RLOS APN).

-	When the UE is RLOS detached from the EPC, the PCRF shall indicate it to the P-CSCF.

RLOS-CSCF additional features for RLOS

-	Receives the RLOS indication from P-CSCF.

-	Routes RLOS session establishment requests to an appropriate destination including anonymous session establishment requests.

-	Generates CDRs for RLOS.

### 7.5.2	Procedures

Editor's note:	This clause will describe the high-level procedures and information flows for the solution.

#### 7.5.2.1	IMS registration

The UE performs IMS registration for RLOS by adding a RLOS indication in the SIP Register message, which allows the P-CSCF to detect that the IMS registration is for RLOS. The P-CSCF, if configured through policies, does not authenticate the UE if it has indicated RLOS, but shall verify that the UE is RLOS attached by querying the PCRF on whether the PDN connection used for IMS signalling was established for RLOS (i.e. to RLOS APN).

#### 7.5.2.2	Location Information retrieval

Not applicable.

#### 7.5.2.3	UE initiated RLOS IMS session establishment

The following flow contains a high-level description of the RLOS procedures.

Figure 7.5.2.3-1: UE initiated RLOS IMS session establishment

The following steps are performed:

1.	The UE sends an Attach Request to the EPC including an RLOS indication. The EPC establishes a local break-out default PDN connection for RLOS to the RLOS specific APN and replies to the UE with an indication that the UE is attached for RLOS as well as with the address of a P-CSCF suitable for RLOS sessions (via PCO). The EPC also informs PCRF that the PDN connection is for RLOS. See solution #4 in the TR for details.

2.	According to clause 7.x.2.1, the UE initiates an IMS registration request for RLOS to the P-CSCF whose address was received in the PCO during RLOS Attach. SIP Register message shall contain an RLOS indication. The P-CSCF , if configured through policies, does not authenticate the UE if it has included RLOS indication in the SIP Register message but it shall verify that the UE is RLOS attached by querying the PCRF on whether the PDN connection used for IMS signalling was established for RLOS.

3.	If the UE requests RLOS, it sends a SIP INVITE for the establishment of an IMS RLOS session with an RLOS indication. The P-CSCF routes the SIP request to the RLOS specific CSCF. Emergency calls are detected the same way as in a regular P-CSCF, and in this case the SIP request is routed to the E-CSCF.

### 7.5.3	Impact on existing entities and interfaces

Editor's note:	This clause will describe the impacts to existing nodes or functionality and interfaces.

## 7.6	Solution #6: Solution to key issue #EPC-3 & #EPC-4 to enable VoLTE calls after initial attachment

### 7.6.1	Functional Description

There are two options to support RLOS services through the RLOS APN.

In the first option, the RLOS APN can lead to a portal that guides the user to various services, and after that, the user can establish the VoLTE session. It is assumed that the SIP signalling in this case may not acquire the QCI 5 that would typically be associated with an APN dedicated for IMS. It is up to the discretion of the operator to associate the necessary QCI for the RLOS APN in this option. Alternatively, if the UE attached to RLOS APN can be IMS authenticated, the UE can use IMS APN to re-(register) in IMS.

In the second option, the RLOS APN may lead directly to the IMS network to initiate the requested RLOS VoLTE service. The IMS network can be a regular IMS network provided that there is sufficient protection to ensure that RLOS users cannot acquire any service other than RLOS or can be a network dedicated to provide only RLOS.

It is up to operator policies to select the option it desires, and which alternative to adopt if the first option is selected.

### 7.6.2	Procedures

Editor's note:	This clause will describe the high-level procedures and information flows for the solution.

### 7.6.3	Impact on existing entities and interfaces

This solution requires a new RLOS indication be carried in an initial attach. The following nodes are impacted:

MME:	MME supports one the above options depending on configuration.

## 7.7	Solution #7 to key issues #IMS-1, #IMS-2, and #IMS-3: IMS support for RLOS users

### 7.7.1	Functional Description

Two RLOS scenarios are addressed in this solution depending on the operator preference:

-	In the first scenario, RLOS users are allowed to make VoLTE calls even if their IMS registration cant be successful.

-	In the second scenario, RLOS users are registered in IMS with identities specially assigned for RLOS that allow them to make VoLTE calls, and indeed can appear as operator owned IMS subscribers. However, these RLOS users have to be tagged by the IMS network as RLOS users for charging purposes and other enforcement policies to be performed by the network.

Both above scenarios must be explicitly identifiable by the IMS network, and distinguishable so that operator policies can be enforced in terms of what type of calls they can make, how long their temporary service lasts, charging, etc.

An operator may choose, based on policies, to support either one of them or both of them.

To address both of the above scenarios, a UE desiring access to RLOS must perform regular IMS registration and must include a feature tag to indicate its support for RLOS. Additionally, a UE desiring access to RLOS inserts a special tag in a session initiation request. Furthermore, a P-CSCF supporting RLOS must be configured with a list of RLOS supported by the operator.

For unauthenticated IMS users in scenario 1, both the P-CSCF and the S-CSCF shall support enabling unauthenticated users access to RLOS when the incoming session includes an RLOS tag as described above. Including a special tag by the UE to access RLOS in conjunction with a configured list of RLOS in the P-CSCF ensures that unauthenticated users can only access RLOS.

For successfully authenticated IMS users desiring access to RLOS according to scenario 2, the UE must include RLOS tag in the session initiation request (to bypass originating services). If an authenticated UE included the RLOS tag for a non-RLOS session, the P-CSCF shall remove the tag prior to forwarding the request to the S-CSCF. If an authenticated UE did not include the RLOS tag for an RLOS session, the P-CSCF may insert one subject to policy or reject the session. For authenticated users, the S-CSCF may include the RLOS tag for an RLOS session in the CDR, regardless if the UE included or not the tag in an RLOS session request.

Editior's note:	Enforcement of the inclusion of RLOS tag by P-CSCF as mentioned above needs to be detailed.

A P-CSCF supporting RLOS can also be used for regular IMS users.

The support for emergency calls is described in other solutions.

### 7.7.2	Procedures

A P-CSCF supporting RLOS shall be configured with the list of RLOS services.

A UE desiring access to RLOS must perform regular IMS registration and must include a feature tag to indicate its support for RLOS. If the user cannot be successfully authenticated, according to scenario 1, the P-CSCF shall create a temporary record and mark the user as "RLOS only user". The P-CSCF shall forward the request to a S-CSCF that supports RLOS. The S-CSCF creates a default profile for the UE, and the registration is accepted. For successfully authenticated IMS users, the I-CSCF must allocate to the UE a S-CSCF that supports RLOS at UE registration. This implies that the I-CSCF must understand a new capability related to RLOS and allocate a S-CSCF supporting it at registration time if needed.

A non-authenticated IMS UE desiring access to RLOS inserts a special RLOS tag in the session initiation request and includes the requested RLOS in the Request-URI of the session initiation request.  Upon receipt by a P-CSCF of such a request and if the P-CSCF supports RLOS, the P-CSCF shall accept the session if the requested RLOS service in the incoming R-URI is configured in the P-CSCF. The P-CSCF shall forward the incoming request to the S-CSCF allocated to the UE at registration.  The P-CSCF shall reject an incoming session without an RLOS tag for users marked as "RLOS only users".

For a successfully authenticated IMS UE desiring access to RLOS, according to scenario 2, the UE must include RLOS tag in the session initiation request (to bypass originating services). If a successfully authenticated UE included the RLOS tag for a non-RLOS session, the P-CSCF shall remove the tag prior to forwarding the request to the S-CSCF.  If a successfully authenticated UE did not include the RLOS tag for an RLOS session, the P-CSCF may insert one, subject to policy, or reject the session. The S-CSCF, based on policy, may include the RLOS tag in the CDR.

Upon receipt by a S-CSCF of an RLOS request and if the incoming request includes the RLOS tag, then the S-CSCF routes the session towards the destination. No originating services shall be permitted, i.e. the UE profile for authenticated users is not considered.

### 7.7.3	Impact on existing entities and interfaces

The solution impacts the following nodes:

UE:

A UE desiring access to RLOS must include a feature tag at IMS registration to indicate its support to RLOS.  A non-authenticated UE desiring access to RLOS inserts a special RLOS tag in the session initiation request. The UE will be pre-configured with the list of RLOS supported by the operator. An authenticated UE desiring access to RLOS may include the RLOS tag in the session initiation request.

P-CSCF:

The P-CSCF must understand the new RLOS feature tag.

The P-CSCF must be configured with the list of RLOS services to enforce originating sessions from unauthenticated RLOS users.

The P-CSCF must support incoming RLOS sessions from authenticated and unauthenticated users. The UE must mark unauthenticated RLOS users as "RLOS only user".

P-CSCF must be configured with a list of S-CSCF(s) that support RLOS for forwarding an incoming registration for unauthenticated RLOS users.

The P-CSCF must remove an RLOS tag from a non-RLOS session for authenticated UEs.

The P-CSCF, subject to policy, may insert the RLOS tag for an authenticated user that initiated an RLOS session and did not include one, or reject the session.

S-CSCF:

The S-CSCF shall route an incoming RLOS to its target for a authenticated and unauthenticated UEs. For authenticated UEs, no originating services shall be permitted, i.e the UE profile is not considered.

The S-CSCF may, based on policy, include in the CDR an RLOS tag to identify RLOS sessions for authenticated users regardless if the UE included or did not include one in the incoming request.

I-CSCF:

I-CSCF must understand the new capabilities related to RLOS and allocate a S-CSCF supporting RLOS at IMS registration when the IMS Register request includes the RLOS feature tag.

HSS:

HSS must support enabling a S-CSCF to declare RLOS as supporting capability.

The Gm interface shall be enhanced to enable conveying the RLOS indicator.

Cx interface shall support new RLOS capability.

## 7.8	Solution #8: Solution to key issues #IMS-4: Support of emergency services by UEs attached for RLOS

### 7.8.1	Functional Description

For a UE that is already attached for RLOS services, if the user would like to request an emergency service, the UE may detach from RLOS and then do emergency Attach, or the UE may decide to stay RLOS attached and to establish an emergency PDN connectivity towards the emergency APN, depending on the UE state (e.g. UE with or without USIM, authenticated or not authenticated).

Editor's note:	Whether one of the solutions is sufficient or both solutions should be supported is FFS. Details on what the UE behaviour is for a given UE state are FFS.

After the emergency attach is completed or after the emergency PDN connection is established, the UE shall perform IMS emergency registration.

### 7.8.2	Procedures

#### 7.8.2.1	UE performs detach from RLOS and then do emergency Attach

In this procedure:

-	For a UE without USIM, after the UE has been detached from the RLOS, the emergency EPS attach will be allowed only if behaviour d) of emergency bearer service support as specified in clause 4.13.12 in TS 23.401 [5] is supported.

-	For a UE with USIM but unauthenticated, after the UE has been detached from the RLOS, the emergency EPS attach will be allowed if behaviours c) or d) of emergency bearer service support as specified in clause 4.13.12 in TS 23.401 [5] are supported.

-	For a UE, which is authenticated but in limited service state, after detached from the RLOS, the emergency EPS attach will be allowed if behaviour b), c) or d) of emergency bearer service support as specified in clause 4.13.12 in TS 23.401 [5] are supported.

Figure 7.8.2.1-1: UE detach from RLOS and perform emergency Attach

1.	An UE is RLOS attached.

2.	The user would like to make an emergency call.

3.	UE performs Detach from RLOS.

4.	UE then perform emergency Attach as specified in TS 23.401 [5].

5.	UE performs IMS emergency registration as specified in TS 23.167 [8].

#### 7.8.2.2	UE stays RLOS attached, and establishes emergency PDN connection

In this procedure,

-	For a UE without USIM, when attached with RLOS, the establishment of an emergency PDN connection will be allowed only if behaviour d) of emergency bearer service support as specified in clause 4.13.12 in TS 23.401 [5] is supported.

-	For a UE with USIM but unauthenticated, when attached with RLOS, the establishment of an emergency PDN connection will be allowed if behaviours c) or d) of emergency bearer service support as specified in clause 4.13.12 in 3GPP TS 23.401 [5] are supported.

-	For a UE, which is authenticated but in limited service state, when attached with RLOS, if behaviour c) or d) of emergency bearer service support as specified in clause 4.13.12 in TS 23.401 [5] are supported, the establishment of an emergency PDN connection will be allowed.

-	If behaviour b) in clause 4.13.12 of TS 23.401 [5] is supported, as authentication is skipped during RLOS attach, the establishment of an emergency PDN connection will be rejected by the network.

Figure 7.8.2.2-1: RLOS attached UE initiate emergency service by establishing emergency PDN connection

1.	An UE is RLOS attached.

2.	The user would like to make an emergency call.

3.	UE initiates UE requested PDN Connectivity with request type = Emergency, as specified in TS 23.401 [5].

4.	After the emergency PDN connection is established, the UE performs IMS emergency registration, as specified in TS 23.167 [8].

### 7.8.3	Impact on existing entities and interfaces

The solution impacts the following nodes:

UE:

A UE is RLOS attach shall be able to detach from RLOS and then do emergency attach, or initiate emergency PDN connection when UE stays RLOS attached, and then perform emergency IMS registration.

MME:

Support of emergency PDN connection for an RLOS attached UE.

## 7.9	Solution #9: PLMN selection and radio access for RLOS

### 7.9.1	Functional Description

This solution is intended to resolve Key Issues EPC-1 and EPC-2.

The solution is described as follows:

-	The E-UTRAN broadcasts system information about the support of RLOS in the PLMN, e.g. as described in Solution #1 in clause 7.1.

-	If the user requests RLOS access and there are more than one PLMNs supporting RLOS, then the UE selects one of the available PLMNs either a) based on the user input solicited through the User Interface, or b) other ME configuration. Once a PLMN has been selected, this PLMN of the current serving cell is considered as the selected PLMN.

-	During the RRC connection establishment procedure, the UE indicates in the RRC signalling that the RRC connection is for RLOS access. Based on the RLOS indication in the RRC signalling, the RAN node applies the following:

-	The E-UTRAN may admit the radio access resources and the E-UTRAN may apply access control (e.g. in case of overload) for UEs accessing RLOS services; and

-	The E-UTRAN may select an appropriate MME serving the RLOS access to EPC. The RAN node forwards the NAS message from the UE to the selected MME.

Editor's note:	Whether specific MME selection is required for RLOS is FFS.

NOTE:	The RAN WGs will specify whether a new RRC Establishment Cause or another indication is used in the RRC signalling.

### 7.9.2	Procedures

Editor's note:	This clause will describe the high-level procedures and information flows for the solution.

### 7.9.3	Impact on existing entities and interfaces

Impacts to UE:

-	UE is able to process new SIB indication;

-	UE is able to indicate request for RLOS service at RRC and at NAS layer;

-	UE NAS layer may interact with the GUI for PLMN selection.

Impacts to RAN:

-	E-UTRAN broadcasts RLOS SIB indication;

-	E-UTRAN may apply RLOS-specific access control;

-	E-UTRAN node (e.g. eNB) is able to select CN node (e.g. MME) based on RLOS indication.

## 7.X	Solution #X: <Solution Title>

### 7.X.1	Functional Description

Editor's note:	This clause will describe the general description, assumption, and principles of the solution. The related key issues it solves will be mentioned.

### 7.X.2	Procedures

Editor's note:	This clause will describe the high-level procedures and information flows for the solution.

### 7.X.3	Impact on existing entities and interfaces

Editor's note:	This clause will describe the impacts to existing nodes or functionality and interfaces.

# 8	Evaluation

Editor's note:	This clause will provide a general evaluation of the solutions.

# 9	Conclusions

Editor's note:	This clause will capture agreed conclusions from the study.

Annex <A>:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-08 | SA2#122BIS | - | - | - | - | TR skeleton (approved in S2-175500) | 0.1.0 |
| 2017-08 | SA2#122BIS | S2-175501 | - | - | - | Scope of TR 23.715 | 0.1.0 |
| 2017-08 | SA2#122BIS | S2-176256 | - | - | - | Assumptions for PARLOS | 0.1.0 |
| 2017-08 | SA2#122BIS | S2-176210 | - | - | - | Key issue on mechanisms to access RLOS | 0.1.0 |
| 2017-08 | SA2#122BIS | S2-176258 | - | - | - | Key issue on RLOS support indication and related UE behaviour | 0.1.0 |
| 2017-08 | SA2#122BIS | S2-176259 | - | - | - | IMS Key Issues for RLOS | 0.1.0 |
| 2017-09 | - | - | - | - | - | MCC Editorial corrections | 0.1.1 |
| 2018-02 | SA2#125 | S2-181001 | - | - | - | Update of RLOS architectural assumptions | 0.2.0 |
| 2018-02 | SA2#125 | S2-181002 | - | - | - | Update to Key issue IMS#1 | 0.2.0 |
| 2018-02 | SA2#125 | S2-181003 | - | - | - | Update to Key issue EPC#3 | 0.2.0 |
| 2018-02 | SA2#125 | S2-180086 | - | - | - | Removal of Key issue EPC#5 | 0.2.0 |
| 2018-02 | SA2#125 | S2-181044 | - | - | - | Update of RLOS key issues | 0.2.0 |
| 2018-03 | SA2#126 | S2-182508 | - | - | - | Update of RLOS architectural assumptions | 0.3.0 |
| 2018-03 | SA2#126 | S2-182888 | - | - | - | Update of RLOS key issues | 0.3.0 |
| 2018-03 | SA2#126 | S2-182510 | - | - | - | Solution for Restricted IMS Local Operator Services | 0.3.0 |
| 2018-03 | SA2#126 | S2-182511 | - | - | - | Solution: EPC procedures for RLOS | 0.3.0 |
| 2018-05 | SA2#127 | S2-184021 | - | - | - | Update to Architectural Assumptions for PARLOS | 0.4.0 |
| 2018-05 | SA2#127 | S2-184022 | - | - | - | Correction on Architectural assumption for emergency service | 0.4.0 |
| 2018-05 | SA2#127 | S2-184027 | - | - | - | Solution : IMS procedures for RLOS | 0.4.0 |
| 2018-05 | SA2#127 | S2-184030 |  |  |  | RLOS service for the attached UE in limited state | 0.4.0 |
| 2018-05 | SA2#127 | S2-184523 |  |  |  | PLMN selection and radio access for RLOS | 0.4.0 |
| 2018-05 | SA2#127 | S2-184524 |  |  |  | Removing some ENs and update in Solution #4 | 0.4.0 |
| 2018-05 | SA2#127 | S2-184525 |  |  |  | How to complete RLOS service | 0.4.0 |
| 2018-05 | SA2#127 | S2-184526 | - | - | - | Solution for Key Issues EPC#3 and EPC#4 | 0.4.0 |
| 2018-05 | SA2#127 | S2-184527 | - | - | - | Solution #1 to key issues #IMS-1, #IMS-2, and #IMS-3 | 0.4.0 |
| 2018-05 | SA2#127 | S2-184639 | - | - | - | Solution to Key issue #IMS-4: Support of emergency services by UEs attached for RLOS | 0.4.0 |