3GPP TSG-RAN1 Meeting #88bis	R1-1706289

# Spokane, USA 3rd - 7th April 2017

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 0885 | rev | - | Current version: | 14.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on tables used for SCI format 1 in 36.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_V2X-Core |  |  |  |  |  | Date: |  |  | 2017-04-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Table index 14.2.1-1 is used for two tables, and one of them is not used in the corresponding UE behaviour description. Also, the second table for the indicated value m mentions SCI but the value is indicated by DCI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Indices of the two tables in 14.2.1 are corrected and the one for the indicated value m is used as reference in the UE behavior description with a corrected table title. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE behaviour regarding SCI setting is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged parts are omitted >

For sidelink transmission mode 3,

-	The UE shall determine the subframes and resource blocks for transmitting SCI format 1 as follows:

-	SCI format 1 is transmitted in two physical resource blocks per slot in each subframe where the corresponding PSSCH is transmitted.

-	If the UE receives in subframe n DCI format 5A with the CRC scrambled by the SL-V-RNTI, one transmission of PSCCH is in the PSCCH resource  (described in subclause 14.2.4) in subframe  where  is the value indicated by "Lowest index of the sub-channel allocated to the initial transmission" associated with the configured sidelink grant (described in [8]) and  is the smallest integer which is larger than or equal to 4+m and fulfils the condition that subframe is included in  which is determined by subclause 14.1.5. The value m is indicated by ‘Subframe offset’ field in the corresponding DCI format 5A according to Table 14.2.1-1 if this field is present and m=0 otherwise.

-	If “Time gap between initial transmission and retransmission" in the configured sidelink grant (described in [8]) is not equal to zero, another transmission of PSCCH is in the PSCCH resource  in subframe  where  is the value indicated by "Time gap between initial transmission and retransmission" field in the configured sidelink grant.  corresponds to the value  determined by the procedure in subclause 14.1.1.4C with the RIV set to the value indicated by "Frequency resource location of the initial transmission and retransmission" field in the configured sidelink grant.

-	If the UE receives in subframe n DCI format 5A with the CRC scrambled by the SL-V-SPS-RNTI, the UE shall consider the received DCI information as a valid sidelink semi-persistent activation or release only for the SPS configuration indicated by the SL SPS configuration index field. If the received DCI activates an SL SPS configuration, one transmission of PSCCH is in the PSCCH resource  (described in subclause 14.2.4) in subframe  where  is the value indicated by "Lowest index of the sub-channel allocated to the initial transmission" associated with the configured sidelink grant (described in [8]) and  is the smallest integer which is larger than or equal to 4+m and fulfils the condition that subframe is included in  which is determined by subclause 14.1.5. The value m is indicated by ‘Subframe offset’ field in the corresponding DCI format 5A according to Table 14.2.1-1 if this field is present and m=0 otherwise.

-	If “Time gap between initial transmission and retransmission" in the configured sidelink grant (described in [8]) is not equal to zero, another transmission of PSCCH is in the PSCCH resource  in subframe  where  is the value indicated by "Time gap between initial transmission and retransmission" field in the configured sidelink grant.  corresponds to the value  determined by the procedure in subclause 14.1.1.4C with the RIV set to the value indicated by "Frequency resource location of the initial transmission and retransmission" field in the configured sidelink grant.

-	The UE shall set the contents of the SCI format 1 as follows:

-	the UE shall set the Modulation and coding scheme as indicated by higher layers.

-	the UE shall set the “Priority” field according to the highest priority among those priority(s) indicated by higher layers corresponding to the transport block.

-	the UE shall set the Time gap between initial transmission and retransmission field, the Frequency resource location of the initial transmission and retransmission field, and the Retransmission index field such that the set of time and frequency resources determined for PSSCH according to subclause 14.1.1.4C is in accordance with the PSSCH resource allocation indicated by the configured sidelink grant.

-	the UE shall set the Resource reservation to zero.

-	Each transmission of SCI format 1 is transmitted in one subframe and two physical resource blocks per slot of the subframe.

-	The UE shall randomly select the cyclic shift  among {0, 3, 6, 9} in each PSCCH transmission.

For sidelink transmission mode 4,

-	The UE shall determine the subframes and resource blocks for transmitting SCI format 1 as follows:

-	SCI format 1 is transmitted in two physical resource blocks per slot in each subframe where the corresponding PSSCH is transmitted.

-	If the configured sidelink grant from higher layer indicates the PSCCH resource in subframe , one transmission of PSCCH is in the indicated PSCCH resource m (described in subclause 14.2.4) in subframe .

-	If "Time gap between initial transmission and retransmission" in the configured sidelink grant (described in [8]) is not equal to zero, another transmission of PSCCH is in the PSCCH resource  in subframe  where  is the value indicated by "Time gap between initial transmission and retransmission" field in the configured sidelink grant,  corresponds to the value  determined by the procedure in subclause 14.1.1.4C with the RIV set to the value indicated by "Frequency resource location of the initial transmission and retransmission" field in the configured sidelink grant.

-	the UE shall set the contents of the SCI format 1 as follows:

-	the UE shall set the Modulation and coding scheme as indicated by higher layers.

-	the UE shall set the “Priority” field according to the highest priority among those priority(s) indicated by higher layers corresponding to the transport block.

-	the UE shall set the Time gap between initial transmission and retransmission field, the Frequency resource location of the initial transmission and retransmission field, and the Retransmission index field such that the set of time and frequency resources determined for PSSCH according to subclause 14.1.1.4C is in accordance with the PSSCH resource allocation indicated by the configured sidelink grant.

-	the UE shall set the Resource reservation field according to table 14.2.1-2 based on indicated value X, where X is equal to the Resource reservation interval provided by higher layers divided by 100.

-	Each transmission of SCI format 1 is transmitted in one subframe and two physical resource blocks per slot of the subframe.

-	The UE shall randomly select the cyclic shift  among {0, 3, 6, 9} in each PSCCH transmission.

Table 14.2.1-1: Mapping of DCI format 5A offset field to indicated value m

| Subframe offset field in DCI format 5A | Indicated value m |
| --- | --- |
| ‘00’ | 0 |
| ‘01’ | 1 |
| ‘10’ | 2 |
| ‘11’ | 3 |

Table 14.2.1-2: Determination of the Resource reservation field in SCI format 1

| Resource reservation field in SCI format 1 | Indicated value X | Condition |
| --- | --- | --- |
| ‘0001’, ‘0010’, …, ‘1010’ | Decimal equivalent of the field | The higher layer decides to keep the resource for the transmission of the next transport block and the value X meets . |
| ‘1011’ | 0.5 | The higher layer decides to keep the resource for the transmission of the next transport block and the value X is 0.5. |
| ‘1100’ | 0.2 | The higher layer decides to keep the resource for the transmission of the next transport block and the value X is 0.2. |
| ‘0000’ | 0 | The higher layer decides not to keep the resource for the transmission of the next transport block. |
| ‘1101’, ‘1110’, ‘1111’ | Reserved |  |

#### < Unchanged parts are omitted >