3GPP TSG-RAN WG4 Meeting #83	R4-1705864

# Hangzhou, P. R. China, 15th – 19th May, 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 4419 | rev | 1 | Current version: | 14.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the additional SE for Band 41 UE in rel-14 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_B41_UE_PC2-Core |  |  |  |  |  |  | Date: |  | 2017-05-15 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The A-SE requirement for Band 41 with NS_04 is inconsistent with FCC regulation. The measurement BW in 3GPP is define with 1MHz to measure 0MHz offset from the lower edge of Band 41. In case, some UEs do not satisfy the A-SE requirement evenif apply A-MPR requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Additional SE requirements have reflected FCC reule in 27.53, but the FCC regulation was changed as below “Therefore, we will change the rules to specify that, except for the 2495–2496 MHz band, in the 1 megahertz bands immediately outside and adjacent to the frequency block under use, a resolution bandwidth of at least 2 percent of the fundamental emission be allowed to measure compliance. In the 2495–2496 MHz band, the existing resolution bandwidth requirement of at least 1 percent would still apply.” So, RAN4 follow the changed the measured frequency band for A-SE in 6.6.3 - A-SE requirements - Changed the frequency band from 2490.5 to 2496MHz will be reduced as 2490.5 <f< 2495. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Commertial Band 41 UE with power class3 and power class 2 do not satisfy the A-SE requirements even if UE received NS_04 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR … CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS/TR ... CR ...36.521-1 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of changed request>

6.6.3.3.19	Minimum requirement (network signalled value "NS_04")

When "NS 04" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.19-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.19-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 10, 15, 20 MHz |  |
| 2490.5 ≤ f < 2495 | -13 | 1 MHz |
| 0 < f < 2490.5 | -25 | 1 MHz |

<End of changed request>