3GPP TSG-RAN WG4 Meeting #82bis		R4-1703929

Spokane, WA, US, 3 - 7 April, 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 37.114 | CR | 0025 | rev |  | Current version: | 13.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 37.114: Isolation of Band 46 and NB-IoT from the AAS BS specification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Ericsson, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AAS_BS_LTE_UTRA-Core/Perf |  |  |  |  |  |  | Date: |  | 2017-03-16 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | During RAN4#81 the following was agreed, as captured in the chairman’s notes in R4-1610387: not include B46, not include NB-IoT AAS BS type, update for other new bands and latest references. This Cat. B CR implements isolation of the NB-IoT and Band 46 from the AAS EMC specification. The receiver exclusion bands notes are based on the EUTRA CR in R4-160654 MSR CR R4-160655. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated scope - Missing refernece added - New definitions added - Abbreviations section aligned with other specifications - Test conditions section updated with the Band 46 isolation - Minor editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment among AAS BS core and test specifications would exist. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 2, 3, 4, 6, 7, 8, 9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 37.105, CR 0030, 0031 |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 37.145-1, CR xxxx, 0026 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

---------------------------------- Modified section ----------------------------------

# 1	Scope

The present document covers the assessment of UTRA TDD, UTRA FDD, E-UTRA and Multi-Standard Radio (MSR) Active Antenna Systems Base Stations in respect of Electromagnetic Compatibility (EMC).

NOTE:	Whenever the UTRA AAS BS is referred in the following section, UTRA TDD and UTRA FDD shall be considered.

The present document specifies the applicable test conditions, performance assessment and performance criteria for base stations and associated ancillary equipment in the following categories:

-	Active Antenna System Base Station for UTRA, E-UTRA and MSR meeting the conducted requirements of 3GPP TS 37.105 [2], with conformance demonstrated by compliance to 3GPP TS 37.145-1 [3],

- 	Active Antenna System Base Station for E-UTRA, UTRA and MSR meeting the OTA requirements of 3GPP TS 37.105 [2], with conformance demonstrated by compliance to 3GPP TS 37.145-2 [10].

The present document does not specify test conditions, performance assessment and performance criteria for the Narrow-Band Internet of Things (NB-IoT) in band, NB-IoT guard band, or standalone NB-IoT operation, for AAS BS in single RAT E-UTRA operation as defined in 3GPP TS 36.113 [6], or for AAS BS in MSR operation using E-UTRA as defined in 3GPP TS 37.113 [4].

The present document does not specify test conditions, performance assessment and performance criteria for Band 46 operation as it is not supported by AAS BS.

The scope of the present document is twofold:

- 	Requirement, procedures and values of an AAS BS with TAB connectors for every transceiver unit at the transceiver array boundary (TAB), subject to conducted requirements,

- 	Requirements, procedures and values of an AAS Base Station without TAB connectors are not covered by the Rel-13 version of this document and are FFS.

The environment classification used in the present document refers to the residential, commercial and light industrial environment classification used in IEC 61000-6-1 [7] and IEC 61000-6-3 [8].

The EMC requirements have been selected to ensure an adequate level of compatibility for apparatus at residential, commercial and light industrial environments. The levels, however, do not cover extreme cases which may occur in any location but with low probability of occurrence.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications"

[2]	3GPP TS 37.105: "Active Antenna System (AAS) Base Station (BS) transmission and reception"

[3]	3GPP TS 37.145-1: "Active Antenna System (AAS) Base Station (BS) conformance testing; Part 1: Conducted conformance testing"

[4]	3GPP TS 37.113: "E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) Electromagnetic Compatibility (EMC)"

[5]	3GPP TS 25.113: "Base Station (BS) and repeater ElectroMagnetic Compatibility (EMC)"

[6]	3GPP TS 36.113: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) and repeater ElectroMagnetic Compatibility (EMC)"

[7]	IEC 61000-6-1: 2016: "Electromagnetic compatibility (EMC) - Part 6-1: Generic standards - Immunity standard for residential, commercial and light-industrial environments"

[8]	IEC 61000-6-3: 2006/AMD1:2010: "Electromagnetic compatibility (EMC) - Part 6-3: Generic standards - Emission standard for residential, commercial and light industrial environments"

[9]	3GPP TR 37.842: "Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)"

[10]	3GPP TS 37.145-2: "Active Antenna System (AAS) Base Station (BS) conformance testing; Part 2: radiated conformance testing"

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1], 3GPP TS 37.113 [4] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1] or 3GPP TS 37.113 [4].

NOTE:	Multi-word definitions are treated as linguistic expressions and printed in italic font throughout this requirement specification. Linguistic expressions may not be split and are printed in their entirety.

active antenna system base station: BS system which combines an Antenna Array with an Active Transceiver Unit Array and a Radio Distribution Network (RDN)

antenna array: group of radiating elements characterized by the geometry and the properties of the array elements

NB-IoT In-band operation: NB-IoT is operating in-band when it utilizes the resource block(s) within a normal E-UTRA carrier

NB-IoT guard band operation: NB-IoT is operating in guard band when it utilizes the unused resource block(s) within a E-UTRA carrier’s guard-band.

NB-IoT standalone operation: NB-IoT is operating standalone when it utilizes its own spectrum, for example the spectrum currently being used by GERAN systems as a replacement of one or more GSM carriers, as well as scattered spectrum for potential IoT deployment.

radio distribution network: linear passive network which distributes the RF power generated by the transceiver unit array to the antenna array, and/or distributes the radio signals collected by the antenna array to the transceiver unit array

NOTE:	In the case when the active transceiver units are physically integrated with the array elements of the antenna array, the radio distribution network is a one-to-one mapping.

TAB connector: transceiver array boundary connector

transceiver array boundary: conducted interface between the transceiver unit array and the composite antenna

## 3.2	Symbols

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AAS	Active Antenna System

AAS BS	AAS Base Station

EMC	ElectroMagnetic Compatibility

MSR	Multi-Standard Radio

NB-IoT	Narrowband – Internet of Things

RDN	Radio Distribution Network

TAB	Transceiver Array Boundary

# 4	Test conditions

Where the AAS BS has multiple TAB connectors which are declared to be equivalent then it is sufficient to perform EMC tests on a single representative TAB connector. For the definition of the TAB connector equivalence declaration (D6.70), refer to 3GPP TS 37.145-1 [3].

EMC test shall not be performed with the AAS BS antenna array radiating, all TAB connectors shall be disconnected from the radio distribution network (RDN)/antenna array as specified in 3GPP TS 37.105 [2] and terminated in an appropriate load impedance. For the description of the general AAS BS radio architecture and relations between the RDN/antenna array and the Transceiver Array Boundary, refer to 3GPP TR 37.842 [9].

For UTRA AAS BS the test conditions from 3GPP TS 25.113 [5] apply.

For E-UTRA AAS BS the test conditions from 3GPP TS 36.113 [6] apply.

NOTE: 	The receiver exclusion band defined in 3GPP TS 36.113 [6] for Band 46 operation is not applicable for AAS BS, as the Band 46 operation is not supported by AAS BS.

For MSR AAS BS the test conditions from 3GPP TS 37.113 [4] apply.

NOTE: 	The receiver exclusion band defined in 3GPP TS 37.113 [4] for Band 46 operation is not applicable for AAS BS, as the Band 46 operation is not supported by AAS BS.

# 5	Performance assessment

For UTRA AAS BS the performance assessment from 3GPP TS 25.113 [5] applies.

For E-UTRA AAS BS the performance assessment from 3GPP TS 36.113 [6] applies.

For MSR AAS BS the performance assessment from 3GPP TS 37.113 [4] applies.

# 6	Performance criteria

For UTRA AAS BS the performance criteria from 3GPP TS 25.113 [5] apply.

For E-UTRA AAS BS the performance criteria from 3GPP TS 36.113 [6] apply.

For MSR AAS BS the performance criteria from 3GPP TS 37.113 [4] apply.

# 7	Applicability overview

For UTRA AAS BS the applicability overview from 3GPP TS 25.113 [5] applies.

For E-UTRA AAS BS the applicability overview from 3GPP TS 36.113 [6] applies.

For MSR AAS BS the applicability overview from 3GPP TS 37.113 [4] applies.

# 8	Emission

For UTRA AAS BS the emission requirement from 3GPP TS 25.113 [5] applies.

For E-UTRA AAS BS the emission requirement from 3GPP TS 36.113 [6] applies.

For MSR AAS BS the emission requirement from 3GPP TS 37.113 [4] applies.

# 9	Immunity

For UTRA AAS BS the immunity requirement from 3GPP TS 25.113 [5] applies.

For E-UTRA AAS BS the immunity requirement from 3GPP TS 36.113 [6] applies.

For MSR AAS BS the immunity requirement from 3GPP TS 37.113 [4] applies.

---------------------------------- End of modified section ----------------------------------