3GPP TSG-RAN WG4 Meeting #83	         	R4-1705801

Hangzhou, China, 15 - 19 May, 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 37.145-1 | CR | 0065 | rev |  | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 37.145-1: MB-MSR update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Ericsson, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AAS_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2017-04-20 |
| Category: | A |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This is Cat. A CR for the Rel-13 CR in R4-1704348 agreed in RAN4#82bis. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Missing definition of the inter-band gap was added - ATC5a and ATC5b test configurations generation description updated - Basic Limits for MSR Band Categories 1, 2, 3 descriptions updated - Basic Limits for E-UTRA description updated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Terminology usage misalignment among MSR and AAS specifications would exists. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 4.11.2.8.1.2, 4.11.2.8.2.2, 6.6.4.5.2.1, 6.6.5.5.2, 6.6.6.5.3, 6.6.5.5.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 37.105, CR 0033 |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

---------------------------------- Modified section ----------------------------------

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

active antenna system base station: base station system which combines an Antenna Array with an Active transceiver unit array and a Radio Distribution Network

active receiver unit: number of active receivers is the same as the [number of receiver diversity branches] to which compliance is declared for clause 8 performance requirements

active transmitter unit: transmitter unit which is ON, and has the ability to send modulated data streams that are parallel and distinct to those sent from other transmitter units to one or more TAB connectors at the transceiver array boundary

band category: group of operating bands for which the same MSR scenarios apply

Base Station RF Bandwidth: bandwidth in which a base station transmits and/or receives single or multiple carrier(s) and/or RATs simultaneously within a supported operating band

NOTE:	In single carrier operation, the Base Station RF Bandwidth is equal to the channel bandwidth.

Base Station RF Bandwidth edge: frequency of one of the edges of the Base Station RF Bandwidth

basic limit: emissions limit taken from the non AAS specifications that is converted into a per TAB connector TX cell group AAS emissions limit by scaling

beam: main lobe of a radiation pattern from an AAS BS

NOTE:	For certain AAS antenna array, there may be more than one beam.

beam centre direction: direction equal to the geometric centre of the -3 dB EIRP contour of the beam

beam direction pair: data set consisting of  the beam centre direction and the related beam peak direction

beam peak direction: direction where the maximum EIRP is supposed to be found

beamwidth: angles describing the major and minor axes of an ellipsoid closest fit to an essentially elliptic half-power contour of a beam

carrier: modulated waveform conveying the physical channels

carrier aggregation: aggregation of two or more E-UTRA component carriers in order to support wider transmission bandwidths

channel bandwidth: RF bandwidth supporting a single RF carrier with the transmission bandwidth configured in the uplink or downlink of a cell

NOTE1:	The channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements.

NOTE 2:	For UTRA FDD, the channel bandwidth is the nominal channel spacing specified in 3GPP TS 25.104 [2], For UTRA TDD 1,28 Mcps option, the channel bandwidth is the nominal channel spacing specified in 3GPP TS 25.105 [3].

NOTE 3:	For E-UTRA, the channel bandwidths are specified in 3GPP TS 36.104 [4].

code domain power: part of the mean power which correlates with a particular (OVSF) code channel in a UTRA signal

NOTE:	The sum of all powers in the code domain equals the mean power in a bandwidth of (1+ ) times the chip rate of the radio access mode.

contiguous spectrum: spectrum consisting of a contiguous block of spectrum with no sub-block gap(s)

downlink operating band: part of the (FDD) operating band designated for downlink

EIRP accuracy directions set: beam peak directions for which the EIRP accuracy requirement is intended to be met.

NOTE:	The beam peak directions are related to a corresponding contiguous range or discrete list of beam centre directions by the beam direction pairs included in the set.

equivalent isotropic radiated power: equivalent power radiated from an isotropic directivity device producing the same field intensity at a point of observation as the field intensity radiated in the direction of the same point of observation by the discussed device

NOTE:	Isotropic directivity is equal in all directions (0 dBi).

equivalent isotropic sensitivity: sensitivity for an isotropic directivity device equivalent to the sensitivity of the discussed device exposed to an incoming wave from a defined AoA

NOTE1:	The sensitivity is the minimum received power level at which a RAT specific requirement is met.

NOTE 2:	Isotropic directivity is equal in all directions (0 dBi).

inter-band gap: frequency gap between two supported consecutive operating bands

Inter RF Bandwidth gap: frequency gap between two consecutive Base Station RF Bandwidths that are placed within two supported operating bands

maximum carrier output power per TAB connector: mean power level measured on a particular carrier at the array boundary antenna connectors, during the transmitter ON period in a specified reference condition

maximum throughput: maximum achievable throughput for a reference measurement channel

multi-band requirements: requirements applying per one single operating band with exclusion bands or other multi-band provisions as defined for each requirement

multi-band TAB connector : TAB connector supporting operation in multiple operating bands through common active electronic components(s)

NOTE:	For common RX and TX TAB connectors, the definition applies where common active electronic components are in both the receive path and in the transmit path. Combinations with active shared components in only Rx or Tx are TBD.

non-AAS BS: BS  conforming to one of  the RF requirement specifications 3GPP TS 25.104 [2], 3GPP TS 25.105 [3], 3GPP TS 36.104 [4] or 3GPP TS37.104 [5]

non-contiguous spectrum: spectrum consisting of two or more sub-blocks separated by sub-block gap(s)

OTA sensitivity directions declaration: set of manufacturer declarations comprising one or more EIS values (with related RAT and channel bandwidth), and the directions where it (they) applies

NOTE:	All the directions apply to all the EIS values in an OSDD.

Radio Bandwidth: frequency difference between the upper edge of the highest used carrier and the lower edge of the lowest used carrier

radio distribution network: passive network which distributes radio signals generated by the transceiver unit array to the antenna array, and/or distributes the radio signals collected by the antenna array to the transceiver unit array

NOTE:	In the case when the active transceiver units are physically integrated with the array elements of the antenna array, the radio distribution network is a one-to-one mapping

rated carrier output power per TAB connector: mean power level associated with a particular carrier the manufacturer has declared to be available at the TAB connector, during the transmitter ON period in a specified reference condition

rated total output power per TAB connector: mean power level associated with a particular operating band the manufacturer has declared to be available at the TAB connector, during the transmitter ON period in a specified reference condition

receiver target: angles of arrival in which reception is performed

receiver target redirection range: union of all the sensitivity RoAoA achievable through redirecting the receiver target related to the OSDD

receiver target reference direction: direction inside the receiver target redirection range declared by the manufacturer for conformance testing. For an OSDD without receiver target redirection range, this is a direction inside the sensitivity RoAoA

reference beam direction: declared beam direction pair, including reference beam centre direction and reference beam peak direction where the reference beam peak direction is the direction for the intended maximum EIRP within the EIRP accuracy compliance directions set

sensitivity RoAoA: RoAoA within which the declared EIS(s) of an OSDD is intended to be achieved at any instance of time for a specific AAS BS direction setting

single band requirements: requirements applying per one single operating band without exclusion bands or other multi-band provisions

single band TAB connector: TAB connector supporting operation either in a single operating band only, or in multiple operating bands but without any common active electronic component(s)

sub-block: one contiguous allocated block of spectrum for use by the same Base Station

NOTE	There may be multiple instances of sub-blocks within an Base StationRF Bandwidth.

sub-block gap: frequency gap between two consecutive sub-blocks within an Base Station RF Bandwidth, where the RF requirements in the gap are based on co-existence for un-coordinated operation

TAB connector: transceiver array boundary connector

TAB connectors beam forming group: Group of TAB connectors associated with an EIRP beam declaration, comprising of the complete set of TAB connectors from which a declared beam is transmitted

TAB connector RX min cell group: operating band specific declared group of TAB connectors to which requirements are applied

NOTE:	The group corresponds to the group of TAB connectors which are responsible for receiving a cell when the AAS BS setting corresponding to the declared minimum number of cells with transmission on all TAB connectors supporting an operating band, but its existence is not limited to that condition.

TAB connector TX min cell group: operating band specific declared group of TAB connectors to which requirements are applied.

NOTE:	The group corresponds to the group of TAB connectors which are responsible for transmitting a cell when the AAS BS setting corresponding to the declared minimum number of cells with transmission on all TAB connectors supporting an operating band, but its existence is not limited to that condition.

throughput: number of payload bits successfully received per second for a reference measurement channel in a specified reference condition

transceiver array boundary: conducted interface between the transceiver unit array and the composite antenna

transmission bandwidth: bandwidth of an instantaneous E-UTRA transmission from a UE or BS, measured in Resource Block units

transmitter OFF period: time period during which the transmitter is scheduled not to transmit

NOTE:	For AAS BS, this definition applies per TAB connector and operating band.

transmitter ON period: time period during which the transmitter is transmitting data and/or reference symbols

NOTE:	For AAS BS, this definition applies per TAB connector and operating band.

transmitter transient period: time period during which the transmitter is changing from the OFF period to the ON period or vice versa

NOTE:	For AAS BS, this definition applies per TAB connector and operating band.

uplink operating band: part of the (FDD) operating band designated for uplink

---------------------------------- Next modified section ----------------------------------

4.11.2.8.1.2	ATC5a generation

ATC5a is based on re-using the existing test configurations applicable per band on multi-band TAB connectors. ATC5a is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum Base Station RF Bandwidth (see table 4.10-1, D6.17) of the multi-band TAB connector.

-	The number of carriers of each supported operating band shall be the declared maximum number of supported carriers by the multi-band TAB connector in each band (see table 4.10-1, D6.25). Carriers shall first be placed at the outermost edges of the declared maximum Radio Bandwidth (see table 4.10-1, D6.16) Additional carriers shall next be placed at the edges of the Base Station RF Bandwidths, if possible.

-	The allocated Base Station RF Bandwidth of the outermost bands shall be located at the outermost edges of the declared maximum Radio Bandwidth (see table 4.10-1, D6.16).

-	Each concerned band shall be considered as an independent band and thecarrier placement in each band shall be according to the test configuration referenced in Table 4.11.2.8.1.2-1, where the declared parameters for multi-band operation shall apply. The mirror image of the single band test configuration shall be used in the highest band being tested for the TAB connector.

-	If a multi-band TAB connector supports three carriers only, two carriers shall be placed in one band according to the relevant test configuration while the remaining carrier shall be placed at the edge of the maximum Radio Bandwidth (DUID9) in the other band.

-	If the sum of the base Station RF bandwidths of each of the supported operating bands is greater than the declared Radio Bandwidth (see table 4.10-1, D6.16) for the declared band combinations (see table 4.10-1, D6.41) of the TAB connector then repeat the steps above for test configurations where the Base Station RF Bandwidth of one of the operating band shall be reduced so that the declared Radio Bandwidth (see table 4.10-1, D6.16) of the TAB connector is not exceeded and vice versa.

-	If the sum of the maximum number of supported carrier of each supported operating bands for the multi-band TAB connector is larger than the declared total number of supported carriers for the declared band combinations (see table 4.10-1, D6.42) of the AAS BS, repeat the steps above for test configurations where in each test configuration the number of carriers of one of the operating band shall be reduced so that the total number of supported carriers is not be exceeded and vice versa.

Table 4.11.2.8.1.2-1: The applicability of test configuration for carrier placement in each band

| BC | CSA 1 | CSA 2 | CSA 3 | CSA 4 | CSA 5 |
| --- | --- | --- | --- | --- | --- |
| BC1 | ATC1a | ATC2a | ATC3a | ATC1a | ATC2a |
| BC2 | ATC1a | ATC2a | ATC3a | ATC1a | ATC2a |
| BC3 | ATC1b | ATC2a | ATC3b | ATC1b | ATC2a |

---------------------------------- Next modified section ----------------------------------

4.11.2.8.2.2	ATC5b generation

ATC5b is based on re-using the existing test configurations applicable per band on multi-band TAB connectors. ATC5b is constructed using the following method:

-	The Base Station RF Bandwidth of each supported operating band shall be the declared maximum Base Station RF Bandwidth (see table 4.10-1, D6.16) of the multi-band TAB connector.

-	The allocated Radio Bandwidth of the outermost bands shall be located at the outermost edges of the declared maximum Radio Bandwidth (see table 4.10-1, D6.16).

-	The maximum number of carriers is limited to two per band. Carriers shall be placed at the outermost edges of the declared maximum Radio Bandwidth (see table 4.10-1, D6.16).

-	Each concerned band shall be considered as an independent band and the carrier placement in each band shall be according to the test configuration referenced in Table 4.11.2.8.2.2-1, where the declared parameters for multi-band operation shall apply. Narrowest supported E-UTRA channel bandwidth shall be used in the test configuration. The mirror image of the single band test configuration shall be used in the highest band being tested for the TAB connector.

-	For AAS BS supporting CSA4 in the band, if a multi-band TAB connector supports three carriers only, two carriers shall be placed in one band according to AUTC2 while the remaining carrier shall be placed at the edge of the Maximum Base Station RF Bandwidth in the other band.

-	If the sum of the base Station RF bandwidths of each of the supported operating bands is greater than the declared Radio Bandwidth (see table 4.10-1, D6.16) for the declared band combinations (see table 4.10-1, D6.41) of the TAB connector then repeat the steps above for test configurations where the Base Station RF Bandwidth of one of the operating band shall be reduced so that the declared Radio Bandwidth (see table 4.10-1, D6.16) of the TAB connector is not exceeded and vice versa.

Table 4.11.2.8.2.2-1: The applicability of test configuration for carrier placement in each band

| BC | CSA 1 | CSA 2 | CSA 3 | CSA 4 | CSA 5 |
| --- | --- | --- | --- | --- | --- |
| BC1 | ANTC1a | ANTC2 | ANTC3a | ANTC1 | ANTC2 |
| BC2 | ANTC1a | ANTC2 | ANTC3a | ANTC1 | ANTC2 |
| BC3 | ATC1b | ANTC2 | ANTC3a | N/A | ANTC2 |

---------------------------------- Next modified section ----------------------------------

6.6.4.5.2.1	UTRA FDD

The basic limit is specified in tables 6.6.4.5.2.1-1 to 6.6.4.5.2.1-11 for the appropriate PRated,c,sys, where:

-	f is the separation between the carrier frequency and the nominal -3 dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the carrier frequency and the centre of the measurement filter;

-	f_offsetmax is either 12.5 MHz or the offset to the UMTS Tx band edge as defined in clause 3.4.1, whichever is the greater.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

Inside any Inter RF Bandwidth gap s with Wgap < 20 MHz for BS operating in multiple bands, emissions shall not exceed the cumulative sum of the test requirements specified at the Base Station RF Bandwidth edges on each side of the Inter RF Bandwidth gap. The basic limit for Base Station RF Bandwidth edge is specified in tables 6.6.4.5.2.1-1 to 6.6.4.5.2.1-11 below, where in this case:

-	f is equal to 2.5 MHz plus the separation between the Base Station RF Bandwidth edge frequency and the nominal -3dB point of the measuring filter closest to the Base Station RF Bandwidth edge.

-	f_offset is equal to 2.5 MHz plus the separation between the Base Station RF Bandwidth edge frequency and the centre of the measuring filter.

-	f_offsetmax is either 12.5 MHz or the offset to the UMTS Tx band edge as defined in section 5.2, whichever is the greater.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

For a multi-band TAB connector, the operating band unwanted emission limits apply also in a supported operating band without any carrier transmitted, in the case where there are carrier(s) transmitted in another supported operating band. In this case, no cumulative limit is applied in the inter-band gap between a supported downlink operating band with carrier(s) transmitted and a supported downlink operating band without any carrier transmitted and

-	In case the inter-band gap between a downlink band with carrier(s) transmitted and a downlink band without any carrier transmitted is less than 20 MHz, f_offsetmax shall be the offset to the frequency 10 MHz outside the outermost edges of the two downlink operating bands and the operating band unwanted emission limit of the band where there are carriers transmitted, as defined in the tables of the present subclause, shall apply across both downlink bands.

-	In other cases, the operating band unwanted emission limit of the band where there are carriers transmitted, as defined in the tables of the present subclause for the largest frequency offset (fmax), shall apply from 10 MHz below the lowest frequency, up to 10 MHz above the highest frequency of the downlink operating band without any carrier transmitted.

Inside any sub-block gap for a TAB connector operating in non-contiguous spectrum, the measurement results shall not exceed the cumulative sum of the test requirements specified for the adjacent sub blocks on each side of the sub block gap. The basic limit for each sub block is specified in tables 6.6.4.5.2.1-1 to 6.6.4.5.2.1-11 below, where in this case:

-	f is equal to 2.5 MHz plus the separation between the sub block edge frequency and the nominal -3 dB point of the measuring filter closest to the sub block edge.

-	f_offset is equal to 2.5 MHz plus the separation between the sub block edge frequency and the centre of the measuring filter.

-	f_offsetmax is equal to the sub block gap bandwidth minus half of the bandwidth of the measuring filter plus 2.5 MHz.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

---------------------------------- Unchanged part omitted----------------------------------

---------------------------------- Next modified section ----------------------------------

##### 6.6.5.5.2	Basic Limits for MSR Band Categories 1 and 3

For an AAS BS of Wide Area BS class operating in Band Category 1 or Band Category 3, the requirement applies outside the Base Station RF Bandwidth edges. In addition, for a Wide Area BS operating in non-contiguous spectrum, it applies inside any sub-block gap. In addition, for an AAS BS of Wide Area BS class operating in multiple bands, it applies inside any Inter RF Bandwidth gap.

For an AAS BS of Medium Range BS class operating in Band Category 1 the requirement applies outside the Base Station RF Bandwidth edges. In addition, for a Medium Range BS operating in non-contiguous spectrum, it applies inside any sub-block gap. In addition, for an AAS BS of Medium Range BS class operating in multiple bands, it applies inside any Inter RF Bandwidth gap.

For an AAS BS of Local Area BS class operating in Band Category 1 the requirement applies outside the Base Station RF Bandwidth edges. In addition, for a Local Area BS operating in non-contiguous spectrum, it applies inside any sub-block gap. In addition, for an AAS BS of Local Area BS class operating in multiple bands, it applies inside any Inter RF Bandwidth gap.

Outside the Base Station RF Bandwidth edges, emissions shall not exceed the maximum levels specified in tables 6.6.2.5.1-1 to 6.6.2.5.1-4 below, where:

-	f is the separation between the Base Station RF Bandwidth edge frequency and the nominal -3 dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the Base Station RF Bandwidth edge frequency and the centre of the measuring filter.

-	f_offsetmax is the offset to the frequency 10 MHz outside the downlink operating band.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

For a multi-band TAB connector, inside any Inter RF Bandwidth gaps with Wgap < 20 MHz, emissions shall not exceed the cumulative sum of the test requirements specified at the Base Station RF Bandwidth edges on each side of the Inter RF Bandwidth gap. The basic limit for Base Station RF Bandwidth edge is specified in tables 6.6.5.5.2-1 to 6.6.5.5.2-8, where in this case:

-	f is the separation between the Base Station RF Bandwidth edge frequency and the nominal -3 dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the Base Station RF Bandwidth edge frequency and the centre of the measuring filter.

-	f_offsetmax is equal to the Inter RF Bandwidth gap divided by two.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

For a multi-band TAB connector, the operating band unwanted emission limits apply also in a supported operating band without any carriers transmitted, in the case where there are carriers transmitted in another operating band. In this case where there is no carrier transmitted in an operating band, no cumulative limits are applied in the inter-band gap between a supported downlink band with carrier(s) transmitted and a supported downlink band without any carrier transmitted and

-	In case the Inter RF Bandwidth gap between a supported downlink band with carrier(s) transmitted and a supported downlink band without any carrier transmitted is less than 20MHz, f_offsetmax shall be the offset to the frequency 10 MHz outside the outermost edges of the two supported downlink operating bands and the operating band unwanted emission limit of the band where there are carriers transmitted, as defined in the tables of the present subclause, shall apply across both supported downlink bands.

-	In other cases, the operating band unwanted emission limit of the band where there are carriers transmitted, as defined in the tables of the present subclause for the largest frequency offset (fmax), shall apply from 10 MHz below the lowest frequency, up to 10 MHz above the highest frequency of the supported downlink operating band without any carrier transmitted.

Inside any sub-block gap for a TAB connector operating in non-contiguous spectrum, emissions shall not exceed the cumulative sum of the test requirements specified for the adjacent sub blocks on each side of the sub block gap. The basic limit for each sub block is specified in tables 6.6.5.5.2-1 to 6.6.5.5.2-8, where in this case:

-	f is the separation between the sub block edge frequency and the nominal -3 dB point of the measuring filter closest to the sub block edge frequency.

-	f_offset is the separation between the sub block edge frequency and the centre of the measuring filter.

-	f_offsetmax is equal to the sub block gap bandwidth divided by two.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

---------------------------------- Unchanged part omitted----------------------------------

---------------------------------- Next modified section ----------------------------------

##### 6.6.5.5.3	Basic Limits for MSR Band Category 2

For a TAB connector operating in Band Category 2 the requirement applies outside the Base Station RF Bandwidth edges. In addition, for a TAB connector operating in non-contiguous spectrum, it applies inside any sub-block gap.

Outside the Base Station RF Bandwidth edges, emissions shall not exceed the maximum levels specified in tables 6.6.5.5.3-1 to 6.6.5.5.3-8, where:

-	f is the separation between the Base Station RF Bandwidth edge frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the Base Station RF Bandwidth edge frequency and the centre of the measuring filter.

-	f_offsetmax is the offset to the frequency 10 MHz outside the downlink operating band.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

For a multi-band TAB connector, inside any Inter RF Bandwidth gaps with Wgap < 20 MHz, emissions shall not exceed the cumulative sum of the test requirements specified at the Base Station RF Bandwidth edges on each side of the Inter RF Bandwidth gap. The basic limit for Base Station RF Bandwidth edge is specified in tables 6.6.5.5.3-1 to 6.6.5.5.3-8, where in this case:

-	f is the separation between the Base Station RF Bandwidth edge frequency and the nominal -3 dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the Base Station RF Bandwidth edge frequency and the centre of the measuring filter.

-	f_offsetmax is equal to the Inter RF Bandwidth gap divided by two.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

For a multi-band TAB connector and where there is no carrier transmitted in an operating band, no cumulative limits are applied in the inter-band gap between a supported downlink band with carrier(s) transmitted and a supported downlink band without any carrier transmitted and

-	In case the inter-band gap between a supported downlink band with carrier(s) transmitted and a supported downlink band without any carrier transmitted less than is 20MHz, f_offsetmax shall be the offset to the frequency 10 MHz outside the outermost edges of the two supported downlink operating bands and the operating band unwanted emission limit of the band where there are carriers transmitted, as defined in the tables of the present subclause, shall apply across both supported downlink bands.

-	In other cases, the operating band unwanted emission limit of the band where there are carriers transmitted, as defined in the tables of the present subclause for the largest frequency offset (fmax), shall apply from 10 MHz below the lowest frequency, up to 10 MHz above the highest frequency of the supported downlink operating band without any carrier transmitted.

---------------------------------- Unchanged part omitted----------------------------------

---------------------------------- Next modified section ----------------------------------

##### 6.6.5.5.5	Basic Limits for E-UTRA

6.6.5.5.5.1	General

The measurement results in subclause 6.6.5.4 shall not exceed the maximum levels specified in the tables below, where:

-	f is the separation between the channel edge frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the channel edge frequency and the centre of the measuring filter.

-	f_offsetmax is the offset to the frequency 10 MHz outside the downlink operating band.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

For a multi-band TAB connector, inside any Inter RF Bandwidth gaps with Wgap < 20 MHz, emissions shall not exceed the cumulative sum of the test requirements specified at the Base Station RF Bandwidth edges on each side of the Inter RF Bandwidth gap. The basic limit for Base Station RF Bandwidth edge is specified in tables 6.6.5.5.5.2-1 to 6.6.5.5.5.2-9, where in this case:

-	f is the separation between the Base Station RF Bandwidth edge frequency and the nominal -3 dB point of the measuring filter closest to the Base Station RF Bandwidth edge.

-	f_offset is the separation between the Base Station RF Bandwidth edge frequency and the centre of the measuring filter.

-	f_offsetmax is equal to the Inter RF Bandwidth gap minus half of the bandwidth of the measuring filter.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

For multi-band TAB connector, the operating band unwanted emission limits apply also in a supported operating band without any carrier transmitted, in the case where there are carrier(s) transmitted in another supported operating band. In this case, no cumulative limit is applied in the inter-band gap between a supported downlink operating band with carrier(s) transmitted and a supported downlink operating band without any carrier transmitted and:

-	In case the inter-band gap between a supported downlink operating band with carrier(s) transmitted and a supported downlink operating band without any carrier transmitted is less than 20 MHz, f_offsetmax shall be the offset to the frequency 10 MHz outside the outermost edges of the two supported downlink operating bands and the operating band unwanted emission limit of the band where there are carriers transmitted, as defined in the tables of the present subclause, shall apply across both downlink bands.

-	In other cases, the operating band unwanted emission limit of the band where there are carriers transmitted, as defined in the tables of the present subclause for the largest frequency offset (fmax), shall apply from 10 MHz below the lowest frequency, up to 10 MHz above the highest frequency of the supported downlink operating band without any carrier transmitted.

For a multicarrier E-UTRA TAB connector or a TAB connector configured for intra-band contiguous or non-contiguous carrier aggregation the definitions above apply to the lower edge of the carrier transmitted at the lowest carrier frequency and the upper edge of the carrier transmitted at the highest carrier frequency within a specified frequency band.

In addition inside any sub-block gap for a TAB connector operating in non-contiguous spectrum, measurement results shall not exceed the cumulative sum of the test requirements specified for the adjacent sub blocks on each side of the sub block gap. The basic limit for each sub block is specified in tables 6.6.5.5.5.2-1 to 6.6.5.5.5.2-9, where in this case:

-	f is the separation between the sub block edge frequency and the nominal -3 dB point of the measuring filter closest to the sub block edge.

-	f_offset is the separation between the sub block edge frequency and the centre of the measuring filter.

-	f_offsetmax is equal to the sub block gap bandwidth minus half of the bandwidth of the measuring filter.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

---------------------------------- End of modified section ----------------------------------