3GPP TSG-RAN WG3 Meeting #96	R3-171911

# Hangzhou, China, 15th -19th  May 2017

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.423 | CR | 1897 | rev | 1 | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of the abnormal case in Dual Cell E-DCH operation enhancements configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | UTRA_MCe-Core |  |  |  |  |  |  | Date: |  | 2017-05-05 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The abnormal case description for the use of DTX Information in Dual Cell E-DCH operation enhancement is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The text procedure for the use of the DTX releasted Information in Dual Cell E-DCH operation enhancement is corrected. Additionally, the abnormal case covers now the situation when the DTX information is not presented. Revision History Rev 1. Editorial Updates. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The incorrect description of abnormal case may lead to ambigous netowrk implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1, 8.3.1.4, 8.3.2, 8.3.2.4, 8.3.4, 8.3.4.4, 8.3.7, 8.3.7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 25.433 CR 2094 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 8.3.1	Radio Link Setup

#### 8.3.1.1	General

This procedure is used for establishing the necessary resources in the DRNS for one or more radio links.

The connection-oriented service of the signalling bearer shall be established in conjunction with this procedure.

------------------------------------------

Skip to next change

------------------------------------------

#### 8.3.1.4	Abnormal Conditions

If the DRNC receives either an S-RNTI or a D-RNTI which already has RL(s) established [FDD – and the Synchronisation Indicator IE is not included in the RADIO LINK SETUP message,] the DRNC shall send the RADIO LINK SETUP FAILURE message to the SRNC, indicating the reason for failure.

[FDD – If the RADIO LINK SETUP REQUEST message includes the Active Pattern Sequence Information IE, but the Transmission Gap Pattern Sequence Information IE is not present, then the DRNC shall reject the procedure using the RADIO LINK SETUP FAILURE message.]

[FDD – If the RADIO LINK SETUP REQUEST message includes both the Initial DL TX Power IE and the Primary CPICH Ec/No IE or does not include either of these IEs, then the DRNC shall reject the procedure using the RADIO LINK SETUP FAILURE message.]

If more than one DCH of a set of co-ordinated DCHs has the QE-Selector IE set to “selected” [TDD – or no DCH of a set of co-ordinated DCHs has the QE-Selector IE set to “selected”] the DRNS shall reject the Radio Link Setup procedure and shall respond with a RADIO LINK SETUP FAILURE message.

[FDD – If only the Initial DL TX Power IE or the Uplink SIR Target IE is included in the RADIO LINK SETUP REQUEST message, then DRNC shall reject the Radio Link Setup procedure and shall respond with the RADIO LINK SETUP FAILURE message.]

If the RADIO LINK SETUP REQUEST message includes a DCH Information IE with multiple DCH Specific Info IEs, and if the DCHs in the DCH Information IE do not have the same Transmission Time Interval IE in the Semi-static Transport Format Information IE, then the DRNC shall reject the procedure using the RADIO LINK SETUP FAILURE message.

[FDD – If the RADIO LINK SETUP REQUEST message includes the Enhanced Primary CPICH Ec/No IE, but not the Primary CPICH Ec/No IE, then the DRNC shall reject the procedure using the RADIO LINK SETUP FAILURE message.]

If the RADIO LINK SETUP REQUEST message includes the Transport Layer Address IE and the Binding ID IE in the RL Specific DCH Information IE [FDD – or in the RL Specific E-DCH Information IE] included in the RL Information IE for a specific RL and the Diversity Control Field IE is set to “Must” [FDD- or the RL is combined with an E-DCH RL previously listed in the RADIO LINK SETUP RESPONSE message in the DRNS], the DRNC shall reject the Radio Link Setup procedure and the DRNC shall respond with the RADIO LINK SETUP FAILURE message.

If ALCAP is not used, if the RADIO LINK SETUP REQUEST message does not include the Transport Layer Address IE and the Binding ID IE in the RL Specific DCH Information IE nor RL Specific E-DCH Information IE in the RL Information IE for a specific RL and the Diversity Control Field IE is set to “May”, the DRNC shall reject the Radio Link Setup procedure and respond with the RADIO LINK SETUP FAILURE message.

If ALCAP is not used, if the RADIO LINK SETUP REQUEST message does not include the Transport Layer Address IE and the Binding ID IE in the RL Specific DCH Information IE in the RL Information IE for a specific RL and the Diversity Control Field IE is set to “Must Not”, the DRNC shall reject the Radio Link Setup procedure and respond with the RADIO LINK SETUP FAILURE message.

If ALCAP is not used, if the RADIO LINK SETUP REQUEST message does not include the Transport Layer Address IE and the Binding ID IE in the RL Specific DCH Information IE in the RL Information IE for the first RL and/or [FDD – in the RL Specific E-DCH Information IE in the RL Information IE for the first E-DCH RL][TDD – in the E-DCH MAC-d Flows Information TDD IE], the DRNC shall reject the Radio Link Setup procedure and respond with the RADIO LINK SETUP FAILURE message.

If ALCAP is not used, if the RADIO LINK SETUP REQUEST message does not include the Transport Layer Address IE and the Binding ID IE for an HS-DSCH MAC-d Flow in the HS-DSCH MAC-d Flows Information IE, the DRNC shall reject the Radio Link Setup procedure and respond with the RADIO LINK SETUP FAILURE message.

[TDD – If ALCAP is not used, if the RADIO LINK SETUP REQUEST message does not include the Transport Layer Address IE and the Binding ID IE for a DSCH in the DSCH TDD Information IE and/or for an USCH in the USCH Information IE, the DRNC shall reject the Radio Link Setup procedure and respond with the RADIO LINK SETUP FAILURE message.]

If the RADIO LINK SETUP REQUEST message includes the Transport Layer Address IE or the Binding ID IE, and not both are present for a transport bearer intended to be established, the DRNC shall reject the Radio Link Setup procedure and the DRNC shall respond with the RADIO LINK SETUP FAILURE message.

If the RADIO LINK SETUP REQUEST message includes an HS-PDSCH RL ID IE not referring to one of the radio links to be established, the DRNC shall reject the procedure using the RADIO LINK SETUP FAILURE message.

If the RADIO LINK SETUP REQUEST message contains the HS-DSCH Information IE and if the Priority Queues associated with the same HS-DSCH MAC-d Flow ID IE have the same Scheduling Priority Indicator IE value, the DRNC shall reject the procedure using the RADIO LINK SETUP FAILURE message.

If the RADIO LINK SETUP REQUEST message includes the Maximum MAC-d PDU Size Extended IE for a Priority Queue in the HS-DSCH MAC-d Flows Information IE in the HS-DSCH Information IE, and the HS-DSCH MAC-d PDU Size Format IE in the HS-DSCH Information IE has the value “Indexed MAC-d PDU Size”, the DRNC shall reject the procedure using the RADIO LINK SETUP FAILURE message.

If the RADIO LINK SETUP REQUEST message does not include the Maximum MAC-d PDU Size Extended IE for a Priority Queue in the HS-DSCH MAC-d Flows Information IE in the HS-DSCH Information IE, and the HS-DSCH MAC-d PDU Size Format IE in the HS-DSCH Information IE has the value “Flexible MAC-d PDU Size”, the DRNC shall reject the procedure using the RADIO LINK SETUP FAILURE message.

[FDD – If the RADIO LINK SETUP REQUEST message contains, for at least one logical channel, the Maximum MAC-d PDU Size Extended IE in the E-DCH MAC-d Flows Information IE in the E-DCH FDD Information IE and there exist a logical channel for which the Maximum MAC-d PDU Size Extended IE in the E-DCH MAC-d Flows Information IE in the E-DCH FDD Information IE is not present, the DRNC shall reject the procedure using the RADIO LINK SETUP FAILURE message.]

[TDD – If the RADIO LINK SETUP REQUEST message contains, for at least one logical channel, the Maximum MAC-d PDU Size Extended IE in the E-DCH MAC-d Flows Information TDD IE in the E-DCH Information IE, and there exist a logical channel for which the Maximum MAC-d PDU Size Extended IE in the E-DCH MAC-d Flows Information TDD IE in the E-DCH Information IE is not present, the DRNC shall reject the procedure using the RADIO LINK SETUP FAILURE message.]

[FDD – If the RADIO LINK SETUP REQUEST message includes the F-DPCH Information IE and the DL DPCH Information IE configured simultaneously to one downlink frequency, then the DRNS shall reject the procedure using the RADIO LINK SETUP FAILURE message.]

[FDD – If the RADIO LINK SETUP REQUEST message contains the E-DCH RL Indication IE set to “E-DCH”, but does not contain the E-DCH FDD Information IE, or if the message contains the E-DCH FDD Information IE, but does not contain the E-DCH RL Indication IE set to “E-DCH”, then the DRNC shall reject the procedure using the RADIO LINK SETUP FAILURE message.]

If the RADIO LINK SETUP REQUEST message does not contain the E-DCH Decoupling Indication IE but contains the HS-PDSCH RL ID IE and the Serving E-DCH RL IE, and the Serving HS-DSCH Radio Link and the Serving E-DCH Radio Link are not configured to be in the same cell, then the DRNC shall reject the procedure using the RADIO LINK SETUP FAILURE message.

[FDD – If the RADIO LINK SETUP REQUEST message contains the HS-PDSCH RL ID IE and the E-DPCH Information IE which includes the HS-DSCH Configured Indicator IE set as “HS-DSCH not configured” then the DRNC shall reject the procedure using the RADIO LINK SETUP FAILURE message.]

[FDD – If the RADIO LINK SETUP REQUEST message contains the E-DPCH Information IE but does not contain the UL DPDCH Indicator for E-DCH operation IE, then the DRNC shall reject the procedure using the RADIO LINK SETUP FAILURE message.]

[FDD – If the RADIO LINK SETUP REQUEST message contains the Serving Cell Change CFN IE, but neither the Serving E-DCH RL IE nor HS-DSCH Information IE is included, then the DRNS shall reject the procedure using the RADIO LINK SETUP FAILURE message.]

[FDD – If the RADIO LINK SETUP REQUEST message contains the Transport Bearer Not Requested Indicator IE for a DCH, but does not contain the Unidirectional DCH indicator IE set to “Uplink DCH only” in the DCH Specific Info IE for the DCH, the DRNC shall reject the procedure using the RADIO LINK SETUP FAILURE message.]

[FDD – If the RADIO LINK SETUP REQUEST message contains the Synchronisation Indicator IE for a RL, but does not contain the D-RNTI IE which already has the RL, the DRNC shall reject the procedure using the RADIO LINK SETUP FAILURE message.]

[FDD – If the UL DPCH Information IE in the RADIO LINK SETUP REQUEST message contains the UL DPCCH Slot Format set to “4” but does not contain the F-DPCH Information IE, then the DRNC shall reject the procedure using the RADIO LINK SETUP FAILURE message.]

[FDD – If the UL DPCH Information IE in the RADIO LINK SETUP REQUEST message contains the UL DPCCH Slot Format set to “0” or “2” and the Continuous Packet Connectivity DTX-DRX Information IE, then the DRNC shall reject the procedure using the RADIO LINK SETUP FAILURE message.]

[FDD – If the UL DPCH Information IE in the RADIO LINK SETUP REQUEST message contains Diversity Mode IE set to “Closed loop mode 1”and UL DPCCH Slot Format not set to “2” or “3”, then the DRNC shall reject the procedure using the RADIO LINK SETUP FAILURE message.]

[FDD – If the RADIO LINK SETUP REQUEST message contains the MIMO Activation Indicator IE, Sixtyfour QAM Usage Allowed Indicator IE set to “Allowed”, the Additional HS Cell Information RL Setup IE, the MIMO with four transmit antennas Activation Indicator IE, the Dual Stream MIMO with four transmit antennas Activation Indicator IE and/or the Single Stream MIMO Activation Indicator IE, but does not contain the HS-DSCH MAC-d PDU Size Format IE set to “Flexible MAC-d PDU Size”, then the DRNC shall reject the procedure using the RADIO LINK SETUP FAILURE message.]

[FDD – If the RADIO LINK SETUP REQUEST message contains the Continuous Packet Connectivity DTX-DRX Information IE but does not contain the F-DPCH Information IE, then the DRNC shall reject the procedure using the RADIO LINK SETUP FAILURE message]

[FDD – If the RADIO LINK SETUP REQUEST message contains the Serving E-DCH RL ID IE but contains the Transport Bearer Not Requested Indicator IE in the RL Specific E-DCH Information for the new Serving E-DCH RL, the DRNC shall reject the procedure using the RADIO LINK SETUP FAILURE message.]

[FDD – If the RADIO LINK SETUP REQUEST message includes the Transport Bearer Not Requested Indicator IE for a DCH or an E-DCH MAC-d Flow for a specific RL and the specific RL is combined with RL which the transport bearer is configured to be established for the DCH or the E-DCH MAC-d Flow, previously listed in the RADIO LINK SETUP RESPONSE message in the DRNS, the DRNC shall reject the procedure using the RADIO LINK SETUP FAILURE message.]

[FDD – If the RADIO LINK SETUP REQUEST message contains the Additional HS Cell Information RL Setup IE indicating a seconadry serving cell that is not in the same Node B as the new serving HS-DSCH cell, then the DRNC shall reject the procedure using the RADIO LINK SETUP FAILURE message]

[FDD – If the RADIO LINK SETUP REQUEST message contains the Additional HS Cell Information RL Setup IE and if the HS-DSCH Information IE is not present, then the DRNC shall reject the procedure using the RADIO LINK SETUP FAILURE message.]

If the RADIO LINK SETUP REQUEST message includes the DL RLC PDU Size Format IE for a Priority Queue in the HS-DSCH MAC-d Flows Information IE in the HS-DSCH Information IE set to “Flexible RLC PDU Size”, and the HS-DSCH MAC-d PDU Size Format IE in the HS-DSCH Information IE has the value “Indexed MAC-d PDU Size”, the DRNC shall reject the procedure using the RADIO LINK SETUP FAILURE message.

If the RADIO LINK SETUP REQUEST message does not include the Maximum MAC-d PDU Size Extended IE for a Priority Queue in the HS-DSCH MAC-d Flows Information IE in the HS-DSCH Information IE, and the DL RLC PDU Size Format IE in the HS-DSCH Information IE has the value “Flexible RLC PDU Size”, the DRNC shall reject the procedure using the RADIO LINK SETUP FAILURE message.

[FDD – If the RADIO LINK SETUP REQUEST message contains more than one of a MIMO Activation Indicator IE, a MIMO with four transmit antennas Activation Indicator IE, a Dual Stream MIMO with four transmit antennas Activation Indicator IE and a Single Stream MIMO Activation Indicator IE in the HS-DSCH FDD Information IE or in the HS-DSCH FDD Secondary Serving Information IE in the Additional HS Cell Information RL Setup IE, then the DRNC shall reject the procedure using the RADIO LINK SETUP FAILURE message.]

[FDD – If the RADIO LINK SETUP REQUEST message contains the Additional E-DCH Cell Information RL Setup Req IE and if the E-DPCH Information IE is not present, then the DRNS shall reject the procedure using the RADIO LINK SETUP FAILURE message.]

[FDD – If the RADIO LINK SETUP REQUEST message contains the Additional E-DCH Cell Information RL Setup Req IE and there exist a logical channel for which the Maximum MAC-d PDU Size Extended IE in the E-DCH MAC-d Flows Information IE in the E-DCH FDD Information IE is not present, the DRNS shall reject the procedure using the RADIO LINK SETUP FAILURE message.]

[FDD – If the RADIO LINK SETUP REQUEST message contains the Additional E-DCH Cell Information RL Setup Req IE and the C-ID IE is not included in the Additional E-DCH RL Specific Information To Setup IE in the Additional E-DCH FDD Setup Information IE in the Additional E-DCH Cell Information Setup IE, the DRNS shall reject the procedure using the RADIO LINK SETUP FAILURE message.]

[FDD – If the RADIO LINK SETUP REQUEST message contains in the HS-DSCH FDD Secondary Serving Information IE in the Additional HS Cell Information RL Setup IE the Diversity Mode IE not set to “None” but not the Transmit Diversity Indicator or contains the Transmit Diversity Indicator but not the Diversity Mode IE not set to “None”, then the DRNS shall reject the procedure using the RADIO LINK SETUP FAILURE message.]

[FDD - If the RADIO LINK SETUP REQUEST message contains the Additional HS Cell Information RL Setup IE containing more than one secondary serving HS-DSCH RL, and all secondary serving HS-DSCH RLs in the new configuration will not be assigned consecutive ordinal numbers starting with the value "1", which are received in the Ordinal Number Of Frequency IE in the in the HS-DSCH FDD Secondary Serving Information IE, the DRNS shall reject the procedure using the RADIO LINK SETUP FAILURE message.]

[FDD - If the RADIO LINK SETUP REQUEST message contains the Additional HS Cell Information RL Setup IE containing more than one secondary serving HS-DSCH RL, the new configuration also contains an Additional E-DCH Serving Radio Link and the secondary serving HS-DSCH Radio link, which is configured in the same cell as the Additional E-DCH Serving Radio Link does not have Ordinal Number Of Frequency value “1”, the DRNS shall reject the procedure using the RADIO LINK SETUP FAILURE message.]

[FDD - If the RADIO LINK SETUP REQUEST message contains the Affected HS-DSCH serving cell List IE in the Active Pattern Sequence Information IE, and theTransmission Gap Pattern Sequence for affected HS-DSCH Serving Cells is activated on the HS-DSCH Primary Serving Cell but not for all the other serving cells, the DRNS shall reject the procedure using the RADIO LINK SETUP FAILURE message with the cause value “Invalid CM settings”.]

[FDD - If the RADIO LINK SETUP REQUEST message contains the UL CLTD Information IE but does not contain the F-TPICH Information IE, or if it contains HS-DSCH Preconfiguration Setup IE with UL CLTD Information IE but without F-TPICH Information IE, then the DRNC shall reject the procedure using the RADIO LINK SETUP FAILURE message.]

[FDD - If the RADIO LINK SETUP REQUEST message contains the UL MIMO Information IE in E-DCH FDD Information IE but does not contain the UL CLTD Information IE, or if it contains HS-DSCH Preconfiguration Setup IE with UL MIMO Information IE but without UL CLTD Information IE, then the DRNS shall reject the procedure using the RADIO LINK SETUP FAILURE message.]

[FDD - If the RADIO LINK SETUP REQUEST message contains more than one of a MIMO Activation Indicator IE, a MIMO with four transmit antennas Activation Indicator IE, a Dual Stream MIMO with four transmit antennas Activation Indicator IE in HS-DSCH Preconfiguration Setup IE or in the Secondary Cells IE in the HS-DSCH Preconfiguration Setup IE, then the DRNS shall reject the procedure using the RADIO LINK SETUP FAILURE message.]

[FDD - If the RADIO LINK SETUP REQUEST message contains the DCH Enhancements Information IE, and either the DL DPCH Slot Format IE is not set to “17” or “18”, or the UL DPCCH Slot Format IE is not set to “5”, then the DRNS shall reject the procedure using the RADIO LINK SETUP FAILURE message.]

[FDD - If the RADIO LINK SETUP REQUEST message does not contain the DCH Enhancements Information IE, and either (i) the DL DPCH Slot Format IE is set to “17”, or (ii) the DL DPCH Slot Format IE is set to “18”, or (iii) the UL DPCCH Slot Format IE is set to “5”, then the DRNS shall reject the procedure using the RADIO LINK SETUP FAILURE message.]

[FDD - If the Dual Cell E-DCH operation enhancements configuration is setup but the Dual Cell E-DCH Operation Enhancements Information IE is not included in the RADIO LINK SETUP REQUEST message, then the DRNS shall reject the procedure using the RADIO LINK SETUP FAILURE message.]

[FDD - If the Dual Cell E-DCH operation enhancements configuration is setup and if  the Continuous Packet Connectivity DTX-DRX Information IE is included in the RADIO LINK SETUP REQUEST message, but the DTX Information IE does not contain any of the value defined for 10ms TTI, then the DRNS shall reject the procedure using the RADIO LINK SETUP FAILURE message.]

------------------------------------------

Skip to next change

------------------------------------------

### 8.3.2	Radio Link Addition

#### 8.3.2.1	General

This procedure is used for establishing the necessary resources in the DRNS for one [FDD – or more] additional RLs towards a UE when there is already at least one RL established to the concerned UE via this DRNS.

This procedure shall use the signalling bearer connection for the relevant UE Context.

The Radio Link Addition procedure shall not be initiated if a Prepared Reconfiguration exists, as defined in subclause 3.1.

[TDD – The Radio Link Addition procedure serves to establish a new Radio Link with the DSCH and USCH included, if they existed before.]

------------------------------------------

Skip to next change

------------------------------------------

#### 8.3.2.4	Abnormal Conditions

If the RADIO LINK ADDITION REQUEST message includes a C-ID IE corresponding to a cell reserved for operator use and the Permanent NAS UE Identity is not available in the DRNC for the considered UE Context, the DRNC shall reject the procedure for this particular Radio Link and send the RADIO LINK ADDITION FAILURE message.

[FDD – If the RADIO LINK ADDITION REQUEST message includes the Transmission Gap Pattern Sequence Status IEs in the Active Pattern Sequence Information IE and it does not address exactly all ongoing compressed mode patterns and frequency specific compressed mode is not supported, the DRNS shall reject the Radio Link Addition procedure and shall respond with a RADIO LINK ADDITION FAILURE message with the Cause IE value “Invalid CM settings”.]

[FDD – If the RADIO LINK ADDITION REQUEST message is used to establish a new RL without compressed mode when compressed mode is active for the existing RL(s) (as specified in subclause 8.3.2.2), and if at least one of the new RLs is to be established in a cell that has the same UARFCN (both UL and DL) as at least one cell with an already existing RL and frequency specific compressed mode is not supported,, the DRNS shall reject the Radio Link Addition procedure and shall respond with a RADIO LINK ADDITION FAILURE message with the cause value “Invalid CM settings”.]

[FDD – If the power balancing is active with the Power Balancing Adjustment Type of the UE Context set to “Individual” in the existing RL(s) and if the DL Reference Power IEs are included in the RL Information IE but the DL Reference Power IE is not present for each RL in the RL Information IE, the DRNC shall reject the Radio Link Addition procedure and shall respond with a RADIO LINK ADDITION FAILURE message.]

[FDD – If the RADIO LINK ADDITION REQUEST message includes the DL Reference Power IEs in the RL Information IE but the power balancing is not active in the existing RL(s) or the power balancing is active with the Power Balancing Adjustment Type of the UE Context set to “Common” in the existing RL(s), the DRNC shall reject the Radio Link Addition procedure and shall respond with a RADIO LINK ADDITION FAILURE message with the cause value “Power Balancing status not compatible”.]

[FDD – If the RADIO LINK ADDITION REQUEST message includes the Enhanced Primary CPICH Ec/No IE, but not the Primary CPICH Ec/No IE, then the DRNC shall reject the procedure using the RADIO LINK ADDITION FAILURE message.]

If the RADIO LINK ADDITION REQUEST message includes the Transport Layer Address IE and the Binding ID IE in the RL Specific DCH Information IE [FDD – or for an E-DCH MAC-d flow in RL Specific E-DCH Information IE] included in the RL Information IE for a specific RL and the Diversity Control Field IE is set to “Must” [FDD – or the RL is combined with existing E-DCH RL which transport bearer is not established in the DRNS], the DRNC shall reject the Radio Link Addition procedure and respond with the RADIO LINK ADDITION FAILURE message.

If ALCAP is not used, if the RADIO LINK ADDITION REQUEST message does not include the Transport Layer Address IE and the Binding ID IE in the RL Specific DCH Information IE nor RL Specific E-DCH Information IE in the RL Information IE for a specific RL and the Diversity Control Field IE is set to “May”, the DRNC shall reject the Radio Link Addition procedure and respond with the RADIO LINK ADDITION FAILURE message.

If ALCAP is not used, if the RADIO LINK ADDITION REQUEST message does not include the Transport Layer Address IE and the Binding ID IE in the RL Specific DCH Information IE in the RL Information IE for a specific RL and the Diversity Control Field IE is set to “Must Not”, the DRNC shall reject the Radio Link Addition procedure and respond with the RADIO LINK ADDITION FAILURE message.

If ALCAP is not used, if the RADIO LINK ADDITION REQUEST message does not include the Transport Layer Address IE and the Binding ID IE in [FDD – the RL Specific E-DCH Information IE in the RL Information IE for the first E-DCH RL][TDD – the E-DCH MAC-d Flows Information TDD IE], the DRNC shall reject the Radio Link Addition procedure and respond with the RADIO LINK ADDITION FAILURE message.

If ALCAP is not used, if the RADIO LINK ADDITION REQUEST message does not include the Transport Layer Address IE and the Binding ID IE for an HS-DSCH MAC-d Flow in the HS-DSCH MAC-d Flows Information IE, the DRNC shall reject the Radio Link Addition procedure and respond with the RADIO LINK ADDITION FAILURE message.

If the RADIO LINK ADDITION REQUEST message includes the Transport Layer Address IE or the Binding ID IE, and not both are present for a transport bearer intended to be established, the DRNC shall reject the Radio Link Addition procedure and respond with the RADIO LINK ADDITION FAILURE message.

[FDD – If the RADIO LINK ADDITION REQUEST message includes the HS-DSCH Serving Cell Change Information IE but not the HS-DSCH FDD Information IE and the UE Context is not configured for HS-DSCH, then the DRNS shall reject the procedure using the RADIO LINK ADDITION FAILURE message.]

[FDD – If the RADIO LINK ADDITION REQUEST message includes the Serving Cell Change CFN IE but neither the Serving E-DCH RL IE nor the HS-DSCH Serving Cell Change Information IE, then the DRNS shall reject the procedure using the RADIO LINK ADDITION FAILURE message.]

[FDD – If the RADIO LINK ADDITION REQUEST message includes the Serving E-DCH RL IE but the UE Context is not configured for E-DCH, then the DRNS shall reject the procedure using the RADIO LINK ADDITION FAILURE message.]

[FDD – If the E-DCH FDD Information IE is present in the RADIO LINK ADDITION REQUEST message, but the E-DPCH Information IE is not present, then the DRNS shall reject the procedure using the RADIO LINK ADDITION FAILURE message.]

[FDD – If the RADIO LINK ADDITION REQUEST message includes the E-DCH RL Indication IE set to “E-DCH”, but no E-DCH FDD Information IE, and the UE Context is not configured for E-DCH, then the DRNC shall reject the procedure using the RADIO LINK ADDITION FAILURE message.]

[FDD – If the RADIO LINK ADDITION REQUEST message includes the E-DCH FDD Information IE but no E-DCH RL Indication IE set to “E-DCH”, then the DRNC shall reject the procedure using the RADIO LINK ADDITION FAILURE message.]

[TDD – If the RADIO LINK ADDITION REQUEST message includes the HS-PDSCH RL ID IE not equal to the RL ID IE, the DRNC shall reject the procedure using the RADIO LINK ADDITION FAILURE message.]

[TDD – If the RADIO LINK ADDITION REQUEST message includes the E-DCH Serving RL IE not equal to the RL ID IE, the DRNS shall reject the procedure using the RADIO LINK ADDITION FAILURE message.]

If the RADIO LINK ADDITION REQUEST message does not contain the E-DCH Decoupling Indication IE but contains the HS-PDSCH RL ID IE [FDD – in the HS-DSCH Serving Cell Change Information IE] and/or Serving E-DCH RL IE, and if both HS-DSCH and E-DCH are configured in the DRNS but the Serving HS-DSCH Radio Link and the Serving E-DCH Radio Link are not in the same cell, then the DRNC shall reject the procedure using the RADIO LINK ADDITION FAILURE message.

[FDD – If the RADIO LINK ADDITION REQUEST message contains the HS-DSCH Serving Cell Change Information IE and the E-DPCH Information IE which includes the HS-DSCH Configured Indicator IE set as “HS-DSCH not configured” then the DRNC shall reject the procedure using the RADIO LINK ADDITION FAILURE message.]

If the RADIO LINK ADDITION REQUEST message includes the Maximum MAC-d PDU Size Extended IE for a Priority Queue in the HS-DSCH MAC-d Flows Information IE in the HS-DSCH Information IE [FDD – in the HS-DSCH Serving Cell Change Information] and the HS-DSCH MAC-d PDU Size Format IE in the HS-DSCH Information IE [FDD – in the HS-DSCH Serving Cell Change Information] has the value “Indexed MAC-d PDU Size”, the DRNC shall reject the procedure using the RADIO LINK ADDITION FAILURE message.

If the RADIO LINK ADDITION REQUEST message does not include the Maximum MAC-d PDU Size Extended IE for a Priority Queue in the HS-DSCH MAC-d Flows Information IE in the HS-DSCH Information IE [FDD – in the HS-DSCH Serving Cell Change Information] and the HS-DSCH MAC-d PDU Size Format IE in the HS-DSCH Information IE [FDD – in the HS-DSCH Serving Cell Change Information] has the value “Flexible MAC-d PDU Size”, the DRNC shall reject the procedure using the RADIO LINK ADDITION FAILURE message.

[FDD – If the RADIO LINK ADDITION REQUEST message contains, for at least one logical channel, the Maximum MAC-d PDU Size Extended IE in the E-DCH MAC-d Flows Information IE in the E-DCH FDD Information IE and there exist a logical channel for which the Maximum MAC-d PDU Size Extended IE in the E-DCH MAC-d Flows Information IE in the E-DCH FDD Information IE is not present, the DRNC shall reject the procedure using the RADIO LINK ADDITION FAILURE message.]

[TDD – If the RADIO LINK ADDITION REQUEST message contains, for at least one logical channel, the Maximum MAC-d PDU Size Extended IE in the E-DCH MAC-d Flows Information TDD IE in the E-DCH Information IE, and there exist a logical channel for which the Maximum MAC-d PDU Size Extended IE in the E-DCH MAC-d Flows Information TDD IE in the E-DCH Information IE is not present, the DRNC shall reject the procedure using the RADIO LINK ADDITION FAILURE message.]

[FDD – If the RADIO LINK ADDITION REQUEST message contains the Transport Bearer Not Requested Indicator IE for a DCH but the DCH is configured to be included as a part of the downlink CCTrCH, the DRNC shall reject the procedure using the RADIO LINK ADDITION FAILURE message.]

[FDD – If the RADIO LINK ADDITION REQUEST message contains the MIMO Activation Indicator IE, Sixtyfour QAM Usage Allowed Indicator IE set to “Allowed”, the Additional HS Cell Information RL Addition IE, the MIMO with four transmit antennas Activation Indicator IE, the Dual Stream MIMO with four transmit antennas Activation Indicator IE and/or the Single Stream MIMO Activation Indicator IE, but does not contain the HS-DSCH MAC-d PDU Size Format IE set to “Flexible MAC-d PDU Size”, then the DRNC shall reject the procedure using the RADIO LINK ADDITION FAILURE message.]

[FDD – If the RADIO LINK ADDITION REQUEST message contains the Serving E-DCH RL ID IE but contains the Transport Bearer Not Requested Indicator IE in the RL Specific E-DCH Information for the new Serving E-DCH RL or there is at least one E-DCH MAC-d flow which transport bearer was not configured in the existing E-DCH RL to be combined with the Serving E-DCH RL, the DRNC shall reject the procedure using the RADIO LINK ADDITION FAILURE message.]

[FDD – If the RADIO LINK ADDITION REQUEST message includes the Transport Bearer Not Requested Indicator IE for a DCH or an E-DCH MAC-d Flow for a specific RL and the specific RL is combined with the existing RL which the transport bearer is established for the DCH or the E-DCH MAC-d Flow in DRNS, the DRNC shall reject the procedure using the RADIO LINK ADDITION FAILURE message.]

[FDD – If the RADIO LINK ADDITION REQUEST message contains the Additional HS Cell Information RL Addition IE indicating a seconadry serving cell that is not in the same Node B as the new serving HS-DSCH cell, then the DRNC shall reject the procedure using the RADIO LINK ADDITION FAILURE message.]

[FDD – If the RADIO LINK ADDITION REQUEST message contains the Additional HS Cell Information RL Addition IE and if the HS-DSCH is not configured in the DRNS Communication Context and the HS-DSCH Information IE is not present, then the DRNS shall reject the procedure using the RADIO LINK SETUP FAILURE message.]

If the RADIO LINK ADDITION REQUEST message includes DL RLC PDU Size Format IE for a Priority Queue in the HS-DSCH MAC-d Flows Information IE in the HS-DSCH Information IE [FDD – in the HS-DSCH Serving Cell Change Information IE] set to “Flexible RLC PDU Size”, HS-DSCH MAC-d PDU Size Format IE in the HS-DSCH Information IE [FDD – in the HS-DSCH Serving Cell Change Information IE] has the value “Indexed MAC-d PDU Size”, the DRNC shall reject the procedure using the RADIO LINK ADDITION FAILURE message.

If the RADIO LINK ADDITION REQUEST message does not include the Maximum MAC-d PDU Size Extended IE for a Priority Queue in the HS-DSCH MAC-d Flows Information IE in the HS-DSCH Information IE [FDD – in the HS-DSCH Serving Cell Change Information IE] and the DL RLC PDU Size Format IE in the HS-DSCH Information IE [FDD – in the HS-DSCH Serving Cell Change Information IE] has the value “Flexible RLC PDU Size”, the DRNC shall reject the procedure using the RADIO LINK ADDITION FAILURE message.

[FDD – If the RADIO LINK ADDITION REQUEST message contains more than one of a MIMO Activation Indicator IE, a MIMO with four transmit antennas Activation Indicator IE, a Dual Stream MIMO with four transmit antennas Activation Indicator IE and a Single Stream MIMO Activation Indicator IE in the HS-DSCH FDD Information IE in the HS-DSCH Serving Cell Change Information IE or in the HS-DSCH FDD Secondary Serving Information IE in the Additional HS Cell Information RL Addition IE, then the DRNC shall reject the procedure using the RADIO LINK ADDITION FAILURE message.]

[FDD – If the RADIO LINK ADDITION REQUEST message contains the Additional E-DCH Cell Information RL Setup Req IE and if the E-DPCH Information IE is not present or the E-DPCH Information was not configured in the UE Context, then the DRNS shall reject the procedure using the RADIO LINK ADDITION FAILURE message.]

[FDD – If the RADIO LINK ADDITION REQUEST message contains the Additional E-DCH Cell Information RL Add Req IE and there exist a logical channel for which the Maximum MAC-d PDU Size Extended IE in the E-DCH MAC-d Flows Information IE in the E-DCH FDD Information IE is not present, the DRNS shall reject the procedure using the RADIO LINK ADDITION FAILURE message.]

[FDD – If the RADIO LINK ADDITION REQUEST message contains the Additional E-DCH Cell Information RL Add Req IE and the C-ID IE is not included in the Additional E-DCH RL Specific Information To Setup IE in the Additional E-DCH FDD Setup Information IE in the Additional E-DCH Cell Information Setup IE, but the Radio Link indicated by the E-DCH Additional RL ID IE is not configured in the current UE context as a Secondary Serving HS-DSCH radio link without any configured Additional E-DCH, the DRNS shall reject the procedure using the RADIO LINK ADDITION FAILURE message.]

[FDD – If the RADIO LINK ADDITION REQUEST message contains the Diversity Mode IE in the HS-DSCH FDD Secondary Serving Information IE in the Additional HS Cell Information RL Addition IE and the secondary serving HS-DSCH is already configured in the UE Context, then the DRNS shall reject the procedure using the RADIO LINK ADDITION FAILURE message.]

[FDD – If the secondary serving HS-DSCH is not configured in the UE Context and if the RADIO LINK ADDITION REQUEST message contains in the HS-DSCH FDD Secondary Serving Information IE in the Additional HS Cell Information RL Addition IE the Diversity Mode IE not set to “None” but not the Transmit Diversity Indicator or contains the Transmit Diversity Indicator but not the Diversity Mode IE not set to “None”, then the DRNS shall reject the procedure using the RADIO LINK SETUP FAILURE message.]

[FDD - If the RADIO LINK ADDITION REQUEST message contains the Additional HS Cell Information RL Addition IE and the new configuration contains more than one secondary serving HS-DSCH RL and all secondary serving HS-DSCH RLs in the new configuration will not be assigned consecutive ordinal numbers starting with the value "1", which are previously assigned to the RL or received in the Ordinal Number Of Frequency IE in the HS-DSCH FDD Secondary Serving Information IE, the DRNS shall reject the procedure using the RADIO LINK ADDITION FAILURE message.]

[FDD - If the RADIO LINK ADDITION REQUEST message contains the Additional HS Cell Information RL Addition IE and the new configuration contains more than one secondary serving HS-DSCH RL, the new configuration also contains an Additional E-DCH Serving Radio Link and the secondary serving HS-DSCH Radio link, which is configured in the same cell as the Additional E-DCH Serving Radio Link does not have Ordinal Number Of Frequency value "1", the DRNS shall reject the procedure using the RADIO LINK ADDITION FAILURE message.]

[FDD - If the RADIO LINK ADDITION REQUEST message contains the Affected HS-DSCH serving cell List IE in the Active Pattern Sequence Information IE, and theTransmission Gap Pattern Sequence for affected HS-DSCH Serving Cells is activated on the HS-DSCH Primary Serving Cell but not for all the other serving cells, the DRNS shall reject the procedure using the RADIO LINK ADDITION FAILURE message with the cause value “Invalid CM settings”.]

[FDD - If the RADIO LINK ADDITION REQUEST message contains the UL CLTD Information IE but does not contain the F-TPICH Information IE, or if it contains HS-DSCH Preconfiguration Setup IE with UL CLTD Information IE but without F-TPICH Information IE, then the DRNC shall reject the procedure using the RADIO LINK ADDITION FAILURE message.]

[FDD - If the RADIO LINK ADDITION REQUEST message contains the UL MIMO Information IE in E-DCH FDD Information IE but does not contain the UL CLTD Information IE, or if it contains HS-DSCH Preconfiguration Setup IE with UL MIMO Information IE but without UL CLTD Information IE, then the DRNS shall reject the procedure using the RADIO LINK ADDITION FAILURE message.]

[FDD - If the RADIO LINK ADDITION REQUEST message contains more than one of a MIMO Activation Indicator IE, a MIMO with four transmit antennas Activation Indicator IE, a Dual Stream MIMO with four transmit antennas Activation Indicator IE in HS-DSCH Preconfiguration Setup IE or in the Secondary Cells IE in the HS-DSCH Preconfiguration Setup IE, then the DRNS shall reject the procedure using the RADIO LINK ADDITION FAILURE message.]

[FDD - If the Dual Cell E-DCH operation enhancements configuration is setup but the Dual Cell E-DCH Operation Enhancements Information IE is not included in the RADIO LINK ADDITION REQUEST message, then the DRNS shall reject the procedure using the RADIO LINK ADDITION FAILURE message.]

[FDD - If the Dual Cell E-DCH operation enhancements configuration is setup and if  the Continuous Packet Connectivity DTX-DRX Information IE in the HS-DSCH Serving Cell Change Information IE is included in the RADIO LINK ADDITION REQUEST message, but the DTX Information IE does not contain any of the value defined for 10ms TTI, then the DRNS shall reject the procedure using the RADIO LINK ADDITION FAILURE message.]

[FDD - If the Dual Cell E-DCH operation enhancements configuration is setup and if the DTX related Information is not signalled but the currently used value is not from the set of values defined for 10ms TTI, then the DRNS shall reject the procedure using the RADIO LINK ADDITION FAILURE message.]

------------------------------------------

Skip to next change

------------------------------------------

### 8.3.4	Synchronised Radio Link Reconfiguration Preparation

#### 8.3.4.1	General

The Synchronised Radio Link Reconfiguration Preparation procedure is used to prepare a new configuration of Radio Link(s) related to one UE-UTRAN connection within a DRNS.

This procedure shall use the signalling bearer connection for the relevant UE Context.

The Synchronised Radio Link Reconfiguration Preparation procedure shall not be initiated if a Prepared Reconfiguration exists, as defined in subclause 3.1.

------------------------------------------

Skip to next change

------------------------------------------

#### 8.3.4.4	Abnormal Conditions

If only a subset of all the DCHs belonging to a set of co-ordinated DCHs is requested to be deleted, the DRNS shall reject the Synchronised Radio Link Reconfiguration Preparation procedure as having failed and shall send the RADIO LINK RECONFIGURATION FAILURE message to the SRNC.

If more than one DCH of a set of co-ordinated DCHs has the QE-Selector IE set to “selected” [TDD – or no DCH of a set of co-ordinated DCHs has the QE-Selector IE set to “selected”] the DRNS shall reject the Synchronised Radio Link Reconfiguration Preparation procedure and the DRNC shall respond with a RADIO LINK RECONFIGURATION FAILURE message.

If the RADIO LINK RECONFIGURATION PREPARE message includes a DCHs To Modify IE or DCHs To Add IE with multiple DCH Specific Info IEs, and if the DCHs in the DCHs To Modify IE or DCHs To Add IE do not have the same Transmission Time Interval IE in the Semi-static Transport Format Information IE, then the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.

[FDD – If the RL Information IE includes the DL Reference Power IE, but the power balancing is not active in the indicated RL(s), the DRNS shall reject the Synchronised Radio Link Reconfiguration Preparation procedure as having failed and the DRNC shall respond with the RADIO LINK RECONFIGURATION FAILURE message with the cause value “Power Balancing status not compatible”.]

[FDD – If the power balancing is active with the Power Balancing Adjustment Type of the UE Context set to “Common” in the existing RL(s) but the RADIO LINK RECONFIGURATION PREPARE message includes more than one DL Reference Power IE, the DRNS shall reject the Synchronised Radio Link Reconfiguration Preparation procedure as having failed and the DRNC shall respond with the RADIO LINK RECONFIGURATION FAILURE message with the cause value “Power Balancing status not compatible”.]

If the RADIO LINK RECONFIGURATION PREPARE message contains the Transport Layer Address IE or the Binding ID IE when establishing a transport bearer for any Transport Channel or HS-DSCH MAC-d flow being added, or any Transport Channel or HS-DSCH MAC-d flow being modified for which a new transport bearer was requested with the Transport Bearer Request Indicator IE., and not both are present for a transport bearer intended to be established, the DRNC shall reject the Synchronised Radio Link Reconfiguration Preparation procedure and the DRNC shall respond with a RADIO LINK RECONFIGURATION FAILURE message.

If the RADIO LINK RECONFIGURATION PREPARE message contains any of the HS-DSCH Information To Modify IE, HS-DSCH MAC-d Flows To Add IE or HS-DSCH MAC-d Flows To Delete IE in addition to the HS-DSCH Information IE, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.

If the RADIO LINK RECONFIGURATION PREPARE message contains any of the HS-DSCH Information To Modify IE, HS-DSCH MAC-d Flows To Add IE, HS-DSCH MAC-d Flows To Delete IE or HS-PDSCH RL ID IE and the Serving HS-DSCH Radio Link is not in the DRNS, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.

If the RADIO LINK RECONFIGURATION PREPARE message includes the HS-DSCH Information IE and does not include the HS-PDSCH RL ID IE, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.

If the RADIO LINK RECONFIGURATION PREPARE message includes the HS-DSCH Information To Modify IE deleting the last remaining Priority Queue of an HS-DSCH MAC-d Flow, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.

If the RADIO LINK RECONFIGURATION PREPARE message includes the HS-PDSCH RL ID IE indicating a Radio Link not existing in the UE Context, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.

If the RADIO LINK RECONFIGURATION PREPARE message contains any of the HS-DSCH Information IE, HS-DSCH Information To Modify IE, or HS-DSCH MAC-d Flows To Add IE and if in the new configuration the Priority Queues associated with the same HS-DSCH MAC-d Flow ID IE have the same Scheduling Priority Indicator IE value, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.

If, in the new configuration, the concerned UE Context is configured to use “Indexed MAC-d PDU Size” for an HS-DSCH but there exist a priority queue of the MAC-d flows of the HS-DSCH that is configured to use Maximum MAC-d PDU Size Extended, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.

If, in the new configuration, the concerned UE Context is configured to use “Flexible MAC-d PDU Size” for an HS-DSCH but there exist a priority queue of the MAC-d flows of the HS-DSCH that is configured to use MAC-d PDU Size Index, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.

If, in the new configuration, the concerned UE Context is configured to use “Fixed MAC-d PDU Size” for an E-DCH and there exist a Logical Channel of the MAC-d flows of the E-DCH that is configured to use Maximum MAC-d PDU Size Extended, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.

If, in the new configuration, the concerned UE Context is configured to use “Flexible MAC-d PDU Size” for an E-DCH and there exist a Logical Channel of the MAC-d flows of the E-DCH that is configured to use MAC-d PDU Size List, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.

[FDD – If the RADIO LINK RECONFIGURATION PREPARE message includes the F-DPCH Information IE and the DL DPCH Information IE configured simultaneously to one downlink frequency, then the DRNS shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

If the RADIO LINK RECONFIGURATION PREPARE message includes HS-DSCH Information IE and the HS-DSCH is already configured in the UE Context, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.

[FDD – If the concerned UE Context is configured to use DPCH in the downlink in the old configuration and if the RADIO LINK RECONFIGURATION PREPARE message includes the DL DPCH Power Information IE, then the DRNS shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the concerned UE Context is configured to use F-DPCH in the downlink in the old configuration and the RADIO LINK RECONFIGURATION PREPARE message includes at least one but not all of the TFCS IE, DL DPCH Slot Format IE, TFCI Signalling Mode IE, Multiplexing Position IE, Limited Power Increase IE and DL DPCH Power Information IE in the DL DPCH Information IE, then the DRNS shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the E-DCH FDD Information IE is present in the RADIO LINK RECONFIGURATION PREPARE message, but the E-DPCH Information IE is not present or if any of the Maximum Set of E-DPDCHs IE, Puncture Limit IE, E-TFCS Information IE, E-TTI IE, E-DPCCH Power Offset IE, E-RGCH 2-Index-Step Threshold IE, E-RGCH 3-Index-Step Threshold IE, HARQ Info for E-DCH IE or HS-DSCH Configured Indicator IE are not present in the E-DPCH Information IE, then the DRNS shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION PREPARE message includes the E-DCH RL Indication IE set to “E-DCH”, but no E-DCH FDD Information IE, and the UE Context is not configured for E-DCH, then the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION PREPARE message includes the E-DCH FDD Information IE but no E-DCH RL Indication IE set to “E-DCH”, then the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

If the RADIO LINK RECONFIGURATION PREPARE message does not contain the E-DCH Decoupling Indication IE but contains the HS-PDSCH RL ID IE and/or the Serving E-DCH RL IE, and if both HS-DSCH and E-DCH are configured in the new configuration but the Serving HS-DSCH Radio Link and the Serving E-DCH Radio Link are not in the same cell, then the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.

[FDD – If the RADIO LINK RECONFIGURATION PREPARE message contains the HS-PDSCH RL ID IE and the E-DPCH Information IE which includes the HS-DSCH Configured Indicator IE set as “HS-DSCH not configured” then the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION PREPARE message contains any of the E-DCH FDD Information To Modify IE, E-DCH MAC-d Flows To Add IE or E-DCH MAC-d Flows To Delete IE in addition to the E-DCH FDD Information IE, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION PREPARE message contains any of the E-DCH FDD Information To Modify IE, E-DCH MAC-d Flows To Add IE, E-DCH MAC-d Flows To Delete IE and the UE Context is not configured for E-DCH, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION PREPARE message includes the E-DCH FDD Information To Modify IE deleting the last remaining E-DCH Logical Channel of an E-DCH MAC-d Flow, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION PREPARE message includes E-DCH FDD Information IE and the E-DCH is already configured in the UE Context, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the Fast Reconfiguration Mode IE is included in the RADIO LINK RECONFIGURATION PREPARE message and the UL Scrambling Code IE does not indicate an uplink scrambling code different from the currently used uplink scrambling code the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION PREPARE message includes the Continuous Packet Connectivity DTX-DRX Information To Modify IE in addition to the Continuous Packet Connectivity DTX-DRX Information IE, then the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION PREPARE message includes the Continuous Packet Connectivity HS-SCCH less Deactivate Indicator IE in addition to the Continuous Packet Connectivity HS-SCCH less Information IE, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message. ]

[FDD – If the RADIO LINK RECONFIGURATION PREPARE message includes the Continuous Packet Connectivity HS-SCCH less Deactivate Indicator IE while the Continuous Packet Connectivity HS-SCCH less configuration isn’t configured in the DRNC, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION PREPARE message includes the Continuous Packet Connectivity DTX-DRX Information To Modify IE while the Continuous Packet Connectivity DTX-DRX configuration isn’t configured in the DRNC, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION PREPARE message includes the DRX Information To Modify IE in Continuous Packet Connectivity DTX-DRX Information To Modify IE while the Continuous Packet Connectivity DRX configuration is not configured in the DRNC, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

If the DCHs to Modify IE contains a DCH Specific Info IE which includes the Unidirectional DCH Indicator IE set to “Uplink DCH only” but no Transport Format Set IE for the uplink for this DCH and the DRNC had ignored the configuration of Transport Format Set for uplink, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.

If the DCHs to Modify IE contains a DCH Specific Info IE which includes the Unidirectional DCH Indicator IE set to “Downlink DCH only” but no Transport Format Set IE for the downlink for this DCH and the DRNC had ignored the configuration of Transport Format Set for downlink, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.

[FDD – If the RADIO LINK RECONFIGURATION PREPARE message contains the Transport Bearer Not Requested Indicator IE for a DCH or an E-DCH MAC-d flow but does not contain the corresponding DCH ID IE and the Unidirectional DCH indicator IE set to “Uplink DCH only” for the DCH in DCH Information To Add IE or does not contain the corresponding E-DCH MAC-d Flow ID IE in E-DCH MAC-d Flows Information IE, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the concerned UE Context is configured to apply UL DPCCH Slot Format 4 but is not configured to use F-DPCH, then the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the concerned UE Context is configured to apply UL DPCCH Slot Format 0 or 2 and execute Continuous Packet Connectivity DTX-DRX operation, then the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the concerned UE Context is configured to apply the “Closed loop mode 1” and if the concerned UE Context is not configured to apply UL DPCCH Slot Format 2 or 3, then the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the concerned UE Context is configured to apply MIMO, allowed to apply 64QAM, establish the secondary serving HS-DSCH Radio Link, apply MIMO with four transmit antennas, or apply Dual Stream MIMO with four transmit antennas, or apply Single Stream MIMO in the new configuration but is not configured to use flexible MAC-d PDU Size, then the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION PREPARE message includes the Transport Bearer Not Requested Indicator IE for a DCH in the RL Specific DCH Information IE but does not include the DCH ID IE for the DCH in the DCHs to Add IE, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION PREPARE message includes the Transport Bearer Not Requested Indicator IE for an E-DCH MAC-d flow in the RL Specific E-DCH Information IE but does not include the E-DCH MAC-d flow ID IE for the E-DCH MAC-d flow in the E-DCH MAC-d flows Information IE, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION PREPARE message contains the Continuous Packet Connectivity DTX-DRX Information IE but does not contain the F-DPCH Information IE and the concerned UE Context is not previously configured to use F-DPCH, then the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the concerned UE Context is configured to have the Serving E-DCH Radio Link but there is at least one E-DCH MAC-d flow which the transport bearer is not configured for the Serving E-DCH Radio Link in DRNS, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION PREPARE message includes the Transport Bearer Not Requested Indicator IE for a DCH or an E-DCH MAC-d Flow for a specific RL and the specific RL is combined with existing RL which the transport bearer is established for the DCH or the E-DCH MAC-d Flow in the DRNS, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION PREPARE message contains the Additional HS Cell Information RL Reconf Prep IE indicating a new seconadry serving cell that is not in the same Node B as the serving HS-DSCH cell (or new serving in case of simultaneous serving HS-DSCH cell change), then the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

If ALCAP is not used, if the concerned UE Context is configured to establish a DCH, an E-DCH MAC-d flow and/or an HS-DSCH MAC-d flow but the RADIO LINK RECONFIGURATION PREPARE message does not include the Transport Layer Address IE and the Binding ID IE for the DCH, the E-DCH MAC-d flow and/or the HS-DSCH MAC-d flow, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.

[TDD – If ALCAP is not used, if the concerned UE Context is configured to establish a DSCH and/or a USCH but the RADIO LINK RECONFIGURATION PREPARE message does not include the Transport Layer Address IE and the Binding ID IE for the DSCH and/or the USCH, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

If, in the new configuration, there exist a priority queue of the MAC-d flows of the HS-DSCH that is configured to use “Flexible RLC PDU Size” for an HS-DSCH but is not configured to use Maximum MAC-d PDU Size Extended, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.

If, in the new configuration, the concerned UE Context is configured to use MAC-d PDU Size Index for an HS-DSCH but there exist a priority queue of the MAC-d flows of the HS-DSCH that is configured to use “Flexible RLC PDU Size”, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.

[FDD – If the RADIO LINK RECONFIGURATION PREPARE message contains more than one of a MIMO Activation Indicator IE, a MIMO with four transmit antennas Activation Indicator IE, a Dual Stream MIMO with four transmit antennas Activation Indicator IE and a Single Stream MIMO Activation Indicator IE in the HS-DSCH FDD Information IE or in the HS-DSCH FDD Secondary Serving Information IE in the Additional HS Cell Information RL Reconf Prep IE, then the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the concerned UE Context is configured to apply more than one of MIMO, MIMO with four transmit antennas, Dual Stream MIMO with four transmit antennas and Single Stream MIMO for the HS-DSCH Radio Link or the Secondary Serving Radio link, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION PREPARE message contains the Additional E-DCH Cell Information RL Reconf Prep IE and if the E-DPCH Information IE is not present or the E-DPCH Information was not configured in the UE Context, then the DRNS shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION PREPARE message contains the Additional E-DCH Cell Information RL Reconf Prep IE and there exist a logical channel for which the Maximum MAC-d PDU Size Extended IE in the E-DCH MAC-d Flows Information IE in the E-DCH FDD Information IE is not present, the DRNS shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION PREPARE message contains the Additional E-DCH RL Specific Information To Setup IE in the Additional E-DCH FDD Setup Information IE in the Additional E-DCH Cell Information Setup IE in the Additional E-DCH Cell Information RL Reconf Prep IE and the C-ID IE is not included but the RL indicated by the E-DCH Additional RL ID IE is not configured in the current UE context as a Secondary Serving HS-DSCH radio link without any configured Additional E-DCH, the DRNS shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION PREPARE message contains the Diversity Mode IE in the HS-DSCH FDD Secondary Serving Information IE in the Additional HS Cell Information RL Reconf Prep IE and the secondary serving HS-DSCH is already configured in the UE Context, then the DRNS shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the secondary serving HS-DSCH is not configured in the UE Context and if the RADIO LINK RECONFIGURATION PREPARE message contains in the HS-DSCH FDD Secondary Serving Information IE in the Additional HS Cell Information RL Reconf Prep IE the Diversity Mode IE not set to “None” but not the Transmit Diversity Indicator or contains the Transmit Diversity Indicator but not the Diversity Mode IE not set to “None”, then the DRNS shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION PREPARE message contains the Diversity Mode IE in the Secondary Serving Information To Modify IE in the Additional HS Cell Information RL Reconf Prep IE and the Non Cell Specific Tx Diversity IE, the DRNS shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD - If the RADIO LINK RECONFIGURATION PREPARE message contains the Additional HS Cell Information RL Reconf Prep IE and the new configuration contains more than one secondary serving HS-DSCH RL, and all secondary serving HS-DSCH RLs in the new configuration will not be assigned consecutive ordinal numbers starting with the value "1", which are previously assigned to the RL or received in the Ordinal Number Of Frequency IE in the HS-DSCH FDD Secondary Serving Information IE or the HS-DSCH FDD Secondary Serving Information To Modify IE, the DRNS shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD - If the RADIO LINK RECONFIGURATION PREPARE message contains the Additional HS Cell Information RL Reconf Prep IE and the new configuration contains more than one secondary serving HS-DSCH RL, the new configuration also contains an Additional E-DCH Serving Radio Link and the secondary serving HS-DSCH Radio link, which is configured in the same cell as the Additional E-DCH Serving Radio Link does not have Ordinal Number Of Frequency value "1", the DRNS shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD - If the RADIO LINK RECONFIGURATION PREPARE message contains the UL CLTD Information IE but does not contain the F-TPICH Information IE, or if it contains HS-DSCH Preconfiguration Setup IE with UL CLTD Information IE but without F-TPICH Information IE, then the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD - If the RADIO LINK RECONFIGURATION PREPARE message contains the UL MIMO Information IE in E-DCH FDD Information IE but does not contain the UL CLTD Information IE, or if it contains HS-DSCH Preconfiguration Setup IE with UL MIMO Information IE but without UL CLTD Information IE, then the DRNS shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD - If the RADIO LINK RECONFIGURATION PREPARE message contains more than one of a MIMO Activation Indicator IE, a MIMO with four transmit antennas Activation Indicator IE, a Dual Stream MIMO with four transmit antennas Activation Indicator IE in HS-DSCH Preconfiguration Setup IE or in the Secondary Cells IE in the HS-DSCH Preconfiguration Setup IE, then the DRNS shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD - If the RADIO LINK RECONFIGURATION PREPARE message contains the DCH Enhancements Information IE, and either the DL DPCH Slot Format IE is not set to “17” or “18”, or the UL DPCCH Slot Format IE is not set to “5”, then the DRNS shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD - If the RADIO LINK RECONFIGURATION PREPARE message does not contain the DCH Enhancements Information IE, and either (i) the DL DPCH Slot Format IE is set to “17”, or (ii) the DL DPCH Slot Format IE is set to “18”, or (iii) the UL DPCCH Slot Format IE is set to “5”, then the DRNS shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD - If the RADIO LINK RECONFIGURATION PREPARE message contains the Fast TTI switching Mode Requested UnSynchronized IE in the E-DCH FDD Information To Modify IE, then the DRNS shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD - If the Dual Cell E-DCH operation enhancements configuration is setup or configured but the Dual Cell E-DCH Operation Enhancements Information IE is not included in the RADIO LINK RECONFIGURATION PREPARE message, then the DRNS shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD - If the Dual Cell E-DCH operation enhancements configuration is setup or configured and if  the Continuous Packet Connectivity DTX-DRX Information IE is included in the RADIO LINK RECONFIGURATION PREPARE message, but the DTX Information IE does not contain any of the value defined for 10ms TTI, then the DRNS shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD - If the Dual Cell E-DCH operation enhancements configuration is setup or configured but the DTX Information To Modify IE, if included in the Continuous Packet Connectivity DTX-DRX Information To Modify IE in the RADIO LINK RECONFIGURATION PREPARE message and the choice is “Modify”, does not contain any of the value defined for 10ms TTI, then the DRNS shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD - If the Dual Cell E-DCH operation enhancements configuration is setup or configured and if the DTX related Information is not signalled but the currently used value is not from the set of values defined for 10ms TTI, then the DRNS shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

------------------------------------------

Skip to next change

------------------------------------------

### 8.3.7	Unsynchronised Radio Link Reconfiguration

#### 8.3.7.1	General

The Unsynchronised Radio Link Reconfiguration procedure is used to reconfigure Radio Link(s) related to one UE-UTRAN connection within a DRNS.

The procedure is used when there is no need to synchronise the time of the switching from the old to the new radio link configuration in the cells used by the UE-UTRAN connection within the DRNS.

This procedure shall use the signalling bearer connection for the relevant UE Context.

The Unsynchronised Radio Link Reconfiguration procedure shall not be initiated if a Prepared Reconfiguration exists, as defined in subclause 3.1.

------------------------------------------

Skip to next change

------------------------------------------

#### 8.3.7.4	Abnormal Conditions

If only a subset of all the DCHs belonging to a set of co-ordinated DCHs is requested to be deleted, the DRNS shall reject the Unsynchronised Radio Link Reconfiguration procedure as having failed, and the DRNC shall send the RADIO LINK RECONFIGURATION FAILURE message to the SRNC.

If more than one DCH of a set of co-ordinated DCHs has the QE-Selector IE set to “selected” [TDD – or no DCH of a set of co-ordinated DCHs has the QE-Selector IE set to “selected”], the DRNS shall reject the Unsynchronised Radio Link Reconfiguration procedure, and the DRNC shall respond with a RADIO LINK RECONFIGURATION FAILURE message.

If the RADIO LINK RECONFIGURATION REQUEST message includes a DCHs To Modify IE or DCHs To Add IE with multiple DCH Specific Info IEs, and if the DCHs in the DCHs To Modify IE or DCHs To Add IE do not have the same Transmission Time Interval IE in the Semi-static Transport Format Information IE, then the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.

[FDD – If the RADIO LINK RECONFIGURATION REQUEST message includes the DL Reference Power Information IE, but the power balancing is not active in the indicated RL(s), the DRNS shall reject the Unsynchronised Radio Link Reconfiguration procedure as having failed and the DRNC shall respond the RADIO LINK RECONFIGURATION FAILURE message with the cause value “Power Balancing status not compatible”.]

[FDD – If the power balancing is active with the Power Balancing Adjustment Type of the UE Context set to “Common” in the existing RL(s) but the DL Reference Power Information IE includes the Individual DL Reference Power Information IE, the DRNS shall reject the Unsynchronised Radio Link Reconfiguration procedure as having failed and the DRNC shall respond with the RADIO LINK RECONFIGURATION FAILURE message with the cause value “Power Balancing status not compatible”.]

[FDD – If the power balancing is active with the Power Balancing Adjustment Type of the UE Context set to “Individual” in the existing RL(s) but the DL Reference Power Information IE includes the Common DL Reference Power IE, the DRNS shall reject the Unsynchronised Radio Link Reconfiguration procedure as having failed and the DRNC shall respond with the RADIO LINK RECONFIGURATION FAILURE message with the cause value “Power Balancing status not compatible”.]

If the RADIO LINK RECONFIGURATION REQUEST message contains the Transport Layer Address IE or the Binding ID IE when establishing a transport bearer for any Transport Channel or HS-DSCH MAC-d flow being added, or any Transport Channel or HS-DSCH MAC-d flow being modified for which a new transport bearer was requested with the Transport Bearer Request Indicator IE., and not both are present for a transport bearer intended to be established, the DRNC shall reject the Unsynchronised Radio Link Reconfiguration procedure, and the DRNC shall respond with a RADIO LINK RECONFIGURATION FAILURE message.

If the RADIO LINK RECONFIGURATION REQUEST message contains any of the HS-DSCH Information To Modify IE, HS-DSCH MAC-d Flows To Add IE or HS-DSCH MAC-d Flows To Delete IE in addition to the HS-DSCH Information IE, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.

If the RADIO LINK RECONFIGURATION REQUEST message contains any of the HS-DSCH Information To Modify IE, HS-DSCH MAC-d Flows To Add IE, HS-DSCH MAC-d Flows To Delete IE or HS-PDSCH RL ID IE and the Serving HS-DSCH Radio Link is not in the DRNS, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.

If the RADIO LINK RECONFIGURATION REQUEST message includes the HS-DSCH Information IE and does not include the HS-PDSCH RL ID IE, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.

If the RADIO LINK RECONFIGURATION REQUEST message includes the HS-PDSCH RL ID IE indicating a Radio Link not existing in the UE Context, the DRNS shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.

If the RADIO LINK RECONFIGURATION REQUEST message contains any of the HS-DSCH Information IE, HS-DSCH Information To Modify IE, or HS-DSCH MAC-d Flows To Add IE and if in the new configuration the Priority Queues associated with the same HS-DSCH MAC-d Flow ID IE have the same Scheduling Priority Indicator IE value, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.

If, in the new configuration, the concerned UE Context is configured to use “Indexed MAC-d PDU Size” for an HS-DSCH but there exist a priority queue of the MAC-d flows of the HS-DSCH that is configured to use Maximum MAC-d PDU Size Extended, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.

If, in the new configuration, the concerned UE Context is configured to use “Flexible MAC-d PDU Size” for an HS-DSCH but there exist a priority queue of the MAC-d flows of the HS-DSCH that is configured to use MAC-d PDU Size Index, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.

If, in the new configuration, the UE Context is configured to use “Fixed MAC-d PDU Size” for an E-DCH and there exist a Logical Channel of the MAC-d flows of the E-DCH that is configured to use Maximum MAC-d PDU Size Extended, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.

If, in the new configuration, the concerned UE Context is configured to use “Flexible MAC-d PDU Size” for an E-DCH and there exist a Logical Channel of the MAC-d flows of the E-DCH that is configured to use MAC-d PDU Size List, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.

If the RADIO LINK RECONFIGURATION REQUEST message includes HS-DSCH Information IE and the HS-DSCH is already configured in the UE Context, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.

[FDD – If the E-DCH FDD Information IE is present in the RADIO LINK RECONFIGURATION REQUEST message, but the E-DPCH Information IE is not present or if any of the Maximum Set of E-DPDCHs IE, Puncture Limit IE, E-TFCS Information IE, E-TTI IE, E-DPCCH Power Offset IE, E-RGCH 2-Index-Step Threshold IE, E-RGCH 3-Index-Step Threshold IE, HARQ Info for E-DCH IE or HS-DSCH Configured Indicator IE are not present in the E-DPCH Information IE, then the DRNS shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If any of the HS-DSCH Configured Indicator IE, Maximum Set of E-DPDCHs IE, Puncture Limit IE or E-TTI IE are present in the E-DPCH Information IE and the E-DCH FDD Information IE is not present in the RADIO LINK RECONFIGURATION REQUEST message, then the DRNS shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION REQUEST message includes the E-DCH RL Indication IE set to “E-DCH”, but no E-DCH FDD Information IE, and the UE Context is not configured for E-DCH, then the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION REQUEST message includes the E-DCH FDD Information IE but no E-DCH RL Indication IE set to “E-DCH”, then the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

If the RADIO LINK RECONFIGURATION REQUEST message does not contain the E-DCH Decoupling Indication IE but contains the HS-PDSCH RL ID IE and/or the Serving E-DCH RL IE, and if both HS-DSCH and E-DCH are configured in the new configuration but the Serving HS-DSCH Radio Link and the Serving E-DCH Radio Link are not in the same cell, then the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.

[FDD – If the RADIO LINK RECONFIGURATION REQUEST message contains the HS-PDSCH RL ID IE and the E-DPCH Information IE which includes the HS-DSCH Configured Indicator IE set as “HS-DSCH not configured” then the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION REQUEST message contains any of the E-DCH FDD Information To Modify IE, E-DCH MAC-d Flows To Add IE or E-DCH MAC-d Flows To Delete IE in addition to the E-DCH FDD Information IE, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION REQUEST message contains any of the E-DCH FDD Information To Modify IE, E-DCH MAC-d Flows To Add IE, E-DCH MAC-d Flows To Delete IE and the UE Context is not configured for E-DCH, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION REQUEST message includes the E-DCH FDD Information To Modify IE deleting the last remaining E-DCH Logical Channel of an E-DCH MAC-d Flow, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION REQUEST message includes E-DCH FDD Information IE and the E-DCH is already configured in the UE Context, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION REQUEST message includes the Continuous Packet Connectivity DTX-DRX Information To Modify IE in addition to the Continuous Packet Connectivity DTX-DRX Information IE, then the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION REQUEST message includes the Continuous Packet Connectivity HS-SCCH less Deactivate Indicator IE in addition to the Continuous Packet Connectivity HS-SCCH less Information IE, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message. ]

[FDD – If the RADIO LINK RECONFIGURATION REQUEST message includes the Continuous Packet Connectivity HS-SCCH less Deactivate Indicator IE while the Continuous Packet Connectivity HS-SCCH less configuration isn’t configured in the DRNC, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION REQUEST message includes the Continuous Packet Connectivity DTX-DRX Information To Modify IE while the Continuous Packet Connectivity DTX-DRX configuration isn’t configured in the DRNC, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION REQUEST message includes the DRX Information To Modify IE in Continuous Packet Connectivity DTX-DRX Information To Modify IE while the Continuous Packet Connectivity DRX configuration is not configured in the DRNC, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

If the DCHs to Modify IE contains a DCH Specific Info IE which includes the Unidirectional DCH Indicator IE set to “Uplink DCH only” but no Transport Format Set IE for the uplink for this DCH and the DRNC had ignored the configuration of Transport Format Set for uplink, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.

If the DCHs to Modify IE contains a DCH Specific Info IE which includes the Unidirectional DCH Indicator IE set to “Downlink DCH only” but no Transport Format Set IE for the downlink for this DCH and the DRNC had ignored the configuration of Transport Format Set for downlink, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.

[FDD – If the RADIO LINK RECONFIGURATION REQUEST message contains the Transport Bearer Not Requested Indicator IE for a DCH or an E-DCH MAC-d flow but does not contain the corresponding DCH ID IE and the Unidirectional DCH indicator IE set to “Uplink DCH only” for the DCH in DCH Information To Add IE or does not contain the corresponding E-DCH MAC-d Flow ID IE in E-DCH MAC-d Flows Information IE, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the concerned UE Context is configured to apply UL DPCCH Slot Format 0 or 2 and execute Continuous Packet Connectivity DTX-DRX operation, then the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the concerned UE Context is configured to apply MIMO, allowed to apply 64QAM, establish the secondary serving HS-DSCH Radio Link, apply MIMO with four transmit antennas or apply Dual Stream MIMO with four transmit antennas or apply Single Stream MIMO in the new configuration but is not configured to use flexible MAC-d PDU Size, then the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION REQUEST message contains the Additional HS Cell Information RL Reconf Req IE indicating a new seconadry serving cell that is not in the same Node B as the serving HS-DSCH cell (or new serving in case of simultaneous serving HS-DSCH cell change), then the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION REQUEST message includes the Transport Bearer Not Requested Indicator IE for a DCH in the RL Specific DCH Information IE but does not include the DCH ID IE for the DCH in the DCHs to Add IE, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION REQUEST message includes the Transport Bearer Not Requested Indicator IE for an E-DCH MAC-d flow in the RL Specific E-DCH Information IE but does not include the E-DCH MAC-d flow ID IE for the E-DCH MAC-d flow in the E-DCH MAC-d flows Information IE, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION REQUEST message contains the Continuous Packet Connectivity DTX-DRX Information IE but the concerned UE Context is not previously configured to use F-DPCH, then the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the concerned UE Context is configured to have the Serving E-DCH Radio Link but there is at least one E-DCH MAC-d flow which the transport bearer is not configured for the Serving E-DCH Radio Link in DRNS, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION REQUEST message includes the Transport Bearer Not Requested Indicator IE for a DCH or an E-DCH MAC-d Flow for a specific RL and the specific RL is combined with existing RL which the transport bearer is established for the DCH or the E-DCH MAC-d Flow in the DRNS, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

If ALCAP is not used, if the concerned UE Context is configured to establish a DCH, an E-DCH MAC-d flow and/or an HS-DSCH MAC-d flow but the RADIO LINK RECONFIGURATION REQUEST message does not include the Transport Layer Address IE and the Binding ID IE for the DCH, the E-DCH MAC-d flow and/or HS-DSCH MAC-d flow, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.

If, in the new configuration, there exist a priority queue of the MAC-d flows of the HS-DSCH that is configured to use “Flexible RLC PDU Size” for an HS-DSCH but is not configured to use Maximum MAC-d PDU Size Extended, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.

If, in the new configuration, the concerned UE Context is configured to use MAC-d PDU Size Index for an HS-DSCH but there exist a priority queue of the MAC-d flows of the HS-DSCH that is configured to use “Flexible RLC PDU Size”, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.

[FDD – If the RADIO LINK RECONFIGURATION REQUEST message contains more than one of a MIMO Activation Indicator IE, a MIMO with four transmit antennas Activation Indicator IE, a Dual Stream MIMO with four transmit antennas Activation Indicator IE and a Single Stream MIMO Activation Indicator IE in the HS-DSCH FDD Information IE or in the HS-DSCH FDD Secondary Serving Information IE in the Additional HS Cell Information RL Reconf Req IE, then the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the concerned UE Context is configured to apply more than one of MIMO, MIMO with four transmit antennas, Dual Stream MIMO with four transmit antennas and Single Stream MIMO for the HS-DSCH Radio Link or the Secondary Serving Radio link, the DRNC shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION REQUEST message contains the Additional E-DCH Cell Information RL Reconf Req IE and if the E-DPCH Information IE is not present or the E-DPCH Information was not configured in the UE Context, then the DRNS shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION REQUEST message contains the Additional E-DCH Cell Information RL Reconf Req IE and there exist a logical channel for which the Maximum MAC-d PDU Size Extended IE in the E-DCH MAC-d Flows Information IE in the E-DCH FDD Information IE is not present, the DRNS shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD – If the RADIO LINK RECONFIGURATION REQUEST message contains the Additional E-DCH RL Specific Information To Setup IE in the Additional E-DCH FDD Setup Information IE in the Additional E-DCH Cell Information Setup IE in the Additional E-DCH Cell Information RL Reconf Req IE and the C-ID IE is not included but the RL indicated by the E-DCH Additional RL ID IE is not configured in the current UE context as a Secondary Serving HS-DSCH radio link without any configured Additional E-DCH, the DRNS shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD - If the RADIO LINK RECONFIGURATION REQUEST message contains the Additional HS Cell Information RL Reconf Req IE and the new configuration contains more than one secondary serving HS-DSCH RL, and all secondary serving HS-DSCH RLs in the new configuration will not be assigned consecutive ordinal numbers starting with the value "1"which are previously assigned to the RL or received in the Ordinal Number Of Frequency IE in the HS-DSCH FDD Secondary Serving Information IE or the HS-DSCH FDD Secondary Serving Information To Modify Unsynchronised IE, the DRNS shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD - If the RADIO LINK RECONFIGURATION REQUEST message contains the Additional HS Cell Information RL Reconf Req IE and the new configuration contains more than one secondary serving HS-DSCH RL, the new configuration also contains an Additional E-DCH Serving Radio Link and the secondary serving HS-DSCH Radio link, which is configured in the same cell as the Additional E-DCH Serving Radio Link does not have Ordinal Number Of Frequency value "1", the DRNS shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD - If the RADIO LINK RECONFIGURATION REQUEST message contains the UL CLTD Information IE but does not contain the F-TPICH Information IE, or if it contains HS-DSCH Preconfiguration Setup IE with UL CLTD Information IE but without F-TPICH Information IE, then the DRNC shall reject the procedure using the RADIO LINK REQUEST FAILURE message.]

[FDD - If the RADIO LINK RECONFIGURATION REQUEST message contains the UL MIMO Information IE in E-DCH FDD Information IE but does not contain the UL CLTD Information IE, or if it contains HS-DSCH Preconfiguration Setup IE with UL MIMO Information IE but without UL CLTD Information IE, then the DRNS shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD - If the RADIO LINK RECONFIGURATION REQUEST message contains more than one of a MIMO Activation Indicator IE, a MIMO with four transmit antennas Activation Indicator IE, a Dual Stream MIMO with four transmit antennas Activation Indicator IE in HS-DSCH Preconfiguration Setup IE or in the Secondary Cells IE in the HS-DSCH Preconfiguration Setup IE, then the DRNS shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD - If the RADIO LINK RECONFIGURATION REQUEST message contains the Fast TTI switching Mode Requested Synchronized IE in the E-DCH FDD Information To Modify IE, then the DRNS shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD - If the Dual Cell E-DCH operation enhancements configuration is setup or configured but the Dual Cell E-DCH operation Enhancements Information IE is not included in the RADIO LINK RECONFIGURATION REQUEST message, then the DRNS shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD - If the Dual Cell E-DCH operation enhancements configuration is setup or configured and if the Continuous Packet Connectivity DTX-DRX Information IE is included in the RADIO LINK RECONFIGURATION REQUEST message, but the DTX Information IE does not contain any of the value defined for 10ms TTI, then the DRNS shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD - If the Dual Cell E-DCH operation enhancements configuration is setup or configured but the DTX Information To Modify IE, if included in the Continuous Packet Connectivity DTX-DRX Information To Modify IE in the RADIO LINK RECONFIGURATION REQUEST message and the choice is “Modify”, does not contain any of the value defined for 10ms TTI, then the DRNS shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]

[FDD - If the Dual Cell E-DCH operation enhancements configuration is setup or configured and if the DTX related Information is not signalled but the currently used value is not from the set of values defined for 10ms TTI, then the DRNS shall reject the procedure using the RADIO LINK RECONFIGURATION FAILURE message.]