3GPP TSG-RAN4 Meeting #84	R4-1707570

# Berlin, Germany, 21-25 August 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.123 | CR | 0569 | rev | - | Current version: | 14.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of the FDD L-band (Band 74) into 25.123 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_FDD_L_Band-Core |  |  |  |  |  | Date: |  |  | 2017-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of Band 74 in 25.123 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add Band 74 to test parameters. - Add NOTE to clarify that the tests for Band 74 shall be performed with the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Band 74 will not be properly introduced. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.9.2.5a.1, A.9.2.5b.1, B.1.1, B.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is based on the draft CR (R4-1706036) endorsed in RAN4#83. |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

A.9.2.5a.1	E-UTRAN FDD RSRP

A.9.2.5a.1.1	Test Purpose and Environment

The purpose of this test is to verify that the E-UTRA FDD RSRP measurement absolute accuracy is within the specified limits. This test will verify the requirements in section 9.1.1.5a and applies to UE supporting this capability.

Cell 1 is a UTRA TDD cell and cell 2 is a E-UTRA cell. In all tests, Cell 1 is the serving cell and Cell 2 the target cell. In the measurement control information it is indicated to the UE that periodic reporting of the E-UTRA RSRP measurement is used.

Idle interval of 80ms period as defined in TS25.331 is provided.

A.9.2.5a.1.2	Test parameters

E-UTRA FDD RSRP accuracy requirements are tested by using test parameters in Table A.9.2.5a.1-1, A.9.2.5a.1-2 and A.9.2.5a.1-3.

Table A.9.2.5a.1-1: General E-UTRA RSRP test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| DCH parameters |  | DL Reference Measurement Channel 12.2 kbps | As specified in TS 25.102 section A.2.2 |
| Power Control |  | On |  |
| Target quality value on DTCH | BLER | 0.01 |  |
| Active cell |  | Cell 1 | 1.28Mcps TDD cell |
| Neighbour cell |  | Cell 2 | E-UTRA FDD cell |
| CP length of cell 2 |  | normal |  |
| Idle intervals period | ms | 80 | As specified in TS 25.331 |
| Filter coefficient |  | 0 | L3 filtering is not used |
| Inter-RAT(E-UTRA FDD) measurement quantity |  | E-UTRA FDD RSRP |  |

Table A.9.2.5a.1-2: Cell specific test parameters for E-UTRA RSRP test parameters (cell 1)

| Parameter |  | Unit | Test 1, Test 2 |  |
| --- | --- | --- | --- | --- |
| DL timeslot number |  | 0 | DwPTS |
| UTRA RF Channel number (NOTE) |  | Channel 1 |  |
| PCCPCH_Ec/Ior | dB | -3 |  |
| DwPCH_Ec/Ior | dB |  | 0 |
| OCNS_Ec/Ior | dB | -3 |  |
| Îor/Ioc | dB | 3 |  |
| Ioc | dBm / 1.28MHz | -75 |  |
| Propagation condition |  | AWGN |  |
| NOTE: In the case of multi-frequency, the UTRA RF Channel Number can be set for the primary frequency in this test. |  |  |  |

# Table A.9.2.5a.1-3: Cell specific test parameters for E-UTRA RSRP test parameters (cell 2)

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | 1 |
| BWchannel |  | MHz | 10 | 10 |
| Measurement bandwidth |  |  | 22—27 | 22—27 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  | R.6 FDD | R.6 FDD |
| OCNG Patterns defined in A.3.2.1.2 (OP.2 FDD) |  |  | OP.2 FDD | OP.2 FDD |
| PBCH_RA |  | dB | 0 | 0 |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |
| Note2 | Bands 1, 4, 6, 10, 11, 18, 19, 21, 23 24 and 70 | dBm/15 kHz | -88.65 | -117 |
|  | Band 66 and 74 Note5 |  |  | -116.5 |
|  | Bands 2, 5, 7 and 27 |  |  | -115 |
|  | Band 25 |  |  | -113.5 |
|  | Band 28 |  |  | -115.5 |
|  | Bands 3, 8, 12, 13, 14, 17, 20 and 22 |  |  | -114 |
|  | Band 65 |  |  | -116.5 |
|  | Band 9 and 30 |  |  | -116 |
|  |  | dB | 10 | -4 |
| RSRPNote3 | Bands 1, 4, 6, 10, 11, 18, 19, 21, 23 and 24 | dBm/15 kHz | -78.65 | -121 |
|  | Band 66 and 74 Note5 |  |  | -120.5 |
|  | Bands 2, 5, 7 and 27 |  |  | -119 |
|  | Band 25 |  |  | -117.5 |
|  | Band 28 |  |  | -119.5 |
|  | Bands 3, 8, 12, 13, 14, 17, 20 and 22 |  |  | -118 |
|  | Band 65 |  |  | -120.5 |
|  | Band 9 and 30 |  |  | -120 |
| IoNote3 | Bands 1, 4, 6, 10, 11, 18, 19, 21, 23 24 and 70 | dBm/9 MHz | -49.5 | -87.76 |
|  | Band 66 and 74 Note5 |  |  | -87.26 |
|  | Bands 2, 5, 7 and 27 |  |  | -85.76 |
|  | Band 25 |  |  | -84.26 |
|  | Band 28 |  |  | -86.26 |
|  | Bands 3, 8, 12, 13, 14, 17, 20 and 22 |  |  | -84.76 |
|  | Band 65 |  |  | Band 65 |
|  | Band 9 and 30 |  |  | -86. 76 |
|  |  | dB | 10 | -4 |
| Propagation condition |  | - | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For Band 74, the tests shall be performed with the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

A.9.2.5b.1	E-UTRAN FDD RSRQ

A.9.2.5b.1.1	Test Purpose and Environment

The purpose of this test is to verify that the E-UTRA FDD RSRQ measurement absolute accuracy is within the specified limits. This test will verify the requirements in section 9.1.1.5b and applies to UE supporting this capability.

Cell 1 is a UTRA TDD cell and cell 2 is a E-UTRA FDD cell. In all tests, Cell 1 is the serving cell and Cell 2 the target cell. In the measurement control information it is indicated to the UE that periodic reporting of the E-UTRA RSRQ measurement is used.

Idle interval of 80ms period as defined in TS25.331 is provided.

A.9.2.5b.1.2	Test parameters

E-UTRA FDD RSRQ accuracy requirements are tested by using test parameters in Table A.9.2.5b.1-1, A.9.2.5b.1-2, and A.9.2.5b.1-3.

Table A.9.2.5b.1-1: General E-UTRA RSRQ test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| DCH parameters |  | DL Reference Measurement Channel 12.2 kbps | As specified in TS 25.102 section A.2.2 |
| Power Control |  | On |  |
| Target quality value on DTCH | BLER | 0.01 |  |
| Active cell |  | Cell 1 | 1.28Mcps TDD cell |
| Neighbour cell |  | Cell 2 | E-UTRA FDD cell |
| CP length of cell 2 |  | normal |  |
| Idle intervals period | ms | 80 | As specified in TS 25.331 |
| Filter coefficient |  | 0 | L3 filtering is not used |
| Inter-RAT(E-UTRA FDD) measurement quantity |  | E-UTRA FDD RSRQ |  |

Table A.9.2.5b.1-2: Cell specific test parameters for E-UTRA RSRQ test parameters (cell 1)

| Parameter |  | Unit | Test 1, Test 2 |  |
| --- | --- | --- | --- | --- |
| DL timeslot number |  | 0 | DwPTS |
| UTRA RF Channel number (NOTE) |  | Channel 1 |  |
| PCCPCH_Ec/Ior | dB | -3 |  |
| DwPCH_Ec/Ior | dB |  | 0 |
| OCNS_Ec/Ior | dB | -3 |  |
| Îor/Ioc | dB | 3 |  |
| Ioc | dBm / 1.28MHz | -75 |  |
| Propagation condition |  | AWGN |  |
| NOTE: In the case of multi-frequency, the UTRA RF Channel Number can be set for the primary frequency in this test. |  |  |  |

# Table A.9.2.5b.1-3: Cell specific test parameters for E-UTRA RSRQ test parameters (cell 2)

| Parameter |  |  | Unit |  | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  |  |  | 1 |  | 1 |  | 1 |  |
| Bwchannel |  |  | MHz |  | 10 |  | 10 |  | 10 |  |
| Measurement bandwidth |  |  |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 |  |  |  |  | R.6 FDD |  | R.6 FDD |  | R.6 FDD |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and A.3.2.1.2 (OP.2 FDD) |  |  |  |  | OP.2 FDD |  | OP.2 FDD |  | OP.2 FDD |  |
| PBCH_RA |  |  | dB |  | 0 |  | 0 |  | 0 |  |
| PBCH_RB |  |  |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | Bands 1, 4, 6, 10, 11, 18, 19, 21, 23 and 24 | dBm/15 kHz |  | -80 |  | -104.70 |  | -119.50 |  |
|  |  | Band 66 and 74 Note5 |  |  |  |  |  |  | -119.00 |  |
|  |  | Bands 2, 5, 7 and 27 |  |  |  |  |  |  | -117.50 |  |
|  |  | Band 25 |  |  |  |  |  |  | -116.00 |  |
|  |  | Band 28 |  |  |  |  |  |  | -118.00 |  |
|  |  | Bands 3, 8, 12, 13, 14, 17, 20 and 22 |  |  |  |  |  |  | -116.50 |  |
|  |  | Band 65 |  |  |  |  |  |  | -119.0 |  |
|  |  | Band 9 and 30 |  |  |  |  |  |  | -118.50 |  |
|  |  |  | dB |  | -1.75 |  | -4.0 |  | -4.0 |  |
| RSRPNote3 |  | Bands 1, 4, 6, 10, 11, 18, 19, 21, 23 and 24 | dBm/15 kHz |  | -81.75 |  | -108.70 |  | -123.50 |  |
|  |  | Band 66 and 74 Note5 |  |  |  |  |  |  | -123.00 |  |
|  |  | Bands 2, 5, 7 and 27 |  |  |  |  |  |  | -121.50 |  |
|  |  | Band 25 |  |  |  |  |  |  | -120.00 |  |
|  |  | Band 28 |  |  |  |  |  |  | -122.00 |  |
|  |  | Bands 3, 8, 12, 13, 14, 17, 20 and 22 |  |  |  |  |  |  | -120.50 |  |
|  |  | Band 65 |  |  |  |  |  |  | -123.0 |  |
|  |  | Band 9 and 30 |  |  |  |  |  |  | -122.50 |  |
| RSRQNote3 |  | Bands 1, 4, 6, 10, 11, 18, 19, 21, 23 and 24 | dB |  | -14.76 |  | -16.25 |  | -16.25 |  |
|  |  | Band 66 and 74 Note5 |  |  |  |  |  |  |  |  |
|  |  | Bands 2, 5, 7 and 27 |  |  |  |  |  |  |  |  |
|  |  | Band 25 |  |  |  |  |  |  |  |  |
|  |  | Band 28 |  |  |  |  |  |  |  |  |
|  |  | Bands 3, 8, 12, 13, 14, 17, 20 and 22 |  |  |  |  |  |  |  |  |
|  |  | Band 65 |  |  |  |  |  |  |  |  |
|  |  | Band 9 and 30 |  |  |  |  |  |  |  |  |
| IoNote3 |  | Bands 1, 4, 6, 10, 11, 18, 19, 21, 23 and 24 | dBm/9 MHz |  | -50 |  | -75.46 |  | -90.26 |  |
|  |  | Band 66 and 74 Note5 |  |  |  |  |  |  | -89.76 |  |
|  |  | Bands 2, 5, 7 and 27 |  |  |  |  |  |  | -88.26 |  |
|  |  | Band 25 |  |  |  |  |  |  | -86.76 |  |
|  |  | Band 28 |  |  |  |  |  |  | -88.76 |  |
|  |  | Bands 3, 8, 12, 13, 14, 17, 20 and 22 |  |  |  |  |  |  | -87.26 |  |
|  |  | Band 65 |  |  |  |  |  |  | -89.76 |  |
|  |  | Band 9 and 30 |  |  |  |  |  |  | -89.26 |  |
|  |  |  | dB |  | -1.75 |  | -4.0 |  | -4.0 |  |
| Propagation condition |  |  | - |  | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For Band 74, the tests shall be performed with the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

B.1.1.	Conditions for measurements of inter-RAT E-UTRA cells

This section defines the E-UTRAN RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot applicable for a corresponding operating band.

The conditions for measurements of inter-RAT E-UTRAN cells are defined in Table B.1.1-1

Table B.1.1-1. Conditions for measurements of inter-RAT E-UTRA cells

| Parameter | E-UTRA operating bands | Minimum RSRP | Minimum SCH_RP | RSRP Ês/Iot | SCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | 1, 4, 6, 10, 11, 18, 19, 21, 23, 24, 33, 34, 35, 36, 37, 38, 39, 40 | -124 | -124 | -4 | -4 |
|  | 66, 74 | -123.5 Note2 | -123.5 Note2 |  |  |
|  | 65 | -123.5 | -123.5 |  |  |
|  | 9, 30, 42, 43, 48 | -123 | -123 |  |  |
|  | 28 | -122.5 | -122.5 |  |  |
|  | 2, 5, 7, 27, 41, 44 | -122 | -122 |  |  |
|  | 26 | -121.5 Note1 | -121.5 Note1 |  |  |
|  | 3, 8, 12, 13, 14, 17, 20, 22 | -121 | -121 |  |  |
|  | 25 | -120.5 | -120.5 |  |  |
| Note 1: The condition is -122 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. Note 2: For Band 74, the condition is -124 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 1475.9-1510.9 MHz. |  |  |  |  |  |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

B.2.1.	Conditions for identification of a new cell in CELL_DCH State (3.84 Mcps option)

This section defines the E-UTRAN RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot applicable for a corresponding operating band.

The conditions for identification of a new cell in CELL_DCH State (3.84 Mcps option) are defined in Table B.2.1-1

Table B.2.1-1. Conditions for identification of a new cell in CELL_DCH State

| Parameter | E-UTRA operating bands | Minimum RSRP | Minimum SCH_RP | RSRP Ês/Iot | SCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | 1, 4, 6, 10, 11, 18, 19, 21, 23, 24, 33, 34, 35, 36, 37, 38, 39, 40 | -125 | -125 | -4 | -4 |
|  | 66, 74 | -124.5 Note2 | -124.5 Note2 |  |  |
|  | 65 | -124.5 | -124.5 |  |  |
|  | 9, 30, 42, 43, 48 | -124 | -124 |  |  |
|  | 28 | -123.5 | -123.5 |  |  |
|  | 2, 5, 7, 27, 41, 44 | -123 | -123 |  |  |
|  | 26 | -122.5 Note1 | -122.5 Note1 |  |  |
|  | 3, 8, 12, 13, 14, 17, 20, 22 | -122 | -122 |  |  |
|  | 25 | -121.5 | -121.5 |  |  |
| Note 1: The condition is -123 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. Note 2: For Band 74, the condition is -125 dBm/15kHz when the carrier frequency of the assigned E-UTRA channel bandwidth is within 1475.9-1510.9 MHz. |  |  |  |  |  |

----------------------------------------------------- end of Change ------------------------------------------------------------