3GPP TSG-RAN4 Meeting #84	R4-1707569

# Berlin, Germany, 21-25 August 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.133 | CR | 1429 | rev | - | Current version: | 14.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of the FDD L-band (Band 74) into 25.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_FDD_L_Band-Core |  |  |  |  |  | Date: |  |  | 2017-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of Band 74 in 25.133. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Separate FDD_B in E-UTRA band groups to FDD_B1 and FDD_B2, and add Band 74 to FDD_B2. - Add FDD_B1 and FDD_B2 to each test case. - Add NOTE to clarify that the minimum Io condition for Band 74 is reduced by 0.5 dB when the carrier frequency of the assigned E-UTRA channel bandwidth is within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Band 74 will not be properly introduced. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.5, A.9.10, A.9.12, B.1.1, B.2.1, B.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 34.121 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is based on the draft CR (R4-1706015) endorsed in RAN4#83. |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

3.5	Additional notation

3.5.1 	Groups of E-UTRA bands

The intention with the band grouping below is to increase the readability of the specification and to reduce impact on the specification if new E-UTRA bands are introduced.

Table 3.5.1-1: E-UTRA band groups

| Group | E-UTRA FDD |  | E-UTRA TDD |  |
| --- | --- | --- | --- | --- |
|  | Band group notation | Operating bands | Band group notation | Operating bands |
| A | FDD_A | 1, 4, 6, 10, 11, 18, 19, 21, 23, 24, 32 Note 4, 67 Note 6, 69Note 7, 70Note 8 | TDD_A | 33, 34, 35, 36, 37, 38, 39, 40, 45 |
| B | FDD_B1 | 65, 66 Note 5 | TDD_B | - |
|  | FDD_B2 | 74 Note 9 |  |  |
| C | FDD_C | 9, 30 | TDD_C | 42, 43, 48 |
| D | FDD_D | 28 | TDD_D | - |
| E | FDD_E | 2, 5, 7, 27 | TDD_E | 41, 44 |
| F | FDD_F | 26 Note 3 | TDD_F | - |
| G | FDD_G | 3, 8, 12, 13, 14, 17, 20, 22, 29 Note 2 | TDD_G | - |
| H | FDD_H | 25 | TDD_H | - |
| I | FDD_I | - | TDD_I | - |
| J | FDD_J | - | TDD_J | - |
| K | FDD_K | - | TDD_K | - |
| L | FDD_L | - | TDD_L | - |
| M | FDD_M | - | TDD_M | - |
| N | FDD_N | 31 | TDD_N | - |
| NOTE 1: The bands within the same group have the same Io conditions in a corresponding requirement in this specification. NOTE 2: Band 29 is used only for E-UTRA carrier aggregation with other E-UTRA bands. NOTE 3: The minimum Io condition for Band 26 is reduced by 0.5 dB when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 4: Band 32 is used only for E-UTRA carrier aggregation with other E-UTRA bands. NOTE 5: The 2180 - 2200 MHz part of Band 66 is used only for E-UTRA carrier aggregation with other E-UTRA bands. NOTE 6: Band 67 is used only for E-UTRA carrier aggregation with other E-UTRA bands. NOTE 7: Band 69 is used only for E-UTRA carrier aggregation with other E-UTRA bands. NOTE 8 The range 2010-2020 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured and TX-RX separation is 300 MHz The range 2005-2020 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured and TX-RX separation is 295 MHz. NOTE 9: The minimum Io condition for Band 74 is reduced by 0.5 dB when the carrier frequency of the assigned E-UTRA channel bandwidth is within 1475.9-1510.9 MHz. |  |  |  |  |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

A.9.1.10	E-UTRAN FDD RSRP absolute accuracy (CELL_DCH)

A.9.1.10.1	Test Purpose and Environment

The purpose of this test is to verify that the E-UTRAN FDD RSRP measurement absolute accuracy is within the specified limits. This test will verify the requirements in section 9.1.4a.

The test is carried out in Cell_DCH state using a compressed mode pattern with purpose “E-UTRAN Measurement”. The compressed mode pattern repeats every 80 ms and uses a gap length of 10 slots. Further details are found in TS 25.101 annex A.5.

Tables A.9.1.10.1-1 and A.9.1.10.1-2 define the limits of signal strengths and code powers on the UTRA FDD cell where the requirement is applicable. In the measurement control information periodic reporting of E-UTRAN FDD RSRP is indicated to the UE. The E-UTRAN FDD test parameters are given in Table A.9.1.10.1-3.

Table A.9.1.10.1-1: General UTRAN test parameters for E-UTRAN FDD RSRP measurements

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| DCH parameters |  | DL Reference Measurement Channel 12.2 kbps | As specified in TS 25.101 section A.3.1 |
| Power Control |  | On |  |
| Target quality value on DTCH | BLER | 0.01 |  |
| Compressed mode patterns - E-UTRAN measurement |  | Compressed mode reference pattern 2 Set 5 | As specified in table A.22 TS 25.101 section A.5 |
| Inter-RAT measurement quantity |  | E-UTRAN FDD RSRP |  |
| Monitored cell list size |  | 1 E-UTRAN FDD neighbour cell | Measurement control information is sent before the compressed mode pattern starts. |

# Table A.9.1.10.1-2: Cell specific UTRAN test parameters for E-UTRAN FDD RSRP measurements

| Parameter | Unit | Cell 1 |
| --- | --- | --- |
| UTRA RF Channel number | - | Channel 1 |
| CPICH_Ec/Ior | dB | -10 |
| PCCPCH_Ec/Ior | dB | -12 |
| SCH_Ec/Ior | dB | -12 |
| PICH_Ec/Ior | dB | -15 |
| DCH_Ec/Ior | dB | Note 1 |
| OCNS_Ec/Ior | dB | Note 2 |
| Îor/Ioc | dB | -1 |
| Ioc | dBm/ 3.84 MHz | -70 |
| CPICH_Ec/Io | dB | -13.54 |
| Propagation condition | - | AWGN |
| Note 1: The DPCH level is controlled by the power control loop. Note 2: The power of the OCNS channel that is added shall make the total power from the cell to be equal to Ior. |  |  |

Table A.9.1.10.1-3: E-UTRAN FDD RSRP test parameters

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
|  |  |  | Cell 2 | Cell 2 |
| BWchannel |  | MHz | 10 | 10 |
| Measurement bandwidth |  |  | 22—27 | 22—27 |
| PDCCH/PCFICH/PHICH Reference measurement channel as defined in TS 36.133 A.3.1.2.1 |  |  | R.6 FDD | R.6 FDD |
| OCNG Pattern as defined in TS 36.133 A.3.2.1.2 |  |  | OP.2 FDD | OP.2 FDD |
| PBCH_RA |  | dB | 0 | 0 |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |
| Note2 | Band group FDD_A | dBm/15 kHz | -88.65 | -117 |
|  | Band groupsFDD_B1 and FDD_B2 (Note 7) |  |  | -116.5 |
|  | Band groups FDD_E and FDD_F (Note 5) |  |  | -115 |
|  | Band group FDD_H |  |  | -113.5 |
|  | Band group FDD_D |  |  | -115.5 |
|  | Band group FDD_G |  |  | -114 |
|  | Band group FDD_C |  |  | -116 |
|  |  | dB | 10 | -4 |
| RSRPNote3 | Band group FDD_A | dBm/15 kHz | -78.65 | -121 |
|  | Band groups FDD_B1 and FDD_B2 (Note 7) |  |  | -120.5 |
|  | Band groups FDD_E and FDD_F (Note 5) |  |  | -119 |
|  | Band group FDD_H |  |  | -117.5 |
|  | Band group FDD_D |  |  | -119.5 |
|  | Band group FDD_G |  |  | -118 |
|  | Band group FDD_C |  |  | -120 |
| IoNote3 | Band group FDD_A | dBm/9 MHz | -49.5 | -87.76 |
|  | Band groups FDD_B1 and FDD_B2 (Note 7) |  |  | -87.26 |
|  | Band groups FDD_E and FDD_F (Note 5) |  |  | -85.76 |
|  | Band group FDD_H |  |  | -84.26 |
|  | Band group FDD_D |  |  | -86.26 |
|  | Bands group FDD_G |  |  | -84.76 |
|  | Band group FDD_C. |  |  | -86.76 |
|  |  | dB | 10 | -4 |
| Propagation condition |  | - | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For Band 26 in band group FDD_F, the tests shall be performed with the assigned E-UTRA channel bandwidth within 865-894. Note 6: E-UTRA operating band groups are as defined in Section 3.5. Note 7: For Band 74 in band group FDD_B2, the tests shall be performed with the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

A.9.1.12	E-UTRA FDD RSRQ absolute accuracy (CELL_DCH)

A.9.1.12.1	Test Purpose and Environment

The purpose of this test is to verify that the E-UTRAN FDD RSRQ measurement absolute accuracy is within the specified limits. This test will verify the requirements in section 9.1.4b.

The test is carried out in Cell_DCH state using a compressed mode pattern with purpose “E-UTRAN Measurement”. The compressed mode pattern repeats every 80 ms and uses a gap length of 10 slots. Further details are found in TS 25.101 annex A.5.

Tables A.9.1.12.1-1 and A.9.1.12.1-2 define the limits of signal strengths and code powers on the UTRA FDD cell where the requirement is applicable. In the measurement control information periodic reporting of E-UTRAN RSRQ is indicated to the UE. The E-UTRAN FDD test parameters are given in Table A.9.1.12.1-3.

Table A.9.1.12.1-1: General UTRAN test parameters for E-UTRAN FDD RSRQ measurements

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| DCH parameters |  | DL Reference Measurement Channel 12.2 kbps | As specified in TS 25.101 section A.3.1 |
| Power Control |  | On |  |
| Target quality value on DTCH | BLER | 0.01 |  |
| Compressed mode patterns - E-UTRAN measurement |  | Compressed mode reference pattern 2 Set 5 | As specified in table A.22 TS 25.101 section A.5 |
| Inter-RAT measurement quantity |  | E-UTRAN FDD RSRQ |  |
| Monitored cell list size |  | 1 E-UTRAN FDD neighbour cell | Measurement control information is sent before the compressed mode pattern starts. |

# Table A.9.1.12.1-2: Cell specific UTRAN test parameters for E-UTRAN FDD RSRQ measurements

| Parameter | Unit | Cell 1 |
| --- | --- | --- |
| UTRA RF Channel number | - | Channel 1 |
| CPICH_Ec/Ior | dB | -10 |
| PCCPCH_Ec/Ior | dB | -12 |
| SCH_Ec/Ior | dB | -12 |
| PICH_Ec/Ior | dB | -15 |
| DCH_Ec/Ior | dB | Note 1 |
| OCNS_Ec/Ior | dB | Note 2 |
| Îor/Ioc | dB | -1 |
| Ioc | dBm/ 3.84 MHz | -70 |
| CPICH_Ec/Io | dB | -13.54 |
| Propagation condition | - | AWGN |
| Note 1: The DPCH level is controlled by the power control loop. Note 2: The power of the OCNS channel that is added shall make the total power from the cell to be equal to Ior. |  |  |

Table A.9.1.12.1-3: E-UTRAN FDD RSRQ test parameters

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 2 | Cell 2 | Cell 2 |
| BWchannel |  | MHz | 10 | 10 | 10 |
| Measurement bandwidth |  |  | 22—27 | 22—27 | 22—27 |
| PDCCH/PCFICH/PHICH Reference measurement channel as defined in TS 36.133 A.3.1.2.1 |  |  | R.6 FDD | R.6 FDD | R.6 FDD |
| OCNG Pattern as defined in TS 36.133 A.3.2.1.2 |  |  | OP.2 FDD | OP.2 FDD | OP.2 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |
| Note2 | Band group FDD_A | dBm/15 kHz | -80 | -104.70 | -119.50 |
|  | Band groups FDD_B1 and FDD_B2 (Note 7) |  |  |  | -119 |
|  | Band groups FDD_E and FDD_F (Note 5) |  |  |  | -117.50 |
|  | Band group FDD_H |  |  |  | -116 |
|  | Band group FDD_D |  |  |  | -118 |
|  | Band group FDD_G |  |  |  | -116.50 |
|  | Band group FDD_C . |  |  |  | -118.50 |
|  |  | dB | -1.75 | -4.0 | -4.0 |
| RSRPNote3 | Band group FDD_A | dBm/15 kHz | -81.75 | -108.70 | -123.50 |
|  | Band groups FDD_B1 and FDD_B2 (Note 7) |  |  |  | -123 |
|  | Band groups FDD_E and FDD_F (Note 5) |  |  |  | -121.50 |
|  | Band group FDD_H |  |  |  | -120 |
|  | Band group FDD_D |  |  |  | -122 |
|  | Band group FDD_G |  |  |  | -120.50 |
|  | Band group FDD_ . |  |  |  | -122.50 |
| RSRQNote3 | Band group FDD_A | dB | -14.76 | -16.25 | -16.25 |
|  | Band groups FDD_B1 and FDD_B2 (Note 7) |  |  |  |  |
|  | Band groups FDD_H, FDD_E and FDD_F |  |  |  |  |
|  | Band group FDD_G |  |  |  |  |
|  | Band group FDD_C |  |  |  |  |
| IoNote3 | Band groups FDD_A | dBm/9 MHz | -50 | -75.46 | -90.26 |
|  | Band groups FDD_B1 and FDD_B2 (Note 7) |  |  |  | -89.76 |
|  | Band groups FDD_E and FDD_F (Note 5) |  |  |  | -88.26 |
|  | Band group FDD_H |  |  |  | -86.76 |
|  | Band group FDD_D |  |  |  | -88.76 |
|  | Band group FDD_G |  |  |  | -87.26 |
|  | Band group FDD_C. |  |  |  | -89.26 |
|  |  | dB | -1.75 | -4.0 | -4.0 |
| Propagation condition |  | - | AWGN | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For Band 26, the tests shall be performed with the assigned E-UTRA channel bandwidth within 865-894. Note 6: E-UTRA operating band groups are as defined in Section 3.5. Note 7: For Band 74, the tests shall be performed with the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

B.1.1. Conditions for measurements of inter-RAT E-UTRA cells

This section defines the inter-RAT E-UTRA RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot applicable for a corresponding operating band.

The conditions for measurements of inter-RAT E-UTRA cells are defined in Table B.1.1-1

Table B.1.1-1: Conditions for measurements of inter-RAT E-UTRA cells

| Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Band groups Conditions | Parameters |  |  |  |
|  | RSRP¦dBm | SCH_RP¦dBm | RSRP Ês/Iot | SCH Ês/Iot |
| FDD_A, TDD_A | -124 dBm | -124 dBm | -4 dB | -4dB |
| FDD_B1, FDD_B2 | -123.5 dBm (Note 2) | -123.5 dBm (Note 2) |  |  |
| FDD_C, TDD_C | -123 dBm | -123 dBm |  |  |
| FDD_E, TDD_E | -122 dBm | -122 dBm |  |  |
| FDD_G | -121 dBm | -121 dBm |  |  |
| FDD_H | -120.5 dBm | -120.5 dBm |  |  |
| FDD_F | -121.5 dBm (Note 1) | -121.5 dBm (Note 1) |  |  |
| FDD_D | -122.5 dBm | -122.5 dBm |  |  |
| Note 1: The condition is -122 dBm when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. Note 2: For Band 74, the condition is -124 dBm when the carrier frequency of the assigned E-UTRA channel bandwidth is within 1475.9-1510.9 MHz. |  |  |  |  |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

B.2.1. Conditions for E-UTRAN measurements

This section defines the E-UTRAN RSRP, RSRP Ês/Iot, SCH_RP and SCH RP/Iot applicable for a corresponding operating band.

The conditions for E-UTRAN measurements are defined in Table B.2.1-1.

Table B.2.1-1: Conditions for E-UTRAN measurements

| Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Band groups Conditions | Parameters |  |  |  |
|  | RSRP¦dBm | SCH_RP¦dBm | RSRP Ês/Iot | SCH Ês/Iot |
| FDD_A, TDD_A | -125 dBm | -125 dBm | -4 dB | -4dB |
| FDD_B1, FDD_B2 | -124.5 dBm (Note 2) | -124.5 dBm (Note 2) |  |  |
| FDD_C, TDD_C | -124 dBm | -124 dBm |  |  |
| FDD_E, TDD_E | -123 dBm | -123 dBm |  |  |
| FDD_G | -122 dBm | -122 dBm |  |  |
| FDD_H | -121.5 dBm | -121.5 dBm |  |  |
| FDD_F | -122.5 dBm (Note 1) | -122.5 dBm (Note 1) |  |  |
| FDD_D | -123.5 dBm | -123.5 dBm |  |  |
| Note 1: The condition is -123 dBm when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. Note 2: For Band 74, the condition is -125 dBm when the carrier frequency of the assigned E-UTRA channel bandwidth is within 1475.9-1510.9 MHz. |  |  |  |  |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

B.4.1. Conditions for identification of a new CGI of inter-RAT E-UTRA cell with autonomous gaps

This section defines the E-UTRAN SCH_RP|dBm and  SCH Ês/Iot  applicable for a corresponding operating band.

The conditions for E-UTRAN measurements are defined in Table B.4.1-1

Table B.4.1-1: Conditions for identification of a new CGI of inter-RAT E-UTRA cell with autonomous gaps

| Conditions |  |  |
| --- | --- | --- |
| E-UTRA band group | Parameter |  |
|  | SCH_RP¦dBm | SCH Ês/Iot |
| FDD_A, TDD_A | -125 dBm | -4 dB |
| FDD_B1, FDD_B2 | -124.5 dBm (Note 2) |  |
| FDD_C, TDD_C | -124 dBm |  |
| FDD_E, TDD_E | -123 dBm |  |
| FDD_G | -122 dBm |  |
| FDD_H | -121.5 dBm |  |
| FDD_F | -122.5 dBm (Note 1) |  |
| FDD_D | -123.5 dBm |  |
| Note 1: The condition is -123 dBm when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. Note 2: For Band 74, the condition is -125 dBm when the carrier frequency of the assigned E-UTRA channel bandwidth is within 1475.9-1510.9 MHz. |  |  |

----------------------------------------------------- end of Change ------------------------------------------------------------