3GPP TSG RAN Plenary Meeting #78	RP-172768

Lisboa, Portugal, 18th -21st December 2017

Source: 	Thales

Title:	NR-NTN: Spectrum for non terrestrial networks

TDOC Type:	pCR

Agenda Item:	9.3.1  (SID = sFS_NR_nonterr_nw)

Document for:	Approval

Release:	Rel-15

# Abstract

The objective of this document is to define the targeted spectrum for the 5G satellite and HAPS based networks.

# Proposed text for approval

It is proposed to add the following texts to TR 38.811 “Study on NR to support Non-Terrestrial Networks”.

* * * Start of changes * * * * (new text)

## 4.8 Spectrum

This clause identifies the frequency bands that can be used for NR via Satellite or HAPS in the different ITU regions which are here below depicted:

Figure : ITU regions and the dividing lines between them.   Region 1   Region 2   Region 3

Ka band spectrum for Satellite services via GEO space segment

The following bands are identified for use by GEO based Satellite services (Fixed and/or Mobile) on a worldwide basis

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (space to earth) | 17.3 – 20.2 GHz | 17.7 – 20.2 GHz | 17.7 – 20.2 GHz |
| Uplink (earth to space) | 27.5 – 30.0 GHz | 27.0 – 30.0 GHz | 27.0 – 30.0 GHz |

Ka band spectrum for Satellite services via Non GEO space segment

The following bands are identified for use by Non GEO based Satellite services (Fixed and/or Mobile) on a worldwide basis, subject to protection of GEO systems.

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (space to earth) | 17.3 – 20.2 GHz | 17.7 – 20.2 GHz | 17.7 – 20.2 GHz |
| Uplink (earth to space) | 27.5 – 29.1 GHz & 29.5 – 30.0 GHz | 27.0 – 29.1 GHz & 29.5 – 30.0 GHz | 27.0 – 29.1 GHz & 29.5 – 30.0 GHz |

S band spectrum for Satellite services via GEO or Non GEO space segment

The following bands are identified for use by Mobile Satellite services to provide International Mobile Telecommunications (IMT) on a worldwide basis

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (space to earth) | 2170-2200 MHz | 2160-2200 MHz | 2170-2200 MHz |
| Uplink (earth to space) | 1980-2010 MHz | 1980-2025 MHz | 1980-2010 MHz |

S band spectrum for HAPS based services

The following bands as identified by ITU may be used by HAPS as base stations to provide International Mobile Telecommunications (IMT) on a worldwide basis

In FDD mode

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (aerial to earth) | 2110-2170 MHz | 2110-2160 MHz | 2110-2170 MHz |
| Uplink (earth to aerial) | 1885-1980 MHz & 2010-2025 MHz | 1885-1980 MHz | 1885-1980 MHz & 2010-2025 MHz |

In TDD mode

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Up and Downlink (aerial to earth and earth to aerial) | 2110-2170 MHz | 2110-2160 MHz | 2110-2170 MHz |

Other bands

The following bands are also identified for use by GEO and in some cases by Non GEO based Satellite services (Fixed and/or Mobile) on a worldwide basis.

The following bands are referred as Q and V bands and is applicable to GEO and Non GEO systems, subject to protection of GEO systems.

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (space to earth) | 37.5 – 42.5 GHz, 47.5 - 47.9 GHz, 48.2-48.54 GHz & 49.44-50.2 GHz | 37.5 – 42.5 GHz | 37.5 – 42.5 GHz |
| Uplink (earth to space) | 42.5 – 43.5 GHz, 47.2 – 50.2 GHz & 50.4 – 51.4 GHz | 42.5 – 43.5 GHz, 47.2 – 50.2 GHz & 50.4 – 51.4 GHz | 42.5 – 43.5 GHz, 47.2 – 50.2 GHz & 50.4 – 51.4 GHz |

The following bands are referred as Ku bands and is applicable to GEO and Non GEO systems, subject to protection of GEO systems.

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (space to earth) | 10.7 - 12.75 GHz | 10.7 - 12.7 GHz | 10.7 - 12.75 GHz |
| Uplink (earth to space) | 12.75-13.25 GHz & 13.75-14.5 GHz | 12.75-13.25 GHz & 13.75-14.5 GHz | 12.75-13.25 GHz & 13.75-14.5 GHz |

Note 1: the use of the bands 12.75-13.25 GHz, 10.7-10.95 GHz and 11.2-11.45 GHz by GEO systems is subject to the provisions of ITU Radio Regulation Appendix 30B.

Note2: the use of the bands 11.7-12.5 GHz in Region 1, 12.2-12.7 GHz in Region 2 and 11.7-12.2 GHz in Region 3 by GEO systems is subject to the provisions of ITU Radio Regulation Appendix 30.

The following bands are referred as C band and is applicable to GEO systems. The sub-bands 3700-4200 MHz and 5925-6425 MHz may be used by Non GEO systems, subject to protection of GEO systems.

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (space to earth) | 3400-4200 MHz and 4500-4800 MHz | 3400-4200 MHz and 4500-4800 MHz | 3400-4200 MHz and 4500-4800 MHz |
| Uplink (earth to space) | 5725-7025 MHz | 5725-7025 MHz | 5725-7025 MHz |

Note: the use of the bands 4500-4800 MHz and  6725-7025 MHz by GEO systems is subject to the provisions of ITU Radio Regulation Appendix 30B.

* * * End of Changes * * * *