3GPP TSG RAN WG1 Meeting 92 		R1-1803115

Athens, Greece, February 26-March 2, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.211 | CR | 0424 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on DM-RS in DwPTS in special subframe configuration 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_UL_CAP_enh-Core |  |  |  |  |  |  | Date: |  | 2018-03-02 |
| Category: | A |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | DM-RS in DwPTS in special subframe configuration 10 for PDSCH and MPDCCH/EPDCCH are not specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add special subframe configuration 10 to the same DM-RS definitions as special subframe configuration 9. Update label of second row of Figure 6.10.3.2-3 to include special subframe configuration 10. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification of special subframe configuration 10. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.3.2, 6.10.3A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 6.10.3.2	Mapping to resource elements

<Unchanged parts are omitted>

For antenna ports , ,,, or , in a physical resource block with frequency-domain index  assigned for the corresponding PDSCH transmission, a part of the reference signal sequence  shall be mapped to complex-valued modulation symbols  in a subframe according to

Normal cyclic prefix:

where

wpi=wpim'+nPRBmod2=0wp3-im'+nPRBmod2=1
k=5m'+NscRBnPRB+k'
k'=1p∈7,8,11,130p∈9,10,12,14
l=l'mod2+2if in a special subframe with configuration 3, 4, 8, or 9, or 10 (see Table 4.2-1)l'mod2+2+3l'2if in a special subframe with configuration 1, 2, 6, or 7 (see Table 4.2-1)l'mod2+5if not in a special subframe
l'=0,1,2,3if nsmod2=0 and in a special subframe with configuration 1, 2, 6, or 7 (see Table 4.2-1)0,1if nsmod2=0 and not in a special subframe with configuration 1, 2, 6, or 7 (see Table 4.2-1)2,3if nsmod2=1 and not in a special subframe with configuration 1, 2, 6, or 7 (see Table 4.2-1)
m'=0,1,2

The sequence  is given by Table 6.10.3.2-1.

<Unchanged parts are omitted>

Figure 6.10.3.2-3 illustrates the resource elements used for UE-specific reference signals for normal cyclic prefix for antenna ports 7, 8, 9 and 10. Figure 6.10.3.2-4 illustrates the resource elements used for UE-specific reference signals for extended cyclic prefix for antenna ports 7, 8.

Figure 6.10.3.2-3: Mapping of UE-specific reference signals, antenna ports 7, 8, 9 and 10 (normal cyclic prefix)

<Unchanged parts are omitted>

#### 6.10.3A.2	Mapping to resource elements

For the antenna port  in a physical resource block  assigned for the associated EPDCCH/MPDCCH, a part of the reference signal sequence  shall be mapped to complex-valued modulation symbols  in a subframe according to

Normal cyclic prefix:

where

wpi=wpim'+nPRBmod2=0wp3-im'+nPRBmod2=1
k=5m'+NscRBnPRB+k'
k'=1p∈107, 1080p∈109, 110
l=l'mod2+2if in a special subframe with configuration 3, 4, 8, or 9, or 10 (see Table 4.2-1)l'mod2+2+3l'2if in a special subframe with configuration 1, 2, 6, or 7 (see Table 4.2-1)l'mod2+5if not in a special subframe
l'=0,1,2,3if nsmod2=0 and in a special subframe with configuration 1, 2, 6, or 7 (see Table 4.2-1)0,1if nsmod2=0 and not in a special subframe with configuration 1, 2, 6, or 7 (see Table 4.2-1)2,3if nsmod2=1 and not in a special subframe with configuration 1, 2, 6, or 7 (see Table 4.2-1)
m'=0,1,2

The sequence  is given by Table 6.10.3A.2-1.