3GPP TSG RAN Meeting #80                                                                    RP-180665

La Jolla, USA, 11th June – 14th June 2018

Source:	Thales, Ligado, HNS, ESA

Title:	NR-NTN: update on link budget analysis

TDOC Type:	pCR

Agenda Item:	9.4.1  (SID = sFS_NR_nonterr_nw)

Document for:	Approval

Release:	Rel-15

Specification:	TR 38.811 (SID = sFS_NR_nonterr_nw)

# Abstract

The objective of this document is to extend the link budget analysis to other deployment scenarios for NR operating via satellite.

1	Abstract	1

2	Proposed text for approval	2

Annex A: Link budget analysis for the different deployment scenarios	2

A1. Introduction	2

A1.1 Path loss function	2

A1.2 UE characteristics	4

A2.	Deployment scenario D1: GEO in Ka band	6

A2.1 Downlink performance	6

A2.2 Uplink performance	8

A3.	Deployment scenario D2 : GEO satellite in S band	12

A3.3.1 Downlink performance	12

A3.3.2 Uplink performance	13

A4. Deployment scenario D3 : LEO satellite at 600 km in S band	15

A4.1 Downlink performance	15

A4.2 Uplink performance	16

A5. Scenario D4 : LEO at 600 km in Ka band	19

A5.1 Downlink performance	19

A5.2 Uplink performance	21

A6. Scenario D5: HAPS at 20 km in S band	25

A6.1 Downlink performance	25

A6.2 Uplink performance	26

A7	Deployment scenario D2 : GEO satellite in L band	29

A7.1 Downlink performance	29

A7.2 Uplink performance	30

A8 : Performances assessment summary	32

A8.1 Ka-band deployment scenarios: D1 and D4	32

A8.2 S-band deployment scenarios: D2, D3 and D5	32

# Proposed text for approval

It is proposed to replace the existing texts of Annex A in TR 38.811 “Study on NR to support Non-Terrestrial Networks” v0.4.0 by the text here below.

* * * Start of changes * * * * (new text, replacing the previous text)

# Annex A: Link budget analysis for the different deployment scenarios

This annex provides examples of Link Budgets associated to the different deployment scenarios.

## A1. Introduction

### A1.1 Path loss function

This section provides a short overview of the path loss function used in the satellite link budgets.

The general formula for link budgets in free space between a transmitter and a receiver is provided here below, expressed in dB.

CN0+I0=-10×Log(10-0.1×CN0+10-0.1×CI0)

Inside the logarithm there are two terms:

The first term is the received carrier power over noise density ratio in dBHz called C/No, and the second term is the C/Io ratio which the carrier power to interference noise density ratio. For convenience this second term includes all the potential sources of interference: other beams interference, other systems interference, and intermodulation products interference.

The first term is the received power carrier to noise density ratio expressed in  dBHz

The second term C/Io corresponds to the carrier power over interference noise density, expressed in dBHz.

We have

- C/No : carrier power over noise density ratio in dBHz

- EIRP: useful transmitter EIRP

- G/T : figure of merit of the receiver, where G is the receiver antenna gain,a nd T the equivalent system temperature. The unit is dB/K

- Loss: free space loss (negative according to the definition)

- : shadowing margins; these are provisioned for shadowing effect for instance when dealing with satellite systems that can suffer from shaded conditions (trees)

- Aloss:  atmospheric losses : due to gases and rain fades: in S or L bands atmospheric losses are very small typically between 0.5 and 1 dB, but they can be very high in Ka band. They depend on the location of the terminal with respect to the satellite, the elevation angle. In this document we provide only clear sky conditions Link Budgets, with a typical atmospheric attenuation of 1 dB.

- : Additional losses, like degradation due to feeder links in case of non-regenerative systems. Typically we provision 0.5 dB feeder link degradation

- k: Boltzmann Constant: -10*LOG(k) =-228.6 dBW/K/Hz

In fact there is interference noise coming from other spot beams, other systems interference, and intermodulation products interference, and so we must consider at the entrance of the receiver the C/Io ratio which the carrier power to interference noise density ratio.

The overal C/( N0+I0) in dBHz can be computed from the C/N0 and C/I0 is provided above:

CN0+I0=-10×Log(10-0.1×CN0+10-0.1×CI0)

And the C/(N+I) can be computed as follow:

C/( N+I) = C/( N0+I0) -10xLOG (BW), where BW is the signal bandwidth expressed in Hz, and C/N is in dB.

The free space loss depends on the wavelength λ, and D the slant range which is the distance between the transmitter and the receiver (satellite and earth station). The free space loss can be computed as follow:

, and in dB LOSS =.

The Slant range D can be computed as follow.

D=-RT×sinα+(RT2×sinα2+HS2+2×RT×Hs)

Where

- RT: Earth Radius

- Hs: Satellite Altitude at nadir

- : elevation angle of the satellite from the earth station location.

All the link budgets presented all over this chapter compute a minimum required C/(N+I) to support the targeted data rate performance.

Note 1: the relationship  between G/T (commonly used in satellite system design) and the minimum Rx level and receiver’s antenna is:

Min_Rx_Level_At_antenna = - (Ga/T )+10*LOG(k)+10*LOG (BW)+ Required(C/N)

Where  BW is the signal bandwidth.

Note 2 : in the link budgets, the  Symbol rate is defined as the ratio  Ncarrier/ Ts. The Time Symbol includes CP length. It is expressed in Msymb/s.

Note 3 :in the Link Budgets in S band involving standard 3GPP UEs, some lines are now added to introduce the Noise Figure, and the system temperature T according to the following equation

Where

- Ga is the antenna gain in dBi

- NF : Noise Figure in dB

- T is called the system temperature with

- To : ambient temperature (290 K)

- Ta : antenna temperature

In the case of Class 3 UE, antenna gain is 0 dBi and antenna temperature is equal  to ambient temperature (typically for handheld terminals in S band deployment scenarios).

In the case of vehicle mounted terminal, antenna typical temperature is in the order of 150 K.

Note 4: concerning the Ka band deployment scenarios, where terminals are based on dishes or phase-array antenna, the use of the figure of merit (G/T) is maintained in the link budgets, as most of the terminals data sheets provide the G/T as part of the terminal radio electrical performances.

Note 5 : Receivers on board the satellite are also characterized by a figure of merit G/T instead of noise figure, antenna gain and ambient temperature.

### A1.2 UE characteristics

Table A1: Performances of the different UE types taken into account in the link budget analysis

|  | HH1 | HH2 | VM | VSAT |
| --- | --- | --- | --- | --- |
| Frequency band | S band | S band | S band | Ka band |
| Antenna gain | 0 dBi | 0 dBi | 4 dBi | - |
| EIRP | 23 dBm | 30 dBm | 37 dBm | +45 dBW |
| Transmit power | 200 mW | 1 W | 2 W | 2 W |
| G/T | - | - | - | 15.7 dB/K |
| Noise figure | 6 dB (Note 6) | 6 dB (Note 6) | 2 dB | - |
| Polarisation | Linear | Circular (possibly with dual linear polarised antenna) | Linear | Circular (dish or phased array antenna) |

|  | UT1 | UT2 | UT3 |
| --- | --- | --- | --- |
| Frequency band | L band | L band | L band |
| Antenna gain | 0 dBi | 3 dBi | 6 dBi |
| EIRP | 23 dBm | 30 dBm | 37 dBm |
| Transmit power | 200 mW | 1 W | 5 W |
| G/T | - | - | - |
| Noise figure | 4 - 6 dB | 4 - 6 dB | 4 -6 dB |
| Polarisation | LP/CP | LP/CP | LP/CP |

Note 6 : For handheld, isotropic antenna, a noise figure of 6 dB has been considered although the standard defines 7 dB. The reason for adopting 6 dB is that most products on the market feature such performance

All link budgets have been computed with a target BLER of 0.1%, considering the maximum block size allowed by the NR LDPC channel coding (i.e 3840 for code rates ≤ ¼ and 8448 otherwise).

## A2.	Deployment scenario D1: GEO in Ka band

The following tables present the downlink and uplink link budgets, for fixed terminal featuring an antenna with 60 cm equivalent aperture (dish or phased array antenna) and maximum 2  W Transmit power.

The radio interface configuration is provided in the different Link Budgets Tables.

On both directions, the impact of feeder links is supposed to be around 0,5 dB degradation.

### A2.1 Downlink performance

An example of the downlink link budgets is presented here below, and the graph presents the spectral efficiency versus the EIRP density.

Table 2.1-1: Example of GEO satellite Ka band Downlink link budgets for fixed terminal

| System performance target |  |  |
| --- | --- | --- |
| Spectral efficiency | bits/s/Hz | 2 |
| Information data rate (after decoding) | Mb/s | 396 |
| Radio interface configuration | Unit | Value |
| Down-link Carrier Frequency | GHz | 20,0 |
| Channel bandwidth | MHz | 198 |
| Sub Carrier Spacing (SCS) | kHz | 60 |
| Cyclic Prefix | µs | 1,2 |
| Total OFDM symbol duration | µs | 17,84 |
| Aggregate Modulation Symbol rate | Msymb/s | 185,0 |
| Terminal RF characteristics |  |  |
| G/T ( figure of merit see Note above) | dB/K | 15,7 |
| Antenna polarization |  | C |
| System assumptions |  |  |
| Elevation angle to satellite as seen from the UE | ° | 30,0 |
| Corresponding slant Range (distance between UE and satellite for the considered elevation angle) | Km | 38613,7 |
| Clear sky conditions |  |  |
| Satellite RF characteristics |  |  |
| Satellite nominal altitude | Km | 35788,0 |
| Typical average C/I within a satellite beam | dB | 16,0 |
| Satellite beam polarization |  | C |
| Link budget |  |  |
| Free space propagation loss | dB | -210,2 |
| Atmospheric loss (in clear sky conditions) | dB | 1 |
| Required C/(N+I) for the targeted data rate | dB | 6,700 |
| Required useful EIRP per channel | dBW | 57,7 |
| Required useful satellite EIRP density | dBW/MHz | 34,7 |

The figure below provides the required useful satellite EIRP density for a range of targeted spectral efficiency. This range of performances is typically supported by current and future satellites.

Figure 2.1-1: Spectral Efficiency versus the required useful satellite EIRP density – GEO in Ka band

### A2.2 Uplink performance

An example of the uplink link budgets is presented here below, and the graph  presents the spectral efficiency versus the satellite G/T.

The return link can use CP-OFDM or DFT-SOFDM.The first table is the results with DFT-SOFDM, the second with CP-OFDM.

Table2.2-1: Example of GEO satellite Ka band Uplink Link budgets with DFT-SOFDM

| System performance target |  |  |
| --- | --- | --- |
| Spectral efficiency | bits/s/Hz | 2 |
| Information data rate (after decoding) | Mb/s | 40.32 |
| Radio interface configuration |  |  |
| Up-link carrier frequency | GHz | 30,00000 |
| Channel Bandwidth | Mhz | 20.16 |
| Cyclic Prefix | µs | 2,3 |
| Total OFDM symbol duration | µs | 35,68 |
| Aggregate Modulation symbol rate | Msymb/s | 18.8 |
| Terminal RF characteristics |  |  |
| Max EIRP (corresponding to 2 W Transmission power) | dBW | 45 |
| System assumptions |  |  |
| Elevation angle to satellite | ° | 30,0 |
| Slant Range (distance between UE and satellite for the considered elevation angle) | km | 38613,7 |
| Clear sky conditions |  |  |
| Satellite RF characteristics |  |  |
| Nominal altitude | km | 35788,0 |
| Typical average C/I in a beam | dB | 16,0 |
| Link budget |  |  |
| Output back off in the terminal | dB | 1,0 |
| Terminal useful EIRP per channel | dBW | 44,0 |
| Free space propagation loss | dB | -213,7 |
| Atmospheric propagation loss (in clear sky conditions) | dB | 1,0 |
| Required C/(N +I) | dB | 6.7 |
| Required satellite G/T (corresponding to Rx sensitivity) | dB/K | 24,0 |

Table2.2-2: Example of GEO satellite Ka band Uplink Link budget with CP-OFDM

| System performance target |  |  |
| --- | --- | --- |
| Spectral efficiency | bits/s/Hz | 2,0 |
| Information data rate (after decoding) | Mb/s | 40.32 |
| Radio interface configuration |  |  |
| Up-link carrier frequency | GHz | 30 |
| Channel Bandwidth | Mhz | 20.16 |
| Cyclic Prefix | µs | 2,3 |
| Total OFDM symbol duration | µs | 35,68 |
| Aggregate Modulation symbol rate | Msymb/s | 8,1 |
| Terminal RF characteristics |  |  |
| Max EIRP (corresponding to 2 W Transmission power) | dBW | 45 |
| System assumptions |  |  |
| Elevation angle to satellite | ° | 30,0 |
| Slant Range (distance between UE and satellite for the considered elevation angle) | km | 38613,7 |
| Clear sky conditions |  |  |
| Satellite RF characteristics |  |  |
| Nominal altitude | km | 35788,0 |
| Typical average C/I in a beam | dB | 16,0 |
| Link budget |  |  |
| Output back off in the terminal | dB | 2,5 |
| Terminal useful EIRP per channel | dBW | 42,5 |
| Free space propagation loss | dB | -213,7 |
| Atmospheric propagation loss (in clear sky conditions) | dB | 1,0 |
| Required C/(N+I) | dB | 6,7 |
| Required satellite G/T (corresponding to Rx sensitivity) | dB/K | 25,0 |

The figure below provides the required satellite G/T for a range of targeted spectral efficiency. This range of performances is typically supported by current and future satellites.

Figure2.2-1: Spectral Efficiency versus the required satellite G/T – GEO in Ka band for DFTS-OFDM and CP-OFDM (20 MHz bandwidth)

## A3.	Deployment scenario D2 : GEO satellite in S band

The following tables present the downlink and uplink link budgets, for the different types of terminals listed in the section A1.

The radio interface configuration is provided in the different Link Budgets Tables.

On both directions, the impact of feeder links is supposed to create around 0,5 dB degradation.

### A3.3.1 Downlink performance

An example of the downlink link budgets is presented here below.

Table3.3.1-1: Example of GEO satellite S band Downlink link budgets for different kinds of terminals

| System performance target |  | HH1 | HH2 | VM |
| --- | --- | --- | --- | --- |
| Spectral efficiency | bits/s/Hz | 0.25 | 0.45 | 2 |
| Information data rate (after decoding) | Mb/s | 1.26 | 2.27 | 10 |
| Radio interface configuration | Unit |  |  |  |
| Down-link Carrier Frequency | (GHz) | 2.170 | 2.170 | 2.170 |
| Channel bandwidth | MHz | 5.04 | 5.04 | 5.04 |
| SCS | kHz | 15 | 15 | 15 |
| Cyclic Prefix | µs | 4.7 | 4.7 | 4.7 |
| Total OFDM symbol duration | µs | 71.36 | 71.36 | 71.36 |
| Number of PRBs |  | 28 | 28 | 28 |
| Aggregate Modulation Symbol rate | Msymb/s | 4.709 | 4.709 | 4.709 |
| Terminal RF characteristics |  |  |  |  |
| Antenna Gain | dBi | 0 | 0 | 4 |
| Noise Figure | dB | 6 | 6 | 2 |
| Antenna Temperature | K | 290 | 290 | 150 |
| Ambient Temperature | K | 290 | 290 | 290 |
| Antenna polarization (Circular or Linear) |  | L | C | C |
| System assumptions |  |  |  |  |
| Elevation angle to satellite as seen from the UE | ° | 30 | 30 | 30 |
| Corresponding slant Range (distance between UE and satellite for the considered elevation angle) | km | 38611.5 | 38611.5 | 38611.5 |
| Satellite RF characteristics |  |  |  |  |
| Satellite nominal altitude | km | 35786 | 35786 | 35786 |
| Typical average C/I within a satellite beam | dB | 16 | 16 | 16 |
| Satellite beam polarization |  | C | C | C |
| Link budget |  |  |  |  |
| Free space propagation loss | dB | -190.9 | -190.9 | -190.9 |
| Atmospheric loss | dB | -0,1 | -0,1 | -0,1 |
| Shadowing margins | dB | 10 | 10 | 10 |
| Required C/(N+I) for the targeted data rate | dB | -4.8 | -1.8 | 6.7 |
| Required useful EIRP per channel | dBW | 69 | 69 | 69 |
| Required useful satellite EIRP density | dBW/MHz | 62 | 62 | 62 |

Note : the link budgets include, when necessary, degradation due to polarization mismatch.

### A3.3.2 Uplink performance

| Table3.3.2-1: Example of GEO satellite S band Uplink Link budget for different kinds of terminalsSystem performance target |  | HH1 | HH2 | VM |
| --- | --- | --- | --- | --- |
| Spectral efficiency | bits/s/Hz | 1.25 | 1.125 | 1.125 |
| Information data rate (after decoding) | kb/s | 18.75 | 202.5 | 202.5 |
| Radio interface configuration |  |  |  |  |
| Up-link carrier frequency | GHz | 1.98 | 1.98 | 1.98 |
| Channel Bandwidth | khz | 15 | 180 | 180 |
| Number of PRB |  | 1 | 1 | 1 |
|  | (µs | 66,6667 | 66,6667 | 66,6667 |
| Cyclic Prefix | µs | 4,7 | 4,7 | 4,7 |
| Total OFDM symbol duration | µs | 71,36 | 71,36 | 71,36 |
| Aggregate Modulation symbol rate | ksymb/s | 14 | 168.2 | 168.2 |
| Terminal RF characteristics |  |  |  |  |
| Max EIRP | dBW | -10 | 0 | 7 |
| System assumptions |  |  |  |  |
| Elevation angle to satellite | ° | 30 | 30 | 30 |
| Slant Range (distance between UE and satellite for the considered elevation angle) | km | 38611.6 | 38611.6 | 38611.6 |
| Satellite RF characteristics |  |  |  |  |
| Nominal altitude | km | 35786 | 35786 | 35786 |
| Typical average C/I in a beam | dB | 16 | 16 | 16 |
| Link budget |  |  |  |  |
| Output back off in the terminal | dB | 0 | 0 | 0 |
| Terminal useful EIRP per channel | dBW | -10 | 0 | 7 |
| Free space propagation loss | dB | -190.1 | -190.1 | -190.1 |
| Atmospheric propagation loss (in clear sky conditions) | dB | -0.1 | -0.1 | -0.1 |
| Shadowing margins | dB | 3 | 3 | 10 |
| Required C/(N+I) | dB | -1,3 | -1,8 | 5,4 |
| Required satellite G/T (corresponding to Rx sensitivity) | dB/K | 20 | 20 | 20 |

## A4. Deployment scenario D3 : LEO satellite at 600 km in S band

The following tables present the downlink and uplink link budgets, for the different types of terminals listed in the section A1.

The radio interface configuration is provided in the different Link Budgets Tables.

On both directions, the impact of feeder links is supposed to be around 0,5 dB degradation.

### A4.1 Downlink performance

An example of the downlink link budgets is presented here below.

Table4.1-1: Example of LEO satellite S band Downlink link budgets for different kinds of terminals

| System performance target |  | HH1 | HH2 | VM |
| --- | --- | --- | --- | --- |
| Spectral efficiency | bits/s/Hz | 0.25 | 0.45 | 2 |
| Information data rate (after decoding) | Mb/s | 1.26 | 2.27 | 10 |
| Radio interface configuration | Unit |  |  |  |
| Down-link Carrier Frequency | (GHz) | 2.170 | 2.170 | 2.170 |
| Channel bandwidth | MHz | 5.04 | 5.04 | 5.04 |
| SCS | kHz | 15 | 15 | 15 |
| Cyclic Prefix | µs | 4.7 | 4.7 | 4.7 |
| Total OFDM symbol duration | µs | 71.36 | 71.36 | 71.36 |
| Number of PRBs |  | 28 | 28 | 28 |
| Aggregate Modulation Symbol rate | Msymb/s | 4.709 | 4.709 | 4.709 |
| Terminal RF characteristics |  |  |  |  |
| Antenna Gain | dBi | 0 | 0 | 4 |
| Noise Figure | dB | 6 | 6 | 2 |
| Antenna Temperature | K | 290 | 290 | 150 |
| Ambient Temperature | K | 290 | 290 | 290 |
| Antenna polarization (Circular or Linear) |  | L | C | C |
| System assumptions |  |  |  |  |
| Elevation angle to satellite as seen from the UE | ° | 30 | 30 | 30 |
| Corresponding slant Range (distance between UE and satellite for the considered elevation angle) | km | 1075,2 | 1075,2 | 1075,2 |
| Satellite RF characteristics |  |  |  |  |
| Satellite nominal altitude | km | 600 | 600 | 600 |
| Typical average C/I within a satellite beam | dB | 16 | 16 | 16 |
| Satellite beam polarization |  | C | C | C |
| Link budget |  |  |  |  |
| Free space propagation loss | dB | -159,8 | -159,8 | -159,8 |
| Atmospheric loss | dB | -0,1 | -0,1 | -0,1 |
| Shadowing margins | dB | 10 | 10 | 10 |
| Required C/(N+I) for the targeted data rate | dB | -4.8 | -2.8 | 6.7 |
| Required useful EIRP per channel | dBW | 37.8 | 37.8 | 37.8 |
| Required useful satellite EIRP density | dBW/MHz | 30.8 | 30.8 | 30.8 |

Note : the link budgets include, when necessary, degradation due to polarization mismatch.

### A4.2 Uplink performance

Table4.2-1: Example of LEO satellite S band Uplink Link budgets for different kinds of terminals

| System performance target |  | HH1 | HH2 | VM |
| --- | --- | --- | --- | --- |
| Spectral efficiency | bits/s/Hz | 0,2000 | 1,375 | 1,3750 |
| Information data rate (after decoding) | kb/s | 36,000 | 247.5,000 | 247,5 |
| Radio interface configuration |  |  |  |  |
| Up-link carrier frequency | GHz | 1.98 | 1.98 | 1.98 |
| Channel Bandwidth | khz | 180 | 180 | 180 |
| Number of PRB |  | 1 | 1 | 1 |
|  | (µs | 66,6667 | 66,6667 | 66,6667 |
| Cyclic Prefix | µs | 4,7 | 4,7 | 4,7 |
| Total OFDM symbol duration | µs | 71,36 | 71,36 | 71,36 |
| Aggregate Modulation symbol rate | ksymb/s | 168.2 | 168.2 | 168.2 |
| Terminal RF characteristics |  |  |  |  |
| Max EIRP | dBW | -10 | 0 | 7 |
| System assumptions |  |  |  |  |
| Elevation angle to satellite | ° | 30 | 30 | 30 |
| Slant Range (distance between UE and satellite for the considered elevation angle) | km | 1075,2 | 1075,2 | 1075,2 |
| Satellite RF characteristics |  |  |  |  |
| Nominal altitude | km | 600 | 600 | 600 |
| Typical average C/I in a beam | dB | 16 | 16 | 16 |
| Link budget |  |  |  |  |
| Output back off in the terminal | dB | 0 | 0 | 0 |
| Terminal useful EIRP per channel | dBW | -10 | 0 | 7 |
| Free space propagation loss | dB | --159 | 159 | 159 |
| Atmospheric propagation loss (in clear sky conditions) | dB | -0.1 | -0.1 | -0.1 |
| Shadowing margins | dB | 3 | 3 | 10,0 |
| Required C/(N+I) | dB | -4,8 | 5.4 | 5.4 |
| Required satellite G/T (corresponding to Rx sensitivity) | dB/K | -8.4 | -8.4 | -8.4 |

## A5. Scenario D4 : LEO at 600 km in Ka band

The following tables present the downlink and uplink link budgets, for fixed VSAT terminal in table A1.

The radio interface configuration is provided in the different Link Budgets Tables.

On both directions, the impact of feeder links is supposed to be around 0,5 dB degradation.

### A5.1 Downlink performance

An example of the downlink link budgets is presented here below, and the graph presents the spectral efficiency versus the EIRP density.

Table5.1-1: Example of LEO satellite Ka band Downlink link budgets for fixed terminal

| System performance target |  |  |
| --- | --- | --- |
| Spectral efficiency | bits/s/Hz | 2,0 |
| Information data rate (after decoding) | Mb/s | 396 |
| Radio interface configuration | Unit | Value |
| Down-link Carrier Frequency | GHz | 20,0 |
| Channel bandwidth | MHz | 198 |
| SCS | kHz | 60 |
| Cyclic Prefix | µs | 1,2 |
| Total OFDM symbol duration | µs | 17,84 |
| Aggregate Modulation Symbol rate | Msymb/s | 185,0 |
| Terminal RF characteristics |  |  |
| G/T ( figure of merit see Note above) | dB/K | 15,7 |
| Antenna polarization (Circular or Linear) |  | C |
| System assumptions |  |  |
| Elevation angle to satellite as seen from the UE | ° | 30,0 |
| Corresponding slant Range (distance between UE and satellite for the considered elevation angle) | km | 1075.2 |
| Clear sky conditions |  |  |
| Satellite RF characteristics |  |  |
| Satellite nominal altitude | km | 600 |
| Typical average C/I within a satellite beam | dB | 16,0 |
| Satellite beam polarization |  | C |
| Link budget |  |  |
| Free space propagation loss | dB | -179.1 |
| Atmospheric loss (in clear sky conditions) | dB | 1 |
| Required C/(N+I) for the targeted data rate | dB | 6.7 |
| Required useful EIRP per channel | dBW | 26.6 |
| Required useful satellite EIRP density | dBW/MHz | 3.6 |

The figure below provides the required useful satellite EIRP density for a range of targeted spectral efficiency. This range of performances is typically supported by current and future satellites.

Figure5.1-1: Spectral Efficiency versus the required useful satellite EIRP density – LEO in Ka band

### A5.2 Uplink performance

An example of the uplink link budgets is presented here below, and the graph  presents the spectral efficiency versus the satellite G/T.

The return link can use CP-OFDM or DFT-SOFDM.The first table is the results with DFT-SOFDM, the second with CP-OFDM.

Table5.2-1: Example of LEO satellite Ka band Uplink Link budgets with DFT-SOFDM

| System performance target |  | Fixed |
| --- | --- | --- |
| Spectral efficiency | bits/s/Hz | 2,0 |
| Information data rate (after decoding) | Mb/s | 40,32 |
| Radio interface configuration |  |  |
| Up-link carrier frequency | GHz | 30,0 |
| Channel Bandwidth | Mhz | 20.16 |
| Cyclic Prefix | µs | 2,3 |
| Total OFDM symbol duration | µs | 35,68 |
| Aggregate Modulation symbol rate | Msymb/s | 8,1 |
| Terminal RF characteristics |  |  |
| Max EIRP (corresponding to 2 W Transmission power) | dBW | 45 |
| System assumptions |  |  |
| Elevation angle to satellite | ° | 30,0 |
| Slant Range (distance between UE and satellite for the considered elevation angle) | km | 1075.2 |
| Clear sky conditions |  |  |
| Satellite RF characteristics |  |  |
| Nominal altitude | km | 600 |
| Typical average C/I in a beam | dB | 16,0 |
| Link budget |  |  |
| Output back off in the terminal | dB | 1,0 |
| Terminal useful EIRP per channel | dBW | 44,0 |
| Free space propagation loss | dB | -182.6 |
| Atmospheric propagation loss (in clear sky conditions) | dB | 1,0 |
| Required C/(N +I) | dB | 6.7 |
| Required satellite G/T (corresponding to Rx sensitivity) | dB/K | -7 |

Table5.2-2: Example of LEO satellite Ka band Uplink Link budget with CP-OFDM

| System performance target |  | Fixed |
| --- | --- | --- |
| Spectral efficiency | bits/s/Hz | 2,0 |
| Information data rate (after decoding) | Mb/s | 40.32 |
| Radio interface configuration |  |  |
| Up-link carrier frequency | GHz | 30, |
| Channel Bandwidth | Mhz | 20.16 |
| Cyclic Prefix | µs | 2,3 |
| Total OFDM symbol duration | µs | 35,68 |
| Aggregate Modulation symbol rate | Msymb/s | 8,1 |
| Terminal RF characteristics |  |  |
| Max EIRP (corresponding to 2 W Transmission power) | dBW | 45 |
| System assumptions |  |  |
| Elevation angle to satellite | ° | 30,0 |
| Slant Range (distance between UE and satellite for the considered elevation angle) | km | 1075.2 |
| Clear sky conditions |  |  |
| Satellite RF characteristics |  |  |
| Nominal altitude | km | 600 |
| Typical average C/I in a beam | dB | 16,0 |
| Link budget |  |  |
| Output back off in the terminal | dB | 2,5 |
| Terminal useful EIRP per channel | dBW | 42,5 |
| Free space propagation loss | dB | -182.6 |
| Atmospheric propagation loss (in clear sky conditions) | dB | 1,0 |
| Required C/(N+I) | dB | 6,7 |
| Required satellite G/T (corresponding to Rx sensitivity) | dB/K | -6 |

The figure below provides the required satellite G/T for a range of targeted spectral efficiency. This range of performances is typically supported by current and future satellites.

Figure5.2-1: Spectral Efficiency versus the required satellite G/T – LEO in Ka band for DFTS-OFDM and CP-OFDM (20 MHz)

## A6. Scenario D5: HAPS at 20 km in S band

The following tables present the downlink and uplink link budgets, for the different types of terminals listed in the table A1.

The radio interface configuration is provided in the different Link Budgets Tables.

We consider an On Board Processing Subsystem, which avoids feeder link degradation.

We consider  a reference elevation angle baseline of 30°.

We assume light indoor coverage conditions for handheld devices.

### A6.1 Downlink performance

Table6.1-1: Example of HAPS S band Downlink link budgets for different kinds of terminal and NR

| System performance target |  | HH1 | HH2 | VM |
| --- | --- | --- | --- | --- |
| Spectral efficiency | bits/s/Hz | 1,625 | 2,375 | 3,750 |
| Information data rate (after decoding) | Mb/s | 8,19 | 11,97 | 18,9 |
| Radio interface configuration | Unit |  |  |  |
| Down-link Carrier Frequency | (GHz) | 2.140 | 2.140 | 2.140 |
| Channel bandwidth | MHz | 5.04 | 5.04 | 5.04 |
| SCS | kHz | 15 | 15 | 15 |
| Cyclic Prefix | µs | 4.7 | 4.7 | 4.7 |
| Total OFDM symbol duration | µs | 71.36 | 71.36 | 71.36 |
| Number of PRBs |  | 28 | 28 | 28 |
| Aggregate Modulation Symbol rate | Msymb/s | 4.709 | 4.709 | 4.709 |
| Terminal RF characteristics |  |  |  |  |
| Antenna Gain | dBi | 0 | 0 | 4 |
| Noise Figure | dB | 6 | 6 | 2 |
| Antenna Temperature | K | 290 | 290 | 150 |
| Ambient Temperature | K | 290 | 290 | 290 |
| Antenna polarization (Circular or Linear) |  | L | C | C |
| System assumptions |  |  |  |  |
| Elevation angle to satellite as seen from the UE | ° | 30 | 30 | 30 |
| Corresponding slant Range (distance between UE and satellite for the considered elevation angle) | km | 40 | 40 | 40 |
| HAPS RF characteristics |  |  |  |  |
| HAPS nominal altitude | km | 20 | 20 | 20 |
| Typical average C/I within a satellite beam | dB | 16 | 16 | 16 |
| HAPS beam polarization |  | C | C | C |
| Link budget |  |  |  |  |
| Free space propagation loss | dB | -131,1 | -131,1 | -131,1 |
| Atmospheric loss | dB | -0,1 | -0,1 | -0,1 |
| Shadowing margins | dB | 10 | 10 | 10 |
| Required C/(N+I) for the targeted data rate | dB | 5,4 | 8,2 | 13,5 |
| Required useful EIRP per channel of 5 MHz | dBW | 19.5 | 19.5 | 19.5 |
| Required useful HAPS EIRP density | dBW/MHz | 12.5 | 12.5 | 12.5 |

### A6.2 Uplink performance

Table6.2-1: Example of HAPS in  S band Uplink Link budget with NR DFT-SOFDM for different kinds of terminals

| System performance target |  | HH1 | HH2 | VM |
| --- | --- | --- | --- | --- |
| Spectral efficiency | bits/s/Hz | 1,000 | 2,875 | 3,750 |
| Information data rate (after decoding) | Mb/s | 5,04 | 14,49 | 18,9 |
| Radio interface configuration |  |  |  |  |
| Up-link carrier frequency | GHz | 1.94 | 1.94 | 1.94 |
| Channel Bandwidth | Mhz | 5.04 | 5.04 | 5.04 |
| Number of PRB |  | 28 | 28 | 28 |
|  | (µs | 66,6667 | 66,6667 | 66,6667 |
| Cyclic Prefix | µs | 4,7 | 4,7 | 4,7 |
| Total OFDM symbol duration | µs | 71,36 | 71,36 | 71,36 |
| Aggregate Modulation symbol rate | Msymb/s | 4.709 | 4.709 | 4.709 |
| Terminal RF characteristics |  |  |  |  |
| Max EIRP | dBW | -10 | 0 | 7 |
| System assumptions |  |  |  |  |
| Elevation angle to satellite | ° | 30 | 30 | 30 |
| Slant Range (distance between UE and satellite for the considered elevation angle) | km | 40 | 40 | 40 |
| HAPS RF characteristics |  |  |  |  |
| Nominal altitude | km | 20 | 20 | 20 |
| Typical average C/I in a beam | dB | 16 | 16 | 16 |
| Link budget |  |  |  |  |
| Output back off in the terminal | dB | 0 | 0 | 0 |
| Terminal useful EIRP per channel | dBW | -10 | 0 | 7 |
| Free space propagation loss | dB | -130,2 | -130,2 | -130,2 |
| Atmospheric propagation loss (in clear sky conditions) | dB | -0.1 | -0.1 | -0.1 |
| Shadowing margins | dB | 10 | 10 | 10 |
| Required C/(N+I) | dB | 1,7 | 10,2 | 13,5 |
| Required HAPS G/T (corresponding to Rx sensitivity) | dB/K | -9 | -9 | -9 |

## A7	Deployment scenario D2 : GEO satellite in L band

The following tables present the downlink and uplink link budgets, for the different types of terminals listed in the section A1.

On both directions, the impact of feeder links is supposed to create around 0,5 dB degradation.

### A7.1 Downlink performance

An example of the downlink link budgets is presented here below.

Table 7.1-1: Example of GEO satellite L band Downlink link budgets for different kinds of terminals

| System performance target |  | UT1 | UT2 | UT3 |
| --- | --- | --- | --- | --- |
| Spectral efficiency | bits/s/Hz | 0.3 | 0.44 | 0.44 |
| Information data rate (IP) per carrier | kbps | 484 | 709 | 709 |
| Radio interface configuration | Unit |  |  |  |
| Down-link Carrier Frequency | (GHz) | 1.550 | 1.550 | 1.550 |
| Channel bandwidth | MHz | 1.4 | 1.4 | 1.4 |
| SCS | kHz | 15 | 15 | 15 |
| Number of PRBs |  | 6 | 6 | 6 |
| Terminal RF characteristics |  |  |  |  |
| Antenna Gain | dBi | 1 | 3 | 6 |
| Noise Figure | dB | 4 | 4 | 4 |
| Antenna polarization (Circular or Linear) |  | C | C | C |
| System assumptions |  |  |  |  |
| Average elevation angle to satellite as seen from the UT | ° | 30 | 30 | 30 |
| Corresponding slant Range (distance between UE and satellite for the considered elevation angle) | km | 38611.5 | 38611.5 | 38611.5 |
| Satellite RF characteristics |  |  |  |  |
| Satellite nominal altitude | km | 35786 | 35786 | 35786 |
| Average C/Io within a satellite beam | dB | 61 | 61 | 61 |
| Satellite transmit polarization |  | RHCP | RHCP | RHCP |
| Link budget |  |  |  |  |
| Free space propagation loss | dB | -188.8 | -188.8 | -188.8 |
| Other losses | dB | 1 | 1 | -0,1 |
| Average shadowing margin | dB | 3 | 3 | 3 |
| Required C/(No+Io) for the targeted data rate | dB | 58.3 | 60.7 | 60.7 |
| Average FL EIRP per carrier | dBW | 52 | 52 | 52 |

Note : the link budgets include, when necessary, degradation due to polarization mismatch.

### A7.2 Uplink performance

Table 7.2-1: Example of GEO satellite S band Uplink Link budget for different kinds of terminals

| System performance target |  | UT1 | UT2 | UT3 |
| --- | --- | --- | --- | --- |
| Spectral efficiency | bits/s/Hz | 0,25 | 0,25 | 0,25 |
| Information data rate (after decoding) | kb/s | 22 | 45 | 90 |
| Radio interface configuration |  |  |  |  |
| Up-link carrier frequency | GHz | 1.657 | 1.657 | 1.657 |
| Channel Bandwidth | kHz | 180 | 180 | 180 |
| Number of PRB |  | 1 | 1 | 1 |
| Terminal RF characteristics |  |  |  |  |
| Max EIRP | dBW | -7 | 0 | 7 |
| System assumptions |  |  |  |  |
| Elevation angle to satellite | ° | 30 | 30 | 30 |
| Slant Range (distance between UE and satellite for the considered elevation angle) | km | 38611.6 | 38611.6 | 38611.6 |
| Satellite RF characteristics |  |  |  |  |
| Nominal altitude | km | 35786 | 35786 | 35786 |
| Typical average C/I in a beam | dB | 16 | 16 | 16 |
| Link budget |  |  |  |  |
| Output back off in the terminal | dB | 0 | 0 | 0 |
| Terminal useful EIRP per channel | dBW | -7 | 0 | 7 |
| Free space propagation loss | dB | -189.3 | -189.3 | -189.3 |
| Atmospheric propagation loss (in clear sky conditions) | dB | -0.1 | -0.1 | -0.1 |
| Shadowing margins | dB | 3 | 3 | 3 |
| Required C/(No+Io) | dB | 50 | 52 | 54 |
| Average satellite G/T | dB/K | 20.5 | 20.5 | 20.5 |

## A8 : Performances assessment summary

### A8.1 Ka-band deployment scenarios: D1 and D4

For the Ka band deployment scenarios (GEO and LEO at 600 km), we summarise here under the minimum satellite payload performance requirements (EIRP density and figure of merit (G/T) for a spectral efficiency of 2.

It assumes for UE, a VSAT terminal which characteristics are listed in table A1.

Table8.1-1: Minimum Required Useful Satellite EIRP density (dBW/MHz) for a Spectral Efficiency of 2 bit/s/Hz

|  | GEO: 35786km | LEO: 600 km |
| --- | --- | --- |
| Minimum EIRP density (dBW/MHz) | 34.7 | 3.6 |

Table8.1-2: Minimum Required Useful Satellite G/T (dB/K) for a Spectral Efficiency of 2 bit/s/Hz

|  | GEO: 35786km DFT SOFDM | GEO: 35786km CP-OFDM | LEO: 600 km DFT SOFDM | LEO: 600 km CP-OFDM |
| --- | --- | --- | --- | --- |
| Minimum G/T (dB/K) | 24 | 25 | -7 | -6 |

### A8.2 S-band deployment scenarios: D2, D3 and D5

For the S band deployment scenarios (GEO, LEO at 600 km and HAPS), we summarise here under,  the minimum space or airborne payload performance requirements (EIRP density and figure of merit, G/T) for various targeted data rates on respectively down and uplink and various UE types.

Table8.2-1: Minimum Required Useful space and airborne EIRP density (dBW/MHz) per 5 MHz channel

|  | Downlink data rate for HH1 UE | Downlink data rate for HH2 & VM UE | Minimum Required EIRP density (dBW/MHz) |
| --- | --- | --- | --- |
| GEO: 36000 km | 1.25 Mbps | 2.27 & 10 Mbps (respectively) | +62 dBW/MHz |
| LEO: 600 km | 1.25 Mbps | 2.27 & 10 Mbps (respectively) | +31 dBW/MHz |
| HAPS: 20 km | 8.2 Mbps | 12 & 18.9 Mbps (respectively) | +12.5 dBW/MHz |

Table8.2-2: Minimum Required space and airborne G/T (in dB/K)

|  | Channel bandwidth | Uplink data rate for HH1 UE | Uplink data rate for HH2 & VM UE | Min required G/T |
| --- | --- | --- | --- | --- |
| GEO: 36000 km | 180 KHz channel | 18.75 kbps@15 kHz | 202.5& 202.5 kbps (respectively) | +20 dB/K |
| LEO: 600 km | 180 KHz channel | 36 kbps | 247.5 & 247.5 kbps (respectively) | -8.4 dB/K |
| HAPS: 20 km | 5 MHz channel | 5 Mbps | 14.5 & 18.9 Mbps (respectively) | -9 dB/K |

* * * End of Changes * * * *