3GPP TSG RAN Meeting 80                            	              RP-180680

La Jolla, CA, U.S.A., June 11st-14th, 2018

Agenda item:	9.3.1

Source: 	Qualcomm Incorporated

Title: 	Solutions for single Tx TDM

Document for:	Discussion/Decision

# Introduction

At RAN1 #93, proposals were discussed on introducing a new single Tx TDM mode for EN-DC with TDD PCell [2][3], even though just one meeting earlier, it was agreed not to introduce such feature in Rel-15.

In this contribution, we discuss the possible resolution to the new proposals.

# Discussion

The proposal in [3] was the following:

- For single UL operation in EN-DC with LTE TDD PCell, where UE operates on only one carrier at a time

  - for LTE carrier, UE can be configured with

    - Case 1: DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell

      - For DL HARQ timing, the DL-reference UL/DL configuration, i.e. “subframeAssignment-r15”, is applied

      - For UL HARQ timing, the PCell’s UL/DL configuration, i.e. “subframeAssignment”, is applied

      - For UE capable of dynamic power sharing, LTE UL transmission can be scheduled in all UL subframes and UpPTS configured by “subframeAssignment”

      - UE is not expected to transmit on the MCG and SCG simultaneously

    - Case 2: uses Release 15 LTE-TDD HARQ timing

      - For DL HARQ timing, the PCell’s UL/DL configuration, i.e. “subframeAssignment”, is applied

      - For UL HARQ timing, the PCell’s UL/DL configuration, i.e. “subframeAssignment”, is applied

The outcome of the discussion in RAN1 #93 was captured in [1] and is copied below:

Conclusion:

When the UE is configured with multiple UL carriers on different frequencies, but the UE operates only on one of the UL carriers at a given time among a pair of LTE and NR carriers, LTE operation with an LTE TDD PCell using the same HARQ timing as LTE operation without EN-DC is supported by the current RAN1 specifications at least when no LTE UL CA is used at least for the following cases

- The UE supports dynamic power sharing

- The UE is configured with P_LTE + P_NR not greater than P_cmax

i.e. only Case 2 was adopted.

We believe that there is no need to reopen the RAN1 decision; however, for the evolution in future releases, we think that all the previous proposals [2][3][4][5] and solutions can be further considered.

The previously discussed proposals are:

- Case 1a : TDM HARQ Case 1, operation A

- Case 1b : TDM HARQ Case 1, operation B

- Case 2 : TDM HARQ Case 2

- ‘Case 3’ : Hybrid TDM [3][5]

Case 3 can be further described as follows:

- LTE DL HARQ follows the eIMTA case for TDD and the FDD-TDD CA with TDD PCell case for FDD

  - LTE UL control is restricted to a subframe subset

- LTE UL HARQ is same as without EN-DC

  - All UL subframe can be used for data

- NR DL HARQ is same as without EN-DC

- NR UL HARQ is same as without EN-DC

Note that the proposal in [3] can be viewed as an adaptation of ‘Case 3’ to the TDD PCell case. An earlier proposal in [5] suggested adopting ‘Case 3’ for the FDD PCell case.

In the following, we give comparison of the various solutions:

|  | Case 1a (semi-static LTE, dynamic NR) | Case 1b (semi-static TDM) | Case 2 (dynamic TDM) | ‘Case 3’ (hybrid TDM) |
| --- | --- | --- | --- | --- |
| New LTE DL HARQ? | Yes | Yes | No | Yes |
| All LTE DL subframes useable? | Yes | Yes | No | Yes |
| New NR DL HARQ? | No | No | No | No |
| All NR DL slots useable? | Yes | Yes | Yes | Yes |
| New LTE UL HARQ? | Yes | Yes | No | No |
| LTE UL link budget loss? | Yes | Yes | No | No |
| New NR UL HARQ? | No | No | No | No |
| NR UL link budget loss? | No | Yes | No | No |

Note that green is ‘positive’, red is ‘negative’ in the table

As it can be seen from the Table above, Case 3 can efficiently combine the benefits of all other cases; therefore, it is an attractive option for operating single Tx TDM. However; we also think it is important that if Case 3 is introduced, it should be introduced both for FDD and TDD at the same time to maintain parity. Therefor we make the following proposal:

Proposal:

Consider adopting ‘Case 3’ in a future release, as long as it is adopted for both EN-DC with FDD PCell and EN-DC with TDD PCell at the same time.

# Conclusion

We have made the following proposal regarding the EN-DC TDM cases:

Proposal:

Consider adopting ‘Case 3’ in a future release, as long as it is adopted for both EN-DC with FDD PCell and EN-DC with TDD PCell at the same time.

# References

- “RAN1 Chair’s Notes”, RAN1 #93, May 25, 2018

- R1-1807281, “On single Tx transmission timing in TDD LTE-NR DC”, Nokia, Nokia Shanghai Bell

- R1-1807713, “WF on Single Tx in EN-DC with TDD PCell”, Apple

- R1-1807865, “Proposals on LTE-NR coexistence”, Qualcomm

- R1-1719194, “WF on UL TDM in single Tx EN-DC”, Qualcomm, Vodafone