3GPP TSG RAN Meeting #80                                                                    RP-181394

La Jolla, USA, 11th June – 14th June 2018				rev from RP-181380

Source:	Thales, HNS, Inter Digital, Nokia and Nokia Shanghai Bell, ESA, Heron, IAESI, Fraunhofer IIS

Title:	NR-NTN: TP for Chap 7.3 NR modifications to support NTN

TDOC Type:	pCR

Agenda Item:	9.4.1  (SID = sFS_NR_nonterr_nw)

Document for:	Approval

Release:	Rel-15

Specification:	TR 38.811 (SID = sFS_NR_nonterr_nw)

# Abstract

This document proposes a text for the clause 7.3 “NR modifications to support the Non-Terrestrial Network deployment scenarios” of the 3GPP TR 38.811 “Study on NR to support non-terrestrial networks”.

This text would replace alltogether the current agreed text in TR 38.811 v0.4.0

# Proposed Text

* * * Start of changes * * * * (adding references in red text)

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.101: "Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception (Release 14)".

[3]	3GPP TR 38.801: "Technical Specification Group Radio Access Network; Study on new radio access technology: Radio access architecture and interfaces (Release 14)".

[4]	3GPP TR 38.804: "Technical Specification Group Radio Access Network; Study on New Radio Access Technology; Radio Interface Protocol Aspects (Release 14)".

[5]	3GPP TR 38.913: "Study on Scenarios and Requirements for Next Generation Access Technologies (Release 14)".

[6]	3GPP TS 22.261: Service requirements for next generation new services and markets.

[7]	A. Guidotti et. al, “Satellite-enabled LTE systems in LEO Constellations”, 2017 IEEE International Conference on Communications (ICC), Paris, May 2017, pp. 876-881, doi: 10.1109/ICCW.2017.7962769

[8]	3GPP TR 38.802 v14.1.0, “Study on New Radio Access Technology Physical Layer Aspects (Release 14)“

[73-1]	3GPP TS 38.321 “Technical Specification Group Radio Access Network; NR; Medium Access Control (MAC) protocol specification (Release 15)”

[73-2]	3GPP TS 38.211, “Technical Specification Group Radio Access Network; NR; Physical channels and modulation (Release 15)”

[73-3]	3GPP TS 38.133 “Technical Specification Group Radio Access Network; NR; Requirements for support of radio resource management (Release 15)”

[73-4]	3GPP TS 38.214, “Technical Specification Group Radio Access Network; NR; Physical layer procedures for data (Release 15)”

[73-5]	3GPP TR 38.802 v14.1.0, “Study on New Radio Access Technology Physical Layer Aspects (Release 14)“, June 2016

[73-6] 	3GPP TS 36.101: "Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception (Release 14)".

[73-7] 	3GPP TS 38.101, “Technical Specification Group Radio Access Network; NR; User Equipment (UE) radio transmission and reception”.

[73-8] 	3GPP TS 36.104, “Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception”.

[73-9] 	3GPP TS 38.104, “Technical Specification Group Radio Access Network; NR; Base Station (BS) radio transmission and reception”.

[73-10]	O. Kodheli, A. Guidotti, and A. Vanelli-Coralli. “Integration of Satellites in 5G through LEO Constellations,” CoRR abs/1706.06013 (2017): pp. 1-6

[73-11]	R. Ali Ahmad, J. Lacan, F. Arnal, M. Gineste and L. Clarac, “Enhancing satellite system throughput using adaptive HARQ for delay tolerant services in mobile communications,” 2015 Wireless Telecommunications Symposium (WTS), New York, NY, 2015, pp. 1-7.

[73-12]	Tae Chul Hong, Kunseok Kang, Bon-Jun Ku, Dae-Ig Chang, “Receiver memory management method for HARQ in LTE-based satellite communication system,” International Journal of Satellite Communications and Networking, 2017, 35, 1, 3

[73-13]	R1-1802613, “NTN NR impacts on the HARQ Operation,” Fraunhofer, 3GPP TSG RAN1 meeting #92, Athens, Greece, Feb. 26 - Mar. 2, 2018.

[73-14]	3GPP TS 36.321, “Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification (Release 15)”, v1.0.0 (2017-12)

[73-15]	3GPP TS 38.212, “Technical Specification Group Radio Access Network; NR; Multiplexing and channel coding (Release 15)”

[73-16]	3GPP TS 38.300, “Technical Specification Group Radio Access Network; NR; NR and NG-RAN Overall Description; Stage 2”, v1.0.0 (2017-09)

[73-17] 	R1-1802632, “Considerations on random access for non-terrestrial networks”, Interdigital Inc., 3GPP TSG RAN WG1 Meeting #93, Busan, Korea, May 21st – 25th, 2018.

[73-18]	R1-1804236, “Discussion on the NR impacts on random access for NTN”, ZTE, 3GPP TSG RAN WG1 Meeting #93, Busan, Korea, May 21st – 25th, 2018.

[73-20]	R1-1806750, “Considerations on random access for NTN”, Samsung, 3GPP TSG RAN WG1 Meeting #93, Busan, Korea, May 21st – 25th, 2018.

[73-21]	R1-1807864 “NR-NTN: solution principles for NR to support non-terrestrial networks”, Thales et al, 3GPP TSG RAN Meeting #80, La Jolla, USA, June 11th – 14th, 2018.

[73-22]	R1-1804822, “On NTN initial access”, Qualcomm Incorporated, 3GPP TSG RAN WG1 Meeting #92bis, Sanya, China, April 16th – 20th, 2018.

[73-23]	R1-1806768 “Considerations on timing advance and random access for NTN”, Nokia, Nokia Shanghai Bell, 3GPP TSG RAN WG1 Meeting #93, Busan, Korea, May 21st – 25th, 2018.

[73-24]	ITU-R M.1225, “Guidelines for evaluation of radio transmission technologies for IMT-2000”, 1997

[73-25]	E. Lutz, M. Werner, and A. Jahn, “Satellite Systems for Personal and Broadband Communications.”, Berlin, Germany, Springer-Verlag, 2000.

[73-26]	T. Rappaport, “Wireless Communications”, New Jersey: Prentice Hall, 1996

[73-29]	DVB Document A171-2, Digital Video Broadcasting (DVB) Implementation guidelines for the second generation system for Broadcasting, Interactive Services, News Gathering and other broadband satellite applications; Part 2 - S2 Extensions (DVB-S2X), March 2015.

[73-30]	TR 38.803 Study on new radio access technology: Radio Frequency (RF) and co-existence aspects v14.2.0.

[73-31]	RP-180664, “NR-NTN: solution principles for NR to support non-terrestrial networks”, Thales et al, 3GPP TSG RAN Meeting #80, La Jolla, USA, June 11th – 14th, 2018.

[73-32]	R1-1804857, “Deactivating HARQ for Non-Terrestrial Networks,” InterDigital Inc, 3GPP TSG RAN WG1 Meeting #93, Busan, Korea, May 21st – 25th, 2018.

[73-33]	R1-1805848, “Consideration on HARQ Impact for NTN”, Nokia, Nokia Shanghai Bell, 3GPP TSG RAN WG1 Meeting #93, Busan, Korea, May 21st – 25th, 2018.

* * * End of changes * * * *

* * * Start of changes * * * * (adding references in red text)

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACM: 	Adaptive Modulation and Coding

AMF:	Access and Mobility Management Function

ARQ:	Automatic Repeat Request

AWGN:	Additive White Gaussian Noise

BLOS:	Beyond Line of Sight

BS:	Base Station

BW:	Bandwidth

C2:	Control and Command

CFO: 	Carrier Frequency Offset

CP:	Cyclic Prefix

CPE: 	Common Phase Error

EIRP: 	Equivalent Isotropically Radiated Power

eMBB:	enhanced Mobile Broadband

FDM: 	Frequency-Division Multiplexed

FOTA:	Firmware Over The Air services

FSS:	Fixed Satellite Services

GEO:	Geostationary Earth Orbiting

gNB:	next Generation Node B

GNSS: 	Global Navigation Satellite System

GSO	Geo Synchronous Orbit

GW:	Gateway

HARQ:	Hybrid Automatic Repeat Request

HD:	High Definition

HEO:	Highly Elliptical Orbiting

ICI:	Inter-Carrier Interference

IoT:	Internet of Things

KPI:	Key Performance Indicator

IMUX:	Input MUltipleXer

ISL:	Inter-Satellite Links

LEO:	Low Earth Orbiting

LMS:	Land Mobile Satellite

LoS:	Line of Sight

Mbps:	Mega bit per second

MEO:	Medium Earth Orbiting

mMTC:	massive Machine Type Communications

MS:	Mobile Services

MSS:	Mobile Satellite Services

NGSO:	Non Geostationary Satellite Orbit

NT:	Non Terrestrial

NTN: 	Non Terrestrial Network

OBO: 	Output Backoff

OMUX:	Output MUltipleXer

PA:	Power Amplifier

PAPR: 	Peak-to-Average Power Ratio

POI:	Point of Interest

PRACH: 	Physical Random Access Channel

PSS: 	Primary Synchronization Signal

RACH: 	Random Access Channel

RAN:	Radio Access Network

RAR: 	Random Access Response

RAT:	Radio Access Technology

RTT:	Round Trip Time

RRH:	Remote Radio Head

SCS: 	Subcarrier Spacing

SIB: 	System Information Block

SNR: 	Signal-to-Noise Ratio

SOTA:	Software Over The Air services

SSPA:	Solid-State Power Amplifier

SSS:	Secondary Synchronization Signal

TA: 	Timing Advance

TAG: 	Timing Advance Group

TV:	Television

UAS:	Unmanned Aerial System

UE:	User Equipment

URLCC:	Ultra-Reliable Low Latency Communications

* * * End of changes * * * *

* * * Start of changes * * * * (new text)

## 7.3	NR modifications to support the Non-Terrestrial Network deployment scenarios

For each deployment scenarios, the actual impacts on NR are identified.

### 7.3.1	Methodology

For each of the NR Features that may require adaptations to support NR operation via Satellite or HAPS, identified in chapter 7.2, the problem statement in terms of issue are assessed, potential areas of impact on NR protocol are identified.

Table 7.3.1-1: Areas of impacts on NR to support Non-Terrestrial networks

| Non-Terrestrial network specifics | Effects | Impacted NR features |
| --- | --- | --- |
| Motion of the space/aerial vehicles (especially for Non GEO based access network) | Moving cell pattern | Hand-over/paging |
|  | Delay variation | TA adjustment |
|  | Doppler | Init synchro downlink |
|  |  | DMRS time density |
| Altitude | Long latency | HARQ |
|  |  | MAC/RLC Procedures |
|  |  | Physical layer Procedures (ACM, power control) |
| Cell size | Differential delay | TA in Random access response message |
|  |  | RACH |
| Propagation channel | Impairments | DMRS frequency density |
|  |  | Cyclic prefix |
| Duplex mode | Regulatory constraints | Access scheme (TDD/FDD) |
| Satellite or aerial Payload performance | Phase noise impairment | PT-RS |
|  | Back-off | PAPR |

### 7.3.2	Motion of the space/aerial vehicles

#### 7.3.2.1 Hand-Over and paging

##### 7.3.2.1.1 Problem statement

NGSO satellites move rapidly with respect to any given UE location.  As an example, on a 2-hour orbit, a LEO satellite is in view of a stationary UE from horizon to horizon for about 20 minutes.  Since each LEO satellite may have many beams, the time such a UE stays within a beam is typically for only a few minutes.  The fast pace of change creates problems for paging as well as handoffs for a stationary UE as well as a moving UE.

Since handover happens in general when the UE or relay is in CM-ACTIVE and RRC-CONNECTED state, the procedure is time critical to avoid loss data. In NTN systems based on NGSO satellites, the cells or spot beams are moving at high speeds and so the handover procedure from one spot beam to the next or from one satellite to the next has to be executed quickly otherwise the UE may not make use the target beam and/or satellite resources efficiently and in the worst case may suffer loss of data.

NR beam management for mobility between spot-beams on the same base station cannot be ported to satellite to minimize the handoff overhead.  The NR beam management might assume same frequency on the adjacent beams, but for, the adjacent beams on the same satellite may use different frequencies or different polarization. Thus, the beam management procedures may have to be modified.

The problem for paging needs more detailed explanation.  In NR based cellular networks, a UE camps on a cell. The cell is uniquely identified by the RAN from which the UE is receiving the radio signals from.  A collection of cells is called a Tracking Area. A collection of Tracking Areas is called as a Registration Area. A cell belongs to a Tracking area and a Registration Area. As long as the UE stays within a Registration Area, no location update is needed. The UE in the CM-IDLE state will perform a Registration Area update when it moves out of a Tracking Area.

The AMF only needs be aware of the UE location to the granularity of Registration Area when a UE is in the CM-IDLE state. If a packet arrives from internet for this UE in CM-IDLE state, the AMF attempts to page the UE on all cells belonging to the Registration Area in order to notify the arrival of packets to it. All RANs that receive the page transmits a page in the corresponding cells to reach UE that may be anywhere in the Registration Area.

In Non-GEO satellite access network, a UE camps on a beam of a satellite, but as beams move, it ends up camping on different beams and different satellites over time even though UE may not have moved. Unlike terrestrial framework where a cell on the ground is tied to radio communication with a RAN, in Non-GEO satellite access network, the satellite beams are moving. There is no correspondence between cells on the ground and satellite beams. The same cell on the ground is covered by different satellites and different beams over time. Therefore, for the initial Registration, the Satellite based radio access network will not be able to provide the Tracking Area information to AMF based on which beam and which satellite the Registration Request was received.  Given that tracking areas are defined on the ground and Non-GEO beams are moving, there is no one-to-one correspondence between moving beams and fixed tracking areas or registration areas on the ground. However, this information is necessary for UE to determine if it needs to perform a registration area update with AMF in NR.

##### 7.3.2.1.2 Assessment of conditions for NR operation in Non-Terrestrial networks

For handover and paging to operate successfully and efficiently in the NGSO satellite networks, the NR UEs may need to be capable of geo-location.  One possibility is that the NR UE may be aware of their locations and may report this to the satellite RAN if needed.   (For fixed installations, their location can be reported once at time of installation.)

The ephemeris information of the NGSO satellites can be used to determine their footprints of each of the beams, and its velocity all the time. Therefore, for a given UE location at any given time, the network has information as to which beam of which satellite covers that location best.  It also knows the duration that UE location would remain to be covered by the beam and which beam on the same satellite, or a different satellite will be the best candidate to switch over next, and at what time. If positioning information of the UEs is available at the network, the possibility to simplify handover procedure and reduce measurement reporting overhead require further study.

##### 7.3.2.1.3 NR impact considerations

No impact for HAPS or GEO satellite based NTN.  For Non-GEO satellite based NTN, adaptation of the NR handover and paging protocols needs further study possibly taking advantage of the knowledge of the UE location and satellite ephemeris information.

#### 7.3.2.2 TA adjustment

##### 7.3.2.2.1 Problem statement

MEO, LEO and HAPS systems feature a strong varying delay because satellite/HAPS and UE are fast-moving and are not relatively static. In this case, the individual timing advances of the UEs may need to be dynamically updated and appropriate TA index values may be needed to solve the long strong delay in the overall distance of the propagation on NTN link.

The issues or technical problems to solve, related to TA alignment in Satellite communications, are as follows:

- A strong delay variation is caused by moving satellites generating a fast change in the overall distance of the propagation from UE over Satellite to BS.

- The delay is much longer over a satellite link than one TTI

A strong delay variation is caused by moving satellites generating a fast change in the overall distance of the radio link between UE and BS via satellite. The delay is much higher and variable over a satellite radio link than over a terrestrial radio link. This delay largely exceeds the TTI (Equivalent to one frame) of NR which is equal to or less than 1 ms. However, the delay variation is quite predictable knowing the satellite orbits and UE position.

Hence, TA alignment is an important feature of NR that will be impacted by introduction of NTN in 5G to ensure that all uplink transmissions are synchronized at gNB reception point.

##### 7.3.2.2.2 Assessment of conditions for NR operation in Non-Terrestrial networks

A timing advance command [73-1], , for a TAG indicates adjustment of the current  value, , to the new  value, , by index values of  = 0, 1, 2,..., 63, where for a subcarrier spacing of  kHz, .

As shown in Figure 7.3.2.2-1, transmission of uplink frame number i from the UE shall start TTA=NTA+NTA, offset×TC before the start of the corresponding downlink frame i at the UE, where NTA can be derived by the UE based on the index value TA from gNB, NTA, offset depends on the duplex mode and frequency range in uplink transmission, and TC is the basic timing unit [73-2][ 73-3][ 73-4].

Figure 7.3.2.2-1: Time alignment at gNB with TA

In case of satellite, the high delay drift of individual non-GEO satellites is quite predictable because the motion of the satellites follows known paths. The very fast update of the TA is neither required in terrestrial links, nor in GEO satellite links. In both scenarios, the terminal mobility is dominating the TA requirements.

Another technical issue that arises is that the delay variation over the satellite link is much more than a TTI. E.g. if the SCS is increasing from 15 kHz to 60 kHz, the TTI goes down from 1 ms to 250 μs. The required TA adjustment range for satellite links will become larger that the TTI with any SCS selection and the transmission timing of the UE has to be adjusted over the borders of individual TTIs.

Table 7.3.2.2-1: TA granularity, and step size with SCS.

| Subcarrier spacing (SCS) configuration parameter, µ | SCS [kHz] | RB band-width [kHz] | TA gran-ularity [Ts] | Tstep [ns] | Max Ta step size [µs] |
| --- | --- | --- | --- | --- | --- |
| 0 | 15 | 180 | 1024 | 520.83 | 16,6 |
| 1 | 30 | 360 | 512 | 260.42 | 8,3 |
| 2 | 60 | 720 | 256 | 130.21 | 4,15 |
| 3 | 120 | 1440 | 128 | 65.10 | 2,1 |
| 4 | 240 | 2880 | 64 | 32.55 | 1 |

This max Ta step should be valid only for extended cyclic prefix. With normal cyclic prefix, we expect this maximum step not to exceed the normal CP length (e.g. CP length is 4.7 µs in 15kHz SCS case). It means that the number of Ta commands to be sent per second to track the maximum drift of 35 µs/s is about 10 per second for 15 kHz SCS case, 40 per second for 60 kHz SCS, and 80 per second for 120 kHz SCS case. The number of Ta commands to be sent per second is important but can be implemented.

##### 7.3.2.2.3 NR impact considerations

Solutions need to be studied to ensure alignment of uplink signals over the NTN links to overcome the predictable delay of NTN.

#### 7.3.2.3 Initial synchronization in downlink

##### 7.3.2.3.1 Problem statement

In order to access the 5G network, the UE has to detect the PSS and the SSS. Those synchronization signals allow time and frequency correction, and Cell Id detection. UE in cellular network has to get good one-shot detection probability at -6 dB received baseband SNR condition with less than 1% false alarm rate as defined in chapter 7.1.5 of [73-5], with robustness against initial frequency offset up to 5 ppm.

We expect that requirements (SNR level of -6 dB, frequency error robustness of 5 ppm and 1% false alarm) defined for terrestrial UE will be kept the same for NTN UE. Even if  the SNR level of NTN systems is typically in the range of -3 to 13 dB SNR.

In cellular networks, transmission equipment (gNB or RRH) are usually fixed, except when on board a moving platform such as a train.

In Non-Terrestrial Networks, the transmission equipment is on board the satellite/HAPS.

- For geostationary systems, the transmission equipment is quasi static with respect to the UE with only small Doppler shift.

- For HAPS, the transmission equipment is moving around or across a theoretical central point but creates small Doppler shift.

- For non-geostationary systems, the satellites move relative to the earth and creates higher Doppler shift than for geostationary systems.

The Doppler shift depends on the relative satellite/HAPS velocity with respect to the UE, and on the frequency band.

In term of Doppler shift, the worst case for NTN systems corresponds to non-geostationary systems, at lowest altitude (i.e. 600 km), where the speed of the satellite embedding transmission equipment is 7.5 km/s.

As detailed in clause 5.3, assuming a worst case NTN terminal velocity of 1000 km/h:

- For LEO in S band (2 GHz): Up to +/- 48kHz Doppler Shift in downlink for the whole satellite coverage (spot)

- For LEO in Ka band (20 GHz): Up to +/- 480kHz Doppler Shift in downlink for the whole satellite coverage (spot)

If frequency error robustness requirement is 5 ppm (i.e. 10kHz for S band, 100 kHz for Ka band), it means that this worst case described above is not covered by current 5G specifications. To be inferior or equal to 5ppm error, the satellite altitude has to be above 13000 km.

Actually, the Doppler shift amplitude to be compensated is less than 48 kHz for S band and 480 kHz for Ka band, because in satellite communication systems, all satellites whether GSO or NGSO generate multi beams and each beam foot print corresponds to a cell. The Max Doppler shift amplitude observed at the satellite coverage foot print edges will be reduced within each beam footprint. The lower the beam width, the less the Doppler shift amplitude.

##### 7.3.2.3.2 Assessment of conditions for NR operation in Non-Terrestrial networks

In case the altitude of the satellite is above a certain altitude (about 13 000 km) or if center beam pre-compensation is sufficient to reach the 5 ppm requirements, no impact is foreseen.

##### 7.3.2.3.3 NR impact considerations

In case the above conditions are not met, further studies are required to accommodate the high Doppler shift during the cell synchronization procedure in non-Terrestrial networks.

Note that this high Doppler shift can be known based on UE location and satellite ephemeris and hence could be for example pre compensated. This could prevent impact on the NR but may need to be confirmed with further study.

#### 7.3.2.4 DMRS time density

##### 7.3.2.4.1 Problem statement

As analyzed in Section 5.3, the maximum Doppler variation rates during the field tests appeared in the scenarios of LEO at the altitude of 600 km, and are -544 Hz/s with carrier frequency of 2 GHz, -5.44 kHz/s with carrier frequency of 20Ghz and -8.16 kHz/s with carrier frequency of 30 GHz, where the maximum Doppler variation rate is the maximum slope (maximum derivative) of the Doppler shift curves. In NR network, there are up to 4 DMRS symbols per slot and the duration of one slot can be configured to 1 ms, 0.5 ms, 0.25 ms, 0.125 ms or 0.0625 ms, which correspond to the subcarrier spacing of 15 kHz, 30 kHz, 60 kHz, 120 kHz and 240 kHz respectively, as shown in Figure 7.3.2.4-1. Table 7.3.2.4-1 summarizes the potential maximum Doppler shift in one slot in NTN based on the observed maximum Doppler variation rate during the field tests. As shown in Table 7.3.2.4-1, it is observed that the potential maximum Doppler shifts are up to 0.544 Hz with carrier frequency of 2 GHz in DL/UL, up to 5.44 Hz with carrier frequency of 20 GHz in DL and up to 8.16 Hz with carrier frequency of 30 GHz in UL.

Figure 7.3.2.4-1: Slot duration depending on µ configuration

Table 7.3.2.4-1 Potential maximum Doppler Shift in one slot in NTN

| Carrier frequency when the altitude of LEO satellite is 600km | Maximum Doppler variation (Hz/s) | Duration of one slot (ms) | Potential maximum Doppler shift in one slot (Hz) |
| --- | --- | --- | --- |
| 2 GHz (DL/UL) | - 544 | 1 | 0.544 |
|  |  | 0.5 | 0.272 |
|  |  | 0.25 | 0.136 |
|  |  | 0.125 | 0.068 |
|  |  | 0.0625 | 0.034 |
| 20 GHz (DL) | -5440 | 1 | 5.44 |
|  |  | 0.5 | 2.72 |
|  |  | 0.25 | 1.36 |
|  |  | 0.125 | 0.68 |
|  |  | 0.0625 | 0.34 |
| 30 GHz (UL) | -8160 | 1 | 8.16 |
|  |  | 0.5 | 4.08 |
|  |  | 0.25 | 2.04 |
|  |  | 0.125 | 1.02 |
|  |  | 0.0625 | 0.51 |

In the existing LTE network, the frequency error at the UE side shall be within ±0.1 PPM observed over a period of 0.5 ms [73-6]. In NR network, the frequency error at the UE side shall be within ±0.1 PPM observed over a period of 1 ms[73-7].

##### 7.3.2.4.2 Assessment of conditions for NR operation in Non-Terrestrial networks

The frequency error corresponding to ±0.1 PPM is ±200 Hz with carrier frequency of 2 GHz, ±2 kHz with carrier frequency of 20 GHz and ±3 kHz with carrier frequency of 30 GHz. Compared to the requirement of frequency error at the UE side, the maximum Doppler shift in one slot in LEO scenarios during field test is negligible, see Table 7.3.2.4-1. This result is also valid for other configurations with higher DMRS time density per time slot as configurable in 5G NR. Similar conclusion could be observed at the eNB side too [73-8][ 73-9].

##### 7.3.2.4.3 NR impact considerations

As a conclusion, no impact on NR specifications is needed for DMRS positioning in time in NTN deployment scenarios because the maximum Doppler Variation in NTN is negligible compared to the minimum requirement of frequency error at both gNB and UE sides in NR.

### 7.3.3	Altitude of the space/aerial vehicles

#### 7.3.3.1 HARQ

The HARQ process is a very time-critical mechanism. HARQ operation becomes even more critical at extremely long RTT, i.e., as in case of NTN and extreme coverage scenarios.

In satellite communication, the RTT normally exceeds the maximum conventional HARQ timers (after which an ACK is received) or the maximum possible number of HARQ processes (i.e., a flexible pool of parallel HARQ processes similar to LTE). This is also true even for LEO constellations, where the RTT varies between 15 to 63 times longer than that of the terrestrial RTT [73-10]. Thereby, simply extending the number of HARQ processes linearly to RTT induced by the satellite channel might not be feasible for some UEs due to memory restrictions and the maximum possible parallel processing channels [73-11, 73-12, 73-13, 73-32, 73-33]. Furthermore, gNBs also have to consider this latency impact on the number of their active HARQ processes. Therefore, it is important to study the impact on NR HARQ operation for the introduced NTN delays. Figure 7.3.3.1-1 depicts the HARQ feedback RTT, i.e., from data retransmission time up to acknowledge reception at a bent-pipe satellite. The figure also shows the backhauling delay Tp1 as well as the forward/reverse satellite link delay Tp2, i.e., where the RTT = 2(Tp1+Tp2)=2*Tp.

Figure 7.3.3.1-1: Bent-pipe Satellite HARQ feedback operation with a maximum RTT (from sending a NACK until receiving a retransmission redundancy version (RV))

7.3.3.1.1 Problem statement

NR has extended the number of HARQ processes in Rel. 15 to 16 processes [73-14]. For NR NTN satellite transmission, the number of HARQ processes may need to be further extended flexibly according to the induced RTT delay. Here, the minimum required number of HARQ processes can be computed directly from the RTT delay of each satellite constellation, e.g., LEO, MEO and GEO, using the following formula [73-10]

NHARQ, min≥THARQTslot ,  	(7.3.3.1-1)

where NHARQ, min is the minimum required number of HARQ processes, Tslot is 1ms assuming a reference numerology 15 kHz subcarrier spacing, and THARQ is the time duration between the initial transmission of one transport block (TB) and the corresponding ACK/NACK complete decoding.

The THARQ is depicted in Figure 7.3.3.1-2 considering the RTT (2*Tp), transmission time (2*Tslot), and processing time, Tproc1 and Tproc2, for decoding the TB and the ACK/NACK frame, respectively. THARQ is illustrated in details in Figure 7.3.3.1-2.

Figure 7.3.3.1-2: Timing diagram of a single HARQ process for a NTN with a single bent-pipe satellite in the link

Table 7.3.3.1-1 gives an overview of the number of HARQ processes, NHARQ, min, based on different THARQ values (including the RTT) for different satellite constellations, LEO, MEO, and GEO [5].

Table 7.3.3.1-1: The minimum required number of the HARQ processes, NHARQ, min, assuming a 1ms slot duration for 15 kHz* reference subcarrier-spacing

| constellation | Max. THARQ | NHARQ, min processes for 1 ms slot operation | UE side feasibility |
| --- | --- | --- | --- |
| Terrestrial | 16ms | 16 | Feasible (Rel. 15) |
| LEO | 50ms | 50 | Feasible (with HARQ extension) |
| MEO | 180ms | 180 | FFS (impact on TBS/MCS) |
| GEO/HEO | 600ms | 600 | FFS (impact on TBS/MCS) |

Note*: For larger subcarrier spacing (SCS), i.e., 2k * 15 kHz, the min. required number of the HARQ processes might be scaled by 2k.

7.3.3.1.2 Assessment of conditions for NR operation in non-terrestrial networks

Handling a higher number of parallel HARQ processes and its feasibility for NTN have been addressed in [73-10, 73-11, 73-31]. The impacts of NTN delays on the existing HARQ-supporting mechanisms need to be addressed in a subsequent study.

At least the following principles can be considered in further study:

- Enhancing existing HARQ operation to extend the HARQ processing accommodating low to moderate NTN RTT delays.

- Limiting HARQ capabilities and/or disabling HARQ for long RTT delays.

7.3.3.1.3 NR impact considerations

It is required to study the impact on NR HARQ operation due to the long RTT delay of a non-terrestrial network. The impacts should be considered as well for the NTN UEs and serving gNBs, when the number of HARQ processes is either extended to satisfy high reliability scenarios or limited/disabled for longer NTN delays.

#### 7.3.3.2 MAC/RLC procedures

##### 7.3.3.2.1 Problem statement

UE operating in GEO satellite access networks can experience a one-way propagation time of 240 ms at the minimum, 270 ms at the maximum between UE and satellite base station.  The base station can observe the acknowledgement of packets sent to the UE only after the round trip plus some processing time, which is more than ½ second later.  Similarly the UE can observe acknowledgement for its packet sent to the base station in about the same time interval.  For the LEO satellite systems with typical 600 km orbit, the one way propagation delay changes continuously between 2 ms when the satellite is directly above, and 7 ms when the satellite is seen with 10° elevation.

ARQ requires that the transmitted packets be buffered in anticipation of potential packet loss and released only after the successful receipt of an acknowledgement, or until a time-out mechanism reinitiating a retransmission. The long round trip delay requires larger transmission buffer, and potentially limits the number of retransmission allowed for each transmitted packet in both the forward and return links. Note that in LEO satellite systems, the ARQ transmit buffer size, and retransmission mechanism must be designed for the longest possible delay, i.e. at the lowest elevation.

Scheduling mechanisms must be able to cope with the long RTT. UL scheduling delay parameters are expected to be redefined to accommodate the RTT of the associated deployment scenario.

##### 7.3.3.2.2 Assessment of conditions for NR operation in Non-Terrestrial networks

For efficient ARQ operation in GEO or NGSO satellite networks, NR UE and base stations must size their transmission buffer and the retransmission time-out mechanism according to the longest round-trip delay to be anticipated.  The number of retransmissions allowed before a packet is dropped from the retransmission buffer may also be adjusted.

UL scheduling delay parameters are expected to be redefined to accommodate the RTT of the associated deployment scenario.

##### 7.3.3.2.3 NR impact considerations

No impacts on the ARQ protocol itself, except parameter sizing needs to account for the longer delay of the NTN network.

#### 7.3.3.3 Physical layer procedures (ACM, power control)

##### 7.3.3.3.1 Problem statement

As mentioned in section 7.3.3.2.1, UE operating in GEO satellite access networks can experience a one-way propagation time up to 270 msec. Using LEO satellite access network with 600 km orbit, the one way propagation delay changes continuously between 2 ms, and 7 ms. The slow reaction time is expected to have performance impact on some of the physical layer procedures particularly those with close loops such as power control and ACM.

##### 7.3.3.3.2 Assessment of conditions for NR operation in Non-Terrestrial networks

While slower reaction on the control loops affects the performance of all the control loops between UE and base station, most of them require some adjustments in implementation, but not fundamentally different design.

While the link margin may be different for specific links and systems depending on applications, satellite power is typically at a premium. Due to the large free space loss and limited EIRP and battery power available at UE, power margin is also limited for mobile terminals.  Thus, a very limited amount of power control, if at all, is available for the GEO satellite links. Due to the long delay in the loop, the power control is not expected to track fast fading, but may be used to track slower power variations.

For Ka-band satellites, ACM is an essential tool that maintains connection through rain fades, which typically changes somewhat slower than the ½ second round trip delay.  It generally works well with some hysteresis to avoid excessive oscillations between two ACM modulation coding modes.  But, this reaction time is too slow for ACM to adapt for changes of signal strength for mobile terminals when line of sight is interrupted by shadowing.

For GEO systems in S-band, the main issue is multipath fading, which can be much faster than ½ second round trip delay.  As such, ACM will not be able to follow it.  ACM algorithm typically attempts to settle on a modulation coding mode that closes the link if possible, by giving up some power to maintain a margin.

For LEO satellites, ACM may also be used to adapt for the large variation of free space loss.  The variation is sufficiently slow compared to the 20 ms worst case round trip delay.  It should also be able to react to shadowing fades to a large extent, but still unable to follow fast fading.

##### 7.3.3.3.3 NR impact considerations

Further studies need to be performed to define the required margin for power and ACM control loops to accommodate to the long RTT.

### 7.3.4	Cell size (Beam foot print)

#### 7.3.4.1 PRACH and Random access

##### 7.3.4.1.1 Problem statement

The RTT in NTN can be much larger than the RTT in terrestrial network as analyzed in Section 5.3. Therefore, it is necessary to consider its impact on PRACH and random access procedure. As shown in Figure 7.3.4.1-1, for one given beam covering a cell, there is one common propagation delay for all served UEs and one relative propagation delay for each served UE. If the common propagation delay can be compensated, then the NTN PRACH signal design will depend on the relative propagation delay, which is limited up to 200km in current specifications regarding TA range. However even if only the difference value left, a NTN PRACH signal design and PRACH procedure design are still needed since the common propagation delay could be thousands of kilometers which cannot be ignored.

Figure 7.3.4.1-1: NTN systems, new geometry and the cell size.

7.3.4.1.2 Assessment of conditions for NR operation in Non-Terrestrial networks

Random Access Response

For non-terrestrial networks, the round-trip time can be much larger than the round-trip time in terrestrial networks (up to 600 ms for the case of GEO satellites, with bent pipe architecture). However the current window for the PRACH response in NR, which starts at  symbols after transmitting the last symbol of the preamble and has the size of “rar-WindowLength”, cannot cover this round-trip time [73-17]. Therefore, the random access response window length in NR should be revisited to accommodate the round-trip time of NTN. However, extending the RA response window size in the existing procedure introduces unnecessary UE monitoring intervals thus more power saving due to large propagation delay in NTNs. Therefore, the solution to handle long propagation delay with the consideration of power saving at the UE side needs to be further studied.

PRACH Sequence and Format

Considering that the cell size of NTN HAPS is not extremely large, the current NR preamble format design should be enough to support HAPS. The current NR preamble format design should be enough to support HAPS but for Satellite it needs to be revisited [73-17][73-18][73-4][ 73-20][73-21]. As shown in Figure 7.3.4.1-1, the difference d3 = d2 – d1 depends on the elevation angle, as listed in Table 7.3.4.1-1.

Table 7.3.4.1-1 Differential delay d3 = d2 – d1 vs. satellite elevation angle

| α2[degree] | cell radius (Smax2) [km] | d3 [km] |
| --- | --- | --- |
| 10 | 200 | 390 |
| 20 | 200 | 372 |
| 30 | 200 | 343 |
| 40 | 200 | 303 |
| 50 | 200 | 254 |
| 60 | 200 | 197 |
| 70 | 200 | 134 |
| 80 | 200 | 67 |

Clearly, the difference d3 exceeds the maximum cell coverage supported by the NR preamble format of 2x 100 km for satellite elevation angles below 60 degrees. In such a case, NR preamble format needs to be extended for NTN system considering different footprint of NTN cells. So it should be further studied if new random access preamble format is needed for spaceborne vehicles.

The effect of the residual differential delay may be mitigated as follows:

- Case 1: NTN differential delay ≤ Max currently specified NR PRACH CP duration

In this case no change is required to the specified NR RACH procedure.

- Case 2: NTN differential delay > Max currently specified NR PRACH CP duration

In this case different solutions are possible which are for further study.

RACH Procedure

For PRACH procedure, huge propagation delay [66.7μs, 120ms] [3] corresponding to BS heights [8km, 35786km] will lead to long RA response procedure [73-23][73-21]. As a result, it is necessary to shorten the RA transmission/retransmission delay and design an NTN-specific RAR window based on the given BS information to make the UE efficient and power saving.

        - NR impact considerations

Regarding NR PRACH waveform/format, no NR spec impacts are identified if the GNSS based techniques are applicable at the UE side, otherwise, potential enhancements w.r.t PRACH waveform/format may be needed which are for further study. For RACH procedures, potential enhancements can be studied regardless of whether the GNSS based techniques are applicable or not.

#### 7.3.4.2 TA in Random access response message

##### 7.3.4.2.1 Problem statement

The Timing advance mechanisms ensure that transmissions from all UEs operating in the same cell are synchronized when received by the same gNB. A TA command is provided to the UE in a RAR message during initial access, as well as later, to adjust the uplink transmission timing for PUCCH/PUSCH/SRS. TA adjustment is analyzed in Section 7.3.2.2.

The timing advance (NTA) is given by:

NTA=TA*16*642µ

where µ=0, 1, 2, 3, 4, 5 and TA is an integer between 0 and 3846.

The maximum value of the TA command sent in RAR message constraints the maximum distance between an UE and the serving base station, which also defines the maximum allowed cell size, as illustrated in Table 7.3.4.2-1.

Table 7.3.4.2-1: Terrestrial link distance for 5G NR

| Subcarrier spacing | 15 kHz µ= 0 | 30 kHz µ= 1 | 60 kHz µ= 2 | 120 kHz µ= 3 | 240 kHz µ= 4 | 480 kHz µ= 5 |
| --- | --- | --- | --- | --- | --- | --- |
| Maximum timing advance (ms) | 0.67 | 0.335 | 0.1675 | 0.0838 | 0.0419 | 0.0209 |
| Maximum link distance (TA = 3846) (km) | 300 | 150 | 75 | 37.5 | 18.75 | 9.38 |

Even at 15kHz, the maximum cell size is below the satellite altitude, for both LEO and GEO scenarios. Therefore, it is not expected that the timing advance mechanism can be reused as is for satellite networks.

For HAPS scenarios, the HAPS altitude is quite negligible compared to the maximum link distance for low numerology. In these cases, the maximum allowed cell size for terrestrial operations needs to be slightly reduced to ensure the maximum link distance is never exceeded. For high numerology however, the maximum link distance may be smaller than the HAPS altitude, leading to the same conclusion as for satellite operations.

##### 7.3.4.2.2 Assessment of conditions for NR operation in Non-Terrestrial networks

Compensation of entire propagation delay by TA

For satellite operations, it is expected that TA mechanism cannot be reused as is to compensate for the propagation delay. For HAPS operations, the mechanism can be reused as is in some cases (low numerology and/or low HAPS altitude). The maximum cell size shall then be redefined to ensure that the maximum allowed link distance is never exceeded.

Common delay known and handled by the network

In this case, TA compensates only for differential delay/distance, so that the maximum TA value defines the maximum allowed differential delay/distance. Considering Figure 7.3.4.1-1, it can be easily shown that the maximum differential distance is achieved when α2 is minimum. As shown in Table 7.3.4.2-2, the maximum differential distance is almost equal to the maximum cell diameter for α2=10°.

Table 7.3.4.2-2: Maximum cell size for  α2=10°, SCS=15kHz, GEO/MEO/LEO cases

| NTN vehicle altitude orbit | Min Elevation angle α2 (deg) | Max Differential distance d3 = d2 – d1 (km) | Maximum cell diameter Smax (km) |
| --- | --- | --- | --- |
| GEO (at 35 786 km) | 10° | 300 km | 304.66 |
| MEO at 10 000 km | 10° | 300 km | 304.73 |
| LEO at 1500 km | 10° | 300 km | 305.05 |
| LEO at 600 km | 10° | 300 km | 305.50 |

In first approximation, it can be stated that the existing TA mechanism can compensate the differential delay if the cell diameter does not exceed the maximum link distance given in Table 7.3.4.2-1.

##### 7.3.4.2.3 NR impact considerations

In HAPS operations, extension of the maximum TA value could be considered to extend the maximum allowed cell size, especially for high numerology.

In satellite operations, it is expected that the network can take into account the common propagation delay. To extend the maximum cell size, different approaches can be studied further.

### 7.3.5	Propagation channel

#### 7.3.5.1 DMRS frequency density

##### 7.3.5.1.1 Problem statement

For PDSCH, the mapping to physical resources of the Demodulation reference signals is described in chapter 7.4.1.1.2 of [73-2] recalled below. There is either one DM-RS symbol per slot or 2 in consecutive OFDM symbols within a slot. The resource elements of this DM-RS symbol are scattered over different subcarriers according to two configuration types.

In configuration type 1, DM-RS symbols are inserted every 2 subcarriers in PDSCH. In configuration type 2, DM-RS symbols are inserted every 5 subcarriers in PDSCH.

Table 7.3.5.1-1: Minimum coherence bandwidth of PDSCH supported for a given SCS value

|  | Minimum supported coherence bandwidth (assumed no orthogonally required) (kHz) |  |
| --- | --- | --- |
| SCS (kHz) | Configuration type 1 | Configuration type 2 |
| 15 | 30 | 75 |
| 30 | 60 | 150 |
| 60 | 120 | 300 |
| 120 | 240 | 600 |

For PBCH, the mapping to physical resources of the Demodulation reference signals is specified in Table 7.4.3.1-1 of [73-2]. DM-RS are inserted every four subcarriers in PBCH on all OFDM symbols.

For PDCCH, the DM-RS symbols are inserted every 4 subcarriers as for PBCH (chapter 7.4.1.3.2 in [73-2]).

Table 7.3.5.1-2: Minimum coherence bandwidth of PBCH/PDCCH supported for a given SCS value

| SCS (kHz) | Minimum supported coherence bandwidth (kHz) |
| --- | --- |
| 15 | 60 |
| 30 | 120 |
| 60 | 240 |
| 120 | 480 |

The minimum coherence bandwidth for each NTN deployment scenario can be computed with the following formula:

Minimum coherence bandwidth = 1/(alpha *delay spread)

with alpha being a constant between 1 and 50. As a worst case, it is set to 50 for the DM-RS mapping selection. The maximum delay spread depends on the UE antenna directivity.

Table 7.3.5.1-3: Maximum delay spread and minimum coherence bandwidth for each deployment scenario

|  | D1, GEO, Ka band | D2, GEO, S band | D3, LEO, S band | D4, LEO, Ka band | D5, HAPS, S band |
| --- | --- | --- | --- | --- | --- |
| Maximum Delay spread (ns) | 10 | 100 | 100 | 10 | 150 |
| Min coherence bandwidth (Note 1 & 2) | >> MHz | 200 kHz | 200 kHz | >> MHz | 133 kHz |

Note 1: In Ka band, typical antenna directivity is taken into account in the delay spead estimate

Note 2: In S band, delay spread in satellite scenarios is lower than in HAPS scenario, because the min operating SNR is lower which leads to discard largest delay spread.

##### 7.3.5.1.2 Assessment of conditions for NR operation in Non-Terrestrial networks

For S band, the minimal coherence bandwidth has been computed with the formula in the previous clause and an alpha value corresponding to a theoretical worst case (Alpha set to 50). This may put some constraints on the SCS value selection which needs to be further studied.

##### 7.3.5.1.3 NR impact considerations

Enhancement to NR specifications with respect to DMRS frequency diversity may not be needed depending on possible SCS choice constraints that needs to be further studied.

#### 7.3.5.2 Cyclic prefix

##### 7.3.5.2.1 Problem statement

Originally, the NR CP has been dimensioned for terrestrial transmissions characterized by high delay spread due to multi path propagation. This clause describes selection criteria of the CP that need to be addressed when considering using NR in Non-Terrestrial Network deployment scenarios.

##### 7.3.5.2.2 Assessment of conditions for NR operation in Non-Terrestrial networks

Delay spread in satellite propagation channels

Signal echoes are associated to the presence of indirect rays that reach the receiver antenna and carry a significant energy with respect to the energy of the direct ray.

ITU-R recommendation [73-24] defines for the 2 GHz band three parameter sets of wideband models, including LOS and NLOS cases, applicable for an elevation range from 15 to 55° and for urban, suburban and rural environments. The delay spread of these three parameter sets ranges between 180 ns to 250 ns, whereas the 250 ns are stated to cover 90% of the cases.

For higher elevations than 55°, we assume that the delay spread of the satellite channel will be in the same range or even lower due to the traveling distances of the echoes arriving at a receiver.

Few papers are available on delay spread measurements in Ka-Band. Reference [73-25] is stating the coherence bandwidth to be 30 MHz at 40 GHz with omnidirectional antennas. According to [73-26], the coherence bandwidth (Δ f)c of a channel with maximum delay spread Tm is

(Δ f)c ≈ 1 / (5T)

For the stated coherence bandwidth in [73-25], this results in a maximum delay spread of Tm = 25 ns for omni-directional antennas. For directional antennas, echoes with significant delay are normally filtered out by the antenna radiation pattern, so flat fading can be assumed for Ka-band signals.

CP length defined in NR

The possible CP lengths currently defined for New Radio [73-2] are summarized in the following table:

Table 7.3.5.2-1: CP lengths and minimum/maximum RF bandwidth of NR as currently defined in [73-2].

| Subcarrier spacing (SCS) configuration parameter µ | SCS [kHz] | normal CP length [µs] | extended CP length [µs] |
| --- | --- | --- | --- |
| 0 | 15 | 4,688 | Not defined |
| 1 | 30 | 2,344 | Not defined |
| 2 | 60 | 1,172 | 16,67 |
| 3 | 120 | 0,586 | Not defined |
| 4 | 240 | 0,293 | Not defined |

##### 7.3.5.2.3 NR impact considerations

It is observed that lower numerologies (µ= 0, 1) are associated with CP lengths exceeding the requirement of NTN, resulting in a slightly reduced spectral efficiency due to the over-dimensioned CP (e.g. overhead is for µ= 0: (4.688µs - 0.25µs) / 66.67 µs = 6.7 %).

High numerologies (µ= 3, 4) result in CP lengths which are well matching to propagation characteristics in Ka-Band.

The extended CP for a SCS of 60 kHz is not required, because it is significantly larger than required for satellite applications.

Enhancement to NR specification is not expected for NTN applications due to the NTN channel model delay spread compatible with the existing specified CP values.

### 7.3.6	Duplex mode

#### 7.3.6.1 FDD/TDD Duplexing mode

##### 7.3.6.1.1 Problem statement

Most of the existing satellite systems operate in the frequency bands designated for FDD mode, with defined transmit direction.  For some frequency bands, TDD mode is possible.

When considering TDD mode, a guard time is necessary to prevent UE to simultaneously Transmit and reception. This guard time directly depends on the propagation delay between UE and gNB. This guard time will directly impact the useful throughput and hence the spectral efficiency.

When considering Non-Terrestrial networks, this guard time should commensurate the round trip delay.

Guard time would range between 2 x 7 ms for LEO at 600 km and 2 x 270 ms for GEO satellite access networks since NTN terminals can experience a one-way propagation time of

- 240 ms at minimum and 270 ms at maximum between UE and satellite base station for GEO

- 2 ms at minimum and 7 ms at maximum between UE and satellite base station for LEO at 600 km altitude

Such excessive guard time would lead to a very inefficient radio interface especially in GEO or even MEO based access.

It may be acceptable in the case of LEO access system with the need to deal with the variable delay.

##### 7.3.6.1.2 Assessment of conditions for NR operation in Non-Terrestrial networks

The applicability of the duplexing mode TDD or FDD depends on the regulations (ITU-R and/or national) associated to the targeted spectrum.

##### 7.3.6.1.3 NR impact considerations

FDD is preferred duplexing mode for most NR based NTN access network.

In case the regulations allow it, TDD mode can be considered for both HAPS and LEO satellite based access network with potential NR impacts if required.

### 7.3.7	Satellite or aerial Payload performance

#### 7.3.7.1 PT-RS

##### 7.3.7.1.1 Problem statement

Phase variations in time domain can be caused by different phenomena: presence of phase noise, frequency drifts due to Doppler shift, or due to insufficient frequency synchronization (e.g. residual CFO), etc. Phase noise, caused by imperfect oscillator implementation technology destroys the orthogonality of subcarriers in OFDM-based systems especially in FR2. Phase noise causes CPE, resulting in a constant rotation angle of the modulation constellation, and ICI, resulting in scattering of the constellation points in OFDM based systems. Doppler effects and/or residual CFO cause frequency shifts translated into time domain phase ramps at OFDM symbol level.

The phase variations translating all the above effects may significantly degrade the performance and need compensation. When mild, these variations can be absorbed by the DM-RS in the channel estimation process. But when strong phase variations are present, which is particularly the case at carrier frequencies above 6GHz, more frequent support than the one provided by DM-RS is necessary in order to track the remaining variations.

In NR, PT-RS has been introduced [73-2, 73-4]. to compensate for phase errors. PT-RS configuration in NR is very flexible and allows user-specific configuration depending on scheduled MCS/bandwidth, UE RF characteristics, DM-RS configuration, waveform, etc.

##### 7.3.7.1.2 Assessment of conditions for NR operation in Non-Terrestrial networks

It has been proposed that eMBB should be supported for NTN and the modulation order might not always be low. PT-RS is needed in NTN at high carrier frequencies, where the maximum received SNR is limited by the phase noise.

Considering the high speed of airborne/spaceborne vehicles, CFO and Doppler shift/spread might be worse in NTN and therefore CFO and Doppler estimation need to be considered.

A number of phase noise masks such as DTH or VSAT use cases considered in [73-29], or other state of the art bent pipe of satellite or HAPS payloads as described in Annex B are considered as pertinent for non-terrestrial communications. Phase noise masks pertinent for terrestrial equipment operating in mmWave have been defined in NR in [73-30].

The performance of phase tracking algorithms is highly dependent on a large set of variables such as carrier frequency, subcarrier spacing, UE specific RF characteristics, configured MCS/bandwidth, frame length, DMRS configuration, Doppler, receiver implementation options, etc.

##### 7.3.7.1.3 NR impact considerations

PTRS is needed in NR supporting NTN for phase error compensation. PTRS configuration in NR is very flexible. Typical phase noise masks of state of the art bent pipe of satellite or HAPS payloads in Non-Terrestrial networks can be efficiently compensated by the current NR design in the absence of important Doppler shifts and/or residual CFO at a carrier frequency of up to 30 GHz. Some further investigation is needed in the case of important Doppler shifts and/or residual CFO, or in the presence of the specific phase noise masks of on board payloads significantly different from the ones considered in cellular network so far, or with very large channel bandwidths.

#### 7.3.7.2 PAPR

##### 7.3.7.2.1 Problem statement

A key component in the satellite payload architecture is a PA. It exhibits nonlinear behavior when driven near saturation in an effort to increase power efficiency. Nonlinear distortion causing constellation warping and clustering, thus complicating signal reception. One measurement that determines the vulnerability of the transmitted signal to nonlinear distortion is the PAPR, with higher values associated with worse impact.

In the NR downlink, CP-OFDM is used. CP-OFDM is composed of superposition of narrowband orthogonal subcarriers, resulting in higher PAPR values compared with the underlying modulation in a single carrier.  The PAPR is a function of the number of subcarriers in the signal, as well as the underlining modulation.

The amount of distortion caused by nonlinear amplifier characteristics is a function of the PAPR of the signal.  The greater the PAPR, the greater the distortion can be.  The distortion can be reduced by increasing the backoff of the amplifier operating point. But this reduces the amplifier efficiency accordingly.  Tradeoffs are often made between the amplifier OBO and the nonlinear signal distortion, such that the total degradation of output power and signal distortion is minimized.

As an example, the figures1 below show the total degradation of CP-OFDM with over a typical satellite transponder is about 6 dB for QPSK modulation and 7.6 dB for 16-QAM, whereas that for DFT-spread-OFDM is about 4 dB for QPSK and 6 dB for 16-QAM.  The difference between CP-OFDM and DFT-spread-OFDM is 1.6-2 dB.

- QPSK Modulation				(b) 16-QAM modulation

Figure 7.3.7.2-1: DFT-S-OFDM vs CP-OFDM total degradation

The additional 2 dB output backoff may represent 20 to 40 percent reduction of link capacity compared to State of the art satellite radio interface, depending on whether the satellite is bandwidth limited or power limited.

_______________

1 See ETSI TR 103 297 “Satellite Earth Stations and Systems (SES), SC-FDMA based radio waveform technology for Ku/Ka band satellite service”, , V.1.1.1., (2017-07). Pages 29-30.

As for HAPS, the payload architecture may be very similar to satellite, and power amplifier remains to be a key component, although SSPA is likely to be used instead of traveling wave tube.  Regardless, the PAPR difference between CP-OFDM and DFT-spread-OFDM also requires different OBO which translates to different total distortion.  The higher total distortion of CP-OFDM ultimately means lower capacity.  In this case, however, the problem may be less acute, since the payload power efficiency may not be as critical.

##### 7.3.7.2.2 Assessment of conditions for NR operation in Non-Terrestrial networks

The use of CP-OFDM in the downlink does not restrict NR operation in non-terrestrial networks, but it may affect the system performance:

- In one case, a satellite transponder is sufficiently wide and sufficiently powerful to accommodate more than one FDM carriers, each of which is a different NR CP-OFDM signal.  The satellite amplifier is backed off to minimize the intermodulation between these FDM carriers within the same transponder.  For such cases, the distortion introduced by the amplifier nonlinearity is small, with little impact.

- In second and much more common case for communicating to small UEs, the satellite amplifier is used to send only one NR CP-OFDM downlink.  It is highly desirable to operate the amplifier with as small OBO as possible.  But, due to the higher PAPR of CP-OFDM signal, sufficient OBO is necessary.  To close the link, it may be necessary to reduce the size of CP-OFDM carrier or operate the CP-OFDM carrier with a lower modulation and coding mode.  Either way, the forward link capacity is reduced significantly.

On the uplink, DFT-spread-OFDM might be beneficial.

##### 7.3.7.2.3 NR impact considerations

PAPR reduction techniques of CP-OFDM signal on the downlink would be beneficial to optimize the capacity of non-terrestrial networks and therefore could be considered in future studies.

* * * End of changes * * * *

* * * Start of changes * * * * (new text)

# Annex B: NTN characteristics

### B.1 NTN Phase noise masks

DTH P1 and P2 and VSAT P1 and P2 phase noise masks were previously defined by DVB [73-29]. in the context of satellite communications.

|  | log10freq | log10freq |  |  |  |
| --- | --- | --- | --- | --- | --- |

* * * End of changes * * * *