3GPP TSG RAN Meeting #81	RP-181659

Gold Coast, Australia, Sep. 10th – 13th, 2018

Source:	vivo, CMCC

Title:	Resolution of In-device issue in NR

Agenda Item:	9.1.2

Document for:	Discussion and Decision

# Introduction

According to the RAN2#103 meeting discussion, RAN2 made the following agreements:

=> IDC for NR SA will not be specified for Rel-15 => Can be discussed offline how this might be included in the Rel-16 work (e.g. TEI or possible WI)

In this contribution, we provide the offline discussion feedbacks and kindly request the RAN plenary to decide how to resolve the in-device issues in Rel-16.

# Discussion

## In-device issue(s) and potential solutions

According to the LTE discussions, the in-device issues include:

- Overheating due to the use of the advanced radio technologies.

- Hardware sharing between different RAT(s)

- Interference between different RAT(s) between adjacent frequencies

- Interference between different RAT(s) due to the inter-modulation within the same cell group and between different cell groups

The specified solutions including:

- The network configuration of the autonomous deny parameters

- The UE reporting of the affected frequency

- The UE reporting of the desired TDM pattern

- The hardware sharing indication

Figure 1: Coexistence interference within the same UE

As the NR UE are expected to support more RAT(s) and more antenna panels, the in-device issues are getting more serious than before. According to the previous UE test, the in-device interference includes diverse wave types, e.g. side-lobe, harmonic (mostly second/third harmonic order) and inter-modulation (e.g. LTE+WiFi inter-modulation). Thus, the potential victim frequency range are given in Annex A. According to the LS from RAN4 [6], RAN2 also confirmed the in-device interference issue in NR. According to the discussion related to the NR IDC issue in the RAN2#103bis meeting, the potential NR solutions for IDC are expected to be more practical and effective.

## Handling of in-device issue(s) in Rel-16

According to the offline discussion with several companies, we could have the following potential ways to handle the in-device issue(s) in Rel-16:

- NR-U

- TEI

- New WI

Regarding the option of including the in-device issue in NR-U, the in-device interference issue seems to be a critical issue for the NR-U UE. As the LTE IDC WI was done before LTE LAA WI, the LTE IDC solutions are re-used for the LTE LAA UE. As the NR-U SI is on-going now and the NR has no IDC solution yet, it is also urgent for the NR-U UE to find some solutions to the in-device issues. However the in-device issues are not just for the unlicensed frequency but also for the licensed frequency, which requires RAN4 to define the corresponding UE requirements. It seems that many companies consider that we should find a common solution for both the unlicensed frequency and the licensed frequency.

Observation 1: The in-device issues affect both the unlicensed frequency and the licensed frequency.

Regarding the option of including the in-device issue in TEI in RAN2, according to the LTE IDC WI discussion, RAN4 needs to define extra UE requirements for the corresponding IDC issues. This is not what RAN2 usually does for the TEI, as a TEI solution should normally try to avoid the impacts on other working groups. Furthermore according to the offline discussion, it seems that to resolve the interference issue for other RAT of the inter-modulation between different cell groups, the TDM solution would require the coordination between MN and SN. This could potentially also require some RAN3 work as the NR solution for the harmonic issue of EN-DC.

Observation 2: The solutions to the in-device issues require the work at least from RAN2 and RAN4.

According to the specification work in LTE, the in-device issues are considered as urgent problems to be resolved. Due to the limited time in the NR discussion, RAN2 determines not to specify any solutions for IDC for NR SA.  Thus we respectfully ask the RAN plenary to provide some guidance(s) on how to handle the in-device issues in RAN.

Proposal: RAN plenary is kindly requested to decide whether to handle the in-device issue in a new WI or TEI.

# Conclusion

According to the discussions give above, we have the following Observations and Proposal:

Observation 1: The in-device issues affect both the unlicensed frequency and the licensed frequency.

Observation 2: The solutions to the in-device issues require the work at least from RAN2 and RAN4.

Proposal: RAN plenary is kindly requested to decide whether to handle the in-device issue in a new WI or TEI.

# Reference

- 3GPP TR 38.816, “Study on signalling and procedure for interference avoidance for in-device coexistence”

- 3GPP TS 36.331, “Radio Resource Control (RRC); Protocol specification”

- 3GPP TS 36.300, “Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2”

- R2-1713684, TDM pattern coordination for 1Tx and Harmonic scenario	vivo

- S2-176689, LSin, From SA2, To RAN1, RAN2, RAN4, “LS on simultaneous transmission and/or reception over EPC/E-UTRAN and 5GC/NR”.

- R4-1803254, LSout, From RAN4, To RAN2, “LS reply on In-Device Coexistence solution for EN-DC”.

- R2-1802609, IDC indication for EN-DC, Qualcomm Incorporated	discussion

- R2-1802101, Discussion on the IDC report of NR frequency in EN-DC, vivo	discussion

- R2-1802100, Challenges for in-device coexistence in NR, vivo, Potevio, CATR, Xiaomi, Spreadtrum Communications, China Telecom, TCL		discussion

- 3GPP TS 38.101-1, “User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

- 3GPP TS 38.101-2, “User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”.

# Annex A

The following table gives the potential victim frequency range between NR band(s) and other systems:

| Victim Systems | Frequency range [MHz] |  |  | Regions | Side-lobe | NR second harmonic order | NR third harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 |  | n50(UL)/n74(UL) |  |  |
| Galileo | 1559 | - | 1591 |  | n50(UL)/n74(UL) |  |  |
| GLONASS | 1591 | - | 1610 |  | n50(UL)/n70(UL) |  |  |
| GPS | 1563 | - | 1587 |  | n50(UL)/n74(UL) |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | US/Europe | n7/n38/n41 |  | n5/n20/n28/n82 |
|  | 2400 | - | 2494 | Asia | n7/n38/n41 |  | n5/n20/n28/n82 |
| ISM band (5GHz) | 5150 | - | 5925 | US | n79 | n7/n38/n41 | n1/n2/n3/n66/n80/n84 |
|  | 5150 | - | 5350 | Europe | n79 | n7/n38/n41 | n1/n2/n3/n66/n80/n84 |
|  | 5470 | - | 5725 |  |  |  |  |
|  | 5150 | - | 5825 | Asia | n79 | n7/n38/n41 | n1/n2/n3/n66/n80/n84 |
| ISM band (45GHz) | 42300 | - | 47000 | China |  | n258 |  |
|  | 47200 | - | 48400 |  |  |  |  |
| ISM band (60GHz) | 57000 | - | 66000 | Europe |  | n257 |  |
|  | 57050 | - | 64000 | USA - Canada |  | n257 |  |
|  | 57000 | - | 64000 | South Korea |  | n257 |  |
|  | 59000 | - | 66000 | Japan |  | n257 |  |
|  | 59000 | - | 64000 | China |  | n257 |  |
|  | 59400 | - | 62900 | Australia |  | n257 |  |

# Annex B

The following table for FR1 is extracted from 38.101-1 [10]:

Table 5.2-1: NR operating bands in FR1

| NR Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
| n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894MHz | FDD |
| n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
| n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| n20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
| n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
| n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| n50 | 1432 MHz | – | 1517 MHz | 1432 MHz | – | 1517 MHz | TDD |
| n51 | 1427 MHz | – | 1432 MHz | 1427 MHz | – | 1432 MHz | TDD |
| n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
| n70 | 1695 MHz | – | 1710 MHz | 1995 MHz | – | 2020 MHz | FDD |
| n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
| n74 | 1427 MHz | – | 1470 MHz | 1475 MHz | – | 1518 MHz | FDD |
| n75 | N/A |  |  | 1432 MHz | – | 1517 MHz | SDL |
| n76 | N/A |  |  | 1427 MHz | – | 1432 MHz | SDL |
| n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
| n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
| n80 | 1710 MHz | – | 1785 MHz | N/A |  |  | SUL |
| n81 | 880 MHz | – | 915 MHz | N/A |  |  | SUL |
| n82 | 832 MHz | – | 862 MHz | N/A |  |  | SUL |
| n83 | 703 MHz | – | 748 MHz | N/A |  |  | SUL |
| n84 | 1920 MHz | – | 1980 MHz | N/A |  |  | SUL |

# Annex C

The following table for FR2 is extracted from 38.101-2 [11]:

Table 5.2-1: NR operating bands in FR2

| NR Operating Band |  | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n257 | 26500 MHz |  | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |
| n258 | 24250 MHz |  | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |
| n260 | 37000 MHz |  | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |