3GPP TSG RAN meeting #81										RP-181669

Gold Coast, Australia, September 10-13, 2018

## Status Report to TSG

Agenda item:			10.2.5

| WI / SI Name | Enhancements for high capacity stationary wireless link and introduction of 1024 QAM for LTE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| included in this status report | Core part: | Yes | Perf. part: | Yes |  | Testing part: | No |
| Acronym | LTE_1024QAM_DL |  |  |  |  |  |  |
| Unique ID | 750064 |  |  |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-171738 |  |  |  |  |  |  |
| Target Completion Date | Core part: 09/2018 |  |  |  | Performance part: 12/2018 |  |  |
| Is the WI/SI complete? | Core part: Yes |  |  |  | Performance Part: No |  |  |

Source:

| Leading WG |  | RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | XIAO, Weimin RICO ALVARINO, Alberto ZHU, Jianchi |
|  | Company | Huawei Qualcomm Incorporated China Telecom |
|  | Email | weimin.xiao@huawei.com albertor@qti.qualcomm.com zhujianchi.bri@chinatelecom.cn |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

No remaining issue.

## 2.2	RAN2

#### 2.2.1	Agreements

RAN2#103:

5 ([1-5]) contributions were submitted to RAN2#103 meeting and following were achieved:

R2-1812980	UE categories for 1024QAM	Qualcomm Incorporated	CR	Rel-15	36.306	15.1.0	1628	1	B	LTE_1024QAM_DL-Core	R2-1811711

=>	Agreed

R2-1813417	UE UL categories for 1024QAM	Qualcomm Incorporated	CR	Rel-15	36.331

=>	Agreed

R2-1812981	Support of 1024QAM in TS 36 331	Huawei, HiSilicon	CR	Rel-15	36.331	15.2.2	3227	4	B	LTE_1024QAM_DL-Core	R2-1807712

=>	Agreed

#### 2.2.2	Remaining Open issues

No remaining issue.

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

No remaining issue.

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4#88:

16 ([6-21]) contributions were submitted to RAN4#88 meeting and following were achieved:

R4-1810435	CR on UE category for DL 1024QAM in TS 36.101
					36.101	  CR-5164  rev  Cat: F (Rel-15) v15.3.0
					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

R4-1811378	Way forward on 1024QAM demodulation and CSI requirements
						  CR-  rev  Cat:  () v
					Source: Huawei

Decision:		Approved

Demodulation requirements under fading conditions

- For 1024QAM demodulation requirements, the test cases given in Table 2 with the common parameters given in Table 1 are identified

Table 1: common parameters

| Parameters | Value |
| --- | --- |
| Bandwidth | 10MHz |
| Channel model | EPA5 |
| CFI (OFDM symbol for control channel) | 1 |
| Duplex mode | FDD, TDD |

Demodulation requirements under fading conditions

- For 1024QAM demodulation requirements, the candidate test cases given in Table 2 with the common parameters given in Table 1 are identified

Table 2: Test cases for demodulation tests under fading conditions

| No. | TM | PMI feedback (pre-coding) | MIMO layer | MCS | Antenna config. |
| --- | --- | --- | --- | --- | --- |
| 1 | TM4 | Wideband PMI Feedback (PUSCH 3-1, Reporting interval =1) | 1-layer | MCS#23 (with 52752 bits) | 4x2 Low |
| 2 | TM4 | Wideband PMI feedback (PUSCH 3-1, Reporting interval =1) | 2-layer | MCS#23 (with 52752 bits) | 4x4 Low |
| 3 | TM9 (Note 1) | Per-PRG Random Precoder | 1-layer | MCS#23 (with 52752 bits) | 4x2 Low |
| 4 | TM9 | Wideband PMI feedback | 2-layer | MCS#23 (with 52752 bits) | 4x4 Low |

Note 1: For TM9, configure 4Tx with 2 CRS ports

- Companies are encouraged to provide the simulation results with impairments for 1024QAM DL FDD and TDD demodulation fading tests

SDR test

Test parameters:

- Transmission mode: TM3

- Bandwidth: 10MHz

- Antenna configuration: 2 x 2 for 2-layer transmission per CC, and 4 x 4 for 4-layer transmission per CC

- MCS 24 for 2 x 2 2-layer transmission per CC, and MCS 24 for 4 x 4 4-layer transmission per CC

- CFI = 1

- Tx EVM: 2%

CQI reporting test (For information)

Definition test:

- Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbols): CRS based TM single codeword with TM1 under static channel in Annex B.1

  - Bandwidth:

    - FDD: 10MHz

    - TDD: 20MHz

Table 3: CQI2MCS table

| CQI Index |  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Target Spectral Efficiency |  |  | OOR | 0.1523 | 0.3770 | 0.8770 | 1.4766 | 2.4063 | 3.3223 | 3.9023 | 4.5234 | 5.1152 | 5.5547 | 6.2266 | 6.9141 | 7.4063 | 8.3321 | 9.2578 |  |
| MCS Scheme | PRB | Available RE-s | Imcs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MCS.x1A | 50 | 6300 | DTX | 0 | 1 | 3 | 5 | 7 | 9 | 11 | 13 | 15 | 17 | 19 | 21 | 22 | 23 | 25 |  |
| MCS.x2A | 100 | 12600 | DTX | 0 | 1 | 3 | 5 | 7 | 9 | 11 | 13 | 15 | 17 | 19 | 21 | 22 | 23 | 25 |  |
| Note 1: Mapping between Imcs and CQI Index according to Tables 7.1.7.1-1A, 7.1.7.2.1-1 and 7.2.3-2 in TS 36.213 [6]. Note 2: 3 symbols allocated to PDCCH. Note 3: Sub-frame#0 and #5 are not used for the corresponding requirement. The next subframe (i.e. sub-frame#1 or #6) shall be used for potential retransmissions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

DMRS overhead reduction

- DMRS ports 7,8,11,13 with OCC=4

- 16QAM, EPA5, 4-layer

- 1dB difference i.e. improvement with reduced DMRS

Agreement:

- Applicability rule is defined such that a UE supporting OCC4 DMRS rank-4 does not need to be tested for the existing OCC2 DMRS rank-4 (defined in Section 8.10.1.1.9 in [2]).

R4-1811376	CR: test case for reduced DMRS
					36.101	  CR-5141  rev  Cat: B (Rel-15) v15.3.0
					Source: Huawei, HiSilicon

Decision:		Agreed

Agreement:

- No PDSCH allocation on subframe 0 and subframe 5 for 1024QAM CQI reporting test.

- Consider CFI 3 and full RB allocation for 1024QAM CQI reporting test.

#### 2.4.2	Remaining Open issues

- For 1024QAM demodulation requirements, the final required SNR at 70% relative throughput needs be decided.

- For CQI test, the final test points need be decided.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

#### 2.5.4	Estimated Level of Completion

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT. Based on the joint RAN-SA#80 session the items relevant for this section are shown in http://www.3gpp.org/ftp/tsg_sa/TSG_SA/TSGS_80/Docs/SP-180561.zip and summarized below:

| Work Area | SA WIDs | Rapporteurs | RAN WIDs | Rapporteurs |
| --- | --- | --- | --- | --- |
| URLLC for 5G | FS_5G_URLLC FS_Vertical_LAN cyberCAV | SA2: Hui.ni@huawei.com SA2: Devaki.chandramouli@nokia.com SA1: joachim.walewski@siemens.com | FS_NR_L1enh_URLLC FS_NR_unlic FS_NR_IIOT | RAN1: chengyan.cheng@huawei.com RAN1: jingsun@qti.qualcomm.com RAN2: dawid.koziol@nokia.com |
| V2X for 5G | FS_eV2XARC | SA2: laeyoung.kim@lge.com SA1: sungduck.chun@lge.com | FS_NR_V2X | RAN1: Hanbyul.seo@lge.com Matthew.webb@huawei.com |
| Positioning | FS_eLCS 5G_HYPOS | SA2: aiming@catt.cn SA1: lionel.ries@esa.int | FS_NR_POS | RAN1:alexey.khoryaev@INTEL.COM asbjorn.grovlen@ERICSSON.COM |
| UE Capabilities | FS_RACS | SA2: harisz@qti.qualcomm.com | FS_RACS_RAN | RAN2: alex.hsu@mediatek.com |
| 5G Satellite Aspects | FS_5GSAT_ARCH 5GSAT | SA2: cyril.michel@thalesaleniaspace.com SA1: cyril.michel@thalesaleniaspace.com | FS_NR_NTN_solutions | RAN3: nicolas.chuberre@thalesaleniaspace.com |

RAN and SA rapporteurs are requested to fill in these clauses jointly.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

- R2-1811711	UE DL categories for 1024QAM	Qualcomm Incorporated

- R2-1812980	UE categories for 1024QAM	Qualcomm Incorporated

- R2-1813417	UE UL categories for 1024QAM	Qualcomm Incorporated

- R2-1811924	Support of 1024QAM in TS 36 331	Huawei, HiSilicon

- R2-1812981	Support of 1024QAM in TS 36 331	Huawei, HiSilicon

- R4-1810435	CR on UE category for DL 1024QAM in TS 36.101 Huawei, HiSilicon

- R4-1809778	Simulation result for 1024QAM FDD PDSCH demodulation	Qualcomm Incorporated

- R4-1811374	Summary of simulation results for 1024QAM	Huawei

- R4-1811378	Way forward on 1024QAM demodulation and CSI requirements	Huawei

- R4-1809851	PDSCH Demodulation requirements under fading conditions with 1024QAM	 Intel Corporation

- R4-1810030	Discussion on 1024QAM DL demodulation requirements under fading propagation conditions	 Huawei, HiSilicon

- R4-1810140	Simulation results of PDSCH demodulation for 1024QAM	Ericsson

- R4-1811375	Draft CR: for 1024QAM DL demodulation under fading condition	Huawei, HiSilicon

- R4-1809852	SDR Requirements with 1024QAM	Intel Corporation

- R4-1810032	Discussion on 1024QAM DL SDR tests	Huawei, HiSilicon

- R4-1809780	Simulation result for reduced DMRS FDD PDSCH demodulation	Qualcomm Incorporated

- R4-1810034	Discussion on reduced DMRS tests	Huawei, HiSilicon

- R4-1811376	CR: test case for reduced DMRS	Huawei, HiSilicon

- R4-1809779	Discussion on 1024QAM FDD CQI reporting test	 Qualcomm Incorporated

- R4-1810141	Simulation results of CQI reporting test for 1024QAM	Ericsson

- R4-1811377	Discussion and simulation results on 1024QAM DL CSI requirements	Huawei, HiSilicon

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template