3GPP TSG RAN meeting #81										RP-181703

Gold Coast, Australia, September 10-13, 2018

## Status Report to TSG

Agenda item:			10.2.4

| WI / SI Name | Work Item on Enhancements to LTE operation in unlicensed spectrum |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| included in this status report | Core part: | Yes | Perf. part: | Yes |  | Testing part: | No |
| Acronym | LTE_unlic |  |  |  |  |  |  |
| Unique ID | 750065 |  |  |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-180402 |  |  |  |  |  |  |
| Target Completion Date | Core part: 09/2018 |  |  |  | Performance part: 12/2018 |  |  |
| Is the WI/SI complete? | Core part: Yes |  |  |  | Performance Part: No |  |  |

Source:

| Leading WG |  | TSG RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | Timo LUNTTILA |
|  | Company | Nokia |
|  | Email | timo.lunttila@nokia.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

In RAN1#94, Gothenburg, Sweden, following agreements were made:

R1-1808139 Correction on the partial PUSCH mode field for FeLAA in 36.212 Huawei, HiSilicon Final CR agreed in R1-1809576 (Rel-15, CR0300, TS 36.212) R1-1809623 Correction on starting position of Partial PUSCH Mode 1 for FeLAA in 37.213 Huawei, HiSilicon, Ericsson Final CR agreed in R1-1809647 (TS37.213, Rel-15, CR0002) R1-1809626 Correction on COT length for AUL transmission LG Electronics Final CR agreed in R1-1809648 (TS37.213, Rel-15, CR0003) R1-1808589 Correction on the interpretation of HARQ-ACK bitmap for FeLAA in 36.212 Huawei, HiSilicon R1-1809577 Correction on the interpretation of HARQ-ACK bitmap for FeLAA in 36.212 Huawei, HiSilicon R1-1809602 Correction on DL CP extension for FeLAA in 36.211 Huawei, HiSilicon Final CR agreed in R1-1809650 (TS36.211, Rel-15, CR0456) R1-1809622 Correction on RRC parameters for FeLAA in 37.213 Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Ericsson Ericsson R1-1809620 Correction to determination of ending symbol for AUL transmissions Nokia, Nokia Shanghai Bell, Ericsson, Huawei, HiSilicon Final CR agreed in R1-1809651 (TS36.211, Rel-15, CR0457) R1-1809621 Correction to AUL-UCI resource allocation in 36.213 Nokia, Nokia Shanghai Bell Final CR agreed in R1-1809652 (TS36.213, Rel-15, CR1155)

#### 2.1.2	Remaining Open issues

No remaining open issues.

## 2.2	RAN2

#### 2.2.1	Agreements

In RAN2#103, Gothenburg, Sweden, following agreements were made:

Stage 2: R2-1812366 Miscellaneous Corrections to Stage-2 Specification Ericsson CR Rel-15 36.300 15.2.0 1177 - F LTE_unlic-Core => Remove the change “except for autonomous HARQ, the UE does not perform any UL (re)transmission and keeps the data in the HARQ buffer. A PDCCH is then required to perform a retransmission i.e. a non-adaptive retransmission cannot follow. For autonomous HARQ, the UE does not perform any UL (re)transmission and it may perform a new transmission for the same HARQ process in any following autonomous uplink resource” => With remove the above change, Agreed to be added to the merged 36.300 CR R2-1812651 Correction on HARQ for FeLAA Huawei, HiSilicon CR Rel-15 36.300 15.2.0 1179 - F LTE_unlic-Core => Not use “Autonomous asynchronous HARQ” => Agreed to be added to the merged 36.300 CR. CB on Friday: merged 36.300 CR (Offline#277, Huawei, R2-181) R2-1812649 Correction on channel access priority class for FeLAA Huawei, HiSilicon CR Rel-15 36.300 15.2.0 1178 - F LTE_unlic-Core => Agreed to be added to the merged 36.300 CR. R2-1812653 Correction on uplink scheduling for FeLAA Huawei, HiSilicon CR Rel-15 36.300 15.2.0 1180 - F LTE_unlic-Core => Agreed to be added to the merged 300 CR. RRC: R2-1812137 Introduce feLAA in TS 36.331 Nokia CR Rel-15 36.331 15.2.2 3473 1 B LTE_unlic-Core R2-1808930 => Endorsed as running CR CB on Friday: Revised in R2-181xxxx to capture ASN.1 review outputs and other agreements from this meeting. (Offline#278, Nokia) R2-1812004 Adding some missing references for feLAA Nokia, Nokia Shanghai Bell draftCR Rel-15 36.331 15.2.2 LTE_unlic-Core => Agreed to be added to the revised running CR. MAC: R2-1812367 Aligning feLAA parameter names to RRC Ericsson CR Rel-15 36.321 15.2.0 1346 - B LTE_unlic-Core R2-1812654 Miscellaneous correction to FeLAA in TS 36.321 Huawei, HiSilicon CR Rel-15 36.321 15.2.0 1351 - F LTE_unlic-Core => Remove the change “except for autonomous asynchronous UL HARQ” => Change “on the configured autonomous uplink grant” to “on the configured autonomous uplink” => The change“Autonomous uplink and Uplink Semi-Persistent Scheduling cannot be active simultaneously on the same SCell.” should not be removed => With these changes, agreed to be added to the merged 36.321 CR. CB on Friday: Merged 36.321 CR in R2-1813348 (Offline# 279, Huawei) R2-1812652 Correction on IRV handling for FeLAA in TS 36.321 Huawei, HiSilicon CR Rel-15 36.321 15.2.0 1350 - F LTE_unlic-Core => Huawei intention is correct. => Not pursued R2-1812380 Correction to UL Grant Intel Corporation CR Rel-15 36.321 15.2.0 1348 - F LTE_unlic-Core => Agreed to be added to the merged CR. R2-1812650 Correction on handling of aul-RetransmissionTimer for FeLAA in TS 36.321 Huawei, HiSilicon CR Rel-15 36.321 15.2.0 1349 - F LTE_unlic-Core => Change “When an uplink grant addressed to C-RNTI is received for this TTI and if the UL HARQ operation is autonomous asynchronous, the HARQ process shall:” to “When an uplink grant addressed to C-RNTI is received for this HARQ process and if the UL HARQ operation is autonomous asynchronous, the HARQ process shall:” => With this change, the first two changes are agreed to be added to the merged CR.

#### 2.2.2	Remaining Open issues

No remaining open issues.

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

In RAN4#88, Gothenburg, Sweden, following agreements were made:

RAN4 agreed CRs to move section 9 of 36.141 into 37.107. The related CRs are: • R4-1811115, Moving Section 9 from 36.141 to 37.107 • R4-1811116, Removing Section 9 from 36.141

#### 2.4.2	Remaining Open issues

No remaining open issues in the Core part. Performance part of the WI is still ongoing.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

#### 2.5.4	Estimated Level of Completion

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT. Based on the joint RAN-SA#80 session the items relevant for this section are shown in http://www.3gpp.org/ftp/tsg_sa/TSG_SA/TSGS_80/Docs/SP-180561.zip and summarized below:

| Work Area | SA WIDs | Rapporteurs | RAN WIDs | Rapporteurs |
| --- | --- | --- | --- | --- |
| URLLC for 5G | FS_5G_URLLC FS_Vertical_LAN cyberCAV | SA2: Hui.ni@huawei.com SA2: Devaki.chandramouli@nokia.com SA1: joachim.walewski@siemens.com | FS_NR_L1enh_URLLC FS_NR_unlic FS_NR_IIOT | RAN1: chengyan.cheng@huawei.com RAN1: jingsun@qti.qualcomm.com RAN2: dawid.koziol@nokia.com |
| V2X for 5G | FS_eV2XARC | SA2: laeyoung.kim@lge.com SA1: sungduck.chun@lge.com | FS_NR_V2X | RAN1: Hanbyul.seo@lge.com Matthew.webb@huawei.com |
| Positioning | FS_eLCS 5G_HYPOS | SA2: aiming@catt.cn SA1: lionel.ries@esa.int | FS_NR_POS | RAN1:alexey.khoryaev@INTEL.COM asbjorn.grovlen@ERICSSON.COM |
| UE Capabilities | FS_RACS | SA2: harisz@qti.qualcomm.com | FS_RACS_RAN | RAN2: alex.hsu@mediatek.com |
| 5G Satellite Aspects | FS_5GSAT_ARCH 5GSAT | SA2: cyril.michel@thalesaleniaspace.com SA1: cyril.michel@thalesaleniaspace.com | FS_NR_NTN_solutions | RAN3: nicolas.chuberre@thalesaleniaspace.com |

RAN and SA rapporteurs are requested to fill in these clauses jointly.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

| 1 | R1-1808139 | Correction on the partial PUSCH mode field for FeLAA in 36.212 | Huawei, HiSilicon |
| --- | --- | --- | --- |
| 2 | R1-1808140 | Correction on starting position of Partial PUSCH Mode 1 for FeLAA in 37.213 | Huawei, HiSilicon |
| 3 | R1-1808585 | Discussion on the AUL-UCI payload for FeLAA in 36.212 | Huawei, HiSilicon |
| 4 | R1-1808587 | Discussion on the partial PUSCH mode field for FeLAA in 36.212 | Huawei, HiSilicon |
| 5 | R1-1808589 | Correction on the interpretation of HARQ-ACK bitmap for FeLAA in 36.212 | Huawei, HiSilicon |
| 6 | R1-1808590 | Correction on DL CP extention for FeLAA in 36.211 | Huawei, HiSilicon |
| 7 | R1-1808591 | Correction on RRC parameters for FeLAA in 37.213 | Huawei, HiSilicon |
| 8 | R1-1808595 | Correction on the AUL-UCI payload for FeLAA in 36.212 | Huawei, HiSilicon |
| 9 | R1-1809576 | Correction on the partial PUSCH mode field for FeLAA in 36.212 | Huawei, HiSilicon |
| 10 | R1-1809577 | Correction on the interpretation of HARQ-ACK bitmap for FeLAA in 36.212 | Huawei, HiSilicon |
| 11 | R1-1809602 | Correction on DL CP extention for FeLAA in 36.211 | Huawei, HiSilicon |
| 12 | R1-1809622 | Correction on RRC parameters for FeLAA in 37.213 | Huawei, HiSilicon |
| 13 | R1-1809650 | Correction on DL CP extention for FeLAA in 36.211 | Huawei, HiSilicon |
| 14 | R1-1809623 | Correction on starting position of Partial PUSCH Mode 1 for FeLAA in 37.213 | Huawei, HiSilicon, Ericsson |
| 15 | R1-1809647 | Correction on starting position of Partial PUSCH Mode 1 for FeLAA in 37.213 | Huawei, HiSilicon, Ericsson |
| 16 | R1-1808464 | Discussion on AUL-initiated COT sharing for LAA | LG Electronics |
| 17 | R1-1809626 | Correction on COT length for AUL transmission | LG Electronics |
| 18 | R1-1809648 | Correction on COT length for AUL transmission | LG Electronics |
| 19 | R1-1809369 | Correction to AUL-UCI resource allocation in 36.213 | Nokia, Nokia Shanghai Bell |
| 20 | R1-1809621 | Correction to AUL-UCI resource allocation in 36.213 | Nokia, Nokia Shanghai Bell |
| 21 | R1-1809652 | Correction to AUL-UCI resource allocation in 36.213 | Nokia, Nokia Shanghai Bell |
| 22 | R1-1809280 | Correction to determination of ending symbol for AUL transmissions | Nokia, Nokia Shanghai Bell, Ericsson |
| 23 | R1-1809281 | Correction to indication of UE-to-eNodeB COT sharing | Nokia, Nokia Shanghai Bell, Ericsson |
| 24 | R1-1809620 | Correction to determination of ending symbol for AUL transmissions | Nokia, Nokia Shanghai Bell, Ericsson, Huawei, HiSilicon |
| 25 | R1-1809651 | Correction to determination of ending symbol for AUL transmissions | Nokia, Nokia Shanghai Bell, Ericsson, Huawei, HiSilicon |
| 26 | R2-1812003 | Introduce feLAA in TS 36.331 | Nokia |
| 27 | R2-1812004 | Adding some missing references for feLAA | Nokia, Nokia Shanghai Bell |
| 28 | R2-1812137 | Introduce feLAA in TS 36.331 | Nokia |
| 29 | R2-1812366 | Miscellaneous Corrections to Stage-2 Specification | Ericsson |
| 30 | R2-1812367 | Aligning feLAA parameter names to RRC | Ericsson |
| 31 | R2-1812368 | Handling Coexistence Between AUL and Dynamically Scheduled UL Grants | Ericsson, Intel |
| 32 | R2-1812369 | Handling Coexistence Between AUL and Dynamically Scheduled UL Grants | Ericsson, Intel |
| 33 | R2-1812370 | Coexistence Between AUL and Dynamically Scheduled UL Grants | Ericsson, Intel |
| 34 | R2-1812380 | Correction to UL Grant | Intel Corporation |
| 35 | R2-1812649 | Correction on channel access priority class for FeLAA | Huawei, HiSilicon |
| 36 | R2-1812650 | Correction on handling of aul-RetransmissionTimer for FeLAA in TS 36.321 | Huawei, HiSilicon |
| 37 | R2-1812651 | Correction on HARQ for FeLAA | Huawei, HiSilicon |
| 38 | R2-1812652 | Correction on IRV handling for FeLAA in TS 36.321 | Huawei, HiSilicon |
| 39 | R2-1812653 | Correction on uplink scheduling for FeLAA | Huawei, HiSilicon |
| 40 | R2-1812654 | Miscellaneous correction to FeLAA in TS 36.321 | Huawei, HiSilicon |
| 41 | R2-1812655 | Issues on coexistence between AUL and SUL | Huawei, HiSilicon |
| 42 | R2-1812656 | Remaining issues with Timer X | Huawei, HiSilicon |

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template