3GPP TSG RAN Meeting 81		 RP-181707

Gold Coast, Australia, September 10th – 13th, 2018

Agenda item:	9.6

Source: 	Qualcomm Incorporated

Title: 	NR UE Features

Document for:	Discussion/Decision

# Discussion

In this document, we discuss proposed changes to the NR UE feature list.

## SRS Tx switch

One of the issues is the unclarity regarding what ‘T=R’ setting means. Our proposal is that this should be clarified as follows: Clarify that ‘T=R’ means that the set of Tx antennas used for PUSCH transmission is a subset of Rx antennas used for PDSCH reception. In addition, we propose that the feature should be optional to enable UE classes that use different antennas for Rx and Tx.

2-13 SRS Tx switch

| 2-55 | SRS Tx switch | 1. Support SRS Tx port switch, 2. Report whether the uplink TX switching impact to downlink receiving in a band, | 2-53 | Yes | SRS Tx Switch is not supported | Type 3 | N.A. | N.A. |  | Component-2 is agreed with conditioned to RAN4’s decision.— | RAN1/4 | Optional with capability signaling Component-1 is a list of TRx pairs, candidates are {“1T2R”, “1T4R”, “2T4R”, “1T4R/2T4R”, “T=R”} Component-2: Candidate value set:, {yes, no}, |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 1:

Clarify that ‘T=R’ means that the set of Tx antennas used for PUSCH transmission is a subset of Rx antennas used for PDSCH reception.

Make the feature optional to enable UE classes that use different antennas for Rx and Tx.

## SRS-based UL beam management

Feature 2-30 is currently named Uplink beam management, although it only covers the version that primarily relates to UEs without beam correspondence capability. This feature should not be mandatory, at least not for UEs wit beam correspondence capability.

2-30 UL beam management

| 2-30 | SRS-based uplink beam management | 1 Support of SRS based beam management 2. Supported max number of SRS resource per set (SRS set use is configured as for beam management). 3. Supported max number of SRS resource sets (SRS set use is configured as for beam management). |  | Yes | Uplink beam management is not supported | Type1 | N.A. | N.A. |  |  |  | Optional, with capability signaling Component-2, candidate value set is {2, 4, 8, 16} Component-3, candidate value set is {from 1 to 8} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 2:

SRS-based uplink beam management should be optional with capability signaling.

## Beam correspondence

Optional/mandatory for beam correspondence is undecided.

2-20 Beam correspondence

| 2-20 | Beam correspondence | 1. Support Beam correspondence 2. When intra-band NR CA is configured in a band, the same beam management/correspondence is applied across all CCs in the band. |  | Yes | Beam correspondence is not supported | Type 1 | N.A. | N.A. |  | Note: Beam correspondence means each Tx port can be beamformed in a desirable direction but does not imply setting phase across ports |  | Mandatory with capability signaling for FR2 Not supported in FR1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 3:

Beam correspondence is mandatory with capability signaling in FR2, not supported in FR1.

Due to the analog beamforming restriction, there is no need to perform individual beam management on each CC in intra-band contiguous CA. Therefore, in the UE capabilities, CCs participating in intra-band contiguous CA should be considered a single beam management/correspondence entity.

## PUCCH spatial relation information

This feature is indicated to have different Optional/Mandatory status for FR1 and FR2 but in the signaling no differentiation is allowed. Make this feature Type 1.

2-24 PUCCH-spatialrelationinfo indication by a MAC CE per PUCCH resource

| 4-24 | PUCCH-spatialrelationinfo indication by a MAC CE per PUCCH resource |  |  | Yes |  | Type 1 | N.A. | N.A. |  |  | RAN1 | Mandatory with capability signaling for FR2 Optional with capability signaling for FR1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4:

Change feature ‘PUCCH-spatialrelationinfo indication by a MAC CE per PUCCH resource’ to Type 1.

## Beam reporting timing

For the case of 30 kHz SCS, the current beam reporting timing is too tight, change the upper limit to a larger value.

2-25 Beam reporting timing

| 2-25 | Beam reporting timing | 1. The number of symbols, Xi, between the last symbol of SSB/CSI-RS and the first symbol of the transmission channel containing beam report is at least RBi, where i is the index of SCS, i=1,2,3,4 corresponding to 15,30,60,120 kHz SCS. | 2-24 | Yes | Beam reporting time capability is not known by gNB | Type 1 | N.A. | N.A. |  |  | RAN1/4 | Candidate value sets: X1 is {2, 4, 8} X2 is {4, 8, 14, 28} X3 is {8, 14, 28} X4 is {14, 28, 56} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 5:

Add 28 symbol as a candidate value for 30kHz SCS beam reporting timing.

## No analog beamforming in FR1

2-26 and 2-27 are analog beamforming related and are not applicable to FR1.

| 2-26 | Receiving beam selection using CSI-RS resource repetition "ON" | 1. Support Rx beam switching procedure using CSI-RS resource repetition "ON" 2. Recommended CSI-RS resource repetition number per resource set, |  | Yes | Rx beam switching is not supported | Type 1 | N.A. | N.A. |  |  |  | Support Rx beam switching is mandatory for bands at least > 6GHz Componet-2: candidate value set {2, 3, 4, 5, 6, 7, 8} Not supported in FR1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-27 | Beam switching | 1. Maximum number of Tx + Rx beam changes a UE can conduct during a slot across the whole band CC BB_Total,. This number is defined as per SCS Note: it is assumed that spec enable the possibility to restrict the same beam across intra-band CCs | 2-24 | Yes. | No restriction on the maximum number of Tx+Rx beam change for a slot | Type 1 | N.A. | N.A |  |  |  | Only applicable to FR2 Candidate value set: {4, 7, 14} |

Proposal 6:

For FR1, there is no need to define features 2-26 and 2-27.

## Limiting number of CSI reports per CC

There is already a capability associated with the number of reports supported across all CCs. There should be a limit on reports per CC as well, to accommodate UEs that process different CCs in different hardware blocks and have a processing limitation per hardware block.

2-35 CSI report framework

| 2-35 | CSI report framework | 1. Maximum number of periodic CSI report setting per BWP 2. Maximum number of aperiodic CSI report setting per BWP 3. Maximum number of semi-persistent CSI report setting per BWP 4. Support of advanced CSI process table (the Minimum duration Z, Z’ for processing a CSI) [Conditioned to the completion of advanced CSI process]4. UE can process Y CSI report(s) simultaneously in a CC. CSI reports can be P/SP/A CSI and any latency class and codebook type. 5. UE can process X CSI report(s) simultaneously across all CCs. CSI reports can be P/SP/A CSI and any latency class and codebook type. FFS: whether X and Y should also count the SRS precoder derivation in case of reciprocity based SRS Tx | 2-32 | Yes | CSI report is not supported | Type 3 | N.A. | N.A. |  | NOTE: Other MIMO capability other than component 5 may further restrict (reduce) the number of simultaneously CSI report that UE is required to update |  | Component-1 candidate values: {1, 2, 3, 4} Component-2 candidate values {1, 2, 3, 4} Component-3 candidate values: {0, 1, 2, 3, 4} Component-4: signaling is a bitmap of size 8 Component-4 candidate values: {from 1 to 8} Component-5: candidate values: {from 5 to 32} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 7:

Introduce a capability for number of simultaneous reports in a CC.

## Codebook based UL MIMO

It is optional to support codebook based UL MIMO. The codebook coherency subset indication should be conditional based on codebook based MIMO support.

2-13 PUSCH codebook coherency subset

| 2-13 | PUSCH codebook coherency subset | Proposal: 1. Supported codebook coherency subset type: Candidate value set: {non-coherent, partial/non-coherent, full/partial/non-coherent} | 2-12 | Yes | Only non-coherent codebook subset is supported | Type 1 | N.A. | N.A. |  | Note: Not included if 2-14 is set to no-codebook based MIMO |  | Candidate values: {non-coherent, partial-non-coherent, full-coherent} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 8:

Add a note saying that PUSCH codebook coherency subset is not included when the UE indicates no codebook-based MIMO capability.

## DL mini-slot capability

With the last change, the inclusion of PDSCH for Message 4 was deleted. This should be added back.

5-11 Up to 2 unicast PDSCHs per slot for different TBs for UE processing time Capability 1

| 5-11 | Up to 2 unicast PDSCHs per slot for different TBs for UE processing time Capability 1 | Up to 2 unicast PDSCHs per slot only in TDM is supported for Capability 1 1) PDSCH(s) for Msg. 4 is included |  | Yes |  | Type 3 | N.A. | N.A. |  | This capability is necessary for each SCS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 9:

Add back the condition that Message 4 is counted as part of the limit on number of TBs per slot.

## Rate matching

The types of rate matching had been previously accidentally deleted and then added back at RAN1 #94. However, the control resource set rate matching was not included in the current wording.

2-33a Supported PDSCH RE-mapping patterns

| 2-33a | Supported PDSCH RE-mapping patterns Note: this FG will be moved to 5-x family | 1. Supported max # of RE mapping patterns (including NZP/ZP CSI-RS, CRS and control resource set and bitmap configured in 5-26/27/27a) Note: patterns are counted as per symbol per CC 2. Supported max # of RE mapping patterns (including NZP/ZP CSI-RS, CRS and control resource set and bitmap configured in 5-26/27/27a) Note: patterns are counted as per slot per CC |  | Yes | PDSCH RE mapping is not supported | Type 4 | No need | Yes |  |  |  | candidate values: {10, 20} for FR1 {6, 20} for FR2 [Compponent-2 candidate values: {from 16: 16: 256 16} for FR1 {16: 16: 256} for FR2] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 10:

Include CORESET in the number of rate matching patterns.

## Remaining decisions on optional/mandatory

In the feature list, there are missing definitions for optional/mandatory for several features. We propose to add the following.

| 1-8 | RLM based on a mix of SS block and CSI-RS signals |  | 1-4 and 1-7 | Yes | UE does not support RLM based on a mix of SS block and CSI-RS signals | Type 4 | No need | No need |  |  | RAN1 | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-34 | NZP-CSI-RS based interference measurement | 1. Support NZP-CSI-RS based interference measurement FFS whether to add the number of NZP-CSI-RS port for interference measurement | 2-33 | Yes | NZP-CSI-RS based interference measurement is not supported | Type 4 | No need | No need |  |  |  | Optional with capability signaling |
| 2-44 | Basic DL PTRS | Support 1 port of PTRS |  | Yes | DL PTRS is not supported | Type 4 | N.A. | Yes |  |  |  | Mandatory with UE capability signaling for FR2 Optional with capability signaling for FR1 |
| 6-16 | Supplemental uplink | 1) RACH, PUSCH, PUCCH, SRS operations in a band combination including SUL 2) Supplemental uplink with same numerology between SUL and non SUL carriers | 6-15 | Yes | UE will not be able to perform the RACH/PUSCH/PUCCH/SRS operation in a band combination including SUL | N.A. | N.A. | N.A. |  | This is conditioned on the support of SUL band combination(s). The band combination definition is up to RAN4. |  | Optional with capability signaling |
| 6-18 | Supplemental uplink with dynamic switch | DCI based selection of PUSCH carrier | 6-16 | Yes |  | Type 3 | N.A. | N.A. |  | This is conditioned on the support of SUL band combination(s). The band combination definition is up to RAN4. |  | Optional with capability signaling |

Proposal 11:

Resolve remaining definitions of optional/mandatory as noted in the tables in this contribution.

# Conclusions

In this contribution, the following proposals have been made:

Proposal 1:

Clarify that ‘T=R’ means that the set of Tx antennas used for PUSCH transmission is a subset of Rx antennas used for PDSCH reception.

Make the feature optional to enable UE classes that use different antennas for Rx and Tx.

Proposal 2:

SRS-based uplink beam management should be optional with capability signaling.

Proposal 3:

Beam correspondence is mandatory with capability signaling in FR2, not supported in FR1.

Due to the analog beamforming restriction, there is no need to perform individual beam management on each CC in intra-band contiguous CA. Therefore, in the UE capabilities, CCs participating in intra-band contiguous CA should be considered a single beam management/correspondence entity.

Proposal 4:

Change feature ‘PUCCH-spatialrelationinfo indication by a MAC CE per PUCCH resource’ to Type 1.

Proposal 5:

Add 28 symbol as a candidate value for 30kHz SCS beam reporting timing.

Proposal 6:

For FR1, there is no need to define features 2-26 and 2-27.

Proposal 7:

Introduce a capability for number of simultaneous reports in a CC.

Proposal 8:

Add a note saying that PUSCH codebook coherency subset is not included when the UE indicates no codebook-based MIMO capability.

Proposal 9:

Add back the condition that Message 4 is counted as part of the limit on number of TBs per slot.

Proposal 10:

Include CORESET in the number of rate matching patterns.

Proposal 11:

Resolve remaining definitions of optional/mandatory as noted in the tables in this contribution.

# References

- R1-1805728, “NR UE feature list”, NTT DOCOMO, AT&T

- R1-1800886, “NR features and capabilities”, Qualcomm

- R1-1802849, “NR features and capabilities”, Qualcomm

- R1-1807374, “NR features and capabilities”, Qualcomm

- R1-1809998, “NR UE feature list”, NTT DOCOMO, AT&T

- R1-1809433, “NR Features and Capabilities”, Qualcomm