3GPP TSG RAN Meeting #81		    RP-181751

Gold Coast, Australia, September 10 - 13, 2018

Source:	CMCC, Ericsson, MTK, ZTE, Qualcomm, Intel, Nokia, Huawei

Title:	Collection of evaluation results for high speed train performance with R15 numerologies

Agenda Item:	9.8

Document for:	Discussion and Decision

- Introduction

As stated in our RAN #79 contribution [1], for high speed scenario, the enhancements of NR in contrast to LTE include scalable subcarrier spacing (SCS) and additional DMRS configuration. Considering the resistance of the ICI caused by high mobility, different SCSs provide different performance. For normal CP configuration, larger SCS indicates shorter CP durations, which is more sensitive to large delay spread. Therefore, in order for NR to provide better UE experience than LTE, the numerology (combination) should be carefully examined.

Link level simulations to compare the high speed performance of different numerologies are conducted among several companies, the evaluation results are shown in this contribution. And observation are made here.

The numerologies considered here are, 15KHz NCP, 30KHz NCP, 60KHz NCP and 60 KHz ECP.

- Evaluation Methodology

A SFN deployment is adopted, as shown in Figure 1, where an infinite number of RRHs are equidistantly distributed along the track, and the UE connects to the 4 nearest RRHs that share the same cell ID.

Figure 1. Deployment of HST-SFN

UE moves along the railway track, and experiences different Doppler shifts and path delays at different locations. To check UE’s performance,

- A number of UE locations along the track are chosen, and link level simulation (LLS) is performed for each location to evaluate the throughput performance for each numerology.

- Then throughput average of all the locations can be the performance of this numerology.

- Performance of different numerologies are compared.

To simplify the evaluation, no handover is considered. Therefore, only the region that between any two adjacent RRHs needs to be examined. A number of UE locations within this region are chosen. To initialize the LLS, some basic assumptions and parameters are provided as follows:

  - Number of UE locations to be evaluated

Denote the inter-RRH distance as, and the distance step between two evaluated UE locations as,　there are  UE locations between the two adjacent RRHs, where , supposing thatdivides . Considering the deployment symmetry, the number of UE locations that to be evaluated is.

  - SNR of each location

In the LLS, the SNR value to be evaluated for each location should be determined, and a simple relative SNR setting is considered here to reflect the SNR relation between the locations. Considering the region between RRH1 and RRH2, the UE location that is closest to RRH1 is set as the reference location A, as shown in figure.2. Denote the SNR of location A as, the sum of LOS path power of all the connected RRHs at location A as , and the sum of LOS path power of all the connected RRHs for another UE location B as , then the SNR of location B is computed by, as depicted in Figure 2.

Figure 2. The relative SNR of the evaluated UE locations

  - MCS for each location

For the changing SNR, adaptive MCS is straight way. However, the impact of adaptive algorithms will bring uncertainty in the results. To eliminate the impact of adaptive algorithms, a series of MCSs can be evaluated at SNR of each location, and the throughput of each location is given by the throughput of MCS which provide the best throughput. Therefore, the MCS changes at different locations.

- Evaluation Assumptions

  - Channel model

The merged model from CDL and 36.101 B.3A is adopted in the evaluation. Assume that an infinite number of RRHs distributed equidistantly along the track with the same Cell ID, and the UE connects to the nearest 4 RRHs, as depicted in Figure 1.

The location of the th RRH is expressed as

(3.1)

where , , and  is the distance between RRHs and the track, and is inter-RRH distance.  and  are in meters. The values of   and  are provided in table 1, which come from average data of some practical deployment.

Table 1 Parameters for channel model

| Parameter | Value |
| --- | --- |
|  | 720 m |
|  | 120 m |

The UE (train) location is denoted as

(3.2)

where  is the speed of the UE. The channel between the th Tx antenna and the th antenna at UE is given by

(3.3)

where  is the channel between the th Tx antenna of the th RRH to the th antenna at UE, which is

(3.4)

with

(3.5)

and

(3.6)

and , where  is the delay of th RRH, which can be derived as

for                (3.7)

and  is the delay of the th channel cluster, shown in Table 7.7.1-1~7.7.1-5 in 38.901. Note that in the evaluation, the AOA, AOD, ZOA, ZOD shall be modified according to UE locations based on AOA, AOD, ZOA, ZOD in Table 7.7.1-1~7.7.1-5 in 38.901. In (3.3), the  (dB) for the signal from th RRH, normalized to the total power received from all visible RRHs, is given by

(3.8)

  - Simulation parameters

The distance step between two evaluated UE locations is 72 m, therefore six locations named A,B,C,D,E,F are evaluated. And SNR for the reference location A is 6 dB / 9 dB / 12 dB. The SNR for the 6 locations are [6, 5, 3, 1, 0, -1] dB, [9, 8, 6, 4, 3, 2] dB, and [12, 11, 9, 7, 6, 5] dB when SNRA=6 dB, 9 dB, and 12 dB, respectively

For each location, PDSCH with the following transmission schemes are considered,

  - For 2 GHz with 2T2R

precoding cycling with 2RB granularity

  - For 3.5 GHz with 64T4R

Single layer transmission, and the CQI feedback behavior are provided as the following in 38.214 section 5.2.1.4:

When a UE is configured with higher layer parameter codebookType set to 'TypeI-SinglePanel' and pmi-FormatIndicator is configured for single PMI reporting, the UE may be configured with reportQuantity to report:

……

- RI (if reported), CRI (if reported), CQI, and a PMI consisting of a single wideband indication ( in Subclause 5.2.2.2.1) for the entire CSI reporting band. The CQI is calculated conditioned on the reported  assuming PDSCH transmission with   precoders (corresponding to the same but different  in sub-clause 5.2.2.2.1), where the UE assumes that one precoder is randomly selected from the set of   precoders for each PRG on PDSCH, where the PRG size for CQI calculation is configured by the higher layer parameter pdsch-BundleSizeForCSI

The antenna related parameters are listed in table.2

Table 2 antenna related parameters and values

| Parameters | Values |  |
| --- | --- | --- |
| Antenna configuration | GHz: gNB: 2T, UE: 2R 3.5 GHz: gNB: 64T, UE: 4R |  |
| RRH | 128 Tx/Rx, (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1), (dH,dV) = (0.5, 0.8)λ, +45°, -45° polarization Map to 64TXRU, (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1) |  |
| UE | 4 Tx/Rx, (M, N, P, Mg, Ng) = (1,2,2,1,1), (dH,dV) = (0.5, N/A)λ, 0°,90° polarization Map to 4TXRU, 1-to-1 mapping |  |
| Downtilt Angle | 5.5°, where is the RRH height |  |
| Azimut Angle | 72° |  |
| Radiation power pattern of a single antenna element for RRH | Vertical cut of the radiation power pattern (dB) |  |
|  | Horizontal cut of the radiation power pattern (dB) |  |
|  | 3D radiation power pattern (dB) |  |
|  | Maximum directional gain of an antenna element GE,max | 8 dBi |
| UE antenna | omni-directional antenna element with 0 dBi |  |

Other simulation parameters are listed in table.3

Table 3 Simulation parameters and values

| Parameters | Values |
| --- | --- |
| bandwidth | 20 MHz |
| Numerologies (subcarriers/CP) | 15K NCP 30KHz NCP 60KHz NCP 60KHz ECP |
| Mobility | 350km/h, 500km/h |
| Number of RBs | 106, 51, 24 corresponding to 15K，30K and 60KHz subcarrier spacing. |
| PDCCH overhead | 2 OFDM symbols |
| RRH height | 35 m |
| UE height | 1.5 m |
| Channel model | CDL-E |
| Delay spread | 100 ns |
| MCS | Depends on the SNR of each evaluated UE location |
| HARQ | off |
| Channel estimation | Practical |
| Receiver type | MMSE |
| DMRS configuration | DMRS Configuration 1. Front-load+additional DMRS: Mandatory: 1+3 for 15KHz ; 1+3 for 30 kHz; 1+1 for 60 kHz Optional: 1+2 for 30 kHz and 60 kHz; |
| CSI-RS port | 16 ports |

- Simulation Results

As to the metric for comparison, since A, B, C, D, E and F only represent half of the inter-site distance, throughput of B,C,D,E should be weighted twice when average throughput is calculated, while A and F weighted only once, as shown in figure.3

Figure. 3 calculation weight for different location

  - Results of 500 km/h at 3.5 GHz

From the collected results, it is observed that one group of results shows that 30 KHz with NCP performs better, and another group of result shows 60KHz with NCP performs better.

    - Resources showing 30KHz with NCP performs better

All companies give results for SNRA =6dB, some also provide results of SNRA =9dB and 12dB. The weighted average throughput results with 500km/h at 3.5 GHz from Ericsson, ZTE, MTK, Qualcomm, Intel and CMCC are provided as follows. Ericsson, Qualcomm and MTK also provide performance of different additional DMRS. Note that Ericsson uses RAN4 SFN model in 36.101 B.3A.

| Source | SNR_A = 6 dB | SNR_A = 9 dB | SNR_A = 12 dB |
| --- | --- | --- | --- |
| Ericsson |  |  |  |
| ZTE |  |  |  |
| MTK |  | N/A | N/A |
| Qualcomm |  | N/A | N/A |
| Intel |  | N/A | N/A |
| CMCC |  |  |  |

The performance gain of 30KHz NCP over 60KHz NCP is shown in the following,

Ericsson: 2%~4.5%;

MTK: 8.8%;

ZTE: 5.6%~12.6%;

Qualcomm: 6%;

Intel: 36%;

CMCC: 6.8%

    - Resources showing 60 kHz with NCP performs better

The weighted average throughput with 500km/h at 3.5 GHz from Huawei and Nokia are provided as follows.

| Source | SNR_A = 6 dB | SNR_A = 9 dB | SNR_A = 12 dB |
| --- | --- | --- | --- |
| Huawei |  |  |  |
| Nokia |  |  |  |

The performance gain of 60 KHz NCP over 30 KHz NCP is shown in the following,

Huawei: 5.7%~20%;

Nokia: 13.7~17%;

Comparing 60 KHz ECP with results of 30 KHz and 60 KHz (for companies providing results of different DMRS configurations, the best results for each numerology is chosen), all results show that 60KHz ECP is worse than 30KHz and 60KHz NCP with the current evaluation assumption.

Observation 1: for 3.5GHz with 500km/h,

- Results of 6 companies, e.g Ericsson, ZTE, MTK, Qualcomm, Intel and CMCC illustrate that 30 kHz NCP performs better than other numerologies. And the performance gain varies from 2%~36%;

- Results of 2 companies, e.g Nokia and Huawei illustrate that 60 kHz NCP performs better than other numerologies. And the performance gain varies from 5.7%~20%;

Observation 2: for 3.5GHz with 500km/h, 15KHz numerology performs worst among all the numerologies.

Observation 3: all results shows that performance of 60KHz ECP is worse than 30KHz and 60KHz NCP for 3.5GHz and 500km/h with current evaluation simulation.

As to the performance of different additional DMRS, Ericsson, Qualcomm, Nokia and MTK provide corresponding results. However, the trends are not aligned. For 3.5GHz with 500km/h, the results show as,

For 30KHz NCP:

- 1 Front loaded+2 additional  DMRS provides better performance than 1+3 DMRS.

  - Ericsson (gain: 2.5%~3.9%), Qualcomm(gain 4.3%), MTK(gain 4.4%)

- 1 Front loaded+3 additional DMRS provide better performance than 1+2 DMRS.

  - Nokia (gain:18.7%~28.6%)

For 60KHz NCP:

- 1 Front loaded+1 additional DMRS provide better performance than 1+2 DMRS.

  - Nokia (gain:4.7%~10%)

- 1 Front loaded+2 additional DMRS provide better performance than 1+1DMRS.

  - Qualcomm (gain:2.8%)

- 1 Front loaded+2 additional DMRS provide similar performance as 1+1DMRS

  - Ericsson

Observation 4: for 3.5GHz with 500km/h deployment and 30KHz NCP,

-	Results of 3 companies show that 1 Front loaded+2 additional DMRS performs better than 1 Front loaded+3 additional DMRS.

- Ericsson (gain: 2.5%~3.9%), Qualcomm (gain 4.3%), MTK (4.4%);

-	Results of 1 company show that 1 Front loaded+3 additional DMRS performs better than 1 Front loaded+2 additional DMRS.

- Nokia (gain: 18.7%~28.6%)

Observation 5: for 3.5GHz with 500km/h deployment and 60KHz NCP,

-	Results of 1 company show that 1 Front loaded+1 additional DMRS performs better than 1 Front loaded+2 additional DMRS.

- Nokia (gain: 4.7%~10%)

-	Results of 1 company show that 1 Front loaded+2 additional DMRS performs better than 1 Front loaded+1 additional DMRS.

- Qualcomm (gain: 2.8%)

- Results of 1 company show that 1 Front loaded+1 additional DMRS provide similar performance as 1 Front loaded+2 additional DMRS

- Ericsson.

  - Results of 350 km/h at 2 GHz

The weighted average throughput with 350km/h at 2 GHz from Ericsson is provided as follows.

| Source | SNR_A = 6 dB | SNR_A = 9 dB | SNR_A = 12 dB |
| --- | --- | --- | --- |
| Ericsson |  |  |  |

It can be seen from Ericsson’s results that for 350 km/h at 2 GHz, 60 kHz ECP performs worse than other numerologies, 30 kHz 1+2 DMRS provides the best performance while the differences with other numerologies are not very significant.

The detailed evaluation results can be seen in following attached excel document. Noted that the results present diverse performance trends when UE changes positions. The diverse may come from different beamforming realization, different channel model details, different receiver algorithms, and etc. Although we have tried to make them aligned, the different implementation details may affect the comparison conclusion.

- Conclusions

In this contribution, we collect and compare the weighted average throughput for different numerologies in the high speed train scenario and have the following observations:

Observation 1: for 3.5GHz with 500km/h,

-	Results of 6 companies, e.g Ericsson, ZTE, MTK, Qualcomm, Intel and CMCC illustrate that 30 kHz NCP performs better than other numerologies.

-	Results of 2 companies, e.g Nokia and Huawei illustrate that 60 kHz NCP performs better than other numerologies.

Observation 2: for 3.5GHz with 500km/h, 15KHz numerology performs worst among all the numerologies.

Observation 3: all results shows that performance of 60KHz ECP is worse than 30KHz and 60KHz NCP for 3.5GHz and 500km/h with current evaluation simulation.

Observation 4: for 3.5GHz with 500km/h deployment and 30KHz NCP,

-	Results of 3 companies show that 1 Front loaded+2 additional DMRS performs better than 1 Front loaded+3 additional DMRS.

- Ericsson (gain: 2.5%~3.9%), Qualcomm (gain 4.3%), MTK (4.4%);

-	Results of 1 company show that 1 Front loaded+3 additional DMRS performs better than 1 Front loaded+2 additional DMRS.

- Nokia (gain: 18.7%~28.6%)

Observation 5: for 3.5GHz with 500km/h deployment and 60KHz NCP,

-	Results of 1 company show that 1 Front loaded+1 additional DMRS performs better than 1 Front loaded+2 additional DMRS.

- Nokia (gain: 4.7%~10%)

-	Results of 1 company show that 1 Front loaded+2 additional DMRS performs better than 1 Front loaded+1 additional DMRS.

- Qualcomm (gain: 2.8%)

- Results of 1 company show that 1 Front loaded+1 additional DMRS provide similar performance as 1 Front loaded+2 additional DMRS

- Ericsson.

References

- RP-180308, Study item on high speed train performance validation with Rel-15 NR numerologies, CMCC, RAN#79.

- TS 38.214, Physical layer procedures for data.

- L. Tian, J. Zhang, and C. Pan, “Small scale fading characteristics of wideband radio channel in the U-shape cutting of high-speed railway,” in Proc. IEEE 78th VTC, 2013.

- T. Zhou, “Research on radio channels in high-speed railway scenarios: propagation characteristics, modeling, and measurement methods,” PhD Dissertation, Beijing: Beijing Jiaotong University, 2016.

Notes: Referring to Table III in [3], the measured mean delay spread is 75ns under the scenario that Tx-Rx distance is 50-400m with LOS path, and referring to Table 2.5 in [4], the measured mean delay spread is 68 ns under the viaduct scenario, and is 56 under the U-shaped cutting scenario. Based on these measurements, the delay spread is set to 100ns in the evaluation.