6	Self evaluation of URLLC technical performance

6.1	Reliability

As defined in Report ITU-R M.2412, reliability is the success probability of transmitting a layer 2/3 packet within a required maximum time, which is the time it takes to deliver a small data packet from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface at a certain channel quality.

6.1.1	NR

NR Reliability is evaluated under Urban Macro – URLLC test environment. Both downlink and uplink are evaluated.

6.1.1.1	DL reliability

For downlink reliability, both evaluation configuration A (carrier frequency = 4 GHz) and evaluation configuration B (carrier frequency = 700 MHz) are evaluated. A variety of configurations are evaluated. Detailed assumptions and results are provided in RP-181820.

The evaluation results of NR FDD for downlink reliability are provided in Table 6.1.1.1-1. All the evaluation results are derived with less than 100 MHz and 40 MHz bandwidth for evaluation configuration A (CF = 4 GHz) and B (CF = 700 MHz), respectively. It is observed that NR FDD fulfils the reliability requirement for downlink in a wide range of configurations.

Table 6.1.1.1-1 Evaluation results of downlink reliability for NR FDD

- Evaluation configuration A (CF = 4 GHz)

| Scheme and antenna configuration | Sub-carrier spacing [kHz] | ITU Requirement | Channel condition | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | Reliability | Number of samples | Reliability |
| 2x2 SU-MIMO 14os1 slot, slot aggregation, (1 PDCCH + 2 PDSCH) | 60 | 99.999% | NLOS | 1 | 99.999899% | 1 | 99.99991% |
| 2x2 SU-MIMO, 4os non-slot, HARQ re-tx (2 PDCCH + 2 PDSCH) | 30 | 99.999% | NLOS | 1 | 99.999898% | 1 | 99.99995% |
| 2x2 SU-MIMO 4os non-slot, TBD | 30 | 99.999% | NLOS | - | - | 1 | 99.99969% |
| 2x4 SU-MIMO 7os non-slot, One shot, (1 PDCCH + 1 PDSCH) | 30 | 99.999% | NLOS | 1 | > 99.9999% | 1 | > 99.9999% |
| 32x8 SU-MIMO 14os slot, One shot (1 PDCCH + 1 PDSCH) | 30 | 99.999% | NLOS | 1 | 99.9999% | － | － |
| NOTE1: os = OFDM symbol |  |  |  |  |  |  |  |

- Evaluation configuration A (CF = 700 MHz)

| Scheme and antenna configuration | Sub-carrier spacing [kHz] | ITU Requirement | Channel condition | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | Reliability | Number of samples | Reliability |
| 2x2 SU-MIMO 14os slot, One shot (1 PDCCH+1 PDSCH) | 30 | 99.999% | NLOS | 1 | 99.9998% | 2 | 99.9994% |
| 2x2 SU-MIMO 7os non-slot, one-shot (1 PDCCH+1 PDSCH) | 30 | 99.999% | NLOS | 3 | 99.9996% | 4 | 99.9997% |
| 2x2 SU-MIMO, 4os non-slot, slot aggregation (1 PDCCH + 2 PDSCH) | 30 | 99.999% | NLOS | 2 | 99.9997% | 2 | 99.9998% |
| 16x4 SU-MIMO, 14os slot, One short (1PDCCH + 1 PDSCH) | 30 | 99.999% | NLOS | 1 | 99.9999% | － | － |

6.1.1.2	UL reliability

For uplink reliability, both evaluation configuration A (carrier frequency = 4 GHz) and evaluation configuration B (carrier frequency = 700 MHz) are evaluated. A variety of configurations are evaluated. Detailed assumptions and results are provided in RP-181820.

The evaluation results of NR FDD for uplink reliability are provided in Table 6.1.1.2-1. All the evaluation results are derived with less than 100 MHz and 40 MHz bandwidth for evaluation configuration A (CF = 4 GHz) and B (CF = 700 MHz), respectively.

Table 6.1.1.2-1 Evaluation results of uplink reliability for NR FDD

(a) Evaluation configuration A (CF = 4 GHz)

| Scheme and antenna configuration | Sub-carrier spacing [kHz] | ITU Requirement | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Channel condition | Number of samples | Reliability | Channel condition | Number of samples | Reliability |
| 1x16 SIMO, OFDMA , 14os slot, Grant free, One shot (1 PUSCH) | 60 | 99.999% | NLOS | 1 | 99.9999% | NLOS | 1 | 99.99999% |
| 1x2 SIMO, OFDMA, 7os non-slot, Grant free, 3 repetitions (3 PUSCH) | 30 | 99.999% | NLOS | － | － | NLOS | 1 | 99.9998% |
| 8x32 SU-MIMO, 14os slot, Grant based, (1 PDCCH + 1 PUSCH) | 30 | 99.999% | LOS | 1 | 99.9999% | LOS | － | － |
| 1x8 SIMO 7os non-slot Grant free One short (1 PUSCH) | 30 | 99.999% | NLOS | 1 | > 99.9999% | NLOS | 2 | 99.9999% |
| 1x2 SIMO 4os non-slot Grant free, 2 repetitions (2 PUSCH) | 30 | 99.999% | NLOS | 1 | 99.9999999992% | NLOS | 1 | 99.9999999887% |
| 1x8 SIMO 14os slot, Grant free One shot (1 PUSCH) | 30 | 99.999% | NLOS | － | － | NLOS | 1 | 99.9992% |

(b) Evaluation configuration B (CF = 700 MHz)

| Scheme and antenna configuration | Sub-carrier spacing [kHz] | ITU Requirement | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Channel condition | Number of samples | Reliability | Channel condition | Number of samples | Reliability |
| 1x8 SIMO, OFDMA, 14os slot, Grant free One shot (1 PUSCH) | 30 | 99.999% | NLOS | 1 | 99.99999% | NLOS | 1 | 99.9999997% |
| 1x2 SIMO, OFDMA, 7os non-slot, Grant free, 2 repetitions (2 PUSCH) | 30 | 99.999% | NLOS | － | － | NLOS | 2 | 99.99996% |
| 4x16 SU-MIMO 14os slot, Grant based (1 PDCCH + 1 PUSCH) | 30 | 99.999% | NLOS | 1 | 99.9999% | NLOS | － | － |
| 1x8 SIMO 7os non-slot, Grant free, One shot (1 PUSCH) | 30 | 99.999% | NLOS | 1 | > 99.9999% | NLOS | 1 | > 99.9999% |
| 1x2 SIMO, OFDMA, 4os non-slot, Grant free, 2 repetitions (2 PUSCH) | 30 | 99.999% | NLOS | 1 | 99.99999999% | NLOS | 1 | 99.99999999% |
| 1x8 SIMO, OFDMA, 14os slot, Grant free, One shot (1 PUSCH, 8RB) | 30 | 99.999% | NLOS | 1 | 99.9993% | NLOS | 1 | 99.9993% |
| 1x8 SIMO, OFDMA, 14os slot, Grant free One shot (1 PUSCH, 12RB) | 30 | 99.999% | NLOS | 1 | 99.9992% | NLOS | 1 | 99.9992% |
| 1x8 SIMO, OFDMA, 7os non-slot, One shot (1 PUSCH, 12RB) | 30 | 99.999% | NLOS | 1 | 99.9993% | NLOS | 1 | 99.9993% |

The evaluation results of NR TDD for uplink reliability are provided in Table 6.1.1.2-2. All the evaluation results are derived with less than 100 MHz and 40 MHz bandwidth for evaluation configuration A (CF = 4 GHz) and B (CF = 700 MHz), respectively.

It is observed that NR fulfils the reliability requirement for uplink in a wide range of configurations.

Table 6.1.1.2-1 Evaluation results of uplink reliability for NR TDD

(a) Evaluation configuration A (CF = 4 GHz)

| Scheme and antenna configuration | Sub-carrier spacing [kHz] | Frame structure | ITU Requirement | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Channel condition | Number of samples | Reliability | Channel condition | Number of samples | Reliability |
| 2x8 SIMO OFDMA, 4os non-slot, Grant free, 2 repetitions (2 PUSCH) | 60 | 0.5ms periodicity DL:UL=1:1 | 99.999% | NLOS | 1 | 99.9999910% | NLOS | － | － |