3GPP TSG RAN meeting #81										RP-181873

Gold Coast, Australia, September 10-13, 2018

## Status Report to TSG

Agenda item:			10.7.1

| WI / SI Name | Additional MTC enhancements for LTE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| included in this status report | Core part: | Yes | Perf. part: | Yes |  | Testing part: | No |
| Acronym | LTE_eMTC5 |  |  |  |  |  |  |
| Unique ID | 800083 |  |  |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-181450 |  |  |  |  |  |  |
| Target Completion Date | Core part: 12/2019 |  |  |  | Performance part: 06/2020 |  |  |
| Is the WI/SI complete? | Core part: No |  |  |  | Performance Part: No |  |  |

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Johan BERGMAN |
|  | Company | Ericsson |
|  | Email | johan.bergman@ericsson.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

Additional explanations/motivations for the time budget changes in the attached Excel table:

Request for RAN3 TUs (which was missed in the original TU request approved in RAN#80)

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

## 2.1	RAN1

#### 2.1.1	Agreements

RAN1#94

To this meeting, 96 contributions were submitted (for details see agenda item 6.2.1 in Tdoc list). A WI work plan was provided and presented in R1-1808033.

RAN1 discussed UE-group wake-up signal, with the following agreements:

Agreement UE-group WUSs are only multiplexed in the same NB as associated PO - FFS TDM/FDM/CDM for UE-group MWUS multiplexing Agreement - UE-group MWUS is supported based on eNB’s and UE’s capability. - Whether the network supports UE-group MWUS is done by higher layer signalling. - FFS: The number of UE groups is configured by SIB. - Note that the UE-group MWUS is UE optional Agreement Rel-16 UE-group MWUS sequence should consider at least - Fallback to legacy UE behavior - Inter-cell interference randomization - UE group ID for different UE-group MWUS - Reuse of Rel-15 sequences is not precluded - Effect of sequence detection on UE complexity Agreement Study the RAN1 consequence of UE-grouping on the following basis: - UE ID - Coverage - DRX/eDRX - Gap configuration - Services

RAN1 discussed support for transmission in preconfigured UL resources, with the following agreements:

Agreement Idle mode based pre-configured UL resources is supported for UEs in possession of a valid TA - FFS: Validation mechanism for TA - FFS: How the pre-configured UL resources is acquired Agreement For transmission in preconfigured UL resources, the UE may use the latest TA of which its validity can be confirmed Agreement Study both shared and dedicated resource for preconfigured UL resources. If both shared and dedicated resources are supported, strive for commonality in design of both resource types. Agreement HARQ procedures for transmission in preconfigured UL resources should be studied and the following aspects should be considered: - Whether to support HARQ; - If supported, details of HARQ design including the number of HARQ processes; - Whether ACK/NACK is necessary Fallback mechanisms should be considered, e.g. fallback to legacy RACH/EDT procedures.

RAN1 discussed scheduling of multiple DL/UL transport blocks, with the following agreements:

Agreement Specify scheduling of multiple transport blocks for both CE Mode A and B Agreement The possibility of scheduling multiple DL/UL transport blocks is configured via RRC. Details TBD Agreement When scheduling of multiple TBs is enabled, the number of scheduled transport blocks (>= 1) should be dynamically selected via DCI. The maximum number of scheduled transport blocks with one single DCI is [TBD]. - The number of blind decodes for MPDCCH is not increased as a result of scheduling multiple TBs Conclusion When multiple TBs are scheduled by one DCI, study interleaving amongst TBs from different HARQ process in cases of repetitions - Companies are encouraged to submit evaluation results in the next RAN1 meeting Agreement One DCI to schedule multiple TBs for SC-MCCH is not supported Working assumption For unicast, when multiple DL/UL transport blocks are assigned by a single DCI, each transport block corresponds to a unique HARQ process.

RAN1 discussed coexistence of LTE-MTC with NR, with the following agreements:

Observation From RAN1 perspective, no issues were identified that would prevent the coexistence of NR and eMTC Agreement RAN1 studies additional specification enhancement for improving the performance of coexistence of eMTC with NR.

RAN1 discussed support of quality report in Msg3, with the following agreements:

Agreement Prioritize the following alternatives for DL quality report in Msg3 in MTC, for CE Mode A and CE Mode B separately: - CQI (for CE Mode A) - Repetition number related to UE decoding of actual or hypothetical MPDCCH/PDSCH - FFS if aggregation level needs to be reported when repetition number equal to 1 - RSRP/RSRQ Companies are encouraged to provide details and/or performance evaluation results Agreement Whether the DL quality report is included in Msg3 is indicated in SIB and/or RAR. Above applies in case the UE supports DL quality report in Msg3.

RAN1 discussed MPDCCH performance improvements, with the following agreements:

Agreement CRS for improving channel estimation on USS MPDCCH is supported Agreement The relation between the MPDCCH DMRS and CRS is defined for enabling improved channel estimation. FFS the details on specifying the relation (e.g., whether the ‘definition’ means predefined in specification or done by signalling, relation between the CRS ports and DMRS ports, power offset between DMRS-CRS). Agreement eNB enables/disables “CRS for improving channel estimation on MPDCCH” via dedicated or broadcasted RRC signaling. FFS the details on the signaling. Agreement CRS for improving channel estimation on Type0-MPDCCH CSS MPDCCH is supported.

RAN1 discussed CE mode A and B improvements for non-BL UEs, with the following agreements:

Agreement Study on the performance benefit of dual layer DL reception when it is supported for non-BL UEs in CE mode A. Companies are encouraged to submit evaluation results. - UE complexity should be also considered - Prioritize SNR region relevant to CE mode A Agreement Study on the performance benefit of CSI-RS based CSI feedback when it is supported for non-BL UEs in CE mode A. - UE complexity should be also considered - Prioritize SNR region relevant to CE mode A

RAN1 discussed use of LTE control channel region for DL transmission, with the following agreements:

Agreement FFS whether and for what broadcast transmission (e.g. SIBs, paging, SC-PTM, Msg2, etc) use of LTE control channel region for MPDCCH and PDSCH are applied Agreement The following options can be considered for transmission of MPDCCH and PDSCH in LTE control channel region: For MPDCCH - Option 1: All or part of the MPDCCH are mapped into the DL control region. - Option 2: MPDCCH are rate-matched to all OFDM symbols. FFS on RE mapping details. For PDSCH - Option A: All or part of the PDSCH are mapped into the DL control region. - Option B: PDSCH are rate-matched to all OFDM symbols. FFS on RE mapping details.

RAN1 discussed use of RSS for measurement improvements, with the following agreements:

Agreement From RAN1 perspective, it is feasible to use RSS for measuring RSRP for cells at least for IDLE mode mobility - RAN1 to identify the related parameters in the next RAN1 meeting

#### 2.1.2	Remaining Open issues

Specify physical layer aspects of the following improvements for BL/CE UEs:

- Specify support for UE-group wake-up signal (WUS)

- Specify support for transmission in preconfigured resources in idle and/or connected mode based on SC-FDMA waveform for UEs with a valid timing advance

- Specify scheduling multiple DL/UL transport blocks with or without DCI for SC-PTM and unicast

- Conclude on additional potential specification enhancement for improving the performance of coexistence of LTE-MTC with NR

- Specify quality report in MSG3 at least for EDT

- Specify MPDCCH performance improvement by using CRS at least for connected mode

- Specify CE mode A and B improvements for non-BL UEs

- Enable the use of LTE control channel region for DL transmission (MPDCCH/PDSCH) to BL/CE UEs

- Consider improving the DL RSRP and, if needed, RSRQ measurement accuracy, through use of RSS

## 2.2	RAN2

Not yet started

## 2.3	RAN3

Not yet started

## 2.4	RAN4

Not yet started

## 2.5	RAN5

N/A

## 2.6	RAN6

N/A

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

N/A

## 4.	References

RAN1#94

96 contributions (for details see agenda item 6.2.1 in Tdoc list)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template