TSG-RAN Working Group 4 (Radio) meeting #88	R4-1811924

# Gothenburg, Sweden, 20th – 24th August 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.817-02 | CR | 0001 | rev | 2 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 38.817-02: Update to OTA Modulation Quality |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-08-20 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Update to section OTA modulation quality |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added OTA modulation quality information for FR2. Update to OTA modulation quality information for FR1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete technical background for OTA modulation quality |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.6.6, 9.6.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 9.6.6	OTA modulation quality for FR1

OTA transmit modulation signal quality is measured in terms of error vector magnitude (EVM).  EVM captures a maximum allowed distortion due to degradations in the BS transmitter.

For NR EVM is defined similar to that of E-UTRA in that the measurement device contains a equalization algorithm and the EVM is assessed after equalization.  EVM is also measured over a root mean square average over 10 sub-frames.

Although EVM is expressed in terms of a percentage it is a ratio, that of the error vector and the reference power. Whilst the reference (or wanted) signal may be subjected to beam forming the error vector power has unknown correlation level between transmitter units and hence may be beam formed or not.

The EVM requirement is important only between the BS and its intended UE. Hence the EVM is only of importance in directions where the BS intends to communicate with the UE’s. It is therefore in a set of specific directions from the AAS BS which is of interest rather than the average or total EVM over the entire sphere. Based upon the method captured in 3GPP TR 37.843 [9], the EVM requirement is defined over a declared OTA coverage range.  The range of directions which the NR BS meets the EVM requirement is declared by the manufacturer as OTA coverage range.

Figure 9.6.6-1. Example of OTA coverage range and 5 conformance points

The EVM is required to be met across the OTA coverage range.  Physical layer specifics such as DM-RS patterns needed are discussed in more detail in 6.5.4 for conducted requirements, can also be applied to OTA for FR1.

### 9.6.7	OTA modulation quality for FR2

The EVM requirement for FR2 shall apply also to the range of directions which is described in section 9.6.6.

The intention of the following description is to summarize the parameters used to determine appropriate EVM levels.

A DM-RS pattern will be transmitted, and a standardized receiver will be used to mitigate some linear aspects of the EVM.  Additionally, common phase error compensation algorithm is also needed to be applied in addition to the simple equalization algorithm.  For this, an additional reference signal is needed, the so-called phase tracking reference signal (PT-RS).

To balance between reference signal overhead and throughput a balance of DM-RS and PT-RS along with achievable EVM levels were studied.

PTRS every 4th OFDM symbol

PDCCH     DM-RS       PT-RS     PDSCH

Figure 9.6.7-1. TYPE-1 Pattern front loaded DM-RS (1 DM-RS symbol)