3GPP TSG RAN Meeting #81					  	RP-182021

# Gold Coast, Australia, Sept 10th – 13th, 2018

Agenda item:	9.6

Source: 	Samsung

Title: 	NR UE features to support dynamic indication of 64 beams

Document for:	Discussion and Decision

Introduction

In RAN1#94 and RAN2#103 meeting, it was agreed to mandate UEs to support a minimum of 64 configured TCI states and to allow up to 128 configured TCI states for PDSCH [1-3]:

| Features | # | Feature group | Components | Prerequisite feature groups (listed in this sheet only) | Need for gNB to know whether the feature is supported by the UE | Consequences if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | RAN5 implication | Note | Responsible WG | RAN WG recommendation | TSG-RAN decision |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. MIMO | 2-4 | TCI states for PDSCH | 1. Support number of active TCI states per BWP per CC , including control and data 2. maximum number of configured TCI states per CC for PDSCH | 2-1 | Yes | Only one TCI state can be supported | Type 1 | N.A. | N.A. |  | Note: UE is required to track only the active TCI states |  | Component-1: Candidate value set: {1, 2, 4, 8 } Component-2: candidate value set: {4, 8, 16, 32, 64, [128]} UE is mandated to signal 64 |  |

This contribution provides Samsung’s view on UE features to support dynamic indication of 64 beams.

Discussion

For FR2, dynamic beam change of 64 beams is a crucial feature to prevent frequent RRC reconfiguration for beam change when the UE moves within the cell. Since 64 TCI states should be divided into two groups for beam indication of TRS and PDCCH/PDSCH/CSI-RS, only up to 32 beams can be utilized via L1/L2 signalling. To cover entire cell area without RRC reconfiguration, 128 TCI states should be made mandatory.

| Features | # | Feature group | Components | Prerequisite feature groups (listed in this sheet only) | Need for gNB to know whether the feature is supported by the UE | Consequences if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | RAN5 implication | Note | Responsible WG | RAN WG recommendation | TSG-RAN decision |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. MIMO | 2-4 | TCI states for PDSCH | 1. Support number of active TCI states per BWP per CC , including control and data 2. maximum number of configured TCI states per CC for PDSCH | 2-1 | Yes | Only one TCI state can be supported | Type 1 | N.A. | N.A. |  | Note: UE is required to track only the active TCI states |  | Component-1: Candidate value set: {1, 2, 4, 8 } Component-2: candidate value set: {4, 8, 16, 32, 64, [128]} UE is mandated to signal 64 | Candidate value set: {1, 2, 4, 8 } Component-2: candidate value set: {4, 8, 16, 32, 64, 128} UE is mandated to signal 128 |

In addition to configuration of 128 TCI states, 64 TRS resource sets per CC should be made mandatory to support 64 beams. In order to provide time/frequency tracking, each beam should be associated to a TRS resource set. Note that 64 TRS resource set should be supported with 128 TCI states, otherwise beam information cannot be configured due to lack of available TCI state configuration.

| Features | # | Feature group | Components | Prerequisite feature groups (listed in this sheet only) | Need for gNB to know whether the feature is supported by the UE | Consequences if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | RAN5 implication | Note | Responsible WG | RAN WG recommendation | TSG-RAN decision |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. MIMO | 2-51 | TRS (CSI-RS for tracking) | 1. Support TRS BW 2. TRS burst length (X), 3. Max # of TRS resource sets (per CC) UE is able to track simultaneously 4. Max # of TRS resource sets configured to UE per CC 5. Max # of TRS resource sets configured to UE across CCs | 2-50 | Yes | TRS is not supported | Type 1 | N.A. | N.A. |  |  |  | Component-1: candidate values set: {BWP, min(52,BWP), both} Component-2: candidate values {1,2} Component-3: Candidate value set: {1 to 8} Component-4: Candidate value set: {1 to 64} Component-5: Candidate value set: {1 to 128} | Component-1: candidate values set: {BWP, min(52,BWP), both} Component-2: candidate values {1,2} Component-3: Candidate value set: {1 to 8} Component-4: Candidate value set: {1 to 64} UE is mandated to signal 64 at least for FR2 Component-5: Candidate value set: {1 to 128} UE is mandated to signal 64*#CC at least for FR2 |

- References

- 3GPP RAN1#94 Chairman note

- R2-1813460, “Changes for MAC CEs to Support the Extended Maximum Number of TCI States”, Samsung

- R2-1813477, “CR on Extension of Maximum Number of TCI States (Non-backward compatible)”, Samsung.