3GPP TSG RAN Meeting #81	RP-182030

Gold Coast, Australia, 10 - 13 Septermber 2018

Agenda item:	9.3.2

Source:	Huawei

Title:	Consideration on IMT-2020 link budget

Document for:	Discussion

1	Introduction

In this contribution, the consideration and evaluation on link budget are provided.

According to Report ITU-R M.2411, the proponent of IMT-2020 should provide link budget template to ITU-R as one of the submission templates to complete Step 3 of IMT-2020 evaluation and submission process. The study of self evaluation of IMT-2020 therefore includes link budget as one of its objectives.

In this contribution, we discuss some important aspects of link budget evaluation.

2	Text proposal to TR37.910

2.1  Link budget template

The link budget template is provided in Section 5.2.3.3 in Report ITU-R M.2411. Complete table is provided for proponent to fill in the information. The downlink and uplink physical channel link budgets are to be provided.

In this contribution, the considerations on link budget are provided. The initial link budget template is provided in the attachment.

2.2  Discussions

2.2.1  Consideration on focusing scenarios and configuration

For 3GPP’s update submission in September 2018, it is considered to focus on a subset of evaluation configurations and technical features. This could facilitate the preparation of the template information. More results can be derived before final submission in June 2019. The proposed focused evaluation configuration is shown in Table 1.

Table 1 Focused evaluation configuration

|  | Indoor hotspot – eMBB | Dense Urban – eMBB | Rural – eMBB | Urban Macro – mMTC | Urban Macro – URLLC |
| --- | --- | --- | --- | --- | --- |
| System configuration | NR 4 GHz TDD | NR 4 GHz TDD | NR 700 MHz FDD LTE 700 MHz FDD | NB-IoT/eMTC 700 MHz FDD | NR 700 MHz FDD |
| Physical channel | DL: PDCCH, PDSCH UL: PUCCH, PUSCH | DL: PDCCH, PDSCH UL: PUCCH, PUSCH | DL: PDCCH, PDSCH UL: PUCCH, PUSCH | DL: PDCCH, PDSCH UL: PUCCH/PUSCH F2, PUSCH | DL: PDCCH -> PDSCH -> PUCCH UL: PUSCH -> PDCCH |
| Channel state | NLOS, LOS | NLOS, NLOS O-to-I, LOS | NLOS, NLOS O-to-I, LOS | NLOS, NLOS O-to-I, LOS | NLOS, NLOS O-to-I, LOS |
| Channel model | LOS: TDL-iv NLOS: TDL-i | NLOS: TDL-iii LOS: TDL-v | NLOS: TDL-iii LOS: TDL-v | NLOS: TDL-iii LOS: TDL-v | NLOS: TDL-iii LOS: TDL-v |
|  | Delay spread, angular spread is to be scaled to SLS model. |  |  |  |  |
| SCS | TDD: 30 kHz | TDD: 30 kHz | 700 MHz FDD: 15 kHz | 15kHz | 700 MHz FDD: 30 kHz |
| UE speed | LOS/NLOS: 3km/h | LOS: 30km/h NLOS: 30km/h NLOS O-to-I: 3km/h (for link level simulation, use TDL-iii channel model) | LOS: 120km/h NLOS: 120km/h NLOS O-to-I: 3km/h | LOS/NLOS/NLOS O-to-I: 3km/h | LOS/NLOS: 30km/h NLOS O-to-I: 3km/h |

2.2.2  Consideration on eMBB link budget evaluation

For eMBB evaluation, the target transmission bit rate, occupied bandwidth, and antenna configuration needs to be considered, which have impact on the link level simulation that derives the required SINR. In addition, the Feeder loss, Cable loss, Receiver interference density, etc. can follow the value used in IMT-Advanced evaluation.

The consideration is listed in Table 2.

Specifically, the target transmission bit rates of DL and UL data channel is set to three times than those in IMT-Advanced. This is under the consideration that the spectral efficiency of IMT-2020 is three times compared to IMT-Advanced.

In terms of transmitter array gain from Table 2, there are two options. Option 1 simplifies the link level simulation while providing flexible antenna configuration support. So in the evaluation, option 1 is used.

Another note is that receive array gain needs to be taken into account in the link budget evaluation. This is because in NR, multiple receive antennas are supported. Therefore receive array gain would contribute to link budget. This item is not included in ITU template, but it is proposed to be added in 3GPP evaluation.

Table 2 eMBB link budget evaluation

|  | DL control channel |  | DL data channel |  | UL control channel |  | UL data channel |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Transmission bit rate (bit/s) | NR PDCCH: DCI format 1-0; DCI size = 64 bit; QPSK, aggregation level = 16 CCE |  | ~3x IMT-A |  | NR PUCCH: Format 1 (long PUCCH with 14 OFDM symbols), 1 bit |  | ~3x IMT-A |  |
|  | 700 MHz | 4 GHz | 700 MHz | 4 GHz | 700 MHz | 4 GHz | 700 MHz | 4 GHz |
| Number of transmit antennas | 64 | 128 | 64 | 128 | 1 | 2 | 1 | 2 |
| Number of TXU | 8 | 32 | 8 | 32 | 1 | 2 | 1 | 2 |
| Number of receive antennas | 2 | 4 | 2 | 4 | 64 | 128 | 64 | 128 |
| Number of RXU | 2 | 4 | 2 | 4 | 8 | 32 | 8 | 32 |
| Transmitter array gain | 10*log10(N_tx_antenna/2) |  | Option 1: 10*log10(N_tx_antenna/2) Option 2: 10*log10(N_tx_antenna/N_TXU) |  | 0 |  | Same options as DL data channel |  |
| Receive array gain (This is not included in ITU template!) | 10*log10(N_rx_antenna/2) |  | Option 1: 10*log10(N_rx_antenna/2) Option 2: 10*log10(N_rx_antenna/N_RXU) |  | Option 1: 10*log10(N_rx_antenna/2) Option 2: 10*log10(N_rx_antenna/N_RXU) |  | Same options as DL data channel |  |
| Required SINR via link level simulation | Conduct link level simulation with a 2x2 system; link level simulation provides a 2x1 precoder for NR PDCCH. |  | For Option 1: Conduct link level simulation with a 2x2 system For Option 2: Conduct link level simulation with a N_TXUxN_RXU system |  | For Option 1: Conduct link level simulation with a 2x2 or 1x2 system For Option 2: Conduct link level simulation with a 2xN_RXU or 1xN_RXU system |  | Same options as DL data channel |  |
| Occupied channel bandwidth | FDD: 10 MHz TDD: 20 MHz |  | FDD: 10 MHz TDD: 20 MHz |  | 1 PRB |  | 4 PRB |  |
| Feeder loss, Cable loss, Receiver interference density | Follow 3GPP’s ITU submission for IMT-Advanced |  |  |  |  |  |  |  |
| Penetration margin | For O-to-I: outdoor-to-indoor penetration loss model is used. - Channel model A and B are different. High penetration is considered for Channel model B. For LOS/NLOS (O-to-O): in-car penetration loss model is used. |  |  |  |  |  |  |  |

2.2.3  Consideration on mMTC link budget evaluation

For mMTC link budget, similar items as eMBB are highlighted. For the target data rate, it is noted that 160bps transmission bit rate is required based on TR38.913. Therefore this value is used as starting point for UL evaluation. For DL, a higher bit rate can be considered due to the larger bandwidth. Other considerations are given in Table 3.

Table 3 mMTC link budget evalution

|  | DL control channel | DL data channel | UL control channel | UL data channel |
| --- | --- | --- | --- | --- |
| Transmission bit rate (bit/s) | NB-IoT: NPDCCH eMTC: PDCCH | 160bps | NPUSCH Format 2, PUCCH, PRACH | 160bps |
|  | 700 MHz | 700 MHz | 700 MHz | 700 MHz |
| Number of transmit antennas | 16 | 16 | 1 | 1 |
| Number of TXU | 2 | 2 | 1 | 1 |
| Number of receive antennas | 1 | 1 | 16 | 16 |
| Number of RXU | 1 | 1 | 2 | 2 |
| Transmitter array gain | 9 | 9 | 0 | 0 |
| Receive array gain (This is not included in ITU template!) | 0 | 0 | 9 | 9 |
| Required SINR via link level simulation | Conduct link level simulation with a 2x1 system; In link level simulation, use the MCS corresponding to the transmission format, and employ the number of repetitions that achieves the target BLER | Conduct link level simulation with a 2x1 system; In link level simulation, use the lowest MCS and employ the number of repetitions that achieves the target data rate with target BLER | Conduct link level simulation with a 1x2 system; In link level simulation, use the MCS corresponding to the transmission format, and employ the number of repetitions that achieves the target BLER | Conduct link level simulation with a 1x2 system In link level simulation, use the lowest MCS and employ the number of repetitions that achieves the target data rate with target BLER |
| Occupied channel bandwidth | NB-IoT: 1 RB eMTC: 6 RBs | NB-IoT: 1 RB eMTC: 6 RB | NPUSCH Format 2: 1 sub-carrier PUCCH: 1 RB | NB-IoT: 1 SCS (15kHz) eMTC: 1 RB |
| Feeder loss, Cable loss, Receiver interference density | Follow 3GPP’s ITU submission for IMT-Advanced |  |  |  |
| Penetration margin | For O-to-I: outdoor-to-indoor penetration loss model is used. - Channel model A and B are different. High penetration is considered for Channel model B. For LOS/NLOS (O-to-O): in-car penetration loss model is used. |  |  |  |

2.2.4  Consideration on URLLC link budget evaluation

Similarly, the items as shown in Table 4 are highlighted for URLLC. It is noted that the transmission bit rate requirements for DL and UL data channels are based on that 32 bytes are transmitted in 1ms. So 256kbps is derived. And the target packet error rate requirement comes from the requirement of 99.999% reliability.

The bandwidths of DL control channel, DL data channel and UL data channel are determined by the packet size and the selected MCS.

Table 4 URLLC link budget evaluation

|  | DL control channel | DL data channel | UL control channel | UL data channel |
| --- | --- | --- | --- | --- |
| Transmission bit rate (bit/s) | NR PDCCH: DCI format 1-0; DCI size = 64 bit; QPSK, aggregation level = 16 CCE | 256kbps | NR PUCCH: Format 0 (short PUCCH with 2 OFDM symbols), 1 bit | 256kbps |
| Target packet error rate | 0.001% | 0.001% (NOTE: This target can be achieved by several repetitions) | 0.001% | 0.001% (NOTE: This target can be achieved by several repetitions) |
|  | 700 MHz | 700 MHz | 700 MHz | 700 MHz |
| Number of transmit antennas | 64 | 64 | 1 | 1 |
| Number of TXU | 8 | 8 | 1 | 1 |
| Number of receive antennas | 2 | 2 | 64 | 64 |
| Number of RXU | 2 | 2 | 8 | 8 |
| Transmitter array gain | Follow option 1 for eMBB | Follow option 1 for eMBB | 0 | 0 |
| Receive array gain (This is not included in ITU template!) | 0 | 0 | Follow option 1 for eMBB | Follow option 1 for eMBB |
| Required SINR via link level simulation | Conduct link level simulation with a 2x2 system; In link level simulation, select the appropriate MCS that achieves target BLER for control. | Conduct link level simulation with a 2x2 system; In link level simulation, select the number of repetition within 1ms, and the appropriate MCS that achieves target data rate = 32byte / 1ms and the target BLER. | Conduct link level simulation with a 1x2 system; In link level simulation, select the appropriate MCS that achieves target BLER for control. | Conduct link level simulation with a 1x2 system; In link level simulation, select the number of repetition within 1ms, and the appropriate MCS that achieves target data rate = 32byte / 1ms and the target BLER. |
| Occupied channel bandwidth | TBD | TBD | 1 RB | TBD |
| Feeder loss, Cable loss, Receiver interference density | Follow 3GPP’s ITU submission for IMT-Advanced |  |  |  |
| Penetration margin | For O-to-I: outdoor-to-indoor penetration loss model is used. - Channel model A and B are different. High penetration is considered for Channel model B. For LOS/NLOS (O-to-O): in-car penetration loss model is used. |  |  |  |

2.2.5  Consideration of beam management

As mentioned in [2], beam management is introduced in NR design. It is a mechanism for massive MIMO system that achieves wide area coverage by employing and sweeping the directional narrow analog beams.

In this case, the impact of beam management on transmit array gain and receive array gain needs to be considered. This mechanism is usually adopted for higher frequency band. Considering that the proposed focusing scenario is primarily on below 6 GHz, it is therefore not urgent to develop this model towards 3GPP’s updated submission in September.

3	Conclusion

In this contribution, the consideration and evaluation results on link budget are provided.

Proposal: It is proposed to take into account the above considerations in the link budget evaluation, and use the attached link budget templates as starting point.

Reference

Report ITU-R M.2411, “Requirements, evaluation criteria and submission templates for the development of IMT-2020”, Nov. 2017.

R1-1805938, “Consideration on self evaluation of IMT-2020 for link budget”, Huawei, HiSilicon, May 2018.

Annex 1  Margin Computation

The calculation methods of shadow fading margin and penetration margin are provided in this Annex.

Method 1:

Method 1 is used to calculate shadow fading margin (Indoor Hotspot/mMTC LOS/NLOS) and shadow fading margin with car penetration (Dense Urban/Rural/URLLC LOS/NLOS).

Assuming the coverage area of a base station is a circle, and the radius is R. The signal strength at radius R follows the Gaussian distribution with mean value x(R) and deviation σ.

px=1σ2πexp⁡-(x-x(R))22σ2

So, if a receiver has threshold level x0 located in the circle, the successful receive probability (meaning that the signal strength at radius R is beyond the threshold level x0) can be calculated as below.

Px0R=x0+∞pxdx=121+erf⁡(xR-x0σ2)

Px0R is the edge coverage reliability. Margin equals to xR-x0.

Then, in the whole coverage area of a base station, the percentage of useful service area, where the signal strength at radius r is beyond the threshold level x0, is area coverage reliability(ACR). ACR can be calculated as below.

Fu=1πR2Px0rdA=2R20RPx0r∙rdr

Where

x(r)=x(R)-10nlgrR

Px0r=121+erf⁡(xR-x0-10nlgrRσ2)

Mark that a=(x(R)-x0)(σ2), b=(10nlge)(σ2), then we have

Fu=2R20R121+erfa-b×lnrR∙rdr

=12+1ba∞erft×exp2a-tbdt

=121+erfa+exp2ab+1b21-erfab+1b

Margin equals to xR-x0. Fu is affected by Margin xR-x0, deviation σ and pathloss factor n.

As for shadow fading, σ=σSF. While for car penetration loss, PL=PLb+Nμ,σp2=xr+N0,σSF2+N(μ,σp2), and the σ2=σSF2+σp2.

Method 2:

Method 2 is used to calculate shadow fading margin with O2I penetration with model A<6GHz (Dense Urban/Rural/ mMTC /URLLC NLOS O2I).

In M.2412, the definition of O2I building penetration is as below.

PLr=PLbr+PLtw+PLin+N0,σp2

Note that when using model A<6GHz, σp2=0

PLr=xr+N0,σSF2+PLtw+0.5U0,M

=xr+PLtw+M4+U-M4,M4+N0,σSF2

Where M=10 or 25, depended on Rural or others. So, we can easily observe that PLin has both mean value M4 and uniform distribution U-M4,M4. Obviously, the uniform distribution U-M4,M4 has influence on margin calculation.

In general, mark σSF2 as σ2 for writing convenience.

PLr=xr+PLtw+M4+U-M4,M4+N0,σ2

=PL1r+U-M4,M4+N0,σ2

Consider the circular area with radius R. At radius R, the signal strength has mean value PR= Pt-PL1R and distrubution as below.

pz=p1t×p2z-PR-tdt=-M4M42M×12πσe-z-PR-t22σ2dt

so the edge coverage reliability

Px0R=x0∞pzdz=x0∞-M4M42M×12πσe-z-PR-t22σ2dtdz

=-M4M4x0∞2M×12πσe-z-PR-t22σ2dzdt

=-M4M41M1+erft+PR-x0σ2dt

Similarly, for radius r inside the coverage area, Pr=PR-10nlgrR, so

Px0r=1M-M4M41+erft+PR-10×n×lgrR-x0σ2dt

Mark a=(t+P(R)-x0)(σ2) and b=(10nlge)(σ2)

Px0r=1M-M4M41+erfa-b×lnrRdt

So the area coverage reliability is

Fu=2R20Rr×Px0rdr=2R20Rr×1M-M4M41+erfa-b×lnrRdtdr

=2σMP(R)-x0-M42σP(R)-x0+M42σ1+2R20Rr×erfa-b×lnrRdrda

=2σMP(R)-x0-M42σP(R)-x0+M42σ1+erfa+exp2ab+1b21-erfab+1bda

=2σMa+e-a2π+a+b2erfa+b2exp2ab+1b21-erfab+1bP(R)-x0+M42σP(R)-x0-M42σ

Margin equals to P(R)-x0.

Method 3:

Method 3 is used to calculate shadow fading margin with O2I penetration with model A>6GHz & model B (Dense Urban/Rural/ mMTC /URLLC NLOS O2I).

In M.2412, the definition of O2I building penetration is as below.

PLr=PLbr+PLtw+PLin+N0,σp2

Note that when using model A>6GHz & model B, d2D-in=min⁡(X,Y) where X,Y are independent and both follow the uniform distribution U(0,M). So, the probability density function of d2D-in is

pd2D-in=2M2M-x  , if 0≤x≤M.  Ed2D-in= M3

Then,

PLR=PLbR+PLtw+PLin+N0,σp2

=xR+PLtw+12d2D-in+N0,σSF2+N0,σp2

=xR+PLtw+M6+p1+N0,σ2

Where

p1x=8M2M3-x  , if-M6≤x≤M3.  Ep1=0

σ2=σSF2+σp2

Mark  PL2R=xR+PLtw+M6, so at radius R, the signal strength has mean value QR=Pt-PL2R and distributuion

qz=p1x×p2z-QR-xdx

=-M6M3 8M2M3-x×12πσe-z-QR-x22σ2dx

Edge coverage reliability

Px0R=x0∞pzdz=x0∞-M6M3 8M2M3-x×12πσe-z-QR-x22σ2dxdz

=-M6M3 8M2M3-x×x0∞12πσe-z-QR-x22σ2dzdx

=-M6M3 8M2M3-x×121+erfx+QR-x0σ2dx

Then, with radius r inside the coverage area,

Px0r=-M6M3 8M2M3-x×121+erfx+QR-x0-10×n×lgrRσ2dx

So, the area coverage reliability is

Fu=2R20Rr×Px0rdr

=2R20R-M6M3 8M2M3-x×121+erfx+QR-x0-10×n×lgrRσ2dxrdr

=01-M6M3 8M2M3-x1+erfx+QR-x0-10×n×lgtσ2dxtdt

This integral can be calculated using Numeral Calculations.

Fu=titi∆t×xi8M2M3-xi1+erfxi+MARGIN-10×n×lgtiσ2∆x

Take Rural environment as an example. Set σ=9.13, n=3.86, M=10, ∆t=0.0005, ∆x=0.0005, then we can get Fu(MARGIN) shown as below.

Annex 2  Simulation assumption of LLS part for link budget

The simulation assumption of link level part for link budget evaluation is listed in Table A2.

Table A2-1. Simulation assumptions of LLS for eMBB/URLLC PDSCH

|  | Indoor Hotspot – eMBB | Dense urban - eMBB | Rural – eMBB | URLLC |
| --- | --- | --- | --- | --- |
| Carrier frequency for evaluation | 4 GHz | 4 GHz | 700 MHz | 700 MHz |
| RIT | NR | NR | NR | NR |
| Waveform | CP-OFDM | CP-OFDM | CP-OFDM | CP-OFDM |
| Duplexing | TDD | TDD | FDD | FDD |
| TDD frame structure | DDDSU | DDDSU | - | - |
| Link-level Channel model | NLOS: TDL-i | NLOS: TDL-iii | NLOS: TDL-iii | NLOS: TDL-iii |
| Scaled delay spread | NLOS: 39 ms | NLOS: 363 ms | NLOS: 37 ms | NLOS: 363 ms |
| UE speed | 3 km/h | O2I: 3 km/h Others: 30 km/h | O2I: 3 km/h Others: 120 km/h | 3 km/h |
| Subcarrier spacing | For TDD: 30 kHz | For TDD: 30 kHz | For FDD: 15 kHz | 15 kHz |
| Transmission mode | SU-MIMO | SU-MIMO | SU-MIMO | SU-MIMO |
| Transmission rank | Rank 1 | Rank 1 | Rank 1 | Rank 1 |
| TRxP receiver type | MMSE-IRC | MMSE-IRC | MMSE-IRC | MMSE-IRC |
| Channel estimation | LMMSE | LMMSE | LMMSE | LMMSE |
| Number of subcarriers per PRB | 12 | 12 | 12 | 12 |
| Modulation | NLOS: Code rate = 0.42,16QAM | NLOS: Code rate = 0.44, QPSK NLOS O2I: Code rate = 0.37, QPSK | NLOS: Code rate = 0.51, QPSK NLOS O2I: Code rate = 0.51, QPSK |  |
| DMRS configuration | 1 symbol DMRS (front loaded) with configuration type 2, no FDM with data and full power utilization in DMRS symbols | - For 3km/h: 1 symbol DMRS (front loaded and one additional) with configuration type 2, no FDM with data and full power utilization in DMRS symbols - For 30km/h: 2 symbol DMRS (front loaded and one additional) with configuration type 2, no FDM with data and full power utilization in DMRS symbols | - For 3km/h: 1 symbol DMRS (front loaded and one additional) with configuration type 2, no FDM with data and full power utilization in DMRS symbols - For 120km/h: 2 symbol DMRS (front loaded and one additional) with configuration type 2, no FDM with data and full power utilization in DMRS symbols | Type 1 2 symbol DMRS for 14 os 1 symbol for 7os and 4os |

Table A2-2. Simulation assumptions of LLS for eMBB/URLLC PDCCH

|  | Indoor Hotspot – eMBB | Dense urban - eMBB | Rural – eMBB | URLLC |
| --- | --- | --- | --- | --- |
| Carrier frequency for evaluation | 4 GHz | 4 GHz | 700 MHz | 700 MHz |
| Waveform | CP-OFDM | CP-OFDM | CP-OFDM | CP-OFDM |
| Duplexing | TDD | TDD | FDD | FDD |
| TDD frame structure | DDDSU | DDDSU | - | - |
| Link-level Channel model | NLOS: TDL-i LOS: TDL-iv | NLOS: TDL-iii LOS: TDL-v | NLOS: TDL-iii LOS: TDL-v | NLOS: TDL-iii LOS: TDL-v |
| Scaled delay spread | NLOS: 39 ms LOS: 20 ms | NLOS: 363 ms LOS: 93 ms | NLOS: 37 ms LOS: 32 ms | NLOS: 363 ms LOS: 93 ms |
| UE speed | 3 km/h | O2I: 3 km/h Others: 30 km/h | O2I: 3 km/h Others: 120 km/h | 3 km/h |
| Subcarrier spacing | For TDD: 30 kHz | For TDD: 30 kHz | For FDD: 15 kHz | 15kHz |
| PDCCH Transmition Scheme | DCI format 1-0. 64bit payload includes CRC. Aggregation level = 16 | DCI format 1-0. 64bit payload includes CRC. Aggregation level = 16 | DCI format 1-0. 64bit payload includes CRC. Aggregation level = 16 | DCI format 1-0. 64bit payload includes CRC. Aggregation level = 16 |
| Modulation | QPSK | QPSK | QPSK | QPSK |

Table A2-3. Simulation assumptions of LLS for eMBB/URLLC PUSCH

|  | Indoor Hotspot – eMBB | Dense urban - eMBB | Rural – eMBB | URLLC |
| --- | --- | --- | --- | --- |
| Carrier frequency for evaluation | 4 GHz | 4 GHz | 700 MHz | 700MHz |
| Duplexing | TDD | TDD | FDD | FDD |
| TDD frame structure | DDDSU | DDDSU | - | - |
| Link-level Channel model | NLOS: TDL-i LOS: TDL-iv | NLOS: TDL-iii LOS: TDL-v | NLOS: TDL-iii LOS: TDL-v | NLOS: TDL-iii LOS: TDL-v |
| Scaled delay spread | NLOS: 39 ms LOS: 20 ms | NLOS: 363 ms LOS: 93 ms | NLOS: 37 ms LOS: 32 ms | NLOS: 363 ms LOS: 93 ms |
| UE speed | 3 km/h | O2I: 3 km/h Others: 30 km/h | O2I: 3 km/h Others: 120 km/h | 3 km/h |
| Subcarrier spacing | For TDD: 30 kHz | For TDD: 30 kHz | For FDD: 15 kHz | 15 kHz |
| Symbols number per slot | 14 | 14 | 14 | 14 |
| TRxP receiver type | MMSE-IRC | MMSE-IRC | MMSE-IRC |  |
| Channel estimation | LMMSE | LMMSE | LMMSE | LMMSE |
| Number of subcarriers per PRB | 12 | 12 | 12 | 12 |
| Modulation | NLOS: Code rate = 0.5, 64QAM | NLOS: Code rate = 0.66, QPSK NLOS O2I: Code rate = 0.59, QPSK | NLOS: Code rate = 0.80, QPSK NLOS O2I: Code rate = 0.78, QPSK |  |
| DMRS configuration | 1 symbol DMRS (front loaded and one additional) with configuration type 2, no FDM with data and full power utilization in DMRS symbols | - For 3km/h: 1 symbol DMRS (front loaded and one additional) with configuration type 2, no FDM with data and full power utilization in DMRS symbols - For 30km/h: 2 symbol DMRS (front loaded and one additional) with configuration type 2, no FDM with data and full power utilization in DMRS symbols | - For 3km/h: 1 symbol DMRS (front loaded and one additional) with configuration type 2, no FDM with data and full power utilization in DMRS symbols - For 120km/h: 2 symbol DMRS (front loaded and one additional) with configuration type 2, no FDM with data and full power utilization in DMRS symbols | Type 1, 2 symbol DMRS |

Table A2-4. Simulation assumptions of LLS for eMBB/URLLC PUCCH

|  | Indoor Hotspot – eMBB | Dense urban - eMBB | Rural – eMBB | URLLC |
| --- | --- | --- | --- | --- |
| Carrier frequency for evaluation | 4 GHz | 4 GHz | 700 MHz | 700 MHz |
| Simulation bandwidth | 180 kHz | 180 kHz | 180 kHz | 180kHz |
| Number of users in simulation | 1 | 1 | 1 | 1 |
| Link-level Channel model | NLOS: TDL-i LOS: TDL-iv | NLOS: TDL-iii LOS: TDL-v | NLOS: TDL-iii LOS: TDL-v | NLOS: TDL-iii LOS: TDL-v |
| Scaled delay spread | NLOS: 39 ms LOS: 20 ms | NLOS: 363 ms LOS: 93 ms | NLOS: 37 ms LOS: 32 ms | NLOS: 363 ms LOS: 93 ms |
| UE speed | 3 km/h | O2I: 3 km/h Others: 30 km/h | O2I: 3 km/h Others: 120 km/h | 3 km/h |
| TRxP receiver type | MMSE-IRC | MMSE-IRC | MMSE-IRC | MMSE-IRC |
| Channel estimation | LMMSE | LMMSE | LMMSE | LMMSE |
| PUCCH Transmition Scheme | format 1 | format 1 | format 1 | Format 1 |
| Modulation | QPSK | QPSK | QPSK | QPSK |

Table A2-5. Simulation assumptions of LLS for mMTC

| Parameters | NPUSCH format 1 | NPUSCH format 2 | NPDSCH | NPDCCH |
| --- | --- | --- | --- | --- |
| Carrier frequency | 700 MHz | 700 MHz | 700 MHz | 700 MHz |
| Evaluated service profiles | Full buffer best effort | Full buffer best effort | Full buffer best effort | Full buffer best effort |
| Simulation bandwidth | 15kHz | 15kHz | 180kHz | 180kHz |
| Link-level Channel model | TDL-iii | TDL-iii | TDL-iii | TDL-iii |
| UE speed | 3 km/h | 3 km/h | 3 km/h | 3 km/h |
| Subcarrier spacing | 15 kHz | 15 kHz | 15 kHz | 15 kHz |
| Symbols number per slot | 14 | 14 | 14 | 14 |
| TBS | 256 | 1 | 256 | 23 |
| Modulation order | BPSK | BPSK | QPSK | QPSK |
| Number of Resource units | 10 RU | 1 RU | 10TTI | 1 TTI |
| Number of repetition | 16 | 64 | 128 | 512 |
| TRxP receiver type | MMSE-IRC | MMSE-IRC | MMSE-IRC | MMSE-IRC |
| Channel estimation | LMMSE | LMMSE | LMMSE | LMMSE |
| Channel coding | Turbo code | Block code | Turbo code | Convolutional code |
| DMRS/NRS configuration | 1 RE for DMRS per slot | 3 REs for DMRS per slot | 16 REs for NRS per RB | 16 REs for NRS per RB |