3GPP TSG-RAN Meeting #81	RP-182085

Gold Coast, Australia, Sept 10th – 13th, 2018

Source:	Ericsson, Sierra Wireless

Title:	IMT-2020 self evaluation: mMTC link budget

Agenda Item:	9.3.2

Document for:	Discussion and Decision

# Introduction

3GPP are as part of the Study Item on Self Evaluation towards IMT-2020 Submission [1] preparing a link budget for the use cases eMBB, mMTC and URLLC. In this contribution we present our input to the mMTC use case in TR 37.919 [4] based on link level simulations performed for NB-IoT and for LTE Bandwidth reduced Low complexity (BL) UEs operating in Coverage Enhanced (CE) modes A and B, hereafter referred to as LTE-M.

This is a revision of RP-182003, now including simulation results also for an eNB configured with 2 RX branches, in addition to the previous 4 RX results.

# Link budget template

In this chapter we present our input to the IMT-2020 mMTC link budget template. Sections 2.1, 2.2, 2.3 and 2.4 presents a discussion around the SNR, Shadow fading margin, HARQ gain and penetration margin parameters. Section 2.5 presents the complete mMTC link budget.

## SNR and Data rate performance

The IMT-2020 link budget requires the proponents of a technology to present ‘transmission bit rates’ and ‘packet error ratios’ for the data and control channels of the presented technology. Here we show NB-IoT and LTE-M (N)/(M)PDCCH, (N)PDSCH, (N)PUSCH (F1/2) and PUCCH performance based on the assumptions summarized in Table 1. These assumptions are aligned with the IMT-2020 evaluation methodology [3].

Table 1: Urban Macro-mMTC test environment definition.

| Parameters | Config. A |
| --- | --- |
| Technologies | LTE-M FDD and NB-IoT FDD |
| Physical channels | NB-IoT: NPDCCH, NPDSCH, NPUSCH F1, NPUSCH F2 LTE-M: MPDCCH, PDSCH, PUSCH, PUCCH |
| Carrier frequency for evaluation | 700 MHz |
| Device mobility | 3 km/h |
| Channel models | TDL-iii (NLOS), TDL-v (LOS) |
| eNB RX/TX | 4/2 and 2/2 |
| Device RX/TX | 1/1 |
| Transmission mode | TM2 at eNB; TM1 at UE |

The simulator was configured to study the coverage achievable at 10% BLER and a physical layer data rate of at least 160 bps on the UL and DL data channels. The control channels were evaluated at a similar coverage as the data channels, but with a 1% BLER target. Table 3 and Table 2 presents the simulated results for NB-IoT and calculates the thereto associated coupling losses when assuming eNB power of 35 dBm/PRB and a device power class of 23 dBm. The data rate calculations take both the selected TBS and the CRC into account.

Table 2: NB-IoT link level results for TDL-iii, 2 Hz.

| Physical channel | NPDSCH | NPDCCH | NPUSCH F1 | NPUSCH F1 | NPUSCH F2 | NPUSCH F2 |
| --- | --- | --- | --- | --- | --- | --- |
| Power | 35 | 35 | 23 | 23 | 23 | 23 |
| Bandwidth [Hz] | 180000 | 180000 | 15000 | 15000 | 15000 | 15000 |
| TBS [Bits] | 256 | 23 | 256 | 256 | 1 | 1 |
| #RU/#SF/Aggregation level | 10 | 2 | 10 | 10 | 1 | 1 |
| #Rep | 128 | 512 | 16 | 16 | 64 | 32 |
| #TX/#RX | 2TX/1RX | 2TX/1RX | 1TX/2RX | 1TX/4RX | 1TX/2RX | 1TX/4RX |
| BLER | 10% | 1% | 10% | 10% | 1% | 1% |
| Phy data rate [bps] | 197 | 75 | 197 | 197 | 8 | 15 |
| SNR [dB] | -17.7 | -16.7 | -14 | -16.6 | -14.3 | -16 |
| NF | 7 | 7 | 5 | 5 | 5 | 5 |
| Resulting coupling loss | 167.1 | 166.1 | 164.2 | 166.8 | 164.5 | 166.2 |

Table 3: NB-IoT link level results for TDL-v, 2 Hz.

| Physical channel | NPDSCH | NPDCCH | NPUSCH F1 | NPUSCH F1 | NPUSCH F2 | NPUSCH F2 |
| --- | --- | --- | --- | --- | --- | --- |
| Power | 35 | 35 | 23 | 23 | 23 | 23 |
| Bandwidth [Hz] | 180000 | 180000 | 15000 | 15000 | 15000 | 15000 |
| TBS [Bits] | 256 | 23 | 512 | 512 | 1 | 1 |
| #RU/#SF/Aggregation level | 10 | 2 | 10 | 10 | 1 | 1 |
| #Rep | 64 | 256 | 16 | 16 | 32 | 32 |
| #TX/#RX | 2TX/1RX | 2TX/1RX | 1TX/2RX | 1TX/4RX | 1TX/2RX | 1TX/4RX |
| BLER | 10% | 1% | 10% | 10% | 1% | 1% |
| Phy. data rate [bps] | 394 | 151 | 197 | 197 | 15 | 15 |
| SNR [dB] | -16.9 | -17.2 | -15.8 | -17.6 | -15.4 | -17.9 |
| NF | 7 | 7 | 5 | 5 | 5 | 5 |
| Resulting coupling loss | 166.3 | 166.6 | 166 | 167.8 | 165.6 | 168.1 |

Table 4 and Table 5 presents LTE-M results for TDL-iii and -v.

Table 4: LTE-M link level results for TDL-iii, 2 Hz.

| Physical channel | PDSCH | MPDCCH | PUSCH | PUSCH | PUCCH | PUCCH |
| --- | --- | --- | --- | --- | --- | --- |
| Power | 36.8 | 36.8 | 23 | 23 | 23 | 23 |
| Bandwidth [Hz] | 1080000 | 1080000 | 180000 | 180000 | 180000 | 180000 |
| TBS [Bits] | 208 | 18 | 256 | 256 | 1 | 1 |
| #RU/#SF/Aggregation level | 1 | 24 | 1 | 1 | 1 | 1 |
| #Rep | 1024 | 256 | 1536 | 768 | 128 | 64 |
| #TX/#RX | 2TX/1RX | 2TX/1RX | 1TX/2RX | 1TX/4RX | 1TX/2RX | 1TX/4RX |
| BLER | 10% | 1% | 5.3% | 4% | 1% | 1% |
| Phy data rate [bps] | 204 | 131 | 173 | 350 | 8 | 15 |
| SNR [dB] | -21.9 | -20.8 | -24.6 | -24.6 | -26.3 | -26 |
| NF | 7 | 7 | 5 | 5 | 5 | 5 |
| Resulting coupling loss | 165.3 | 164.2 | 164 | 164 | 165.7 | 165.4 |

Table 5: LTE-M link level results for TDL-v, 2 Hz.

| Physical channel | PDSCH | MPDCCH | PUSCH | PUSCH | PUCCH | PUCCH |
| --- | --- | --- | --- | --- | --- | --- |
| Power | 36.8 | 36.8 | 23 | 23 | 23 | 23 |
| Bandwidth [Hz] | 1080000 | 1080000 | 180000 | 180000 | 180000 | 180000 |
| TBS [Bits] | 208 | 23 | 256 | 256 | 1 | 1 |
| #RU/#SF/Aggregation level | 1 | 24 | 1 | 1 | 1 | 1 |
| #Rep | 1024 | 256 | 1024 | 512 | 32 | 16 |
| #TX/#RX | 2TX/1RX | 2TX/1RX | 1TX/2RX | 1TX/4RX | 1TX/2RX | 1TX/4RX |
| BLER | 10% | 1% | 7.7% | 8.7% | 1% | 1% |
| Phy data rate [bps] | 204 | 131 | 252 | 499 | 31 | 62 |
| SNR [dB] | -24.1 | -21 | -24.6 | -24.6 | -25.6 | -25.6 |
| NF | 7 | 7 | 5 | 5 | 5 | 5 |
| Resulting coupling loss | 167.5 | 164.4 | 164 | 164 | 165 | 165 |

## Link budget template: Cell area reliability and Shadow fading margin

Since the Connection Density evaluations have been performed for a 99% Grade of service requirement it is in the link budget motivated to assume a ‘Cell area reliability’ of 99%. Based on this requirement a ‘Shadow fading margin’ can be calculated, using an assumption of a lognormal distributed shadowing with standard deviation σ and a pie shaped cell with an area πR2/3.

Figure 1: Cell shape assumed in the derivation of shadow fading margin.

First assume that a maximum path loss PLMAX is supported by the studied technology. The outage likelihood due to shadow fading (SF) at a certain distance r from the base station can then be calculated based on the Normal distribution Cumulative distribution function:

ProbPLr+SF>PLMAX=12(1+erf⁡(PLr-PLMAX2σ))

Here erf corresponds to the Error function, and the path loss PL(r) is defined by the studied channel model.

The total outage can then be calculated by weighting ProbPLr+SF>PLMAX with the fraction of the cell area associated with a circular arc of length 2πr/3 and thickness dr, and integrating over the full cell radius rϵ[0,R]:

Outage= r=0r=R12(1+erfPLr-PLMAX2σ)∙ 2πrdrπR2

The cell radius R is given by the channel model and a nominal path loss that equals PLMAX minus the shadow fading margin. Figure 2 illustrates the relation between outage probability and the shadow fading margin for LTE-M and UMA A, LOS conditions.

Figure 2: Outage probability versus shadow fading margin.

The shadow fading margin that results in 1% outage is used in the attached link budgets to provide 99% Cell area reliability.

## Link budget template: HARQ gain

The link budget template requires a ‘HARQ gain’ to be established. Figure 3 and Figure 4 therefore presents for TDL-iii and TDL-v (assuming an eNB configured with 4 RX branches):

- The BLER performance of the NPUSCH F1 reference case (1 sub-carrier, 10 RUs, 16 repetition) presented in Table 2 and Table 3.

- The initial and residual BLER performance of an 8-repetition NPUSCH F1 transmission using a single HARQ retransmission.

It can be observed that for TDL-iii there is a negligible 0.2 dB coverage gain by using 8 repetitions, and 1 HARQ retransmission compared to the 16 repetitions, no HARQ reference case. The gain can be attributed to the added time diversity due to the scheduling gap inserted between the first and second HARQ transmission.

For TDL-v there is no time diversity coverage gain. This is due to the limited diversity offered by this LOS type of channel.

Similar conclusions with regards to the HARQ cell edge coverage gain can be made for the NB-IoT DL (see link results presented in the Annex), as well as for LTE-M UL and DL. The HARQ gain for the data transmission is therefore reported as 0 dB in Table 7 and Table 8.

For both NB-IoT and LTE-M there is however an important data rate gain thanks to HARQ retransmissions that deserves to be highlighted. Table 6 presents the average (N)PUSCH PHY cell edge data rate with and without HARQ. These improvements in data rates are for simplicity not considered in the attached link budgets. The data rate calculations includes the CRC overhead.

Note that the simulations for LTE-M focused on achieving maximum data rate for the given coverage of 164 dB. For NB-IoT the focus was to maximize coverage given a data rate requirement of 160 bps. At the same coupling loss level, the performance is expected to be similar for the two technologies.

Table 6: Average (N)PUSCH cell edge PHY data rates for NB-IoT and LTE-M.

| Technology | Channel | Coupling loss | Ref: No HARQ | HARQ |
| --- | --- | --- | --- | --- |
| NB-IoT | TDL-iii | 166.8 dB | 197 bps | 300 bps |
| NB-IoT | TDL-v | 167.8 dB | 197 bps | 219 bps |
| LTE-M | TDL-iii | 164 dB | 350 bps | 766 bps |
| LTE-M | TDL-v | 164 dB | 499 bps | 766 bps |

Figure 3: Assessment of NB-IoT NPUSCH F1 cell edge HARQ performance for TDL-iii.

Figure 4: Assessment of NB-IoT NPUSCH F1 cell edge HARQ performance for TDL-v.

## Link budget template: Penetration margin

For the ‘Penetration margin’ parameter we are making a close to worst case assumption. This is motivated since the link budget is exploring the maximum range supported by mMTC.

For UMA A the penetration loss (PL) is calculated as [5]:

PLUMAA = PLtw + PLin

With the PLtw equal to the external wall loss, and PLin equal to the indoor loss:

PLtw = 20 dB

PLin = 0.5d2D-in, with d2D-in ~ U(0, 25m)

With d2D-in = 25 m we get a pessimistic assessment of the penetration loss:

PLUMAA = PLtw + PLin = 20 + 12.5 = 32.5 dB

For UMA B the high loss model [5] gives a penetration loss (PL) calculated as:

PLUMAB = PLtw + PLin + N(0, σP2)

With:

PLtw =5-10log100.7⋅10-LIRRglass10+0.3⋅10-Lconcrete10, for LIRRglass = 23+0.3f, Lconcrete = 5+4f

PLin = 0.5d2D-in, with d2D-in = min(A,B), for A and B~ U(0, 25m)

σP = 6.5 dB

With a frequency f = 0.7 GHz,  d2D-in = 25 m and under the assumption that the N(0, σP2) delivers a value of 2 times σP (to cover ~95% of the N(0, σP2) distribution), i.e. 13 dB we get a pessimistic assessment of the penetration loss that equals:

PLUMAB = PLtw + PLin + N(0, σP2) = 17.74 + 12.5 + 13 = 43.2 dB

These two values for UMA A and UMA B are used as ‘Penetration margin’ in the attached link budget table when calculating the indoor scenario link budget.

## Link budget template: Proposal

The attached .xls document contains the link budget for the test environments LOS, NLOS and NLOS for indoor users. Results are presented for TDL-iii and TDL-v link level models, and for the eNB configured with 2 or 4 RX antennas.

Many of the parameter values declared in the link budget tables are taken from the ITU-R report M.2411-0 [3]. The tables are also complemented with a set of notes explaining parts of the data. Worth to mention is that item “11a Receiver array gain” is not part of the IMT-2020 template and has been added since its appears to be missing in the ITU-R template.

# Conclusions

In this contribution, we have presented input to the IMT-2020 mMTC link budget based on NB-IoT and LTE-M link level performance. It is proposed that the link budgets for an eNB configured with 4 RX antennas are added to TR 37.919 Study on Self Evaluation towards IMT-2020 Submission and used as basis for the LS to ITU-R on IMT-2020.

Proposal: The herein presented mMTC link budget analysis for an eNB with 4 RX antennas is added to TR 37.919 Study on Self Evaluation towards IMT-2020 Submission [4] and used in the LS to ITU-R on IMT-2020.

# References

RP-171451, “New Study Item on Self Evaluation towards IMT-2020 Submission”, RAN#75, Ericsson et al

RP-181889, “IMT-2020 self-evaluation: mMTC link budget”, RAN#81, Ericsson

Report ITU-R M.2411-0 “Requirements, evaluation criteria and submission templated for the development of IMT-2020”, ITU-R

RP-180523, TR 37.919 v0.1.0 Study on Self Evaluation towards IMT-2020 Submission (Rel-15), RAN#79

Report ITU-R M.2412-0 “Guidelines for evaluation of radio interface technologies for IMT-2020”, ITU-R

# Annex: NPDSCH HARQ performance

Figure 5: Assessment of NB-IoT NPDSCH cell edge HARQ performance for TDL-iii.

Figure 6: Assessment of NB-IoT NPDSCH cell edge HARQ performance for TDL-v.