3GPP TSG RAN meeting #82						RP-182335

Sorrento, Italy, December 10-13, 2018

Agenda item:	10.4.3

Source: 	Qualcomm Incorporated

Title: 	FGIs for TM9

# Introduction

In RAN#81, it was agreed to set the TM9 FGI bit to 1 for UEs of certain categories and release:

RP-182137	Way forward on setting TM9 FGI bits to 1

Conclusion: endorsed with the clarifications: this does not apply to REL-14 and earlier, categories will be decided in Dec.18

As a result of this decision, the following actions were taken in the working groups:

- In RAN5, the LS RP-182238 was agreed indicating that, from RAN5 perspective, the test cases are already defined and RAN5 could implement the necessary changes to mandate the FGI setting by RAN#83 (Q1’19)

- In RAN2, a Rel-15 CR R2-1818598 was technically endorsed, where the applicable category(ies) is TBD.

In this document, we present our views on the process of setting TM9 FGI bits to 1.

# Category and Release

In RAN#81 it was agreed that the mandatory setting does not apply to Rel-14 and earlier. Given the interest from several companies to set these FGI bit under discussion starting as early as Release 15, we propose:

Proposal 1: FGI 103/104 are set to ‘1’ for Rel-15+.

For determining the category, the following principles should be taken into account:

- Low complexity/low cost UEs (e.g. Cat-M1, Cat-1) should not be mandated to implement TM9.

- UEs that are already deployed and are not hardware-ready to support TM9 should be allowed to be upgraded to Rel-15 while being spec compliant (e.g. to support a Rel-15 feature).

In our view, the first category that meets both requirements above is Category 11 – there are a large number of UEs with categories 6 and 9 that support over-the-air updates and are not capable of TM9. Thus, we make the following proposal.

Proposal 2: FGI 103/104 are set to ‘1’ for UE with categories 11 and above (or DL categories with peak data rate larger than or equal to 603008).

For reference, we include in Table 1 the DL categories for which support of FGI 103/104 would be mandatory according to the proposal above.

Table 1 DL Categories with mandatory support of FGI 103/104

| UE DL Category | Maximum number of DL-SCH transport block bits received within a TTI | FGI 103/104 mandatory to set to ‘1’ |
| --- | --- | --- |
| DL Category M1 | 1000 | NO |
| DL Category M2 | 4008 | NO |
| DL Category 0 | 1000 | NO |
| DL Category 1bis | 10296 | NO |
| DL Category 4 | 150752 | NO |
| DL Category 6 | 301504 | NO |
| DL Category 7 | 301504 | NO |
| DL Category 9 | 452256 | NO |
| DL Category 10 | 452256 | NO |
| DL Category 11 | 603008 | YES |
| DL Category 12 | 603008 | YES |
| DL Category 13 | 391632 | NO |
| DL Category 14 | 3916560 | YES |
| DL Category 15 | 749856-807744 | YES |
| DL Category 16 | 978960 -1051360 | YES |
| DL Category 17 | 25065984 | YES |
| DL Category 18 | 1174752-1211616 | YES |
| DL Category 19 | 1566336 -1658272 | YES |
| DL Category 20 | 1948064 - 2019360 | YES |
| DL Category 21 | 1348960 - 1413120 | YES |
| DL Category 22 | 2349504 – 2562784 | YES |
| DL Category 23 | 2695968 – 2869920 | YES |
| DL Category 24 | 2936880 – 3028608 | YES |
| DL Category 25 | 3132672 – 3316544 | YES |
| DL Category 26 | 3422400– 3531888 | YES |

# Timeline

The standard timeline for FGI setting was described in RP-130411 as follows:

- The conditions for mandating FGI as agreed by consensus by TSG-RAN (Update of Proposal 1 in [1]):

- Sufficient IOT facilities should be readily available and be confirmed by 2 network vendors and 2 operators at first. This should finally be decided by TSG-RAN on a case by case basis.

- Mandating FGIs does not require vendors whose UEs are already in the market to pass the IOT and change the FGI settings.

- The timing of when to mandate FGI bits is not dependent on any associated testing regime as test cases will be developed as required in a timely manner.

- Assuming the precondition that the proponents believe that there are sufficient IOT facilities for features to be trialled, the proposed order of events for setting the FGI bits is as follows (Update of Proposal 3 in [1]):

At TSG RAN #X:

Step 1:	The case is made to TSG RAN Plenary where the extent of IOT facilities will be analysed  and the status of any related conformance test cases will be reviewed..

Step 2:	Based on step 1, if there is consensus, TSG RAN to agree to set the bit(s) at a future date that takes into account the need for a preparation period to enable successful IOT. The length of this preparation time is decided on a case by case basis but typically this could be up to 2 Plenary cycles.

Step 3:	At this point, an LS is sent to RAN5 to make them aware of the bit setting situation and to ask RAN5 to take the appropriate action i.e. provide feedback and/or CRs to the following Plenary.

At TSG RAN #(X+2) where the preparation period is 2 Plenary meeting cycles:

Step 4:	After the stipulated preparation time, TSG RAN review and decide whether CRs to change FGI bit settings can be approved or whether additional preparation time is needed. Once decided, an update is to be sent to RAN5. The expectation will be that RAN5takes the appropriate CR related action and informs/updates the certification bodies accordingly.

There was no discussion at RAN#81 on availability of testing/IOT opportunities for the FGI under discussion. There are currently 8 test cases (out of the 22 identified by RAN5) that are not available from any test equipment vendor and thus have never been validated. This is verified as per current GCF DCC V3.72 and PTCRB V5.37 versions.

Test cases that have not been validated

Applicable to Rel.10 and higher UEs

| 9.4.1.3.2_D | TDD PMI Reporting - PUSCH 3-1 (Single PMI) for eDL-MIMO |
| --- | --- |
| 9.4.2.3.2_D | TDD PMI Reporting - PUSCH 1-2 (Multiple PMI) for eDL-MIMO |

Applicable to Rel12 and higher UEs

| Clause | Title |
| --- | --- |
| 9.3.7.1 | FDD CQI Reporting under fading conditions - PUSCH 3-2 for eDL MIMO Enhancement |
| 9.3.7.2 | TDD CQI Reporting under fading conditions - PUSCH 3-2 for eDL MIMO Enhancement |
| 9.4.1.4.1 | FDD PMI Reporting with 4Tx enhanced codebook - PUCCH 1-1 (Single PMI) for eDL MIMO Enhancement |
| 9.4.1.4.2 | TDD PMI Reporting with 4Tx enhanced codebook - PUCCH 1-1 (Single PMI) for eDL MIMO Enhancement |
| 9.4.2.3.3 | FDD PMI Reporting with 4Tx enhanced codebook - PUSCH 1-2 (Multiple PMI) for eDL-MIMO Enhancement |
| 9.4.2.3.4 | TDD PMI Reporting with 4Tx enhanced codebook - PUSCH 1-2 (Multiple PMI) for eDL-MIMO Enhancement |

Observation 1: 8 test cases related to FGI 103/104 have not been implemented by any TE vendor and thus have not been verified

The absence of verification of these tests means that today the TM9 certified UEs are not tested against these for certification. In our view, all the test cases should be verified before specifying the FGI bits to be mandatory. Otherwise, current UEs in development cannot be tested against these, but if/when the test become verified, they will become mandatory for certification. In view of the unavailability of these tests, we propose to target RAN#84 as the meeting for approval of corresponding RAN2/RAN5 CRs. The CRs to introduce these bits as mandatory should only be agreed after all relevant test cases have been verified.

Proposal 3: RAN to target approval of RAN5/RAN2 CRs in RAN#84, with the pre-requisite that all relevant test cases have been verified by then.

# Conclusions

In this contribution we presented our views on the setting of FGI 103/104 to mandatory. We made the following observations and proposals

Proposal 1: FGI 103/104 are set to ‘1’ for Rel-15.

Proposal 2: FGI 103/104 are set to ‘1’ for UE with categories 11 and above (or DL categories with peak data rate larger than or equal to 603008).

Observation 1: 8 test cases related to FGI 103/104 have not been implemented by any TE vendor and thus have not been verified

Proposal 3: RAN to target approval of RAN5/RAN2 CRs in RAN#84, with the pre-requisite that all relevant test cases have been verified by then.