3GPP TSG RAN meeting #82										RP-182355

Sorrento, Italy, December 10th -13th, 2018

Status Report to TSG

Agenda item:		9.3.7

| WI / SI Name | Study on UE Power Saving in NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_NR_UE_pow_sav |  |  |  |  |
| Unique ID | 800094 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-181463 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 06/2019 | Core part: NA | Performance part: NA | Testing part: NA |  |
| Overall Completion level | Study Item: 60 % | Core part: NA | Performance Part: NA | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | Fang-Chen Cheng |
|  | Company | CATT |
|  | Email | fcc@catt.cn |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

      - RAN1 #94bis Agreements

The skeleton is endorsed

R1-1811896	Skeleton of TR38.840 UE Power Saving	CATT

Agreements in Evaluation methodology

Agreements:

- The baseline for UE power saving evaluation is NR Rel-15 including support for DRX, BWP adaptation, cross-slot scheduling, PDCCH monitoring parameters, etc.

Agreements:

- At least dense urban (with single layer) deployment scenario should be included for evaluation. Indoor hotspot and rural macro can be additionally considered.

  - Note: For RRM measurement evaluation, UE power consumption contributed by inter-frequency and/or inter-RAT measurement can be considered.

Agreements:

- The following frequency ranges with the respective carrier frequency and reference sub-carrier spacing should be included for evaluation if a proposed scheme intends to be used for the respective frequency range(s):

  - FR1: 4GHz, 30kHz SCS

  - FR2: 30GHz, 120kHz SCS

Agreements:

- At least TDD should be included for evaluation. FDD can be optionally considered.

Agreements:

- The following reference configuration should be considered for evaluation:

  - Downlink: TDD, FR1, 30 kHz SCS, 100 MHz CC BW, PDCCH region of 2 symbols at beginning of a slot, PDSCH up to 256QAM 4x4 MIMO, #RB for TRS = 52, max 4RX, UE Capability 1

  - Uplink: TDD, FR1, 30 kHz SCS, 100MHz CC BW, 1TX

- Different and/or additional parameter assumptions used for evaluation should be clearly stated.

Agreements:

- Percentage power consumption reduction from the baseline scheme will be used to express the power saving gain

  - FFS: For the case multiple applications are evaluated, whether power consumption is the overall DoU power across the applications

- Latency of packet or scheduling delay, user throughput, system throughput and/or resource utilization/overhead (if applicable) should be reported as the result of the evaluation, in addition to power saving gain.

- If a new signal/channel is introduced, performance, complexity, & overhead for reception of the signal/channel should be evaluated. If the new signal is used for detection, false alarm rate and misdetection rate should be evaluated.

- Numerical analysis, system level simulation, and link level simulation are included as evaluation methods for power saving proposals. At least one of the methods should be selected and used for evaluation of a specific power saving proposal.

  - FFS: Criteria for selection of the methods, including the possibility of up to each company

Agreements:

- FTP model 3 should be included in the evaluation for at least FTP application. Modification to the parameters is not precluded. Other bursty traffic arrival models can be considered.

- Applications including FTP, web-browsing, video streaming, instant messaging, VoIP, gaming, background app sync can be considered for traffic modelling for power saving proposal evaluation.

Agreements:

- Simulation assumptions as specified in Table A1.5-1 in TR38.802 should be the basis for link-level simulation evaluation.

  - Antenna configuration may use IMT-2020 as reference. Companies to state assumptions different from the reference if any

- Simulation assumptions as specified in Table A2.1-1 in TR38.802 should be the basis for system-level simulation evaluation.

  - Antenna configuration may use IMT-2020 as reference. Companies to state assumptions different from the reference if any

Agreements:

- The following power states and relative power values for the reference configuration are adopted as working assumption for power saving SI.

  - FFS: Power modeling/scaling for the case more than one power states in a slot.

  - FFS: Power saving signal processing power and transition energy.

  - FFS: Power scaling for other configurations from the reference configuration

| Reference Configuration | Power State | Characteristics | Relative Power |
| --- | --- | --- | --- |
| Downlink: TDD, FR1, 30 kHz SCS, 1CC, 100 MHz BW, PDCCH region of 2 symbol at beginning of a slot, k0 = 0, max. #CCE = 56, 36 PDCCH blind decoding, PDSCH of max data rate with 256QAM 4x4 MIMO, #RB for TRS = 52, 4RX, Capability 1 Uplink: TDD, FR1, 30 kHz SCS, 1CC, 100MHz BW, 1TX, 2 power levels 0dBm and 23dBm Power values are averaged over the operations within a slot. | Deep Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing may not be maintained. | 1 (Optional: 0.5) |
|  | Light Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. | 20 |
|  | Micro sleep | Immediate transition is assumed for power saving study purpose from or to a non-sleep state | 45 |
|  | PDCCH-only | No PDSCH and same-slot scheduling; this includes time for PDCCH decoding and any micro-sleep within the slot. | 100 |
|  | SSB or CSI-RS proc. | SSB can be used for fine time-frequency sync. and RSRP measurement of the serving/camping cell. FFS the power scaling for RRM of neighbor cells . TRS is the considered CSI-RS for sync. FFS the power scaling for processing other configurations of CSI-RS. | 100 |
|  | PDCCH + PDSCH | PDCCH + PDSCH. ACK/NACK in long PUCCH is modeled by UL power state. FFS the power scaling for PDSCH-only slot. | 300 |
|  | UL | Long PUCCH or PUSCH. FFS the power scaling for short PUCCH and SRS. | 250 (0 dBm) 700 (23 dBm) |

Agreements:

- For evaluation, the following additional transition energy and total transition time for the three sleep types are adopted as working assumption for power saving SI:

  - Both ramp down and ramp up transitions are included.

  - Additional processing time for acquiring synchronization is not included in total transition time.

  - There is always a non-sleep power state between adjacent sleeps.

  - FFS: Specify the times and relative power values for the ramp down and ramp up transitions separately.

| Sleep type | Additional transition energy: (Relative power x ms) | Total transition time |
| --- | --- | --- |
| Deep sleep | 450 | 20 ms |
| Light sleep | 100 | 6 ms |
| Micro sleep | 0 | 0 ms* |
| * Immediate transition is assumed for power saving study purpose from or to a non-sleep state |  |  |

Agreements:

The following DRX scenarios can be considered as reference for evaluation:

- C-DRX cycle 320msec, inactivity timer {200, 80} msec

- FR1 On duration: 10 msec

- FR2 On duration: 5 msec

- C-DRX cycle 160msec, inactivity timer {100, 40} msec

- FR1 On duration: 8 msec

- FR2 On duration: 4 msec

- C-DRX cycle 40msec, inactivity timer {25, 10} msec

- FR1 On duration: 4 msec

- FR2 On duration: 2 msec

- I-DRX cycle 1.28 sec

- Group paging rate (for a PO): [10%]

  - P-RNTI is detected but PDSCH decoding results in no match

  - Note: Statistics for the matching case may be further considered based on use case

Note: Companies may select and report the settings for short DRX cycle, short DRX cycle timer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, and/or drx-HARQ-RTT-TimerUL.

Agreements:

- It is clarified that user throughput measured from system level simulation does not preclude user perceived throughput taking into account TCP slow start if applicable

Agreements:

- For system level impact evaluation, IMT-2020 simulation assumptions can be adopted.

- For system level simulation, whether to apply DRX depends on the particular power saving proposal being evaluated and the purpose of the simulation. Companies may justify if DRX is not applied in the simulation.

Agreements:

- Power scaling scheme for at least FR1 power states:

| Scaling for FR1 | Proposal | Comment |
| --- | --- | --- |
| BWP Bandwidth (DL) | Scaling of X MHz = 0.4 + 0.6 * (X - 20) / 80. Linear interpolation for intermediate bandwidths. Valid only for X = 10, 20, 40, 80, and 100. | For 10MHz BW, only AL up to 8 can be used for PDCCH The transition time is the same as DCI-based BWP switching delay for Rel-15. FFS: transition energy for BWP switching |
| BWP Bandwidth (UL) | No scaling at 0dBm or 23dBm |  |
| CA (DL) | 2CC is 1.7x1CC | Higher CA is FFS Activation/deactivation delay follows RAN4 specification; FFS transition energy |
| CA (UL) | As downlink at 0dBm. No scaling at 23dBm |  |
| Antenna scaling (DL) | 2Rx power is 0.7x 4Rx power | Other antenna counts are FFS |
| Antenna scaling (UL) | 2Tx power is 1.4x 1Tx power at 0dBm. [No scaling] at 23dBm | Other antenna counts are FFS |
| PDCCH-only | Power of cross-slot scheduling is 0.7x same-slot scheduling FFS for the scaling w.r.t. #blinding decoding |  |
| SSB | FFS for #SSB to be processed in one slot (Note 2 SSBs in a slot for the ref. config.) FFS for neighbor cell measurement including cell detection FFS for #measured cells/SSBs |  |
| PDSCH-only slot | [280] |  |
| CSI-RS | FFS for scaling w.r.t. #symbols for CSI-RS FFS for neighbor cell measurement FFS for #measured cells |  |
| Short PUCCH | Short PUCCH power = [0.6] x uplink power |  |
| SRS | SRS power = [0.6] x uplink power |  |

Note: Scaling applies only to non-sleep power states.

FFS: Rule for generating the power values for combination of more than one power states within one slot.

Note: latency involved in antenna scaling is FFS

Agreements:

Reference configuration for FR2:

- Downlink: TDD, FR2, 120 kHz SCS,  1CC, 100 MHz BW, PDCCH region of 2 symbol at beginning of a slot, k0 = 0, max. #CCE = 32, 20 PDCCH blind decoding, PDSCH of max data rate with [64QAM] 2x2 MIMO, #RB for TRS = TBD, 2RX, UE Capability 1

- Uplink: TDD, FR2, 120 kHz SCS, 1CC, 100MHz BW, 1TX, power levels [FFS dBm]

- Power values are averaged over the operations within a slot.

| Power State | Characteristics | Relative Power |  |
| --- | --- | --- | --- |
|  |  | FR1 | FR2 (for reference only) |
| PDCCH-only | No PDSCH and same-slot scheduling; this includes time for PDCCH decoding and any micro-sleep within the slot. | 100 | [175] |
| SSB or CSI-RS proc. | SSB can be used for fine time-frequency sync. and RSRP measurement of the serving/camping cell. FFS the power scaling for RRM of neighbor cells . TRS is the considered CSI-RS for sync. FFS the power scaling for processing other configurations of CSI-RS. (Note 2 SSBs in a slot for the ref. config.) | 100 | [175] |
| PDCCH + PDSCH | PDCCH + PDSCH. ACK/NACK in long PUCCH is modeled by UL power state. FFS the power scaling for PDSCH-only slot. | 300 | [300] |
| UL | Long PUCCH or PUSCH. FFS the power scaling for short PUCCH and SRS. | 250 ( 0 dBm) 700 (23 dBm) | [300] (FFS Tx power level) |

Sleep states power level, energy overhead, and transitions are same as FR1.

Other scaling relationships for FR2: FFS

Agreements:

Power scaling scheme for PDCCH candidates processing reduction:

- Scaling for the power reduction due to PDCCH candidates processing (e.g. AL/CCE/BD) reduction is modelled solely based on its effect on microsleep portion of the PDCCH-only slot

- It is assumed that for PDCCH-only state baseline agreed for FR1, some portion of the slot is in microsleep

  - For scaling determination, companies can report the portion that is assumed

- Companies may state their assumption on how much portion of PDCCH processing time can scale in accordance to PDCCH candidates (e.g. AL/CCE/BD) reduction

  - Note: In the reference configuration, the first two symbols are PDCCH symbols.

Agreements:

For the purpose of basic calibration of traffic modeling, FTP model 3 (use 0.1 Mbytes packet size, mean inter-arrival time 200msec) and VoIP model (as defined in R1-070674) should be used to generate time distribution for different power states, for the following scenarios

- No C-DRX configured

  - For both VoIP and FTP

- C-DRX cycle 40msec for VoIP

  - 10 msec inactivity timer

  - Assume max two packets bundled

- C-DRX cycle 160msec for FTP

  - 100 msec inactivity timer

The time distribution for different power states shall be reported (e.g. x% in PDCCH-only, y% in PDCCH+PDSCH, z% in microsleep, etc), as a result of the calibration exercise.

The following simplifying assumptions can be made: Power modelling reference configuration for FR1. Peak throughput. 100MHz DL BWP. 10-symbol PDSCH (one symbol occupied by DMRS), capable of carrying 868584 information bits per slot (Note: a packet can fit within a PDSCH transmission). All packets can be successfully decoded on the first transmission. No HARQ retransmission. No UL slot. Single user. Short DRX is not configured.

Agreements after email discussion [94-NR-10]

Assumptions:

- SSB periodicity is 20msec and 2 SSBs per a slot are measured

- SMTC periodicity is 20msec while SMTC duration is TBD.

- For inter-band, up to [2] frequency layers and gap pattern ID = 0 defined in TS 38.133 (i.e., measurement gap length of 6msec and measurement gap periodicity of 40msec)

For intra-frequency measurements:

| N: Number of cells for intra-frequency measurement | Synchronous case |  | Asynchronous case |  |
| --- | --- | --- | --- | --- |
|  | FR1 | FR2 | FR1 | FR2 |
| N=8 | 150 | FFS | 170 | FFS |
| N=4 | 120 | FFS | 140 | FFS |

All above values are slot-averaged power (P_fr1 or P_fr2 for FR1 or FR2 respectively).

Synchronous case means actual SSB tranmissions from cells are time-aligned e.g., timing of SSBi from cell i is aligned with timing of SSBi from cell j.

Asynchronous case means actual SSB transmissions from cells are not time-aligned e.g., timing of SSBi from cell i is not aligned with timing of SSBi from cell j

For inter-frequency measurement as function of the number of frequency layers:

UE needs to monitor Nf frequency layers within a measurement gap.

E3=i=0Nf-1Ei+Et∙Nf+1

Where

- Ei is either P_fr1*Ns (for FR1) or P_fr2*Ns (for FR2) for each frequency layer i

- Ns is the number of slot over which measurements (for each frequency layer i) are carried out

- Nf is the number of frequency layers measured

- Et = Pt * Tt, where Pt is the switching power consumption and Tt is 0.5ms for FR1 and 0.25ms for FR2 (from RAN4)

  - [Assume microsleep power for Pt]

And it can be simplified to the following if Ei is same across frequency layers (i.e. Ei = E for all i).

E3 = E*Nf + Et*(Nf+1)

Note: RAN4 requirement assumes only one frequency layer per measurement gap.

For full neighbor cell search:

The baseline neighbor cell search power is independent of the number of cells on the first order and can be approximated as:

| FR1 | FR2 |
| --- | --- |
| [180] | FFS |

It is expressed as the power averaged over a slot during which search is performed.

For combined measurement and search:

FFS

Agreements in UE power saving schemes with UE adaptation to the traffic and UE power consumption characteristics

Agreements:

At least the following power saving techniques are to be further studied (including detailed scheme, performance, complexity, overhead, etc.)

- Time domain

  - Cross-slot scheduling – enhancement from Rel-15

  - Multi-slot scheduling

  - Slot aggregation– enhancement from Rel-15

- DRX configuration – C-DRX enhancement

  - Dynamic Adaptation in configuration

  - Adaptive parameters

- Frequency domain

  - BWP – enhancement of dynamic switching from Rel-15

    - RS (including SRS) configuration for channel tracking, CSI measurements etc.

    - Efficient configuration/switching

    - Association with DRX

  - CA/DC

    - Quick activation/de-activation (e.g.,L1 signaling, MAC signaling)

    - Efficient configuration of SCell

      - E.g., Power saving with CSI/RRM measurements and beam management only but no PDCCH monitoring before activation (SCell dormancy)

- Antenna domain

  - Antenna adaptation

    - Adaptation of number of panels/antenna with consideration of aspects, such as CSI measurements (for both Rx/Tx)

    - Adaptation multi-antenna processing

- UE processing time

  - Adaptation in UE processing time

  - Timeline relaxing of UE processing

- Reduce PDCCH Monitoring

  - Reduced number of PDCCH processing

    - Further reduce the number of PDCCH blind decoding

    - Adaptation in CORESET, search space, PDCCH candidate, AL, CCE, DCI formats/RNTI monitoring

    - UE/Network Assistant Information

    - Adaptive configured parameters

    - Association with C-DRX

  - Network assistance

    - Decoupling of DL and UL grant

    - Configured RS for channel tracking and estimation

- Assistant information for adaptation

  - UE assistance:  UE assistant information/feedback used for adaptation in some domains

  - Adaptation profile

Agreements in Trigger UE adaptation of UE power consumption characteristics

Agreements:

- Study further the following for Triggering adaptation of UE power consumption characteristics

- Existing signal/channel based approach

  - Signals based on PDCCH channel

  - Signals based on RS

  - MAC signalling

  - RRC signalling

- New power saving signal/channel

- Performance metrics based on the agreements in the evaluation methodology

Agreements:

Study further:

- Triggering for UE time domain processing adaptation

  - Trigger UE adaptation to DRX operation

    - UE DRX PDCCH monitoring and efficient UE wakeup

      - Constraint on scheduling DCI during DRX ON

    - Performing CSI measurement/feedbacks and RRM measurements

    - Reducing power consumption during DRX ON

      - e.g., go-to-sleep signalling to assist UE to the sleep state

    - Triggering dynamic adaptation among multiple DRX configurations

  - Trigger UE adaptation in reducing PDCCH monitoring

    - Indication to change PDCCH monitoring behaviour, e.g., to monitor, to skip, to adapt to different PDCCH parameters

- Triggering for UE frequency domain processing adaptation

  - Trigger for the adaptation of BWP

    - State transition during BWP switching

      - Pre-processing information, e.g., CSI measurement/feedback, before switching/activation for gNB to use after switching to the new BWP

    - UE assistance approach for network configuration

      - UE initiates the request to the network

      - Network trigger UE feedback for the adaptation

  - Trigger for the carrier adaptation in CA/DC

    - State transition in CA/DC activation/deactivation

      - Pre-processing information, e.g., CSI measurement/feedback, before switching/activation for gNB to use after carrier activation

    - Adaptation among different cells with different power consumption charateristics

      - Bundle adaptation among different cells

    - UE assistance approach for network configuration

      - UE initiates the request to the network

      - Network trigger UE feedback for the adaptation

- Triggering UE processing adaptation

  - Trigger for adaptation in number of Tx/Rx antenna and/or maximum number of MIMO layers and/or number of antenna panels

    - UE assistance approach for network configuration

      - UE initiates the request to the network

      - Network trigger UE feedback for the adaptation

    - Network instructed

  - Trigger for adaptation in UE processing time

    - UE assistance information, e.g., k0, k1, k2 value

    - Network instructed

  - Trigger for adaptation of UE processing e.g., in maximum modulation orders, TB sizes, HARQ operation

- Triggering in reducing PDCCH blind decoding

# Conclusion:

- Proposal of UE power schemes not listed or FFS in the UE power consumption model should also include the proposal of modeling of UE power consumption for the discussion at the next meeting.

Agreements in UE power Consumption Reduction in RRM Measurements

Agreements:

- RRM power saving study should consider RRC IDLE, INACTIVE and CONNECTED states.

Agreements:

- Further study on relaxing/adapting RRM measurement for UE power saving,

  - Relaxing/adapting the number of RRM measurements in time domain for serving cell and neighbor cells based on certain conditions, e.g., number of SSB/CSI-RS/measurement occasions.

  - Relaxing/adapting the number of measured cells/frequencies for neighbor cells based on different conditions

  - Schemes considered should disclose the mechanism used to determine the mobility/speed assumed

  - FFS conditions, e.g., channel condition, UE location, UE speed, measurement configuration, measurement results, cell type, etc.

  - FFS impact on measurement/mobility performance

  - Relaxed/adapting measurement report (if any)

  - Other solutions are not precluded.

Agreements:

- Further study on the benefit/impact for additional resource for RRM measurements for power saving for mobility performance not inferior to that of Rel-15

  - The schemes proposed should at least include the following aspects:

    - The signals/channels used as additional resource

    - Procedure on how to use additional resources.

    - Overhead from network perspective should be considered.

Agreements:

- Focus on stationary (e.g., 0km/h), pedestrian (e.g., 3km/h) and vehicular (e.g., 30km/h) scenarios for further studies when considering UE power saving techniques for RRM measurements,

- Further studies UE power saving for RRM measurement by considering the following issues

  - Multi-beam/multi-cell related RRM measurement issues.

  - Possible misalignment between SSB(s)/SMTC and DRX ON duration

      - Agreements in RAN1#95

R1-1812639	Update of TR38.840v0.0.1 UE Power Saving	CATT

R1-1814387	TR38.840v0.1.0 UE Power Saving in NR	CATT

Agreements in Evaluation methodology

Agreements:

# To capture the following in the TR:

|  | Power States | HW | Vivo | MTK | ZTE | Intel | LGE | CATT | SS [10] | E/// | QCOM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FTP, w/o C-DRX | PDCCH only |  | 99.75% | 99.75% | 99.73% | 99.75% | 99.75% | 99.71% | 99.75% | 99.80% | 99.75% |
|  | PDCCH+PDSCH |  | 0.25% | 0.25% | 0.27% | 0.25% | 0.25% | 0.29% | 0.25% | 0.20% | 0.25% |
|  | Micro sleep |  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Light sleep |  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Deep sleep |  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| FTP, w/ C-DRX | PDCCH only | 34.96% | 34.68% | 34.62% | 35% | 34.96% | 35.27% | 37.92% | 32.85% | 38.20% | 35.01% |
|  | PDCCH+PDSCH | 0.21% | 0.25% | 0.20% | 0.27% | 0.25% | 0.26% | 0.29% | 0.20% | 0.20% | 0.25% |
|  | Micro sleep | 0.02% | 0.01% | 0.02% | 0.03% | 0.02% | 0.02% | 0.00% | 0.01% | 0% | 0.01% |
|  | Light sleep | 0.15% | 0.13% | 0.17% | 0% | 0.15% | 0.17% | 0.00% | 0.14% | 0% | 0.15% |
|  | Deep sleep | 64.66% | 64.94% | 64.99% | 65% | 64.62% | 64.28% | 61.85% | 66.80% | 61.60% | 64.58% |
| VoIP, w/o C-DRX | PDCCH only |  | 98.59% | 98.56% | 98.43% | 98.75% |  | 98.70% | 98.46% | 98.50% | 98.60% |
|  | PDCCH+PDSCH |  | 1.41% | 1.44% | 1.57% | 1.25% |  | 1.31% | 1.54% | 1.50% | 1.40% |
|  | Micro sleep |  | 0.00% | 0.00% | 0.00% | 0.00% |  | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Light sleep |  | 0.00% | 0.00% | 0.00% | 0.00% |  | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Deep sleep |  | 0.00% | 0.00% | 0.00% | 0.00% |  | 0.00% | 0.00% | 0.00% | 0.00% |
| VoIP, w/ C-DRX | PDCCH only | 22.09% | 22.40% | 21.46% | 19.31% | 18.72% |  | 14.43% | 20.16% | 32.40% | 22.31% |
|  | PDCCH+PDSCH | 1.08% | 1.09% | 1.09% | 0.78% | 1.25% |  | 0.40% | 1.96% | 1.50% | 1.08% |
|  | Micro sleep | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |  | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Light sleep | 1.37% | 0.61% | 0.41% | 0.00% | 0.00% |  | 0.00% | 0.34% | 26.50% | 0.54% |
|  | Deep sleep | 75.45% | 75.90% | 77.04% | 79.92% | 80.03% |  | 85.16% | 77.54% | 39.60% | 76.07% |

Agreements:

Update the intra-frequency measurement power table for FR2 as follows (shown in change marks):

| N: Number of cells for intra-frequency measurement | Synchronous case |  | Asynchronous case |  |
| --- | --- | --- | --- | --- |
|  | FR1 | FR2 | FR1 | FR2 |
| N=8 | 150 | FFS [225] | 170 | FFS [285] |
| N=4 | 120 | FFS [195] | 140 | FFS [255] |

Agreements:

For power modelling purpose, adopt the following maximum number of cells for intra-frequency measurement within a slot:

| FR1 | FR2 |
| --- | --- |
| Nmax = 19 | Nmax = 9 |

Agreements:

Additions or modifications to the previously agreed table in RAN1#94bis are marked in red below:

|  | Proposal | Comment |
| --- | --- | --- |
| BWP Bandwidth (DL) | Scaling of X MHz = 0.4 + 0.6 * (X - 20) / 80. Linear interpolation for intermediate bandwidths. Valid only for X = 10, 20, 40, 80, and 100. Above scaling is applicable for FR1 only. In case scaling is needed for FR2, companies can report the assumed scaling factor. | For 10MHz BW, only AL up to 8 can be used for PDCCH The transition time is the same as DCI-based BWP switching delay for Rel-15. FFS: transition energy for BWP switching |
| BWP Bandwidth (UL) | No scaling at 0dBm or 23dBm Above scaling is applicable for FR1 only. In case scaling is needed for FR2, companies can report the assumed scaling factor. |  |
| CA (DL) | 2CC is 1.7x1CC 4CC is 3.4x1CC (i.e. 2x 2CC) Above refers to the worst case CA configuration in terms of power consumption. | Higher CA is FFS Activation/deactivation delay follows RAN4 specification; FFS transition energy Applicable for FR1 and FR2 |
| CA (UL) | Same as downlink at 0dBm. 2CC is 1.2x1CC at 23dBm Limit scaling up to 2CC. | Applicable for FR1 and FR2 |
| Antenna scaling (DL) | 2Rx power is 0.7x 4Rx power for FR1 1Rx power is 0.7x 2Rx power for FR2 | Other antenna counts are FFS Assume same number of antenna elements per Rx chain |
| Antenna scaling (UL) | 2Tx power is 1.4x 1Tx power at 0dBm; [No scaling] 1.2x at 23dBm. FR1 only. | Other antenna counts are FFS 2Tx support is not considered for FR2. |
| PDCCH-only | Power of cross-slot scheduling is 0.7x same-slot scheduling FFS for the scaling w.r.t. # of blinding decoding | Applicable for FR1 and FR2 |
| SSB | FFS for # of SSB to be processed in one slot (Note 2 SSBs in a slot for the ref. config.) One SSB power is 0.75 of two SSB power, i.e. 75 power units FFS for neighbor cell measurement including cell detection FFS for #measured cells/SSBs |  |
| PDSCH-only slot | [280] for FR1 [280325] for FR2 | This assumes the same number of PDSCH symbols as in the PDCCH+PDSCH case. |
| CSI-RS | FFS for scaling w.r.t. # of symbols for CSI-RS FFS for neighbor cell measurement FFS for #measured cells |  |
| Short PUCCH | Short PUCCH power = [0.6 0.3] x uplink power Reference config consists of 1-symbol PUCCH | Applicable for FR1 and FR2. |
| SRS | SRS power = [0.6 0.3] x uplink power | Applicable for FR1 and FR2. |

Agreements:

For power scaling for PDCCH candidate reduction (for same slot scheduling only):

# P(α) = α ∙ Pt + (1 – α) ∙ 0.7Pt

where α is the ratio of PDCCH candidates to the max number of PDCCH candidates in the reference configuration (α>0). Pt is the PDCCH-only power for same-slot scheduling.

Agreements:

- Update the baseline power table for FR2 as follows (shown in change marks):

| Power State | Characteristics | Relative Power |  |
| --- | --- | --- | --- |
|  |  | FR1 | FR2 (for reference only) |
| PDCCH-only | No PDSCH and same-slot scheduling; this includes time for PDCCH decoding and any micro-sleep within the slot. | 100 | [175] |
| SSB or CSI-RS proc. | SSB can be used for fine time-frequency sync. and RSRP measurement of the serving/camping cell. FFS the power scaling for RRM of neighbor cells . TRS is the considered CSI-RS for sync. FFS the power scaling for processing other configurations of CSI-RS. (Note 2 SSBs in a slot for the ref. config.) | 100 | [175] |
| PDCCH + PDSCH | PDCCH + PDSCH. ACK/NACK in long PUCCH is modeled by UL power state. FFS the power scaling for PDSCH-only slot. | 300 | [300 350] |
| UL | Long PUCCH or PUSCH. FFS the power scaling for short PUCCH and SRS. | 250 ( 0 dBm) 700 (23 dBm) | [300 350] (FFS Tx power level) |

Agreements:

Update the full neighbour cell search power table as follows (shown in change marks):

# For full neighbor cell search:

The baseline neighbor cell search power is independent of the number of cells on the first order and can be approximated as:

| FR1 | FR2 |
| --- | --- |
| [150] | [270] |

# It is expressed as the power averaged over a slot during which search is performed.

Agreements:

# For combined measurement and search:

To obtain combined neighbor cell measurement and search power, add the difference between full neighbor cell search state power and SSB/CSI-RS proc. state power to the neighbor cell measurement power, as shown below:

| N: Number of cells for intra-frequency measurement & search | Synchronous case |  | Asynchronous case |  |
| --- | --- | --- | --- | --- |
|  | FR1 | FR2 | FR1 | FR2 |
| N=8 | 200 | [320] | 220 | [380] |
| N=4 | 170 | [290] | 190 | [350] |

Agreements:

For UE power modelling, the following power scaling relationship is assumed:

- For simplicity,

  - The slot-averaged power for “PDCCH+PDSCH+PUCCH” is same as “PDCCH+PDSCH”

  - The slot-averaged power for “PDSCH+PUCCH” is same as “PDSCH-only”

  - The slot-averaged power for “PDCCH+PUCCH” is the sum of “PDCCH-only” power and “short PUCCH” power.

    - Note: PDCCH-only with cross-slot scheduling scaling is also applicable

  - Note: it is observed via evaluations (where 0dBm PUCCH tx power is assumed) that the difference is not significant

- For non-RRM evaluation, for “PDCCH+PDSCH” concurrent in the same slot as “SSB or CSI-RS processing”, assume “PDCCH+PDSCH” power

- For non-RRM evaluation, for “PDSCH only” concurrent in the same slot as “SSB or CSI-RS processing”, assume “PDSCH only” power

- For “PDCCH-only” and “SSB or CSI-RS processing” concurrent in a slot, the slot-averaged power is 0.85x the sum of the respective power, at least for FR1

  - For FR1, 0.85*(100+100) = 170

  - For FR2, 0.85*(175+175) = [300]

- For “SSB processing” and “CSI-RS processing” concurrent in a slot, the slot-averaged power is 0.85x the sum of the respective power, at least for FR1

  - For FR1, 0.85*(100+100) = 170

  - For FR2, 0.85*(175+175) = [300]

  - 2-SSB within a slot is assumed

# Conclusion:

- For reducing simulation time for system level simulation, scaling of the bandwidth and packet size of the traffic model can be considered (e.g. factor of 1/5). Companies to report the scale factor used, if any.

Agreements:

- For LLS simulation on power saving signal detection performance, at least the following SINR values should be evaluated: -6dB, 3dB, 20dB.

Agreements:

- For system level simulation, at least assume that the traffic model is FTP model 3 with 0.5Mbyte payload and mean inter-arrival time of 200 milliseconds

  - The mean inter-arrival time can be adjusted to achieve different cell loading scenarios.

  - Note: Other variations including different traffic models are not precluded

- The same traffic model settings should be used across all users.

Agreements:

For FTP, instant messaging, and VoIP application, the following traffic models and DRX configuration should be included for evaluation:

|  | FTP traffic | Instant messaging | VoIP |
| --- | --- | --- | --- |
| Model | FTP model 3 | FTP model 3 | As defined in R1-070674. Assume max two packets bundled. |
| Packet size | 0.5 Mbytes | 0.1 Mbytes |  |
| Mean inter-arrival time | 200 ms | 2 sec |  |
| DRX setting | Period = 160 ms Inactivity timer = [100] ms | Period = 320 ms Inactivity timer = 80 ms | Period = 40 ms Inactivity timer = 10 ms |

Note: For ON duration setting, following reference DRX configurations as previously agreed.

Agreements:

- For web-browsing, video streaming, and gaming applications, the traffic models and the delay requirements defined in R1-070674 can be used in the evaluation. The parameters (e.g. packet size) may be updated to be in line with EMBB traffic requirements.

- For background app sync application, for power consumption evaluation purpose, it can be assumed that idle mode operations (inclusive of page detection, RRM, deep sleep and transition overhead) contributes to X% of the use case power. The remaining portion is contributed by intermittent RRC connections due to background activities (FFS: value of X)

- Companies should report the assumptions made in the evaluation

Agreements:

For power evaluation,

- For evaluation of DRX scenarios, the RRC release timer is set to infinity to ensure C-DRX and I-DRX cases are treated separately.

- Power consumption for channel/time/frequency and beam tracking operations should be taken into account.

Agreements:

Use the following as a reference for RRM power evaluation:

- Assume SMTC window duration of 2ms (corresponding to 30kHz SCS) for synchronised FR1 scenario.

- Assume SMTC window duration of 5ms for all other cases FR2 scenario.

- Note: other values are not precluded

Agreements in UE power saving schemes with UE adaptation to the traffic and UE power consumption characteristics

Agreements:

RAN1 to study further following:

  - UE switching to micro sleep after PDCCH reception

  - Cross-slot scheduling

    - Minimum K0 > 0 and aperiodic CSI-RS triggering offset is not within the duration - UE could switch to micro sleep right away after PDCCH reception – no addition PDSCH and CSI-RS signals reception within the given duration (e.g. the same slot)

      - It is known to the UE at PDCCH decoding

      - Extended micro sleep time and reduce the PDCCH processing in reducing UE power consumption

      - FFS: whether minimum K2 > 0 is essential to avoid the requirements of fast PDCCH processing

  - Same slot scheduling

      - Adaptation of TDRA configurations to achieve UE power saving – ensure the gap between PDCCH reception and PDSCH transmission known to the UE

      - Adjustment of TDRA configuration

      - Selection of TDRA entry in the TDRA table (e.g. K0 > 0)

        - Note: cross-slot scheduling could be incorporated in the TDRA configuration

      - FFS: Power model for TDRA power saving scheme

  - Multi-slot scheduling – PDCCH decoding in one slot (e.g., one DCI, multiple DCI) supports scheduled PDSCH/PUSCH transmission over multiple slots.

    - Achieving UE power consumption reduction by potentially skipping PDCCH monitoring at subsequent slots of PDSCH/PUSCH transmission.

- Other approach(es) not precluded

# Companies are encouraged to perform more analysis/evaluations for the above schemes

Agreements:

# For power saving scheme with UE adaptation to the DRX operation for further study

  - UE adaptation of its behavior to the DRX operation for UE power consumption reduction

    - Power saving signal as the signal for the indication whether to wakeup or not before or at the beginning of DRX ON duration

      - At least for the indication of PDCCH monitoring

    - Preparation period in advance of DRX ON, e.g., to perform channel tracking, CSI measurements, beam tracking, in preparation for the PDCCH decoding

    - Go-to-sleep signaling as the indication allowing UE going back to sleep state

    - Constraints on scheduling DCI during DRX_ON

  - Dynamic DRX configuration – parameters could be dynamic adapted to the traffic arrival

    - Dynamic selection of DRX configuration from multiple DRX configurations to adapt to the traffic arrival

    - Adaptive parameters setting of one DRX configuration

Other power saving schemes with UE adaptation to the DRX operation are not precluded.

Note: UE background processing, e.g.,periodic RRM/CSI measurements, beam management, RLM, time/frequency tracking,  needs to be studied in accordance to UE adaptation to DRX operation.

Agreements:

The UE power power saving schemes for the UE adaptation in frequency domain for further study are as follows,

  - BWP -  UE adaptation to different BWP

    - RS to assist UE channel tracking and measurements to assist BWP switching

    - Enhancement of L1 signaling, e.g., power saving signal or DCI for power saving, in triggering the BWP switching

    - Association of BWP and DRX configuration

  - CA/DC –

    - Quick activation/de-activation (e.g.,L1 signaling, MAC CE enhancement)

    - Adaptation of PDCCH monitoring/search space on activated SCell

    - Power adaptation based on the operation in a group of cell in power efficient way

    - CSI/RRM measurements and beam management at non-active SCell

# Other power saving schemes with UE adaptation in frequency domain are not precluded.

Agreements:

RAN1 to further study the following,

    - The power saving schemes of indicated processing time of K0, K1, K2 and aperiodic CSI-RS offset in advance allows the UE in staying low power consumption state for the UE power saving.

    - Other schemes are not precluded

Agreements:

The power saving schemes to reduce PDCCH monitoring and blind decoding for further studies are as follows,

  - Triggering of PDCCH monitoring – dynamic trigger through L1 signal/signaling

    - Power saving signal triggering PDCCH monintoring

    - Go-to-sleep signaling to skip PDCCH monitoring

  - PDCCH skipping -

    - DCI based indication  for PDCCH skipping (e.g., indication in DCI content, new SFI state).

    - L1 signal/signaling (other than DCI) based triggering  -

  - Mulitple CORESET/search space configurations

    - Configuration of different PDCCH periodicities with dynamic signaling

    - Adaptation of CORSET/search space configuration – DCI/timer/HARQ-ACK based indication

    - Dynamic/semi-persistent CORSET/search space ON/OFF

    - Adaptation between DRX ONduration timer and inactivitytimer

    - Separated PDCCH monitoring of DL and UL

  - L1 signaling triggering to assist  UE in reducing the number of PDCCH blind decoding –

  - Reduced PDCCH monitoring on SCell (including cross carrier scheduling)

  - Network assistance –  RS is dynamically transmitted based on the need to assist UE performing synchronization, channel tracking, measurements and  channel estimations before PDCCH decoding

# Other power saving schemes for the reduction of PDCCH monitoring and blind decoding are not precluded.

Agreements:

The UE assistance information for the power saving schemes for further studies are as follows,

  - UE assistance information/feedback to assist network in configurations for UE adaptation

    - UE preferred processing timeline parameters, e.g., K0, K1, K2 values

    - UE preferred BWP information/configuration

    - UE preferred antenna configuration, including MIMO layers, antenna panel awareness information

    - UE assistance/feedback on the DRX configurations/parameters

    - UE preferred BWP provided to assist network in BWP switching

    - UE request on SCell/SCG activation/de-activation/configuration

    - UE preferred PDCCH monitoring parameters/search space configuration/maximum number of blind decoding

# Other UE assistance information for the power saving schemes is not precluded.

Agreements in Trigger UE adaptation of UE power consumption characteristics

Agreements:

# The power saving signal/channel for UE adaptation includes the following signals/channels for further study

- Existing signal/channel based power saving signal/channel

  - PDCCH channel

  - TRS, CSI-RS type  RS, SSS-like and DMRS

  - PDSCH channel carried MAC CE and/or RRC signaling

- New power saving signal/channel – sequence based

The aspects of the power saving signal/channel used for the UE adaptation to the traffic used for further evaluation of power saving signal design in addition to its triggering to the power saving gain.

- Network resource overhead

  - Resource and/or periodicity of power saving signal/channel

  - Multiplexing capability

  - Usage of resource

- Coexistence/multiplexing with existed signal/channel of Rel-15

- UE-specific, group-specific, cell-specific power saving signal/channel

- Detection performance

- Complexity

- Power consumption of the power saving signal/channel

Agreements:

- The power saving signal/channel in trigger UE adaptation to DRX operation for further study is that the power saving signal/channel can be configured along the DRX configuration as the indication for UE to wake up from the sleep state.  RS resources can be considered to assist UE in performing RRM/CSI measurement and channel time/frequency and/or beam tracking.

  - Other schemes are not precluded

Agreements:

- The power saving signal/channel candidate in triggering UE to achieve reducing PDCCH monitoring for further study  is that the power saving signal/channel can be used to trigger UE to skip the PDCCH monitoring and/or to go to sleep for a period of time.

  - Other schemes are not precluded

Agreements:

- The power saving signal/channel in trigger UE frequency domain processing adaptation for further study is the power saving signal/channel can be used to trigger the indication of RS configuration for channel tracking, CSI measurents, and beam management for the additional assistance of dynamic switching of BWP or activation of SCell in achieving the power saving gain.  The power saving signal/channel can be used for BWP switching, activation/deactivation of SCell or adaptation of PDCCH monitoring and/or CORESET/search space of PCell/SCell.

  - Other schemes are not precluded

Agreements:

For further study, the power saving signal/channel in triggering adaption to the UE processing, such as MIMO configuration/layers, antenna configuration, UE processing time, and background processing can be used as the indication for UE adaptation.  The power saving signal/channel in triggering UE adaptation to the processing is to allow UE in reducing the power consumption by indication of the processing time, such as PDCCH/PDSCH/PUSCH/PUCCH processing or the essential background processing, such as periodic CSI and RRM measurements.

  - Other schemes are not precluded

Agreements in UE power Consumption Reduction in RRM Measurements

Agreements:

- Observation: for certain conditions (e.g., low mobility deployment/UE speed/favorable RSRP conditions), the number of RSRP measurement samples for a given duration (e.g., measurement period / evaluation period) can be relaxed with negligible impact on accuracy achieved by existing Rel-15 measurement.

Agreements:

- Observation:

  - For certain conditions (e.g., favorable RSRP conditions, etc.), reducing RRM measurement activities (e.g., measurement, reporting) for a given time period is beneficial from UE power saving perspective for RRC IDLE/INACTIVE/CONNECTED states.

Agreements:

- For intra frequency and/or inter frequency measurement, the following approaches are to be studied for UE power saving in time domain, including impact on mobility performance

  - Increasing measurement period

  - Reducing number of samples (e.g., OFDM symbols / slots) within a measurement period (e.g., SMTC window)

  - Confining RRM measurements within a measurement window and increasing the periodicity of the measurement window for intra frequency and/or inter frequency measurement

  - Other approaches are not precluded

  - Note: this does not necessarily mean that the techniques studied will have spec impact

Agreements:

To further study the following adaption mechanism of RRM measurement activities for UE power saving:

- gNB controlled RRM measurement operation with UE assistance information reported to gNB, e.g.,

  - mobility related information (e.g., mobility state, history of mobility state, UE's visited cells and cells not reselected due to the ping-pong effect, the number of handovers for certain period, etc.)

  - channel condition (e.g. change in serving RS/signal)

- gNB controlled RRM measurement operation without UE assistance information reported to gNB based on certain conditions, e.g.,

  - Doppler estimation for RRC CONNECTED states

  - cell type (e.g., small cell/macro cell)

- gNB controlled threshold to support UE autonomous RRM measurement adaptation based on e.g.,

  - signal measurements (e.g., RSRP)

  - UE mobility state (e.g., low/medium/high mobility)

  - UE location in the cell (e.g., cell-center/cell-edge)

  - S-measure enhancement (e.g. S-measure for SCell, CSI-RS)

- Other mechanisms/approaches are not precluded

Agreements:

Observation:

- Reducing the number of cells for intra-frequency measurement can be beneficial for UE power saving,

  - Assuming that UE can limit the processing for measurement within a constrained time period and/or with reduced complexity.

  - Assuming number of neighbouring cells to be measured is reduced.

- For UE power saving perspective, reducing the need in neighbour cell intra-frequency measurement can be beneficial.

Agreements:

Observation: Under certain conditions and certain deployment scenarios, additional resource for RRM measurement can be beneficial for UE power saving, including at least the following aspects:

- Minimizing/reducing the timing gap between measurement (e.g., SSB) and DRX ON duration (e.g., paging monitoring occasion/reception, data reception, etc.)

- Additional resource around the measurement occasion, e.g., for AGC assistance

- Reducing measurement activities by providing additional resource may provide sufficient measurement/T-F accuracy.

2.1.2	Remaining Open issues

- Evaluation methodology –

  - TDRA power model

  - The power scaling for processing other configurations of CSI-RS.(note: 2 SSBs in a slot for the ref. config)

  - Power saving signal processing power and transition energy.

  - UL Tx power level for FR2

  - The power scaling for PDSCH-only slot for FR2

  - Transition energy for BWP switching

  - Transition energy for DL CA

  - CSI-RS power consumption model

    - Scaling with respect to  number of symbols for CSI-RS

    - Neighbor cell measurements

    - Number of measured cells

  - For the case multiple applications are evaluated, whether power consumption is the overall DoU power across the applications

  - The value of X in percentage is assumed for background app sync application.   For power consumption evaluation purpose, it can be assumed that idle mode operations (inclusive of page detection, RRM, deep sleep and transition overhead) contributes to X% of the use case power. The remaining portion is contributed by intermittent RRC connections due to background activities

- The detail description and procedure for the UE power saving schemes with UE adaptation in time, frequency, antenna, DRX operation, UE processing timeline.

- The detail design of the power saving signal/channel and procedure in triggering UE adaptation to the power consumption characteristic.

- The UE and network performance impact and complexity of the power saving schemes.

- Details of power saving schemes in reducing the UE power consumption for RRM measurements.

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

- Timer based power saving schemes

- Higher layer procedures of UE power consumption reduction in RRM measurements in synchronous and asynchronous network deployment

- Higher layer procedures for UE power saving

  - Study the enhancement of  UE paging procedure based on the additional power saving signal/channel/procedure

  - Study the enhancement of  UE power saving procedure in supporting efficient transition from RRC_CONNECTED to RRC_IDLE/RRC_INACTIVE mode

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

2.4.2	Remaining Open issues

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTx

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

- R1-1810559	Work plan for UE Power Saving	CATT

- R1-1810560	Skeleton of TR38.840 UE Power Saving	CATT

- R1-1812639	Update of TR38.840 UE Power Saving	CATT

- R1-1814387	TR38.840v0.1.0 UE Power Saving in NR	CATT

- R1-1811281	UE Power Saving Evaluation Methodology	Qualcomm Incorporated

- R1-1810447	Evaluation methodology for NR UE power saving study	MediaTek Inc.

- R1-1810153	Evaluation methodology for UE power saving	Huawei, HiSilicon

- R1-1810306	Discussion on evaluation methodology for UE power saving	LG Electronics

- R1-1810336	Discussion on UE power consumption model and simulation assumptions	ZTE Corporation

- R1-1810412	Evaluation methodology for NR UE power saving	vivo

- R1-1810561	Evaluation Methodology for UE Power Saving Scheme	CATT

- R1-1810794	Evaluation methodology for UE power saving techniques	Intel Corporation

- R1-1810891	Evaluation Methodology	Samsung

- R1-1811016	Study on modeling of UE power consumption	Spreadtrum Communications

- R1-1811126	Power model and evaluation assumptoins for UE power saving mechanism evaluation	Apple Inc.

- R1-1811136	Power state model and IDLE mode evaluation assumptions for NR power consumption study	Sony

- R1-1811224	On evaluation methodology for power saving 	InterDigital, Inc.

- R1-1811478	Evaluation methodology for UE power consumption estimation	Nokia, Nokia Shanghai Bell

- R1-1811500	Evaluation Methodology for NR UE power saving techniques	Ericsson

- R1-1810337	Consideration on potential techniques for UE power saving	ZTE Corporation

- R1-1811524	Overall views on Potential Techniques for UE Power Saving	CAICT

- R1-1811050	Considerations for UE power saving	CMCC

- R1-1811127	Network-indication based Approaches for UE power saving	Apple Inc.

- R1-1810154	Power consumption reduction based on time/frequency/antenna adaptation	Huawei, HiSilicon

- R1-1810307	Discussion on UE power saving in NR	LG Electronics

- R1-1810338	Consideration on UE adaptation to the traffic and UE power consumption characteristics	ZTE Corporation

- R1-1810413	Techniques on UE adaptation to the traffic and UE power consumption characteristics	vivo

- R1-1810448	UE adaptation to the traffic and UE power consumption characteristics	MediaTek Inc.

- R1-1810468	Discussion on UE traffic adaptation and power consumption characteristics	PANASONIC

- R1-1810562	UE Power saving schemes with multi-dimensional adaptation	CATT

- R1-1810795	UE adaptation to the traffic and UE power consumption characteristics	Intel Corporation

- R1-1810892	Discussion on UE adaptation schemes	Samsung

- R1-1810976	UE adaptation to the traffic for UE power saving	Guangdong OPPO Mobile Telecom.

- R1-1811137	Enhancements of NR power saving feature	Sony

- R1-1811225	On UE power consumption characteristics 	InterDigital, Inc.

- R1-1811282	UE Adaptation to the Traffic and UE Power Consumption Characteristics	Qualcomm Incorporated

- R1-1811382	Discussion on UE adaptation to the traffic and UE power consumption characteristics	NTT DOCOMO, INC.

- R1-1811392	Initial views on the PDCCH monitoring for power saving	Xiaomi Communications

- R1-1811458	PDSCH resource allocation restriction for power saving	ASUSTEK COMPUTER (SHANGHAI)

- R1-1811479	On UE adaptation to the traffic	Nokia, Nokia Shanghai Bell

- R1-1811501	Adaptation aspects of NR UE power saving	Ericsson

- R1-1810155	Signaling based mechanisms for UE power saving	Huawei, HiSilicon

- R1-1810563	UE Power saving schemes with power saving signal/channel/procedures	CATT

- R1-1810308	Discussions on triggering signaling for UE power saving	LG Electronics

- R1-1810414	Discussion on triggering adaptation of UE power consumption characteristics	vivo

- R1-1810449	Considerations on triggering adaptation and the potential power saving benefit	MediaTek Inc.

- R1-1810469	Discussion on UE traffic adaptation procedures	PANASONIC

- R1-1810645	Channels and procedures for adapting UE power consumption characteristics	Sony

- R1-1810796	Triggering adaptation of UE power consumption characteristics	Intel Corporation

- R1-1810893	Discussion on triggering adaptation schemes	Samsung

- R1-1810997	Triggering adaptation  to the traffic for UE power saving	Guangdong OPPO Mobile Telecom.

- R1-1811015	Discussion on triggering adaptation for UE power saving	Spreadtrum Communications

- R1-1811051	Discussion on CSI report enhancement for CDRX	CMCC

- R1-1811089	Triggering adaptation of UE power consumption	ASUSTEK COMPUTER (SHANGHAI)

- R1-1811128	UE-assisted Approaches for UE power saving	Apple Inc.

- R1-1811226	On UE adaptation for power saving 	InterDigital, Inc.

- R1-1811283	Triggering Adaptation of UE Power Consumption Characteristics	Qualcomm Incorporated

- R1-1811480	On UE Power Saving Triggering Mechanisms for Rel-16 NR	Nokia, Nokia Shanghai Bell

- R1-1811502	Triggering aspects of adaptation of NR UE power saving	Ericsson

- R1-1811627	Triggering Adaptation of UE Power Consumption Characteristics	Convida Wireless

- R1-1810415	On UE power Consumption Reduction in RRM Measurements	vivo

- R1-1811481	UE Power Consumption Reduction in RRM Measurements	Nokia, Nokia Shanghai Bell

- R1-1810156	UE power consumption reduction in RRM measurements	Huawei, HiSilicon

- R1-1810309	Considerations on RRM measurements to reduce UE power consumption	LG Electronics

- R1-1810450	Considerations on NR RRM power saving	MediaTek Inc.

- R1-1810564	UE Power saving scheme for RRM measurements	CATT

- R1-1810797	UE power Consumption Reduction in RRM Measurements	Intel Corporation

- R1-1810894	Discussion on UE power consumption reduction in RRM measurement	Samsung

- R1-1810988	UE power Consumption Reduction in RRM Measurements	Guangdong OPPO Mobile Telecom.

- R1-1811012	NR power saving on RRM measurement	Spreadtrum Communications

- R1-1811138	Reducing UE power consumption in RRM measurements	Sony

- R1-1811284	UE Power Consumption Reduction in RRM Measurements	Qualcomm Incorporated

- R1-1811383	Discussion on UE power Consumption Reduction in RRM Measurements	NTT DOCOMO, INC.

- R1-1811503	RRM aspects of NR UE power saving	Ericsson

- R1-1810310	Discussion on power saving for DRX operation	LG Electronics

- R1-1810311	Discussion on power saving for PDCCH monitoring	LG Electronics

- R1-1810312	Discussion on power saving for CA operation	LG Electronics

- R1-1810416	Considerations on traffic modelling for UE power saving	vivo

- R1-1810451	More aspects for NR UE power saving	MediaTek Inc.

- R1-1810714	UE Power saving in RRC_IDLE mode	Huawei, HiSilicon

- R1-1811296	Optimizing power consumption of connected mode UE during RACH	Motorola Mobility, Lenovo

- R1-1811504	Network impact of NR UE power saving	Ericsson

- R1-1811505	NR UE power consumption modeling details	Ericsson

- R1-1811506	Uplink aspects of NR UE power saving	Ericsson

- R1-1812329	Remaining aspects for evaluation methodology for power saving	vivo

- R1-1812420	Consideration on UE power consumption model and preliminary evaluation results	ZTE Corporation

- R1-1812640	Evaluation Methodology for UE Power Saving Scheme	CATT

- R1-1812230	Evaluation methodology for UE power saving	Huawei, HiSilicon

- R1-1812360	Evaluation methodology for UE power saving	MediaTek Inc.

- R1-1812512	Evaluation methodology for UE power saving techniques	Intel Corporation

- R1-1812586	Discussion on evaluation methodology for UE power consumption	LG Electronics

- R1-1812823	Evaluation methodology for UE power saving	OPPO

- R1-1812924	Evaluation methodology for UE power saving	Apple Inc.

- R1-1813010	Evaluation Methodology for UE power saving	Samsung

- R1-1813181	Evaluations and modeling of UE power consumption	Ericsson

- R1-1813242	Discussion on power saving evaluation methodology 	InterDigital, Inc.

- R1-1813446	UE Power Saving Evaluation Methodology	Qualcomm Incorporated

- R1-1813619	Evaluation methodology for UE power consumption estimation	Nokia, Nokia Shanghai Bell

- R1-1813634	Traffic modelling for power consumption evaluation	Sony

- R1-1812231	Power saving techniques	Huawei, HiSilicon

- R1-1812513	UE adaptation to the traffic and UE power consumption characteristics	Intel Corporation

- R1-1812925	Power saving techniques based on UE adaptation	Apple Inc.

- R1-1812330	Discussion on UE adaptation to the traffic and UE power consumption characteristics	vivo

- R1-1812361	Adaptation framework for UE power saving	MediaTek Inc.

- R1-1812421	On adaptation aspects for NR UE power consumption reduction	ZTE Corporation

- R1-1812587	Discussion on UE adaptation to UE power consumption characteristics	LG Electronics

- R1-1812641	UE Power saving schemes with adaptation	CATT

- R1-1812749	Adaptive power saving techniques based on UE power consumption characteristics	Sony

- R1-1812824	UE Adaptation to the Traffic and UE Power Consumption Characteristics	OPPO

- R1-1812891	Discussion on CSI report enhancement for CDRX	CMCC

- R1-1813011	On UE adaptation schemes	Samsung

- R1-1813111	PDSCH resource allocation restriction for power saving	ASUSTeK

- R1-1813182	Adaptation aspects of NR UE power saving	Ericsson

- R1-1813243	Discussion on power saving techniques 	InterDigital, Inc.

- R1-1813358	UE power saving in MIMO operation	Motorola Mobility, Lenovo

- R1-1813447	UE Adaptation to the Traffic and UE Power Consumption Characteristics	Qualcomm Incorporated

- R1-1813494	Discussion on UE traffic adaptation and power consumption characteristics	PANASONIC

- R1-1813620	On UE adaptation to the traffic	Nokia, Nokia Shanghai Bell

- R1-1812362	Triggering adaptation for UE power saving	MediaTek Inc.

- R1-1813012	On triggering adaptation schemes	Samsung

- R1-1813621	On UE Power Saving Triggering Mechanisms	Nokia, Nokia Shanghai Bell

- R1-1812232	Design of power saving signal	Huawei, HiSilicon

- R1-1812331	Discussion on triggering adaptation of UE power consumption characteristics	vivo

- R1-1812422	Consideration on triggering for UE power saving	ZTE Corporation

- R1-1812514	Triggering adaptation of UE power consumption characteristics	Intel Corporation

- R1-1812642	UE Power saving schemes with power saving signal/channel/procedures	CATT

- R1-1812750	Conditions and procedures for adaptation of power consumption characteristics	Sony

- R1-1812825	Triggering Adaptation of UE Power Consumption Characteristics	OPPO

- R1-1812890	Considerations on power saving signal design	CMCC

- R1-1812926	Power saving techniques based on explicit indication	Apple Inc.

- R1-1813076	Discussion on triggering adaptation for UE power saving	Spreadtrum Communications

- R1-1813183	Triggers for NR UE power saving	Ericsson

- R1-1813244	Discussion on triggering of power mode adaptation 	InterDigital, Inc.

- R1-1813448	Triggering Adaptation of UE Power Consumption Characteristics	Qualcomm Incorporated

- R1-1813495	Discussion on UE traffic adaptation procedures	PANASONIC

- R1-1813516	Triggering adaptation of UE power consumption	ASUSTEK COMPUTER (SHANGHAI)

- R1-1813625	Triggering Adaptation of UE Power Consumption Characteristics	Convida Wireless

- R1-1813184	RRM aspects of NR UE power saving	Ericsson

- R1-1813335	Discussion on UE power Consumption Reduction in RRM Measurements	NTT DOCOMO, INC.

- R1-1813449	UE Power Consumption Reduction in RRM Measurements	Qualcomm Incorporated

- R1-1812233	UE power consumption reduction in RRM measurements	Huawei, HiSilicon

- R1-1813862	UE power Consumption Reduction in RRM Measurements	vivo

- R1-1812363	UE power saving for RRM	MediaTek Inc.

- R1-1812423	On UE Power Saving for RRM Measurement	ZTE Corporation

- R1-1812515	On RRM aspects of NR UE power reduction	Intel Corporation

- R1-1812589	Considerations on RRM measurements to reduce UE power consumption	LG Electronics

- R1-1812643	UE Power saving scheme for RRM measurements	CATT

- R1-1812826	UE power Consumption Reduction in RRM Measurements	OPPO

- R1-1812892	Discussion on power saving in intra-band RRM measurements	CMCC

- R1-1813013	UE power consumption reduction in RRM measurement	Samsung

- R1-1813081	Considerations on NR RRM for UE power saving	Spreadtrum Communications

- R1-1813622	UE Power Consumption Reduction in RRM Measurements	Nokia, Nokia Shanghai Bell

- R1-1812333	Power consumption simulation results	vivo

- R1-1812334	Online gaming traffic modelling for UE power saving	vivo

- R1-1812516	On evaluation results for UE Power Saving signals	Intel Corporation

- R1-1812590	Discussion on power saving for DRX operation	LG Electronics

- R1-1812591	Discussion on power saving for CA operation	LG Electronics

- R1-1812592	Discussion on power saving for PDCCH monitoring	LG Electronics

- R1-1812685	UE power saving for paging	Huawei, HiSilicon

- R1-1813185	Network impact of NR UE power saving	Ericsson

- R1-1813186	Detection performance and correlation analysis of PDCCH-based wakeup signal	Ericsson

- R1-1813187	Further details on power consumption modeling	Ericsson

- R1-1813188	Aspects of UE power saving in UL transmission	Ericsson

- R1-1813189	UE power saving techniques for Idle and Inactive states	Ericsson

- R1-1813190	DoU Traffic modeling	Ericsson

- R1-1813684	Calibration results for power saving study	Huawei, HiSilicon

- R1-1813685	Initial evaluation results for power saving	Huawei, HiSilicon

- R1-1813686	Simulation results of C-DRX parameters	Huawei, HiSilicon

- R1-1813687	Initial link-level performances for PDCCH/TRS-based power saving signal	Huawei, HiSilicon