3GPP TSG RAN meeting #82										RP-182424

Sorrento, Italy, December 10-13, 2018

## Status Report to TSG

Agenda item:			9.3.8

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_NR_L1enh_URLLC |  |  |  |  |
| Unique ID | 800095 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-182089 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 03/2019 | Core part: N/A | Performance part: N/A | Testing part: N/A |  |
| Overall Completion level | Study Item: 45% | Core part: N/A | Performance Part: N/A | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | Yan Cheng |
|  | Company | Huawei |
|  | Email | chengyan.cheng@huawei.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

RAN1#94bis meeting

TR 38.824 v0.0.2 (R1-1811909) was endorsed.

##### Remaining details on evaluation methodology

Agreements:

- Take the following table as the table of representative use cases for selection for Rel-16 NR URLLC evaluation.

| Use case (Clause #) | Reliability (%) | Latency (ms) | Data packet size and traffic model | Description |
| --- | --- | --- | --- | --- |
| Power distribution (22.804:5.6.4 &5.6.6) | 99.9999 | 5(end to end latency) Note: 2-3 ms air interface latency | DL & UL: 100 bytes ftp model 3 with arrival interval 100 ms | Power distribution grid fault and outage management |
|  | 99.999 | 15(end to end latency) Note: 6-7 ms air interface latency | DL & UL: 250 bytes Periodic and deterministic with arrival interval 0.833 ms Random offset between UEs | Differential protection |
| Factory automation | 99.9999 | 2(end to end latency) Note: 1 ms air interface latency | DL & UL: 32 bytes Periodic deterministic traffic model with data arrival interval 2 ms | Motion control |
| Rel-15 enabled use case (e.g. AR/VR) | 99.999 | 1ms (air interface delay) for 32 bytes 1 ms and 4 ms (air interface delay) for 200 bytes | DL & UL: 32 and 200 bytes FTP model 3 or periodic with different arrival rates |  |
|  | 99.9 | 7ms (air interface delay) | DL & UL: 4096, 10 K FTP model 3 or periodic with different arrival rates |  |
| Transport Industry (22.186: 5.5) | 99.999 | 5 (end to end latency) Note: 3ms air interface latency | For UL: 2.5 Mpbs; Packet size 5220 bytes For DL: 1Mbps; Packet size 2083 bytes Note: Data arrival rate 60 packets per second for periodic traffic model | Remote driving |
| Transport Industry (23.501, 22.261) | 99.999 | 10(end to end latency) Note: 7ms air interface latency | UL&DL: 1.1 Mbps, Packet size 1370 bytes Note: Data arrival rate 100 packets per second for periodic traffic model | Intelligent transport system (ITS) |

  - Note: The above packet size already includes header overhead.

- FFS whether or not to additionally simulate aperiodic traffic model for factor automation, and if so, details (latency, packet sizes, etc.)

- Note: UL and DL simulation is independent

Agreements:

- For periodic traffic model for factory automation, it is assumed that the data for UEs in a group will arrive simultaneously in the evaluations.

  - Data for UEs in different groups can arrive at different time either in a random manner or in a pre-planned manner

    - Companies report what manner used in the evaluations

  - Companies can report the number of groups and the number of users in each group used in the evaluations

    - The number of users in a group can be one or more, up to companies to report

Agreements:

- Evaluate aperiodic traffic model (FTP model 3) for DL for remote driving and ITS.

  - Companies report the packet size, data arrival rate and data rate

  - Aim to conclude the packet size, data arrival rate and data rate in RAN1#95 meeting

Agreements:

- For evaluation for 4GHz for at least the case of power distribution, adopt the following:

| BS antenna configuration | 4 Tx/4 Rx antenna ports and 8 Tx/8 Rx antenna ports dH = 0.5λ, dV = 0.8λ; Companies report the antenna tilt Note: Other BS antenna configurations for evaluation are not precluded. Detailed antenna configuration parameters are FFS |
| --- | --- |
| UE antenna configuration | 2 Tx/4 Rx antenna ports Panel model 1: Mg=1, Ng=1, P=2, dH=0.5 Note: Other UE antenna configurations for evaluation are not precluded Detailed antenna configuration parameters are FFS |

Note: for other cases, to be further discussed, including the possibility of having each company to report the assumed antenna configurations in the evaluation

Agreements:

- For evaluation for 4GHz for the case of factory automation, adopt the following BS antenna configuration:

| BS antenna configuration | 4 Tx/4 Rx antenna ports and 8 Tx/8 Rx antenna ports Note: Other values are not precluded for evaluation For 4 Tx/4 Rx antenna ports: (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2) For 8 Tx/8 Rx antenna ports: (M, N, P, Mg, Ng; Mp, Np) = (2, 2, 2, 1, 1; 2, 2) |
| --- | --- |

Agreements:

- For evaluation for 4GHz for at least the case of power distribution, adopt the following antenna configuration parameters for BS antenna:

  - For 4 Tx/4 Rx antenna ports: (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1; 1, 2)

  - For 8 Tx/8 Rx antenna ports: (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1; 1, 4)

- For evaluation for 4GHz for at least the case of power distribution, adopt the following antenna configuration parameters for UE antenna:

  - For 4 Rx: (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2)

  - For 2 Tx: (M, N, P, Mg, Ng; Mp, Np) = (1, 1, 2, 1, 1; 1, 1)

Agreements:

- For evaluation for 4GHz for the case of factory automation, reuse the UE antenna configuration for 4 GHz for power distribution.

Agreements:

- Reuse the gNB/UE Tx/Rx antenna ports number for power distribution use case for all urban macro use cases evaluated at 4 GHz.

  - If 16Tx/16Rx antenna ports are additionally used, adopt the following configuration (working assumption): (M, N, P, Mg, Ng; Mp, Np) = (8, 8, 2, 1, 1; 1, 8)

Agreements:

- For URLLC evaluation for 30 GHz for factory automation, adopt the following BS/UE antenna configuration:

| BS antenna configuration | 2 TX/Rx antenna ports (M, N, P, Mg, Ng; Mp, Np)=(4,4,2,1,1; 1,1) dH = dV = 0.5 λ Note: Other antenna configurations are not precluded |
| --- | --- |
| UE antenna configuration | 2 Tx/Rx antenna ports (M, N, P, Mg, Ng; Mp, Np) = (2, 4, 2, 1, 2; 1,1), (dH,dV)=(0.5,0.5)λ Static panel selection Note: Other antenna configurations are not precluded |

- For evaluation for 30 GHz for factory automation, other than the antenna configuration above, reuse the assumptions for 4 GHz with the following modifications:

| Carrier frequency | 30GHz |
| --- | --- |
| BS receiver noise figure | 7dB as defined in TR 38.802 |
| UE antenna gain | 5dBi (Table 2.1-8 in TR38.802) |
| UE receiver noise figure | 10dB |
| BS Tx power | 23dBm for 80MHz bandwidth |
| Channel model | 5GCM office for 30 GHz Companies report the modification of the channel model |
| Simulation bandwidth | 160MHz |
| SCS | 120kHz Note: Other values for evaluation are not precluded. |

Agreements:

- Update UE distribution for power distribution as below:

| Parameters | Value |
| --- | --- |
| UE distribution | 100% of users are outdoors Use 3km/h for modeling fading channel |

- Update the BS antenna height for factory automation as below:

| BS antenna height | 10 m Note: Other value (e.g. 3 m) is not precluded for evaluation |
| --- | --- |

- Update the layout for factory automation as below:

| Layout | Single layer as defined in 38.802 Indoor floor: 12 BSs per 120 m x 50 m |
| --- | --- |

Agreements:

- Reuse the simulation settings for power distribution use case for Rel-15 enabled use case with urban macro (applicable data packet size 32 bytes and 200 bytes) with the following modification:

| Number of UEs per cell | Up to 20 Companies to report the value used in the evaluations Note: Example of the number of users can be 5, 10, 15, 20 |
| --- | --- |
| UE distribution | 80% of users are outdoors and 20% of users are indoors Indoor penetration loss is modelled according to low loss model |

Agreements:

- Reuse the simulation settings for factory automation use case for Rel-15 enabled use case with indoor hotspot with the following modification:

| Number of UEs per cell | Up to 20 Companies to report the value used in the evaluations Note: Example of the number of users can be 5, 10, 15, 20 |
| --- | --- |
| UE distribution | 100% of users are indoor: 3 km/h UE-speed |
| BS antenna height | 3 m |
| Channel model | ITU InH for 4 GHz |

Agreements:

- The number of UEs in the above two agreements is the number of pure URLLC UEs.

- For the case of evaluating multiplexing of eMBB and URLLC UEs sharing the same carrier, companies report the number of eMBB UEs used in the evaluations.

Agreements:

- Update the number of users per cell for factory automation in the table of simulation assumptions for evaluation as below:

| Number of UEs per cell | Up to 40 Note: Example of the number of users can be 5, 10, 20, 30, 40 |
| --- | --- |

Note: The number of UEs in the above table is the number of pure URLLC UEs

Agreements:

- Update the number of users per cell for power distribution in the table of simulation assumptions for evaluation as below:

| Number of UEs per cell | Up to 10 Note: Example of the number of users can be 5, 10 |
| --- | --- |

Note: The number of UEs in the above table is the number of pure URLLC UEs

Note: the number of UEs per cell for all simulated cases are for evaluation purpose only and are not intended to reflect or to meet any requirements

Agreements:

For evaluating multiplexing of eMBB and URLLC UEs sharing the same carrier,

- Take FTP model 3 with 0.5 Mb file size or full buffer as the traffic model for eMBB

- Companies describe eMBB UE dropping

- Evaluate spectral efficiency for eMBB UEs

- Use cases with aperiodic traffics are prioritized for the evaluation of inter-UE multiplexing. Periodic traffic is not precluded for evaluation.

- A certain ratio(s) of UEs that is not capable of the enhanced schemes can be assumed in the evaluation and company should report the ratio(s).

- Performance impact to eMBB and URLLC UEs will be studied for inter-UE multiplexing.

- Evaluating URLLC UEs following the agreed performance metric for URLLC UEs in Rel-16

- eMBB UEs and URLLC UEs have the same subcarrier spacing (for evaluation purpose only)

Agreements:

Companies describes the following assumptions for evaluation:

- Duplex mode: FDD or TDD (DL/UL configuration)

- Blockage due to moving metal parts for channel model for factory automation

- Other assumptions like TTI size, gNB/UE processing time, CSI measurement and reporting

- Detailed assumptions for carrier frequency 700 MHz and 2 GHz if evaluation is performed based on these carrier frequencies

- If any, details on re-dropping or discarding UEs which do not satisfy certain channel quality

Agreements:

- In addition to existing agreements, reuse the simulation settings for power distribution use case for transport industry with the following modification:

| Parameters | Value |
| --- | --- |
| Layout | Single layer - Macro layer: Road configuration in Figure 6.1.9-1 in 38.913 and BS placement as depicted in Figure A.1.3-1 in 36.885. |
| UE antenna height | 3.0 m |

Agreements:

- Take the simulation settings in the following table for link-level simulation for all cases with urban macro:

| Parameter | Value |
| --- | --- |
| Carrier frequency for evaluation | 4GHz |
| Channel model | TDL-C (delay spread: 300ns) as in 38.901 |
| UE speed | 3 km/h for power distribution and Rel-15 enabled use case; 60 km/h for remote driving and ITS; |
| BS antenna configuration | 4 Tx/4 Rx antenna ports and 8 Tx/8 Rx antenna ports Higher BS antenna configurations for evaluation are not precluded |
| UE antenna configuration | 2 Tx/4 Rx antenna ports Higher UE antenna configurations for evaluation are not precluded |
| System bandwidth | 40 MHz |
| Sub-carrier spacing | 30 kHz Note: Other values for evaluation are not precluded. |
| Channel estimation | Practical |
| Receiver type | MMSE |
| Q value (i.e. SINR range) | Companies report the 5% Q value |

  - Evaluation of 700 MHz and 2 GHz carrier frequency are not precluded.

Agreements:

- Take the simulation settings in the following table for link-level simulation applicable for all cases with indoor hot-spot and factory automation for 4 GHz:

| Parameter | Value |
| --- | --- |
| Carrier frequency for evaluation | 4GHz |
| Channel model | TDL-D (delay spread: 30ns) as in 38.901 TDL-C (delay spread: 100ns) as in 38.901 Note: Companies report the modification of the channel model if any |
| UE speed | 3 km/h, 30 km/h |
| BS antenna configuration | 4 Tx/4 Rx antenna ports and 8 Tx/8 Rx antenna ports Higher BS antenna configurations for evaluation are not precluded |
| UE antenna configuration | 2 Tx/4 Rx antenna ports Higher UE antenna configurations for evaluation are not precluded |
| System bandwidth | 40 MHz |
| Sub-carrier spacing | 30 kHz Note: Other values for evaluation are not precluded. |
| Channel estimation | Practical |
| Receiver type | MMSE |
| Q value (i.e. SINR range) | Companies report the 5% Q value |

Agreements:

- Take the simulation settings in the following table for link-level simulation applicable for all cases with indoor hot-spot and factory automation for 30 GHz:

| Parameter | Value |
| --- | --- |
| Carrier frequency for evaluation | 30 GHz |
| Channel model | (working assumption) CDL-A (delay spread: 20 ns) as in 38.901 |
| UE speed | 3 km/h, 30 km/h |
| BS antenna configuration | 2 Tx/2 Rx antenna ports |
| UE antenna configuration | 2 Tx/2 Rx antenna ports |
| System bandwidth | 160 MHz |
| Sub-carrier spacing | 120 kHz Note: Other values for evaluation are not precluded. |
| Channel estimation | Practical |
| Receiver type | MMSE |
| Q value (i.e. SINR range) | Companies report the 5% Q value |

Agreements:

- The number of users per cell for remote driving/ITS is up to 10.

  - Examples for evaluation 2, 6, 10

Agreements:

- For evaluating remote driving and ITS, UE speed of 60 km/h is assumed for urban macro

Conclusion:

- PDCP duplication is not evaluated in RAN1 in this study item.

- PDCP duplication is not always available/applicable.

- Rel-15 higher layer mechanisms PDCP duplication may be applicable for improving reliability.

Agreements:

The performance metric for Rel-16 NR URLLC evaluation is either option 1 or option 2 below:

- Option 1: Percentage of users satisfying reliability and latency requirements

  - Intend for the case with fixed number of UEs and fixed traffic model per UE

- Option 2: URLLC capacity as defined in TR 38.802 with the modification as below:

- URLLC capacity and URLLC / eMBB multiplexing capacity - Definition: URLLC system capacity is calculated as follows: - C(L, R) is the maximum offered cell load under which Y% of URLLC UEs in a cell operate with target link reliability R under L latency bound - X = (100 – Y) % is the percentage of UEs in outage - A UE in outage is defined as the UE cannot meet both latency L and link reliability R bound - Companies report their assumption on X (e.g. 5% or 0%) - Companies report their assumption on the number of eMBB UEs deployed together with the URLLC UEs

  - Intend for the case that the number of UEs and/or the data arrival rate is adjustable

    - Adjusting the number of UEs should be applied to periodic deterministic traffic model

  - The value of X can be revisited in the next meeting

Layer 1 enhancements

Agreements:

- One PUSCH transmission instance is not allowed to cross the slot boundary at least for grant-based PUSCH.

UL inter UE Tx prioritization/multiplexing

Agreements:

- Potential UL power control enhancements are to be studied further:

  - Enhanced dynamic power boost for URLLC UE

    - Dynamic change of power control parameters, e.g. P0, alpha without SRI configured

    - Enhanced TPC, e.g. increased TPC range, finer granularity

    - Currently, the need of URLLC UE power change during one transmission instance is not envisioned

  - Study the Enhanced dynamic power boost for URLLC UE, including at least the following aspects

    - Feasibility of boosting UE power in power limited or interference limited scenarios

    - Physical channel/signal used for the signalling

    - UE Processing timeline for the signalling

    - UE monitoring behaviours for the signalling

    - UE PDCCH monitoring capability, if the signalling is by PDCCH

    - Methods to ensure the reliability of the signalling

    - Type of gNB receiver should be reported

- Note:

  - Other power control enhancements are not precluded.

  - No change of eMBB UE power control scheme is assumed in this study.

##### Enhanced UL grant-free transmissions

Agreements:

- To study further from at least the following:

  - Option 1: multiple active configured grant configurations for a BWP of a serving cell

  - Option 2: repetition(s) across the boundary of a period P

  - Option 3: one transmission cross boundary of a period P

  - FFS the UE behavior when repetitions are collided with the resource which are not available for UL transmissions

  - Note: Switch grant free to grant based retransmission which is available in Rel.15

RAN1#95 meeting

TR 38.824 v0.0.5 (R1-1814036) was endorsed.

TR 38.824 v0.0.6 (R1-1814401) incorporating the agreements achieved in RAN1#95 meeting was endorsed by email approval.

##### Remaining details on evaluation methodology

Agreements:

- Companies report the value of X used in the evaluations (either ~5% or 0%).

  - The value of X should be 0% if re-dropping is used in the evaluations.

Agreements:

- No need to additionally evaluate aperiodic traffic model for factory automation.

Agreements:

- Confirm the working assumption: If 16Tx/16Rx antenna ports are additionally used, adopt the following configuration: (M, N, P, Mg, Ng; Mp, Np) = (8, 8, 2, 1, 1; 1, 8).

Agreements:

- Confirm the working assumption: for indoor hot-spot and factory automation at 30 GHz, the channel model is CDL-A (delay spread: 20 ns) as in 38.901.

Agreements:

Update simulation bandwidth in the tables of simulation assumptions for evaluation as below:

| Simulation bandwidth | 40 MHz NOTE: For FDD, 40 MHz for DL and 40 MHz for UL. Note that this is for evaluation purpose because there is no FDD bands identified at 4 GHz currently. For TDD, 40 MHz for DL/UL. |
| --- | --- |

| Simulation bandwidth | 160MHz NOTE: For TDD, 160 MHz for DL/UL. No FDD bands identified at 30 GHz currently. |
| --- | --- |

Agreements:

Reuse the system level simulation assumptions at 4 GHz for power distribution and transport industry with urban macro scenario for evaluating 700 MHz with the following modifications:

| BS antenna configuration | 2 Tx/2 Rx antenna ports (M, N, P, Mg, Ng; Mp, Np) = (8,1,2,1,1;1,1) (dH, dV) = (N/A, 0.8)λ +45°, -45° polarization Note: 4 Tx/4 Rx as agreed for 4 GHz should be evaluated also |
| --- | --- |
| UE antenna configuration | 2Tx/2 Rx antenna ports (M, N, P, Mg, Ng) = (1,1,2,1,1) 2 Tx/4 Rx antenna ports Panel model 1: Mg=1, Ng=1, P=2, dH=0.5 (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2) for 4 Rx; (M, N, P, Mg, Ng; Mp, Np) = (1, 1, 2, 1, 1; 1, 1) for 2 Tx; 0°, 90° polarization |
| Simulation bandwidth | 10 MHz, 20 MHz Note: 10 MHz for DL and 10 MHz for UL for simulation bandwidth of 10 MHz; 20 MHz for DL and 20 MHz for UL for simulation bandwidth of 20 MHz |
| SCS | 30 KHz Note: Other values for evaluation are not precluded. |

Agreements:

Reuse the link level simulation assumptions at 4 GHz for urban macro for evaluating 700 MHz with the following modifications:

| BS antenna configuration | 2 Tx/2 Rx antenna ports (M, N, P, Mg, Ng; Mp, Np) = (8,1,2,1,1;1,1) (dH, dV) = (N/A, 0.8)λ +45°, -45° polarization |
| --- | --- |
| UE antenna configuration | 2Tx/2 Rx antenna ports (M, N, P, Mg, Ng) = (1,1,2,1,1) 2 Tx/4 Rx antenna ports Panel model 1: Mg=1, Ng=1, P=2, dH=0.5 (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2) for 4 Rx; (M, N, P, Mg, Ng; Mp, Np) = (1, 1, 2, 1, 1; 1, 1) for 2 Tx; 0°, 90° polarization |
| Simulation bandwidth | 10 MHz, 20 MHz Note: 10 MHz for DL and 10 MHz for UL for simulation bandwidth of 10 MHz; 20 MHz for DL and 20 MHz for UL for simulation bandwidth of 20 MHz |
| SCS | 30 KHz Note: Other values for evaluation are not precluded. |

Layer 1 enhancements: Potential enhancements to PDCCH

Agreements:

For link-level PDCCH evaluation, the target operating BLER of DCI(s) scheduling HARQ-less PDSCH/PUSCH should be smaller than 1e-x in Rel-16 NR URLLC, at the 5%-tile SINR geometry.

- x is the reliability requirement given in the table of representative use case for evaluation agreed in the RAN1#94bis meeting.

- The 5%-tile SINR geometry is obtained by system-level simulation assuming full buffer for a given evaluation scenario.

This target assumes no HARQ re-transmssion

Agreements:

- No change of DCI format 0_0/1_0 in CSS from Rel-16 URLLC study item perspective

Agreements:

- To further study DCI for URLLC with a size potentially smaller than that of Rel-15 fallback DCI

  - Consider using Rel-15 fallback DCI as a starting point for Rel-16 URLLC DCI

  - Target a reduction of at least 10-16 bits compared to Rel-15 fallback DCI

    - Companies report how to achieve the DCI size reduction

  - The link level performance gain from PDCCH reliability perspective

    - Check at least AL=16

  - PDCCH resource utilization considering all UEs in the cell

    - Check AL=1/2/4/8/16

    - If retransmission is feasible with the latency bound, different BLER target can be used

  - The PDCCH blocking probability when applicable

  - The performance impact from compact DCI including impact to PDSCH/PUSCH capacity when applicable

  - The impact on PDCCH blind decoding/DCI size budget

  - The impact on PDSCH/PUSCH scheduling flexibility

  - At least Rel-15 enabled use cases should be evaluated for the above study

Layer 1 enhancements: Potential enhancements to UCI

Agreements:

- Multiple PUCCHs for HARQ-ACK within a slot should be supported in R16.

Conclusion:

For supporting multiple PUCCHs for HARQ-ACK within a slot, companies are encouraged to provide following details when proposing a solution:

- How to separate HARQ-ACK multiplexing windows for different PUCCHs?

- How to indicate the starting symbol of different PUCCHs?

- How to indicate K1, e.g. in unit of slot, half-slot, a number of symbols or symbol?

- How to determine dynamic HARQ codebook?

- How to determine semi-static HARQ-ACK codebook?

- How to configure PUCCH resource sets, e.g. reuse R15 PUCCH resource set configurations or not?

- How to determine PUCCH resource for each PUCCH?

- How to do PUCCH resource overriding for HARQ-ACK multiplexing?

- Maximum number of PUCCH transmissions for HARQ-ACK allowed in a slot?

Layer 1 enhancements: Potential enhancements to PUSCH

Agreements:

Support at least one of the following for one TB:

- One UL grant scheduling two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots

- One UL grant scheduling two or more PUSCH repetitions in consecutive available slots, with one repetition in each slot with possibly different starting symbols and/or durations

- N (N>=2) UL grants scheduling N PUSCH repetitions on consecutive available slots, with one repetition in each slot, and the i-th UL grant can be received before the end of the PUSCH transmission scheduled by the (i-1)th UL grant.

- FFS the definition of available slots

Layer 1 enhancements: Potential enhancements to Scheduling/HARQ/CSI Processing timeline

Agreements:

- In order to evaluate the necessity to introduce a new N1/N2 timing capability in Rel. 16 eURLLC, the following aspects should be considered:

  - Perform latency analysis to identify the set of scheduling configuration parameters for which the eURLLC latency requirement(s) can/cannot be satisfied under the NR Rel. 15 timing capabilities.

    - To do this, the worst-case achievable latency should be considered.

  - Perform system-level and/or link-level evaluations to investigate the gains brought by reducing N1/N2 and allowing for more (re-)transmissions within the eURLLC latency budget.

    - For system-level evaluation, the performance metrics agreed for Rel. 16 eURLLC SI are applied.

    - For link-level evaluation, at least the resource efficiency, i.e., the average number of REs used for completing the transmission of a TB, should be reported. The number of transmissions for successfully decoding a TB and the target BLER for each transmission should be reported.

    - For both system-level and link-level evaluations, the simulation parameters agreed for Rel. 16 eURLLC SI are the baseline.

- For all aspects, the comparison reference point is Rel. 15 NR capability timing 2 for FR1 and Rel. 15 NR capability timing 1 for FR2.

- For all aspects, companies should report the assumed values for the following parameters:

  - Alignment latency

  - The considered N1/N2 values

  - SR periodicity in case the first PUSCH Tx is based on a dynamic grant

  - SR reception to initial PUSCH grant processing time at the gNB

  - PDCCH monitoring periodicity

    - The number of BDs/non-overlapping CCEs per monitoring occasion should be reported.

      - For the purpose of this study, the possibility of enhancing the number of non-overlapping CCEs/BDs for NR eURLLC can be considered.

  - Type-B time-domain allocation length for PDSCH/PUSCH channels

  - Time-domain allocation length for PDCCH, SR and PUCCH

  - UE and gNB PDSCH/PUSCH decoding time

  - The HARQ-ACK to reTx PDCCH  and PUSCH to reTx PDCCH processing time at the gNB

  - The maximum number of possible PUCCH transmissions carrying HARQ-ACK per slot

- Companies can report operation constraints (e.g., compact DCI, TB size, #RBs, #layers, #CCs, etc.) needed to enable reducing N1/N2.

- Note: If TDD is assumed, the DL/UL configurations should be reported.

UL inter UE Tx prioritization/multiplexing

Agreements:

- Use cases

  - At least Rel-15 enabled use cases should be assumed for evaluation

    - 1ms air interface delay for 32bytes should be evaluated as the baseline.

    - Others assumptions (e.g. 1 or 4ms for 200bytes) should be considered, if provided.

  - Evaluation of power distribution should be considered, if provided

    - 2ms air interface delay is assumed

| Use case (Clause #) | Reliability (%) | Latency (ms) | Data packet size and traffic model | Description |
| --- | --- | --- | --- | --- |
| Power distribution (22.804:5.6.4 &5.6.6) | 99.9999 | 5(end to end latency) Note: 2-3 ms air interface latency | DL & UL: 100 bytes ftp model 3 with arrival interval 100 ms | Power distribution grid fault and outage management |
| Rel-15 enabled use case (e.g. AR/VR) | 99.999 | 1ms (air interface delay) for 32 bytes 1 ms and 4 ms (air interface delay) for 200 bytes | DL & UL: 32 and 200 bytes FTP model 3 or periodic with different arrival rates |  |

- Traffic model

  - eMBB: company can select between the following options

    - Full buffer, 2 eMBB UEs per cell

    - FTP model 3, 10 eMBB UEs per cell, with medium to high cell load for eMBB traffic.

  - URLLC:

    - For Rel-15 enabled use cases: 10 URLLC UEs per cell

    - For power distribution : 10 URLLC UEs per cell

- Metrics

  - eMBB: Cell throughput for full buffer traffic; UE perceived throughput for FTP model 3 traffic.

  - URLLC:

    - Company shall report whether maximum URLLC capacity has been reached

    - URLLC metrics as previous agreement

      - Option 1: Percentage of users satisfying reliability and latency requirements

      - Option 2: URLLC capacity as defined in TR 38.802 with the modification as below:

- URLLC capacity and URLLC / eMBB multiplexing capacity - Definition: URLLC system capacity is calculated as follows: - C(L, R) is the maximum offered cell load under which Y% of URLLC UEs in a cell operate with target link reliability R under L latency bound - X = (100 – Y) % is the percentage of UEs in outage - A UE in outage is defined as the UE can not meet both latency L and link reliability R bound - Companies report their assumption on X (e.g. 5% or 0%) - Companies report their assumption on the number of eMBB UEs deployed together with the URLLC UEs

- Rel-15 processing timeline capability #2 is used for URLLC UEs

- The following shall be reported

  - Resource utilization

  - Number of packets generated per URLLC user in the simulation

  - Coupling loss CDFs of URLLC and eMBB UEs

  - Percentage of UEs in outage

    - ~5% if re-dropping is not used

    - 0% if re-dropping is used

- Company can optionally report

  - PDCCH overhead, for example the number of cancelation indications in the simulation.

- Detailed modelling shall be described, including at least the following

  - For UL cancelation indication: UE monitoring periodicity, processing timeline, cancelation with or without resuming

  - For power control: exact power control scheme, e.g. semi-static or dynamic power control with details

  - Retransmission modelling

Enhanced UL grant-free transmissions

The following agreements on multiple active configured grant configurations are relevant for RAN2 (FS_NR_IIOT):

Agreements:

- Multiple active configured grant configurations for a given BWP of a serving cell should be supported at least for different services/traffic types and/or for enhancing reliability and reducing latency

  - FFS details

  - Note: it is understood that the above may be related to RAN2-led work on intra-UE multiplexing

- Send LS to RAN2 – R1-1814112, which is endorsed by updating “kindly asks”, and final LS is in R1-1814116

  - RAN1 kindly ask RAN2 to take RAN1 agreements into account in their future work

Agreements:

- One PUSCH transmission instance is not allowed to cross the slot boundary for UL configured grant

Agreements:

- For whether to support explicit HARQ-ACK for configured grant for UL, at least study further gNB’s missed detection performance of the PUSCH under configured grant

  - Study how to resolve gNB’s missed detection if it is an issue

  - Study should take at least following into account:

    - Companies report the false alarm target

    - Companies report the DMRS configuration assumptions

    - The number of UEs sharing the time/frequency-domain grant free resource: 1 is the baseline, larger than 1 can also be considered

The following agreements are relevant for SA2/RAN2/RAN3 and are related to SA2 LS in S2-1811555 (SA2 study FS_5G_URLLC):

The draft LS R1-1814128 “Draft response to LS on redundant transmission for URLLC” is endorsed by add ccing to RAN2 & RAN3. Final LS is approved in R1-1814191. The main content of the LS reply R1-1814191 is as below:

With respect to the third deployment scenario in terms of frequency planning for the network, frequently changing frequency allocation between gNBs in geographic area may result in inter-frequency measurements and frequent inter-frequency handover.

For uniform and dedicated frequency allocation between gNBs and uniform frequency planning in a portion of the network, given that the coverage for different bands or carriers may be different, there may be potential challenge for network planning to guarantee the two RGs operate on different bands or carriers but achieve almost the same coverage.

RAN1 is not able to provide any recommendation on the deployment scenarios to be assumed in the SA study. RAN1 does not have the expertise to assess if solution#10/solution#2 is feasible or not in all deployment scenarios.

#### 2.1.2	Remaining Open issues

- Establish baseline performance achievable with Rel-15 URLLC

- Further investigate Layer 1 enhancements for further improved reliability/latency and for other requirements related to the use cases identified

  - PDCCH enhancements

  - UCI enhancements

  - PUSCH enhancements

  - Enhancements to scheduling/HARQ/CSI processing timeline (UE and gNB)

- Further study UL inter UE Tx prioritization/multiplexing

- Further study enhanced UL grant-free transmission

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

#### 2.4.2	Remaining Open issues

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN#81

- RP-181677       “Status report for SI Study on physical layer enhancements for NR ultra-reliable and low latency communication (URLLC)”, 3GPP TSG RAN#81 meeting, September 2018.

RAN1#94bis

- “Draft_Minutes_report_RAN1#94b_v010”, 3GPP TSG RAN WG1 Meeting 94bis, October 2018.

- R1-1811105	TR 38.824 v0.0.1 for Study on physical layer enhancements for NR ultra-reliable low latency communication (URLLC)	Huawei

- R1-1811909	TR 38.824 v0.0.2 for Study on physical layer enhancements for NR ultra-reliable low latency communication (URLLC)	Huawei

- R1-1810173	Evaluation Methodology and Simulation Assumptions for Rel-16 NR URLLC	Ericsson

- R1-1810784	Remaining details on evaluation methodologies for eURLLC	Intel Corporation

- R1-1811949	Summary of 7.2.6.1 Remaining details on evaluation methodology	Huawei, HiSilicon

- R1-1812006	Summary of 7.2.6.1 Remaining details on evaluation methodology	Huawei, HiSilicon

- R1-1811980	[Draft] LS on remote driving traffic model	Huawei

- R1-1812039	Summary of 7.2.6.1 Remaining details on evaluation methodology	Huawei, HiSilicon

- R1-1812069	Summary of 7.2.6.1 Remaining details on evaluation methodology	Huawei, HiSilicon

- R1-1810224	Summary of email discussion [94-NR-06] on additional simulation assumptions for Rel-16 NR URLLC	Huawei

- R1-1810293	Remaining issues on evaluation methodology for NR URLLC	LG Electronics

- R1-1811637	Discussion on evaluation methodology	ZTE	(Revision of R1-1810344)

- R1-1810462	Remaining details on URLLC evaluation methodology	MediaTek Inc.

- R1-1810470	Remaining details on URLLC Evaluation Methodology    	Idaho National Laboratory

- R1-1810550	On evaluation methodology for URLLC	CATT

- R1-1810659	Field Measurement Results from an Operational Factory Floor at 3.5 GHz and 28 GHz	Nokia, Nokia Shanghai Bell

- R1-1810716	Evaluation assumptions and methodology for Rel-16 NR URLLC	Huawei,China Southern Power Grid, HiSilicon

- R1-1810878	Remaining details on evaluation methodology for eURLLC	Samsung

- R1-1811377	Remaining details on evaluation methodology	NTT DOCOMO, INC.

- R1-1810294	Discussion on layer 1 enhancements for URLLC	LG Electronics

- R1-1810463	Evaluations and enhancements of NR control channels for URLLC	MediaTek Inc.

- R1-1811888	Summary of 7.2.6.2 Layer 1 enhancements	Huawei, HiSilicon

- R1-1812070	Summary of 7.2.6.2 Layer 1 enhancements	Huawei, HiSilicon

- R1-1810157	L1 enhancements for URLLC	Huawei, HiSilicon

- R1-1810174	Enhancements to Uplink and Downlink Physical Channels for NR URLLC	Ericsson

- R1-1810209	Achieving ultra-low latency in SR-based URLLC UL transmission with minimum overhead	Idaho National Laboratory

- R1-1810345	On URLLC Layer 1 enhancements	ZTE

- R1-1810395	Layer 1 enhancements for URLLC	vivo

- R1-1810474	On NR URLLC L1 enhancements 	Panasonic

- R1-1810486	Scheduling/HARQ Processing Timeline Enhancements for NR URLLC	Fraunhofer HHI

- R1-1810551	On PHY enhancements for Rel-16 URLLC	CATT

- R1-1810596	Discussion on HARQ-ACK feedback over PUCCH and PUSCH	Fujitsu

- R1-1810639	L1 enhancement for URLLC	Sony

- R1-1810660	On the L1 enhancements for NR URLLC	Nokia, Nokia Shanghai Bell

- R1-1810685	Overview of Physical Layer Enhancements for eURLLC	AT&T

- R1-1810785	Layer 1 enhancements for eURLLC	Intel Corporation

- R1-1810879	Layer 1 enhancement for URLLC	Samsung

- R1-1810949	Discussion on L1 enhancements for URLLC	ETRI

- R1-1810991	Layer 1 enhancement for URLLC	Guangdong OPPO Mobile Telecom.

- R1-1811020	Consideration on URLLC physical layer enhancement	Spreadtrum Communications

- R1-1811044	Discussion on out-of-order scheduling and HARQ	CMCC, China Southern Power Grid

- R1-1811107	On PDCCH enhancements for URLLC	Mitsubishi Electric RCE

- R1-1811218	On potential physical layer enhancements for URLLC 	InterDigital, Inc.

- R1-1811272	Layer 1 enhancements for eURLLC	Qualcomm Incorporated

- R1-1811289	Discussion on PUCCH Resource Allocation and HARQ Codebook Determination in URLLC	China Telecommunications

- R1-1811294	Discussion on L1 enhancements for URLLC	Motorola Mobility, Lenovo

- R1-1811378	Layer 1 enhancements for NR URLLC	NTT DOCOMO, INC.

- R1-1811391	HARQ-ACK feedback enhancement for URLLC	III

- R1-1811396	Consideration on URLLC physical layer enhancement	Spreadtrum Communications

- R1-1811401	Discussion on the PDCCH repetition for NR URLLC	Xiaomi

- R1-1811461	L1 enhancements for URLLC	Sequans Communications

- R1-1811522	L1 enhancements to support URLLC	CAICT

- R1-1811613	On VRB-to-PRB interleaving for URLLC	KT Corp.

- R1-1811892	UL inter-UE transmission prioritization and multiplexing	Huawei, HiSilicon

- R1-1811273	UL inter UE Tx prioritization-multiplexing	Qualcomm Incorporated

- R1-1811905	Summary of UL inter UE TX prioritization/multiplexing	vivo

- R1-1810175	Inter-UE Prioritization and Multiplexing of  UL Transmissions	Ericsson

- R1-1810295	Discussion on UL inter UE Tx prioritization	LG Electronics

- R1-1810346	On UL inter UE Tx prioritization/multiplexing	ZTE

- R1-1810396	UL inter UE Tx prioritization for URLLC	vivo

- R1-1810464	Feasibility analysis and SLS results for UL multiplexing between eMBB and URLLC	MediaTek Inc.

- R1-1810475	On NR URLLC UL inter UE Tx prioritization/multiplexing	Panasonic

- R1-1810552	Discussion on inter-UE UL multiplexing	CATT

- R1-1810597	Discussion on UL preemption indication	Fujitsu

- R1-1810640	Considerations in UL Inter UE Pre-emption	Sony

- R1-1810658	UL Pre-emption and multiplexing of eMBB and URLLC	NEC

- R1-1810661	Solution for UL inter-UE multiplexing between eMBB and URLLC	Nokia, Nokia Shanghai Bell

- R1-1810786	On inter-UE prioritization/multiplexing in the UL	Intel Corporation

- R1-1810880	Uplink inter UE multiplexing/prioritization for enhanced URLLC	Samsung

- R1-1810935	UL inter-UE multiplexing between eMBB and URLLC	China Telecommunications

- R1-1810992	Consideration on UL inter UE Tx prioritization and multiplexing	Guangdong OPPO Mobile Telecom.

- R1-1811019	Discussion on UL inter UE Tx prioritization/multiplexing	Spreadtrum Communications

- R1-1811043	Discussion on UL inter UE Tx prioritization/multiplexing	CMCC

- R1-1811087	Discussion on power control mechanism for UL inter UE Tx multiplexing	ASUSTEK COMPUTER (SHANGHAI)

- R1-1811094	Discussion on UL inter UE Tx multiplexing	China Unicom

- R1-1811155	Multiplexing among PUSCHs with different latency and reliability requirements	Sharp

- R1-1811219	On UL inter-UE multiplexing for URLLC	InterDigital, Inc.

- R1-1811379	Uplink transmission prioritization/multiplexing for NR URLLC	NTT DOCOMO, INC.

- R1-1811445	Discussion on UL inter UE Tx prioritization/multiplexing	Potevio

- R1-1811447	Discussion on UL Inter UE Tx prioritization/multiplexing	III

- R1-1811462	Considerations on UL inter-UE multiplexing for URLLC	Sequans Communications

- R1-1811482	Discussions on UL inter UE multiplexing	Mitsubishi Electric Co.

- R1-1811575	Views on UL inter UE Tx prioritization/multiplexing	KT Corp.

- R1-1810881	Potential enhancement for UL grant-free transmission	Samsung

- R1-1811380	Enhanced UL transmission with configured grant for URLLC	NTT DOCOMO, INC.

- R1-1812055	Enhanced UL grant-free transmissions for URLLC	NTT DOCOMO

- R1-1811822	Summary of 7.2.6.4 Enhanced UL grant-free transmissions	NTT DOCOMO

- R1-1810159	Enhanced UL configured grant transmissions	Huawei, HiSilicon

- R1-1810176	Enhancements to UL Grant-free Transmission for NR URLLC	Ericsson

- R1-1810296	Discussion on enhancement for grant-free transmission	LG Electronics

- R1-1810347	Enhancement for UL grant-free transmissions	ZTE

- R1-1810397	Enhanced UL grant-free transmissions  for URLLC	vivo

- R1-1810465	Study and evaluation of configured-grant enhancements for URLLC	MediaTek Inc.

- R1-1810553	On enhancements to configured UL grant operation	CATT

- R1-1810641	Discussion on enhanced UL grant-free transmissions	Sony

- R1-1810662	On Configured Grant enhancements for NR URLLC	Nokia, Nokia Shanghai Bell

- R1-1810731	URLLC enhanced grant-free transmission	Panasonic Corporation

- R1-1810787	Enhancements to configured grant PUSCH	Intel Corporation

- R1-1810993	Grant free transmission enhancement	Guangdong OPPO Mobile Telecom.

- R1-1811006	Discussion on UL grant-free transmission enhancement	Spreadtrum Communications

- R1-1811156	Reliability enhancements for UL configured grant	Sharp

- R1-1811220	Enhanced UL configured grant transmissions for URLLC 	InterDigital, Inc.

- R1-1811274	Enhanced SPS and grant-free transmissions	Qualcomm Incorporated

- R1-1811463	Enhancements for grant free transmissions	Sequans Communications

- R1-1811511	Discussion on enhanced UL grant-free transmissions	KDDI Corporation

- R1-1811523	Enhanced UL grant-free transmissions-	CAICT

- R1-1811552	Discussion on Configured Grant Enhancements	III

- R1-1810136	HARQ-ACK enhancements for URLLC	Huawei, HiSilicon

- R1-1810177	Latency Evaluation of Rel-15 URLLC	Ericsson

- R1-1810178	Reliability Evaluation of Rel-15 Enabled URLLC Use Cases	Ericsson

- R1-1810297	Discussion on differentiation of eMBB and URLLC services	LG Electronics

- R1-1810299	Discussions on PUCCH performance for NR URLLC	LG Electronics

- R1-1810300	Discussion on HARQ-ACK enhancement for URLLC	LG Electronics

- R1-1810317	Intra-UE Prioritization and Multiplexing of  UL Transmissions	Ericsson

- R1-1810318	Triggered A-CSI on PUCCH	Ericsson

- R1-1810348	Further issues related to URLLC enhancement	ZTE

- R1-1810349	PUSCH and PUCCH time line for URLLC	ZTE

- R1-1810398	Other enhancements for URLLC	vivo

- R1-1810399	UL intra UE Tx prioritization for URLLC	vivo

- R1-1810400	DL intra UE Tx prioritization for URLLC	vivo

- R1-1810466	Further enhancements of NR PUCCH for URLLC	MediaTek Inc.

- R1-1810642	Multiple active BWPs	Sony

- R1-1810663	URLLC downlink system-level performance evaluation	Nokia, Nokia Shanghai Bell

- R1-1810664	On compact DCI for NR URLLC	Nokia, Nokia Shanghai Bell

- R1-1810665	On PDCCH repetition for NR URLLC	Nokia, Nokia Shanghai Bell

- R1-1810666	On the PDCCH limits on the maximum number of CCEs for channel estimation and BDs for URLLC	Nokia, Nokia Shanghai Bell

- R1-1810667	On enhanced HARQ-ACK feedback for URLLC	Nokia, Nokia Shanghai Bell

- R1-1810668	On CSI feedback enhancements for URLLC	Nokia, Nokia Shanghai Bell

- R1-1810669	On scheduled PUSCH (& PDSCH) repetition enhancements for NR URLLC	Nokia, Nokia Shanghai Bell

- R1-1810670	Enhancements to scheduling/HARQ/CSI processing timeline for NR URLLC	Nokia, Nokia Shanghai Bell

- R1-1810671	Work Plan for NR Industrial IoT SI	Nokia

- R1-1810672	Draft skeleton of TR 38.825: Study on NR Industrial Internet of Things (IoT)	Nokia

- R1-1810709	PDCCH enhancements for URLLC	Huawei, HiSilicon

- R1-1810717	Simulation results on the baseline performance achievable with Rel-15 URLLC for the identified use cases	Huawei, HiSilicon

- R1-1810718	DL/UL intra-UE transmission prioritization and multiplexing	Huawei, HiSilicon

- R1-1810720	CSI enhancements for URLLC	Huawei, HiSilicon

- R1-1810974	Resource allocation enhancements for NR URLLC	Guangdong OPPO Mobile Telecom.

- R1-1810975	On eMBB and URLLC data differentiation	Guangdong OPPO Mobile Telecom.

- R1-1810994	Consideration on UL intra UE Tx prioritization and multiplexing	Guangdong OPPO Mobile Telecom.

- R1-1811108	Reliability Evaluation of URLLC Factory Automation Scenario	Ericsson

- R1-1811109	Performance Evaluation of PDCCH for URLLC	Ericsson

- R1-1811110	Channel Model for Factory Automation Scenario	Ericsson

- R1-1811111	Uplink Power Control Enhancement for NR URLLC	Ericsson

- R1-1811112	Alignment Delay Study For URLLC Latency	Ericsson

- R1-1811113	HARQ-ACK feedback for URLLC	InterDigital, Inc.

- R1-1811114	URLLC UCI prioritization	InterDigital, Inc.

- R1-1811221	On downlink data reception with configured scheduling 	InterDigital, Inc.

- R1-1811275	eURLLC Considerations for Factory Automation	Qualcomm Incorporated

- R1-1811468	PUSCH enhancements for URLLC	Huawei, HiSilicon

- R1-1811469	Enhancements to scheduling/HARQ/CSI processing timeline for URLLC	Huawei, HiSilicon

- R1-1811617	Transmit Diversity for Rel-16 URLLC	Ericsson Inc.

RAN1#95

- “Draft_Minutes_report_RAN1#95_v010”, 3GPP TSG RAN WG1 Meeting 95, November 2018.

- R1-1814025	TR 38.824 v0.0.4 Study on physical layer enhancements for NR ultra-reliable low latency communication (URLLC)	Huawei, HiSilicon

- R1-1813480	Text proposal on capturing specific use case and requirements for Rel-16 NR URLLC in TR 38.824		Huawei, HiSilicon

- R1-1813891	Preliminary Uplink URLLC Latency Results	Idaho National Laboratory	(rev of R1-1812177)

- R1-1812572	PDCCH enhancements for NR URLLC	LG Electronics

- R1-1813987	PDCCH enhancements for URLLC	Huawei, HiSilicon

- R1-1814023	Summary of 7.2.6.1.1 Potential enhancements to PDCCH	Huawei, HiSilicon

- R1-1814178	Summary of 7.2.6.1.1 Potential enhancements to PDCCH	Huawei

- R1-1812153	PDCCH Enhancements for NR URLLC	Ericsson

- R1-1812312	PDCCH enhancements for URLLC	vivo

- R1-1812374	Evaluations and enhancements of NR PDCCH for URLLC	MediaTek Inc.

- R1-1813905	On PDCCH enhancements for URLLC	ZTE	(rev of R1-1812384)

- R1-1812424	On PDCCH enhancements for URLLC	Mitsubishi Electric RCE

- R1-1813875	On PDCCH enhancements for eURLLC	Intel Corporation	(rev of R1-1812501)

- R1-1812628	DL control enhancements for URLLC	CATT

- R1-1812742	L1 enhancement on PDCCH for URLLC	Sony

- R1-1812815	PDCCH enhancement for URLLC	OPPO

- R1-1812853	On Potential enhancements to PDDCH for URLLC	AT&T

- R1-1812899	PDCCH enhancements for NR URLLC	Panasonic Corporation

- R1-1812994	Potential enhanements for PDCCH	Samsung

- R1-1813064	Discussion on PDCCH enhancement on URLLC	Spreadtrum Communications

- R1-1813088	PDCCH enhancements for URLLC	China Telecommunications

- R1-1813097	On PDCCH enhancements for eURLLC	ETRI

- R1-1813113	On the PDCCH enhancements for NR URLLC	Nokia, Nokia Shanghai Bell

- R1-1813234	Potential PDCCH enhancements for URLLC 	InterDigital, Inc.

- R1-1813324	PDCCH enhancements for URLLC	NTT DOCOMO, INC.

- R1-1813353	Potential PDCCH enhancements for URLLC operation	Motorola Mobility, Lenovo

- R1-1813433	PDCCH enhancements for eURLLC	Qualcomm Incorporated

- R1-1813506	Enhancements to PDCCH to support URLLC	CAICT

- R1-1813538	PDCCH enhancements for URLLC	Sequans Communications

- R1-1812816	UCI enhancement for URLLC	OPPO

- R1-1813434	UCI enhancements for eURLLC	Qualcomm Incorporated

- R1-1814044	Offline Summary on UCI enhancements for URLLC	OPPO

- R1-1812154	UCI Enhancements for NR URLLC	Ericsson

- R1-1813986	UCI enhancements for URLLC	Huawei, HiSilicon

- R1-1812313	UCI enhancements for URLLC	vivo

- R1-1812375	Evaluations and enhancements of NR PUCCH	MediaTek Inc.

- R1-1812385	UL control enhancements for URLLC	ZTE

- R1-1812414	Discussion on HARQ-ACK feedback over PUCCH and PUSCH	Fujitsu

- R1-1812502	On UCI enhancements for eURLLC	Intel Corporation

- R1-1812573	UCI enhancements for NR URLLC	LG Electronics

- R1-1812629	UL control enhancements for URLLC	CATT

- R1-1812743	L1 enhancement on UCI for URLLC	Sony

- R1-1812796	Discussion on UCI and PUCCH enhancement for URLLC	Panasonic Corporation

- R1-1812854	UCI Enhancements  for NR	AT&T

- R1-1812995	Potential enhanements for UCI	Samsung

- R1-1813062	Discussion on UCI enhancement on URLLC	Spreadtrum Communications

- R1-1813114	On UCI Enhancements for URLLC	Nokia, Nokia Shanghai Bell

- R1-1813144	Enhanced HARQ-ACK for NR URLLC	Fraunhofer HHI

- R1-1813163	Potential UCI enhancements for URLLC	InterDigital, Inc.

- R1-1813325	UCI enhancements for URLLC	NTT DOCOMO, INC.

- R1-1813507	Enhancements to UCI to support URLLC	CAICT

- R1-1813539	UCI enhancements for URLLC	Sequans Communications

- R1-1813326	PUSCH enhancements for URLLC	NTT DOCOMO, INC.

- R1-1813115	On scheduled PUSCH (& PDSCH) repetition enhancements for NR URLLC	Nokia, Nokia Shanghai Bell

- R1-1814098	Summary of 7.2.6.1.3 potential enhancements for PUSCH for NR URLLC	Nokia, Nokia Shanghai Bell

- R1-1814326	Summary of offline discussion on the potential enhancements for PUSCH for NR URLLC	Nokia, Nokia Shanghai Bell

- R1-1812315	Enhancement for Scheduling/HARQ/CSI Processing timeline	vivo

- R1-1812377	Study and evaluation of scheduling, HARQ and CSI processing timeline	MediaTek Inc.

- R1-1814094	Summary for Potential Enhancements to Scheduling/HARQ/CSI Processing Timeline	Qualcomm

- R1-1814173	Summary for Potential Enhancements to Scheduling/HARQ/CSI Processing Timeline	Qualcomm

- R1-1814283	Summary for Potential Enhancements to Scheduling/HARQ/CSI Processing Timeline	Qualcomm Incorporated

- R1-1812156	Scheduling/HARQ/CSI Processing Timeline Enhancements for NR URLLC	Ericsson

- R1-1812224	Enhancements to scheduling/HARQ/CSI processing timeline for URLLC	Huawei, HiSilicon

- R1-1812387	DL/UL scheduling and HARQ for URLLC	ZTE

- R1-1812504	On enhancements to UE processing times for eURLLC	Intel Corporation

- R1-1812575	Scheduling/HARQ processing timeline enhancements for NR URLLC	LG Electronics

- R1-1812631	Scheduling enhancements for URLLC	CATT

- R1-1812818	Schedule and HARQ enhancement	OPPO

- R1-1812856	HARQ Enhancements to URLLC	AT&T

- R1-1812885	Discussion on out-of-order scheduling and HARQ	CMCC, China Southern Power Grid

- R1-1812997	Potential enhancements for scheduling/HARQ/CSI Processing timeline	Samsung

- R1-1813116	Enhancements to scheduling/HARQ/CSI processing timeline for NR URLLC	Nokia, Nokia Shanghai Bell

- R1-1813146	Scheduling/HARQ Processing Timeline Enhancements for NR URLLC	Fraunhofer HHI

- R1-1813284	Discussion on flexible HARQ scheduling for URLLC	III

- R1-1813327	Enhancements to Scheduling/HARQ/CSI Processing timeline for URLLC	NTT DOCOMO, INC.

- R1-1813343	Enhancement of processing timeline for intra-UE prioritization and multiplexing	ITRI

- R1-1813355	HARQ enhancement for URLLC	Motorola Mobility, Lenovo

- R1-1813436	Scheduling/HARQ/CSI Processing Timeline for eURLLC	Qualcomm Incorporated

- R1-1813540	HARQ enhancements for URLLC	Sequans Communications

- R1-1813577	On Physical Layer Enhancements for NR URLLC	CEWiT

- R1-1812157	Intra-UE Prioritization and Multiplexing of  UL Transmissions	Ericsson

- R1-1812159	Transmit Diversity for Rel-16 URLLC	Ericsson

- R1-1812160	Uplink Power Control Enhancement for NR URLLC	Ericsson

- R1-1812316	CSI measurement for URLLC	vivo

- R1-1812317	Power control for URLLC	vivo

- R1-1812391	Further issues related to URLLC enhancement	ZTE

- R1-1812392	Issues on URLLC resource allocation	ZTE

- R1-1812576	HARQ-ACK enhancements with BWP switching for NR URLLC	LG Electronics

- R1-1812832	Resource allocation enhancements for NR URLLC	OPPO

- R1-1812833	On eMBB and URLLC data differentiation	OPPO

- R1-1813556	Enhanced CSI measurement and reporting for URLLC	Huawei, HiSilicon

- R1-1813669	Compact DCI  for URLLC	Huawei, HiSilicon

- R1-1813670	PDCCH repetition for URLLC	Huawei, HiSilicon

- R1-1812161	Inter-UE Prioritization and Multiplexing of  UL Transmissions	Ericsson

- R1-1813328	UL inter UE transmission multiplexing	NTT DOCOMO, INC.

- R1-1814031	Summary of UL inter UE Tx prioritization/multiplexing	vivo

- R1-1814261	Summary#2 of UL inter UE Tx prioritization/multiplexing	vivo

- R1-1813926	UL inter-UE transmission prioritization and multiplexing	Huawei, HiSilicon	(rev of R1-1812225)

- R1-1812318	UL inter UE Tx prioritization for URLLC	vivo

- R1-1812378	Power control enhancements for dynamic UL multiplexing between URLLC and eMBB	MediaTek Inc.

- R1-1812389	On UL inter UE Tx prioritization/multiplexing	ZTE

- R1-1812415	Discussion on UL preemption indication	Fujitsu

- R1-1812419	UL inter-UE multiplexing of eMBB and URLLC	NEC

- R1-1812505	On enhanced inter-UE multiplexing for eURLLC	Intel Corporation

- R1-1812577	Discussion on UL inter UE Tx prioritization	LG Electronics

- R1-1812632	Discussion on inter-UE UL multiplexing	CATT

- R1-1812745	Inter-UE uplink multiplexing of URLLC & eMBB traffics	Sony

- R1-1812819	Consideration on UL inter UE Tx prioritization and multiplexing	OPPO

- R1-1812886	Discussion on UL inter UE Tx prioritization/multiplexing	CMCC

- R1-1812998	Uplink inter UE multiplexing/prioritization for enhanced URLLC	Samsung

- R1-1813063	Discussion on UL inter UE Tx prioritizationmultiplexing	Spreadtrum Communications

- R1-1813089	UL inter-UE multiplexing between eMBB and URLLC	China Telecommunications

- R1-1813098	UL inter UE Tx prioritization/multiplexing	ETRI

- R1-1813117	Solutions for UL inter-UE multiplexing between eMBB and URLLC	Nokia, Nokia Shanghai Bell

- R1-1813134	On NR URLLC UL inter UE Tx prioritization/multiplexing	Panasonic

- R1-1813236	Potential enhancements for UL inter-UE multiplexing 	InterDigital, Inc.

- R1-1813385	Views on pre-emption for UL inter UE Tx multiplexing	Mitsubishi Electric Co.

- R1-1813388	Views on inter-UE Prioritization and Multiplexing of UL Transmission	China Unicom

- R1-1813929	UL inter UE Tx prioritization-multiplexing	Qualcomm Incorporated	(rev of R1-1813437)

- R1-1813513	Discussion on power control mechanism for UL inter UE Tx multiplexing	ASUSTEK COMPUTER (SHANGHAI)

- R1-1813542	Considerations on UL inter-UE multiplexing for URLLC	Sequans Communications

- R1-1812797	On NR URLLC enhancements for grant-free transmission	Panasonic Corporation

- R1-1813884	Enhancement for UL grant-free transmissions	ZTE

- R1-1813936	Summary of 7.2.6.3 Enhanced UL grant-free transmissions	NTT DOCOMO

- R1-1814112	[DRAFT] LS on multiple active configured grant configurations	NTT DOCOMO

- R1-1814318	Offline discussions for 7.2.6.3 Enhanced UL grant-free transmissions	NTT DOCOMO

- R1-1812162	Enhancement of Configured Grant for NR URLLC	Ericsson

- R1-1812226	Enhanced UL configured grant transmissions	Huawei, HiSilicon

- R1-1812319	Enhanced UL grant-free transmissions  for URLLC	vivo

- R1-1812379	Study and evaluation of configured-grant enhancements for URLLC	MediaTek Inc.

- R1-1812506	On enhanced Configured Grant PUSCH for eURLLC	Intel Corporation

- R1-1812578	Discussion on enhancement for grant-free transmission	LG Electronics

- R1-1812633	On enhancements to configured UL grant operation	CATT

- R1-1812746	Discussion on enhanced UL grant-free transmissions	Sony

- R1-1812820	Grant free transmission enhancement	OPPO

- R1-1812999	Potential enhancement for UL grant-free transmission	Samsung

- R1-1813069	Discussion on UL grant-free transmission enhancements	Spreadtrum Communications

- R1-1813118	On Configured Grant enhancements for NR URLLC	Nokia, Nokia Shanghai Bell

- R1-1813137	On Uplink Grant Free Transmission for Rel-16 NR	Dish Network

- R1-1813237	Potential enhancements for UL configured grant 	InterDigital, Inc.

- R1-1813329	Enhanced UL transmission with configured grant for URLLC	NTT DOCOMO, INC.

- R1-1813356	Enhanced UL grant-free transmissions for URLLC operation	Motorola Mobility, Lenovo

- R1-1813438	Enhanced SPS and grant-free transmissions	Qualcomm Incorporated

- R1-1813509	Enhanced UL grant-free transmissions to support URLLC	CAICT

- R1-1813528	Discussion on Configured Grant Enhancements	III

- R1-1813537	Discussion on enhanced UL grant-free transmissions	KDDI Corporation

- R1-1813543	Enhancements for grant free transmissions	Sequans Communications

- R1-1813709	Support for Enhanced UL Grant Free Transmission	Fraunhofer IIS

- R1-1814024	Summary of remaining details on evaluation assumptions and evaluation results	Huawei, HiSilicon

- R1-1814285	Summary of remaining details on evaluation assumptions and evaluation results	Huawei

- R1-1812158	Initial Transmission Opportunity in UL Configured Grant	Ericsson

- R1-1812163	Remaining Details on Evaluation Methodology and Simulation Assumptions for Rel-16 NR URLLC		Ericsson

- R1-1813969	Channel Model for Factory Automation Scenario	Ericsson

- R1-1813970	Evaluation of URLLC Power Distribution Scenario	Ericsson

- R1-1812166	Evaluation of URLLC Factory Automation Scenario	Ericsson

- R1-1813971	Evaluation of URLLC Transport Industry Scenario	Ericsson

- R1-1812168	Evaluation of Rel-15 Enabled URLLC Use Cases	Ericsson

- R1-1812169	Latency Evaluation of Rel-15 URLLC	Ericsson

- R1-1813972	Alignment Delay Study for URLLC Latency	Ericsson

- R1-1812219	Remaining evaluation assumptions and methodology for Rel-16 NR URLLC	Huawei, China Southern Power Grid, HiSilicon

- R1-1812220	Simulation results on the baseline performance achievable with Rel-15 URLLC for the identified use cases	Huawei, HiSilicon

- R1-1812320	UL intra UE Tx prioritization for URLLC	vivo

- R1-1812321	DL intra UE Tx prioritization for URLLC	vivo

- R1-1812388	Remaining issues on URLLC evaluation	ZTE

- R1-1812579	Preliminary evaluation results for NR URLLC	LG Electronics

- Late submission

- R1-1812681	UL intra-UE transmission prioritization and multiplexing	Huawei, HiSilicon

- R1-1812689	Discussion on differentiation of eMBB and URLLC services	Huawei, HiSilicon

- R1-1812690	DL intra-UE transmission prioritization and multiplexing	Huawei, HiSilicon

- R1-1812821	Consideration on UL intra UE Tx prioritization and multiplexing	OPPO

- R1-1813850	Use cases and simulation assumptions on 700MHz	ZTE

- R1-1813851	Geometry calibration and preliminary results of URLLC evaluation	ZTE	(rev of R1-1812933)

- R1-1813000	Remaining details for evaluation assumption	Samsung

- R1-1813119	URLLC downlink system-level performance evaluation for factory automation	Nokia, Nokia Shanghai Bell

- R1-1813175	URLLC Use Cases and Deployments for Beyond 52.6 GHz	Ericsson Inc.

- R1-1813238	Preliminary performance evaluations for AR-VR	InterDigital, Inc.

- R1-1813330	URLLC system-level performance evaluation	NTT DOCOMO, INC.

- R1-1813439	eURLLC Considerations for Factory Automation	Qualcomm Incorporated

- R1-1813440	Simulation Results for Factory Automation	Qualcomm Incorporated

- R1-1813536	Discussion on UL intra UE Tx prioritization and multiplexing	WILUS Inc.

- R1-1813557	Scheduling reliable retransmission	Huawei, HiSilicon

- R1-1813664	Intra-UE UCI multiplexing	Huawei, HiSilicon

- R1-1813682	Baseline performance achievable with Rel-15 URLLC for power distribution	Huawei, HiSilicon

- R1-1813683	Baseline performance achievable with Rel-15 URLLC for factory automation	Huawei, HiSilicon

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template