3GPP TSG RAN Meeting #82					RP-182468

Sorrento, Italy, December 10-13, 2018

Source:	vivo

Title:	Discussion on UL multi-panel transmission

Agenda Item:	9.4.1

Document for:	Discussion and Decision

- Introduction

In RAN#81, a revised WID [2] for NR MIMO was agreed and one of the objectives is as follows:

  - Enhancements on multi-TRP/panel transmission including improved reliability and robustness with both ideal and non-ideal backhaul:

    - Specify downlink control signalling enhancement(s) for efficient support of non-coherent joint transmission

    - Perform study and, if needed, specify enhancements on uplink control signalling and/or reference signal(s) for non-coherent joint transmission

    - Multi-TRP techniques for URLLC requirements are included in this WI

While the WID was being drafted, it was intensively discussed UL multi-panel transmission especially for URLLC enhancement.

- Discussion

In RAN1#95, there was debate on whether simultaneous multi-panel transmission under multi-beam operation is within the scope of WID or not. It is clearly mentioned in the WID that:

    - Specify UL transmit beam selection for multi-panel operation that facilitates panel-specific beam selection

The reason for panel-specific beam selection is that for the UEs equipped with multiple antenna panels either panel selection or multi-panel transmission is possible. From the scope of multi-TRP/panel transmission and enhancement on multi-beam operation, it is hard to delineate a clear boundary in the case of UL multi-panel transmission. UL multi-panel transmission is very crucial to meet the URLLC requirements in UL especially in non-ideal backhaul scenario. In general, enhancement of multi-TRP transmission for meeting URLLC requirement in DL only doesn’t make sense if URLLC requirement in UL is not met. Therefore in our view, simultaneous multi-panel transmission in UL should be considered either under multi-TRP/panel transmission objective or multi-beam operation objective. Further clarification on simultaneous multi-panel transmission in UL can be considered in WID if deemed necessary.

- Conclusion

In this contribution, we discussed the use case of simultaneous multi-panel transmission, from WID scope point of view at least for URLLC in UL should support simultaneous multi-panel transmission.

- References

- RP-181453, WID: Enhancements on MIMO for NR, RAN#80

- RP-182067, Revised WID: Enhancements on MIMO for NR, RAN#81