3GPP TSG RAN meeting #82														RP-182502

Sorrento, Italy, 10-13, Dec. 2018

Source:	ZTE Corporation

Title:	Consideration on EN-DC and NR CA/DC combination request template

Agenda Item:	9.5.1

Document for:	Approval

# 1	Introduction

In the RAN4 #89 meeting, the template for the introduction of band combinations in LTE and NR was extensively discussed [1]. However, there was no consensus for the template for EN-DC, NR DC/CA configurations. The only agreement was that uplink configuration shall be included in the band combination proposals.

In this contribution, we discuss further on EN-DC and NR CA/DC combination request template.

# 2	Discussion

2.1 Current WIDs status

Actually, the request templates for EN-DC and SUL were shared via email by the NR rapporteur before the RAN4 May meeting, and also the request template for NR CA/DC was agreed in [2]. Although the request templates were agreed, different companies use different configuration notations when they request their configurations. Consequently, the requested configuration notations for the configurations shown in the WIDs from different companies are not exactly aligned, which was mentioned in [1].

Here, we firstly review the table formats (highlighted in yellow) used in current WIDs, and then compare with the table format proposed in [1]. We take some following WIDs as the examples:

LTE 1DL/1UL + NR 1DL/1UL EN-DC WID [3]

Table 1. Table format used for EN-DC in [3] (for most configurations.):

| EN-DC configuration | REL-indep. from | contact name, company | contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) | supported next level fallback modes (in DL and UL) |
| --- | --- | --- | --- | --- | --- | --- |
| DL_XA_nYA_UL_XA_nYA | Rel-15 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | New | NONE |
| DL_XA-_nYC_UL_XA_nYA | Rel-15 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | New | DL_XA_nYA_UL_XA_nYA |
| DL_XA-_nYC_UL_XA_nYC | Rel-15 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | New | DL_XA_nYC_UL_XA_nYA |
| DL_(n)XAA_UL_(n)XAA | Rel-15 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | New | NONE |
| DL_(n)XA2A_UL_(n)XAA | Rel-15 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | New | DL_nX(2A)_UL_nXA DL_XA-nXA_UL_XA-nXA |

In the above table, only the configuration notations with a prefix “DL_” are listed. Actually there are also some configurations requested with the prefix of “DC_” in current WID, resulting in that the configurations notations from different companies are not exactly aligned.

The UL configurations are already captured in the current format, including 1 UL and 2 UL respectively. However, two types of information are missing:

- For the supported next level fallback mode, no progress status information are added.

- There are no explicit information of the supported bandwidth for each band.

NR Inter-band CA/DC WID [6]

Table 2.1 Table format for 1 band UL used in [6] (for most NR CA configurations.):

| Combination | # CC for DL | CA configuration | REL-indep. from | contact name, company | contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) | supported next level fallback modes (in DL and UL) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| nX-nY | 2 | nXA-nYA_BCS0 | Rel-15 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | New | None |
|  | 3 | DL_nXA-nYC_UL_nYC_BCS0 | Rel-15 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | New | 2B_3CC_DL_nXA-nYC_BCS0 -- new 1B_2CC_DL_nYC_2CC_UL_nYC_BCS0 -- new |

Table 2.2 2 band UL format used in [6] (for most NR CA configurations.):

| Combination | # CC for DL | CA configuration | REL-indep. from | contact name, company | contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) | supported next level fallback modes (in DL and UL) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| nX-nY | 2 | 2CC_DL_nXA-nYA_2CC_UL_nXA-YA_BCS0 | Rel-15 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | New | 2B_2CC_DL_nY1A-nY2A_BCS0 -- new |
|  | 3 | 3CC_DL_nXA-nYC_2CC_UL_nXA-nYA_BCS0 | Rel-15 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | New | 2B_3CC_DL_nXA-nYC_BCS0 -- new 2B_2CC_DL_nXA-nYA_2CC_UL_nXA-nYA_BCS0 -- new |

Compared with EN-DC WID, the table for NR CA/DC WID contains two extra rows (i.e. combination and #CC for DL). Actually, this template refers to the templates of LTE inter-band CA basket WIDs, which were widely used in many years.

For the configuration notations in table 2.1, if no UL configurations is specified, it implicitly means that single carrier either in band X or Y can be used as the UL configuration. In this simple way the number of configurations can be reduced.

In addition, the UL configurations (including BCS) are already captured in the format, including 1 UL and 2 UL respectively, but BCS is not necessarily specified in the UL configuration information for EN-DC. Meanwhile, there is explicit information of the supported bandwidth for each band in the NR CA/DC WID.

For the supported next level fallback mode, the progress status information are also added.

NR intra-band CA WID [7]

Table 3.1 FR1 intra-band contiguous CA format used in [7] (for most configurations.):

| Num CC | CA configuration | REL-indep. from | contact name, company | contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) | supported next level fallback modes (in DL and UL) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | DL_nYC_UL_nYA | Rel-15 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | New | NONE |
| 2 | DL_nYC_UL_nYC | Rel-15 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | New | 1B_DL_nYC_UL_nYA --new |

Table 3.2 FR1 intra-band non-contiguous CA format used in [7] (for most configurations.):

| Num CC | CA configuration | REL-indep. from | contact name, company | contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) | supported next level fallback modes (in DL and UL) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | DL_nY(2A)_UL_nYA | Rel-15 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | New | NONE |
| 2 | DL_nY(2A)_UL_nY(2A) | Rel-15 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | New | 1B_DL_nY(2A)_UL_nYA --new |

Table 3.3 FR2 intra-band contiguous and non-contiguous CA format used in [7] (for most configurations.):

| Num CC | CA configuration | REL-indep. from | contact name, company | contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) | supported next level fallback modes (in DL and UL) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | CA_nYG_UL_nYG | Rel-15 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | New | CA_nYG_UL_nYA -- new |
| 5 | CA_nY(5A) | Rel-15 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | New | CA_nY(4A) |

There are some differences between the tables for NR intra-band CA WID and NR inter-band CA/DC WID:

- Only the row of  #CC for DL included in NR intra-band CA/DC WID. This template also refers to the templates of LTE intra-band CA basket WIDs.

- In Addition, the BCS information is only captured in the tables of the supported bandwidth for each band.

It should be noted that the configuration notations are totally different for FR1 and FR2 intra-band contiguous and non-contiguous CA.

Based on the above summarize, different table formats (highlighted in yellow) are used in the different WIDs, and the companies have already requested lots of configurations based on the table formats used in each WID.

Observation 1. The table formats are different for EN-DC WID, NR inter-band CA/DC WID and NR intra-band CA WID.

Next, let’s come back to the template for EN-DC, NR DC/CA configurations proposed in [1], which are shown in table 4:

Table 4: Template for EN-DC, DC/CA configurations (proposed in [1])

| EN-DC, DC/CA configuration | Uplink Configuration | BCS* | Contact name, company | Contact email | other supporting companies (min. 3) | Status (new, ongoing, completed, stopped) | supported next level fallback modes (in DL and UL) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_XA_nXA | DC_XA_nXA | 0 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | NEW | NONE |
| DC_XC_nXA | DC_XA_nXA | 0 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | NEW | (new) DL_XA_nXA_UL_XA_nXA |
| DC_(n)XAA | DC_(n)XAA | 0 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | NEW | NONE |
| DC_(n)XCA | DC_(n)XAA | 0 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | NEW | (new) DL_(n)XAA_UL_(n)XAA |
| DC_XC_nYA | DC_XC_nYA | 0 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | NEW | (new) DL_XA_nYA_UL_XC_nYA |
| DC_XC_nYA | DC_XA_nYA | 0 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | NEW | NONE |
| DC_XC-ZA_nYA | DC_XC_nYA | 0 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | NEW | (new)DL_XA-ZA_nYA_UL_XA_nYA (new)DL_XC_nYA_UL_XC_nYA |
| DC_XC-ZA_nYA | DC_ZA_nYA | 0 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | NEW | (new) DL_XA-ZA_nYA_UL_ZA_nYA (new) DL_XC-ZA_nYA_UL_ZA_nYA |
| CA_nXA-nYA | (empty) | 0 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | NEW | NONE |
| CA_nXA-nYA | CA_nXA-nYA | 0 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | NEW | (new) DL_nXA_nYA_UL_nXA (new) DL_nXA_nYA_UL_nYA |
| CA_nXA-nYC | CA_nXA-nYA | 0 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | NEW | (new) DL_nXA_nYA_UL_nXA-nYA |
| CA_XA-YA | CA_XA-YA | 0 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | NEW | NONE |
| CA_XC | CA_XC | 0 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | NEW | DL_XC_UL_XA |
| DC_nXA_nYA | DC_nXA-nYA | 0 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | NEW | NONE |

In table 4, a unified table format is proposed for EN-DC and CA, different from the table format used in the current WIDs shown in the above.

Observation 2. The table format proposed in [1] is different from that used in the current WIDs.

Since the configuration request has been already started for half a year, hundreds of configurations have been already captured in each WID. It requires huge amount of extra work load to harmonize all the WID targeting a unified template table by correcting the existing configurations notations and rewriting the current WIDs. It seems not possible if considering the tight time-line of the current standardization works. A reasonable way is to keep the current table formats unchanged for each WIDs, and separate template tables are used for different WIDs with the left work to align the notations for the configurations.

Proposal 1. Keep the current table formats unchanged for each WIDs.

2.2 Template Proposal

Based on the above discussion, the current table formats are kept unchanged for each WID and separate template tables are used for different WIDs. In addition, the configuration notations shall be aligned in each WID, otherwise the WID looks very chaotic and also it is time consumed to review all the band combination with different configuration notations.

Hence, in order to improve the future work of the configuration requested, it is proposed the following tables for the templates for each configuration on top of the table formats in current WIDs, where the configuration notations are corrected and the status for the fallback mode are added for some WIDs.

For EN-DC:

Table 5: Template for EN-DC configuration

| EN-DC configuration | REL-indep. from | contact name, company | contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) | supported next level fallback modes (in DL and UL) |
| --- | --- | --- | --- | --- | --- | --- |
| DL_XA_nYA_UL_XA_nYA | Rel-15 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | New | NONE |
| DL_XA-_nYC_UL_XA_nYA | Rel-15 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | New | DL_XA_nYA_UL_XA_nYA --new |
| DL_XA-_nYC_UL_XA_nYC | Rel-15 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | New | DL_XA_nYC_UL_XA_nYA --new |
| DL_(n)XAA_UL_(n)XAA | Rel-15 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | New | NONE |
| DL_(n)XA2A_UL_(n)XAA | Rel-15 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | New | DL_nX(2A)_UL_nXA --new DL_XA-nXA_UL_XA-nXA --new |

It is not necessary to specify that the supported bandwidths for each band should be associated with the requested configuration. Since all of the bandwidths are supported for inter-band EN-DC by default.

For NR inter-band CA/DC:

Table 6: Template for NR CA/DC configuration

Table 6(a): For 1 band UL NR CA

| Combination | # CC for DL | CA configuration | REL-indep. from | contact name, company | contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) | supported next level fallback modes (in DL and UL) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| nX-nY | 2 | DL_nXA-nYA_BCS0 | Rel-15 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | New | NONE |
|  | 3 | DL_nXA-nYC_UL_nYC_BCS0 | Rel-15 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | New | 2B_DL_nXA-nYC_BCS0 -- new 1B_DL_nXC_UL_nYC_BCS0 -- new |

Table 6(b): For 2 band UL NR CA:

| Combination | # CC for DL | CA configuration | REL-indep. from | contact name, company | contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) | supported next level fallback modes (in DL and UL) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| nX-nY | 2 | DL_nXA-nYA_UL_nXA-nYA_BCS0 | Rel-15 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | New | 2B_DL_nXA-nYA_BCS0 -- new |
|  | 3 | DL_nXA-nYC_UL_nXA-nYA_BCS0 | Rel-15 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | New | 2B_DL_nXA-nYC_BCS0 -- new 2B_DL_nXA-nYA_UL_nXA-nYA_BCS0 -- new |

Table 6(c): For inter-band NR DC:

| DC configuration | REL-indep. from | contact name, company | contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) | supported next level fallback modes (in DL and UL) |
| --- | --- | --- | --- | --- | --- | --- |
| DL_nXA-nYA_UL_nXA-nYA_BCS0 | Rel-15 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | New | NONE |
| DL_nXA-nYC_UL_nXA-nYA_BCS0 | Rel-15 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | New | 2B_DL_nXA-nYA_UL_nXA-nYA_BCS0 -- new |

Also the supported bandwidths for each band should be associated with the requested configuration.

For NR intra-band CA:

Table 7. Template for intra-band contiguous and non-contiguous CA (Both FR1 and FR2):

| Num CC | CA configuration | REL-indep. from | contact name, company | contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) | supported next level fallback modes (in DL and UL) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | DL_nYC_UL_nYA_BCS0 | Rel-15 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | New | NONE |
| 2 | DL_nYC_UL_nYC_BCS0 | Rel-15 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | New | 1B_DL_nYC_UL_nYA_BCS0 -new |
| 2 | DL_nY(2A)_UL_nYA_BCS0 | Rel-15 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | New | NONE |
| 2 | DL_nY(2A)_UL_nY(2A)_BCS0 | Rel-15 | Contact Name, Company name | contact@company.com | [comp1, comp2, comp3] | New | 1B_DL_nY(2A)_UL_nYA_BCS0 --new |

Also the supported bandwidths for each band should be associated with the requested configuration.

Proposal 2. Use the table 5-7 for the introduction of band combinations in LTE and NR, as well as for the basket WIDs.

Since there may be lots of work to do to align the WIDs with the template, the WIDs aligned with the proposed template should be available in RAN #84 meeting.  For the requested combination during the transition time from the companies, it is proposed to use the proposed templates from the RAN4 #90 meeting.

Proposal 3. Use the proposed templates to request the configurations from RAN4 #90 meeting, and the WIDs aligned with the proposed templates should be available in RAN #84 meeting

# 3	Conclusion

In this contribution, we give some further discussions on ENDC_NR CA&DC combination request template. To reduce the reviews time for all the band combination requested, the alignment works shall be done for all of the WIDs, meanwhile to reduce the work of each rapporteur and also to avoid to re-write the WIDs, the proposals in this contributions are:

Observation 1. The table formats are different between ENDC WID, NR inter-band CA/DC WID and NR intra-band CA WID.

Observation 2. The table format proposed in [1] is different from that used in the current WIDs.

Proposal 1. Keep the current table formats unchanged for each WIDs.

Proposal 2. Use the table 5-7 for the introduction of band combinations in LTE and NR, as well as for the basket WIDs.

Proposal 3. Use the proposed templates to request the configurations from RAN4 #90 meeting, and the WIDs aligned with the proposed templates should be available in RAN #84 meeting

# 4	Reference

[1] R4-1816472, LTE and NR band combinations definition, Intel

[2] R4-1810176, Proposal on the request format for Rel.16 NR CA combination, ZTE Corporation

[3]RP-181885,Revised WID on Dual Connectivity (EN-DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL),NTT DOCOMO, INC.

[4] RP-181855, Revised WID on Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL), Huawei

[5] RP-181550, Revised WID on Dual Connectivity (EN-DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL), Ericsson

[6] RP-181730, Revised WID on Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity  for 2 bands DL with x bands UL (x=1,2), ZTE Corporation

[7] RP-181549,Revised WID on Rel-16 NR intra band Carrier Aggregation for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y),Ericsson