3GPP TSG RAN Meeting #82	RP-182540

Sorrento, Italy, December 10 - 13, 2018

Agenda Item:	9.3.2

Source:	Huawei

Title:	Text proposal to TR37.910

Document for:	Discussion and decision

- Introduction

In this contribution, the text proposal to TR37.910 is provided. The text proposal is to correct a typo in user plane latency evalution. This is due to a “column error” when the source input is captured based on R1-1808450 and R1-1808451. In Table 5.7.1.1.2-6 in TR37.910 v1.0.0, the fourth column “0.59, 0.67, 0.40, 0.45, …” should be under “120 kHz SCS” for UE capability 1. And one column of “60 kHz SCS” for UE capability 2 is missed.

- Conclusions

It is proposed to capture the text proposal as in Appendix A to TR37.910.

Appendix A Text proposals

<Unchanged parts are omitted>

5.7	Latency

5.7.1	User plane latency

5.7.1.1	NR

5.7.1.1.2	Uplink

In addition, NR supports X slot that can be flexibly configured based on the traffic pattern. The evaluation of DDDXU is provided for this capability, where ‘X’ slot = ‘U’ slot for UL data transmission and ‘X’=‘D’ for PDCCH transmission for UL user plane latency evaluation. The evaluation results are provided in Table 5.7.1.1.2-6.

Table 5.7.1.1.2-6 UL user plane latency for NR TDD with grant free transmission (ms)
(Frame structure: DDDXU, with ‘X’=’U’ for UL traffic)

| UL user plane latency – NR TDD (DDDXU) |  |  | UE capability 1 |  |  |  | UE capability 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS |  |  |  | SCS |  |  |
|  |  |  | 15 kHz | 30 kHz | 60 kHz | 120 kHz | 15 kHz | 30 kHz | 60 kHz |
| Resource mapping Type A | M=14 (14OS slot) | p=0 | 3.34 | 1.74 | 1.04 | 0.59 | 3.02 | 1.54 | 0.86 |
|  |  | p=0.1 | 3.82 | 1.98 | 1.16 | 0.67 | 3.50 | 1.78 | 0.98 |
| Resource mapping Type B | M=2 (2OS non-slot) | p=0 | 1.80 | 0.97 | 0.66 | 0.40 | 1.48 | 0.77 | 0.48 |
|  |  | p=0.1 | 2.04 | 1.10 | 0.77 | 0.45 | 1.64 | 0.86 | 0.54 |
|  | M=7 (7OS non-slot) | p=0 | 2.44 | 1.29 | 0.82 | 0.48 | 2.12 | 1.09 | 0.64 |
|  |  | p=0.1 | 2.91 | 1.53 | 0.93 | 0.54 | 2.35 | 1.20 | 0.76 |

<Unchanged parts are omitted>