3GPP TSG-RAN2 Meeting #104	R2-1818830

# Spokane, USA, 12th - 16th November 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0125 | rev | 1 | Current version: | 15.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on power ramping counter |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO, Fujitsu, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-10-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In current MAC specification (TS 38.321, v15.3.0, 2018-09), it’s specified that the UE increments the PREAMBLE_POWER_RAMPING_COUNTER if the following three conditions are met: 1> if PREAMBLE_TRANSMISSION_COUNTER is greater than one; and 1> if the notification of suspending power ramping counter has not been received from lower layers; and 1> if SSB selected is not changed (i.e. same as the previous Random Access Preamble transmission): 2> increment PREAMBLE_POWER_RAMPING_COUNTER by 1. It should be noted that in last RAN2#103bis Chengdu meeting, an CR was agreed in principle to update the power ramping counter in TS 38.321, i.e., the 3rd if: R2-1815986 CR to power ramping (option1) Fujitsu CR Rel-15 38.321 15.3.0 0399 2 F NR_newRAT-Core R2-1813018 => revision should be updated to 4 when submitted for approval next meeting => The CR is in principle agreed Based on the CR, the 3rd “if” is changed to “1> if SSB or CSI-RS selected is not changed from the selection in the last Random Access Preamble transmission:” However, in TS 38.300, regarding the power ramping counter, it specifies in the following: The UE calculates the PRACH transmit power for the retransmission of the preamble based on the most recent estimate pathloss and power ramping counter. If the UE conducts beam switching, the counter of power ramping remains unchanged. Regarding the sentence “If the UE conducts beam switching, the counter of power ramping remains unchanged”, it seems the power ramping counter is only related to whether UE conducts beam switching which is corresponding to the second “if” in the MAC spec. However, when the selected reference signal is changed, the counter of power ramping also remains unchanged. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the whole sentence of “If the UE conducts beam switching, the counter of power ramping remains unchanged”, so that there is no ambiguity. Impact analysis Impacted functionality: Random access Inter-operability: - If the network is implemented according to the CR and the UE is not, there is no inter-operability issue. - If the UE is implemented according to the CR and the network is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The condition of not incrementing power ramping counter is TS 38.300 is not accurate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of change

### 5.3.4	Random access

Random access preamble sequences, of two different lengths are supported. Long sequence length 839 is applied with subcarrier spacings of 1.25 and 5 kHz and short sequence length 139 is applied with sub-carrier spacings 15, 30, 60 and 120 kHz. Long sequences support unrestricted sets and restricted sets of Type A and Type B, while short sequences support unrestricted sets only.

Multiple PRACH preamble formats are defined with one or more PRACH OFDM symbols, and different cyclic prefix and guard time. The PRACH preamble configuration to use is provided to the UE in the system information.

The UE calculates the PRACH transmit power for the retransmission of the preamble based on the most recent estimate pathloss and power ramping counter.

The system information provides information for the UE to determine the association between the SSB and the RACH resources. The RSRP threshold for SSB selection for RACH resource association is configurable by network.

End of change