3GPP TSG-RAN2 Meeting #104	R2-1818824

# Spokane, USA, 12th Nov 2018 - 16th Nov 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0134 | rev | 1 | Current version: | 15.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on the carrier selection for random access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ITRI, Rapporteur (Nokia) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-11-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the current stage 2 specification, the carrier to use for the random access procedure is described clearly for the case of initial access in a cell configured with SUL. However, it is not clear which carrier shall be used for the random access procedure in other cases. According to the latest MAC specification, the carrier to use for the random access procedure can be explicitly signalled, or selected by the MAC entity based on the RSRP of the downlink pathloss reference. It shall be captured in the stage 2 specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In subclause 9.2.6, revise the description of carrier selection for the case of random access in a cell configured with SUL based on the latest MAC specification. Impact analysis Impacted functionality: Random Access Procedure Inter-operability: none as the corresponding Stage 3 mechanisms are already captured 38.331. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE doesn’t know which carrier shall be used for random access if the cell is configured with SUL. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of modification>

9.2.6	Random Access Procedure

The random access procedure is triggered by a number of events, for instance:

-	Initial access from RRC_IDLE;

-	RRC Connection Re-establishment procedure;

-	Handover;

-	DL or UL data arrival during RRC_CONNECTED when UL synchronisation status is "non-synchronised";

-	Transition from RRC_INACTIVE;

-	To establish time alignment at SCell addition;

-	Request for Other SI (see subclause 7.3);

-	Beam failure recovery.

Furthermore, the random access procedure takes two distinct forms: contention-based random access (CBRA) and contention-free random access (CFRA) as shown on Figure 9.2.6-1 below.

For random access in a cell configured with SUL, the network can explicitly signal which carrier to use (UL or SUL). Otherwise, the UE selects the SUL carrier if and only if the measured quality of the DL is lower than a broadcast threshold. Once started, all uplink transmissions of the random access procedure remain on the selected carrier.

(a)	Contention-Based											(b)	Contention-Free

Figure 9.2.6-1: Random Access Procedures

<end of modification>