3GPP TSG-RAN Meeting #82	RP-182710

Sorrento, Italy, December 10-13, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | 0093 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification for EN-DC SN change scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RP |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-12-05 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The EN-DC bearer type changes were designed to allow changing also security keys when necessary, but it is also possible to only change the security key and not the bearer type when doing reconfiguration. The current RRC specification condition does not take the “security key change only” scenario into account. Therefore, the sections in TS 37.340 describing the bearer type change need to include also the when only the security key changes but bearer type remains the same. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are proposed: - in section 8.3, clarify that only the security key may be changed while the bearer type remains the same. - in section 8.3 remove the text which limits the bearer termination point change to only MN/SN terminated split bearer and vice-versa. - in Annex A, remove the text which limits the bearer termination point change (in the part of the cell marked as “key change”) to also include key change during SN change scenarios (i.e. change of S-KgNB) Impact analysis Impacted functionality: Handling of “Bearer Type Change”. Inter-operability: None. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current specification text does not include the bearer type change scenario from split to split (in this case the SN terminated split bearer) during the SN change. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3, Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  | TS/TR 36.331 CR3814 |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

## 8.3	Bearer type/key change

In MR-DC, all the possible bearer type/key change options are supported:

-	MCG bearer to/from split bearer;

-	MCG bearer to/from SCG bearer;

-	SCG bearer to/from split bearer.

Bearer termination point change is supported for all bearer types, and can be performed with or without bearer type change.

For EN-DC:

-	when the security key is changed for a bearer, the associated PDCP and RLC entities are re-established, while MAC behavior might depend on the solution selected by the network, e.g. MAC reset, change of LCID, etc. (see Annex A);

-	for MCG bearer, split bearer and SCG bearer, during handover MCG/SCG PDCP and RLC are re-established and MCG/SCG MAC is reset;

-	if a bearer type change happens through handover procedure then for MCG bearer, split bearer and SCG bearer, MCG/SCG PDCP/RLC are re-established and MCG/SCG MAC is reset;

-	if a bearer type change happens through SN change procedure, then SN terminated PDCP /RLC are re-established and SCG MAC is reset;

-	one step (direct) bearer type change between MN terminated bearer types without using the handover procedure is supported;

-	one step (direct) bearer type change between SN terminated bearer types without using the handover or SN change procedure is supported;

-    one step (direct) bearer type change from/to MN terminated bearer to/from SN terminated bearer without using the handover procedure is supported;

-	Upon bearer type change from SCG bearer to MCG bearer MAC is not reset; the associated NR RLC entity is released and the associated LTE RLC entity is established;

-	PDCP version change for DRB or PDCP SN length change for an AM DRB or RLC mode change for DRB is performed using a release and add of the DRBs (in a single message) or full configuration;

-	One step (direct) bearer type change with PDCP version change is supported;

-	Upon bearer type change from MCG bearer to SCG bearer or from split bearer to SCG bearer, the associated LTE RLC entity is first re-established and then released, the associated NR RLC entity is established for MCG bearer to SCG bearer:

NOTE:	L2 handling for bearer type change in EN-DC is also summarized in Annex A (the table does not consider the cases that PDCP SN length is changed and avoiding reuse of COUNT).

Second Modified Subclause

Annex A (informative):
Layer 2 handling for bearer type/key change

This subclause provides for information an overview on L2 handling for bearer type/key change in EN-DC, with and without security key change, i.e. with and without a change of the termination point.

Table A-1: L2 handling for bearer type/key change

| Bearer type change from row to col | MCG |  | Split |  | SCG |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | no key change | with key change | no key change | with key change | no key change | with key change |
| MCG | N/A | PDCP: Re-establish MCG RLC: See Note 1 MCG MAC: See Note 1 SCG RLC: No action SCG MAC: No action | PDCP: Reconfigure MCG RLC: No action MCG MAC: No action SCG RLC: Establish SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: See Note 1 MCG MAC: See Note 1 SCG RLC: Establish SCG MAC: Reconfigure | PDCP: Recovery MCG RLC: Re-est+release MCG MAC: Reconfigure SCG RLC: Establish SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: Re-est+release MCG MAC: Reconfigure SCG RLC: Establish SCG MAC: Reconfigure |
| Split | PDCP: Recovery MCG RLC: No action MCG MAC: No action SCG RLC: Release SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: See Note 1 MCG MAC: See Note 1 SCG RLC: Release SCG MAC: Reconfigure | N/A | PDCP: Re-establish MCG RLC: See Note 1 MCG MAC: See Note 1 SCG RLC: See Note 2 SCG MAC: See Note 2 | PDCP: Recovery MCG RLC: Re-est+release MCG MAC: Reconfigure SCG RLC: No action SCG MAC: No action | PDCP: Re-establish MCG RLC: Re-est+release MCG MAC: Reconfigure SCG RLC: See Note 2 SCG MAC: See Note 2 |
| SCG | PDCP: Recovery MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: Release SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: Release SCG MAC: Reconfigure | PDCP: Reconfigure MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: No action SCG MAC: No action | PDCP: Re-establish MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: See Note 2SCG MAC: See Note 2 | N/A | PDCP: Re-establish MCG RLC: No action MCG MAC: No action SCG RLC: See Note 2SCG MAC: See Note 2 |

NOTE 1:	For MCG, the MAC/RLC behaviour depends on the solution selected by the network. It can be handover, which triggers MAC reset and RLC re-establishment. Alternatively, the logical channel identity can be changed, either via RLC bearer release and add (including RLC re-establishment), or via reconfiguration of the RLC bearer with RLC-re-establishment.

NOTE 2:	For SCG, MAC/RLC behaviour depends on the solution selected by the network. It can be reconfiguration with sync, with MAC reset and RLC re-establishment. Alternatively, the logical channel identity can be changed via RLC bearer release and add.