3GPP TSG RAN Meeting #82							RP-182747

Sorrento, Italy, December 10-13, 2018

Agenda Item:	9.1.2

Source:	AT&T

Title:	Motivation for Key Features of IAB WID

Document for:	Discussion/Approval

# Introduction

The Study Item on Integrated Access and Backhaul for NR [1], has been completed and identified and evaluated potential solutions for efficient operation of integrated access and wireless backhaul for NR.

An example of a network with integrated access and backhaul links is shown in Figure 1 below.

Figure 1: Integrated access and backhaul links

Three key components of the IAB design are support for topology management, route management, and frame structure optimization and backhaul link scheduling as shown in Figure 2.

Figure 2: Key components of IAB design

Topology management includes the following characteristics: 1) happens on long time scales, 2) manages static hop order, 3) handles initial access of relay nodes, and 4) changes every time a node is added or removed. On the other hand route management happens at a much faster time scale (e.g. happens over 10s or 100s of ms) and routes are updated for load variance and blocking. Finally, the scheduling of backhaul links is performed by the parent IAB nodes or donor nodes, but also requires coordination and exchange of resource allocation/route selection information to support different constraints (e.g. half or full duplex) and resource multiplexing options (e.g. time/frequency/space).

Observation: The IAB design should support long-term topology management, short-term route management, and mechanisms for optimizing resource allocation/route selection across multiple IAB nodes/backhaul hops.

This contribution discusses the following mechanisms to support those key components of the IAB design and should be included in a Rel-16 NR IAB WID based on solutions recommended for support in the IAB SI TR [2]:

- NSA Support for IAB Topology Formation and Management

- Multi-Connectivity to support fast route selection

- Resource allocation to support TDM/FDM/SDM of access and backhaul links

# NSA support for IAB Topology Formation and Management

Initial NR network deployments based on Release 15 3GPP specifications may be based on the non-standalone (NSA) scenario (Option 3x), using existing EPC deployments for the core network. Additionally, some other NR network deployments may be based on the standalone (SA) scenario (Option 2). Furthermore, some networks deployed in Option 3x scenario using EPC could migrate to Option 7x or 4 using NGC.

Hence, it is very conceivable that any IAB solutions potentially standardized in Release 16 specifications, may need to operate with either EPC or NGC as the core network, and may also need to continue to operate efficiently in the event of a core network transition from EPC to NGC. As a result the IAB SI TR [2] has captured solutions for IAB operation under both SA and NSA deployments.

The IAB deployment in Figure 3 illustrates a SA deployment of IAB where the access UEs and IAB DUs receive both control and data bearers on NR.

Figure 3: IAB operation under in a SA deployment

IAB operation (UEs and IAB nodes) under a NSA architecture is shown in Figure 4, where the control plane signaling for both access UEs and IAB nodes are provided over LTE.

Figure 4: IAB operation in a NSA deployment

Under NSA operation, access UEs are connected to the network by performing initial access procedures (e.g. synchronization signal detection and random access procedure) via LTE and then are able to access one or more NR cells via dual connectivity procedures such as secondary node addition.

In the case of IAB operation under an NSA architecture, initial access and mobility are provided by the LTE MCG in case of EN-DC, can be much more efficient than if a SA architecture is required for IAB nodes where system information broadcast, paging, and other IDLE mode transmissions are required, thus reducing the overall overhead required to support multiplexing of access and backhaul traffic. However the IAB node’s MT must also have LTE receiver functionality.  The LTE MCG can also be used for providing future routing updates and assist in topology adaptation procedures (e.g. SN change).

Observation: IAB operation under NSA supports early deployments of NR and provides benefits in terms of reduced overhead and increased robustness for IAB topology management and control plane signalling.

Proposal 1: Both NSA and SA operation for IAB-nodes should be supported in the Rel-16 IAB WI.

# Multi-Connectivity for IAB Route Management

For densely deployed mmWave NR systems, the area covered by an NR node can be quite small, so a dense deployment of NR nodes in a given area could require a significantly larger number of deployment sites. There is considerable interest in being able to deploy NR nodes without fiber access to every deployment site. Due to large available bandwidths in mmWave spectrum bands, and the ability to deploy narrow beams using multi-antenna element arrays, it may be more feasible to support self-backhauled links for NR systems operating in mmWave spectrum as shown in Figure 5.

Figure 5 below shows an example scenario where IAB backhaul links could be used to connect multiple IAB DUs DU3 and DU4 (e.g. across one or more hops) to DU1, which has fiber access to the CU.

Figure 5: Support for UE mobility in an IAB Scenario

At mmWave frequencies, the channel experienced by a UE may suffer from blockage events that could result in sudden sharp drops in signal strength (of the order of 30 dB) due to physical objects blocking the UE-IAB node link. Depending on environmental factors and user mobility, frequent beam failure events due to blockage can occur, potentially resulting in frequent beam switches. However if intra-frequency dual connectivity or multi-connectivity is enabled, it could significantly help in reduction of user plane interruption. As a result, multi-connectivity is a critical feature for IAB to support robustness and fast route selection in case of blockage events as shown in Figure 6.

Figure 6: Multi-connectivity for IAB

In addition to support for redundant routes via multi-connectivity, IAB should have support for low-latency control plane and data routing procedures with dynamic route selection (faster than RRC time scales). This ensures that latency and user interruption is minimized while the network finds an alternate route to the UE when the current route is blocked. Since blocking in mmWave happens over short time-scales, this constraint should be considered as an integral part of the IAB design and mechanisms for blockage and load-aware route selection should be specified in Rel-16.

Solutions studied in the SI include utilizing NR-NR DC and dynamic flow control mechanisms to determine the routing of traffic across different links and upon beam failure events as described above. The latter may benefit from utilization of fast centralized retransmissions of lost RLC PDUs across different multi-connectivity legs to quickly recover from failed transmissions which is already supported by NR in Rel. 15 and may require minimal enhancements in order to be supported across multiple backhaul hops [3].

Proposal 2: The IAB design in Rel-16 should support route switching based on intra-frequency multi-connectivity, leveraging existing NR solutions of fast centralized retransmission of pending PDCP PDUs as well as NR-NR DC.

# TDM/FDM/SDM Access and Backhaul Multiplexing

A key advantage of IAB is that backhaul and access are integrated and multiplexed in the scheduler, allowing very dynamic resource allocation between the backhaul and access links (in both DL and UL directions). Even though the IAB relay node consists of two logical nodes each with its own protocol stack, the physical transceiver is shared between them. In other words the PHY of the MT and the PHY of the DU run on the same transceiver systems.

The backhaul link and the access link can therefore be multiplexed in the following manner:

- Time Division Multiplexing: The access link and the backhaul link are time multiplexed with each other. This implies that IAB-MT PHY and IAB-DU PHY are not active simultaneously.

- Frequency Division Multiplexing: The access link and the backhaul link are active at the same time but on different frequency resources, e.g. on separate CC or on separate PRB on the same CC.

- Spatial Multiplexing: The access link and the backhaul link are active at the same time on the same frequency resources. This case further be divided in to the two following cases:

  - Intra Panel SDM: The access and backhaul use the same panel but different spatial layers

  - Inter Panel SDM: The access and backhaul use different panels

Due to the half-duplex constraint at each relay node it can only transmit or receive at any given instance but not both. The MT PHY is in receive mode during a backhaul DL allocation and in transmit mode during a backhaul UL allocation. Similarly the DU PHY is in receive mode during an UL allocation and in transmit mode during a DL allocation as shown in Figure 7.

Figure 7: Half-duplex constraint at the relay

In Figure 8, TDM partitioning is shown with DL/UL switching gaps between both the backhaul directions as well as for the access links while a guard band is introduced between backhaul subframes in the case of FDM.

Figure 8: TDM/FDM of access and backhaul links

In addition, the multiplexing of the access and backhaul links should be backwards compatible to ensure Rel.15 UEs can be supported by IAB. Specifically, the latency/overhead introduced by orthogonal partitioning of resources in either time or frequency should be carefully considered, especially for mmWave frequencies which are typically TDD.

Considering the multi-hop nature of the IAB network, more efficient usage of the DL/UL resources than strict TDM between parent and child links can be used even when taking into account the half-duplex constraint. When the donor gNB (hop 0) sends DL transmissions to the relay node of hop order 1, said rely node is receiving, hence it can schedule access UEs whose gNB it is in the UL. Alternatively, a second order relay node can transmit to the first order relay node when the latter is receiving from the donor node (hop 0). At the network side, the frame structure, can be coordinated across multiple hops in a staggered manner as shown in Figure 9.

Figure 9: IAB frame structure under SDM operation

While the IAB TR captures that TDM resource allocation is recommended to be the first priority for the IAB WI, introduction of signalling to support FDM/SDM can be done in a forward compatible manner that enables IAB nodes based on Rel-16 specification to support TDM/FDM/SDM and even further optimizations (including full-duplex operation) introduced in later releases. This is critical as it has been shown in evaluation results submitted during the SI that SDM operation can have significant performance benefits in terms of increased throughput and reduced latency compared to TDM only partitioning.

Proposal 3: In addition to TDM partitioning of access and backhaul links, the resource allocation singling developed in the Rel-16 IAB WI should support FDM/SDM operation and coordination mechanisms which enable alignment of DL transmissions of the IAB node’s DU with UL transmission slots of the IAB node’s MT as well as alignment of DL reception slots of the IAB nodes’ MT with UL reception slots at the IAB node’s DU.

# Conclusion

This contribution described key features to be supported in the Rel-16 IAB WI and made the following proposals:

Proposal 1: Both NSA and SA operation for IAB-nodes should be supported in the Rel-16 IAB WI.

Proposal 2: The IAB design in Rel-16 should support route switching based on intra-frequency multi-connectivity, leveraging existing NR solutions of fast centralized retransmission of pending PDCP PDUs as well as NR-NR DC.

Proposal 3: In addition to TDM partitioning of access and backhaul links, the resource allocation singling developed in the Rel-16 IAB WI should support FDM/SDM operation and coordination mechanisms which enable alignment of DL transmissions of the IAB node’s DU with UL transmission slots of the IAB node’s MT as well as alignment of DL reception slots of the IAB nodes’ MT with UL reception slots at the IAB node’s DU.

# Reference

- RP-170831, Study on Integrated Access and Backhaul for NR, AT&T, KDDI, Qualcomm, Samsung.

- RP-182329, TR 38.874 v1.0.0 on NR; Study on integrated access and backhaul

- R3-173235, Mechanism for fast retransmission of lost PDUs, Ericsson, AT&T