3GPP TSG RAN meeting #82								                       RP-182750

Sorrento, Italy, December 10-13, 2018

## Status Report to TSG

Agenda item:			9.3.10

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_NR_IAB |  |  |  |  |
| Unique ID | 750047 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-181349 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 12/2018 | Core part: N/A | Performance part: N/A | Testing part: N/A |  |
| Overall Completion level | Study Item: 100% | Core part: N/A | Performance Part: N/A | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN-2 |
| --- | --- | --- |
| Rapporteur | Name | Georg Hampel |
|  | Company | Qualcomm Incorporated |
|  | Email | ghampel@qti.qualcomm.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

RAN1#94bis

Agreements:

- Solution 1-B means SSB, that may get muted, for inter-IAB cell search and measurement in stage 2 is not on the currently defined sync raster for a SA frequency layer, while for a NSA frequency layer the SSBs are transmitted outside of the SMTC configured for access UEs

- Solution 1-A means SSB for inter-IAB cell search in stage 2 is on the currently defined sync raster for a SA frequency layer, while for a NSA frequency layer the SSBs are transmitted inside of the SMTC configured for access UEs

Agreements:

An IAB node should not mute SSB transmissions targeting UE cell search and measurement when doing inter-IAB cell search in stage 2

- For SA, means that SSBs transmitted on the currently defined sync raster follows the currently defined periodicity for initial access

- Means that Solution 1-B implies SSB, that may get muted, for inter-IAB stage 2 cell search is at least TDM with SSB used for UE cell search and measurements

Agreements:

- Solution 1-A and Solution 1-B are both supported

  - Enhancements for off-raster SSB, e.g. new periodicities and time-domain mapping can be considered

Agreements:

- CSI-RS can be used for inter-IAB detection in synchronous network

  - Extended CSI-RS periodicities can be considered.

Agreements:

- For IAB node random access support

  - Longer RACH periodicity

  - Additional preamble formats allowing for longer RTT

  - Details left for WI phase

Agreements:

- Based on Rel-15 PRACH configurations, NR allows network to configure offset(s) for PRACH occasions for MT IAB node(s), to TDM backhaul RACH resources across adjacent hops. The detailed granularity of the offset (e.g., radio frame, subframe, slot, etc.) is left for WI phase

Agreements:

- For Timing and Synchronization

  - For case #1 & case #7, if DL TX and UL RX are not well aligned at the parent node, additional information about the alignment is needed for the child node to properly set its DL TX timing for OTA based timing & synchronization

  - Case #7 to be supported if and only if compatible with release 15 Ues

    - Further check w.r.t. compatibility

  - Support of case #6 is FFS

  - No other cases are supported

Agreements:

From an MT point-of-view, the following time-domain resources can be indicated for the parent link:

- Downlink time resource,

- uplink time resource,

- flexible time resource

as in release 15.

From a DU point-of-view, the child link has the following types of time resources

- Downlink time resource,

- uplink time resource,

- flexible time resource

- not available time resources (not to be used for communication on the DU child links)

Exact configuration for DU is FFS

Agreements:

- For each of the downlink, uplink and flexible time-resource types of the DU child link there are two flavors, hard and soft:

  - Hard: The corresponding time resource is always available for the DU child link

  - Soft: The availability of the corresponding time resource for the DU child link is explicitly and/or implicitly controlled by the parent node.

Agreement of TP in R1-1812078: capturing RAN1#94bis agreements in the TR

RAN1#95

Agreements:

- In an NSA deployment (from an access UE perspective):

  - When the IAB node MT performs initial access on the NR carrier, it follows the same Stage 1 initial access as in SA deployments (from an access UE perspective).

      - The SSB/RMSI periodicity assumed by the MTs for initial access may be longer than 20ms assumed by Rel-15 UEs, e.g., 40ms, 80ms, 160ms, etc.

        - Note: one value is to be finalized during WI phase

    - Note: This implies that the candidate parent IAB nodes/donors must support both NSA functionality for the UE and SA functionality for the MT on the NR carrier

  - When IAB node MT performs initial access on a LTE carrier, Stage 2 solutions can be used for inter-IAB node discovery and measurement by the MT on the NR carrier.

- Details of signalling for IAB node initial access and Stage 2 inter-IAB node discovery and measurement, including SMTC/CSI-RS/RACH configurations and RMSI enhancements, as well as for coordination across IAB nodes are to be further considered in the WI stage.

Agreements:

- Enhancements to Beam Failure Recovery and Radio Link Failure procedures are beneficial and should be supported for NR IAB, including:

  - Enhancements to support interaction between Beam Recovery Procedure and RLF particularly beam failure recovery success indication and RLF

  - Enhancements to existing beam management procedures for faster beam reporting/switching/coordination/recovery between active and backup/candidate beams

  - Details of the signalling and procedures are to be further considered in the WI stage and may rely on solutions developed in other WIs (i.e. Rel-16 MIMO enhancements for BFR/RLF procedures)

- Solutions to avoid RLF at a child IAB node due to parent backhaul link failure should be supported

  - Details of the signalling and procedures are to be further considered in the WI stage

Conclusion:

- Although support of up to 1024QAM is beneficial for backhaul links, it is not deemed essential for Rel-16 IAB operation from RAN1 perspective

Agreements:

- Capture the following observation in the TR: With the assumption of a <=3us timing requirement across IAB nodes within overlapping coverage, TA-based OTA synchronization can support a multi-hop IAB network (up to 5 hops) for FR2. TA-based OTA synchronization may not be sufficient to support multiple hops in FR1.

- Capture the following observation in the TR: In addition to OTA synchronization, other techniques such as GNSS and PTP, can be used to achieve synchronization across IAB nodes.

Agreements:

- For Case 7, the following solution is compatible with Rel. 15 UEs: Introduce “effective” negative TA, and TDM between child IAB nodes/Rel-16 UEs which support the new TA values and child IAB nodes/UEs which do not support the new TA values

  - Details of required TA enhancements and signalling between parent and child IAB nodes to achieve timing alignment are to be further considered in the WI stage

Agreements:

- The use of Case 6, if supported, at the IAB node should be under control of the parent or network

Agreements:

Capture the following in the TR:

Case#7 
To enable alignment between DL and UL reception within the IAB node the following solutions have been identified:

- Alt 1: Introduce negative initial time alignment (TA) for IAB nodes, to be applied to child nodes of the IAB node applying case #7 timing

- Alt 2: Apply a positive TA that enables symbol alignment, but not slot alignment, between the DL reception and the UL reception at the IAB node

- Alt. 3: Signalling of a relative offset w.r.t the most recent TA value, to be applied to child nodes of the IAB node applying case #7 timing to achieve an effective negative TA

Agreements:

Case#6, if supported:
To enable alignment of DL transmissions among IAB nodes:

- Alt. 1: The IAB node may need to carry out parallel (always time multiplexed) case #1 and case #6 uplink transmissions

- Alt 2: Signalling between the parent and IAB node of the time difference of the DL Tx and UL Rx timing at the parent node in order to correct potential misalignment of the DL Tx timing at the child node:

  - The child IAB node compares the corresponding difference of its own DL Tx timing and BH Rx timing; if the signalled difference of the parent node is larger than measured at the child node, the child node advances its TX timing, if smaller the TX timing is delayed.

- Note: Alt 1 & Alt 2 may require maintenance of separate Rx timings at the parent node for Case 6 UL transmissions from different child nodes

Agreements:

- MT-to-DU, MT-to-MT and DU-to-DU, and DU-to-MT CLI measurements and required measurement coordination/configuration should be supported including the following aspects:

  - CLI measurements can be made based on existing RS (e.g. CSI-RS/SRS/DMRS)

    - Enhancements to RS configuration for CLI measurements and transmission timing can be considered in the WI phase

  - Rely on solutions developed in the CLI WI as a baseline and enhance if needed for IAB-specific aspects:

    - Time/frequency resource configuration for measurements across multiple hops

    - Inter IAB-node Tx/Rx beam sweeping and selection

    - Taking DU/MT power control into account in the CLI measurements

Agreements:

- Semi-static configuration should be supported for IAB node DU resources

- Dynamic indication (L1 signalling) to an IAB node of the availability of soft resources for an IAB node DU is supported

  - Existing Rel.15 L1 signalling methods are the baseline for the WI phase

- Potential enhancements (e.g. new slot formats), rules for DU/MT behaviour in case of conflicts across multiple hops, and processing time constraints at the IAB node are to be further considered in the WI stage.

Agreements:

Capture the following in the TR:

In the tables below:

- “MT: Tx” means that the MT should transmit if scheduled

- “DU: Tx” means that the DU may transmit

- “MT: Rx” means that the MT should be able to receive (if there is anything to receive)

- “DU: Rx” means that the DU may schedule uplink transmissions from child nodes or UEs

- “MT: Tx/Rx” means that the MT should transmit if scheduled and should be able to receive, but not simultaneously

- “DU: Tx/Rx” means that the DU may transmit and may schedule uplink transmission from child nodes and UEs, but not simultaneously

- “IA” means that the DU resource is explicitly or implicitly indicated as available

- “INA” means that the DU resource is explicitly or implicitly indicated as not available

- “MT: NULL” means that the MT does not transmit and does not have to be able to receive

- “DU: NULL” means that the DU does not transmit and does not schedule uplink transmission from child nodes and UEs

- The tables assume an IAB not capable of full-duplex operation

In case of TDM operation, where there can be no simultaneous transmission in the DU and the MT, nor any simultaneous reception in the DU and the MT, the following are all the possible combinations of DU and MT behavior (signaling details are to be considered in the WI phase):

| DU Configuration | MT configuration |  |  |
| --- | --- | --- | --- |
|  | DL | UL | F |
| DL-H | DU: Tx MT: NULL | DU: Tx MT: NULL | DU: Tx MT: NULL |
| DL-S | When DU resource: IA DU: Tx MT: NULL When DU resource: INA DU: NULL MT: Rx | When DU resource: IA DU: Tx MT: NULL When DU resource: INA If DU: NULL MT: Tx | When DU resource: IA DU: Tx MT: NULL When DU resource: INA If DU: NULL MT: Tx/Rx |
| UL-H | DU: Rx MT: NULL | DU: Rx MT: NULL | DU: Rx MT: NULL |
| UL-S | When DU resource: IA DU: Rx MT: NULL When DU resource: INA DU: NULL MT: Rx | When DU resource: IA DU: Rx MT: NULL When DU resource: INA DU: NULL MT: Tx | When DU resource: IA DU: Rx MT: NULL When DU resource: INA DU: NULL MT: Tx/Rx |
| F-H | DU: Tx/Rx MT: NULL | DU: Tx/Rx MT: NULL | DU: Tx/Rx MT: NULL |
| F-S | When DU resource: IA DU: Tx/Rx MT: NULL When DU resource: INA DU: NULL MT: Rx | When DU resource: IA DU: Tx/Rx MT: NULL When DU resource: INA DU: NULL MT: Tx | When DU resource: IA DU: Tx/Rx MT: NULL When DU resource: INA DU: NULL MT: Tx/Rx |
| NA | DU: NULL MT: Rx | DU: NULL MT: Tx | DU: NULL MT: Tx/Rx |

In case of SDM operation, where there can be simultaneous transmission in the DU and the MT, alternatively simultaneous reception in the DU and the MT, the following are the possible combinations of DU and MT behavior:

|  | DL | UL | F |
| --- | --- | --- | --- |
| DL-H | DU: Tx MT: NULL | DU: Tx MT: Tx | DU: Tx MT: Tx |
| DL-S | When DU resource: IA DU: Tx MT: NULL When DU resource: INA DU: NULL MT: Rx | When DU resource: IA DU: Tx MT: Tx When DU resource: INA DU: NULL MT: Tx | When DU resource: IA DU: Tx MT: Tx When DU resource: INA DU: NULL MT: Tx/Rx |
| UL-H | DU: Rx MT: Rx | DU: Rx MT: NULL | DU: Rx MT: Rx |
| UL-S | When DU resource: IA DU: Rx MT: Rx When DU resource: INA DU: NULL MT: Rx | When DU resource: IA DU: Rx MT: NULL When DU resource: INA DU: NULL MT: Tx | When DU resource: IA DU: Rx (only if MT is Rx and the DU knows that ahead of time) MT: Rx When DU resource: INA DU: NULL MT: Tx/Rx |
| F-H | DU: Tx/Rx MT: Rx (only if DU is Rx and the parent DU is aware in advance) | DU: Tx/Rx MT: Tx (only if DU is Tx and the parent is aware in advance) | DU: Tx/Rx MT: Tx (only if DU is Tx and the parent DU knows that ahead of time), Rx (only if DU is Rx and the parent DU is aware in advance) |
| F-S | When DU resource: IA DU: Tx/Rx MT: Rx (only if DU is Rx and the parent DU is aware in advance)* When DU resource: INA DU: NULL MT: Rx | When DU resource: IA DU: Tx/Rx MT: Tx (only if DU is Tx and the parent DU is aware in advance) When DU resource: INA DU: NULL MT: Tx | When DU resource: IA DU: Tx/Rx MT: Tx (only if DU is Tx and the parent DU knows that ahead of time), Rx (only if DU is Rx and the parent DU is aware in advance) When DU resource: INA DU: NULL MT: Tx/Rx |
| NA | DU: NULL MT: Rx | DU: NULL MT: Tx | DU: NULL MT: Tx/Rx |

Agreements:

- DL and UL transmit power coordination between IAB nodes is supported, including mechanisms for DL power control between a parent and child IAB node.

Agreements:

- Whether a parent node or the network needs to be aware of a child IAB node’s capability should be considered in the WI phase, including the following:

  - Support for full duplex

  - Supporting SDM/FDM TX of parent and child links

  - Supporting SDM/FDM RX of parent and child links

  - Supported timing alignment cases

Agreements:

Capture the following observations for the IAB SI TR:

- From the results provided, it can be concluded that IAB provides significant gains in downlink and uplink user perceived throughput and coverage compared to a baseline deployment without IAB nodes and the same number of wired nodes.

- The gains are present for low, medium, and high load scenarios and different resource allocation approaches (including semi-static and dynamic TDM and SDM) which take into account a half-duplex constraint at the IAB nodes.

- Higher modulation order using 256QAM & 1024QAM has been evaluated and shown performance gains under the evaluated conditions for IAB

- In addition, it was shown that topology adaption based on for example loading or interference, provides benefits compared to a static IAB topology.

Agreements:

Capture the following conclusions for the IAB SI TR:

RAN1 has studied various physical layer aspects for Integrated Access and Backhaul, and from a RAN1 perspective, support for the following features and solutions has been determined to be beneficial and feasible:

- Mechanisms for discovery of IAB nodes and management of backhaul links in both SA and NSA deployments, taking into account the half-duplex constraint at an IAB node and multi-hop topologies, including:

  - Solutions reusing the same set of SSBs used for access UEs and solutions which use of SSBs which are orthogonal (TDM and/or FDM) with SSBs used for access UEs

  - CSI-RS-based IAB node discovery in synchronized deployments

  - Backhaul link RSRP/RSRQ RRM measurements which are SSB-based and CSI-RS based

  - Enhancements to support configuration of backhaul RACH resources with different occasions, longer RACH periodicities, and additional preamble formats allowing for longer RTT, compared to access RACH resources without impacting Rel-15 UEs

  - Enhancements to Beam Failure Recovery and Radio Link Failure procedures, including solutions to avoid RLF at a child IAB node due to parent backhaul link failure

- Mechanisms for supporting both in-band and out-of-band relaying by multiplexing access and backhaul links in time (TDM), frequency (FDM), or space (SDM) under a per-link half-duplex constraint at the IAB node and across multiple backhaul hops, including:

  - Semi-static configuration for IAB node DU resources

  - Dynamic indication to an IAB node of the availability of soft resources for an IAB node DU

  - Power control/coordination for FDM/SDM of access and backhaul links

- Over-the-air (OTA) timing alignment across multiple backhaul hops, including:

  - Mechanisms for DL timing alignment across IAB nodes

  - Alignment of an IAB node’s UL transmission timing and DL transmission timing

  - Alignment of an IAB node’s UL reception timing and DL reception timing

- Inter-IAB node cross-link interference (CLI) measurements and measurement coordination/configuration

- Support of up to 1024QAM for backhaul links

Agreements:

Capture the following recommendations for the IAB SI TR:

The following physical layer features and solutions are recommended to be specified as part of a Rel.16 IAB WI from a RAN1 perspective:

- SSB/RMSI periodicity values assumed by the IAB node MT for initial access

- Enhancements for use of SSBs which are orthogonal (TDM and/or FDM) with SSBs used for access UEs, including new periodicities and time-domain mapping/muting patterns

- Enhancements to support configuration of backhaul RACH resources with different occasion and longer RACH periodicities, compared to access RACH resources without impacting Rel-15 UEs.

- Efficient mechanisms for multiplexing access and backhaul traffic:

  - Semi-static configuration for IAB node DU resources in case of TDM operation subject to a half-duplex constraint, with further consideration for forward compability for potential support of FDM/SDM operation

  - Case 1 signaling and IAB node behavior is supported in Rel. 16

- Dynamic indication (L1 signalling) to an IAB node of the availability of soft resources for an IAB node DU

As a secondary priority:

- Enhancements to support additional preamble formats for PRACH allowing for longer RTT, without impacting Rel-15 UEs

- Efficient mechanisms for multiplexing access and backhaul traffic:

  - Semi-static configuration for IAB node DU resources in case of FDM/SDM operation is additionally supported

- Inter-IAB node CLI measurement coordination/configuration

  - It is expected that the definition of CLI measurements will be specified in a different WI and can be utilized for Inter-IAB node CLI measurement

Agreement of TP in R1-1814190: Capturing all RAN1 agreements and evaluation results in the TR

#### 2.1.2	Remaining Open issues

None.

## 2.2	RAN2

#### 2.2.1	Agreements

RAN2 #103bis

Agreement of TP in R2-1814371: A TP clarifies transport and security aspects for CP alternatives 1 to 5

Agreement of TP in R2-1814878: A comparison of Arch 1a CP alternatives clarifies the impact of each CP alternative on F1-AP and Adaptation layer placement.

Agreement of TP in R2-1815656: Based on R2-1814878, agreement was reached to down select CP alternatives to 2 and 4. The TP captures this agreement.

Agreement of TP in R2-1815656: A TP captures a proposal for a unified design, which combines support of many-to-one and one-to-one bearer mapping. The unified design ensures future compliance for IAB deployments.

Agreement of TP in R2-1816061: The TP discusses latency due to UL service requests and buffer status reports in multi-hop IAB and proposes optimizations.

Agreement of TP in R2-1816062: This TP establishes many-to-one and one-to-one mapping of SRBs in analogy to the mapping options for DRBs to RLC-channels.

Agreement of TP in R2-1815660: The TP considers encapsulation of F1-AP in an RRC header for CP alternative 2. This retains the legacy RRC/PDCP layering and avoid introducing a new F1-AP/PDCP layering.

Agreement of TP in R2-1815349: This TP discusses E2E reliability for hop-by-hop RLC ARQ. In certain topology adaptation scenarios, e.g. after BH RLF, hop-by-hop ARQ may lead to packet loss. This TP captures three options on how to avoiding packet loss in these scenarios together with a comparison table.

RAN2 #104

Agreement of TP in R2-1816873: This TP discusses two options for the IAB-node-MT’s UP transport, e.g. used for OAM connectivity. Two options are discussed. In one option, the MT uses a separate access RLC channel independently of BH RLC channels. In the other option, the MT piggybacks its access traffic onto its BH RLC channels with its parent.

Agreement of TP in R2-1816874: This TP includes two specific examples for the unified design including high-level features, potential variants, identifiers carried on the air interface and processing steps on IAB-node and IAB-donor DU.

Agreement of TP in R2-1818742: This TP provides two examples to secure F1*-U between IAB-node DU and CU. One example uses PDCP and is derived from CP alternative 2 and the other uses NDS and is derived from CP alternative 4. Both examples are compliant with unified design concept.

Agreement of TP in R2-1818763: This TP proposes removal or handling of “FFS” in TR 38.874.

Agreement of TP in R2-1818765: This TP discusses handling of BH RLF scenarios. It proposes notification of child nodes of upstream BH RLF and pre-emptive preparation for RLF recovery in response to such notification.

Agreement of TP in R2-1818790: This TP establishes the procedure for bearer setup as well as QoS parameters that need to be considered on the UE’s access IAB-node and on intermediate IAB-nodes.

Agreement of TP in R2-1818748: LS from RAN2 to SA3 to inquire on security mandate for F1*-U and consideration of NDS and PDCP for F1*-U security.

Agreement of TP in R2-1818931: LS reply by SA3 to RAN2 indicating that same rules apply for F1*-U as specified for F1-U in TS 33.501 and that NDS and PDCP may be considered.

Agreement of TP in R2-1818762: This TP includes RAN-2 recommendations for WI. This includes selection of unified design example 1, IP termination at the IAB-node, Adapt on top of RLC and hop-by-hop RLC ARQ.

The IAB study item was formally closed.

#### 2.2.2	Remaining Open issues

None.

## 2.3	RAN3

#### 2.3.1	Agreements

RAN2 #101bis

Agreement of TP in R3-186259: This TP captures a detailed comparison among architectures 1a, 1b and 2a. This comparison serves for architecture down selection in next following meeting.

Agreement of TP in R3-186208: This TP proposes removal or handling of “FFS” in TR 38.874.

Agreement of TP in R3-186260: This TP compares various RAN-3 related aspects of the 5 CP alternatives. This comparison serves for down selection in the next following meeting.

Agreement of TP in R3-186263: This TP discusses IAB set up and operation in NSA mode. It proposes to leverage EPC registration, LTE bearer setup, NR measurements and gNB selection using E-UTRAN procedures. It further proposes to conduct RRC and F1-AP between IAB-node and CU either via SRB3 on NR or via SRB1/2 on LTE and X2.

RAN2 #102

Agreement of TP in R3-186259: This TP captures a detailed comparison among architectures 1a, 1b and 2a. This comparison serves for architecture down selection in next following meeting.

Agreement of TP in R3-187183: This TP captures alternatives to IP address allocation for CP alternative 4 and it captures a brief problem description of NR backhauling of LTE access traffic.

Agreement of TP in R3-187184: This TP adds further description to the routing update during IAB-node integration.

Agreement of TP in R3-187186: This TP discusses inter-CU topology adaptation. This discussion complements intra-CU topology adaptation which has already been captured in the TR.

Agreement of TP in R3-187256: This TP proposes dynamic route selection in redundant topologies by using local information available to the IAB-node as well as via centralized configuration from the CU.

Agreement of TP in R3-187257: This TP discusses detailed procedures for BH RLF recovery for three scenarios, i.e. (1) via switching to an existing backup link/route, (2) via migration to a new parent at the same CU, and (3) via migration to a new parent at a different CU. The TP further introduces options to inform descendent nodes about upstream BH RLF.

Agreement of TP in R3-187272: This TP discusses RAN-3 recommendation for WI, which was based on architecture down-selection discussion before. RAN-3 decided to recommend architecture 1a. The TP further includes handling of multiple “FFS” references in TR.

The IAB study item was formally closed.

#### 2.3.2	Remaining Open issues

None.

## 2.4	RAN4

#### 2.4.1	Agreements

#### 2.4.2	Remaining Open issues

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN2 #103bis

| R2-1813709 | Selection of Parent for IAB-Node | vivo |
| --- | --- | --- |
| R2-1813710 | consideration of flow control | CATT |
| R2-1813711 | lossless transmission for hbh ARQ | CATT |
| R2-1813712 | Bearer mapping and QoS Handling for IAB | CATT |
| R2-1813866 | Comparison between Bearer Mapping Solutions | vivo |
| R2-1813867 | Discussion on further downselection of remaining IAB architecture options | Samsung R&D Institute UK |
| R2-1813868 | Solutions for reliable IAB end-to-end transmission for the hop-by-hop RLC ARQ case | Samsung R&D Institute UK |
| R2-1813883 | TP for TR 38.874 on L2 structures enabling one-to-one mapping between UE bearers and RLC BH channels | Samsung R&D Institute UK |
| R2-1813982 | Analysis of options for Adaptation layer placement | Intel Corporation |
| R2-1813983 | Downlink Flow Control and Congestion Control for IAB | Intel Corporation |
| R2-1813984 | Uplink Flow Control and Congestion Control for IAB | Intel Corporation, Convida Wireless |
| R2-1814069 | TR 38.874 | Qualcomm Inc (Rapporteur) |
| R2-1814070 | Comparison of IAB bearer mapping options | Qualcomm Inc |
| R2-1814071 | Comparison of Adapt with vs. without IP for IAB arch 1 | Qualcomm Inc |
| R2-1814072 | Correction to ARQ comparison table for IAB arch 1 | Qualcomm Inc |
| R2-1814073 | UP support for IAB node in arch 1a | Qualcomm Inc |
| R2-1814279 | The mobility in IAB | Potevio |
| R2-1814360 | End to end flow control in multi-hop IAB networks | Ericsson |
| R2-1814361 | Hop by hop flow control in multi-hop IAB networks | Ericsson |
| R2-1814362 | Way forward on RLC termination | Ericsson, AT&T, KDDI, LG, Kyocera, ZTE, Vivo, Intel |
| R2-1814363 | Logical channel grouping for BSR in IAB networks | Ericsson, AT&T, KDDI, LG, CATT, Kyocera, Intel |
| R2-1814364 | Way forward on Bearer Mapping | Ericsson, AT&T, KDDI, LG, CATT, ZTE, Kyocera, Intel |
| R2-1814365 | Uplink Scheduling in IAB networks | Ericsson |
| R2-1814366 | Supporting slicing in IAB networks | Ericsson |
| R2-1814367 | Way forward on CU-DU separation | Ericsson, AT&T, KDDI |
| R2-1814368 | Way forward on F1-AP termiantion | Ericsson, AT&T, KDDI, Qualcomm |
| R2-1814369 | Way forward on F1-U termination | Ericsson, AT&T, KDDI |
| R2-1814370 | Usage of SCTP over the IAB backhaul interface | Ericsson |
| R2-1814371 | Summary of email discussion [103#54] on CP message transport in IAB | Ericsson |
| R2-1814378 | E2E flow control based on ARQ mechanism | PANASONIC R&D Center Germany |
| R2-1814404 | Discussion on UE performing cell selection/reselection for IAB node | FibeHome |
| R2-1814407 | Consideration on IAB startup procedure | FiberHome |
| R2-1814483 | How to indicate which cell/IAB node support child IAB access | Lenovo, Motorola Mobility |
| R2-1814487 | Control Plane Latency issues in IAB | Intel Corporation |
| R2-1814496 | Text proposal for control plane latency issues in IAB | Intel Corporation |
| R2-1814501 | Radio link failure in backhaul link | Lenovo, Motorola Mobility |
| R2-1814530 | Setup procedure for adaptation layer | Ericsson |
| R2-1814592 | Overview of ARQ modes for NR relay networks | Samsung |
| R2-1814593 | Overview of flow control solutions for architecture 1 and 2 | Samsung |
| R2-1814651 | Consideration on Reliability Problem of Hop-by-hop RLC ARQ | ZTE Corporation |
| R2-1814684 | Comparison of hop by hop and end to end RLC ARQ | ZTE Corporation |
| R2-1814702 | Discussion on flow control in IAB | ZTE Corporation |
| R2-1814703 | Discussion on backhaul bearer setup in IAB network | ZTE Corporation |
| R2-1814722 | Latency analysis for IAB network | ZTE Corporation |
| R2-1814723 | Discussion on user plane bearer mapping and comparison | ZTE Corporation |
| R2-1814724 | Comparison of control plane alternatives of architecture 1a | ZTE Corporation |
| R2-1814728 | Discussion on bearer mapping for control plane signalling | ZTE Corporation |
| R2-1814730 | Discussion on IAB node discovery and selection | ZTE Corporation |
| R2-1814731 | Consideration on Routing in IAB | ZTE Corporation |
| R2-1814733 | Resource allocation in IAB | Sony |
| R2-1814734 | Route management in IAB | Sony |
| R2-1814735 | Open issues related to IAB power on procedure | Sony |
| R2-1814756 | Enhancement to CP Alt. 2 for Architecture 1a | Samsung |
| R2-1814757 | Control plane data mapping for Architecture 1a | Samsung |
| R2-1814758 | F1AP mapping options for IAB | Samsung |
| R2-1814819 | Backhual link selection for IAB access | Lenovo, Motorola Mobility |
| R2-1814821 | Analysis on topology in IAB system | Lenovo, Motorola Mobility |
| R2-1814838 | Consideration of RLF recovery in IAB | Kyocera |
| R2-1814878 | Comparison of Cplane protocol stack alternatives for Architecture 1a | Nokia, Nokia Shanghai Bell |
| R2-1814880 | IAB Architecture comparison | Nokia, Nokia Shanghai Bell |
| R2-1814881 | IAB Routing for Architecture 2a | Nokia, Nokia Shanghai Bell |
| R2-1814997 | IAB node discovery and monitoring | Nokia, Nokia Shanghai Bell |
| R2-1814998 | Service interruption time minimization for backhaul links | Nokia, Nokia Shanghai Bell |
| R2-1814999 | Adaptation layer in IAB MT | Nokia, Nokia Shanghai Bell |
| R2-1815000 | Comparison of Architecture 1a alternatives for user plane | Nokia, Nokia Shanghai Bell |
| R2-1815054 | Performance results for hop-by-hop vs. end-to-end RLC ARQ | AT&T |
| R2-1815055 | E2E reliability in hop-by-hop RLC ARQ | AT&T |
| R2-1815081 | LTE Access Impact on UP architectures | Sequans Communications |
| R2-1815083 | Queue Management vs Flow Control for Congestion Handling | Sequans Communications |
| R2-1815112 | Service Interruption Minimization during Topology Adaptation | ITRI |
| R2-1815132 | Flexible hop-by-hop RLC ARQ | LG Electronics Inc. |
| R2-1815135 | Topology management for IAB | KT Corp. |
| R2-1815161 | Handling of the RLF on wireless backhaul link | LG Electronics Inc. |
| R2-1815172 | Further consideration on uplink data congestion handling | LG Electronics Inc. |
| R2-1815173 | Further consideration on downlink data congestion handling | LG Electronics Inc. |
| R2-1815174 | Consideration on disorder of data arriving at the PDCP layer in IAB | LG Electronics Inc. |
| R2-1815176 | Consideration on SN length for multi-hop RLC ARQ | LG Electronics Inc. |
| R2-1815183 | Flow control for PDCP operation | Sequans Communications |
| R2-1815184 | Handling of the RLF on wireless backhaul link | LG Electronics Inc. |
| R2-1815192 | Scheduling enhancement in IAB | LG Electronics Inc. |
| R2-1815230 | Prioritization for IAB-node in random access procedure | ASUSTeK |
| R2-1815235 | Consideration on routing table update in IAB | ASUSTeK |
| R2-1815345 | Considerations on remaining “FFS” in the TR 38.874 | KDDI Corporation |
| R2-1815348 | Discussion for routing operation of IAB topologies | LG Electronics |
| R2-1815349 | Summary of [103#53][IAB] E2E reliability in hop-by-hop RLC ARQ | LG Electronics |
| R2-1815366 | Consideration on backhaul link enhancement for IAB | LG Electronics France |
| R2-1815394 | Summary of [103#53] E2E reliability in hop-by-hop RLC ARQ | LG Electronics |
| R2-1815423 | Supporting Prioritization of F1-AP message | LG Electronics |
| R2-1815437 | Access Control for IAB node | LG Electronics Inc. |
| R2-1815449 | Consideration on cell reselection of IAB | LG Electronics Inc. |
| R2-1815504 | Pre-BSR Enabling Fast Scheduling | Huawei Technologies France |
| R2-1815505 | Distributed RRC functions for IAB | Huawei Technologies France |
| R2-1815506 | Route selection method for architecture 1a | Huawei Technologies France |
| R2-1815509 | Some considerations about congestion handling for IAB networks | Huawei Technologies France |
| R2-1815513 | Some considerations about flow control for IAB networks | Huawei Technologies France |
| R2-1815516 | Comparison between Bearer Mapping Solutions | vivo |
| R2-1815517 | Selection of Parent for IAB-Node | vivo |
| R2-1815518 | QoS parameters for IAB QoS handling | Huawei Technologies France |
| R2-1815520 | IAB bearer mapping decisions | Huawei Technologies France |
| R2-1815530 | Discussion on flow control | ITL |
| R2-1815532 | Support of Multiple connectivity for IAB nodes | Futurewei Technologies |
| R2-1815533 | Analysis of impact of different ARQ modes on PDCP layer | Huawei Technologies France |
| R2-1815538 | Analysis of solutions for reliable transmission with HbH ARQ | Futurewei Technologies |
| R2-1815544 | SRB Types for Control Plane Alternative 2 | Huawei Technologies France |
| R2-1815545 | Adaptation layer design | Futurewei Technologies |
| R2-1815547 | Discussion on reassembly function for architecture 1a | Futurewei Technologies |
| R2-1815551 | Comparisons of C-plane alternatives | Huawei Technologies France |
| R2-1815553 | Further comparison between hop-by-hop ARQ and E2E ARQ | Futurewei Technologies |
| R2-1815590 | Performance results for hop-by-hop vs. end-to-end RLC ARQ | AT&T |
| R2-1815591 | E2E reliability in hop-by-hop RLC ARQ | AT&T |
| R2-1815660 | TP for TR 38.874 on Control Plane Alt 2 | Samsung Electronics R&D Institute UK |
| R2-1815791 | Consolidation of options & alternatives for IAB arch 1a | Qualcomm, AT&T, KDDI (Rapporteurs) |
| R2-1815930 | Unified design for IAB arch 1a | Qualcomm (Rapporteur) |
| R2-1815960 | Unified design for IAB arch 1a | Qualcomm (Rapporteur) |

RAN2 #104

| R2-1816298 | Overview of flow control solutions for architecture 1 and 2 | Samsung |
| --- | --- | --- |
| R2-1816299 | Unified design for IAB CP and UP | Samsung |
| R2-1816508 | Duplication support in IAB | vivo |
| R2-1816509 | Selection of Parent for IAB-Node | vivo |
| R2-1816558 | End to end flow control for IAB networks | Ericsson |
| R2-1816559 | Hop by hop flow control for IAB networks | Ericsson |
| R2-1816560 | Summary of email discussion [103bis_30][NR-IAB] Adaptation layer non-IP or IP | Ericsson |
| R2-1816561 | IAB node selection and reselection in RRC_IDLE | Ericsson |
| R2-1816562 | IAB node relocation | Ericsson |
| R2-1816563 | Mapping of CP data over backhaul links | Ericsson |
| R2-1816564 | Minimumizing CN functionalities for IAB network | Ericsson |
| R2-1816565 | Way forward on CU-DU separation | Ericsson, AT&T, KDDI |
| R2-1816566 | Considerations on SCTP overhead | Ericsson |
| R2-1816567 | Network slicing in IAB networks | Ericsson |
| R2-1816575 | Scheduling in IAB networks | Ericsson |
| R2-1816576 | User plane latency in IAB networks | Ericsson |
| R2-1816577 | PDCP Design Aspects for hop-by-hop | Ericsson |
| R2-1816578 | RLC UM design aspects | Ericsson |
| R2-1816579 | Suspension of Transmission upon Failure of Backhaul links | Ericsson |
| R2-1816580 | Setup Procedure for the Adaptation Layer of an IAB Network | Ericsson |
| R2-1816710 | E2E flow control based on ARQ mechanism | PANASONIC R&D Center Germany |
| R2-1816872 | TR 38.874 v060 | Qualcomm Inc. (Rapporteur) |
| R2-1816873 | Email discussion [103bis#31][NR - IAB] Adaptation layer in MT | Qualcomm Inc. (Rapporteur) |
| R2-1816874 | Email discussion [103bis#32][NR - IAB] Unified Design | Qualcomm Inc. (Rapporteur) |
| R2-1816881 | Text proposal for FFS resolution | KDDI Corporation |
| R2-1816885 | Unified design and QoS Handling for IAB | CATT |
| R2-1816887 | RLC ARQ comparison for IAB | CATT |
| R2-1816888 | Consideration of flow control | CATT |
| R2-1817072 | Resource allocation in IAB | Sony |
| R2-1817073 | Route management in IAB | Sony |
| R2-1817074 | Open issues related to IAB power on procedure | Sony |
| R2-1817169 | Parent node selection for IAB access | Lenovo, Motorola Mobility |
| R2-1817170 | RLF in backhaul link | Lenovo, Motorola Mobility |
| R2-1817271 | Topology Management for Spanning Tree topologies | Nokia, Nokia Shanghai Bell |
| R2-1817272 | Comparison of Arch 1a alternatives for user plane | Nokia, Nokia Shanghai Bell |
| R2-1817273 | Further discussion on unified design | Nokia, Nokia Shanghai Bell |
| R2-1817337 | Queue Management vs Flow Control for Congestion Handling | Sequans Communications |
| R2-1817341 | TP for TR 38.874 on bearer mapping for the unified design | Samsung R&D Institute UK |
| R2-1817344 | Bearer mapping for the unified design | Samsung R&D Institute UK |
| R2-1817349 | Downselection consideration on IAB architecture group 1 | ZTE Corporation |
| R2-1817394 | Consideration on IAB unified design | ZTE Corporation |
| R2-1817409 | Discussion on flow control in IAB | ZTE Corporation |
| R2-1817411 | Discussion on backhaul bearer setup in IAB network | ZTE Corporation |
| R2-1817413 | Discussion on user plane many-to-one bearer mapping | ZTE Corporation |
| R2-1817418 | Discussion on IAB node discovery and selection | ZTE Corporation |
| R2-1817419 | Consideration on Routing in IAB | ZTE Corporation |
| R2-1817420 | Control plane signaling delivery in NSA deployment | ZTE Corporation |
| R2-1817421 | Solutions for reliable IAB end-to-end transmission for the hop-by-hop RLC ARQ case | Samsung R&D Institute UK |
| R2-1817520 | Topology in IAB system | Lenovo, Motorola Mobility |
| R2-1817543 | Which cell/IAB node support child IAB access | Lenovo, Motorola Mobility |
| R2-1817573 | Consideration of RLF recovery in IAB | Kyocera |
| R2-1817616 | Discovery and measurements for IAB | Nokia, Nokia Shanghai Bell |
| R2-1817617 | Resource allocation and coordination for IAB | Nokia, Nokia Shanghai Bell |
| R2-1817697 | Downlink Flow Control and Congestion Control for IAB | Intel Corporation |
| R2-1817698 | Uplink Flow Control and Congestion Control for IAB | Intel Corporation, Convida |
| R2-1817699 | Route Adaptation Upon Backhaul Failure | Intel Corporation |
| R2-1817700 | Uplink Latency in IAB networks | Intel Corporation |
| R2-1817716 | Text proposal for Route Adaptation Upon Backhaul Failure | Intel Corporation |
| R2-1817769 | Comparison of Consolidated Unified Design Examples for Architecture 1a | AT&T |
| R2-1817775 | Route selection method for architecture 1a | Huawei Technologies France |
| R2-1817809 | Harmonizing UP alternatives for architecture group 1a | Ericsson, AT&T, KDDI, KT |
| R2-1817814 | Discussion on reassembly function for architecture 1a | Futurewei Technologies |
| R2-1817830 | Way Forward on Adaptation Layer Placement for IAB Network | Ericsson, AT&T, KDDI, CATT, LG, Intel, Lenovo, Motorola Mobility, Kyocera, ZTE, Sony, Verizon |
| R2-1817836 | CP signalling transmission in IAB NSA | Futurewei Technologies |
| R2-1817903 | Support of LTE Access over NR backhaul in Architecture 1a | Futurewei Technologies |
| R2-1817906 | IAB bearer mapping decisions | Huawei Technologies France |
| R2-1817916 | Way Forward on RLC termination | Ericsson, AT&T, KDDI, CATT, LG, Intel, Lenovo, Motorola Mobility, Kyocera, ZTE, Sony, Verizon |
| R2-1817917 | Using NDS for IAB_NB | Ericsson |
| R2-1817931 | QoS parameters for IAB QoS handling | Huawei Technologies France |
| R2-1817939 | LTE Access Impact on UP architectures | Sequans Communications |
| R2-1817990 | Service Interruption Minimization during Topology Adaptation | ITRI |
| R2-1818059 | Further consideration on uplink data congestion handling | LG Electronics Inc. |
| R2-1818060 | Further consideration on downlink data congestion handling | LG Electronics Inc. |
| R2-1818061 | Consideration on disorder of data arriving at the PDCP layer in IAB | LG Electronics Inc. |
| R2-1818062 | Consideration on SN length for multi-hop RLC ARQ | LG Electronics Inc. |
| R2-1818071 | Some considerations about flow control for IAB networks | Huawei Technologies France |
| R2-1818099 | pCR IAB_Arch_recommnedation(1a,1b and 2a) | KDDI Corporation |
| R2-1818105 | Flexible hop-by-hop RLC ARQ | LG Electronics Inc. |
| R2-1818166 | Some considerations about congestion handling for IAB networks | Huawei Technologies France |
| R2-1818209 | Discussion on flow control | ITL |
| R2-1818231 | Consideration on backhaul link enhancement for IAB | LG Electronics France |
| R2-1818277 | SRB Types for Control Plane Alternative 2 | Huawei Technologies France |
| R2-1818286 | Way forward on ARQ mode | Futurewei Technologies |
| R2-1818292 | Discussion on cell reselection of IAB nodes | LG Electronics Inc. |
| R2-1818317 | Comparisons of C-plane alternatives | Huawei Technologies France |
| R2-1818336 | Support of Multiple connectivity for IAB nodes | Futurewei Technologies |
| R2-1818367 | Handling of the RLF on wireless backhaul link | LG Electronics Inc. |
| R2-1818371 | Adapt function for unified design | Futurewei Technologies |
| R2-1818374 | Unified design for IAB | Futurewei Technologies |
| R2-1818377 | IAB routing and topology management for Architecture 1a | Nokia, Nokia Shanghai Bell |
| R2-1818383 | Way forward on UP alternatives of architecture group 1a | Futurewei Technologies |
| R2-1818386 | Comparison of IP vs non-IP routing for Arch 1a | Nokia, Nokia Shanghai Bell |
| R2-1818388 | Overview of changes required for Arch1a to support non 3GPP access and LTE Access | Nokia, Nokia Shanghai Bell |
| R2-1818415 | Access Control for IAB node | LG Electronics Inc. |
| R2-1818516 | Securing F1-U in IAB arch 1a | Qualcomm Inc. (Rapporteur) |
| R2-1818534 | TR 38.874 v061 | Qualcomm Inc. (Rapporteur) |
| R2-1818544 | IAB QoS implementation options and roadmap | Telstra Corporation Limited |
| R2-1818568 | TR 38.874 v062 | Qualcomm (Rapporteur) |
| R2-1818569 | Securing F1-U in IAB arch 1a | Qualcomm Inc. (Rapporteur) |
| R2-1818741 | Text proposal for FFS resolution | KDDI Corporation |
| R2-1818742 | Securing F1-U in IAB arch 1a | Qualcomm Inc. (Rapporteur) |
| R2-1818743 | [DRAFT] LS on IAB security | Ericsson |
| R2-1818744 | TP for TR 38.874 on bearer mapping for the unified design | Samsung R&D Institute UK |
| R2-1818745 | TP QoS parameters for IAB QoS handling | Huawei Technologies France |
| R2-1818746 | Route Adaptation Upon Backhaul Failure | Intel Corporation |
| R2-1818747 | IAB agreements without TP | Qualcomm (Rapporteur) |
| R2-1818748 | LS on IAB security | RAN2 |
| R2-1818762 | IAB agreements without TP | Qualcomm (Rapporteur) |
| R2-1818763 | Text proposal for FFS resolution | KDDI Corporation |
| R2-1818764 | TP QoS parameters for IAB QoS handling | Huawei Technologies France |
| R2-1818765 | Route Adaptation Upon Backhaul Failure | Intel Corporation |
| R2-1818790 | TP on QoS parameters for IAB QoS handling | Huawei Technologies France |
| R2-1818834 | Operators’ joint proposal on IP termination for the IAB architecture | KDDI, AT&T, NTT DOCOMO, Softbank |
| R2-1818889 | PDCP-secured F1* for IP termination at access-IAB-node | Qualcomm (Rapporteur) |
| R2-1818931 | Reply on LS on IAB security (S3-183711; contact: Ericsson) | SA3 |

RAN3 #101bis

| R3-185400 | Consideration on “CP alternatives for architecture 1a” with NSA operation | KDDI Corporation, Kyocera |
| --- | --- | --- |
| R3-185401 | Considerations on remaining “FFS” in the TR 38.874 | KDDI Corporation |
| R3-185402 | Use of Multi-connectivity for Load Balancing and Reliability in IAB | OMESH Networks |
| R3-185403 | Removal of the FFS on IAB Routing update procedure | KDDI Corporation |
| R3-185404 | On Reliability and Load Balancing in IAB | OMESH Networks |
| R3-185462 | TP for TR 38.874 on recovery procedures for different IAB failure scenarios | Samsung R&D Institute UK |
| R3-185463 | Discussion on further downselection of remaining IAB architecture options | Samsung R&D Institute UK |
| R3-185518 | TR 38874 v050 | Qualcomm Incorporated (Rapporteur) |
| R3-185519 | IAB work plan | Qualcomm Incorporated, Samsung, AT&T, KDDI (Rapporteurs) |
| R3-185520 | Comparison of IAB architecture groups | Qualcomm Incorporated |
| R3-185521 | Comparison of IAB architectures 1a and 1b | Qualcomm Incorporated |
| R3-185522 | IAB backhaul RLF recovery for arch 1a | Qualcomm Incorporated, Ericsson, AT&T |
| R3-185534 | Comparison of architecture 1a and 1b | ZTE Corporation |
| R3-185535 | Comparison of IAB architecture group 1 and architecture group 2 | ZTE Corporation |
| R3-185536 | Control plane signaling delivery in NSA deployment | ZTE Corporation |
| R3-185537 | Discussion on backhaul bearer setup in IAB network | ZTE Corporation |
| R3-185538 | Discussion on IAB node discovery and selection | ZTE Corporation |
| R3-185539 | Discussion on backhaul link failure and recovery | ZTE Corporation |
| R3-185551 | Setup and connection establishment for IAB with NSA | AT&T |
| R3-185552 | Performance results for hop-by-hop vs. end-to-end RLC ARQ | AT&T |
| R3-185560 | Discussion on IAB failure | CATT |
| R3-185684 | IAB solution down-selection criteria | Intel Corporation |
| R3-185685 | IAB flow control | Intel Corporation |
| R3-185686 | IAB route adaptation upon RLF | Intel Corporation |
| R3-185690 | Comparison of Arch 1a alternatives for user plane | Nokia, Nokia Shanghai Bell |
| R3-185692 | Comparison of Arch 1a alternatives for C-plane | Nokia, Nokia Shanghai Bell |
| R3-185693 | IAB Architecture comparison | Nokia, Nokia Shanghai Bell |
| R3-185695 | QoS for Architecture 2a | Nokia, Nokia Shanghai Bell |
| R3-185696 | IAB Node Integration Option 2a | Nokia, Nokia Shanghai Bell |
| R3-185697 | Discussion on Inter-CU Topology Adaptation | Nokia, Nokia Shanghai Bell |
| R3-185698 | IAB Routing for Architecture 2a | Nokia, Nokia Shanghai Bell |
| R3-185731 | Enhancement to CP Alt. 2 for Architecture 1a | Samsung |
| R3-185732 | F1AP mapping options for IAB | Samsung |
| R3-185733 | Control plane data mapping for Architecture 1a | Samsung |
| R3-185827 | TP to TR 38.874: Comparison of CP Alternatives of Architecture 1a | Ericsson |
| R3-185828 | Usage of SCTP Over the IAB Backhaul Interface | Ericsson |
| R3-185829 | TP to TR 38.874: Way Forward on CU-DU Separation | Ericsson, AT&T, KDDI |
| R3-185830 | TP to TR 38.874: Way Forward on F1-AP Termination for CP Alternatives of Architecture Group 1a | Ericsson, AT&T, KDDI, Qualcomm |
| R3-185831 | TP to TR 38.874: Way Forward on F1-U Termination | Ericsson, AT&T, KDDI |
| R3-185832 | TP to TR 38.874: IAB node Access Procedure for Architecture Group 1 | Ericsson |
| R3-185833 | TP to TR 38.874: Setup Procedure for the Adaptation Layer of an IAB Network | Ericsson |
| R3-185834 | TP to TR 38.874: End-to-end Flow Control in Multi-hop IAB Networks | Ericsson |
| R3-185835 | TP to 38.874: Hop-by-hop Flow Control in Multi-hop IAB Networks | Ericsson |
| R3-185836 | TP to TR 38.874: Supporting Network Slicing in IAB Networks | Ericsson |
| R3-185837 | TP to TR 38.874: Using F1 Functionality to Support Redudant Paths | Ericsson, AT&T, KDDI |
| R3-185967 | SRB Types for Control Plane Alternative 2 | Huawei |
| R3-185968 | Route selection method for architecture 1a | Huawei |
| R3-185969 | Topology Table Establishment and Update | Huawei |
| R3-185999 | IAB bearer mapping decisions | Huawei |
| R3-186000 | QoS parameters for IAB QoS handling | Huawei |
| R3-186001 | IAB for vehicular networks | Mitsubishi Electric RCE |
| R3-186009 | Further Consideration on Multiple connectivity for IAB nodes | Huawei |
| R3-186014 | Slice support of IAB nodes | Huawei |
| R3-186016 | Comparisons of C-plane alternatives | Huawei |
| R3-186021 | Distributed RRC functions for IAB | Huawei |
| R3-186022 | Comparison with proposed IAB architectures | LG Electronics Inc. |
| R3-186023 | Comparison with CP alternatives for architecture 1a | LG Electronics Inc. |
| R3-186095 | Performance results for hop-by-hop vs. end-to-end RLC ARQ | AT&T |
| R3-186208 | Considerations on remaining “FFS” in the TR 38.874 | KDDI Corporation |
| R3-186209 | pCR on comparison of IAB architectures | Qualcomm |
| R3-186210 | TP on IAB with NSA operation | AT&T |
| R3-186211 | TP to TR 38.874: Way Forward on F1-AP Termination for CP Alternatives of Architecture Group 1a | Ericsson, AT&T, KDDI, Qualcomm |
| R3-186259 | pCR on comparison of IAB architectures | Qualcomm |
| R3-186260 | TP to TR 38.874: Way Forward on F1-AP Termination for CP Alternatives of Architecture Group 1a | Ericsson, AT&T, KDDI, Qualcomm |
| R3-186263 | TP on IAB with NSA operation | AT&T |

RAN3 #102

| R3-186316 | Removal of the FFS on IAB Routing update procedure | KDDI Corporation |
| --- | --- | --- |
| R3-186317 | Detailed design on topology redundancy | KDDI Corporation |
| R3-186318 | Text proposal for FFS resolution | KDDI Corporation |
| R3-186371 | Discussion on IAB failure | CATT |
| R3-186416 | Downselection consideration on IAB architecture group 1 | ZTE Corporation |
| R3-186417 | Consideration on IAB architecture group 1 and 2 | ZTE Corporation |
| R3-186418 | Discussion on user plane many-to-one bearer mapping | ZTE Corporation |
| R3-186419 | Discussion on backhaul bearer setup in IAB network | ZTE Corporation |
| R3-186420 | Discussion on IAB node discovery and selection | ZTE Corporation |
| R3-186421 | Discussion on backhaul link failure and recovery | ZTE Corporation |
| R3-186440 | TP for Soft Topology Adaption | OMESH Technologies Co. Ltd. |
| R3-186441 | TP on Soft Topology Adaptation | OMESH Technologies Co. Ltd. |
| R3-186454 | TR 38.874 v060 | Qualcomm Incorporated |
| R3-186455 | IAB backhaul RLF recovery for arch 1a | Qualcomm Incorporated, Ericsson, AT&T, Nokia, Huawei, Samsung |
| R3-186456 | IAB inter-CU Topology Adaptation for Arch 1a | Qualcomm Incorporated, Nokia, Samsung |
| R3-186507 | pCR IAB_Arch_recommnedation(1a,1b and 2a) | KDDI Corporation |
| R3-186520 | Route selection method for architecture 1a | Huawei |
| R3-186524 | Topology Table Establishment and Update | Huawei |
| R3-186529 | Principal Steps of Inter-CU Topology Adaptation | Huawei |
| R3-186538 | CP signalling transmission in IAB NSA | Huawei |
| R3-186543 | Support of LTE Access over NR backhaul in Architecture 1a | Huawei |
| R3-186544 | Slice Requirement Assurance for IAB E2E Link | Huawei |
| R3-186545 | IAB bearer mapping decisions | Huawei |
| R3-186564 | QoS parameters for IAB QoS handling | Huawei |
| R3-186565 | SRB Types for Control Plane Alternative 2 | Huawei |
| R3-186571 | IAB architecture down-selection | Samsung |
| R3-186585 | Way forward on CP alternatives of architecture group 1a | Huawei |
| R3-186586 | Support of Multiple connectivity for IAB nodes | Huawei |
| R3-186624 | Xn termination and Xn based mobility in IAB | CMCC |
| R3-186664 | Analysis on IPv6 address management in Architetcure Group 1a | Nokia, Nokia Shanghai Bell |
| R3-186665 | IAB Routing and Topology Management for Architecture 1a | Nokia, Nokia Shanghai Bell |
| R3-186708 | IAB architecture down-selection | Samsung |
| R3-186738 | Mapping of CP data over backhaul links in IAB Architecture group 1a | Ericsson |
| R3-186739 | Using NDS to protect F1 signaling without any standardization impact | Ericsson |
| R3-186740 | P to TR 38.874: Harmonizing UP alternatives for Architecture group 1a | Ericsson, AT&T, KDDI, KT |
| R3-186741 | TP to TR 38.874: IAB node Access Procedure for Architecture Group 1 | Ericsson |
| R3-186742 | TP to TR 38.874: Setup Procedure for the Adaptation Layer of an IAB Network | Ericsson |
| R3-186743 | TP to TR 38.874: End-to-end Flow Control in Multi-hop IAB Networks | Ericsson |
| R3-186744 | TP to 38.874: Hop-by-hop Flow Control in Multi-hop IAB Networks | Ericsson |
| R3-186745 | TP to TR 38.874: Supporting Network Slicing in IAB Networks | Ericsson |
| R3-186746 | TP to TR 38.874: Minimizing the CN functionalities for IAB network | Ericsson |
| R3-186747 | TP to TR 38.874: Using F1 Functionality to Support Redudant Paths | Ericsson, AT&T, KDDI |
| R3-186750 | Selection of IAB architecture for future work | LG Electronics Inc. |
| R3-186751 | Inter-donor topology adaptation for architecture 1a | LG Electronics Inc. |
| R3-186761 | Architecture considerations for IAB | AT&T |
| R3-187007 | Route Adaptation upon Backhaul Failure | Intel Corporation |
| R3-187031 | IAB QoS implementation options and roadmap | telstra |
| R3-187091 | TR 38.874 v062 | Qualcomm Incorporated |
| R3-187164 | Revision of recommendations by RAN-2 for IAB WI | Qualcomm Incorporated |
| R3-187166 | Text proposal for FFS resolution (RAN2 agreed) | KDDI Corporation |
| R3-187182 | Revision of recommendations by RAN-2 for IAB WI | Qualcomm Incorporated |
| R3-187183 | pCR for TR38.874: CB63_TR_IABenhancements | Qualcomm |
| R3-187184 | Removal of the FFS on IAB Routing update procedure | KDDI Corporation |
| R3-187185 | TP to TR 38.874: Setup Procedure for the Adaptation Layer of an IAB Network | Ericsson |
| R3-187186 | IAB inter-CU Topology Adaptation for Arch 1a | Qualcomm Incorporated, Nokia, Samsung |
| R3-187188 | TP to TR 38.874: Using F1 Functionality to Support Redudant Paths | Ericsson, AT&T, KDDI |
| R3-187189 | IAB backhaul RLF recovery for arch 1a | Qualcomm Incorporated, Ericsson, AT&T, Nokia, Huawei, Samsung |
| R3-187190 | Route selection method for architecture 1a | Huawei |
| R3-187256 | Route selection method for architecture 1a | Huawei, OMESH Technologies Co. Ltd. |
| R3-187257 | IAB backhaul RLF recovery for arch 1a | Qualcomm Incorporated, Ericsson, AT&T, Nokia, Huawei, Samsung |
| R3-187272 | Revision of recommendations by RAN-2 for IAB WI | Qualcomm Incorporated |

RAN1 #94bis

| R1-1810130 | Physical layer design for NR IAB | Huawei, HiSilicon |
| --- | --- | --- |
| R1-1810131 | System performance evaluation in multi-hop topology IAB network | Huawei, HiSilicon |
| R1-1810132 | On frame structure of IAB | Huawei, HiSilicon |
| R1-1810133 | Further consideration on timing alignment for IAB nodes | Huawei, HiSilicon |
| R1-1810134 | On cross-link interference in IAB | Huawei, HiSilicon |
| R1-1810135 | On high order modulation for backhaul link | Huawei, HiSilicon |
| R1-1810273 | Discussions on mechanisms to support NR IAB scenarios | LG Electronics |
| R1-1810274 | Discussions on detection and measurement among IAB nodes | LG Electronics |
| R1-1810275 | Discussions on access and backhaul link multiplexing and timing | LG Electronics |
| R1-1810276 | Power control in IAB scenario | LG Electronics |
| R1-1810277 | Measurement enhancement for stage 2 in IAB scenario | LG Electronics |
| R1-1810278 | Discussions on node behavior for IAB link management | LG Electronics |
| R1-1810279 | Discussions on HARQ-ACK transmission for NR IAB | LG Electronics |
| R1-1810387 | Enhancements to support NR backhaul links | vivo |
| R1-1810538 | NR Physical Layer design for IAB backhual link | CATT |
| R1-1810577 | Discussion on resource partitioning for IAB network | Lenovo, Motorola Mobility |
| R1-1810578 | Discussion on discovery and measurement for IAB network | Lenovo, Motorola Mobility |
| R1-1810673 | NR enhancements to support IAB | Nokia, Nokia Shanghai Bell |
| R1-1810674 | Evaluation of IAB | Nokia, Nokia Shanghai Bell |
| R1-1810675 | Resource allocation/coordination between parent BH and child links | Nokia, Nokia Shanghai Bell |
| R1-1810676 | Enhacements on RACH for IAB | Nokia, Nokia Shanghai Bell |
| R1-1810677 | Discovery and measurements for IAB | Nokia, Nokia Shanghai Bell |
| R1-1810678 | IAB Timing | Nokia, Nokia Shanghai Bell |
| R1-1810679 | Cross-link interference management | Nokia, Nokia Shanghai Bell |
| R1-1810680 | Considerations of single panel vs. multi-panel operation for IAB node | Nokia, Nokia Shanghai Bell |
| R1-1810690 | Enhancements to support NR backhaul links | AT&T |
| R1-1810691 | Evaluation results for NR IAB | AT&T |
| R1-1810770 | PHY enhancements on NR IAB | Intel Corporation |
| R1-1810771 | Evalutation methodology and results on NR IAB | Intel Corporation |
| R1-1810806 | Discussion on IAB | NEC |
| R1-1810864 | Necessary Enhancements for NR IAB | Samsung |
| R1-1810865 | Evaluation Methodology for NR IAB | Samsung |
| R1-1810866 | CLI management in NR IAB | Samsung |
| R1-1811035 | Discussions on enhancements to support NR Backhaul links | CMCC |
| R1-1811135 | Discussion on Enhancements to support NR backhaul links | Sony |
| R1-1811160 | Discussion on physical layer enhancements for NR IAB | ZTE, Sanechips |
| R1-1811161 | Further consideration on CLI | ZTE, Sanechips |
| R1-1811166 | Discussion on Timing Alignment for IAB nodes | ASUSTEK COMPUTER (SHANGHAI) |
| R1-1811196 | IAB evaluations based on different topology constructions | ZTE, Sanechips |
| R1-1811208 | On Integrated Access Link and Backhaul in NR | InterDigital, Inc. |
| R1-1811256 | Enhancements to support NR backhaul links | Qualcomm Incorporated |
| R1-1811257 | System Level Simulation Results for IAB Networks | Qualcomm Incorporated |
| R1-1811258 | IAB resource management | Qualcomm Incorporated |
| R1-1811259 | Network Synchronization for Multi-hop IAB Networks | Qualcomm Incorporated |
| R1-1811311 | TR38.874, Version 0.5.0 | AT&T, KDDI, Qualcomm, Samsung |
| R1-1811359 | Discussion on enhancements to support NR backhaul links | NTT DOCOMO, INC. |
| R1-1811449 | Timing of IAB-node transmissions | Ericsson |
| R1-1811455 | What is uplink in the context of IAB nodes | Ericsson |
| R1-1811512 | Physical layer aspects of IAB | Ericsson |
| R1-1811513 | Resource allocation and scheduling of IAB networks | Ericsson |
| R1-1811514 | Evaluation of topology formation for IAB | Ericsson |
| R1-1811519 | Discussion on enhancement of IAB discovery and Resource allocation | CAICT |
| R1-1811554 | On IAB timing alignment | CEWiT |
| R1-1811813 | Evaluation Methodology for NR IAB | Samsung |
| R1-1811844 | Summary of 7.2.3.1 Enhancements to support NR backhaul links | Ericsson |
| R1-1811899 | Updated summary of 7.2.3.1 Enhancements to support NR backhaul links | Ericsson |
| R1-1811954 | TP for 38.874 on Evaluation Results for NR IAB | AT&T |
| R1-1811972 | Updated summary of 7.2.3.1 Enhancements to support NR backhaul links | Ericsson |
| R1-1812024 | Updated summary of 7.2.3.1 Enhancements to support NR backhaul links | Ericsson |
| R1-1812042 | Updated summary of 7.2.3.1 Enhancements to support NR backhaul links | Ericsson |
| R1-1812050 | TP for 38.874 on Initial Access Enhancements for NR IAB | AT&T |
| R1-1812078 | TP for 38.874 on PHY Enhancements for NR IAB | AT&T |

RAN1 #95

| R1-1812198 | Physical layer design for NR IAB | Huawei, HiSilicon |
| --- | --- | --- |
| R1-1812199 | System performance evaluation in multi-hop IAB network | Huawei, HiSilicon |
| R1-1812200 | On frame structure in IAB | Huawei, HiSilicon |
| R1-1812201 | On resource coordination and dynamic scheduling in IAB | Huawei, HiSilicon |
| R1-1812202 | On IAB node synchronization and timing alignment | Huawei, HiSilicon |
| R1-1812203 | Consideration on cross-link interference in IAB | Huawei, HiSilicon |
| R1-1812204 | On high order modulation in IAB | Huawei, HiSilicon |
| R1-1812305 | Enhancements to support NR backhaul links | vivo |
| R1-1812486 | PHY enhancements for NR IAB | Intel Corporation |
| R1-1812487 | Evaluation methodology and results on NR IAB | Intel Corporation |
| R1-1812564 | Discussions on mechanisms to support NR IAB scenarios | LG Electronics |
| R1-1812565 | Discussions on detection and measurement among IAB nodes | LG Electronics |
| R1-1812566 | Discussions on access and backhaul link multiplexing and timing | LG Electronics |
| R1-1812567 | Discussions on node behavior for IAB link management | LG Electronics |
| R1-1812568 | Power control in IAB scenario | LG Electronics |
| R1-1812647 | Discussion on IAB | NEC |
| R1-1812675 | Text Proposal for sections 7 of TR 38.874 | Huawei, HiSilicon |
| R1-1812700 | NR enhancements to support IAB | Nokia, Nokia Shanghai Bell |
| R1-1812701 | Evaluation of IAB | Nokia, Nokia Shanghai Bell |
| R1-1812702 | Resource allocation/coordination between Parent BH and Child links | Nokia, Nokia Shanghai Bell |
| R1-1812703 | IAB Timing | Nokia, Nokia Shanghai Bell |
| R1-1812704 | Discovery and measurements for IAB | Nokia, Nokia Shanghai Bell |
| R1-1812705 | Enhancements on RACH for IAB | Nokia, Nokia Shanghai Bell |
| R1-1812706 | Considerations of single panel vs. multi-panel operation for IAB node | Nokia, Nokia Shanghai Bell |
| R1-1812707 | On Cross-link Interference Management | Nokia, Nokia Shanghai Bell |
| R1-1812729 | Discussion on physical layer enhancements for NR IAB | ZTE, Sanechips |
| R1-1812786 | Discussion on resource partitioning for IAB network | Lenovo, Motorola Mobility |
| R1-1812787 | Discussion on discovery and measurement for IAB network | Lenovo, Motorola Mobility |
| R1-1812858 | TR38.874, Version 0.6.0 | AT&T, KDDI, Qualcomm, Samsung |
| R1-1812859 | Enhancements to support NR backhaul links | AT&T |
| R1-1812860 | Evaluation results for NR IAB | AT&T |
| R1-1812878 | Discussions on enhancements to support NR Backhaul links | CMCC |
| R1-1812981 | Necessary Enhancements for NR IAB | Samsung |
| R1-1812982 | Evaluation Results for NR IAB | Samsung |
| R1-1812983 | CLI Management in NR IAB | Samsung |
| R1-1813209 | Enhancements to Robustize IAB Networks to Radio Link Failure | Sharp |
| R1-1813224 | Discussion on Integrated Access and Backhaul Links in NR | InterDigital, Inc. |
| R1-1813225 | On Integrated Access and Backhaul Enhancements in NR | InterDigital, Inc. |
| R1-1813316 | Discussion on enhancements to support NR backhaul links | NTT DOCOMO, INC. |
| R1-1813359 | Further evaluations on NR IAB | ZTE, Sanechips |
| R1-1813417 | Enhancements to support NR backhaul links | Qualcomm Incorporated |
| R1-1813418 | System Level Simulation Results for IAB Networks | Qualcomm Incorporated |
| R1-1813419 | IAB resource management | Qualcomm Incorporated |
| R1-1813420 | Network Synchronization for Multi-hop IAB Networks | Qualcomm Incorporated |
| R1-1813504 | Discussion on enhancement of IAB discovery and Resource allocation | CAICT |
| R1-1813566 | IAB resource allocation and multiplexing | Ericsson |
| R1-1813568 | Timing of IAB node transmissions | Ericsson |
| R1-1813570 | Deriving Child Node Timing from Multiple Parent IAB Nodes | Ericsson |
| R1-1813571 | IAB node discovery | Ericsson |
| R1-1813572 | IAB - Physical layer enhancements | Ericsson |
| R1-1813573 | Evaluation of multi-hop IAB networks | Ericsson |
| R1-1813864 | Updated text proposals to TR38.874, Version 0.6.2 | AT&T, KDDI, Qualcomm, Samsung |
| R1-1813975 | Summary of 7.2.3.1 Enhancements to support NR backhaul links | AT&T |
| R1-1814060 | Evaluation Results for NR IAB | Samsung |
| R1-1814090 | IAB resource allocation and multiplexing | Ericsson |
| R1-1814091 | IAB - Physical layer enhancements | Ericsson |
| R1-1814127 | Summary of 7.2.3.1 Enhancements to support NR backhaul links | AT&T |
| R1-1814154 | Timing of IAB node transmissions | Ericsson |
| R1-1814190 | TP for 38.874 on PHY Enhancements for NR IAB | AT&T |

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template