3GPP TSG-RAN meeting #82	RP-182757

Sorrento, Italy, December 10th – 13th, 2018

Agenda Item:	9.3.6

Source:	FirstNet, AT&T, The UK Home Office, Telstra

Title:	Mission Critical Services and NR V2X

Document for:	Discussion, Decision

# 1	Introduction

The NR V2X Study item is scheduled to conclude at RAN#83 (March 2019) and be followed by a work item as part of NR Release 16. As a part of this work, an NR sidelink for V2X services was introduced. In this paper we argue for the support of ProSe public safety (PS) services (MCPTT, MCVideo, and MCData) as another use case for the NR sidelink V2X.

# 2	Discussion

ProSe D2D functionality was added to the 3GPP system in Release 12. The driving use case behind this was the need from public safety responder communities to replace traditional narrow-band mobile radio communication systems and add richer communication with video streaming, data services etc. In 3GPP RAN, the new form of communication link was introduced, called the sidelink. In releases 13 and 14, this service was extended to commercial use cases and V2X. V2X services include communication between vehicles and communication between vehicles and road side units (RSUs) which can provide an advanced set of services targeted to enhance safety and efficiency of road transportation.

In Rel-16 the introduction of sidelink is being investigated in the V2X study item. The services studied so far in the V2X work include both periodic services (e.g. vehicular co-operative awareness messaging, CAM) and aperiodic services (e.g. sensor and data sharing). We note that there is no explicit mention of the ProSe public safety services in the V2X SI, so support for it cannot be expected in Rel-16 as per the current plan. However, we expect the amount of work required to support ProSe PS services in Rel-16 to be small given the work planned in NR V2X.

Some of the ProSe public safety functionalities can leverage the recent progress of the V2X SI. For example, group management is currently discussed in the V2X SI as a key enabler of platoon use cases. Even though it is expected that MCPTT may be characterized by different traffic types compared with V2X, e.g. voice, it is foreseen that to a large extent, group management procedures defined by RAN WGs in the V2X SI could be reused for MCPTT use cases.

Another important aspect is the handling of Quality of Service. In the V2X SI, a SL QoS framework is currently being investigated in both RAN and SA WGs. We expect that the fundamental principles of such SL QoS framework defined in V2X can be beneficial in the context of ProSe public safety services.  Also, please note that SA6 WG has initiated a study on the support of Mission Critical Services over 5GS (FS_MCOver5GS) which is expected to be completed in TSG#85 (09/2019).

# 4	Conclusion and proposal

Based on the above discussion we propose that the support of ProSe public safety services (MCPTT, MCVideo, and MCData) be included in the scope of the upcoming NR V2X work item.