3GPP TSG-RAN4 WG4 Meeting #90	R4-1901056

# Athens, Greece,  Feb.25 - Mar.01, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 5361 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR: Addition of performance requirements for FeNB-IoT TDD NPBCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-Perf |  |  |  |  |  | Date: |  |  | 2018-12-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As per the agreement R4-1811370: Reuse the NPBCH FDD performance requirements for NPBCH TDD, but with the RMC and test parameters updates for TDD. Considering the differences between NPBCH FDD and NPBCH TDD are only the NSSS and NPBCH locations, from demodulation performance point of view, no difference between FDD and TDD for NPBCH performance. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the performance requirements for NPBCH TDD. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance requirements for NPBCH TDD will still be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.12.3.2(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

8.12.3	Demodulation of NPBCH

The receiver characteristics of the NPBCH are determined by the probability of miss-detection of the NPBCH (Pm-bch), which is defined as

For the performance with single a NPBCH TTI decoding, A is the number of correctly decoded MIB-NB PDUs and B is the number of transmitted MIB-NB PDUs.

For the performance with multiple NPBCH TTIs decoding, A is the number of 5120ms durations consisting of contiguous NPBCH TTIs where there is at least one correctly decoded MIB-NB PDU, and B is the number of 5120ms durations consisting of contiguous NPBCH TTIs during the test.

8.12.3.1	HD-FDD

Table 8.12.3.1-1: Test Parameters for NPBCH

| Parameter |  | Unit | Single antenna port | Transmit diversity |
| --- | --- | --- | --- | --- |
| Downlink power allocation | NPBCH_RA | dB | 0 | -3 |
|  | NPBCH_RB | dB | 0 | -3 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Cyclic prefix |  |  | Normal | Normal |
| Cell ID |  |  | 0 | 0 |

8.12.3.1.1	Single-antenna port performance with single NPBCH TTI

For the parameters specified in Table 8.12.3.1-1 the average probability of a miss-detecting NPBCH (Pm-bch) shall be below the specified value in Table 8.12.3.1.1-1. The downlink physical setup is in accordance with Annex C.3.6.

Table 8.12.3.1.1-1: Minimum performance NPBCH

| Test number | Bandwidth | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) |  |
| 1 | 200 KHz | R.NB1.1 | EPA1 | 1 x 1 | 1 | -2.0 | NB1, NB2 |

8.12.3.1.2	Transmit diversity performance

8.12.3.1.2.1	Minimum Requirement 2 Tx Antenna Port with multiple NPBCH TTIs

For the parameters specified in Table 8.12.3.1-1 the average probability of a miss-detected NPBCH (Pm-bch) shall be below the specified value in Table 8.12.3.1.2.1-1. The downlink physical setup is in accordance with Annex C.3.6.

Table 8.12.3.1.2.1-1: Minimum performance NPBCH

| Test number | Bandwidth | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) |  |
| 1 | 200 KHz | R.NB1.2 | EPA1 | 2 x 1 | 1 | -11.5 | NB1, NB2 |

8.12.3.2	TDD

Table 8.12.3.2-1: Test Parameters for NPBCH TDD

| Parameter |  | Unit | Single antenna port | Transmit diversity |
| --- | --- | --- | --- | --- |
| Downlink power allocation | NPBCH_RA | dB | 0 | -3 |
|  | NPBCH_RB | dB | 0 | -3 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Cyclic prefix |  |  | Normal | Normal |
| Cell ID |  |  | 0 | 0 |
| Uplink downlink configuration (Note 1) |  |  | 4 | 4 |
| Special subframe configuration (Note 2) |  |  | 4 | 4 |
| Note 1: as specified in Table 4.2-2 in TS 36.211 [4]. Note 2: as specified in Table 4.2-1 in TS 36.211 [4]. |  |  |  |  |

8.12.3.2.1	Single-antenna port performance with single NPBCH TTI

For the parameters specified in Table 8.12.3.2-1 the average probability of a miss-detecting NPBCH (Pm-bch) shall be below the specified value in Table 8.12.3.2.1-1. The downlink physical setup is in accordance with Annex C.3.6.

Table 8.12.3.2.1-1: Minimum performance NPBCH

| Test number | Bandwidth | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) |  |
| 1 | 200 KHz | R.NB1.1 | EPA1 | 1 x 1 | 1 | -2.0 | NB1, NB2 |

8.12.3.2.2	Transmit diversity performance

8.12.3.2.2.1	Minimum Requirement 2 Tx Antenna Port with multiple NPBCH TTIs

For the parameters specified in Table 8.12.3.2-1 the average probability of a miss-detected NPBCH (Pm-bch) shall be below the specified value in Table 8.12.3.2.2.1-1. The downlink physical setup is in accordance with Annex C.3.6.

Table 8.12.3.2.2.1-1: Minimum performance NPBCH

| Test number | Bandwidth | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) |  |
| 1 | 200 KHz | R.NB1.2 | EPA1 | 2 x 1 | 1 | -11.5 | NB1, NB2 |