3GPP TSG-RAN WG4 Meeting #90 	R4-1901350

Athens, Greece, 25th Feb -  1st Mar 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 5385 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TDD requirements for network-based CRS interference mitigation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NW_CRS_IM-Perf |  |  |  |  |  | Date: |  |  | 2019-03-01 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR is to introduce the new TDD requirement for network based CRS-IM test case, together to have some editorial changes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New requirement for TM3 TDD test is added for PDSCH UE demod tests for nw-based CRS-IM (See R4-1902048). FRC index is updated for both FDD and TDD together with the overview table in Annex updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance requirements with network CRS-IM can’t be verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.3.6, 8.2.2.3.6, Annex |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<start of the change>

##### 8.2.1.3.6	Minimum Requirement 2 Tx Antenna Port (network-based CRS interference mitigation)

The requirements are specified in Table 8.2.1.3.6-2, with the addition of parameters in Table 8.2.1.3.6-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of open-loop spatial multiplexing performence with 2 transmit antennas when the PDSCH transmission in the serving cell is interfered by CRS of one dominant interfering cell with network-based CRS interference mitigation. In Table 8.2.1.3.6-1, Cell 1 is the serving cell, and Cell 2 is interfering cell. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1 and Cell 2 respectively.

Table 8.2.1.3.6-1: Test parameters for Larger Delay CDD (FRC) with network-based CRS interference mitigation

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz |  |
| Downlink power allocation |  | dB | -3 | 0 |
|  |  |  | -3 (Note 1) | 0 |
|  |  |  | 0 | 0 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna port 0,1 |
| Cyclic Prefix |  |  | Normal | Normal |
| Cell ID |  |  | 0 | 1 |
| Transmission mode |  |  | 3 | NA (Note 4) |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| (Note 2) |  | dB | Reference Value in Table 8.2.1.3.6-2 | 10 |
| Correlation and antenna configuration |  |  | Low (2x2) | Low (2x2) |
| Max number of HARQ transmissions |  |  | 4 | N/A |
| Redundancy version coding sequence |  |  | {0,1,2,3} | N/A |
| nw-BasedCRS-InterferenceMitigation-r15 |  |  | Disabled | Enabled |
| CRS transmission |  |  | On | CRS is configured with CRS muting pattern as [00000011111111111000] where every 20 subframes consist of [11] subframes with full system BW CRS and [9] subframes with CRS only on the center 6 PRBs (Note 5) |
| Note 1: Note 2: Cell 1 is the serving cell. Cell 2 is the interfering cell. Note 3: All cells are time-synchronous. Note 4: Cell 2 has no PDSCH/PDCCH configured Note 5: In the muting pattern [00000011111111111000] 1 indicates DL subframes with full system BW CRS configured and 0 indicates DL subframes with CRS only on the center 6 PRBs configured. |  |  |  |  |

Table 8.2.1.3.6-2: Minimum performance Large Delay CDD (FRC) with network-based CRS interference mitigation

| Test Number | Reference Channel | OCNG Pattern |  | Propagation Conditions (Note 1) |  | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Fraction of Maximum Throughput (%) | SNR (dB) (Note 2) |  |
| 1 | R.11-5 FDD | OP.1 FDD | NA | EVA5 | EVA5 | 70 | [15.1] | ≥2 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SNR corresponds to of Cell 1. |  |  |  |  |  |  |  |  |

<next change>

##### 8.2.2.3.6	Minimum Requirement 2 Tx Antenna Port (network-based CRS interference mitigation)

The requirements are specified in Table 8.2.2.3.6-2, with the addition of parameters in Table 8.2.2.3.6-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of open-loop spatial multiplexing performence with 2 transmit antennas when the PDSCH transmission in the serving cell is interfered by CRS of one dominant interfering cell with network-based CRS interference mitigation. In Table 8.2.2.3.6-1, Cell 1 is the serving cell, and Cell 2 is interfering cell. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1 and Cell 2 respectively.

Table 8.2.2.3.6-1: Test parameters for Larger Delay CDD (FRC) with network-based CRS interference mitigation

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz |  |
| Uplink downlink configuration |  |  | 4 |  |
| Special subframe configuration |  |  | 4 |  |
| Downlink power allocation |  | dB | -3 | 0 |
|  |  |  | -3 (Note 1) | 0 |
|  |  |  | 0 | 0 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| Cyclic Prefix |  |  | Normal | Normal |
| Cell ID |  |  | 0 | 1 |
| Transmission mode |  |  | 3 | NA (Note 4) |
| at antenna port |  | dBm/15kHz | -98 | N/A |
| (Note 3) |  | dB | Reference Value in Table 8.2.2.3.6-2 | 10 |
| Correlation and antenna configuration |  |  | Low (2x2) | Low (2x2) |
| Max number of HARQ transmissions |  |  | 4 | N/A |
| Redundancy version coding sequence |  |  | {0,1,2,3} | N/A |
| nw-BasedCRS-InterferenceMitigation-r15 |  |  | Disabled | Enabled |
| CRS transmission |  |  | On | CRS is configured with CRS muting pattern as [10UU11111110UU111000] where every 20 subframes consist of [11] subframes with full system BW CRS and [9] subframes with CRS only on the center 6 PRBs (Note 5) |
| Note 1: Note 2: Cell 1 is the serving cell. Cell 2 is the interfering cell. Note 3: All cells are time-synchronous. Note 4: Cell 2 has no PDSCH/PDCCH configured Note 5: The muting pattern [10UU11111110UU111000] follows the Uplink downlink configuration 4 where U indicates uplink subframes, 1 indicates DL subframes with full system BW CRS configured and 0 indicateds DL subframes including special subframes with CRS only on the center 6 PRBs configured. |  |  |  |  |

Table 8.2.2.3.6-2: Minimum performance Large Delay CDD (FRC) with network-based CRS interference mitigation

| Test Number | Reference Channel | OCNG Pattern |  | Propagation Conditions (Note 1) |  | Reference Value |  | UE Category |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Fraction of Maximum Throughput (%) | SNR (dB) (Note 2) |  |  |
| 1 | R.11-5 TDD | OP.1 TDD | NA | EVA5 | EVA5 | 70 | [15.0] | ≥2 |  |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SNR corresponds to of Cell 1. |  |  |  |  |  |  |  |  |  |

<next change>

Table A.3.1.1-1F: Overview of DL reference measurement channels (FDD, PDSCH Performance, Multi-antenna transmission (CRS))

| Duplex | Table | Name | BW | Mod | TCR | RB | RB Offset | UE Categ | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Two antenna ports |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.3.2.1-1 | R.10 FDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-1 | R.11 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-1 | R.11-1 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-1 | R.11-2 FDD | 5 | 16QAM | 1/2 | 25 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-1 | R.11-3 FDD | 10 | 16QAM | 1/2 | 40 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-1 | R.11-4 FDD | 10 | QPSK | 1/2 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-9 | R.11-5 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-1 | R.30 FDD | 20 | 16QAM | 1/2 | 100 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-1 | R.30-1 FDD | 15 | 16QAM | 1/2 | 75 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-1 | R.35 FDD | 10 | 64QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-1 | R.35-1 FDD | 20 | 64QAM | 0.39 | 100 |  | 4 |  |
| FDD | Table A.3.3.2.1-1 | R.35-2 FDD | 15 | 64QAM | 0.39 | 75 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-1 | R.35-3 FDD | 10 | 64QAM | 0.39 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-2 | R.35-4 FDD | 10 | 64QAM | 0.47 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-2 | R.46 FDD | 10 | QPSK |  | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-2 | R.47 FDD | 10 | 16QAM |  | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-7 | R.47-1 FDD | 5 | 16QAM |  | 25 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-7 | R.47-2 FDD | 15 | 16QAM |  | 75 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-7 | R.47-3 FDD | 20 | 16QAM |  | 100 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-2 | R.11-5 FDD | 1.4 | 16QAM | 1/2 | 6 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-2 | R.11-6 FDD | 3 | 16QAM | 1/2 | 15 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-2 | R.11-7 FDD | 15 | 16QAM | 1/2 | 75 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-2 | R.11-8 FDD | 10 | QPSK | 3/5 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-2 | R.11-9 FDD | 10 | QPSK | 0.58 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-2 | R.11-10 FDD | 10 | QPSK | 0.67 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-2 | R.10-2 FDD | 5 | QPSK | 1/3 | 25 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-2 | R.10-3 FDD | 10 | 16QAM | 0.58 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-2 | R.65 FDD | 10 | 256QAM | 0.55 | 50 |  | 11-15 |  |
| FDD | Table A.3.3.2.1-3 | R.62 FDD | 10 | 16QAM | 1/2 | 3 |  | 0 |  |
| FDD | Table A.3.3.2.1-3 | R.63 FDD | 10 | 64QAM | 1/2 | 1 |  | 0 |  |
| FDD | Table A.3.3.2.1-4 | R.79 FDD | 10 | 16QAM | 1/2 | 3 |  | M1, M2,≥ 0 |  |
| FDD | Table A.3.3.2.1-5 | R.81 FDD | 10 | QPSK | 1/10 | 6 |  | M1, ≥ 0 |  |
| FDD | Table A.3.3.2.1-6 | R.84 FDD | 10 | 16QAM | 1/2 | 39 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-6 | R.aa FDD | 10 | QPSK | 0.6-0.65 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.1-6 | R.bb FDD | 10 | 16QAM | 0.3-0.32 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.1-6 | R.87 FDD | 10 | 64QAM | 0.39 | 50 |  | ≥1 |  |
| FDD | Table A.3.3.2.1-8 | R.90 FDD | 10 | QPSK | 1/3 | 18 |  | M2 |  |
| FDD | Table A.3.3.2.1-8 | R.91 FDD | 10 | QPSK | 1/10 | 18 |  | M2 |  |
| FDD | Table A.3.3.2.1-8 | R.92-1 FDD | 10 | QPSK | 1/2 | 36 |  | ≥1 |  |
| FDD | Table A.3.3.2.1-8 | R.92-2 FDD | 10 | QPSK | 1/2 | 36 |  | ≥1 |  |
| FDD | Table A.3.3.2.1-4 | R.xx FDD | 10 | QPSK | 1/3 | 3 |  | M1, M2 |  |
| FDD | Table A.3.3.2.1-4 | R.yy FDD | 10 | 64QAM | 0.4 | 3 |  | M1, M2 |  |
| Four antenna ports |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.3.2.2-1 | R.12 FDD | 1.4 | QPSK | 1/3 | 6 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.2-1 | R.13 FDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.2-1 | R.14 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.2-1 | R.14-1 FDD | 10 | 16QAM | 1/2 | 6 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.2-1 | R.14-2 FDD | 10 | 16QAM | 1/2 | 3 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.2-1 | R.14-3 FDD | 20 | 16QAM | 1/2 | 100 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.2-1 | R.36 FDD | 10 | 64QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.2-1 | R.14-4 FDD | 1.4 | 16QAM | 1/2 | 6 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.2-1 | R.14-5 FDD | 3 | 16QAM | 1/2 | 15 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.2-1 | R.14-6 FDD | 5 | 16QAM | 1/2 | 25 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.2-1 | R.14-7 FDD | 15 | 16QAM | 1/2 | 75 |  | ≥ 2 |  |
| FDD | Table A.3.3.2.2-2 | R.72 FDD | 10 | 256QAM | 0.62 | 50 |  | ≥ 11 |  |
| FDD | Table A.3.3.2.2-2 | R.72-1 FDD | 5 | 256QAM | 0.62 | 25 |  | ≥ 11 |  |
| FDD | Table A.3.3.2.2-2 | R.72-2 FDD | 15 | 256QAM | 0.62 | 75 |  | ≥ 11 |  |
| FDD | Table A.3.3.2.2-2 | R.72-3 FDD | 20 | 256QAM | 0.62 | 100 |  | ≥ 11 |  |
| FDD | Table A.3.3.2.2-2 | R.73 FDD | 10 | 64QAM | 0.43 | 50 |  | ≥ 5 |  |
| FDD | Table A.3.3.2.2-2 | R.74 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 5 |  |
| FDD | Table A.3.3.2.2-3 | R.74-1 FDD | 5 | 16QAM | 1/2 | 25 |  | ≥ 5 |  |
| FDD | Table A.3.3.2.2-3 | R.74-2 FDD | 15 | 16QAM | 1/2 | 75 |  | ≥ 5 |  |
| FDD | Table A.3.3.2.2-3 | R.74-3 FDD | 20 | 16QAM | 1/2 | 100 |  | ≥ 5 |  |
| FDD | Table A.3.3.2.2-2 | R.85 FDD | 10 | 64QAM | 1/2 | 24 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.2-2 | R.93 FDD | 10 | 64QAM | 0.52 | 24 |  | ≥ 1 |  |
| FDD | Table A.3.3.2.2-4 | R.95 FDD | 10 | 16QAM | 1/2 | 3 |  | M2 |  |
| FDD | Table A.3.3.2.2-3 | R.xx1 FDD | 10 | 1024QAM |  | 50 |  | 20, ≥ 22 | UE DL Category |

<next change>

Table A.3.1.1-1I: Overview of DL reference measurement channels (TDD, PDSCH Performance, Multi-antenna transmission (CRS))

| Duplex | Table | Name | BW | Mod | TCR | RB | RB Offset | UE Categ | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Two antenna ports |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.4.2.1-1 | R.10 TDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-1 | R.11 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-1 | R.11-1 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-1 | R.11-2 TDD | 5 | 16QAM | 1/2 | 25 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-1 | R.11-3 TDD | 10 | 16QAM | 1/2 | 40 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-1 | R.11-4 TDD | 10 | QPSK | 1/2 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-11 | R.11-5 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-1 | R.30 TDD | 20 | 16QAM | 1/2 | 100 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-1 | R.30-1 TDD | 20 | 16QAM | 1/2 | 100 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-1 | R.30-2 TDD | 20 | 16QAM | 1/2 | 100 |  | 3 |  |
| TDD | Table A.3.4.2.1-1 | R.35 TDD | 10 | 64QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-1 | R.35-1 TDD | 20 | 64QAM | 0.39 | 100 |  | 4 |  |
| TDD | Table A.3.4.2.1-2 | R.35-2 TDD | 10 | 64QAM | 0.47 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-2 | R.46 TDD | 10 | QPSK |  | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-2 | R.47 TDD | 10 | 16QAM |  | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-9 | R.47-1 TDD | 5 | 16QAM |  | 25 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-9 | R.47-2 TDD | 15 | 16QAM |  | 75 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-9 | R.47-3 TDD | 20 | 16QAM |  | 100 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-2 | R.11-5 TDD | 1.4 | 16QAM | 1/2 | 6 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-2 | R.11-6 TDD | 3 | 16QAM | 1/2 | 15 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-2 | R.11-7 TDD | 5 | 16QAM | 1/2 | 25 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-2 | R.11-8 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-2 | R.11-9 TDD | 15 | 16QAM | 1/2 | 75 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-2 | R.11-10 TDD | 10 | QPSK | 3/5 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.1-2 | R.11-11 TDD | 10 | QPSK | 0.48-0.58 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-2 | R.11-12 TDD | 10 | QPSK | 0.54-0.66 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-2 | R.10-3 TDD | 10 | 16QAM | 0.57-0.58 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-3 | R.62 TDD | 10 | 16QAM | 1/2 | 3 |  | 0 |  |
| TDD | Table A.3.4.2.1-3 | R.63 TDD | 10 | 64QAM | 1/2 | 1 |  | 0 |  |
| TDD | Table A.3.4.2.1-4 | R.65 TDD | 20 | 256QAM | 0.6 | 100 |  | 11-15 |  |
| TDD | Table A.3.4.2.1-5 | R.67 TDD | 10 | 16QAM | 0.4 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-6 | R.79 TDD | 10 | 16QAM | 1/2 | 3 |  | M1, M2, ≥ 0 |  |
| TDD | Table A.3.4.2.1-7 | R.81 TDD | 10 | QPSK | 1/10 | 6 |  | M1, ≥ 0 |  |
| TDD | Table A.3.4.2.1-4 | R.84 TDD | 10 | 16QAM | 1/2 | 39 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-8 | R.aa TDD | 10 | QPSK | 0.54-0.64 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.1-8 | R.bb TDD | 10 | 16QAM | 0.27-0.32 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.4.2.1-8 | R.87 FDD | 10 | 64QAM | 0.39 | 50 |  | ≥1 |  |
| TDD | Table A.3.4.2.1-10 | R.90 TDD | 10 | QPSK | 1/3 | 18 |  | M2 |  |
| TDD | Table A.3.4.2.1-10 | R.91 TDD | 10 | QPSK | 1/10 | 18 |  | M2 |  |
| TDD | Table A.3.4.2.1-10 | R.92-1 TDD | 10 | QPSK | 1/2 | 36 |  | ≥1 |  |
| TDD | Table A.3.4.2.1-10 | R.92-2 TDD | 10 | QPSK | 1/2 | 36 |  | ≥1 |  |
| TDD | Table A.3.4.2.1-6 | R.xx TDD | 10 | QPSK | 1/3 | 3 |  | M1, M2 |  |
| TDD | Table A.3.4.2.1-6 | R.yy TDD | 10 | 64QAM | 0.4 | 3 |  | M1, M2 |  |
| Four antenna ports |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.4.2.2-1 | R.12 TDD | 1.4 | QPSK | 1/3 | 6 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.2-1 | R.13 TDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.2-1 | R.14 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.2-1 | R.14-1 TDD | 10 | 16QAM | 1/2 | 6 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.2-1 | R.14-2 TDD | 10 | 16QAM | 1/2 | 3 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.2-1 | R.43 TDD | 20 | 16QAM | 1/2 | 100 |  | ≥2 |  |
| TDD | Table A.3.4.2.2-1 | R.36 TDD | 10 | 64QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.2-1 | R.43-1 TDD | 1.4 | 16QAM | 1/2 | 6 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.2-1 | R.43-2 TDD | 3 | 16QAM | 1/2 | 15 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.2-1 | R.43-3 TDD | 5 | 16QAM | 1/2 | 25 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.2-1 | R.43-4 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.2-1 | R.43-5 TDD | 15 | 16QAM | 1/2 | 75 |  | ≥ 2 |  |
| TDD | Table A.3.4.2.2-2 | R.72 TDD | 10 | 256QAM | 0.62 | 50 |  | ≥ 11 |  |
| TDD | Table A.3.4.2.2-2 | R.72-1 TDD | 5 | 256QAM | 0.62 | 25 |  | ≥ 11 |  |
| TDD | Table A.3.4.2.2-2 | R.72-2 TDD | 15 | 256QAM | 0.62 | 75 |  | ≥ 11 |  |
| TDD | Table A.3.4.2.2-2 | R.72-3 TDD | 20 | 256QAM | 0.62 | 100 |  | ≥ 11 |  |
| TDD | Table A.3.4.2.2-2 | R.73 TDD | 10 | 64QAM | 0.44 | 50 |  | ≥ 5 |  |
| TDD | Table A.3.4.2.2-2 | R.74 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 5 |  |
| TDD | Table A.3.4.2.2-3 | R.74-1 TDD | 5 | 16QAM | 1/2 | 25 |  | ≥ 5 |  |
| TDD | Table A.3.4.2.2-3 | R.74-2 TDD | 15 | 16QAM | 1/2 | 75 |  | ≥ 5 |  |
| TDD | Table A.3.4.2.2-3 | R.74-3 TDD | 20 | 16QAM | 1/2 | 100 |  | ≥ 5 |  |
| TDD | Table A.3.4.2.2-2 | R.85 TDD | 10 | 64QAM | 1/2 | 24 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.2-2 | R.93 TDD | 10 | 64QAM | 0.50 | 24 |  | ≥ 1 |  |
| TDD | Table A.3.4.2.2-4 | R.95 TDD | 10 | 16QAM | 1/2 | 3 |  | M2 |  |

<next change>

Table A.3.3.2.1-9: Fixed Reference Channel two antenna ports

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | R.11-5 FDD |  |
| Channel bandwidth | MHz | 10 |  |
| Allocated resource blocks (Note 4) |  | 50 |  |
| Allocated subframes per Radio Frame |  | 9 |  |
| Modulation |  | 16QAM |  |
| Target Coding Rate |  | 1/2 |  |
| Information Bit Payload (Note 4) |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 10680 |  |
| For Sub-Frame 5 | Bits | N/A |  |
| For Sub-Frame 0 | Bits | 10680 |  |
| Number of Code Blocks (Notes 3 and 4) |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 2 |  |
| For Sub-Frame 5 | Bits | N/A |  |
| For Sub-Frame 0 | Bits | 2 |  |
| Binary Channel Bits (Note 4) |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 21648 |  |
| For Sub-Frame 5 | Bits | N/A |  |
| For Sub-Frame 0 | Bits | 17424 |  |
| Max. Throughput averaged over 1 frame (Note 4) | Mbps | 9.612 |  |
| UE Category |  | ≥ 2 |  |
| Note 1: 2 symbols allocated to PDCCH. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 4: Given per component carrier per codeword. Note 5: 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0,1,2,3,4,6,7,8,9. |  |  |  |

<next change>

Table A.3.4.2.1-11: Fixed Reference Channel two antenna ports

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | R.11-5 TDD |  |
| Channel bandwidth | MHz | 10 |  |
| Allocated resource blocks (Note 5) |  | 50 |  |
| Uplink-Downlink Configuration (Note 3) |  | 4 |  |
| Special subframe configuration |  | 4 |  |
| Allocated subframes per Radio Frame (D+S) |  | 7+1 |  |
| Modulation |  | 16QAM |  |
| Target Coding Rate |  | 1/2 |  |
| Information Bit Payload (Note 5) |  |  |  |
| For Sub-Frames 0,4,6,7,8,9 | Bits | 10680 |  |
| For Sub-Frames 1 |  | 7736 |  |
| For Sub-Frames 5 |  | NA |  |
| Number of Code Blocks (Notes 4 and 5) |  |  |  |
| For Sub-Frames 0,4,5,6,7,8,9 |  | 2 |  |
| For Sub-Frames 1 |  | 2 |  |
| For Sub-Frames 5 |  | NA |  |
| Binary Channel Bits (Note 5) |  |  |  |
| For Sub-Frames 0,4,5,6,7,8,9 | Bits | 21648 |  |
| For Sub-Frames 1 |  | 17424 |  |
| For Sub-Frames 5 |  | NA |  |
| Max. Throughput averaged over 1 frame (Note 5) | Mbps | 7.1816 |  |
| UE Category |  | ≥ 2 |  |
| Note 1: 2 symbols allocated to PDCCH. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4]. Note 3: As per Table 4.2-2 in TS 36.211 [4]. Note 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 5: Given per component carrier per codeword. Note 6: 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0,1,4,5,6,7,8,9. |  |  |  |

<end of the change>