3GPP TSG RAN Meeting #83																			RP-190472

Shenzhen, China, March 18 - 21, 2019

Agenda Item:	9.3.12

Source:	Ericsson

Title:		TP for TR 38.807: updates on the Global and CEPT spectrum availability and regulatory requirements for bands beyond 52.6GHz

Document for:	Discussion and Decision

# Introduction

At RAN plenary meeting #80 the study item for NR waveform beyond 52.6 GHz was approved [1]. One of the objectives defined in [1] is to “Identify target spectrum ranges”. According to the work plan this objective should be discussed in RAN plenary meetings #82 to #84. Ericsson submitted a contribution containing a survey on spectrum availability and regulatory requirements for CEPT/ECC for frequencies between 52.6GHz and 100GHz [2]. This contribution provides additional information for CEPT/ECC considering the extension of the frequency range of interest, which is between 52.6 GHz and 114.25 GHz.

Furthermore, this contribution provides amendments to contribution [3] which contains a draft text proposal for the TR 38.807 based on the submitted contributions to RAN#82 for study of NR waveform beyond 52.6 GHz. These amendments, focusing on Section 4.1, 4.2, and 4.2.1.1, contain clarifications and additional information considering the current status of the work towards WRC-19 agenda item 1.13.

Additionally, it is important that the final text for Section 4 of TR 38.807 gets an additional update capturing the final outcomes of WRC-19 AI 1.13. Particularly for the candidate frequencies bands currently studied under WRC-19 AI 1.13, where regulatory requirements and possible IMT identification are still undefined.

# Draft Text Proposal for Section 4 of TR38.807

The following is a draft text proposal for section 4 of TR38.807 and is for clarity made on top of the text proposal in [3].

==================== Start of Text Proposal =====================

# 4	Operational Requirements

## 4.1	Overview of Global Spectrum Availability

In 2015, the international telecommunication union (ITU) proposed 11 millimeter-wave bands between 24 and 86 GHz to be studied towards WRC-19 for possible identification for IMT. See Table 4.1-1 with the bands to be studied under AI 1.13 towards WRC-19.

Table 4.1-1 Bands to be studied under AI 1.13 towards WRC-19

| Candidate frequency bands which have allocations to the mobile service on a primary basis | Candidate frequency bands which required a co-primary mobile allocation |
| --- | --- |
| 24.25-27.5 GHz | 31.8-33.4 GHz, |
| 37-40.5 GHz | 40.5-42.5 GHz |
| 42.5-43.5 GHz | 47-47.2 GHz |
| 45.5-47 GHz |  |
| 47.2-50.2 GHz |  |
| 50.4-52.6 GHz |  |
| 66-76 GHz |  |
| 81-86 GHz |  |

Table 4.1-2 provides information on the allocation within the frequency range 52.6 GHz to 116 GHz in ITU Radio Regulation [5]. The column with comments contains information on protection requirements for incumbent services.

Table 4.1-2: Frequency bands in the range 52.6 to 116 GHz in radio regulation

| Frequency band (GHz) | Allocated to Mobile Service on a primary basis | Allocated to Fixed Service on a primary basis | Comments |
| --- | --- | --- | --- |
| 52.6-54.25 | No | No | EESS (passive) and SRS (passive), All emissions are prohibited in this band, footnote 5.340 |
| 54.25-55.78 | No | No | EESS (passive) and SRS (passive) |
| 55.78-59 | Yes | Yes | EESS (passive) and SRS (passive) This band available for high-density applications in the fixed service, footnote 5.547 |
| 59-59.3 | Yes | Yes | EESS (passive) and SRS (passive) Radiolocation |
| 59.3-64 | Yes | Yes | Radiolocation |
| 64-65 | Yes | Yes | This band available for high-density applications in the fixed service, footnote 5.547 |
| 65-66 | Yes | Yes | This band available for high-density applications in the fixed service, footnote 5.547 |
| 66-71 | Yes | No | WRC-19 AI 1.13 frequency band, sharing and compatibility studies and potential limitations information in section 2.2. |
| 71-76 | Yes | Yes | WRC-19 AI 1.13 frequency band, sharing and compatibility studies and potential limitations information in section 2.2. |
| 76-81 | No | No | Radiolocation |
| 81-86 | Yes | Yes | WRC-19 AI 1.13 frequency band, sharing and compatibility studies and potential limitations information in section 2.2. |
| 86-92 | No | No | EESS (passive) and SRS (passive), All emissions are prohibited in this band, footnote 5.340 |
| 92-94 | Yes | Yes | Radiolocation |
| 94-94.1 | No | No | Radiolocation |
| 94.1-95 | Yes | Yes | Radiolocation |
| 95-100 | Yes | Yes | Radiolocation |
| 100-102 | No | No | EESS (passive) and SRS (passive), All emissions are prohibited in this band, footnote 5.340 |
| 102-105 | Yes | Yes | N/A |
| 105-109.5 | Yes | Yes | SRS (passive) |
| 109.5-111.8 | No | No | EESS (passive) and SRS (passive), All emissions are prohibited in this band, footnote 5.340 |
| 111.8-114.25 | Yes | Yes | SRS (passive) |
| 114.25-116 | No | No | EESS (passive) and SRS (passive), All emissions are prohibited in this band, footnote 5.340 |

Within the range 52.6 to 116 GHz, the frequency bands 66-76 GHz (including 66-71 and 71-76 GHz) and 81-86 GHz are being studied under WRC-19 Agenda Item 1.13 for potential IMT identification. Results of sharing and compatibility studies, potential technical and regulatory conditions are included in Draft CPM Report [6], and the final decisions are to be made in WRC-19 with respect to IMT identification or no IMT identification, along with the corresponding technical and regulatory conditions.

For 66-71 GHz, Studies were carried out for the ISS, MSS (Earth-to-space) and MSS (space-to-Earth) indicating that sharing is feasible, with a need for separation distance in the order of few kilometers for the case of MSS (space-to-Earth). The need for studies addressing interference from IMT towards RNS is still under debate. Thus, final conclusions in the regulatory and technical conditions for this band cannot be drawn.

For 71-76 GHz, the studies performed towards WRC-23 indicate that Sharing with FS and FSS would be feasible. However, reduction of the IMT BS and UE unwanted emissions is needed to protect RLS in this frequency band.  Table 4.1-3 depicts the limits of the IMT BS and UE maximum unwanted emission levels according to the individual compatibility studies.

Table 4.1-3: Maximum unwanted emission levels to protect RLS in the frequency band 76-77GHz and 77-81 GHz

| Limits of unwanted emissions into 76-81 GHz from IMT stations Station | 76-77 GHz dB(W/200 MHz) | 77-81 GHz dB(W/200 MHz) |
| --- | --- | --- |
| BS | [TBD/−29.6/−31.5/< −37] | [TBD/−33/< −37] |
| UE | [TBD/−20/< −37] | [TBD/−35/< −37] |

For 81-86 GHz, Studies towards WRC-23 were carried out for the FS, FSS, RAS (in band and adjacent band), EESS (passive) and RLS. The results of those studies indicate that sharing with FS, FSS and RAS (in band and adjacent band) would be feasible. Notice that additional restrictions on the IMT BS and UE would be needed to ensure protection of EESS (passive) and RLS.

- In order to protect the RLS in the frequency band 76-81 GHz, the studies performed towards WRC-23 indicate that the unwanted emission limits into the 76-81 GHz band from IMT BSs and UEs operating in the 81-86 GHz band should comply with the limits given in Table below (note: the exact values are from individual compatibility studies):

Table 4.1-4: Maximum unwanted emission levels to protect RLS in the frequency band 76-77GHz and 77-81 GHz

| Limits of unwanted emissions into 76-81 GHz from IMT stations Station | 76-77 GHz dB(W/200 MHz) | 77-81 GHz dB(W/200 MHz) |
| --- | --- | --- |
| BS | [TBD/−29.6/−31.5/< −37] | [TBD/−33/< −37] |
| UE | [TBD/−20/< −37] | [TBD/−35/< −37] |

- In order to protect the EESS passive service in the frequency band 86-92 GHz, the studies performed towards WRC-23 indicate that the unwanted emission limits into the 86-92 GHz band from IMT BSs and UEs operating in the 81-86 GHz bands should comply with the limits given in Table below (note: the exact values are from individual compatibility studies):

Table 4.1-5: Maximum unwanted emission levels to protect EESS passive in the frequency band 86-92 GHz

| Limits of unwanted emissions into 86-92 GHz from IMT stations Station | 86-92 GHz dB(W/100 MHz) |
| --- | --- |
| BS | TBD/-43.6/-49.9/-31.3/-20 |
| UE | TBD/-43.5/-49.8/-31.2/-19.9 |

## 4.2	Country Specific Spectrum Availability and Spectrum Regulatory Requirements

4.2.1. Country Specific Licensing situation

Table 4.2-1 and 4.2-2 provide an overall summary of the current licensing situation  for data communication (Mobile and Fixed, excluding radar based telemetry) between frequencies 52.6GHz and 100GHz for various countries under ITU region 1, 2, and 3. Symbol ‘U’, and ‘L’, represent unlicensed spectrum and licensed spectrum (including light licensing and other licensing forms), respectively. Notice that Table 4.2-1 and 4.2-2 does not include information about the service allocated to each frequency band. It does neither include information about the frequency bands currently studied in AI 1.13 WRC-19 which are listed in Table 4.1-1. Also, it is worthwhile noticing that type of licensing for each frequency band is a national decision.

Table 4.2-1: Current Licensing situation for various countries between frequency 52.6GHz and 71GHz

| Region | Country | Frequency (GHz) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 52.6-54.25 | 54.25-55.78 | 55.78-56.9 | 56.9-57 | 57-58.2 | 58.2-59 | 59-59.3 | 59.3-64 | 64-65 | 65-66 | 66-71* |
| ITU Region 1 | Europe/CEPT |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Israel |  |  |  |  |  |  |  |  |  |  |  |
|  | South Africa |  |  |  |  | U (Mobile) |  |  |  | U (Mobile) |  |  |
| ITU Region 2 | USA |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Canada |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Brazil |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Mexico |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
| ITU Region 3 | China |  |  |  |  |  |  | U (Mobile) |  |  |  |  |
|  | Japan |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Korea |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | India |  |  |  |  |  |  |  |  |  |  |  |
|  | Taiwan |  |  |  |  |  |  |  |  |  |  |  |
|  | Singapore |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Australia |  |  |  |  | U (Mobile) |  |  |  |  |  |  |

* candidate frequency band for IMT identification under WRC-19 AI 1.13

Table 4.2-2: Current Licensing situation for various countries between frequency 71GHz and 100GHz

| Region | Country | Frequency (GHz) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 71-74* | 74-76* | 76-77 | 77-81 | 81-84* | 84-86* | 86-92 | 92-94 | 94-94.1 | 94.1-95 | 95-100 |
| ITU Region 1 | Europe/CEPT | L (Fixed) |  |  |  | L (Fixed) |  |  | L (Fixed) | L (Fixed) |  |  |
|  | Israel |  |  |  |  |  |  |  |  |  |  |  |
|  | South Africa | L |  |  |  | L (Fixed) |  |  |  |  |  |  |
| ITU Region 2 | USA | L (Fixed/Mobile) |  |  |  | L (Fixed/Mobile) |  |  | L (Fixed/Mobile) |  | L (Fixed/Mobile) |  |
|  | Canada | L |  |  |  | L |  |  |  |  |  |  |
|  | Brazil | L (Fixed) |  |  |  | L (Fixed) |  |  |  |  |  |  |
|  | Mexico | L (Fixed) |  |  |  | L (Fixed) |  |  |  |  |  |  |
| ITU Region 3 | China |  |  |  |  |  |  |  |  |  |  |  |
|  | Japan | L (Fixed) |  |  |  | L (Fixed) |  |  |  |  |  |  |
|  | Korea | L (Fixed) |  |  |  | L (Fixed) |  |  |  |  |  |  |
|  | India |  |  |  |  |  |  |  |  |  |  |  |
|  | Taiwan |  |  |  |  |  |  |  |  |  |  |  |
|  | Singapore | L (Fixed) |  |  |  | L (Fixed) |  |  |  |  |  |  |
|  | Australia |  |  |  |  |  |  |  |  |  |  |  |

* candidate frequency band for IMT identification under WRC-19 AI 1.13

### 4.2.1	ITU Region 1

#### 4.2.1.1	Europe and CEPT

General information for usages in CEPT can be found in THE EUROPEAN TABLE OF FREQUENCY ALLOCATIONS AND APPLICATIONS IN THE FREQUENCY RANGE 8.3 kHz to 3000 GHz (ECA TABLE). When it comes to Regulation about Short Range Devices additional information can be found in Recommendation ERC/REC 70-03 “Relating to the use of Short Range Devices (SRD)”.  Unwanted emission limits can be found in ECC Recommendation (02)/05 “Unwanted Emissions” and ERC/RECOMMENDATION 74-01 “UNWANTED EMISSIONS IN THE SPURIOUS DOMAIN”. It should however be pointed out that the Rec 74-01 is under review with estimated finalization date 2019-05-24.

The bands above 52.6 GHz that are discussed due to WRC-19 AI 1.13 (IMT), within CEPT are three, 66-71 GHz as IMT candidate band and the bands 71-76/81-86 GHz as no IMT bands. The band 71-76 GHz, paired with 81-86 GHz, is a fixed link band important for backhauling of 5G. Therefore, fixed link usage is expected to increase in the future.

From a European perspective, the focus should be on bands that are proposed for IMT as well as on the unlicensed band 57-66 GHz. The current status, seen by CEPT, for IMT bands can be found in the draft ECP, European Common Proposal and Brief for WRC-19 Agenda Item 1.13. There are no limitations concerning output powers or channel bandwidth in the 66-71 GHz band, only that the band 66-71 GHz is planned for “Future Civil systems”.

Table 4.2.1.1-1 and Table 4.2.1.1-2 provide summary of frequency allocations in the range of 52.6 and 114.25 GHz and regulatory requirements for existing Mobile and Fixed services in the frequency bands between 52.6 GHz and 114.25 GHz, respectively.

Table 4.2.1.1-1: Frequency bands in the range 52.6 to 100 GHz

| Frequency range in GHz | Comments | Region |
| --- | --- | --- |
| 52.6 to 54.25 | EESS service, all emissions are prohibited, (ref ITU-R Radio Regulation footnote 5.340 | ITU/CEPT |
| 55.78 to 57 | Recommended to be used by fixed services (ref: ERC Recommendation 12-12 and ITU-R Radio Regulation footnote 5.547) | ITU/CEPT |
| 57 to 66 | Several usage of Short range devices (ref ERC/REC 70-03) and EC Decision about SRD (ref COMMISSION IMPLEMENTING DECISION (EU) 2017/1483 of 8 August 2017) 57-64 GHz recommended by CEPT for fixed service limited to point to point (ref: ECC recommendation (09)01) 64-66 GHz recommended by CEPT for fixed service limited to point to point (ref: ECC recommendation (05)02) 63-64 GHz ITS band in Europe (ref: ECC decision (09)01) | EC/CEPT |
| 66 to 71 | Planned for “Future civil systems” There is currently no limitations on channel bandwidths, output powers etc. Recommended by CEPT for IMT identification (ref: draft new resolution [AI1.13-IMT 66/71 GHz, Output from ECC PT1 meeting in January 2019]). | CEPT |
| 71 to 76 and 81 to 86 | Recommended by CEPT for fixed services (ref: ECC recommendation (05)07) | CEPT |
| 77 to 81 | Automotive RADAR in Europe (ref: ECC decision (04)03 and ref Commission Decision of 8 July 2004 on the harmonisation of radio spectrum in the 79 GHz range for the use of automotive short-range radar equipment in the Community | EC/CEPT |
| 86 to 92, 100 to 102, 109.5 to 111.8 | EESS service, all emissions are prohibited (ref ITU-R Radio Regulation footnote 5.340) | ITU/CEPT |
| 92 to 94, 94.1 to 100, 102 to 109.5 and 111.8 to 114.25 | Recommended by CEPT for fixed services (ref: ECC recommendation (18)02) | CEPT |

Table 4.2.1.1-2: Europe/CEPT – Current Regulatory Requirements for spectrum between 52.6 GHz and 114.25 GHz

| Frequency band (GHz) | Power/Magnetic Field Requirements | Spectrum access and mitigation requirements | Modulation / maximum occupied bandwidth | Purpose/Node Placement requirements | Notes |
| --- | --- | --- | --- | --- | --- |
| 57 – 66 | 40 dBm mean e.i.r.p.. This refers to the highest power level of the transmitter power control range during the transmission burst if transmitter power control is implemented | Adequate spectrum sharing mechanism (e.g. Listen-before-Talk, Detect-And-Avoid) shall be implemented by the equipment | Not specified | Short Rage Devices - Wide Data Transmissions, defined in ERC Rec 70-03 | Fixed outdoor installations are not allowed. The maximum mean e.i.r.p. density is limited to 13 dBm/MHz. Point-to-point links of the Fixed Service are regulated by ECC/REC/ (05)02 and ECC/REC/ (09)01 |
| 66 – 71 | To be defined | To be defined | To be defined | To be defined | Recommended by CEPT for IMT identification WRC-19 AI 1.13 |
| 71-76, 81-86 | Maximum power limit= 35dBm Maximum EIRP= 85dBm (55 dBW) Minimum antenna gain= Pout (dBm) + 15; or 38 (whichever is the greater) maximum antenna gain= 85-Pout(dBm) Complete set of specification, ETSI EN 302 217 part 2-2 and part 3 | Access to the frequency bands by Authorization regimes or by Block assignment/auction regimes . | An aggregation of 250 MHz channels for wider channels as defined in ECC/REC/(05)07 (channel arrangements for channel sizes ranging from 250 to 4750 MHz) | Fixed Services defined in ECC Recommendation (05)07 To assist the planning of fixed links, self-coordination approach, similar to the “light licensing”, described in ECC Report 80 can be considered. |  |
| 92-94, 94.1-100, 102-109.5, 111.8-114.25 | Not defined See ECC Recommendation (18)02 | Not defined See ECC Recommendation (18)02 | Radio frequency channel/block arrangements should be derived by aggregation of 250 MHz channels. Inside a specific sub-band no upper limit is given to block size, provided that the blocks´ duplex separation is maintained. Details in ECC Recommendation (18)02 – Annex 3 | Fixed Services defined in ECC Recommendation (18)02. The choice of the appropriate assignment method and licensing regime for Fixed Service remains a decision for national administrations, based on several technical and not technical factors | To assist the planning of fixed links, self-coordination approach, similar to the “light licensing”, described in ECC Report 80 can be considered. |

=================== End of Text Proposal ========================

# References

- RP-181435, New SID: Study on NR beyond 52.6GHz, Intel, RAN Meeting #80, June 2018

- RP-182629, “Survey on spectrum availability and regulatory requirements for Region CEPT/EC for frequencies above 52.6 GHz,” Ericsson España S.A.

- RP-182486, “Draft TP to TR 38.807 – Operational requirements: spectrum availability and regulatory requirements”, Intel, RAN Meeting #82, Dec. 2018

When conducting studies in the band 24.5-27.5 GHz, to take into account the need to ensure the protection of existing earth stations and the deployment of future receiving earth stations under the EESS (space-to-Earth) and SRS (space-to-Earth) allocation in the frequency band 25.5-27 GHz