3GPP TSG-RAN4 Meeting #91	R4-1907053

Reno, US, May 13th – 17th 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.141 | CR | 0869 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 37.141: Simplification of capability sets definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-Perf |  |  |  |  |  | Date: |  |  | 2019-05-14 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Existing definition of capability sets is too complex, the readability will decrease further with introduction of new configurations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Any configuration with NB-IoT in-band and/or guard band operation is replaced by a new note. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Capability sets definition would remain too complex. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 4.7.1	Definition of Capability Sets (CS)

Capability set is defined as the BS capability to support certain RAT combinations in an operating band.

The manufacturer shall declare the supported capability set(s) according to Table 4.7.1-1 and Table 4.7.1.-2 for each supported operating band.

Table 4.7.1-1 Capability sets

| Capability Set supported by the BS | CS1 | CS2 | CS3 | CS4 | CS5 | CS6 | CS7 | CS16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Supported RATs | UTRA (MC) | E-UTRA (MC) 3 | UTRA, E-UTRA3 | GSM, UTRA | GSM, E-UTRA3 | GSM, UTRA, E-UTRA | GSM, UTRA, E-UTRA3 | NR, E-UTRA3 |
| Supported configurations | SR UTRA (SC, MC) | SR E-UTRA3 (SC, MC, CA) | MR UTRA + E-UTRA3 SR UTRA (SC, MC) SR E-UTRA3 (SC, MC, CA) | MR GSM + UTRA SR GSM (MCBTS) SR UTRA (SC, MC) | MR GSM + E-UTRA3 SR GSM (MCBTS) SR E-UTRA3 (SC, MC, CA) | MR GSM + UTRA + E-UTRA MR GSM + UTRA MR GSM + E-UTRA MR UTRA + E-UTRA SR GSM (MCBTS) SR UTRA (SC, MC) SR E-UTRA (SC, MC, CA) | MR GSM + UTRA2 MR GSM + E-UTRA3 MR E-UTRA3 + UTRA2 SR UTRA (SC, MC)2 SR E-UTRA3 (SC, MC) | MR E-UTRA3 + NR SR NR (SC, MC, CA) SR E-UTRA3 (SC, MC, CA) - -- - -- --- |
| Applicable BC | BC1, BC2 or BC3 | BC1, BC2 or BC3 | BC1, BC2 or BC3 | BC2 | BC2 | BC2 | BC2 | BC1, BC2 or BC3 |
| NOTE 1: MC denotes multi-carrier in single RAT; SC denotes single carrier; MR denotes multi-RAT; SR denotes single-RAT. NOTE 2: For this configuration related to BC2 bands, the support of UTRA in band 3 is declared by the manufacturer. NOTE 3: Includes optional (declared by the manufacturer) support of NB-IoT in-band and/or NB-IoT guard band operation within E-UTRA carrier(s). |  |  |  |  |  |  |  |  |

Table 4.7.1-2 Capability sets with NB-IoT standalone operation

| Capability Set supported by the BS | CS8 | CS9 | CS10 | CS11 | CS12 | CS13 | CS14 | CS15 | CS17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Supported RATs | NB-IoT standalone | GSM, NB-IoT standalone | UTRA, NB-IoT standalone | E-UTRA, NB-IoT standalone | GSM, UTRA, NB-IoT standalone | GSM, E-UTRA, NB-IoT standalone | UTRA, E-UTRA, NB-IoT standalone | GSM, UTRA, E-UTRA, NB-IoT standalone | NR, E-UTRA, NB-IoT standalone |
| e | SR NB-IoT standalone (SC, MC) | MR GSM + NB-IoT standalone SR GSM (MCBTS) SR NB-IoT standalone (SC, MC) | ee | MR E-UTRA + NB-IoT standalone SR E-UTRA (SC, MC, CA) SR NB-IoT standalone (SC, MC) | MR GSM + UTRA + NB-IoT standalone SR GSM (MCBTS) SR UTRA (SC, MC) SR NB-IoT standalone (SC, MC) MR GSM + NB-IoT standalone MR UTRA + NB-IoT standalone MR GSM + UTRA | MR GSM + E-UTRA + NB-IoT standalone SR GSM (MCBTS) SR E-UTRA (SC, MC, CA) SR NB-IoT standalone (SC, MC) MR GSM + NB-IoT standalone MR E-UTRA + NB-IoT standalone MR GSM + E-UTRA | MR UTRA + E-UTRA + NB-IoT standalone SR UTRA (SC, MC) SR E-UTRA (SC, MC, CA) SR NB-IoT standalone (SC, MC) MR UTRA + NB-IoT standalone MR E-UTRA + NB-IoT standalone MR UTRA + E-UTRA | MR GSM + UTRA2 + NB-IoT standalone MR GSM + E-UTRA + NB-IoT standalone MR UTRA2 + E-UTRA + NB-IoT standalone MR GSM + NB-IoT standalone MR UTRA2 + NB-IoT standalone MR E-UTRA + NB-IoT standalone MR GSM + UTRA2 MR GSM + E-UTRA MR E-UTRA + UTRA2 SR UTRA (SC, MC)2 SR E-UTRA (SC, MC) SR NB-IoT standalone (SC, MC) | e (SC, MC, CA) SR E-UTRA3 (SC, MC, CA) SR NB-IoT standalone (SC, MC) - - MR E-UTRA + NB-IoT standalone MR NR + NB-IoT standalone -- -- MR NR + E-UTRA + NB-IoT standalone - -- |
| Applicable BC | BC1, BC2 or BC3 | BC2 | BC1, BC2 or BC3 | BC1, BC2 or BC3 | BC2 | BC2 | BC1, BC2 or BC3 | BC2 | BC1, BC2 or BC3 |
| NOTE 1: MC denotes multi-carrier in single RAT; SC denotes single carrier; MR denotes multi-RAT; SR denotes single-RAT. NOTE 2: For this configuration related to BC2 bands, the support of UTRA in band 3 is declared by the manufacturer. NOTE 3: Includes optional (declared by the manufacturer) support of NB-IoT in-band and/or NB-IoT guard band operation within E-UTRA carrier(s). |  |  |  |  |  |  |  |  |  |

The applicable test configurations for each RF requirement are defined in sub-clause 5.1 and 5.2 for the declared capability set(s). For a BS declared to be capable of multi-band operation, the applicable test configurations for each RF requirement are defined in sub-clause 5.3 for the declared capability set(s).

NOTE:	Not every supported configuration within a CS is tested, but the tables in sub-clause 5.1, 5.2 and 5.3 provide a judicious choice among the supported configurations and test configurations to ensure proper test coverage.