3GPP TSG RAN Meeting #84		 RP-191593

Newport Beach, CA, U.S.A., June 3rd – 6th, 2019	Revision of RP-190991

Agenda item:	9.2.1

Source: 	Qualcomm Incorporated

Title: 	Single Tx UL LTE-NR DC

Document for:	Discussion/Decision

# Discussion

In the current Rel-15 RAN1 feature list [1], there are two features targeting SUO Case 1 operation; FG 6-13 and 8-1a, both listed below.

| 6-13 | Case 1 Single Tx UL LTE-NR DC | 1) Case 1: DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell 2) HARQ subframe offset |  | Yes |  | Type 2 | Yes | Yes |  | This is a UE feature for LTE for a LTE/NR dual connectivity UE |  |  | Mandatory with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8-1a | Operation A with single UL Tx case 1 |  | EN-DC | Yes |  | Type 2 | Yes | Yes |  | RP-172833 |  | Mandatory with capability signaling conditioned that UE does not support dynamic power sharing, optional for UEs supporting dynamic power sharing | Mandatory with capability signaling conditioned that UE does not support dynamic power sharing, i.e., UE indicate “0” as non-support for 8-1, optional for UEs supporting dynamic power sharing |

These two features were introduced for two different use cases:

- FG 6-13: Introduced for single Tx capability, such as band combinations with difficult IMD, or certain intra-band contiguous EN-DC band combinations, e.g. b71-n71

  - Based on the agreed Way Forward in [2]

- FG 8-1a: Introduced for power sharing for Type 2 (not dynamic power sharing capable) UEs

  - Based on the agreed Way Forward in [3]

Mandatory / optional status for FG 6-13 has not been discussed in RAN1. The current mandatory designation was made at the Plenary, but not based on prior agreements and the current description is not aligned with the common understanding.

We make the following proposal.

Proposal:  Adopt either Option (A) or Option (B) below

Option A

| 6-13 | Case 1 Single Tx UL LTE-NR DC | 1) Case 1: DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell 2) HARQ subframe offset |  | Yes |  | Type 2 | Yes | Yes |  | This is a UE feature for LTE for a LTE/NR dual connectivity UE |  |  | Mandatory with capability signaling conditional on the UE not supporting simultaneous dual-Tx operation in the band combination; optional if the UE supports simultaneous dual-Tx operation in the band combination |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Option B

| 6-13 | Case 1 Single Tx UL LTE-NR DC | 1) Case 1: DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell 2) HARQ subframe offset |  | Yes |  | Type 2 | Yes | Yes |  | This is a UE feature for LTE for a LTE/NR dual connectivity UE |  |  | Mandatory with capability signaling Conditionally mandatory with capability signalling for UEs that do not support dynamic power sharing and for UEs that indicate single UL transmission for any BC, and optional otherwise. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| […] | […] | […] | […] | […] |  | […] | […] | […] |  | […] |  | […] | […] |
| 8-1a | Operation A with single UL Tx case 1 |  | EN-DC | Yes |  | Type 2 | Yes | Yes |  | RP-172833 |  | Mandatory with capability signaling conditioned that UE does not support dynamic power sharing, optional for UEs supporting dynamic power sharing | Mandatory with capability signaling conditioned that UE does not support dynamic power sharing, i.e., UE indicate “0” as non-support for 8-1, optional for UEs supporting dynamic power sharing Conditionally mandatory with capability signalling for UEs that do not support dynamic power sharing and for UEs that indicate single UL transmission for any BC, and optional otherwise. |

Isolated impact analysis:

Note that agreeing to the proposal above would not introduce backward compatibility issues and there is no need to introduce any new signaling.

# Conclusions

In this contribution, the following proposal has been made:

Proposal:  Make changes according to either Option (A) or Option (B) in the NR feature list.

# References

- R1-1907862, “RAN1 NR UE features”, NTT DOCOMO

- RP-172064, “Single Tx switched UL”, Qualcomm, Intel

- RP-172833, “Way Forward on NR EN-DC power sharing capability”, Qualcomm

- RP-172100, “LS on single Tx switched UL”, RAN, Qualcomm, Intel

- RP-190991, “Single Tx UL LTE-NR DC”